| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: San Diego, USA, 20–26 February 2016 | Document: JVET-B0057 |
| --- | --- |

| Title: | Evaluation of some intra-coding tools of JEM1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy Huawei Research Center, 1/7, Altufievskoe shosse, Moscow, Russia | Tel: Email: | +7-495-660-44-59 alexey.filippov@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This contribution presents an evaluation of some of JEM1.0 intra-coding tools, specifically: 4-tap interpolation filter for intra prediction, position dependent intra prediction combination, adaptive reference sample smoothing and MPI. Simulations include “off-tests” as provided in [1] as well as a brief tools efficiency analysis. Tools efficiency is estimated by calculating a ration of coding gain increase to encoder complexity.

# Introduction

The proposal studies some of JEM1.0 intra-coding tools (see Table 1). “Off-tests” have been performed within the same conditions as in [1] using the same macro values given in table 2.

Table 1. Studied JEM1.0 intra-coding tools.

| Acronym | Macro name | Tool |
| --- | --- | --- |
| 4TAP | VCEG_AZ07_INTRA_4TAP_FILTER | 4-tap interpolation filter for intra prediction |
| PDPC | COM16_C1046_PDPC_INTRA | Position dependent intra prediction combination |
| ARSS | COM16_C983_RSAF | Adaptive reference sample smoothing |
| MPI | VCEG_AZ05_INTRA_MPI | Multi-parameter intra prediction |

Table 2. Macro values used in “tool-off” simulations as specified in [1].

| MACRO VALUES | Variations | 4TAP | PDPC & MPI | ARSS | w/o PREVENT_ OVERSMOOTHING |
| --- | --- | --- | --- | --- | --- |
| ALF_HM3_REFACTOR |  | 1 | 1 | 1 | 1 |
| COM16_C806_VCEG_AZ10_SUB_PU_TMVP |  | 1 | 1 | 1 | 1 |
|  | COM16_C806_HEVC_MOTION_CONSTRAINT_REMOVAL | 1 | 1 | 1 | 1 |
|  | COM16_C806_DISABLE_4X4_PU | 1 | 1 | 1 | 1 |
|  | COM16_C806_GEN_MRG_IMPROVEMENT | 1 | 1 | 1 | 1 |
| COM16_C806_OBMC |  | 1 | 1 | 1 | 1 |
| COM16_C806_EMT |  | 1 | 1 | 1 | 1 |
| COM16_C806_T64 |  | 1 | 1 | 1 | 1 |
| COM16_C806_LARGE_CTU |  | 1 | 1 | 1 | 1 |
| COM16_C806_LMCHROMA | (ChromaQPOffset = 0) | 1 | 1 | 1 | 1 |
|  | COM16_C806_CR_FROM_CB_LAMBDA_ADJUSTMENT | 1 | 1 | 1 | 1 |
| VCEG_AZ07_IMV |  | 1 | 1 | 1 | 1 |
| VCEG_AZ07_FRUC_MERGE |  | 1 | 1 | 1 | 1 |
| VCEG_AZ06_IC |  | 1 | 1 | 1 | 1 |
| VCEG_AZ07_INTRA_4TAP_FILTER |  | 0 | 1 | 1 | 1 |
| VCEG_AZ07_INTRA_BOUNDARY_FILTER |  | 1 | 1 | 1 | 1 |
|  | VCEG_AZ07_INTRA_BOUNDARY_FILTER_MULTI_LINE | 1 | 1 | 1 | 1 |
| VCEG_AZ07_INTRA_65ANG_MODES |  | 1 | 1 | 1 | 1 |
| VCEG_AZ07_ECABAC |  | 1 | 1 | 1 | 1 |
|  | VCEG_AZ07_CTX_RESIDUALCODING | 1 | 1 | 1 | 1 |
|  | VCEG_AZ07_BAC_ADAPT_WDOW | 1 | 1 | 1 | 1 |
|  | VCEG_AZ07_INIT_PREVFRAME | 1 | 1 | 1 | 1 |
| VCEG_AZ05_MULTI_PARAM_CABAC |  | 1 | 1 | 1 | 1 |
| VCEG_AZ05_BIO |  | 1 | 1 | 1 | 1 |
|  | COM16_C1045_BIO_HARMO_IMPROV | 1 | 1 | 1 | 1 |
| COM16_C1016_AFFINE |  | 1 | 1 | 1 | 1 |
| COM16_C983_RSAF |  | 1 | 1 | 0 | 1 |
|  | COM16_C983_RSAF_PREVENT_OVERSMOOTHING | 1 | 1 | 0 | 0 |
|  | COM16_C983_RSAF_ESTIMATION_MODE_FULL | 1 | 1 | 1 | 1 |
| COM16_C1044_NSST |  | 1 | 1 | 1 | 1 |
|  | VCEG_AZ05_ROT_TR | 0 | 0 | 0 | 0 |
| COM16_C1046_PDPC_INTRA |  | 1 | 0 | 1 | 1 |
|  | VCEG_AZ05_INTRA_MPI | 0 | 0 | 0 | 0 |
|  | COM16_C1046_PDPC_RSAF_HARMONIZATION | 1 | 1 | 1 | 1 |

# Technical description

Simulations for “off-tests” were performed for CTC, full-length test sequences. Results are given in Tables 3-6. Reference is JEM1.0, second reference is HM16.6.

Table 3. JEM1 without 4-tap inta interpolation filter

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | 0.12% | 0.18% | 0.27% | 99% | 110% |
| Class B | 0.53% | 0.67% | 0.77% | 101% | 106% |
| Class C | 0.74% | 1.02% | 0.95% | 99% | 100% |
| Class D | 0.48% | 0.77% | 0.78% | 99% | 96% |
| Class E | 1.00% | 1.41% | 1.67% | 99% | 107% |
| Class F | 0.39% | 0.73% | 0.67% | 99% | 102% |
| Overall (Ref) | 0.52% | 0.77% | 0.81% | 99% | 103% |
| Overall (Ref-2) | -13.55% | -11.79% | -11.73% | 1940% | 155% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | 0.02% | 0.37% | 0.17% | 101% | 105% |
| Class B | 0.25% | 0.37% | 0.54% | 102% | 106% |
| Class C | 0.30% | 0.52% | 0.26% | 100% | 104% |
| Class D | 0.21% | 0.49% | 0.24% | 99% | 103% |
| Class E |  |  |  |  |  |
| Class F | 0.13% | 0.09% | -0.02% | 101% | 109% |
| Overall (Ref) | 0.19% | 0.37% | 0.25% | 101% | 106% |
| Overall (Ref-2) | -19.86% | -16.09% | -15.06% | 547% | 678% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.10% | 0.41% | 0.51% | 101% | 107% |
| Class C | 0.13% | 0.62% | 0.46% | 99% | 103% |
| Class D | 0.08% | 0.27% | 0.27% | 99% | 98% |
| Class E | 0.13% | 1.10% | 0.38% | 99% | 104% |
| Class F | 0.06% | -0.20% | -0.19% | 100% | 106% |
| Overall (Ref) | 0.10% | 0.41% | 0.29% | 100% | 104% |
| Overall (Ref-2) | -16.53% | -21.79% | -22.32% | 385% | 466% |
|  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.09% | 0.30% | 0.34% | 101% | 110% |
| Class C | 0.14% | 0.38% | -0.14% | 99% | 102% |
| Class D | 0.15% | 0.74% | 0.07% | 99% | 105% |
| Class E | 0.13% | 1.14% | 1.82% | 99% | 115% |
| Class F | 0.03% | 0.47% | 0.19% | 101% | 108% |
| Overall (Ref) | 0.11% | 0.56% | 0.38% | 100% | 108% |
| Overall (Ref-2) | -19.24% | -23.70% | -23.93% | 349% | 292% |

Table 4. JEM1 without PDPC and MPI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | 1.40% | 0.78% | 1.43% | 85% | 111% |
| Class B | 0.94% | 0.62% | 0.75% | 86% | 107% |
| Class C | 0.63% | 0.63% | 0.69% | 84% | 102% |
| Class D | 0.58% | 0.40% | 0.28% | 84% | 97% |
| Class E | 1.04% | 0.61% | 0.17% | 86% | 104% |
| Class F | -0.13% | -0.07% | -0.08% | 84% | 102% |
| Overall (Ref) | 0.74% | 0.50% | 0.56% | 85% | 104% |
| Overall (Ref-2) | -13.37% | -12.03% | -11.94% | 1653% | 156% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | 0.48% | -0.27% | 0.52% | 99% | 103% |
| Class B | 0.57% | 0.13% | 0.29% | 100% | 105% |
| Class C | 0.38% | 0.17% | 0.34% | 98% | 104% |
| Class D | 0.29% | 0.11% | 0.16% | 97% | 100% |
| Class E |  |  |  |  |  |
| Class F | 0.00% | -0.20% | -0.11% | 99% | 109% |
| Overall (Ref) | 0.35% | -0.01% | 0.24% | 99% | 104% |
| Overall (Ref-2) | -19.73% | -16.33% | -15.05% | 535% | 670% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.11% | 0.28% | 0.29% | 100% | 108% |
| Class C | 0.12% | 0.35% | 0.42% | 97% | 103% |
| Class D | 0.11% | -0.38% | 0.30% | 97% | 101% |
| Class E | 0.04% | 0.13% | -0.34% | 99% | 104% |
| Class F | 0.03% | -0.06% | -0.32% | 99% | 103% |
| Overall (Ref) | 0.08% | 0.07% | 0.10% | 98% | 104% |
| Overall (Ref-2) | -16.55% | -22.03% | -22.43% | 379% | 466% |
|  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.12% | 0.32% | 0.38% | 98% | 110% |
| Class C | 0.13% | 0.48% | -0.13% | 96% | 99% |
| Class D | 0.16% | 0.50% | 1.27% | 96% | 103% |
| Class E | 0.21% | 0.55% | 0.43% | 98% | 109% |
| Class F | 0.16% | 0.17% | -0.12% | 99% | 106% |
| Overall (Ref) | 0.15% | 0.39% | 0.36% | 97% | 105% |
| Overall (Ref-2) | -19.20% | -23.82% | -23.89% | 341% | 285% |

Table 5. JEM1 without ARSS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | 0.52% | 1.05% | 0.82% | 54% | 114% |
| Class B | 0.81% | 1.55% | 1.67% | 58% | 110% |
| Class C | 0.80% | 1.21% | 1.20% | 57% | 101% |
| Class D | 0.61% | 0.95% | 0.81% | 56% | 98% |
| Class E | 0.83% | 0.85% | 0.63% | 58% | 102% |
| Class F | 0.41% | 0.65% | 0.44% | 60% | 101% |
| Overall (Ref) | 0.66% | 1.07% | 0.97% | 57% | 105% |
| Overall (Ref-2) | -13.42% | -11.52% | -11.57% | 1115% | 157% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | 0.11% | 0.57% | 0.42% | 92% | 106% |
| Class B | 0.34% | 1.06% | 1.02% | 94% | 104% |
| Class C | 0.29% | 0.58% | 0.70% | 91% | 105% |
| Class D | 0.20% | 0.59% | 0.97% | 90% | 102% |
| Class E |  |  |  |  |  |
| Class F | 0.31% | 0.28% | 0.49% | 92% | 108% |
| Overall (Ref) | 0.25% | 0.64% | 0.73% | 92% | 105% |
| Overall (Ref-2) | -19.80% | -15.82% | -14.63% | 498% | 676% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.08% | 0.60% | 0.62% | 94% | 110% |
| Class C | 0.16% | 0.51% | 0.93% | 90% | 100% |
| Class D | 0.08% | 0.23% | 0.31% | 90% | 98% |
| Class E | 0.28% | 1.49% | 0.24% | 97% | 104% |
| Class F | 0.24% | 0.59% | 0.42% | 94% | 103% |
| Overall (Ref) | 0.16% | 0.64% | 0.52% | 93% | 103% |
| Overall (Ref-2) | -16.48% | -21.61% | -22.16% | 358% | 464% |
|  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.04% | 0.46% | 0.70% | 89% | 113% |
| Class C | 0.10% | 0.69% | 0.22% | 86% | 105% |
| Class D | 0.08% | 0.29% | 0.58% | 86% | 100% |
| Class E | 0.28% | 1.37% | 0.87% | 94% | 108% |
| Class F | 0.14% | 0.67% | 0.12% | 91% | 103% |
| Overall (Ref) | 0.12% | 0.65% | 0.49% | 89% | 106% |
| Overall (Ref-2) | -19.23% | -23.62% | -23.83% | 311% | 287% |

Table 6. JEM1 without COM16_C983_RSAF_PREVENT_OVERSMOOTHING macro

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | -0.03% | 0.00% | 0.05% | 101% | 106% |
| Class B | 0.18% | 0.61% | 0.51% | 102% | 107% |
| Class C | 0.12% | 0.34% | 0.34% | 100% | 102% |
| Class D | 0.07% | 0.22% | 0.13% | 100% | 98% |
| Class E | 0.23% | 0.96% | 0.18% | 101% | 104% |
| Class F | 0.08% | 0.25% | 0.23% | 100% | 103% |
| Overall (Ref) | 0.11% | 0.38% | 0.25% | 101% | 103% |
| Overall (Ref-2) | -13.90% | -12.11% | -12.21% | 1963% | 155% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | -0.06% | -0.22% | -0.20% | 102% | 101% |
| Class B | 0.08% | 0.18% | 0.01% | 103% | 104% |
| Class C | -0.01% | -0.18% | -0.24% | 101% | 106% |
| Class D | -0.04% | 0.02% | 0.29% | 100% | 99% |
| Class E |  |  |  |  |  |
| Class F | 0.05% | -0.13% | -0.24% | 102% | 112% |
| Overall (Ref) | 0.01% | -0.05% | -0.07% | 102% | 104% |
| Overall (Ref-2) | -20.00% | -16.38% | -15.31% | 551% | 671% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.02% | 0.34% | 0.16% | 101% | 104% |
| Class C | -0.02% | -0.15% | 0.28% | 100% | 100% |
| Class D | 0.00% | 0.45% | 0.53% | 100% | 97% |
| Class E | 0.19% | 1.35% | 0.19% | 100% | 104% |
| Class F | 0.12% | -0.05% | -0.22% | 101% | 106% |
| Overall (Ref) | 0.05% | 0.34% | 0.19% | 100% | 102% |
| Overall (Ref-2) | -16.57% | -21.83% | -22.41% | 386% | 458% |
|  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A |  |  |  |  |  |
| Class B | 0.02% | 0.10% | 0.06% | 101% | 110% |
| Class C | -0.04% | 0.13% | -0.36% | 100% | 103% |
| Class D | 0.03% | 0.75% | 0.45% | 100% | 100% |
| Class E | 0.11% | 1.38% | 1.21% | 100% | 105% |
| Class F | 0.13% | -0.07% | -0.31% | 102% | 108% |
| Overall (Ref) | 0.05% | 0.39% | 0.15% | 101% | 105% |
| Overall (Ref-2) | -19.29% | -23.82% | -24.08% | 352% | 286% |

Tools efficiency analysis is performed as described in [2]. The basic idea behind this evaluation is to normalize coding gains of different tools by their complexity. We perform similar analysis, but for All intra mode and both “tool-off” (see Table 7) and “tool-on” simulations (see Table 8).

However, in Table 7 and Table 8 weighting parameter w is different from the one used in [2], because all intra mode and random access have different encoder to decoder complexity ratios.

For Class B simulation for “on-tests” we obtained the following ratios:

;.

Evidently , and hence importance factor w should be shifted to encoder complexity. For all intra modes, we found value of w equal to 3 to be a reasonable balance to estimate improvement of encoding and decoding times.

Table 7. Evaluation of tools efficiency, AI tool-off tests, full length

| Tool test | BD-rate |  |  |  | Complexity |  |  | Slope |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | BDYUV | Enc | Dec | C | tan(Angle) | Angle (degrees) |
| 4TAP | 0.52% | 0.77% | 0.81% | -0.59% | 99.41% | 103.26% | 97.71% | -0.2575 | -14.4400 |
| PDPC | 0.74% | 0.50% | 0.56% | -0.69% | 84.72% | 103.75% | 101.01% | 0.6816 | 34.2775 |
| ARSS | 0.66% | 1.07% | 0.97% | -0.75% | 57.12% | 104.64% | 107.24% | 0.1039 | 5.9299 |
| w/o PREVENT_OVERSMOOTHING | 0.11% | 0.38% | 0.25% | -0.16% | 100.61% | 103.39% | 97.30% | -0.0597 | -3.4142 |

Table 7 reveal that there could be problems to estimate the slope using JEM1 “Tool-off” tests. The most evident reason is tool interference in existing JEM1 tool set. Careful study of tools interaction is required to reveal and harmonize interfering JEM1 tools.

Evaluatoin results presented in Table 8 include two MPI experiments, named “MPI2” and “MPI4”. The difference between MPI2 and MPI4 tools is analyzed in details in [1].

Table 8. Evaluation of tools efficiency, AI tool-on tests, first 30 frames simulations

| Tool test | BD-rate |  |  |  | Complexity |  |  | Slope |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | BDYUV | Enc | Dec | C | tan(Angle) | Angle (degrees) |
| MPI2 | -0.98% | -1.06% | -1.21% | 1.01% | 176.58% | 97.91% | 117.57% | 0.0578 | 3.3053 |
| MPI4 | -1.41% | -1.55% | -1.70% | 1.46% | 328.16% | 97.37% | 155.07% | 0.0266 | 1.5212 |
| PDPC | -1.36% | -1.33% | -1.39% | 1.36% | 191.03% | 101.23% | 123.68% | 0.0574 | 3.2867 |
| ARSS | -0.90% | -1.14% | -1.03% | 0.95% | 160.34% | 98.30% | 113.81% | 0.0687 | 3.9313 |
| ARSS w/o full RQT based estimation | -0.38% | -0.55% | -0.55% | 0.43% | 127.61% | 99.04% | 106.18% | 0.0688 | 3.9342 |

From Table 8 it could be noticed that ARSS encoder complexity is rather high but still is much lower than in case of JEM1 “off test”. Having lowest value of C in Table 8, it has the highest value of C in Table 7. We suppose this effect is caused by lack of harmonization between ARSS and other intra-related coding tools.

# Conclusions

We fully support suggestions given in [1] to set up a set of exploration experiments (EEs) to achieve more harmonized JEM1 tool set. We expect EEs to reveal tool interferences to make it possible to provide a appropriate harmonization solution.

Based on JEM analysis presented above we would like to suggest thorough analysis of JEM1.0 intra-coding tools interaction at both encoder and decoder sides.

# References

[1] E.Alshina, A.Alshin, K. Choi and M. Park, “Performance of JEM1.0 tools analysis by Samsung,” JVET-B0022, February 2016.

[2] H. Schwarz, C. Rudat, M. Siekmann, B. Bross, D. Marpe and T. Wiegand, “Coding Efficiency / Complexity Analysis of JEM 1.0 coding tools for the Random Access Configuration,” JVET-B0044, February 2016.

# Patent rights declaration

Huawei Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).