| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JVET-W0072 |
| --- | --- |

| Title: | SJTU 4k test sequences evaluation report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Namuk Kim Seungsu Jeon Huik Jae Shim Byeungwoo Jeon Sungkyunkwan Univ, Cheoncheon-dong, Jangan-gu, Suwon-si, Gyeonggi-do, Korea | Tel: Email: | +82-31-290-7186 namukss@skku.edu jhsl0812@skku.edu waitnual@skku.edu bjeon@skku.edu |
| Source: | Sungkyunkwan University. |  |  |

_____________________________

- Abstract

This contribution presents encoding results of 10sec Shanghai Jiao Tong University (SJTU) test sequences reported in JCTVC-V0083 to evaluate the content objectively and observe clear visual artifact subjectively. For the evaluation, this contribution uses the HM-16.6 reference software, and PSNR is used to evaluate the performance. Some observed artifacts and distortions are reported in this document while all evaluation results are in attached excel table.

Characteristics of original sequences

This part pre-investigates the original 15 SJTU sequences subjectively to see some features which look complicated or hard to encode. Following, the 15 sequences are grouped to 4 sets under the criteria below:

Set 1: Dynamic sequences

(Bund Nightscape, Campfire Party, Fountains)

Set 2: Sequences with objects repeatedly appearing and disappearing

(Marathon, Runners, Rush Hour, Traffic Flow)

Set 3: Sequences with static and changing focus

(Library, Residential Building, Tall Buildings, Wood)

Set 4: Sequences with static and unchanging focus

(Construction Field, Traffic and Building, Tree Shade, Scarf)

Set 1: Dynamic

| Sequence name | First frame | Features of sequence |
| --- | --- | --- |
| Bund Nightscape |  | ㆍThere is a stretching phenomenon as each frame is recorded using long exposure ㆍThe screen at the center shines brightly ㆍPeople are walking in dark part on the right side of video ㆍThere are some textures in the lake on the right side of video and both part casting a shadow or not because of an object in the lake ㆍOverall, the buildings look complicated ㆍThe focus of video does not change |
| Campfire Party |  | ㆍThe ball moves dynamically ㆍThe person behind the fire and firewood appears and then disappears repeatedly ㆍAshes are often flying too much or too little ㆍThere are some words on the dark area and the flag on which some words are written is waving ㆍOverall, it is dark except where there is a ball ㆍThe focus does not change |
| Fountains |  | ㆍWater drops from the fountain are well expressed ㆍThe stream of water repeatedly appear and disappear due to discontinuous generation ㆍThe stream of water is overlapped with words ㆍWhen the stream of water falls into the fountain, waves are made in the fountain ㆍIt is static except the fountain ㆍThe focus does not change |

Set 2: Objects repeat appearing and disappearing

| Sequence name | First frame | Features of sequence |
| --- | --- | --- |
| Marathon |  | ㆍThe texture of the street is colorful from rain ㆍThere is a light on the puddle of the street ㆍThere are many people and they are wearing colorful clothes ㆍThere is a waving flag ㆍPeople are moving fast and repeatedly appearing and disappearing ㆍA raincoat reveals the wearing ㆍThere are detailed movement of muscles on a thigh ㆍThe focus does not change |
| Runners |  | ㆍIt is similar to ‘Marathon’ ㆍCompared to ‘Marathon’, the features are more detailed as the size of each person is large ㆍThere are movements of face, muscles, and clothes ㆍThe background is more complicated than that of ‘Marathon’ such as trees, buildings, and fountain ㆍThe focus does not change |
| Rush Hour |  | ㆍMany people are walking and the distant background is a blur ㆍThe features of people’s look and clothes are detailed ㆍEach person often is overlapped and then appears ㆍThe focus does not change |
| Traffic Flow |  | ㆍMany cars move in different direction and speed ㆍThe focus does not change |

Set 3: Static and Changing Focus

| Sequence name | First frame | Features of sequence |
| --- | --- | --- |
| Tall Buildings |  | ㆍMany skyscrapers look complicated but regular ㆍThere are many areas covered with building’s shadow or not ㆍToo small people are moving ㆍToo small cars are also moving ㆍMany areas are small and detailed such as people and cars as it is filmed from high ㆍThe focus changes |
| Library |  | ㆍPeople are moving slowly ㆍThere are a lot of thin lines ㆍThere are many areas covered with shadow or not to the banner ㆍThere is a cloud but it is not moving ㆍThe background is static ㆍThe focus changes |
| Wood |  | ㆍThere are many trees with different sized leaves ㆍThere is a distortion on the trees due to the strong sunlight and an occluded area ㆍThe focus changes |
| Residential Building |  | ㆍIt is static ㆍThere are a lot of dust on the window of old building ㆍThere is a window reflecting the sunlight ㆍMany electrical wires and pipes in the middle are twisted hard ㆍStairs, newspaper, and curtain loom on the window ㆍOverall, the buildings are covered with dust ㆍThere is an outdoor of air conditioner and laundry ㆍThe focus changes |

Set 4: Static and Unchanging Focus

| Sequence name | First frame | Features of sequence |
| --- | --- | --- |
| Construction Field |  | ㆍOverall, the video is static ㆍMachines are stained by dirt ㆍThere is a grainy texture on the concrete in the background of machines ㆍThere are many thin steel-frame structures ㆍThere is a detailed texture where some materials are stacked ㆍThe crane on thin rope appears in the middle of video ㆍThere is texture with various stones in the ground ㆍThe focus does not change |
| Traffic and Buildings |  | ㆍIt is divided into a static upper area and dynamic area where cars are moving fast ㆍThere are complicated buildings and the sky in the static area ㆍThere are moving cars and people in the dynamic area ㆍThe focus does not change |
| Tree Shade |  | ㆍThere are trees shaken with the wind and grass ㆍBesides them, it is static ㆍGrass has many color such as green, yellow, red, and brown ㆍIt is divided into a dark area and bright area due to the shadow of trees ㆍThe focus does not change |
| Scarf |  | ㆍIt films image hanging the fixed scarf ㆍIt is too blur on the right of the background ㆍSmall and round light repeats appearing and disappearing ㆍA man covering the whole picture is walking in the middle ㆍThe focus does not change |

Results and analysis

Objective evaluation

The tested SJTU sequences are 420, 8bits. This contribution uses the HM-16.6 under CTC LD-B and RA main with 32 I-periods to obtain encoding results for objective evaluation. Tested QP ranges from 17 to 42 with 5 steps (17, 22, 27, 32, 37, and 42). In the R-D curve, x-axis represents ‘log10(bitrate)’. The experimental results are in the attached ‘JCTVC-W0083.xls’.

Set 1: Dynamic

Figure 1: Set 1 R-D Curve under LD-B

Figure 2: Set 1 R-D Curve under RA

In Figures 1 and 2, ‘Bund Nightscape’ has ordinary R-D curve. But both ‘Campfire Party’ and ‘Fountains’ need more bits and have low PSNR than ‘Bund Nightscape’ at the high QP. But PSNR increases steeply by QP goes lower.

Set 2: Objects repeat appearing and disappearing

Figure 3: Set 2 R-D Curve with LD-B

Figure 4: Set 2 R-D Curve with RA

In Figures 3 and 4, ‘Rush Hour’ and ‘Traffic Flow’ show ordinary R-D curve. But, ‘Traffic Flow’ sequence needs a lot of bits to increase small amount of PSNR. Both ‘Marathon’ and ‘Runners’ need more bits and have low PSNR at the high QP. But PSNR increases steeply by QP change.

Set 3: Static and changing focus

Figure 5: Set 3 R-D Curve with LD-B

Figure 6: Set 3 R-D Curve with RA

In Figures 5 and 6, all the set 3 sequences have ordinary R-D curve. But to be the same PSNR, they need different amount of bits. ‘Wood’ needs the largest amount of bits. ‘Library’ needs the smallest amount of bits.

Set 4: Static and unchanging focus

Figure 7 : Set 4 R-D Curve with LD-B

Figure 8: Set 4 R-D Curve with RA

In Figures 7 and 8, ‘Traffic and Building’ and ‘Scarf’ have ordinary R-D curve. ‘Construction field’ sequence has high PSNR and low bitrate at high QP relatively, and ‘Tree Shade’ sequence has low PSNR and hight bitrate at high QP relatively. But at the low QP, ‘Construction field’ obtains low PSNR and high bitrate relatively. ‘Tree Shade’ obtains high PSNR and low bitrate relatively.

Subjective evaluation

This contribution also subjectively evaluates the encoding result of QP 37. The original sequences and encoded sequences are alternately displayed, and viewers compare both two videos. The results of subjective evaluation are as followings:

Set 1: Dynamic

| Sequence name | Subjective evaluation |
| --- | --- |
| Bund Nightscape | ㆍLoss of texture on the right of video such as lines on the road, waves on the lake, and trees ㆍVisible blocking artifacts in the dark background ㆍSmall people are blurred or disappear due to blocking artifacts ㆍ Frame # 7(LD) |
| Campfire Party | ㆍ Frame # 100(LD) Weak degradation on people’s face and clothes ㆍ Frame # 100(LD) Loss of texture on the soil |
| Fountains | ㆍ Frame # 267(LD) Loss of texture on the water drops and trees ㆍ Frame # 267(LD) Visible strong degradation on the water drops |

Set 2: Objects repeat appearing and disappearing

| Sequence name | Subjective evaluation |
| --- | --- |
| Marathon | ㆍ Frame # 32(LD) Loss of texture and degradation on the people and their clothes. ㆍ Frame # 13(RA) Visible distortion and ringing artifacts on the road. |
| Runners | ㆍ Frame # 6(LD) Loss of texture on the people’s muscles and trees ㆍ Frame # 6(LD) Visible distortion on the person’s face |
| Rush Hour | ㆍ Frame # 213(RA) Loss of texture on the people’s hair and clothes |
| Traffic Flow | ㆍ Frame # 155(RA) Strong degradation on the words and yellow lines on the street ㆍ Frame # 162(LD) Loss of texture on the grass and fence ㆍ Frame # 162(LD) Visible blocking artifacts on the dark area of the street |

Set 3: Static and Changing Focus

| Sequence name | Subjective evaluation |
| --- | --- |
| Tall Buildings | ㆍFrame # 6(RA) Loss of texture on the left top side building windows, river, tall buildings windows detail, road signatures. ㆍ Frame # 53(RA) Visible blocking artifacts in the building shadow. ㆍ Frame # 144(RA) Visible ringing artifacts on the road. ㆍ Frame # 159(RA) Visible distortion on the building shadow on the river. ㆍFrame # 159(LD) Visible degradation on building windows. ㆍ Frame # 236(RA) Small objects(Car, people, and architecture) are reduced on the building shadow side. |
| Library | ㆍ Frame # 36(RA) Loss of texture on the road. ㆍ Frame # 79(RA) Visible distortion on the people shadow. |
| Wood | ㆍ Frame # 132(RA) Loss of texture on the leaves and trees. ㆍ Frame # 214(LD) Visible blocking artifacts nearby piercing lights. |
| Residential Building | ㆍ Frame # 171(RA) Loss of texture on the wall |

Set 4: Static and Unchanging Focus

| Sequence name | Subjective evaluation |
| --- | --- |
| Construction Field | ㆍ Frame # 254(RA) Loss of texture on the concrete, stacked materials, and thin steel-frame structures ㆍ Frame # 53(LD) Visible blocking artifacts on stone walls ㆍ Frame # 244(RA) Degradation on the crane on thin rope |
| Traffic and Buildings | ㆍ Frame # 15(LD) Loss of texture on buildings in the background, trees, and grass on the guardrail ㆍ Frame # 15(LD) Ringing artifacts and visible blocking artifacts on the road ㆍ Frame # 224(RA) Blocking artifact on the guardrail |
| Tree Shade | ㆍ Frame # 125(LD) Degradation on the leaves |
| Scarf | ㆍ Frame # 102(RA) Visible blocking artifacts on the acrossing man's hair and face. |

Conclusion

From the obtained HEVC encoding results, this contribution suggests ordering the SJTU test sequences from the most interesting to the least by set:

Set 1: Dynamic sequences

- Campfire Party

- Fountains

- Bund Nightscape

Set 2: Objects repeatedly appearing and disappearing

- Traffic Flow

- Marathon

- Runners

- Rush Hour

Se t3: Static and changing focus sequences

- Tall Buildings

- Wood

- Library

- Residential Building

Set 4: Static and unchanging focus sequences

- Construction Field

- Tree Shade

- Traffic and Building

- Scarf

References

[1] W15791, “Work plan for investigation of test sequence encoding”, MPEG, Geneva, October 2015.

[2] JCTVC-V0083, “SJTU Test Sequences for video coding development”, JCT-VC, Geneva, October 2015.

[3] HM-16.6, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.6/