| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 01 June 2016 | Document: JVET-C_Notes_d0 |
| --- | --- |

| Title: | Meeting notes of the 3rd meeting of the Joint Video Exploration Team (JVET), Geneva, CH, 26 May – 01 June 2016 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from responsible coordinators of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Responsible coordinators |  |  |

_____________________________

- Summary

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its third meeting during 26 May – 01 June 2016 at the ITU-T premises in Geneva, CH. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations. For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.13 of this document.

The JVET meeting sessions began at approximately 1400 hours on Thursday 26 May 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wednesday 01 June 2016. Approximately XX people attended the JVET meeting, and approximately XX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard and evaluate compression technology designs proposed in this area.

One primary goal of the meeting was to review the work that was performed in the interim period since the second JVET meeting in producing the Joint Exploration Test Model 2 (JEM2). In this context, results from seven exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM3 were planned.

The JVET produced 4 output documents from the meeting (update):

- Algorithm description of Joint Exploration Test Model 2 (JEM2)

- Call for test materials for future video coding standardization

- JVET common test conditions and software reference configurations

- Description of Exploration Experiments on coding tools

For the organization and planning of its future work, the JCT-VC established X "ad hoc groups" (AHGs) to progress the work on particular subject areas. X Exploration Experiments (EE) were defined on particular subject areas of coding tool testing. The next four JVET meetings are planned for Sat. 15 – Fri. 21 Oct. 2016 under WG 11 auspices in Chengdu, CN, during Thu. 12 – Wed. 18 Jan. 2017 under ITU-T auspices in Geneva, CH, during Sat. 1 – Fri. 7 Apr. 2017 under WG 11 auspices in Hobart, AU, and during … .

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Exploration Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its second meeting during 20–26 Feb 2016 at the San Diego Marriott La Jolla in San Diego, US. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations.

## Meeting logistics

The JVET meeting sessions began at approximately 1400 hours on Thursday 26 May 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wednesday 01 June 2016. Approximately XX people attended the JVET meeting, and approximately XX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard and evaluate compression technology designs proposed in this area.

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2016_02_B_SanDiego/.

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the second JVET meeting in producing the Joint Exploration Test Model 2 (JEM2). In this context, results from seven exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM3 were planned.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the JEM software but have no normative effect are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the JEM description (an error, oversight, or messiness) or in the software are marked by the string "Decision (BF):".

This meeting report is based primarily on notes taken by the responsible leaders. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 16 May 2016. Any documents uploaded after 1159 hours Paris/Geneva time on Tuesday 17 May were considered "officially late", as around that time the 16th had ended in any time zone of the world.

All contribution documents with registration numbers JVET-C0071 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "B0071+" range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- JVET-C00XX (a proposal on …),

- …

The following technical design proposal contributions were both registered late and uploaded late:

- JVET-C00XX (a proposal on …),

- …

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JVET-C00XX (an information document on …),

- …

The following cross-verification reports were registered on time but were uploaded late: JVET-C00XX [uploaded 05-XX], … .

(Documents that were both registered late and uploaded late, other than technical proposal documents, are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-C00XX, … .

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly the meeting report JVET-B1000, JEM2 algorithm description JVET-B1001, the call for test materials JVET-B1002, JVET common test conditions and reference software configuration JVET-B1010, and the description of exploration experiments JVET-B1011, were approved. The JEM2 software implementation was also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the JEM software uses the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the JEM software, and for evaluating proposals for technology to be included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

For distribution of test sequences, a password protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI.

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT: Adaptive multi-core transform.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP: Advanced temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BIO: Bi-directional optical flow.

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPMVP: Contol-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EE: Exploration Experiment – a coordinated experiment conducted toward assessment of coding technology.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MDNSST: Mode dependent non-separable secondary transform.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- QTBT: Quadtree plus binary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatio-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

## Opening remarks

- Reviewed logistics, agenda, working practices

- Results of previous meeting: JEM, web site, software, etc.

- Goals of the meeting

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800–2000 hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

(Meeting sessions were chaired by J.-R. Ohm, G. J. Sullivan, and/or J. Boyce)

- Thu. 26 May, 1st day

  - 1400-1700 AHG reports and JEM analysis (chaired by JRO and GJS)

  - …

- Fri. 27 May, 2nd day

  - …

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized

- AHG reports (5) (section 2)

- Analysis and improvement of JEM (4) (section 3)

- Test material (9) (section 4)

- Exploration experiments (22) (section 5)

- Non-EE technology proposals (40) (section 6)

  - Transforms and coefficient coding (6)

  - Motion compensation and vector coding (14)

  - Intra coding (10)

  - Partitioning (3)

  - Other (7)

- Perceptual metrics and evaluation criteria (2) (section 7)

- Withdrawn (0) (section 8)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 9)

- Project planning (section 10)

- Output documents, AHGs (section 11)

# AHG reports (5)

Tool evaluation (AHG1) [E. Alshina, M. Karczewicz (co-chairs)]

JVET-C0002 JVET AHG report: JEM algorithm description editing (AHG2) [J. Chen, E. Alshina, J. Boyce]

JVET-C0003 JVET AHG report: JEM software development [X. Li, K.Suehring] [miss]

JVET-C0004 JVET AHG report: Test material (AHG4) [T. Suzuki, J. Chen, A. Norkin, J. Boyce] [miss]

Objective quality metrics (AHG5) [P. Nasiopoulos, M. Pourazad (co-chairs)]

# Analysis, development and improvement of JEM (4)

JVET-C0034 Open-source inspired workflow for JEM maintenance [E. Thomas (??)] [miss] [late]

JVET-C0037 Sequential/Parallel bitstreams unification for JVET CTC [R. Mullakhmetov, I. Sharonov, M. Sychev (Huawei)]

JVET-C0046 RExt coding tools support on JEM [T. Tsukuba, O. Nakagami, T. Suzuki (Sony)] [miss] [late]

JVET-C0070 NEXT test model software [A. Hallapuro, M. Hannuksela, J. Ridge, J. Lainema, A. Aminlou (Nokia)] [late]

# Test material (9)

JVET-C0021 GoPro test sequences for Virtual Reality video coding [A. Abbas (GoPro)]

JVET-C0028 Suggested 1080P Test Sequences Downsampled from 4K Sequences [H. Zhang, X. Ma, H. Yang (Huawei)]

JVET-C0029 Surveillance sequences for video coding development [H. Zhang, X. Ma, H. Yang (Huawei), W.Qiu (Hisilicon)]

JVET-C0041 Proposed test sequences for 1080p class [A. Norkin (Netflix)]

JVET-C0044 Response to B1002 Call for test materials: Five test sequences for screen content video coding [J. Guo, L. Zhao, T. Lin (Tongji Uni.), H. Yu (Futurewei)]

JVET-C0048 Lens distorted test sequence by an action camera for future video coding [K. Kawamura, S. Naito (KDDI Corp.)]

JVET-C0050 Test sequence formats for virtual reality video coding [K. Choi, V. Zakharchenko, M. Choi, E. Alshina (Samsung)] [miss] [late]

JVET-C0064 Nokia test sequences for virtual reality video coding [J. Ridge, M. M. Hannuksela (Nokia)] [late]

JVET-C0067 Ultra High Resolution (UHR) 360 Video [C. J. Murray (Panoaction)] [late]

# Exploration experiments (22)

JVET-C0010 Exploration Experiments on Coding Tools Report [E. Alshina, J. Boyce, Y.-W. Huang, S.-H. Kim, L. Zhang]

## EE1: Quad-tree plus binary-tree (QTBT)

JVET-C0024 EE2.1: Quadtree plus binary tree structure integration with JEM tools [H. Huang, K. Zhang, Y.-W. Huang, S. Lei (MediaTek)]

JVET-C0056 Cross-check of JVET-C0024 (QTBT) [M. W. Park, B. Jin, E. Alshina, C. Kim (Samsung)] [miss] [late]

JVET-C0088 EE: Cross-check of EE2.1 QTBT (JVET-C0024) [V. Seregin, J. Chen (Qualcomm)] [miss] [late]

## EE2: Non Square TU Partitioning

JVET-C0077 EE2.2: Non Square TU Partitioning [K. Rapaka, J. Chen, L. Zhang, W. –J. Chien, M. Karczewicz (Qualcomm)] [late]

## EE3: NSST and PDPC index coding

JVET-C0042 EE2.3: NSST-PDPC Harmonization [S.-H. Kim, A. Segall (Sharp)]

JVET-C0059 Cross-check of JVET-C0042 (NSST-PDPC) [K. Choi, M. Park, E. Alshina, C. Kim (Samsung)] [miss] [late]

JVET-C0087 EE: Cross-check of EE2.3 NSST-PDPC Harmonization (JVET-C0042) [V. Seregin (Qualcomm)] [miss] [late]

## EE4: De-quantization and scaling for next generation containers

JVET-C0095 EE2.4: De-quantization and Scaling for Next Generation Containers [Jie Zhao, Andrew Segall, Seung-Hwan Kim] [late]

## EE5: Improvements on adaptive loop filter

JVET-C0038 EE2.5: Improvements on adaptive loop filter [M. Karczewicz, L. Zhang, W.-J. Chien, X. Li (Qualcomm)]

JVET-C0057 Cross-check of JVET-C0038 (GALF) [K. Choi, M. Park, E. Alshina, C. Kim (Samsung)] [miss] [late]

JVET-C0036 Crosscheck of JVET-C0038 (EE2.5 Improvements on adaptive loop filter) [R. Chernyak (Huawei)] [late]

JVET-C0074 Cross-check of JVET-C0038 (EE2.5: Improvements on adaptive loop filter) [Y. Yamamoto, T. Ikai (Sharp)] [miss] [late]

JVET-C0091 Crosscheck for EE2.5 Improvements on adaptive loop filter (JVET-C0038) [X. Cai, L. Xu, Y. Chiu (Intel)] [miss] [late]

## EE6: Modification of Merge candidate derivation

JVET-C0035 EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning [S. Lee, W.-J. Chien, L. Zhang, J. Chen, M. Karczewicz (Qualcomm)]

JVET-C0085 Cross-check of JVET-C0035 (EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning) [H. Chen, H. Yang (Huawei)] [miss] [late]

JVET-C0060 Cross-check of JVET-C0035 (Modification of merge candidate derivation) [A. Tamse, E. Alshina, C. Kim (Samsung)] [miss] [late]

JVET-C0073 Cross-check of JVET-C0035 (EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning) [Y. Yasugi (Sharp)] [miss] [late]

## EE7: TU-level non-separable secondary transform

JVET-C0053 EE2.7: TU-level non-separable secondary transform [X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi (Qualcomm)]

JVET-C0058 Cross-check of JVET-C0053 (TU-NSST) [K. Choi, M. Park, E. Alshina, C. Kim (Samsung)] [miss] [late]

JVET-C0076 Cross-check of JVET-C0053 (TU-NSST) [V. Lorcy (bcom), P. Philippe (Orange), T. Biatek (TDF)] [miss] [late]

JVET-C0086 Cross-check of EE2.7: TU-level non-separable secondary transform [S.-H. Kim (Sharp)] [miss] [late]

# Non-EE Technology proposals (XX)

## Transforms and coefficient coding (6)

JVET-C0022 Proposed improvements to the Adaptive multiple Core transform [Pierrick Philippe (Orange), Victorien Lorcy (bcom)]

JVET-C0045 EE2.7-related: On secondary transform when primary transform is skipped [T. Tsukuba, O. Nakagami, T. Suzuki (Sony)]

JVET-C0089 Cross-check of JVET-C0045 On secondary transform when primary transform is skipped [J. Chen, K. Rapaka (Qualcomm)] [miss] [late]

JVET-C0063 EE2.7 related: Improved non-separable secondary transform [X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen (Qualcomm)]

JVET-C0075 Cross-check of JVET-0063 EE2.7 related: Improved non-separable secondary transform [S.-H. Kim(Sharp)] [miss] [late]

JVET-C0066 On Coefficient Scaling [D. B. Sansli, D. Rusanovskyy, J. Sole, A.K. Ramasubramonian, M. Karczewicz (Qualcomm)]

## Motion compensation and vector coding (14)

JVET-C0023 Predictors Elimination Technique for HEVC [M. Korman, O. Prosekov (??)]

JVET-C0025 Simplification of motion compensation filter for affine inter prediction [J. Nam, H. Jang, J. Lee, B. Lee, J. Lim (LGE)]

JVET-C0093 Cross-check of JVET-C0025: Simplification of motion compensation filter for affine inter prediction [H.Zhang, H. Yang (Huawei)] [miss] [late]

JVET-C0027 Simplification and improvements for BIO design in JEM2.0 [E. Alshina, A. Alshin (Samsung)]

JVET-C0065 Cross-check of JVET-C0027 [J. Lee, S. Yoo, J. Nam, J. Lim (LGE)] [miss] [late]

JVET-C0072 Cross-check of JVET-C0027 (BIO simplification and improvements) [X. Li (Qualcomm)] [miss] [late]

JVET-C0080 Cross-check of JVET-C0027 (Simplification and improvements for BIO design in JEM2.0) [T. Biatek (TDF), V. Lorcy (bcom), P. Philippe, P. Boissonade (Orange)] [miss] [late]

JVET-C0031 BIO improvement to reduce the encoder and decoder complexities [J. Lee, N. Park, J. Nam, J. Lim (LGE)]

JVET-C0052 The cross-check for BIO improvement to reduce the encoder and decoder complexities in JVET-C0031 [E. Alshina (Samsung)] [late]

JVET-C0047 Generalized bi-prediction for inter coding [C.-C. Chen, X. Xiu, Y. He, Y. Ye (InterDigital)]

JVET-C0092 Cross-check of JVET-C0047: Generalized bi-prediction for inter coding [S.-H. Kim (Sharp)] [miss] [late]

JVET-C0062 Improved affine motion prediction [F. Zou, J. Chen, M. Karczewicz, X. Li, H.-C. Chuang, W.-J. Chien (Qualcomm)]

JVET-C0083 Cross-check of JVET-C0062 Improved affine motion prediction [O. Nakagami (Sony)] [miss] [late]

JVET-C0068 Motion vector coding optimizations [J. Samuelsson, P. Wennersten, R. Yu, U. Hakeem (Ericsson)]

## Intra coding (10)

JVET-C0043 Arbitrary reference tier for intra directional modes [Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI)]

JVET-C0090 Cross-check of JVET-C0043 (Arbitrary reference tier for intra directional modes) [T. Ikai (Sharp)] [miss] [late]

JVET-C0055 Neighbor based intra most probable modes list derivation [V. Seregin, X. Zhao, A. Said, M. Karczewicz (Qualcomm)]

JVET-C0081 Crosscheck of neighbor based intra most probable modes list derivation in JVET-C0055 [H. Huang (MediaTek)] [miss] [late]

JVET-C0061 Decoder-side intra mode derivation [X. Xiu, Y. He, Y. Ye (InterDigital)]

JVET-C0094 Cross-check of JVET-C0061: Decoder-side intra mode derivation [X. Ma, H. Yang (Huawei)] [miss] [late]

JVET-C0071 Multiple line-based intra prediction [J. Li (Peking Univ.), B. Li, J. Xu (Microsoft), R. Xiong (Peking Univ.), G.-J. Sullivan (Microsoft)] [late]

JVET-C0082 Cross-check of JVET-C0071 (Multiple line-based intra prediction) [T. Ikai (Sharp)] [miss] [late]

JVET-C0096 Cross-check of JVET-C0071 (Multiple line-based intra prediction) [L. Zhang, V. Seregin (Qualcomm)] [miss] [late]

JVET-C0097 Cross-check of JVET-C0071 (Multiple line-based intra prediction) [P.-H. Lin, Y.-J. Chang, J.-S. Tu, C.-C. Lin, C.-L. Lin (ITRI)] [late]

## Partitioning (3)

JVET-C0039 Decoupled Luma/Chroma Transform Trees for Intra [F. Urban, T. Poirier, F. Le Léannec (Technicolor)]

JVET-C0079 Cross-check of JVET-C0039 Decoupled Luma/Chroma Transform Trees for Intra [O. Nakagami (Sony)] [miss] [late]

JVET-C0054 Grouped signalling for transform in QTBT [X. Zhao, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

## Other (7)

JVET-C0026 Tiles coding improvement for Inter pictures by improved merge list at tile boundaries [S. B. Raut (??)]

JVET-C0032 Experiment on polyphase subsampled sequence coding [E. Thomas (??)]

JVET-C0078 Cross-check of C0032 (polyphase subsampled sequence coding) [P. Philippe (Orange), V. Lorcy (bcom), T. Biatek (TDF)] [miss] [late]

JVET-C0040 Adaptive Clipping in JEM2.0 [F. Galpin, P. Bordes, F. Le Léannec (Technicolor)]

JVET-C0084 Cross-check of C0040 (Adaptive Clipping) [P. Philippe (Orange), V. Lorcy (bcom), T. Biatek (TDF)] [miss] [late]

JVET-C0049 Extended deblocking-filter process for large block boundary [K. Kawamura, Q. Yao, S. Naito (KDDI Corp.)] [miss] [late]

JVET-C0069 Direction-dependent scan order with JEM tools [S. Iwamura, A. Ichigaya (??)] [late]

# Perceptual metrics and evaluation criteria (2)

JVET-C0030 Perceptual Quality Assessment Metric MS-SSIM [H.Zhang, X. Ma, Y.Zhao, H. Yang (Huawei)]

JVET-C0033 On comparison criteria for Virtual Reality video coding schemes [E. Thomas (??)]

# Withdrawn (0)

# Joint Meetings, BoG Reports, and Summary of Actions Taken

## General (update)

The setup of Exploration Experiments was discussed, and an initial draft of the EE document was reviewed in the plenary (chaire by JRO). This included the list of all tools that are intended to be investigated in EEs during the subsequent meeting cycle:

EE1: Quad-tree plus binary-tree (QTBT)

EE2: Non Square TU Partitioning

EE3: NSST and PDPC index coding

EE4: De-quantization and scaling for next generation containers

EE5: Improvements on adaptive loop filter

EE6: Modification of Merge candidate derivation

EE7: TU-level non-separable secondary transform

It was agreed to give the editors the discretion to finalize the document during the two weeks after the meeting, and circulate/discuss it on the reflector appropriately.

## Joint meetings

No joint meetings were held.

## BoGs

## List of actions taken affecting the JEM3 (update)

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the JEM2 description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

  - Encoder only

    - JVET-B0036

      - IRAP-level parallel encoding

      - AI subsampling

    - JVET-B0039

      - Cfg: GOP16

      - QP lambda change

    - JVET-B0041

      - Simplification #1a and #2

  - Normative change

    - JVET-B0038

      - Harmonization of AFFINE, OBMC and DBF

    - JVET-B0051

      - non-MPM mode coding

    - JVET-B0058

      - 1/16 pel motion vector storage accuracy

# Project planning

## JEM description drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- JEM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be Monday 16 May 2016.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for Experiments

Group coordinated experiments have been planned. These may generally fall into one category:

- "Exploration experiments" (EEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the main branch of JEM by the next meeting.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. (E. Alshina will edit the document based on input from the proponents, review is performed in the plenary)

- Software for tools investigated in EE is provided in a separate branch of the software repository

- During the experiment, further improvements can be made

- By the next meeting it is expected that at least one independent party will report a detailed analysis about the tool, confirms that the implementation is correct, and gives reasons to include the tool in JEM

- As part of the experiment description, it should be captured whether performance relative to JEM as well as HM (with all other tools of JEM disabled) should be reported by the next meeting.

It is possible to define sub-experiments within particular EEs, for example designated as EEX.a, EEX.b, etc., where X is the basic EE number.

As a general rule, it was agreed that each EE should be run under the same testing conditions using one software codebase, which should be based on the JEM software codebase. An experiment is not to be established as a EE unless there is access given to the participants in (any part of) the TE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-B1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to EE work should identify individuals in addition to company names.

EE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the EE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish an EE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to EE activities were established as follows:

- Only qualified JVET members can participate in an EE.

- Participation in an EE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the EE should be announced and made available to all EE participants in a timely manner.

Further discussed Tuesday AM, chaired by JRO and J. Boyce.

A separate branch under the experimental section will be created for each new tool include in the EE. The proponent of that tool is the gatekeeper for that separate software branch. (This differs from the main branch of the JEM, which is maintained by the software coordinators.)

New branches may be created which combine two or more tools included in the EE document or the JEM. Requests for new branches should be made to the software coordinators.

Don’t need to formally name cross-checkers in the EE document. To promote the tool to the JEM at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

Timeline: (update)

T1 = JEM2.0 SW release + 4 weeks: Integration of all tools into separate EE branch of JEM is completed and announced to JVET reflector.

Initial study by cross-checkers can begin.

Proponents may continue to modify the software in this branch until T2

3rd parties encouraged to study and make contributions to the next meeting with proposed changes

T2: JVET-C meeting start – 3 weeks: Any changes to the exploration branch software must be frozen, so the cross-checkers can know exactly what they are cross-checking. An SVN tag should be created at this time and announced on the JVET reflector.

This procedure was agreed on Tuesday.

Common test conditions:

- Intra-frame sub-sampling of 8

- Parallel encoding of RA

- Replacing Class A sequences this meeting, if possible. Maintain other sequences for this meeting cycle.

- Tools not currently included in the main branch are QTBT and signal dependent transforms. A tool can be in the main branch without being enabled in the common test conditions.

- QTBT should be included as an EE. Not included in the common test conditions defined at this meeting.

- Signal dependent transforms not enabled in the common test conditions defined at this meeting.

Above common test conditions characteristics agreed on Tuesday.

## Software development

Software coordinators will work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- JEM2 will be released within 2 weeks (2016-03-11)

- The results about coding performance will be reported by 2016-03-18

# Output documents and AHGs

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JVET-B1000 Meeting Report of the 2nd JVET Meeting [G. J. Sullivan, J.-R. Ohm] [2016-XX-XX] (near next meeting)

JVET-B1001 Algorithm description of Joint Exploration Test Model 2 (JEM2) [J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce] [2016-03-25]

Based on JVET-B0021 plus the new adoptions:

  - Encoder only

    - JVET-B0036

      - IRAP-level parallel encoding

      - AI subsampling

    - JVET-B0039

      - Cfg: GOP16

      - QP lambda change

    - JVET-B0041

      - Simplification #1a and #2

  - Normative change

    - JVET-B0038

      - Harmonization of AFFINE, OBMC and DBF

    - JVET-B0051

      - non-MPM mode coding

    - JVET-B0058

      - 1/16 pel motion vector storage accuracy

JVET-B1002 Call for test materials for future video coding standardization [A. Norkin, H. Yang, J.-R. Ohm, G. J. Sullivan, T. Suzuki] [2016-03-05]

JVET-B1010 JVET common test conditions and software reference configurations [K. Suehring, X. Li] [2016-03-11]

will reflect the decisions recorded elsewhere in this report.

JVET-B1011 Description of Exploration Experiments on coding tools [E. Alshina, J. Boyce, S.-H. Kim, L. Zhang, Y.-W. Huang] [2016-03-11]

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Tool evaluation (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate the exploration experiments. - Investigate interaction of tools in JEM and exploration experiment branches. - Discuss and evaluate methodologies and criteria to assess the benefit of tool. - Study and summarize new technology proposals. | E. Alshina, M. Karczewicz (co-chairs) | N |
| JEM algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-B1001 Algorithm Description of Joint Exploration Test Model 2 - Gather and address comments for refinement of the document - Coordinate with the JEM software development AHG to address issues relating to mismatches between software and text. | J. Chen (chair) E. Alshina, J. Boyce (vice chairs) | N |
| JEM software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of the JEM2 software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver JEM2 software version and the reference configuration encodings according to JVET-B1010 common conditions. - Suggest configuration files for additional testing of tools. - Coordinate with AHG on JEM model editing and errata reporting to identify any mismatches between software and text. - Investigate parallelization for speedup of simulations. | X. Li, K. Suehring (co-chairs) | N |
| Test material (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of future video coding standards. - Identify and recommend appropriate test materials and corresponding test conditions for use in the development of future video coding standards. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics in relation to video test materials. | T. Suzuki (chair), J. Chen, J. Boyce, A. Norkin (vice chairs) | N |
| Objective quality metrics (AHG5) (jvet@lists.rwth-aachen.de) - Study metrics which are useful to evaluate the quality of video compression algorithms with closer match to human perception. - Collect and make software implementing the computation of such metrics available. | P. Nasiopoulos, M. Pourazad (co-chairs) | N |

# Future meeting plans, expressions of thanks, and closing of the meeting (update)

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–6.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Saturday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 6.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Sat. 15 – Fri. 21 Oct. 2016, 4th meeting under WG 11 auspices in Chengdu, CN.

- Thu. 12 – Wed. 18 Jan 2017, 5th meeting under ITU-T auspices in Geneva, CH.

- Sat. 1 – Fri. 7 Apr. 2017, 6th meeting under WG 11 auspices in Hobart, AU.

- …

The agreed document deadline for the 4th JVET meeting is XXday XX Oct 2016. Plans for scheduling of agenda items within that meeting remain TBA.

ITU was thanked for the excellent hosting of the 3rd meeting of the JVET. EBU and XXX were thanked for providing viewing equipment.

The 3rd JVET meeting was closed at approximately XXXX hours on Wednesday 01 June 2016.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the second meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately XX people in total), were as follows:

- …