| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C_Notes_d2 |
| --- | --- |

| Title: | Meeting notes of the 3rd meeting of the Joint Video Exploration Team (JVET), Geneva, CH, 26 May – 1 June 2016 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from responsible coordinators of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Responsible coordinators |  |  |

_____________________________

- Summary

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its third meeting during 26 May – 01 June 2016 at the ITU-T premises in Geneva, CH. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations. For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.13 of this document.

The JVET meeting sessions began at approximately 1400 hours on Thursday 26 May 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wednesday 01 June 2016. Approximately XX people attended the JVET meeting, and approximately XX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard and evaluate compression technology designs proposed in this area.

One primary goal of the meeting was to review the work that was performed in the interim period since the second JVET meeting in producing the Joint Exploration Test Model 2 (JEM2). In this context, results from seven exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM3 were planned.

The JVET produced 4 output documents from the meeting (update):

- Algorithm description of Joint Exploration Test Model 2 (JEM2)

- Call for test materials for future video coding standardization

- JVET common test conditions and software reference configurations

- Description of Exploration Experiments on coding tools

For the organization and planning of its future work, the JCT-VC established X "ad hoc groups" (AHGs) to progress the work on particular subject areas. X Exploration Experiments (EE) were defined on particular subject areas of coding tool testing. The next four JVET meetings are planned for Sat. 15 – Fri. 21 Oct. 2016 under WG 11 auspices in Chengdu, CN, during Thu. 12 – Wed. 18 Jan. 2017 under ITU-T auspices in Geneva, CH, during Sat. 1 – Fri. 7 Apr. 2017 under WG 11 auspices in Hobart, AU, and during … .

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Exploration Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its second meeting during 20–26 Feb 2016 at the San Diego Marriott La Jolla in San Diego, US. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations.

## Meeting logistics

The JVET meeting sessions began at approximately 1400 hours on Thursday 26 May 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wednesday 01 June 2016. Approximately XX people attended the JVET meeting, and approximately XX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard and evaluate compression technology designs proposed in this area.

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2016_05_C_Geneva/.

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the second JVET meeting in producing the Joint Exploration Test Model 2 (JEM2). In this context, results from seven exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM3 were planned.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the JEM software but have no normative effect are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the JEM description (an error, oversight, or messiness) or in the software are marked by the string "Decision (BF):".

This meeting report is based primarily on notes taken by the responsible leaders. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 16 May 2016. Any documents uploaded after 1200 hours Paris/Geneva time on Tuesday 17 May were considered "officially late", as around that time the 16th had ended in any time zone of the world.

All contribution documents with registration numbers JVET-C0069 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "C0069+" range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- JVET-C00XX (a proposal on …),

- …

The following technical design proposal contributions were both registered late and uploaded late:

- JVET-C00XX (a proposal on …),

- …

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JVET-C00XX (an information document on …),

- …

The following cross-verification reports were registered on time but were uploaded late: JVET-C00XX [uploaded 05-XX], … .

(Documents that were both registered late and uploaded late, other than technical proposal documents, are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-C00XX, … .

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly the meeting report JVET-B1000, JEM2 algorithm description JVET-B1001, the call for test materials JVET-B1002, JVET common test conditions and reference software configuration JVET-B1010, and the description of exploration experiments JVET-B1011, were approved. The JEM2 software implementation was also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the JEM software uses the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the JEM software, and for evaluating proposals for technology to be included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

For distribution of test sequences, a password protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI.

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT: Adaptive multi-core transform.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP: Advanced temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BIO: Bi-directional optical flow.

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPMVP: Contol-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EE: Exploration Experiment – a coordinated experiment conducted toward assessment of coding technology.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MDNSST: Mode dependent non-separable secondary transform.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- QTBT: Quadtree plus binary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatio-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

## Opening remarks

- Reviewed logistics, agenda, working practices

- Results of previous meeting: JEM, meeting report, etc.

- Goals of the meeting: New version of JEM, evaluation of status progress in EEs and new proposals, provide summary to parent bodies, define new EEs.

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800–2000 hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

(Meeting sessions were chaired by J.-R. Ohm, G. J. Sullivan, and/or J. Boyce)

- Thu. 26 May, 1st day

  - 1400-1530 Opening, AHG reports (chaired by JRO and GJS)

  - 1600-1900 EE1-EE3 (chaired by JRO)

- Fri. 27 May, 2nd day

  - 900-1100 BoG on QTBT (chaired by Kiho Choi)

  - 1100-1300 EE4-7

  - 1400-1600 BoG on JEM software & SCC tools (chaired by Xiang Li)

  - 1600- …

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized

- AHG reports (5) (section 2)

- Analysis and improvement of JEM (4) (section 3)

- Test material (9) (section 4)

- Exploration experiments (22) (section 5)

- Non-EE technology proposals (40) (section 6)

  - Transforms and coefficient coding (6)

  - Motion compensation and vector coding (14)

  - Intra coding (10)

  - Partitioning (3)

  - Other (7)

- Perceptual metrics and evaluation criteria (2) (section 7)

- Withdrawn (0) (section 8)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 9)

- Project planning (section 10)

- Output documents, AHGs (section 11)

# AHG reports (5)

JVET-C0001 JVET AHG report: Tool evaluation (AHG1) [M. Karczewicz, E. Alshina]

This document reports the work of the JVET ad hoc group on Tool evaluation (AHG1) between the 2nd JVET meeting at San Diego, USA (20–26 February 2016) and the 3rd Meeting at Geneva, Switzerland (26 May – 1 June 2016).

Joint Exploration Test Model Software (HM-16.6-JEM-2.0) was released 19th of March, 2016. The software can be downloaded at:

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-2.0

Algorithms included into JEM2.0 include

- Block structure

  - Larger Coding Tree Unit (up to 256x256) and transforms (up to 64x64)

  - Quadtree plus binary tree (QTBT) block structure  only in EE SW branch

- Intra prediction improvements

  - 65 intra prediction directions  modified in JEM2.0 compared to JEM1.0

  - 4-tap interpolation filter for intra prediction

  - Boundary filter applied to other directions in addition to horizontal and vertical ones

  - Cross-component linear model (CCLM) prediction

  - Position dependent intra prediction combination (PDPC)

  - Adaptive reference sample smoothing

- Inter prediction improvements

  - Sub-PU level motion vector prediction

  - Locally adaptive motion vector resolution (AMVR)

  - 1/16 pel motion vector storage accuracy  modified in JEM2.0 compared to JEM1.0

  - Overlapped block motion compensation (OBMC)

  - Local illumination compensation (LIC)

  - Affine motion prediction  modified in JEM2.0 compared to JEM1.0

  - Pattern matched motion vector derivation

  - Bi-directional optical flow (BIO)

- Transform

  - Explicit multiple core transform

  - Mode dependent non-separable secondary transforms

  - Signal dependent transform (SDT)  disabled by default

- Adaptive loop filter (ALF)

- Enhanced CABAC design

  - Context model selection for transform coefficient levels

  - Multi-hypothesis probability estimation

  - Initialization for context models

At the 2nd JVET meeting common test conditions were modified and test sequences were updated. So direct performance comparison between JEM1.0 and JEM2.0 is difficult.

The table below shows JEM1.0 and JEM2.0 performance compared to HM if both GOP-size and QP/lambda selection are the same for test and reference.

JEM coding performance summary in RA test configuration compared to HEVC.

|  | JEM1.0 vs HM16.6 GOP-size =8 for both |  |  | JEM2.0 vs HM16.9 GOP-size =16 for both |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A1 |  |  |  | −22.4% | −21.1% | −24.2% |
| Class A2 |  |  |  | −28.8% | −29.1% | −22.8% |
| Class B | −21.3% | −13.2% | −9.2% | −21.8% | −21.5% | −15.8% |
| Class C | −20.7% | −14.8% | −18.2% | −20.7% | −19.4% | −21.7% |
| Class D | −20.5% | −9.8% | −12.1% | −21.2% | −16.4% | −17.1% |
| Overall | −20.8% | −16.7% | −15.4% | −22.9% | −21.5% | −20.1% |
| Enc. Time | 6.0 |  |  | 5.3 |  |  |
| Dec. Time | 8.0 |  |  | 8.4 |  |  |

It was commented that the relationship between QP and lambda has been under study and modification, and noted that further consideration of this issue is under way in the JCT-VC and should be coordinated with JVET. This was taken into account for the table above, but not for the table below.

Net effect of enlarging of GOP size, QP/lambda selection modification and tools up-date in JEM2.0 compared to JEM1.0 is shown in Table 2. Test data were provided by JEM SW coordinators.

JEM2.0 (GOP-size =16) vs JEM1.0 (GOP-size =8) performance in RA test.

|  | JEM2.0 vs JEM1.0 (GOP-size is different) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | −3.8% | −17.9% | −16.4% |
| Class A2 | −4.4% | −17.7% | −15.7% |
| Class B | −7.2% | −22.8% | −24.5% |
| Class C | −8.2% | −18.7% | −18.1% |
| Class D | −7.9% | −20.8% | −20.3% |
| Overall | −6.4% | −19.7% | −19.3% |
| Enc. Time | 1.0 |  |  |
| Dec. Time | 1.0 |  |  |

At the 2nd JVET meeting Exploration Experiments practice was established. For each new coding tool under consideration special SW branch was created. After implementation of each tool announcement via JVET reflector was done. There were 7 exploration experiments on new coding tools. For all of them input contribution for this meeting were submitted.

| JVET-C number | Title | Authors |
| --- | --- | --- |
| JVET-C0024 | EE2.1: Quadtree plus binary tree structure integration with JEM tools | H. Huang, K. Zhang, Y.-W. Huang, S. Lei (MediaTek) |
| JVET-C0035 | EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning | S. Lee, W.-J. Chien, L. Zhang, J. Chen, M. Karczewicz (Qualcomm) |
| JVET-C0038 | EE2.5: Improvements on adaptive loop filter | M. Karczewicz, L. Zhang, W.-J. Chien, X. Li (Qualcomm) |
| JVET-C0042 | EE2.3: NSST-PDPC Harmonization | S.-H. Kim, A. Segall (Sharp) |
| JVET-C0053 | EE2.7: TU-level non-separable secondary transform | X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi (Qualcomm) |
| JVET-C0077 | EE2.2: Non Square TU Partitioning | K. Rapaka, J. Chen, L. Zhang, W.-J. Chien, M. Karczewicz (Qualcomm) |
| JVET-C0095 | EE2.4: De-quantization and Scaling for Next Generation Containers | J. Zhao, A. Segall, S.-H. Kim (Sharp) |

In total 20 contributions proposing new coding tools for JEM or improvements of JEM design were submitted in following categories:

- Intra (5)

- Inter (6)

- Transform (2)

- Partitioning (2)

- HDR/WCG (1)

- De-blocking (1)

- Other (3)

| JVET-C number | Title | Authors | Category |
| --- | --- | --- | --- |
| JVET-C0022 | Proposed improvements to the Adaptive multiple Core transform | P. Philippe (Orange), V. Lorcy (bcom) | Transform |
| JVET-C0023 | Predictors Elimination Technique for HEVC | M. Korman, O. Prosekov (Synopsys) | Intra |
| JVET-C0025 | Simplification of motion compensation filter for affine inter prediction | J. Nam, H. Jang, J. Lee, B. Lee, J. Lim (LGE) | Inter |
| JVET-C0026 | Tiles coding improvement for Inter pictures by improved merge list at tile boundaries | S. Biplab Raut (Samsung) | Inter |
| JVET-C0031 | BIO improvement to reduce the encoder and decoder complexities | J. Lee, N. Park, J. Nam, J. Lim (LGE) | Inter |
| JVET-C0039 | Decoupled Luma/Chroma Transform Trees for Intra | F. Urban, T. Poirier, F. Le Léannec (Technicolor) | Partitioning |
| JVET-C0040 | Adaptive Clipping in JEM2.0 | F. Galpin, P. Bordes, F. Le Léannec (Technicolor) | Other |
| JVET-C0043 | Arbitrary reference tier for intra directional modes | Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI) | Intra |
| JVET-C0046 | RExt coding tools support on JEM | T. Tsukuba, O. Nakagami, T. Suzuki (Sony) | Other |
| JVET-C0047 | Generalized bi-prediction for inter coding | C.-C. Chen, X. Xiu, Y. He, Y. Ye (InterDigital) | Inter |
| JVET-C0049 | Extended deblocking-filter process for large block boundary | K. Kawamura, Q. Yao, S. Naito (KDDI Corp.) | De-blocking |
| JVET-C0054 | Grouped signalling for transform in QTBT | X. Zhao, V. Seregin, A. Said, M. Karczewicz (Qualcomm) | Partitioning |
| JVET-C0055 | Neighbor based intra most probable modes list derivation | V. Seregin, X. Zhao, A. Said, M. Karczewicz (Qualcomm) | Intra |
| JVET-C0061 | Decoder-side intra mode derivation | X. Xiu, Y. He, Y. Ye (InterDigital) | Intra |
| JVET-C0062 | Improved affine motion prediction | F. Zou, J. Chen, M. Karczewicz, X. Li, H.-C. Chuang, W.-J. Chien (Qualcomm) | Inter |
| JVET-C0063 | EE2.7 related: Improved non-separable secondary transform | X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen (Qualcomm) | Transform |
| JVET-C0066 | On Coefficient Scaling | D. B. Sansli, D. Rusanovskyy, J. Sole, A.K. Ramasubramonian, M. Karczewicz (Qualcomm) | HDR/WCG |
| JVET-C0068 | Motion vector coding optimizations | J. Samuelsson, P. Wennersten, R. Yu, U. Hakeem (Ericsson) | Inter |
| JVET-C0069 | Direction-dependent scan order with JEM tools | S. Iwamura, A. Ichigaya (NHK) | Other |
| JVET-C0071 | Multiple line-based intra prediction | J. Li (Peking Univ.), B. Li, J. Xu (Microsoft), R. Xiong (Peking Univ.), G.-J. Sullivan (Microsoft) | Intra |

The AHG recommended:

- To review all the related contribution.

- To continue Exploration Experiments practice.

In the next round of tests, also provide results with same bit rate HEVC/JEM, such that it would be possible to identify whether the compression improvement is visible in terms of subjective quality. This should be done at rate points which are required by applications but not reachable with sufficient quality by HEVC.

It is also noted that the relatively high gain of class A2 may be misleading since some of these sequences are relatively easy to encode (rollercoaster, trafficflow)

JVET-C0002 JVET AHG report: JEM algorithm description editing (AHG2) [J. Chen, E. Alshina, J. Boyce]

This document reports the work of the JVET ad hoc group on JEM algorithm description editing (AHG2) between the 2nd JVET meeting at San Diego, USA (20–26 February 2016) and the 3rd Meeting at Geneva, Switzerland (26 May – 1 June 2016).

During the editing period, on top of JVET-A1001 Algorithm Description of Joint Exploration Test Model 1, the editorial team worked on the following three aspects to produce the final version of JVET-B1001 Algorithm Description of Joint Exploration Test Model 2.

- Integrate the following normative adoptions of the 2nd JVET meeting

- JVET-B0038: Harmonization of AFFINE, OBMC and DBF

- JVET-B0051: Non-MPM intra mode coding

- JVET-B0058: 1/16 pel motion vector storage accuracy

- Add brief encoding logic description of the following JEM2 coding tools

- Locally adaptive motion vector resolution (AMVR)

- Overlapped block motion compensation (OBMC)

- Local illumination compensation (LIC)

- Mode dependent non-separable secondary transforms

- Adaptive loop filter (ALF)

- Overall text refinement and quality improvement

Currently the document contains the algorithm description as well as encoding logic description for all the new coding features in JEM2.0.

The AHG recommended to:

- Continue to edit the Algorithm Description of Joint Exploration Test Model document to ensure that all agreed elements of JEM are described

- Continue to improve the editorial quality of the Algorithm Description of Joint Exploration Test Model document.

JVET-C0003 JVET AHG report: JEM software development [X. Li, K.Suehring]

Software development was continued based on the HM-16.6-JEM-1.0 version. A branch was created in the software repository to implement the JEM-2 tools based on the decisions noted in the notes of 2nd JVET meeting. All integrated tools were included in macros to highlight the changes in the software related to that specific tool.

HM-16.6-JEM-2.0 was released on Mar. 22nd, 2016.

Several minor fixes were added to the trunk after the release of HM-16.6-JEM-2.0. Those fixes will be included in the next release of JEM.

As decided on the last meeting, several branches were created for exploration experiments. These branches are maintained by the proponents of exploration experiments.

The performance of HM-16.6-JEM-2.0 over HM-16.6-JEM-1.0 and HM-16.9 under test conditions defined in JVET-B1010 is summarized as follows.

As decided on the 2nd JVET meeting, SDT (Signal Dependent Transform) is disabled in HM-16.6-JEM-2.0 by default. The performance of JEM-2 with SDT on is summarized as follows.

Further discussion on software issues necessary (see section 3).

The aspect of integration of screen content tools was shortly discussed. At first sight, it appears simpler to integrate SC tools in JEM rather than porting JEM on top of HM 17 (which is likely the version with SCC). We would anyway expect that HM and JEM codebases are somewhat diverging, and not every encoder optimization trick used in HM would give benefit for JEM tools.

Both software packages should be aligned in a way that they can be run with similar coding conditions.

Give AHG a mandate to investigate the implementation of SCC tools in JEM (e.g. studying whether palette mode crashes with larger CTU, or in combination with other tools).

BoG (X.Li) to discuss this with more detail and identify potential difficulties that may occur. Revisit.

This mainly relates to palette and CPR; ACT is only relevant for 4:4:4 and full-pel MV resolution may anyway be obsolete by some of the new tools.

Several experts expressed the opinion that the reduction of memory usage by JEM software would be important to investigate.

It was further mentioned that the presence of SDT in the main branch imposes some difficulties due to the long run time and high memory usage; it is currently necessary to test SDT in combination with all newly adopted tools.

JVET-C0004 JVET AHG report: Test material (AHG4) [T. Suzuki, J. Chen, A. Norkin, J. Boyce]

TBR (after arrival of T. Suzuki)

The review of new test material and refinement of test cases is expected again to be an important topic in this meeting.

Objective quality metrics (AHG5) [P. Nasiopoulos, M. Pourazad (co-chairs)]

TBR (after arrival of M. Pourazad)

1 related input doc

# Analysis, development and improvement of JEM (4)

JVET-C0034 Open-source inspired workflow for JEM maintenance [E. Thomas (??)][late]

JVET-C0037 Sequential/Parallel bitstreams unification for JVET CTC [R. Mullakhmetov, I. Sharonov, M. Sychev (Huawei)]

JVET-C0046 RExt coding tools support on JEM [T. Tsukuba, O. Nakagami, T. Suzuki (Sony)] [late]

JVET-C0103 Cross-check of JVET-C0046 RExt coding tools support on JEM [J. Chen, F. Zou (??)]

JVET-C0070 NEXT test model software [A. Hallapuro, M. Hannuksela, J. Ridge, J. Lainema, A. Aminlou (Nokia)] [late]

JVET-C0099 Further JVET CTC simplification for RA [M. Sychew (Huawei)]

# Test material (9)

JVET-C0021 GoPro test sequences for Virtual Reality video coding [A. Abbas (GoPro)]

JVET-C0028 Suggested 1080P Test Sequences Downsampled from 4K Sequences [H. Zhang, X. Ma, H. Yang (Huawei)]

JVET-C0029 Surveillance sequences for video coding development [H. Zhang, X. Ma, H. Yang (Huawei), W.Qiu (Hisilicon)]

JVET-C0041 Proposed test sequences for 1080p class [A. Norkin (Netflix)]

JVET-C0044 Response to B1002 Call for test materials: Five test sequences for screen content video coding [J. Guo, L. Zhao, T. Lin (Tongji Uni.), H. Yu (Futurewei)]

JVET-C0048 Lens distorted test sequence by an action camera for future video coding [K. Kawamura, S. Naito (KDDI Corp.)]

JVET-C0050 Test sequence formats for virtual reality video coding [K. Choi, V. Zakharchenko, M. Choi, E. Alshina (Samsung)] [late]

JVET-C0064 Nokia test sequences for virtual reality video coding [J. Ridge, M. M. Hannuksela (Nokia)] [late]

JVET-C0067 Ultra High Resolution (UHR) 360 Video [C. J. Murray (Panoaction)] [late]

# Exploration experiments (22)

JVET-C0010 Exploration Experiments on Coding Tools Report [E. Alshina, J. Boyce, Y.-W. Huang, S.-H. Kim, L. Zhang]

Summary of Exploration Experiments.

| # | Main test and sub-tests | Document | Y-BD-rate (Enc/DecTime) | Cross-check |
| --- | --- | --- | --- | --- |
| 2.1 | Quad-tree plus binary-tree (QTBT) (*) SW released at April, 19, modified during EE. | JVET-C0024 | AI: -3.3% (ET 5.4, DT 1.0) RA: -3.8% (ET 2.?, DT 1.?) LD: -4.5% (ET 2.4, DT 1.1) LDP: -4.5% (ET 2.2, DT 1.2) | JVET-C0056 Samsung |
|  | - Low-complexity Intra configuration |  | AI: -2% (ET 2.5, DT 1.0) |  |
| 2.2 | Non Square TU Partitioning(**) SW released and unchanged since April, 19 | JVET-C0077 JVET-B0047 | AI: -1.5% (ET 1.9, DT 1,0) RA: -1.0% (ET 1.1, DT 1.0) LD: -0.7% (ET 1.1, DT 1.0) LDP: -0.8% (ET 1.1, DT 1.0) | JVET-B0068 Sony |
| 2.3 | NSST and PDPC index coding (all modifications enabled) - w/o removing PDPC restriction | JVET-C0042 | AI: -0.6% (ET 1.5, DT 1,0) RA: -0.2% (ET 1.1, DT 1.0) LD: -0.1% (ET 1.1, DT 1.0) LDP: -0.1% (ET 1.2, DT 1.0) AI: -0.2% (ET 0.9, DT 1,0) RA: -0.1% (ET 1.0, DT 1.0) LD: -0.0% (ET 1.1, DT 1.1) LDP: -0.0% (ET 1.0, DT 1.0) | JVET-C0059 Samsung JVET-C0087 Qualcomm |
| 2.4 | De-quantization and scaling for next generation containers SW released and unchanged since April, 19 | JVET-C0095 registered May.25 |  |  |
| 2.5 | Improvements on adaptive loop filter SW released and unchanged since April, 19 | JVET-C0038 | AI: -1,0% (ET 1.0, DT 1.1) RA: -1,2% (ET 1.0, DT 1.0) LD: -1.1% (ET 1.0, DT 1.0) LDP: -1.5% (ET 1.0, DT 1.0) | JVET-C0036 Huawei JVET-C0057 Samsung JVET-C0074 Sharp JVET-C0091 Intel |
|  | - W/o chroma filter vs whole package |  | AI: 0,1% (ET 1.0, DT 1.0) RA: 0,0% (ET 1.0, DT 1.0) LD: 0.0% (ET 1.0, DT 1.0) LDP: 0.0% (ET 1.0, DT 1.0) |  |
|  | - W/o prediction from fixed filters vs whole package |  | AI: 0,3% (ET 1.0, DT 1.0) RA: 0,2% (ET 1.0, DT 1.0) LD: 0.1% (ET 1.0, DT 1.0) LDP: 0.1% (ET 1.0, DT 1.0) |  |
| 2.6 | Modification of Merge candidate derivation SW released and unchanged since April, 19 | JVET-C0035 | RA: -0.1% (ET 1.0, DT 1.0) LD: -0.2% (ET 1.0, DT 1.0) LDP: -0.2% (ET 1.0, DT 1.0) | JVET-C0060 Samsung JVET-C0073 Sharp JVET-C0085 Huawei |
|  | - ATMVP simplification |  | RA: -0.0% (ET 1.0, DT 1.0) LD: -0.0% (ET 1.0, DT 1.0) LDP: -0.0% (ET 1.0, DT 1.0) |  |
|  | - Merge pruning |  | RA: -0.1% (ET 1.0, DT 1.0) LD: -0.2% (ET 1.0, DT 1.0) LDP: -0.2% (ET 1.0, DT 1.0) |  |
| 2.7 | TU-level non-separable secondary transform (***) SW released at April, 19, modified during EE. | JVET-C0053 | AI: -0,5% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: -0.1% (ET 1.0, DT 1.0) LDP: -0.0% (ET 1.0, DT 1.0) | JVET-C0058 Samsung JVET-C0086 Sharp JVET-C0076 Orange, B-com |
|  | - W/o HyGT |  | AI: -0.1% (ET 0.8, DT 1.0) RA: ?% (ET 1.0, DT 1.0) LD: -0.1% (ET 1.0, DT 1.0) LDP: -0.0% (ET 1.0, DT 1.0) |  |
|  | - All coeff. sub-groups use secondary transform |  | AI: -0.5% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: -0.1% (ET 1.0, DT 1.0) LDP: -0.1% (ET 1.0, DT 1.0) |  |
|  | - Secondary transform is applied for all non-zero TUs (default 2 non zero coeff) |  | AI: -0.2% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: -0.1% (ET 1.0, DT 1.0) LDP: -0.1% (ET 1.0, DT 1.0) |  |
|  | - Secondary transform is applied for transform-skip and LM mode |  | AI: -0.5% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: -0.1% (ET 1.0, DT 1.0) LDP: -0.1% (ET 1.0, DT 1.0) |  |
|  | - CU-level signalling with HyGT |  | AI: -0.6% (ET 1.5, DT 1.1) RA: ?% (ET ?, DT ?) LD: -0.1% (ET 1.1, DT 1.0) LDP: -0.0% (ET 1.1, DT 1.0) |  |

Comments:

(*) Full tests data available in cross-check report (not in original contribution), significant Chroma gain is observed (~5% AI, LDB and LDP, ~8,5% in RA), significant Class F gain (not included in the previous average number by the CTC template) is observed (AI Y: ~4%, UV: ~7%, RA Y: ~5%, UV: ~9%, LDB Y: ~8%, UV: ~10%, LDP Y: ~8%, (**) Tested vs HM16.6.

(***) Luma BD-rate gain is accompanied by Chroma drop. Only partial test data are available by May, 24.

EE1: QTBT: Gain is slightly higher with other tools off (using QTBT with HM), Has significant increase in encoder runtime, particularly AI

EE2: Gives some evidence how much of the QTBT gain comes from non-square transform

EE3: NSST/PDPC: Most gain is obtained via removing the PDPC restriction. NSST gives about 0.2%, but is not increasing the complexity

EE4: Dequant: Late document, further review

EE5: ALF modifications provide gain without change in encoding/decoding runtime. Modification of chroma filter gives only small benefit.

EE6: No loss by ATMVP simplification; the second aspect avoids duplicate merge candidate, which gives a small gain.

EE7: Secondary transform (hypercubic Givens transf) Results (RA) not fully available yet. AI provides most gain (0.5% on average). While there is gain in luma, some losses occur in chroma in some cases. Reduction of run time because TU level operation does not require a second prediction.

## EE1: Quad-tree plus binary-tree (QTBT)

JVET-C0024 EE2.1: Quadtree plus binary tree structure integration with JEM tools [H. Huang, K. Zhang, Y.-W. Huang, S. Lei (MediaTek)]

This contribution reports the results of Exploration Experiment (EE) 2.1, which is the integration of quadtree plus binary tree (QTBT) structure on top of JEM-2.0. The Y BD-rates and the averages of the U BD-rates and the V BD-rates in all intra (AI), random access (RA), low delay B (LB), and low delay P (LP) common test conditions (CTC) are reported as follows.

[QTBT+JEM-2.0 compared with JEM-2.0] 
AI: Y = -3.3%, UV = -5.1%
RA: Y = -3.8%, UV = -8.9%
LB: Y = -4.5%, UV = -5.4%
LP: Y = -4.4%, UV = -5.4%

For I slices, luma and chroma can have different partition structure

Significant increase of encoder runtime 500% AI, 250% RA/LD; according to the proponents, this is due to the fact that combination with other tools is still suboptimum.

The max CTB size is restricted to 128x128 in the experiments with QTBT, whereas JEM uses 256x256.

Smallest luma PB is 4x8. Additional implementation of non-square transform necessary.

Probably the performance of JEM could also be improved by testing more options at the encoder.

Several experts expressed support for QTBT, in particular because the impact on decoder complexity is marginal, while giving attractive gain. Even though the encoder runtime is significantly increased, the tradeoff with the compression benefit is still attractive. Furthermore, the detailed results indicate that the gain of other tools is largely retained when combined with QTBT.

Decision: Adopt QTBT into the main branch of JEM and CTC.

It is noted that this is a major change, as the current QTBT approach does not distinguish partitioning into CU, PU, TU, but instead gives the option to make all of them non-square and same size.

Revisit: Breakout group (Kiho Choi) to further study detailed results of the EE; study possibilities of reducing the encoder complexity; define default settings of QTBT for CTC (e.g. max CTU size, separate or non-separate trees for luma and chroma in case of intra).

From the follow-up discussion in context of EE4, it should also be made mandatory that the JEM with QTBT supports adaptive QP (which has not been tested so far).

JVET-C0056 Cross-check of JVET-C0024 (QTBT) [M. W. Park, B. Jin, E. Alshina, C. Kim (Samsung)] [late]

JVET-C0088 EE: Cross-check of EE2.1 QTBT (JVET-C0024) [V. Seregin, J. Chen (Qualcomm)] [late]

## EE2: Non Square TU Partitioning

JVET-C0077 EE2.2: Non Square TU Partitioning [K. Rapaka, J. Chen, L. Zhang, W. –J. Chien, M. Karczewicz (Qualcomm)] [late]

This contribution reports the results of Exploration Experiment (EE) 2.2 on non-square TU partitioning for intra and inter prediction modes. Two partition types (2NxN and Nx2N) are added for intra mode. For non-square partitions, a binary split is allowed at root level (level 0) for intra and inter prediction modes. Further TU splitting process follows the HEVC mechanism. It is reported that the proposed method provides 1.5%, 1.0%, 0.7%, 0.8% BD-rate saving for AI, RA, LDB and LDP configurations respectively over HM 16.6.

No need for separate presentation, already covered in EE summary report. Non-square TU partitioning is implicitly included in QTBT, no need to continue this EE at this time. However, further study is recommended whether elements of the proposal could be beneficial in combination with QTBT.

## EE3: NSST and PDPC index coding

JVET-C0042 EE2.3: NSST-PDPC Harmonization [S.-H. Kim, A. Segall (Sharp)]

This contribution proposes changes to the non-separable secondary transform (NSST) process in JEM 2.0, with (i) using a unified binarization for NSST index coding and (ii) adaptively signalling the NSST index on a CU and TU level.  The specific changes are as follows: First, instead of using two binarization methods based on intra prediction mode and partition size as is done in JEM 2.0, the contribution proposes to code the NSST index with a truncated unary binarization method and adjust the context model to reflect the statistics of the index based on the intra prediction mode and partition size. Second, the contribution proposes to code the NSST index first at a CU level and then conditionally signal a TU level flag to indicate whether NSST is applied. Finally, the contribution proposes to remove the bit-stream restriction currently precluding enabling NSST and PDPC at the same time. Using these three proposals, it is reported that an improvement of 0.6%, 0.3%, 0.1%, 0.1% luma BD-rate savings is observed for AI, RA, LD (B), and LD (P) configurations, respectively.

Unification of binarization does not give loss or gain, but is a slight simplification (also proposed in EE7)

Decision: Adopt this aspect.

Additional TU level flag for NSST gives no relevant gain (0.03%) (and anyway obsolete if QTBT is adopted) – no action.

Alternative NSST kernel gives approx. 0.2% BR reduction, but might increase memory usage (same as EE7, see further discussion there)

Decoupling PDPC and NSST gives approx. 0.4% gain, but increases encoder runtime by approx. 50% in AI. If it would not be implemented as bitstream restriction but encoder option, it would increase the bitrate. No attractive tradeoff – no action.

JVET-C0059 Cross-check of JVET-C0042 (NSST-PDPC) [K. Choi, M. Park, E. Alshina, C. Kim (Samsung)] [late]

JVET-C0087 EE: Cross-check of EE2.3 NSST-PDPC Harmonization (JVET-C0042) [V. Seregin (Qualcomm)] [late]

## EE4: De-quantization and scaling for next generation containers

JVET-C0095 EE2.4: De-quantization and Scaling for Next Generation Containers [Jie Zhao, Andrew Segall, Seung-Hwan Kim] [late]

This document provides an update on the EE2.4 exploration of “De-quantization and scaling for next generation containers”.  The asserted goal of the document is to provide answers to some of the questions raised as part of the EE process.  Specifically, information is provided about the complexity of the function f(), the overhead of delta QP signalling as employed in the HDR super anchors developed in the MPEG process, and the performance of the tool when coding ST-2084 content.  Additionally though, the document reports and proposes a methodology to measure the performance of the EE2.4-like tools that attempt to re-shape the quantization noise.  The proposed method uses a weighted PSNR metric that is derived directly from the re-shaping function under test.  Use of this weighting is then combined with traditional BD-rate and BD-PSNR calculations.  Using this approach, the proposed method is shown to provide approximately 1.9% gain relative to signaling delta QP information.

It is questioned how valid these results are, considering the fact that they are not obtained by genuine HDR material.

The claimed bit rate gain may be misleading since the explicit signaling uses 256x256 areas (CTU) for adaptation is likely inappropriate (too large in case of local brightness changes).

Continuation of EE should investigate the local adaptation with a more granular level by using quantization groups in cases of 256x256 CTUs with local brightness changes.

Tests should be performed with genuine HDR sequences (from the HDR test set used in JCT-VC).

The implicit approach works at the TU level and uses a lookup table with 16 entries (?) that is signalled in the bitstream. This still allows sequence specific encoder optimization.

Note: It has to be clarified how much QTBT would support local quantization adaptation at the sub-CTU level. It should be supported and be further investigated in the QTBT BoG. Revisit.

## EE5: Improvements on adaptive loop filter

JVET-C0038 EE2.5: Improvements on adaptive loop filter [M. Karczewicz, L. Zhang, W.-J. Chien, X. Li (Qualcomm)]

In this contribution, results of the EE 2.5 testing several modifications to the adaptive loop filter (ALF) in HM16.6 JEM-2.0 are presented. In the modified ALF, the diagonal gradients are taken into consideration during classification, geometric transformations of filter coefficients and prediction from fixed filters are also employed. In addition, some cleanups of the ALF design and software are included. It is reported that 1.0%, 1.2%, 1.1% and 1.5% BD-rate reduction is achieved for AI, RA, LDB and LDP configurations, respectively.

Elements of the proposal:

- Diagonal gradients taken into consideration during classification.

- Geometric transformations of filter coefficients (diagonal, vertical flip and rotation) based on the classification results.

- Prediction from set of the fixed filters.

- Design and software clean ups, e.g.:

  - Chroma filter shape is aligned with the luma 5x5 filter shape,

  - Context coded bins used for ALF coefficients are replaced by by-pass coded bins with the exception of the on/off flag signalled on the CU level.

The gain is mainly due to the diagonal classification and filtering. Prediction from fixed filters gives a small additional gain

The additional diagonal classification increases complexity. Decoder runtime is increased by roughly 5%, which may also be partially due to the fact that ALF is used more frequently.

Cross-checks confirm results, where however the reported decoding time increase is higher in the reports JVET-C0057, JVET-C0074 and JVET-C0091.

Several experts expressed the opinion that the additional gain in compression performance is attractive and still a reasonable tradeoff versus the increased complexity.

Decision: Adopt all aspects of the proposal into JEM3 and CTC.

JVET-C0057 Cross-check of JVET-C0038 (GALF) [K. Choi, M. Park, E. Alshina, C. Kim (Samsung)] [late]

JVET-C0036 Crosscheck of JVET-C0038 (EE2.5 Improvements on adaptive loop filter) [R. Chernyak (Huawei)] [late]

Subjective testing was performed, and it is reported that mainly the quality is increased, whereas there are few cases where a decrease of quality is observed. This however applies when looking at single frames, whereas for moving video it is hardly recognizable.

JVET-C0074 Cross-check of JVET-C0038 (EE2.5: Improvements on adaptive loop filter) [Y. Yamamoto, T. Ikai (Sharp)] [late]

JVET-C0091 Crosscheck for EE2.5 Improvements on adaptive loop filter (JVET-C0038) [X. Cai, L. Xu, Y. Chiu (Intel)] [late]

## EE6: Modification of Merge candidate derivation

JVET-C0035 EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning [S. Lee, W.-J. Chien, L. Zhang, J. Chen, M. Karczewicz (Qualcomm)]

In this contribution, merge pruning and ATMVP simplification are proposed to achieve BD rate reduction without complexity increase of both encoder and decoder. With the proposed modifications, additional 0.06%, 0.13%, and 0.25% BD rate reduction over JEM2.0 is achieved with same encoder/decoder complexity for random access, low delay B, and low delay P configurations, respectively.

The first aspect (ATMVP) is a slight simplification, whereas the second aspect has a slight increase in complexity.

In general, the change is small and it gives small benefit for all classes, where however the gain is largest for class E and for specific sequences (e.g. rollercoaster). In very few cases, a small loss is observed.

Some support is expressed for the simplification of ATMVP. For the modification of the pruning in merge, no support is given by other experts.

Decision: Adopt the ATMVP simplification to JEM3.

JVET-C0085 Cross-check of JVET-C0035 (EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning) [H. Chen, H. Yang (Huawei)] [late]

JVET-C0060 Cross-check of JVET-C0035 (Modification of merge candidate derivation) [A. Tamse, E. Alshina, C. Kim (Samsung)] [late]

JVET-C0073 Cross-check of JVET-C0035 (EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning) [Y. Yasugi (Sharp)] [late]

## EE7: TU-level non-separable secondary transform

JVET-C0053 EE2.7: TU-level non-separable secondary transform [X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi (Qualcomm)]

This contribution reports the results of Exploration Experiment (EE) 2.7 “TU-level non-separable secondary transform”. On top of JEM-2.0, the overall BD-rate gain for luma component is -0.46%, -0.21%, -0.14% and -0.04% for All Intra (AI), Random Access (RA), Low Delay B (LDB) and Low Delay P (LDP) configurations, respectively, with 17% overall encoder run-time saving for AI.

Powerpoint deck not uploaded in v3.

The main elements of the proposal are

- Hypercubic Givens transform replacing the current secondary transform

- signalling it at TU level instead of CU (which does mainly impact encoder complexity and only gives small benefit in compression) (note with QTBT CU and TU are identical anyway)

If the new transform is enabled at CU level and can be used in combination with PDPC, the BR reduction reaches 0.62%, but encoder runtime is increased by 50%.

It is claimed that the memory consumption of the new transform is less than current NSST (80%) when it is implemented in butterfly structure. With direct implementation via matrix multiply, it would be increased 3x compared to NSST.

C0053 also contains results “test 1b” which show the replacement of NSST by HyGT in combination with QTBT. In this case, the encoder runtime increases by approx. 25%, giving BR reduction of 0.4-0.5% luma (class A results not available yet). Chroma shows small loss.

The TU level decision does not consider chroma in the decision, the same is the case with the current QTBT combination.

Continue EE with the goal of further investigating the HyGT in combination with QTBT

- investigate the impact of conditional usage in combination with PDPC (encoder runtime, compression performance)

- further investigate the chroma loss that is observed.

The results of test 5 indicate that usage of HyGT is useless when TS or LM are on. The same could be the case for the current NSST (note that C0045 investigates that).

JVET-C0058 Cross-check of JVET-C0053 (TU-NSST) [K. Choi, M. Park, E. Alshina, C. Kim (Samsung)] [late]

JVET-C0076 Cross-check of JVET-C0053 (TU-NSST) [V. Lorcy (bcom), P. Philippe (Orange), T. Biatek (TDF)] [late]

JVET-C0086 Cross-check of EE2.7: TU-level non-separable secondary transform [S.-H. Kim (Sharp)] [miss] [late]

# Non-EE Technology proposals (XX)

## Transforms (6)

JVET-C0022 Proposed improvements to the Adaptive multiple Core transform [Pierrick Philippe (Orange), Victorien Lorcy (bcom)]

This contribution proposes a change in the Adaptive Multiple Core transform tool in the current JEM2 design. The changes are limited to the introduction of two additional core transforms and corresponding tables.

It is reported that these changes bring -0.37% and -0.23% Y-BDrate improvement for AI and RA configurations respectively with a limited impact on the encoding/decoding complexity.

Powerpoint presentation not included in v3.

The two transforms suggested to be added are DST-IV (which is similar to DST-VII but has a fast algorithm), and ID (no transform, but not TS as it can be selected separately per direction).

Gain in LDB and LDP is small.

What would happen when just replacing DST-VII by DST-IV in current configuration? According to proponents, this would likely give loss.

The lookup table is modified and increased in size. It is also observed that the lookup table is looking somewhat random and not symmetric; some concern is raised whether it is overtrained.

Further investigation in EE. Investigate

- dependency on block size, and extension of the scheme for QTBT with non-square blocks

- investigate possibility of introducing symmetry in LUT

JVET-C0045 EE2.7-related: On secondary transform when primary transform is skipped [T. Tsukuba, O. Nakagami, T. Suzuki (Sony)]

This contribution proposes to (a) disable secondary transform when primary transform is skipped, and (b) omit signaling transform_skip_flag when secondary transform is signalled.

Experimental results show that:

- BDrate changes for (a) are 0.00%, -0.01%, -0.01% and 0.00% for AI, RA, LB and LP for class A to class E, respectively.

- BDrate changes for (a+b) are 0.17%, 0.BB%, 0.01% and 0.02% for AI, RA, LB and LP for class A to class E, respectively.

It is noted that EE2.7 includes the same aspect discussed in the contribution. It is recommended to adopt the proposal or the same aspect of EE2.7 to JEM.

In revision 1, experimental results are updated and also a patch is attached. Note that several tests are still under simulation for (a+b) case.

The proposals were implemented on top of JEM-2.0 [1]. Two tests were performed on common test condition [2] as below:

TEST1: only disabling secondary transform when primary transform is skipped

TEST2: TEST1 + signaling modification

Test 1 (a) gives benefit (0.18% in AI) for class F (as this is the only class where TS is used).

The approach is straightforward and results are in line with the EE7 case 5. There is clear indication that the combination of NSST and TS is useless (or may even harm in the JEM2 context, since within a CU that signals the use of NSST, some TUs may use TS, and would then apply NSST on residuals directly). This is likely the explanation for the gain in class F.

In context of QTBT, this case does no longer occur, since CU and TU size are identical. Nevertheless, there is clear evidence that the combination of NSST and TS is useless from C0045 and C0053, where however none of the solutions suggested in the context of JEM2 is directly applicable to JEM3 with QTBT.

Decision: In JEM3, disable usage of TS and do not signal the TS flag when NSST is enabled. This means that the sequence of signalling needs to be changed in the current QTBT implementation.

Note: This approach (making TS dependent on NSST) is likely more efficient for cases where NSST is used more frequently than TS. In other cases, e.g. screen content, this may not be the case.

JVET-C0089 Cross-check of JVET-C0045 On secondary transform when primary transform is skipped [J. Chen, K. Rapaka (Qualcomm)] [miss] [late]

JVET-C0063 EE2.7 related: Improved non-separable secondary transform [X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen (Qualcomm)]

In this contribution, an improved non-separable secondary transform is proposed on top of JEM-2.0. Compared to the current CU-level NSST design in JEM-2.0, the proposed method applies 4x4 NSST for 4x4 TUs, and 8x8 NSST for the top-left (lower frequency) 8x8 coefficients in 8x8 and larger TUs, while NSST signaling is still applied on CU-level. With the proposed method, compared with JEM-2.0, additional average coding gain of -1.1% is achieved over JEM-2.0 for AI configuration.

Powerpoint presentation not included in v3.

Memory usage compared to the 4x4 case investigated in EE7 would be 5x,

Additional gain of 8x8 is approx. 0.6%.

To be included in the ongoing EE7.

JVET-C0075 Cross-check of JVET-0063 EE2.7 related: Improved non-separable secondary transform [S.-H. Kim(Sharp)] [miss] [late]

## Coefficient scaling (2)

JVET-C0066 On Coefficient Scaling [D. B. Sansli, D. Rusanovskyy, J. Sole, A.K. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-C0102 Luma delta QP adjustment based on video statistical information [J. Kim, J. Lee, E. Alshina, Y. Park (Samsung)]

## Motion compensation and vector coding (14)

JVET-C0023 Predictors Elimination Technique for HEVC [M. Korman, O. Prosekov (??)]

JVET-C0025 Simplification of motion compensation filter for affine inter prediction [J. Nam, H. Jang, J. Lee, B. Lee, J. Lim (LGE)]

JVET-C0093 Cross-check of JVET-C0025: Simplification of motion compensation filter for affine inter prediction [H.Zhang, H. Yang (Huawei)] [late]

JVET-C0027 Simplification and improvements for BIO design in JEM2.0 [E. Alshina, A. Alshin (Samsung)]

JVET-C0065 Cross-check of JVET-C0027 [J. Lee, S. Yoo, J. Nam, J. Lim (LGE)] [late]

JVET-C0072 Cross-check of JVET-C0027 (BIO simplification and improvements) [X. Li (Qualcomm)] [late]

JVET-C0080 Cross-check of JVET-C0027 (Simplification and improvements for BIO design in JEM2.0) [T. Biatek (TDF), V. Lorcy (bcom), P. Philippe, P. Boissonade (Orange)] [late]

JVET-C0031 BIO improvement to reduce the encoder and decoder complexities [J. Lee, N. Park, J. Nam, J. Lim (LGE)]

JVET-C0052 The cross-check for BIO improvement to reduce the encoder and decoder complexities in JVET-C0031 [E. Alshina (Samsung)] [late]

JVET-C0047 Generalized bi-prediction for inter coding [C.-C. Chen, X. Xiu, Y. He, Y. Ye (InterDigital)]

JVET-C0092 Cross-check of JVET-C0047: Generalized bi-prediction for inter coding [S.-H. Kim (Sharp)] [miss] [late]

JVET-C0062 Improved affine motion prediction [F. Zou, J. Chen, M. Karczewicz, X. Li, H.-C. Chuang, W.-J. Chien (Qualcomm)]

JVET-C0083 Cross-check of JVET-C0062 Improved affine motion prediction [O. Nakagami (Sony)] [miss] [late]

JVET-C0068 Motion vector coding optimizations [J. Samuelsson, P. Wennersten, R. Yu, U. Hakeem (Ericsson)]

## Intra coding (10)

JVET-C0043 Arbitrary reference tier for intra directional modes [Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI)]

JVET-C0090 Cross-check of JVET-C0043 (Arbitrary reference tier for intra directional modes) [T. Ikai (Sharp)] [late]

JVET-C0098 Cross-check of arbitrary reference tier for intra directional modes (JVET-C0043) [B. Li, J. Xu (Microsoft)] [late]

JVET-C0055 Neighbor based intra most probable modes list derivation [V. Seregin, X. Zhao, A. Said, M. Karczewicz (Qualcomm)]

JVET-C0081 Crosscheck of neighbor based intra most probable modes list derivation in JVET-C0055 [H. Huang (MediaTek)] [late]

JVET-C0061 Decoder-side intra mode derivation [X. Xiu, Y. He, Y. Ye (InterDigital)]

JVET-C0094 Cross-check of JVET-C0061: Decoder-side intra mode derivation [X. Ma, H. Yang (Huawei)] [late]

JVET-C0071 Multiple line-based intra prediction [J. Li (Peking Univ.), B. Li, J. Xu (Microsoft), R. Xiong (Peking Univ.), G.-J. Sullivan (Microsoft)] [late]

JVET-C0082 Cross-check of JVET-C0071 (Multiple line-based intra prediction) [T. Ikai (Sharp)] [late]

JVET-C0096 Cross-check of JVET-C0071 (Multiple line-based intra prediction) [L. Zhang, V. Seregin (Qualcomm)] [late]

JVET-C0097 Cross-check of JVET-C0071 (Multiple line-based intra prediction) [P.-H. Lin, Y.-J. Chang, J.-S. Tu, C.-C. Lin, C.-L. Lin (ITRI)] [late]

## Partitioning (3)

JVET-C0039 Decoupled Luma/Chroma Transform Trees for Intra [F. Urban, T. Poirier, F. Le Léannec (Technicolor)]

JVET-C0079 Cross-check of JVET-C0039 Decoupled Luma/Chroma Transform Trees for Intra [O. Nakagami (Sony)] [late]

JVET-C0054 Grouped signalling for transform in QTBT [X. Zhao, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

## Other (7)

JVET-C0026 Tiles coding improvement for Inter pictures by improved merge list at tile boundaries [S. B. Raut (??)]

JVET-C0032 Experiment on polyphase subsampled sequence coding [E. Thomas (??)]

JVET-C0078 Cross-check of C0032 (polyphase subsampled sequence coding) [P. Philippe (Orange), V. Lorcy (bcom), T. Biatek (TDF)] [late]

JVET-C0040 Adaptive Clipping in JEM2.0 [F. Galpin, P. Bordes, F. Le Léannec (Technicolor)]

JVET-C0084 Cross-check of C0040 (Adaptive Clipping) [P. Philippe (Orange), V. Lorcy (bcom), T. Biatek (TDF)] [late]

JVET-C0049 Extended deblocking-filter process for large block boundary [K. Kawamura, Q. Yao, S. Naito (KDDI Corp.)] [late]

JVET-C0069 Direction-dependent scan order with JEM tools [S. Iwamura, A. Ichigaya (??)] [late]

# Perceptual metrics and evaluation criteria (2)

JVET-C0030 Perceptual Quality Assessment Metric MS-SSIM [H.Zhang, X. Ma, Y.Zhao, H. Yang (Huawei)]

JVET-C0033 On comparison criteria for Virtual Reality video coding schemes [E. Thomas (??)]

# Withdrawn (0)

# Joint Meetings, BoG Reports, and Summary of Actions Taken

## General (update)

The setup of Exploration Experiments was discussed, and an initial draft of the EE document was reviewed in the plenary (chaire by JRO). This included the list of all tools that are intended to be investigated in EEs during the subsequent meeting cycle:

EE1: Quad-tree plus binary-tree (QTBT)

EE2: Non Square TU Partitioning

EE3: NSST and PDPC index coding

EE4: De-quantization and scaling for next generation containers

EE5: Improvements on adaptive loop filter

EE6: Modification of Merge candidate derivation

EE7: TU-level non-separable secondary transform

It was agreed to give the editors the discretion to finalize the document during the two weeks after the meeting, and circulate/discuss it on the reflector appropriately.

## Joint meetings

No joint meetings were held.

## BoGs

JVET-C0100 Report of BoG on JEM software and SCC tools [X. Li]

This is the report of BoG activity on JEM software and SCC tools. The mandates of the BoG are as follows

- Investigate the implementation of SCC tools in JEM

- Identify potential difficulties that may occur during integration

The meeting of the BoG was held on Friday, May 26, 2016 at 2:30pm

General discussion:

- SCC tools to be integrated

  - Current Picture Referencing (IntraBC)

  - Palette mode

  - Motion vector resolution control

    - Slice level tool. Potential overlapped with block level adaptive MV resolution

  - Residual adaptive color transform

    - For 444 content only.

It is agreed to focus on two tools (Current Picture Referencing/intraBC and palette mode) for now.

- Implementation difficulties of each tool

|  | CPR (IntraBC) | Palette Mode |
| --- | --- | --- |
| Independent module or not | A lot of changes may be needed in code | Relative independent module |
| Encoder only | Many encoder only optimizations which may not be efficient for natural content. May be implemented with lower priority. | - |
| Memory consumption | Hash-based search (encoder only) may be with lower priority | It should not be an issue in the latest SCM software |
| Interaction with other tools | A) On top of QTBT, there are separated trees for luma and chroma. Then luma component and chroma components may have different block vectors, which may lead to visual artefact. Solution: When CPR (intraBC) is enabled, I slice will be regarded as P/B slice so that no separated trees will be there. Then no issue. B) Potential interaction with other inter coding tools, e.g. ATMVP. | A) Similar to CPR (intraBC), when I slice is regarded as P/B slice, no issue. A sequence level flag is needed to enable/disable separated trees in I slice. A question here, if we enable palette mode but disable CPR, what to do for palette mode? - One solution would be disable separated tree when palette is enabled. - Another solution would be separated palette modes for luma and chroma in I slices with separated trees. |

It is noted that not all related codes of a SCC tool are marked by macros in SCM-7, which increases porting difficulty when changes are needed in many modules.

It is remarked that there may be many ways to implement the details of SCC tools on top of JEM software with QTBT adopted. It would be better to discuss the details before integration. It may be desirable to have a defined process for such implementation.

Due to the adoption of QTBT, simple transfer of SCC tool software does not seem to be possible.

Conduct further study of these aspects in AHG (new mandate of SW AHG).

JVET-C0101 Report of BoG on QTBT configuration setting [K. Choi]

This document provides a report on QTBT configuration settings. The meeting was held in C2 at 9:00-10:00

Mandates

- To further study detailed results of the EE

- To study possibilities of reducing the encoder complexity

- To define default settings of QTBT for CTC (e.g. max CTU size, separate or non-separate trees for luma and chroma in case of intra).

Mandate1 & 2

Parameters regarding QTBT

- CTUSize: Basic unit structure of CBs by a QTBT structure

- MinQTLumaISlice: Quadtree leaf node size for Luma in I-Slice

- MinQTChromaISlice: Quadtree leaf node size for Chroma in I-Slice

- MinQTNonISlice: Quadtree leaf node size in B,P-Slice

- MaxBTDepthISliceL: Binary tree depth for Luma in I-Slice

- MaxBTDepthISliceC: Binary tree depth for Chroma in I-Slice

- MaxBTDepth: Binary tree depth in B,P-Slice

During EE stage of QTBT, three tests (i.e., Test1, Test2, and Test 3 in Table1) have been tested and some of the results are available. The configuration set and results are shown in the Table 1 and 2.

- Anchor: JEM 2.0

- Test Sets: QTBT code in EE branch with Test 1, 2, and 3

Table 1. Configuration sets of QTBT

| Param. | Test1 | Test2 | Test3 | Test4 | Test5 |
| --- | --- | --- | --- | --- | --- |
| CTUSize | 128 | 128 | 128 | 256 | 256 |
| MinQTLumaISlice | 16 | 8 | 8 | 8 | 8 |
| MinQTChromaISlice | 4 | 4 | 4 | 4 | 4 |
| MinQTNonISlice | 16 | 8 | 8 | 8 | 8 |
| MaxBTDepthISliceL | 4 | 2 | 3 | 2 | 3 |
| MaxBTDepthISliceC | 0 | 2 | 3 | 2 | 3 |
| MaxBTDepth | 4 | 2 | 3 | 2 | 3 |
| MaxBTSizeISliceL | 32 | 32 | 32 | 32 | 32 |
| MaxBTSizeISliceC | 16 | 16 | 16 | 16 | 16 |
| MaxBTSizeBPSlice | 128 | 128 | 128 | 128 | 128 |

Table 2. Available test results

|  | Result | Test1 | Test2 | Test3 |
| --- | --- | --- | --- | --- |
| AI | BD-rate(%) | -3.3%/-5.6%/-4.6% | -2.0%/-10.4%/-9.8% |  |
|  | Encoding time(%) | 561% | 251% |  |
|  | Decoding time(%) | 110% | 110% |  |
| RA | BD-rate(%) | -3.8%/-8.9%/-8.3% | -1.6%/-9.9%/-9.7% |  |
|  | Encoding time(%) | 2xx% | 137% |  |
|  | Decoding time(%) | 1xx% | 108% |  |
| LD | BD-rate(%) | -4.5%/-5.0%/-5.8% | -2.9%/-4.6%/-5% | -3.9%/-5.6%/-6.0% |
|  | Encoding time(%) | 241% | 131% | 194% |
|  | Decoding time(%) | 106% | 109% | 106% |
| LDP | BD-rate(%) | -4.4%/-5.3%/-5.6% | -2.8%/-4.7%/-5.0% | -3.8%/-5.8%/-6.3% |
|  | Encoding time(%) | 221% | 118% | 175% |
|  | Decoding time(%) | 118% | 118% | 118% |

Note that the better performance of test 2 in chroma compared to test 1 is due to the increase of MaxBTDepthISliceC from 0 to 2. It might be interesting to modify test 2 with this parameter also set to 0 to bring the encoder runtime further down. (denoted as “Test 2a”) further below.

Test1 (configuration of EE) has been cross checked by two companies and the results were matched with the result of proponent. Test2 with AI has been cross checked, but the other scenarios has not been cross checked. The proponent provided additional test result based on Test3 configuration, but the set has not been cross checked yet.

Two additional test sets were suggested to verify the performance of QTBT (i.e., Test 4 and 5). However, these would likely have even higher runtime than Test 2 and Test 3.

During the Bog meeting, people agreed that practically testing all test sets is impossible especially in ClassA sequences (4k) because testing ClassA requires more than 2 weeks.

It is suggested that the other test sets are evaluated in next meeting by establishing Adhog group regarding QTBT encoder configuration.

After Bog meeting, one company expressed that they will Test 3 during this meeting. Table3 shows the volunteers to test configuration sets from Test 2 to 4.

Table 3. Testers of QTBT during this meeting

|  | Test1 | Test2 | Test3 |
| --- | --- | --- | --- |
| Tester1 | Samsung (Available) | Samsung | Qualcomm |
| Tester2 | Qualcomm (Available) |  |  |

Mandate3

To be decided based on the test results of mandate and 2 during this meeting

Recommendation by BO group

- To test and verify Test2 and Test3 during this meeting.

- To define default settings of QTBT in additional Bog meeting during this meeting

- To establish Adhoc group to get the best configuration setting for QTBT

Initial version presented in JVET Fri afternoon. It is confirmed that the BoG should further investigate Test 2 and Test 3 cases and report results later. If it is unrealistic to get full results for class A during the meeting, results with partial sequences should be reported along with an analysis how homogeneous the performance in Test 1 was over the entire sequence.

If possible, additional results should be reported for Test2a.

Revisit: Based on a more complete set of results, define the CTC of QTBT.

Establish AHG on fast encoding and configuration settings.

## List of actions taken affecting the JEM3 (update)

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the JEM2 description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

  - Encoder only

    - JVET-B0036

      - IRAP-level parallel encoding

      - AI subsampling

    - JVET-B0039

      - Cfg: GOP16

      - QP lambda change

    - JVET-B0041

      - Simplification #1a and #2

  - Normative change

    - JVET-B0038

      - Harmonization of AFFINE, OBMC and DBF

    - JVET-B0051

      - non-MPM mode coding

    - JVET-B0058

      - 1/16 pel motion vector storage accuracy

# Project planning

## JEM description drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- JEM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be Monday 16 May 2016.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for Experiments

Group coordinated experiments have been planned. These may generally fall into one category:

- "Exploration experiments" (EEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the main branch of JEM by the next meeting.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. (E. Alshina will edit the document based on input from the proponents, review is performed in the plenary)

- Software for tools investigated in EE is provided in a separate branch of the software repository

- During the experiment, further improvements can be made

- By the next meeting it is expected that at least one independent party will report a detailed analysis about the tool, confirms that the implementation is correct, and gives reasons to include the tool in JEM

- As part of the experiment description, it should be captured whether performance relative to JEM as well as HM (with all other tools of JEM disabled) should be reported by the next meeting.

It is possible to define sub-experiments within particular EEs, for example designated as EEX.a, EEX.b, etc., where X is the basic EE number.

As a general rule, it was agreed that each EE should be run under the same testing conditions using one software codebase, which should be based on the JEM software codebase. An experiment is not to be established as a EE unless there is access given to the participants in (any part of) the TE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-B1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to EE work should identify individuals in addition to company names.

EE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the EE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish an EE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to EE activities were established as follows:

- Only qualified JVET members can participate in an EE.

- Participation in an EE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the EE should be announced and made available to all EE participants in a timely manner.

Further discussed Tuesday AM, chaired by JRO and J. Boyce.

A separate branch under the experimental section will be created for each new tool include in the EE. The proponent of that tool is the gatekeeper for that separate software branch. (This differs from the main branch of the JEM, which is maintained by the software coordinators.)

New branches may be created which combine two or more tools included in the EE document or the JEM. Requests for new branches should be made to the software coordinators.

Don’t need to formally name cross-checkers in the EE document. To promote the tool to the JEM at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

Timeline: (update)

T1 = JEM2.0 SW release + 4 weeks: Integration of all tools into separate EE branch of JEM is completed and announced to JVET reflector.

Initial study by cross-checkers can begin.

Proponents may continue to modify the software in this branch until T2

3rd parties encouraged to study and make contributions to the next meeting with proposed changes

T2: JVET-C meeting start – 3 weeks: Any changes to the exploration branch software must be frozen, so the cross-checkers can know exactly what they are cross-checking. An SVN tag should be created at this time and announced on the JVET reflector.

This procedure was agreed on Tuesday.

Common test conditions:

- Intra-frame sub-sampling of 8

- Parallel encoding of RA

- Replacing Class A sequences this meeting, if possible. Maintain other sequences for this meeting cycle.

- Tools not currently included in the main branch are QTBT and signal dependent transforms. A tool can be in the main branch without being enabled in the common test conditions.

- QTBT should be included as an EE. Not included in the common test conditions defined at this meeting.

- Signal dependent transforms not enabled in the common test conditions defined at this meeting.

Above common test conditions characteristics agreed on Tuesday.

## Software development

Software coordinators will work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- JEM2 will be released within 2 weeks (2016-03-11)

- The results about coding performance will be reported by 2016-03-18

# Output documents and AHGs

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JVET-B1000 Meeting Report of the 2nd JVET Meeting [G. J. Sullivan, J.-R. Ohm] [2016-XX-XX] (near next meeting)

JVET-B1001 Algorithm description of Joint Exploration Test Model 2 (JEM2) [J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce] [2016-03-25]

Based on JVET-B0021 plus the new adoptions:

  - Encoder only

    - JVET-B0036

      - IRAP-level parallel encoding

      - AI subsampling

    - JVET-B0039

      - Cfg: GOP16

      - QP lambda change

    - JVET-B0041

      - Simplification #1a and #2

  - Normative change

    - JVET-B0038

      - Harmonization of AFFINE, OBMC and DBF

    - JVET-B0051

      - non-MPM mode coding

    - JVET-B0058

      - 1/16 pel motion vector storage accuracy

JVET-B1002 Call for test materials for future video coding standardization [A. Norkin, H. Yang, J.-R. Ohm, G. J. Sullivan, T. Suzuki] [2016-03-05]

JVET-B1010 JVET common test conditions and software reference configurations [K. Suehring, X. Li] [2016-03-11]

will reflect the decisions recorded elsewhere in this report.

JVET-B1011 Description of Exploration Experiments on coding tools [E. Alshina, J. Boyce, S.-H. Kim, L. Zhang, Y.-W. Huang] [2016-03-11]

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Tool evaluation (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate the exploration experiments. - Investigate interaction of tools in JEM and exploration experiment branches. - Discuss and evaluate methodologies and criteria to assess the benefit of tool. - Study and summarize new technology proposals. | E. Alshina, M. Karczewicz (co-chairs) | N |
| JEM algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-B1001 Algorithm Description of Joint Exploration Test Model 2 - Gather and address comments for refinement of the document - Coordinate with the JEM software development AHG to address issues relating to mismatches between software and text. | J. Chen (chair) E. Alshina, J. Boyce (vice chairs) | N |
| JEM software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of the JEM2 software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver JEM2 software version and the reference configuration encodings according to JVET-B1010 common conditions. - Suggest configuration files for additional testing of tools. - Coordinate with AHG on JEM model editing and errata reporting to identify any mismatches between software and text. - Investigate parallelization for speedup of simulations. | X. Li, K. Suehring (co-chairs) | N |
| Test material (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of future video coding standards. - Identify and recommend appropriate test materials and corresponding test conditions for use in the development of future video coding standards. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics in relation to video test materials. | T. Suzuki (chair), J. Chen, J. Boyce, A. Norkin (vice chairs) | N |
| Objective quality metrics (AHG5) (jvet@lists.rwth-aachen.de) - Study metrics which are useful to evaluate the quality of video compression algorithms with closer match to human perception. - Collect and make software implementing the computation of such metrics available. | P. Nasiopoulos, M. Pourazad (co-chairs) | N |

# Future meeting plans, expressions of thanks, and closing of the meeting (update)

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–6.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Saturday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 6.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Sat. 15 – Fri. 21 Oct. 2016, 4th meeting under WG 11 auspices in Chengdu, CN.

- Thu. 12 – Wed. 18 Jan 2017, 5th meeting under ITU-T auspices in Geneva, CH.

- Sat. 1 – Fri. 7 Apr. 2017, 6th meeting under WG 11 auspices in Hobart, AU.

- …

The agreed document deadline for the 4th JVET meeting is XXday XX Oct 2016. Plans for scheduling of agenda items within that meeting remain TBA.

ITU was thanked for the excellent hosting of the 3rd meeting of the JVET. EBU and XXX were thanked for providing viewing equipment.

The 3rd JVET meeting was closed at approximately XXXX hours on Wednesday 01 June 2016.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the second meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately XX people in total), were as follows:

- …