| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C_Notes_d9 |
| --- | --- |

| Title: | Meeting notes of the 3rd meeting of the Joint Video Exploration Team (JVET), Geneva, CH, 26 May – 1 June 2016 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from responsible coordinators of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Responsible coordinators |  |  |

_____________________________

- Summary

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its third meeting during 26 May – 1 June 2016 at the ITU-T premises in Geneva, CH. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations. For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.13 of this document.

The JVET meeting sessions began at approximately 1400 hours on Thursday 26 May 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1225 hours on Wednesday 1 June 2016. Approximately 130 people attended the JVET meeting, and approximately 85 input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard and evaluate compression technology designs proposed in this area.

One primary goal of the meeting was to review the work that was performed in the interim period since the second JVET meeting in producing the Joint Exploration Test Model 2 (JEM2). In this context, results from seven exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM3 were planned.

The JVET produced 3 output documents from the meeting:

- Algorithm description of Joint Exploration Test Model 3 (JEM3)

- Work plan for assessment of test materials

- Description of Exploration Experiments on coding tools

For the organization and planning of its future work, JVET established 9 "ad hoc groups" (AHGs) to progress the work on particular subject areas. 9 Exploration Experiments (EE) were defined on particular subject areas of coding tool testing. The next three JVET meetings are planned for Sat. 15 – Fri. 21 Oct. 2016 under WG 11 auspices in Chengdu, CN, during Thu. 12 – Wed. 18 Jan. 2017 under ITU-T auspices in Geneva, CH, and during Sat. 1 – Fri. 7 Apr. 2017 under WG 11 auspices in Hobart, AU.

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Exploration Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its third meeting during 26 May – 1 June 2016 at the ITU-T premises in Geneva, CH. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations.

## Meeting logistics

The JVET meeting sessions began at approximately 1400 hours on Thursday 26 May 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1225 hours on Wednesday 1 June 2016. Approximately 130 people attended the JVET meeting, and approximately 85 input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard and evaluate compression technology designs proposed in this area.

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2016_05_C_Geneva/.

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the second JVET meeting in producing the Joint Exploration Test Model 2 (JEM2). In this context, results from seven exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM3 were planned.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report, as of the time of preparation of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the JEM software but have no normative effect are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the JEM description (an error, oversight, or messiness) or in the software are marked by the string "Decision (BF):".

This meeting report is based primarily on notes taken by the responsible leaders. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 16 May 2016. Any documents uploaded after 1200 hours Paris/Geneva time on Tuesday 17 May were considered "officially late", as around that time the 16th had ended in any time zone of the world.

All contribution documents with registration numbers JVET-C0069 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "C0069+" range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- JVET-C0046 (a proposal on support for RExt tools in JEM, with some design changes), uploaded 05-23,

- JVET-C0049 (a proposal on deblocking filter modification), uploaded 05-24.

The following technical design proposal contributions were both registered late and uploaded late:

- JVET-C0077 (a proposal on non-square TU partitioning), uploaded 05-20,

- JVET-C0095 (a proposal on receiver-side derivation of quantization scaling), uploaded 05-25,

- JVET-C0069 (a proposal on direction dependent scan order), uploaded 05-18,

- JVET-C0071 (a proposal on multiple line based intra prediction), uploaded 05-17.

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JVET-C0034 (an information document on software development methods and tools), uploaded 05-26,

- JVET-C0050 (a document proposing test materials), uploaded 05-24,

- JVET-C0064 (a document proposing test materials), uploaded 05-17,

- JVET-C0067 (a document proposing test materials), uploaded 05-18,

- JVET-C0102 (an encoder optimization for brightness adaptive derivation of quantization scaling), uploaded 05-27,

- JVET-C0105 (a document requesting reduction of encoding complexity), uploaded 05-30.

The following cross-verification reports were registered on time but were uploaded late: JVET-C0036 [uploaded 05-20], JVET-C0052 [uploaded 05-19], JVET-C0056 [uploaded 05-24], JVET-C0057 [uploaded 05-24], JVET-C0058 [uploaded 05-24], JVET-C0059 [uploaded 05-24], JVET-C0060 [uploaded 05-24], JVET-C0065 [uploaded 05-26].

(Documents that were both registered late and uploaded late, other than technical proposal documents, are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-C00XX (this case did not occur in the current meeting).

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, uploading of corrupted unreadable files, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly the meeting report JVET-B1000, JEM2 algorithm description JVET-B1001, the call for test materials JVET-B1002, JVET common test conditions and reference software configuration JVET-B1010, and the description of exploration experiments JVET-B1011, were approved. The JEM2 software implementation was also approved.

The group was initially asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the JEM software uses the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the JEM software, and for evaluating proposals for technology to be included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

For distribution of test sequences, a password protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI.

## Terminology

Some terminology used in this report is explained below:

- 3D-HEVC: A set of extensions of HEVC that includes the combined coding of depth and texture information for 3D video coding.

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT: Adaptive multi-core transform.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP: Advanced temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BIO: Bi-directional optical flow.

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPMVP: Contol-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions – a set of agreed conditions for coding experiments.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EE: Exploration Experiment – a coordinated experiment conducted toward assessment of coding technology.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- GOP size: Group-of-pictures size – a term used to refer to the referencing structure of pictures in a coded video sequence; in the context of AVC or HEVC, typically refers to the spacing between forward-predictive pictures (i.e., not the spacing between intra pictures, which differs from some older uses of this term in the H.264/MPEG-2 context), between which a hierarchy of bipredictive pictures is typically used.

- HDR: High dynamic range – referring to video content having a brightness range that includes values greater than approximately 100 nits (often implicitly including WCG as well, since HDR video is typically also WCG video).

- HDR10: A term that refers to the single-layer coding of HDR/WCG video content using the HEVC Main 10 profile with a Y′CbCr 4:2:0 10 bit per sample colour representation with ITU-R BT.2020 colour primaries and the PQ transfer characteristics EOTF.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – the draft reference software and its (non-normative) encoder algorithms used for HEVC experiments.

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (as in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase and associated (non-normative) encoding algorithms that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference picture.

- MANE: Media-aware network element.

- MC: Motion compensation.

- MOS: Mean opinion score – a measurement of subjective video quality as reported by human test subjects.

- MDNSST: Mode dependent non-separable secondary transform.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MV: Motion vector; alternativey, multiview.

- MV-HEVC: A set of extensions of HEVC using layered coding to enable the coding of video with multiple views or depth maps.

- NAL: Network abstraction layer (as in AVC and HEVC, contrast with VCL).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NCL: Non-constant luminance, a type of colour difference representation.

- Nits: Candelas per square metre.

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- PQ: Perceptual quantization – the name given to an HDR EOTF curve specified in SMPTE ST 2084 and Rec. ITU-R BT.[HDR-TV].

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- QTBT: Quadtree plus binary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RExt: Format range extensions – a set of extensions of HEVC addressing high bit rate operation, high bit depths, and alternative chroma formats such as monochrome, 4:2:2, 4:4:4, high bit depths, and high throughput.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SD: Slice data; alternatively, standard-definition.

- SDR: Standard dynamic range – referring to video content having a brightness range that would produce a maximum brightness of approximately 100 nits on a reference display under reference viewing conditions.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM (for SHVC).

- SHVC: Scalable high efficiency video coding – a set of extensions of HEVC that uses layered coding to enable the coding of supplemental pictures, quality enhancement layers, spatial resolution enhancement layers, and colour gamut enhancement layers.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatio-temporal motion vector prediction.

- SVC: Scalable video coding, especially when referring to the associated extensions of AVC.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VCL: Video coding layer (as in AVC and HEVC, contrast with NAL).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WCG: Wide colour gamut – referring to video content having a colour gamut that includes colours substantially outside of the range of values that is representable using Rec. ITU-R BT.709.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

## Opening remarks

- Reviewed logistics, agenda, working practices

- Results of previous meeting: JEM, meeting report, etc.

- Goals of the meeting: New version of JEM, evaluation of status progress in EEs and new proposals, provide summary to parent bodies, define new EEs.

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800–2000 hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Thu. 26 May, 1st day

  - 1400–1530 Opening, AHG reports (chaired by JRO and GJS)

  - 1600–1900 EE1-EE3 (chaired by JRO)

- Fri. 27 May, 2nd day

  - 0900–1100 BoG on QTBT (chaired by Kiho Choi)

  - 1100–1300 EE4-6 (chaired by JRO)

  - 1400–1600 BoG on JEM software & SCC tools (chaired by Xiang Li)

  - 1600–19:30 Review BoG results, EE7, 6.1 (chaired by JRO)

- Sat. 28 May, 3rd day

  - 0900–1245 6.2+6.3 (chaired by JRO)

  - 1415–1930 6.4-6.6 (chaired by JRO)

- Sun. 29 May, 4th day

  - 0900–1300 3+7; revisits; remaining docs; EE establishment (chaired by JRO)

  - 1400–1930 BoG on test material including 4 (chaired by Teruhiko Suzuki)

- Mon. 30 May, 5th day

  - 1600–1800 BoG on QTBT config (chaired by Kiho Choi)

- Tue. 31 May, 6th day

  - 0900–1000 Joint meeting to discuss JVET scope and requirements (see section 9.2)

  - 1015–1215 BoG reports (chaired by JRO)

  - 1315–1500 JVET-C0023, JVET-C0105, JVET-C0044; revisits; AHG planning (chaired by JRO)

  - 1530–1630 EE description review (chaired by Jill Boyce)

- Wed. 1 June, 7th day

  - 1100–1230 Plenary (chaired by JRO): Approval of output documents, work plan for JEM text and software development, work plan for sequence testing, establishment of AHGs, closing of meeting.

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting are summarized as follows:

- AHG reports (5) (section 2)

- Analysis and improvement of JEM (4) (section 3)

- Test material (9) (section 4)

- Exploration experiments (22) (section 5)

- Non-EE technology proposals (42) (section 6)

  - Transforms and coefficient coding (5)

  - Coefficient scaling (EE4 related) (2)

  - Motion compensation and vector coding (13)

  - Intra coding (13)

  - Partitioning (3)

  - Other (6)

- Perceptual metrics and evaluation criteria (2) (section 7)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 9)

- Project planning (section 10)

- Output documents, AHGs (section 11)

# AHG reports (5)

JVET-C0001 JVET AHG report: Tool evaluation (AHG1) [M. Karczewicz, E. Alshina]

This document reports the work of the JVET ad hoc group on Tool evaluation (AHG1) between the 2nd JVET meeting at San Diego, USA (20–26 February 2016) and the 3rd Meeting at Geneva, Switzerland (26 May – 1 June 2016).

Joint Exploration Test Model Software (HM-16.6-JEM-2.0) was released 19th of March, 2016. The software can be downloaded at:

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-2.0

Algorithms included into JEM2.0 include

- Block structure

  - Larger Coding Tree Unit (up to 256x256) and transforms (up to 64x64)

  - Quadtree plus binary tree (QTBT) block structure  only in EE SW branch

- Intra prediction improvements

  - 65 intra prediction directions  modified in JEM2.0 compared to JEM1.0

  - 4-tap interpolation filter for intra prediction

  - Boundary filter applied to other directions in addition to horizontal and vertical ones

  - Cross-component linear model (CCLM) prediction

  - Position-dependent intra prediction combination (PDPC)

  - Adaptive reference sample smoothing

- Inter prediction improvements

  - Sub-PU level motion vector prediction

  - Locally adaptive motion vector resolution (AMVR)

  - 1/16 pel motion vector storage accuracy  modified in JEM2.0 compared to JEM1.0

  - Overlapped block motion compensation (OBMC)

  - Local illumination compensation (LIC)

  - Affine motion prediction  modified in JEM2.0 compared to JEM1.0

  - Pattern matched motion vector derivation

  - Bi-directional optical flow (BIO)

- Transform

  - Explicit multiple core transform

  - Mode dependent non-separable secondary transforms

  - Signal dependent transform (SDT)  disabled by default

- Adaptive loop filter (ALF)

- Enhanced CABAC design

  - Context model selection for transform coefficient levels

  - Multi-hypothesis probability estimation

  - Initialization for context models

At the 2nd JVET meeting, common test conditions were modified and test sequences were updated. So direct performance comparison between JEM1.0 and JEM2.0 is not especially straightforward.

The table below shows JEM1.0 and JEM2.0 performance compared to the HM if both GOP-size and QP/lambda selection are the same for the test and reference.

JEM coding performance summary in RA test configuration compared to HEVC.

|  | JEM1.0 vs HM16.6 (GOP-size =8 for both) |  |  | JEM2.0 vs HM16.9 (GOP-size =16 for both) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A1 |  |  |  | −22.4% | −21.1% | −24.2% |
| Class A2 |  |  |  | −28.8% | −29.1% | −22.8% |
| Class B | −21.3% | −13.2% | −9.2% | −21.8% | −21.5% | −15.8% |
| Class C | −20.7% | −14.8% | −18.2% | −20.7% | −19.4% | −21.7% |
| Class D | −20.5% | −9.8% | −12.1% | −21.2% | −16.4% | −17.1% |
| Overall | −20.8% | −16.7% | −15.4% | −22.9% | −21.5% | −20.1% |
| Enc. Time | 6.0 |  |  | 5.3 |  |  |
| Dec. Time | 8.0 |  |  | 8.4 |  |  |

It was commented that the relationship between QP and lambda has been under study and modification, and noted that further consideration of this issue is under way in the JCT-VC and should be coordinated with JVET. This was taken into account for the table above, but not for the table below.

The net effect of enlarging of GOP size, QP/lambda selection modification and tools up-date in JEM2.0 compared to JEM1.0 is shown in the table below. Test data were provided by the JEM software coordinators.

JEM2.0 (GOP-size =16) vs JEM1.0 (GOP-size =8) performance in RA test.

|  | JEM2.0 vs JEM1.0 (GOP-size is different) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | −3.8% | −17.9% | −16.4% |
| Class A2 | −4.4% | −17.7% | −15.7% |
| Class B | −7.2% | −22.8% | −24.5% |
| Class C | −8.2% | −18.7% | −18.1% |
| Class D | −7.9% | −20.8% | −20.3% |
| Overall | −6.4% | −19.7% | −19.3% |
| Enc. Time | 1.0 |  |  |
| Dec. Time | 1.0 |  |  |

At the 2nd JVET meeting, an Exploration Experiments practice was established. For each new coding tool under consideration special SW branch was created. After implementation of each tool, announcement via the JVET reflector was done. There were 7 exploration experiments on new coding tools. For all of them, input contribution for this meeting were submitted, as listed below.

| JVET-C number | Title | Authors |
| --- | --- | --- |
| JVET-C0024 | EE2.1: Quadtree plus binary tree structure integration with JEM tools | H. Huang, K. Zhang, Y.-W. Huang, S. Lei (MediaTek) |
| JVET-C0035 | EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning | S. Lee, W.-J. Chien, L. Zhang, J. Chen, M. Karczewicz (Qualcomm) |
| JVET-C0038 | EE2.5: Improvements on adaptive loop filter | M. Karczewicz, L. Zhang, W.-J. Chien, X. Li (Qualcomm) |
| JVET-C0042 | EE2.3: NSST-PDPC Harmonization | S.-H. Kim, A. Segall (Sharp) |
| JVET-C0053 | EE2.7: TU-level non-separable secondary transform | X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi (Qualcomm) |
| JVET-C0077 | EE2.2: Non Square TU Partitioning | K. Rapaka, J. Chen, L. Zhang, W.-J. Chien, M. Karczewicz (Qualcomm) |
| JVET-C0095 | EE2.4: De-quantization and Scaling for Next Generation Containers | J. Zhao, A. Segall, S.-H. Kim (Sharp) |

In total, 20 contributions proposing new coding tools for the JEM or improvements of JEM design were submitted in the following categories, as further detailed below:

- Intra (5)

- Inter (6)

- Transform (2)

- Partitioning (2)

- HDR/WCG (1)

- De-blocking (1)

- Other (3)

| JVET-C number | Title | Authors | Category |
| --- | --- | --- | --- |
| JVET-C0022 | Proposed improvements to the Adaptive multiple Core transform | P. Philippe (Orange), V. Lorcy (bcom) | Transform |
| JVET-C0023 | Predictors Elimination Technique for HEVC | M. Korman, O. Prosekov (Synopsys) | Intra |
| JVET-C0025 | Simplification of motion compensation filter for affine inter prediction | J. Nam, H. Jang, J. Lee, B. Lee, J. Lim (LGE) | Inter |
| JVET-C0026 | Tiles coding improvement for Inter pictures by improved merge list at tile boundaries | S. Biplab Raut (Samsung) | Inter |
| JVET-C0031 | BIO improvement to reduce the encoder and decoder complexities | J. Lee, N. Park, J. Nam, J. Lim (LGE) | Inter |
| JVET-C0039 | Decoupled Luma/Chroma Transform Trees for Intra | F. Urban, T. Poirier, F. Le Léannec (Technicolor) | Partitioning |
| JVET-C0040 | Adaptive Clipping in JEM2.0 | F. Galpin, P. Bordes, F. Le Léannec (Technicolor) | Other |
| JVET-C0043 | Arbitrary reference tier for intra directional modes | Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI) | Intra |
| JVET-C0046 | RExt coding tools support on JEM | T. Tsukuba, O. Nakagami, T. Suzuki (Sony) | Other |
| JVET-C0047 | Generalized bi-prediction for inter coding | C.-C. Chen, X. Xiu, Y. He, Y. Ye (InterDigital) | Inter |
| JVET-C0049 | Extended deblocking-filter process for large block boundary | K. Kawamura, Q. Yao, S. Naito (KDDI Corp.) | De-blocking |
| JVET-C0054 | Grouped signalling for transform in QTBT | X. Zhao, V. Seregin, A. Said, M. Karczewicz (Qualcomm) | Partitioning |
| JVET-C0055 | Neighbour based intra most probable modes list derivation | V. Seregin, X. Zhao, A. Said, M. Karczewicz (Qualcomm) | Intra |
| JVET-C0061 | Decoder-side intra mode derivation | X. Xiu, Y. He, Y. Ye (InterDigital) | Intra |
| JVET-C0062 | Improved affine motion prediction | F. Zou, J. Chen, M. Karczewicz, X. Li, H.-C. Chuang, W.-J. Chien (Qualcomm) | Inter |
| JVET-C0063 | EE2.7 related: Improved non-separable secondary transform | X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen (Qualcomm) | Transform |
| JVET-C0066 | On Coefficient Scaling | D. B. Sansli, D. Rusanovskyy, J. Sole, A.K. Ramasubramonian, M. Karczewicz (Qualcomm) | HDR/WCG |
| JVET-C0068 | Motion vector coding optimizations | J. Samuelsson, P. Wennersten, R. Yu, U. Hakeem (Ericsson) | Inter |
| JVET-C0069 | Direction-dependent scan order with JEM tools | S. Iwamura, A. Ichigaya (NHK) | Other |
| JVET-C0071 | Multiple line-based intra prediction | J. Li (Peking Univ.), B. Li, J. Xu (Microsoft), R. Xiong (Peking Univ.), G.-J. Sullivan (Microsoft) | Intra |

The AHG recommended:

- To review all the related contribution.

- To continue Exploration Experiments practice.

In the next round of tests, it was suggested to also provide results with same bit rate for HEVC/JEM, such that it would be possible to identify whether the compression improvement is visible in terms of subjective quality. This should be done at rate points which are required by applications but not reachable with sufficient quality by HEVC.

It is also noted that the relatively high gain of class A2 may be misleading since some of these sequences are relatively easy to encode (rollercoaster, trafficflow).

JVET-C0002 JVET AHG report: JEM algorithm description editing (AHG2) [J. Chen, E. Alshina, J. Boyce]

This document reports the work of the JVET ad hoc group on JEM algorithm description editing (AHG2) between the 2nd JVET meeting at San Diego, USA (20–26 February 2016) and the 3rd Meeting at Geneva, Switzerland (26 May – 1 June 2016).

During the editing period, on top of JVET-A1001 Algorithm Description of Joint Exploration Test Model 1, the editorial team worked on the following three aspects to produce the final version of JVET-B1001 Algorithm Description of Joint Exploration Test Model 2.

- Integrate the following normative adoptions of the 2nd JVET meeting

- JVET-B0038: Harmonization of AFFINE, OBMC and DBF

- JVET-B0051: Non-MPM intra mode coding

- JVET-B0058: 1/16 pel motion vector storage accuracy

- Add brief encoding logic description of the following JEM2 coding tools

- Locally adaptive motion vector resolution (AMVR)

- Overlapped block motion compensation (OBMC)

- Local illumination compensation (LIC)

- Mode dependent non-separable secondary transforms

- Adaptive loop filter (ALF)

- Overall text refinement and quality improvement

Currently the document contains the algorithm description as well as encoding logic description for all the new coding features in JEM2.0.

The AHG recommended to:

- Continue to edit the Algorithm Description of Joint Exploration Test Model document to ensure that all agreed elements of JEM are described

- Continue to improve the editorial quality of the Algorithm Description of Joint Exploration Test Model document.

JVET-C0003 JVET AHG report: JEM software development [X. Li, K. Suehring]

Software development was continued based on the HM-16.6-JEM-1.0 version. A branch was created in the software repository to implement the JEM-2 tools based on the decisions noted in the notes of 2nd JVET meeting. All integrated tools were included in macros to highlight the changes in the software related to that specific tool.

HM-16.6-JEM-2.0 was released on Mar. 22nd, 2016.

Several minor fixes were added to the trunk after the release of HM-16.6-JEM-2.0. Those fixes will be included in the next release of JEM.

As decided on the last meeting, several branches were created for exploration experiments. These branches are maintained by the proponents of exploration experiments.

The performance of HM-16.6-JEM-2.0 over HM-16.6-JEM-1.0 and HM-16.9 under test conditions defined in JVET-B1010 is summarized as follows.

As decided on the 2nd JVET meeting, SDT (Signal Dependent Transform) is disabled in HM-16.6-JEM-2.0 by default. The performance of JEM-2 with SDT on is summarized as follows.

Further discussion on software issues was necessary (see section 3).

The aspect of integration of screen content tools was briefly discussed. At first sight, it appears simpler to integrate SC tools into the JEM rather than porting the JEM on top of the HM+SCM. We would anyway expect that HM/SCM and JEM codebases are somewhat diverging, and not every encoder optimization trick used in HM/SCM would give benefit for JEM tools.

Both software packages should be aligned in a way that they can be run with similar coding conditions.

An AHG was given a mandate to investigate the implementation of SCC tools in the JEM (e.g. studying whether palette mode crashes with a larger CTU, or in combination with other tools).

A BoG (X. Li) was asked to discuss this with more detail and identify potential difficulties that may occur.

This mainly relates to palette and CPR; ACT is only relevant for 4:4:4, and full-pel MV resolution may anyway be obsoleted by some of the new tools.

Several experts expressed the opinion that the reduction of the memory usage of the JEM software would be important to investigate.

It was further mentioned that the presence of SDT in the main branch imposes some difficulties due to the long run time and high memory usage; it is currently necessary to test SDT in combination with all newly adopted tools.

JVET-C0004 JVET AHG report: Test material (AHG4) [T. Suzuki, J. Chen, A. Norkin, J. Boyce]

Reviewed Sat. morning.

The document summarizes activities on test sequences selection between the 2nd JVET meeting at San Diego, USA (20–26 February 2016) and the 3rd Meeting at Geneva, Switzerland (26 May – 1 June 2016). Sequences chosen to be used in CTC at the last meeting are made available at the Aachen university ftp site. As a response to the call of test material issued at San Diego meeting, several contributions with new test sequences were proposed.

JVET-B1002 “	Call for test material for future video coding standardization” was issued after San Diego meeting. As a response to the call, several test sequences were proposed.

Relevant contributions to this meeting include the following:

- JVET-C0021 "GoPro test sequences for Virtual Reality video coding", A. Abbas (GoPro).

- JVET-C0028 "Suggested 1080P Test Sequences Downsampled from 4K Sequences", H.Zhang, X. Ma, H. Yang (Huawei).

- JVET-C0029 "Surveillance sequences for video coding development", H.Zhang, X. Ma, H. Yang (Huawei), W.Qiu (Hisilicon).

- JVET-C0041 "Proposed test sequences for 1080p class", A. Norkin (Netflix).

- JVET-C0044 "Response to B1002 Call for test materials: Five test sequences for screen content video coding", J. Guo, L. Zhao, T. Lin (Tongji Univ.), H. Yu (Futurewei).

- JVET-C0048 "Lens distorted test sequence by an action camera for future video coding", K. Kawamura, S. Naito (KDDI Corp.).

- JVET-C0050 "Test sequence formats for virtual reality video coding", K. Choi, V. Zakharchenko, M. Choi, E. Alshina (Samsung).

- JVET-C0064 "Nokia test sequences for virtual reality video coding", J. Ridge, M. M. Hannuksela, E. B. Aksu, J. Lainema, A. Aminlou (Nokia).

- JVET-C0067 "Ultra High Resolution (UHR) 360 Video", C. J. Murray (Panoaction).

The AHG recommended:

- To review all related contributions.

- To perform viewing of the test material.

- To discuss further actions to select new test materials for JVET activity.

The review of new test material and refinement of test cases was an important topic in this meeting.

A BoG meeting (T. Suzuki, tentatively scheduled Sunday afternoon room C2) was asked to:

- Review the class A1/A2 selection made by last meeting, and propose possible changes

- Establish work plan towards the next meeting for investigating the 1080p sequences (with the goal to establish new class or replace class B by the next meeting

- Summarize the material offered in VR, identify if it covers all common methods of projection/rendering/stitching, and discuss possible methods of quality assessment

For the VR material, it should also be discussed with parent bodies how to coordinate the different activities in this area. The work of JVET should not be dominated by VR.

The new material for screen content should be brought to the attention of JCT-VC, and could be used in the SCC verification test. Currently, the development of higher compression technology specifically for screen content is not the main focus of JVET.

JVET-C0005 JVET AHG report: Visual quality metrics (AHG5) [P. Nasiopoulos, M. T. Pourazad]

Reviewed Sat. morning.

The document summarizes activities on visual quality metrics at the 3rd Meeting at Geneva, Switzerland (26 May – 1 June 2016).

There was one contribution in this meeting that suggests to include the MS-SSIM metric for evaluating video quality as follows:

- JVET-C0030 	“Perceptual Quality Assessment Metric MS-SSIM", H. Zhang, X. Ma, Y. Zhao, H. Yang (Huawei)

The AHG recommended

to review the related contribution.

# Analysis, development and improvement of JEM (4)

JVET-C0034 Open-source inspired workflow for JEM maintenance [E. Thomas (TNO)] [late]

This contribution proposes that JVET use a new workflow that is said to be inspired by common practices in open-source developments. That is, the source code is versioned using a decentralized code versioning system such as git. This allows every participant to create branches for new features and to submit the new branches for integration when accepted by the group. In addition, it is proposed to use a modern repository hosting service such as Gitlab.com to ease the management of proponent contributions to the reference software. The contribution reports that for collaborative projects the decentralized approach of git is more common in recent projects.

It was commented that with our mode of operation the centralized (more controlled) approach is more appropriate. The matter was left to the discretion of the software coordinators to decide which tool is more appropriate.

JVET-C0037 Sequential/Parallel bitstreams unification for JVET CTC [R. Mullakhmetov, I. Sharonov, M. Sychev (Huawei)]

This contribution provides a method for comparing sequential and parallel simulations introduced by JVET-B0036 that is reported as being accurate to floating-point error, along with a tool to concatenate output bitstreams from parallel test conditions introduced by JVET-B0036. The output of this tool is reported to be bit-exact with the output of sequential test conditions. With the help of this tool it is reportedly possible to measure accurate sequential decoding complexity, provide and transfer a single output bitstream, and play it back while using parallel encoding from JVET-B0036. Note: This should rather be marked as an information document, for which a patent statement should not be necessary.

Decision(SW): Integrate this tool into the software package (to be checked by software coordinators).

JVET-C0046 RExt coding tools support on JEM [T. Tsukuba, O. Nakagami, T. Suzuki (Sony)] [late]

This contribution proposes a) a software fix to support RExt coding tools on top of JEM-2.0 and b) transform skip support for 64x64 transform blocks. Experimental results reportedly show that, by enabling RExt tools and 64x64 transform skip (Fast Search is off), average coding gains of Class A to Class E are −0.1 %, −0.1%, −0.1% and −0.0% for AI, RA, LB and LP, respectively. For Class F, coding gains are reportedly −3.4%, −3.0%, −2.6% and −2.7% for AI, RA, LB and LP, respectively.

A patch to support RExt coding tools (including 64x64 Transform Skip) except Cross Component Prediction and Single Significance Map Context is provided.

In revision 1, the experimental results (Table 3-1) are revised.

An assertion is corrected in implicit DPCM.

Since only 32x32 coefficients are encoded in JEM for a 64x64 transform, transform skip would set 3/4 of the residual to zero.

Decision(SW): Adopt.

JVET-C0103 Cross-check of JVET-C0046 RExt coding tools support on JEM [J. Chen, F. Zou (Qualcomm)]

JVET-C0070 NEXT test model software [A. Hallapuro, M. Hannuksela, J. Ridge, J. Lainema, A. Aminlou (Nokia)] [late]

This document describes "NEXT" (Nokia EXploratory Test model), codec software that is suggested to be useful in testing future video coding technologies. The reported intention of this contribution is to provide users with a simple, modular and efficient software platform for testing new coding tools in an environment with coding efficiency close to that of the HEVC HM. Nokia is making the software available to interested JVET participants at this meeting.

This was an information contribution, not a technical proposal.

NEXT was reportedly designed as a simplified version of HEVC. It reduces the compression performance by 5% in LD, 2% in AI, but only keeping 15% of the lines in the software codebase. (The software was written from scratch.)

It does not support B pictures, and running in RA mode may also have some limitations.

JVET-C0099 Further JVET CTC simplification for RA [M. Sychew (Huawei)]

This contribution suggests to apply a modification to the current JVET CTC for further simplification of parallel RA simulations.

It was commented that RA settings should not be made on a sequence by sequence basis. More effort should be made to reduce the encoder complexity.

No action was taken on the suggested approach.

JVET-C0105 Request for actions regarding JEM complexity [G. Barroux (Fujitsu), T. Ikai (Sharp), Y. Bandoh (NTT), H. Yang (Huawei), A. Ichigaya (NHK), W. Hamidouche (INSA), R. Hashimoto (Renesas)] [late]

This document presents the view of several organizations regarding the increasing complexity of the JEM software and proposes to create a new Ad-hoc group in order to study possible ways to reduce the general complexity of the software.

The contribution proposed the following:

- Although the authors do not have a perfect solution to the described issue, they said that the JVET group should work toward solving it. To achieve this, it was proposed to establish a new Ad-hoc group whose goal would be to reduce the current complexity of the JEM software while keeping the capability to perform new tools evaluation with a good quality.

- It was stressed that a new Ad-hoc group alone may not be enough to resolve the issue. The authors request that JVET carefully defines the Common Test Conditions considering tool development and ease of testing as well as performance-complexity trade-offs.

The decision to establish an AHG for the first bullet had already been taken before presentation of the contribution.

As one possible method for the second bullet, it is suggested to not enforce testing new tools against full Common Test Conditions. This, however, would mean that interaction between a new tool and some other tools would not be known.

Another suggested approach would be to reduce the length of the video test sequences (in particular when the size of the test set is further increased). However, short sequences would not be usable when subjective viewing has to be performed – only PSNR could be tested.

It was agreed to add the aspects of the second bullet to the mandates of AHG1.

# Test material (9)

JVET-C0021 GoPro test sequences for Virtual Reality video coding [A. Abbas (GoPro)]

Reviewed in BoG report JVET-C0104.

JVET-C0028 Suggested 1080P Test Sequences Downsampled from 4K Sequences [H. Zhang, X. Ma, H. Yang (Huawei)]

Reviewed in BoG report JVET-C0104.

JVET-C0029 Surveillance sequences for video coding development [H. Zhang, X. Ma, H. Yang (Huawei), W. Qiu (Hisilicon)]

Reviewed in BoG report JVET-C0104.

JVET-C0041 Proposed test sequences for 1080p class [A. Norkin (Netflix)]

Reviewed in BoG report JVET-C0104.

JVET-C0044 Response to B1002 Call for test materials: Five test sequences for screen content video coding [J. Guo, L. Zhao, T. Lin (Tongji Univ.), H. Yu (Futurewei)]

Was presented in JVET plenary Tuesday afternoon.

This contribution provided five new screen content test sequences for future video coding standardization activity. These test sequences can be categorized as TGM (text and graphics with motion) content. It was reported that the five test sequences represent a wide range of typical screen content commonly seen in cloud/cloud-mobile computing, remote desktop, PC-over-IP, interactive TV and so on. It was also reported that this new content exhibits different coding characteristics when compared with the existing SCC test sequences and thus are useful to evaluate future SCC tools. It was reported that the five sequences are difficult to code and that, using the HEVC-SCC CTC AI lossy coding configurations and HM16.6-SCM5.2 reference software, the total compression ratio and average PSNR of the five sequences are 46.7 and 45.26 dB, respectively. On the other hand, the total compression ratio and average PSNR of the eight TGM sequences in HEVC-SCC CTC are reportedly 69.6 and 46.96 dB, respectively, which are 49% and 1.7 dB higher than that of the five new sequences. It is also reported that the five sequences have coding mode distribution statistics quite different from the eight TGM sequences in the current HEVC-SCC CTC.

According to the results, the five new test sequences seem to be more difficult to code than the current test set used in JCT-VC. It is also observed that IBC and palette are used more frequently.

According to advice from parent bodies, JVET should not put special emphasis on assessing the performance with computer-generated content. No action at this moment was taken to add a new class of test material.

JVET-C0048 Lens distorted test sequence by an action camera for future video coding [K. Kawamura, S. Naito (KDDI Corp.)]

Reviewed in BoG report JVET-C0104.

JVET-C0050 Test sequence formats for virtual reality video coding [K. Choi, V. Zakharchenko, M. Choi, E. Alshina (Samsung)] [late]

Reviewed in BoG report JVET-C0104.

JVET-C0064 Nokia test sequences for virtual reality video coding [J. Ridge, M. M. Hannuksela (Nokia)] [late]

Reviewed in BoG report JVET-C0104.

JVET-C0067 Ultra High Resolution (UHR) 360 Video [C. J. Murray (Panoaction)] [late]

Reviewed in BoG report JVET-C0104.

# Exploration experiments (22)

JVET-C0010 Exploration Experiments on Coding Tools Report [E. Alshina, J. Boyce, Y.-W. Huang, S.-H. Kim, L. Zhang]

Summary of Exploration Experiments.

| # | Main test and sub-tests | Document | Y-BD-rate (Enc/DecTime) | Cross-check |
| --- | --- | --- | --- | --- |
| 2.1 | Quad-tree plus binary-tree (QTBT) (*) SW released at April, 19, modified during EE. | JVET-C0024 | AI: −3.3% (ET 5.4, DT 1.0) RA: −3.8% (ET 2.?, DT 1.?) LD: −4.5% (ET 2.4, DT 1.1) LDP: −4.5% (ET 2.2, DT 1.2) | JVET-C0056 Samsung |
|  | - Low-complexity Intra configuration |  | AI: −2% (ET 2.5, DT 1.0) |  |
| 2.2 | Non Square TU Partitioning(**) SW released and unchanged since April, 19 | JVET-C0077 JVET-B0047 | AI: −1.5% (ET 1.9, DT 1.0) RA: −1.0% (ET 1.1, DT 1.0) LD: −0.7% (ET 1.1, DT 1.0) LDP: −0.8% (ET 1.1, DT 1.0) | JVET-B0068 Sony |
| 2.3 | NSST and PDPC index coding (all modifications enabled) | JVET-C0042 | AI: −0.6% (ET 1.5, DT 1.0) RA: −0.2% (ET 1.1, DT 1.0) LD: −0.1% (ET 1.1, DT 1.0) LDP: −0.1% (ET 1.2, DT 1.0) | JVET-C0059 Samsung JVET-C0087 Qualcomm |
|  | - w/o removing PDPC restriction |  | AI: −0.2% (ET 0.9, DT 1.0) RA: −0.1% (ET 1.0, DT 1.0) LD: −0.0% (ET 1.1, DT 1.1) LDP: −0.0% (ET 1.0, DT 1.0) |  |
| 2.4 | De-quantization and scaling for next generation containers SW released and unchanged since April 19 | JVET-C0095 (registered May 25) |  |  |
| 2.5 | Improvements on adaptive loop filter SW released and unchanged since April, 19 | JVET-C0038 | AI: −1.0% (ET 1.0, DT 1.1) RA: −1.2% (ET 1.0, DT 1.0) LD: −1.1% (ET 1.0, DT 1.0) LDP: −1.5% (ET 1.0, DT 1.0) | JVET-C0036 Huawei JVET-C0057 Samsung JVET-C0074 Sharp JVET-C0091 Intel |
|  | - W/o chroma filter vs whole package |  | AI: 0.1% (ET 1.0, DT 1.0) RA: 0.0% (ET 1.0, DT 1.0) LD: 0.0% (ET 1.0, DT 1.0) LDP: 0.0% (ET 1.0, DT 1.0) |  |
|  | - W/o prediction from fixed filters vs whole package |  | AI: 0.3% (ET 1.0, DT 1.0) RA: 0.2% (ET 1.0, DT 1.0) LD: 0.1% (ET 1.0, DT 1.0) LDP: 0.1% (ET 1.0, DT 1.0) |  |
| 2.6 | Modification of Merge candidate derivation SW released and unchanged since April, 19 | JVET-C0035 | RA: −0.1% (ET 1.0, DT 1.0) LD: −0.2% (ET 1.0, DT 1.0) LDP: −0.2% (ET 1.0, DT 1.0) | JVET-C0060 Samsung JVET-C0073 Sharp JVET-C0085 Huawei |
|  | - ATMVP simplification |  | RA: −0.0% (ET 1.0, DT 1.0) LD: −0.0% (ET 1.0, DT 1.0) LDP: −0.0% (ET 1.0, DT 1.0) |  |
|  | - Merge pruning |  | RA: −0.1% (ET 1.0, DT 1.0) LD: −0.2% (ET 1.0, DT 1.0) LDP: −0.2% (ET 1.0, DT 1.0) |  |
| 2.7 | TU-level non-separable secondary transform (***) SW released at April, 19, modified during EE. | JVET-C0053 | AI: −0.5% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: −0.1% (ET 1.0, DT 1.0) LDP: −0.0% (ET 1.0, DT 1.0) | JVET-C0058 Samsung JVET-C0086 Sharp JVET-C0076 Orange, B-com |
|  | - W/o HyGT |  | AI: −0.1% (ET 0.8, DT 1.0) RA: ?% (ET 1.0, DT 1.0) LD: −0.1% (ET 1.0, DT 1.0) LDP: −0.0% (ET 1.0, DT 1.0) |  |
|  | - All coeff. sub-groups use secondary transform |  | AI: −0.5% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: −0.1% (ET 1.0, DT 1.0) LDP: −0.1% (ET 1.0, DT 1.0) |  |
|  | - Secondary transform is applied for all non-zero TUs (default 2 non zero coeff) |  | AI: −0.2% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: −0.1% (ET 1.0, DT 1.0) LDP: −0.1% (ET 1.0, DT 1.0) |  |
|  | - Secondary transform is applied for transform-skip and LM mode |  | AI: −0.5% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: −0.1% (ET 1.0, DT 1.0) LDP: −0.1% (ET 1.0, DT 1.0) |  |
|  | - CU-level signalling with HyGT |  | AI: −0.6% (ET 1.5, DT 1.1) RA: ?% (ET ?, DT ?) LD: −0.1% (ET 1.1, DT 1.0) LDP: −0.0% (ET 1.1, DT 1.0) |  |

Comments:

(*) Full tests data available in cross-check report (not in original contribution), significant chroma gain is observed (~5% AI, LDB and LDP, ~8.5% in RA), significant Class F gain (not included in the previous average number by the CTC template) is observed (AI Y: ~4%, UV: ~7%, RA Y: ~5%, UV: ~9%, LDB Y: ~8%, UV: ~10%, LDP Y: ~8%

(**) Tested vs HM16.6.

(***) Luma BD-rate gain is accompanied by chroma drop. Only partial test data available by May 24.

EE1: QTBT: Gain is slightly higher with other tools off (using QTBT with HM), Has significant increase in encoder runtime, particularly for AI

EE2: Gives some evidence how much of the QTBT gain comes from non-square transform

EE3: NSST/PDPC: Most gain is obtained via removing the PDPC restriction. NSST gives about 0.2%, but is not increasing the complexity

EE4: Dequant: Late document, further review needed

EE5: ALF modifications provide gain without change in encoding/decoding runtime. Modification of chroma filter gives only small benefit.

EE6: No loss by ATMVP simplification; the second aspect avoids duplicate merge candidate, which gives a small gain.

EE7: Secondary transform (hypercubic Givens transform) Results (RA) not fully available yet. AI provides most gain (0.5% on average). While there is gain in luma, some losses occur in chroma in some cases. Reduction of run time because TU level operation does not require a second prediction.

EE1: Quad-tree plus binary-tree (QTBT)

JVET-C0024 EE2.1: Quadtree plus binary tree structure integration with JEM tools [H. Huang, K. Zhang, Y.-W. Huang, S. Lei (MediaTek)]

This contribution reports the results of Exploration Experiment (EE) 2.1, which is the integration of the quadtree plus binary tree (QTBT) structure on top of JEM-2.0. The Y BD-rates and the averages of the U BD-rates and the V BD-rates in all-intra (AI), random access (RA), low delay B (LB), and low delay P (LP) common test conditions (CTC) are reported as follows.

[QTBT+JEM-2.0 compared with JEM-2.0]

AI: Y = −3.3%, UV = −5.1%
RA: Y = −3.8%, UV = −8.9%
LB: Y = −4.5%, UV = −5.4%
LP: Y = −4.4%, UV = −5.4%

For I slices, luma and chroma can have a different partition structure.

Significant increase of encoder runtime was observed: 500% AI, 250% RA/LD; according to the proponents, this is due to the fact that the combination with other tools is still suboptimum.

The max CTB size is restricted to 128x128 in the experiments with QTBT, whereas the JEM uses 256x256.

The smallest luma PB is 4x4. Additional implementation of non-square transform necessary.

Probably the performance of the JEM could also be improved by testing more options at the encoder.

Several experts expressed support for QTBT, especially because the impact on decoder complexity is marginal, while giving attractive gain. Even though the encoder runtime is significantly increased, the tradeoff with the compression benefit is still attractive. Furthermore, the detailed results indicate that the gain of other tools is largely retained when their use is combined with QTBT.

Decision: Adopt QTBT into the main branch of JEM and into the CTC.

It is noted that this is a major change, as the current QTBT approach does not distinguish partitioning into CU, PU, TU, but instead gives the option to make all of them non-square and the same size.

A breakout group (Kiho Choi) was asked to further study the detailed results of the EE; study possibilities of reducing the encoder complexity, and define default settings of QTBT for CTC (e.g. max CTU size, separate or non-separate trees for luma and chroma in case of intra).

From the follow-up discussion in the context of EE4, it should also be made mandatory that the JEM with QTBT supports adaptive QP (which has not been tested so far).

JVET-C0056 Cross-check of JVET-C0024 (QTBT) [M. W. Park, B. Jin, E. Alshina, C. Kim (Samsung)] [late]

JVET-C0088 EE: Cross-check of EE2.1 QTBT (JVET-C0024) [V. Seregin, J. Chen (Qualcomm)] [late]

## EE2: Non Square TU Partitioning

JVET-C0077 EE2.2: Non Square TU Partitioning [K. Rapaka, J. Chen, L. Zhang, W.–J. Chien, M. Karczewicz (Qualcomm)] [late]

This contribution reports the results of Exploration Experiment (EE) 2.2 on non-square TU partitioning for intra and inter prediction modes. Two partition types (2NxN and Nx2N) are added for intra mode. For non-square partitions, a binary split is allowed at root level (level 0) for intra and inter prediction modes. Further TU splitting process follows the HEVC mechanism. It is reported that the proposed method provides 1.5%, 1.0%, 0.7%, and 0.8% BD-rate saving for AI, RA, LDB and LDP configurations, respectively, over HM 16.6.

There was no need for separate presentation, as this was already covered in the EE summary report. Non-square TU partitioning is implicitly included in QTBT, and there was no need to continue this EE at this time. However, further study is recommended whether elements of the proposal could be beneficial in combination with QTBT.

## EE3: NSST and PDPC index coding

JVET-C0042 EE2.3: NSST-PDPC Harmonization [S.-H. Kim, A. Segall (Sharp)]

This contribution proposes changes to the non-separable secondary transform (NSST) process in JEM 2.0, with (i) using a unified binarization for NSST index coding and (ii) adaptively signalling the NSST index on a CU and TU level. The specific changes are as follows: First, instead of using two binarization methods based on intra prediction mode and partition size as is done in JEM 2.0, the contribution proposes to code the NSST index with a truncated unary binarization method and adjust the context model to reflect the statistics of the index based on the intra prediction mode and partition size. Second, the contribution proposes to code the NSST index first at a CU level and then conditionally signal a TU-level flag to indicate whether NSST is applied. Finally, the contribution proposes to remove the bitstream restriction currently precluding enabling NSST and PDPC at the same time. Using these three proposals, it is reported that an improvement of 0.6%, 0.3%, 0.1%, and 0.1% luma BD-rate savings is observed for AI, RA, LD (B), and LD (P) configurations, respectively.

Unification of binarization does not give loss or gain, but is a slight simplification (also proposed in EE7).

Decision: Adopt this aspect.

The additional TU level flag for NSST gives no relevant gain (0.03%) (and anyway is obsolete if QTBT is adopted) – no action was taken on that.

The alternative NSST kernel gives approximately 0.2% BR reduction, but might increase memory usage (same as EE7, see further discussion there).

Decoupling PDPC and NSST gives approximately 0.4% gain, but increases encoder runtime by approximately 50% in AI. If it would not be implemented as a bitstream restriction but rather as an encoder option, it would increase the bit rate. No attractive tradeoff was shown, and thus no action was taken on that.

JVET-C0059 Cross-check of JVET-C0042 (NSST-PDPC) [K. Choi, M. Park, E. Alshina, C. Kim (Samsung)] [late]

JVET-C0087 EE: Cross-check of EE2.3 NSST-PDPC Harmonization (JVET-C0042) [V. Seregin (Qualcomm)] [late]

## EE4: De-quantization and scaling for next generation containers

JVET-C0095 EE2.4: De-quantization and Scaling for Next Generation Containers [J. Zhao, A. Segall, S.-H. Kim (Sharp)] [late]

This document provides an update on the EE2.4 exploration of “De-quantization and scaling for next generation containers”. The asserted goal of the document is to provide answers to some of the questions raised as part of the EE process. Specifically, information is provided about the complexity of the function f(), the overhead of delta QP signalling as employed in the HDR super anchors developed in the MPEG process, and the performance of the tool when coding ST 2084 content. Additionally though, the document reports and proposes a methodology to measure the performance of the EE2.4-like tools that attempt to re-shape the quantization noise. The proposed method uses a weighted PSNR metric that is derived directly from the re-shaping function under test. Use of this weighting is then combined with traditional BD-rate and BD-PSNR calculations. Using this approach, the proposed method is reported to provide approximately 1.9% gain relative to signalling delta QP information.

It was questioned how valid these results are, considering the fact that they are not obtained by genuine HDR material.

The claimed bit rate gain may be misleading since the explicit signalling uses 256x256 areas (CTU) for adaptation, which is likely inappropriate (too large in case of local brightness changes).

Continuation of the EE should investigate the local adaptation with a more granular level by using quantization groups in cases of 256x256 CTUs with local brightness changes.

Tests should be performed with genuine HDR sequences (from the HDR test set used in JCT-VC).

The implicit approach works at the TU level and uses a look-up table with 16 entries (?) that is signalled in the bitstream. This still allows sequence specific encoder optimization.

It was later reported by other experts that some EE4 experiments might have an encoder/decoder mismatch. There is no official cross-check, and it is therefore difficult to assess this.

In general, EE4 has various aspects, including the definition of evaluation metrics, usage of other test material than CTC, etc.

This investigation could better be continued in an AHG on JEM coding of extended colour volume material.

## EE5: Improvements on adaptive loop filter

JVET-C0038 EE2.5: Improvements on adaptive loop filter [M. Karczewicz, L. Zhang, W.-J. Chien, X. Li (Qualcomm)]

In this contribution, results of the EE 2.5 testing several modifications to the adaptive loop filter (ALF) in HM16.6 JEM-2.0 are presented. In the modified ALF, diagonal gradients are taken into consideration during classification, and geometric transformations of filter coefficients and prediction from fixed filters are also employed. In addition, some cleanups of the ALF design and software are included. It is reported that 1.0%, 1.2%, 1.1% and 1.5% BD-rate reduction is achieved for AI, RA, LDB and LDP configurations, respectively.

Elements of the proposal:

- Diagonal gradients used during classification.

- Geometric transformations of filter coefficients (diagonal, vertical flip and rotation) based on the classification results.

- Prediction from set of the fixed filters.

- Design and software clean ups, e.g.:

  - Chroma filter shape is aligned with the luma 5x5 filter shape,

  - Context coded bins used for ALF coefficients are replaced by bypass coded bins, with the exception of the on/off flag signalled on the CU level.

The gain is reported to be mainly due to the diagonal classification and filtering. Prediction from fixed filters gives a small additional gain.

The additional diagonal classification increases complexity. Decoder runtime is increased by roughly 5%, which may also be partially due to the fact that ALF is used more frequently.

Cross-checks confirmed the results, where however the reported decoding time increase is higher in the reports JVET-C0057, JVET-C0074 and JVET-C0091.

Several experts expressed the opinion that the additional gain in compression performance is attractive and still has a reasonable tradeoff versus the increased complexity.

Decision: Adopt all aspects of the proposal into JEM3 and CTC.

JVET-C0057 Cross-check of JVET-C0038 (GALF) [K. Choi, M. Park, E. Alshina, C. Kim (Samsung)] [late]

JVET-C0036 Crosscheck of JVET-C0038 (EE2.5 Improvements on adaptive loop filter) [R. Chernyak (Huawei)] [late]

Subjective testing was performed, and it is reported that mainly the quality is increased, whereas there are a few cases where a decrease of quality is observed. This, however, applies when looking at single frames, whereas for moving video it is hardly recognizable.

JVET-C0074 Cross-check of JVET-C0038 (EE2.5: Improvements on adaptive loop filter) [Y. Yamamoto, T. Ikai (Sharp)] [late]

JVET-C0091 Crosscheck for EE2.5 Improvements on adaptive loop filter (JVET-C0038) [X. Cai, L. Xu, Y. Chiu (Intel)] [late]

## EE6: Modification of Merge candidate derivation

JVET-C0035 EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning [S. Lee, W.-J. Chien, L. Zhang, J. Chen, M. Karczewicz (Qualcomm)]

In this contribution, merge pruning and ATMVP simplification are proposed to achieve BD rate reduction, reportedly without complexity increase of both encoder and decoder. With the proposed modifications, an additional 0.06%, 0.13%, and 0.25% BD rate reduction over JEM2.0 is reportedly achieved with same encoder/decoder complexity for random access, low delay B, and low delay P configurations, respectively.

The first aspect (ATMVP) is a slight simplification, whereas the second aspect has a slight increase in complexity.

In general, the change is small and it gives small benefit for all classes, where however the gain is largest for class E and for specific sequences (e.g., rollercoaster). In a very few cases, a small loss is observed.

Some support was expressed for the simplification of ATMVP. For the modification of the pruning in the merge, no support was expressed by other experts.

Decision: Adopt the ATMVP simplification to JEM3.

JVET-C0085 Cross-check of JVET-C0035 (EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning) [H. Chen, H. Yang (Huawei)] [late]

JVET-C0060 Cross-check of JVET-C0035 (Modification of merge candidate derivation) [A. Tamse, E. Alshina, C. Kim (Samsung)] [late]

JVET-C0073 Cross-check of JVET-C0035 (EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning) [Y. Yasugi (Sharp)] [late]

## EE7: TU-level non-separable secondary transform

JVET-C0053 EE2.7: TU-level non-separable secondary transform [X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi (Qualcomm)]

This contribution reports the results of Exploration Experiment (EE) 2.7 “TU-level non-separable secondary transform”. On top of JEM-2.0, the overall reported BD-rate gain for the luma component is −0.46%, −0.21%, −0.14% and −0.04% for All Intra (AI), Random Access (RA), Low Delay B (LDB) and Low Delay P (LDP) configurations, respectively, with 17% overall encoder run-time saving for AI.

The main elements of the proposal are

- Hypercubic Givens transform replacing the current secondary transform

- Signalling it at the TU level instead of CU level (which mainly impacts encoder complexity and only gives a small benefit in compression). (Note with QTBT, CU and TU are identical anyway.)

If the new transform is enabled at the CU level and can be used in combination with PDPC, the BD-rate reduction reaches 0.62%, but encoder runtime is increased by 50%.

It is claimed that the memory consumption of the new transform is less than the current NSST (80%) when it is implemented by a butterfly structure. With direct implementation via matrix multiply, it would be increased 3x compared to NSST.

C0053 also contains results for “test 1b” which shows the replacement of NSST by HyGT in combination with QTBT. In this case, the encoder runtime increases by approximately 25%, giving BD-rate reduction of 0.4–0.5% luma (class A results not available yet). Chroma shows a small loss.

The TU-level decision does not consider chroma in the decision; the same is the case with the current QTBT combination.

It was agreed to continue the EE with the goal of further investigating the HyGT in combination with QTBT

- investigate the impact of conditional usage in combination with PDPC (encoder runtime, compression performance)

- further investigate the chroma loss that is observed.

The results of test 5 indicate that HyGT is useless when TS or LM are on. The same could be the case for the current NSST (note that C0045 investigates that).

JVET-C0058 Cross-check of JVET-C0053 (TU-NSST) [K. Choi, M. Park, E. Alshina, C. Kim (Samsung)] [late]

JVET-C0076 Cross-check of JVET-C0053 (TU-NSST) [V. Lorcy (bcom), P. Philippe (Orange), T. Biatek (TDF)] [late]

JVET-C0086 Cross-check of EE2.7: TU-level non-separable secondary transform [S.-H. Kim (Sharp)] [late]

# Non-EE Technology proposals (42)

## Transforms (5)

JVET-C0022 Proposed improvements to the Adaptive multiple Core transform [P. Philippe (Orange), V. Lorcy (bcom)]

This contribution proposes a change in the Adaptive Multiple Core transform tool in the current JEM2 design. The changes are the introduction of two additional core transforms and corresponding tables.

It is reported that these changes bring −0.37% and −0.23% Y-BD rate improvement for AI and RA configurations respectively with a limited impact on the encoding/decoding complexity.

The two transforms suggested to be added are DST-IV (which is similar to DST-VII but has a fast algorithm), and ID (no transform, but not TS as it can be selected separately per direction).

The gain in LDB and LDP is small.

What would happen when just replacing DST-VII by DST-IV in the current configuration? According to proponents, this would likely give a loss.

The lookup table is modified and increased in size. It is also observed that the lookup table is looking somewhat random and not symmetric; some concern was raised whether it is overtrained.

Further investigation in an EE was recommended. Investigate:

- dependency on block size, and extension of the scheme for QTBT with non-square blocks

- investigate possibility of introducing symmetry in LUT

JVET-C0045 EE2.7-related: On secondary transform when primary transform is skipped [T. Tsukuba, O. Nakagami, T. Suzuki (Sony)]

This contribution proposes to a) to disable the secondary transform when the primary transform is skipped, and b) to omit signalling the transform_skip_flag when the secondary transform is signalled.

Experimental results reportedly show that:

- BD-rate changes for (a) are 0.0%, −0.0%, −0.0% and 0.0% for AI, RA, LB and LP for class A to class E, respectively.

- BD-rate changes for (a+b) are 0.2%, 0.BB%, 0.0% and 0.0% for AI, RA, LB and LP for class A to class E, respectively.

It is noted that EE2.7 includes the same aspect discussed in the contribution. It was suggested to adopt the proposal or the same aspect of EE2.7 to the JEM.

In revision 1, experimental results are updated and also a patch is attached. Several tests were still under simulation for (a+b) case.

The proposals were implemented on top of JEM-2.0. Two tests were performed on common test condition as below:

- TEST1: only disabling secondary transform when primary transform is skipped

- TEST2: TEST1 + signalling modification

Test 1 (a) gives benefit (0.18% in AI) for class F (as this is the only class where TS is used).

The approach is straightforward and results are in line with the EE7 case 5. There is clear indication that the combination of NSST and TS is useless (or may even harm in the JEM2 context, since within a CU that signals the use of NSST, some TUs may use TS, and would then apply NSST on residuals directly). This is likely the explanation for the gain in class F.

In the context of QTBT, this case no longer occurs, since the CU and TU size are identical. Nevertheless, there is clear evidence that the combination of NSST and TS is useless from C0045 and C0053, where, however, none of the solutions suggested in the context of JEM2 are directly applicable to JEM3 with QTBT.

Initially (Fri) the following solution was envisaged: In JEM3, disable usage of TS and do not signal the TS flag when NSST is enabled. This would mean that the sequence of signalling needs to be changed in the current QTBT implementation. It was assumed that this approach (making TS dependent on NSST) is likely more efficient for cases where NSST is used more frequently than TS. In other cases, e.g., for screen content, this may not be the case.

It was later detected and discussed on Sat. evening that also in QTBT, multiple TS flags exist for the different components in a block that has only one NSST index. Therefore, the following solution was adopted (Decision):

- Disable NSST and do not code NSST index if all components use TS.

- Otherwise, if NSST is on, it shall not be used for a block of a component that uses TS.

JVET-C0089 Cross-check of JVET-C0045 On secondary transform when primary transform is skipped [J. Chen, K. Rapaka (Qualcomm)] [late]

JVET-C0063 EE2.7 related: Improved non-separable secondary transform [X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen (Qualcomm)]

In this contribution, a modified non-separable secondary transform is proposed on top of JEM-2.0. Compared to the current CU-level NSST design in JEM-2.0, the proposed method applies 4x4 NSST for 4x4 TUs, and 8x8 NSST for the top-left (lower frequency) 8x8 coefficients in 8x8 and larger TUs, while NSST signalling is still applied at the CU-level. With the proposed method, compared with JEM-2.0, an additional average coding gain of −1.1% is reported over JEM-2.0 for the AI configuration.

Memory usage compared to the 4x4 case investigated in EE7 would be 5x.

The additional gain of 8x8 is approximately 0.6%.

To be included in the ongoing EE7.

JVET-C0075 Cross-check of JVET-C0063 EE2.7 related: Improved non-separable secondary transform [S.-H. Kim(Sharp)] [late]

## Coefficient scaling (EE4 related) (2)

JVET-C0066 On Coefficient Scaling [D. B. Sansli, D. Rusanovskyy, J. Sole, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

This contribution proposes an alternative to the residual scaling proposed in JVET-B0054. The motivation in JVET-B0054 is to remove the overhead of delta QP that is signalled by the encoder to compensate for the reshaping of quantization noise as a function of brightness in containers such as ST 2084. This is done by adapting the scaling of AC coefficients based on the DC coefficient and reconstructed prediction mean. In the proposal, a scale determination method is modified to allow the scaling of all coefficients equally, and the scaling operation is moved after the dequantization operation in the decoder for a simpler design. Simulations were conducted using HDR/WCG sequences from JCT-VC. Random access configuration with 64x64 CTU size reportedly showed an average of 2.5 % BD-rate gain for delta-E.

What would be the benefit compared to EE4/JVET-B0054? This was not possible to assess, since the contributors of C0066 found a mismatch in the EE4 software.

Further study of C0066 was requested in an AHG on JEM coding of extended colour volume material.

JVET-C0102 Luma delta QP adjustment based on video statistical information [J. Kim, J. Lee, E. Alshina, Y. Park (Samsung)] [late]

This contribution is identical to JCTVC-W0039. The contribution is targeting performance improvement of high dynamic range video compression using existing HEVC tools. It was asserted that the performance can be improved by adjusting the luma QP based on video statistics. Depending on the average of the luma components in a picture and the luma block average and variance, the algorithm adjusts the delta QP for each CTU. The performance of the algorithm is measured with the same method of SuperAnchor v3.2 and reportedly shows consistently better compression performance.

One expert pointed out that with this method enabled, it might be even more difficult to use objective metrics.

Further study in an AHG on JEM coding of extended colour volume material was requested.

## Motion compensation and vector coding (13)

JVET-C0025 Simplification of motion compensation filter for affine inter prediction [J. Nam, H. Jang, J. Lee, B. Lee, J. Lim (LGE)]

Motion compensation for affine prediction uses 8-bit precision filters with 1/64 pel motion vector accuracy. This contribution proposes simplification of the motion compensation filter for affine inter prediction. The proposed method uses 6-bit filter taps with 1/16 pel motion vector accuracy, which is already used for motion compensation in some case in the JEM. The proposed modification reportedly shows negligible (0.0%, 0.1%, and 0.1%) BD rate impact for random access, low delay B, and low delay P configurations, respectively.

Modification 1 unifies the accuracy to 1/16 pel for luma and 1/32 pel for chroma.

Modiification 2 uses 6-bit filter taps. Other than described in the contribution, 6 bit precision is also used for chroma, such that the design of interpolation filters would be fully unified for the affine and non-affine cases.

Decision: Adopt for JEM3.

JVET-C0093 Cross-check of JVET-C0025: Simplification of motion compensation filter for affine inter prediction [H. Zhang, H. Yang (Huawei)] [late]

JVET-C0027 Simplification and improvements for BIO design in JEM2.0 [E. Alshina, A. Alshin (Samsung)]

This contribution suggests 3 modifications of the bi-directional optical flow (BIO) design in JEM. The first modification is to change the maximum allowed magnitude for motion vector refinement in BIO. An average performance improvement from enlarging the MV regiment threshold is reportedly 0.1% (RA). The second change is to eliminate some redundant calculations per PU, thus reducing encoding and decoding run time (with no change of compression performance). The third modification is BIO extension to support up to 1/16 MV accuracy. This modification reportedly provides 0.07% average performance improvement in RA and 0.04% in LBD. Together the three modifications reportedly provide 0.2% average performance improvement in the RA configuration. The gain is reportedly higher for high resolution classes.

The reduction in complexity is almost negligible (mostly aspect 2), and some more memory may be needed for aspects 1 and 3.

BIO uses 6-tap filters for the motion estimation; it was pointed out by other experts that it might be useful to investigate the usage of even shorter filters.

One question was raised whether the threshold for the maximum range is still appropriate for the new larger GOP size.

One expert suggested that there could also be an option to adapt/signal the threshold value.

Decision: Adopt C0027 (all three aspects).

It was also agreed to establish an AHG on simplification of decoder-side motion derivation tools, to study the aspects listed above (and others).

JVET-C0065 Cross-check of JVET-C0027 [J. Lee, S. Yoo, J. Nam, J. Lim (LGE)] [late]

JVET-C0072 Cross-check of JVET-C0027 (BIO simplification and improvements) [X. Li (Qualcomm)] [late]

The cross-checker has some concern that the adaptation of the threshold may be mainly beneficial due to the change of the GOP size in the CTC, and that it may cause a loss when GOP size is changed to a lower value. Further, it is pointed out that some chroma loss occurs.

JVET-C0080 Cross-check of JVET-C0027 (Simplification and improvements for BIO design in JEM2.0) [T. Biatek (TDF), V. Lorcy (bcom), P. Philippe, P. Boissonade (Orange)] [late]

JVET-C0031 BIO improvement to reduce the encoder and decoder complexities [J. Lee, N. Park, J. Nam, J. Lim (LGE)]

In this contribution, BIO is proposed to be skipped when a constant motion condition is satisfied, to reduce the encoder and decoder complexity. The proposed method reportedly reduces the encoder and decoder complexity in JEM2.0 about 4–10% and 15–26%, respectively, with a marginal BD-rate increase according to the block size.

The condition checks for equal time distance and equal motion vectors (i.e. opposite sign), as detailed below.

| Test | BIO skip condition description |
| --- | --- |
| Test 1 | Skip BIO when constant motion condition is satisfied and block size is 4x4. |
| Test 2 | Skip BIO when constant motion condition is satisfied and block sizes are <= 8x8. |
| Test 3 | Skip BIO when constant motion condition is satisfied and block sizes are <= 16x16. |
| Test 4 | Skip BIO when constant motion condition is satisfied. |

One expert pointed out that the method would not decrease the worst-case complexity, and would even add an additional check. Furthermore, the encoder runtime is only marginally reduced, such that the main concern for experimentation is not resolved.

It was suggested rather to develop an overall solution for complexity reduction of BIO, where the proposal C0031 can be one of the aspects.

This was agreed to be further investigated in an AHG on simplification of decoder-side motion derivation tools.

JVET-C0052 Cross-check for BIO improvement to reduce the encoder and decoder complexities in JVET-C0031 [E. Alshina (Samsung)] [late]

The cross-checker reported that in some cases the disabling of BIO under the conditions suggested in C0031 leads to a performance drop, and pointed out some possible apporaches for how this could be resolved.

JVET-C0047 Generalized bi-prediction for inter coding [C.-C. Chen, X. Xiu, Y. He, Y. Ye (InterDigital)]

This proposal presents a generalized bi-prediction (GBi) mode that allows bi-prediction weights to be signalled at the CU level and used for the bi-predicted PUs in that CU. The candidate set of weights in the GBi mode includes a total of 7 weights, including the value 0.5 which corresponds to the conventional bi-prediction mode. An index pointing to the entry position of a weight value in the candidate weight set is proposed to be signalled. At most one index per CU is signalled, and the corresponding weight values are shared across all the PUs and all colour components in that CU. When compared with the JEM-2.0 anchor, GBi reportedly performs consistently better in all test cases, with average Y BD-rate reduction of 1.0% for Random Access cases and 1.0% for Low Delay B cases.

The Powerpoint deck used when presenting the proposal had some additional slides compared to the uploaded word file version.

There were significant increase in encoding time (>300%)

One configuration uses a negative weight. Further study of the impact of different weights would be interesting.

It is commented that the gain is practically equal when using the tool with the HM or JEM, which indicates that there is no conflict with other coding tools.

Further study in an EE was planned. Reduction of encoder complexity and alignment with QTBT need to be investigated.

JVET-C0092 Cross-check of JVET-C0047: Generalized bi-prediction for inter coding [S.-H. Kim (Sharp)] [late]

JVET-C0062 Improved affine motion prediction [F. Zou, J. Chen, M. Karczewicz, X. Li, H.-C. Chuang, W.-J. Chien (Qualcomm)]

This document presents an improved affine motion prediction scheme in the current reference software JEM2.0. A 6-parameter affine motion model is used in addition to the existing 4-parameter affine motion model. The scheme was tested on top of JEM2.0. Simulation results reportedly show that the proposed scheme achieves 0.3% BD rate reduction on the random access configuration in JEM2.0, and 0.5% BD rate reduction on the random access configuration in an HM test.

Encoder runtime increased by approximately 10%

This proposal switched between 4 and 6 parameter models at the PU level, only for the 2Nx2N case. In the case of QTBT, it would always be applied. In the case of 6 parameters, three corner vectors have to be signalled.

It was reported that always using 6 parameters does not have good performance.

In affine merge mode, no additional signalling is necessary.

The cross-checker reports that in affine merge mode, the order is modified.

Higher gain was shown for class A2.

Investigation in an EE was encouraged, potentially including additional material with affine motion for better understanding of the effects.

JVET-C0083 Cross-check of JVET-C0062 Improved affine motion prediction [O. Nakagami (Sony)] [late]

JVET-C0068 Motion vector coding optimizations [J. Samuelsson, P. Wennersten, R. Yu, U. Hakeem (Ericsson)]

This contribution proposes three new motion vector coding tools for inclusion in the JEM software. The first tool derives the sign of one of the motion vector components from other coded information, the second tool allows one of the motion vectors in bi-prediction to be used as predictor for the other, and the third tool uses a new rounding criteria for full-pixel motion vectors that depends on the selected MVP index.

Preliminary results indicate that with FRUC, high-precision motion vectors and affine motion disabled, the overall BD-rate change for non-4k sequences is −0.6% / −0.6% / −0.7% Y/U/V for RA.

The presentation gives substantial additional information in terms of results which are not contained in the version 1 file that was available at the time of discussion. The contribution was later revised to include the presentation and a modified version of the document.

The method seems to provide interesting gain, but no results using the CTC were initially given.

This was later reviewed again after a new version is uploaded. The new results were based on CTC conditions with all JEM tools of CTC enabled. However, the results excluded some test sequences for which the encoding crashed. Otherwise, it was reported that no mismatch was found between the encoder and decoder.

Gains of individual tools:

- motion sign hiding 0.1%

- prediction list candidate update 0.2%

- full and half pel candidate rounding 0.1%

The combined improvement was 0.5%; and the three tools seems to be additive in terms of gain.

Further study in an EE was planned.

## Intra coding (13)

JVET-C0023 Predictors Elimination Technique for HEVC [M. Korman, O. Prosekov (Synopsis)]

(Presented Tue 13:15.)

The HEVC specification defines multiple different Intra Prediction modes for spatial prediction on the PU level. Indexes identifying selected prediction modes for luma and chroma prediction blocks constituting each CU are coded into the CU header. Prediction mode coding involves an orientation-aware MPM selection technique impacting the predictability of intra prediction directions and thus reduced representation of mode index information. Intra Mode indexes still may occupy up to 50% or even 70% of the rate for Intra-only coded sequences and still images, depending on prediction unit size.

While HEVC intra prediction algorithms are all different, for a particular square region of an image application of those algorithms may lead to predictor blocks that do not differ from each other, or to predictor blocks that can be considered close to each other with respect to a given QP value and thus grouped, or eliminated. Grouping reduces the source alphabet for mode indexes, and the reduced alphabet leads to a shorter code required for prediction algorithm representation in the bitstream. On the other hand, modes grouping might lead to direction information lost and, as a consequence, MPM selection coding efficiency reduced.

This contribution proposes a QP dependent similarity criteria for predictor blocks as well as modifications to existent intra modes coding that allow content-dependent grouping of equal (or similar) predictors (aka a predictors elimination technique) for the price of computational complexity in the order of the prediction modes number. A modified MPM selection algorithm enabling orientation-aware operation on group numbers in place of mode indexes is also proposed.

Parsing of content-dependent group indexes can only be by performed by the decoder when all the pixels required for predictors construction are available, thus the contribution proposes CU bitstream reordering.

Proposed modifications were verified with intra PUs of size 4x4, and average BD-PSNR results computed on the whole range of HEVC quantization parameter values from 1 to 50 reportedly indicate an average of 16% gain in BD-rate and about 0.9 dB improvement in BD-PSNR values on a set of 25 test sequences compared to HEVC operating with the same PU size.

Comparison is made in the context of using fixed block sizes. Results using RD optimization for variable block size decision should be used, this is, de facto, what the proposed “grouping” of 4x4 blocks does in a different way

Investigation of this in the context of the JEM was requested (currently it was tested only for HEVC).

The scheme might be similarly be implemented as encoder-side method to decide about block sizes without making a syntax change.

Further study was encouraged.

JVET-C0043 Arbitrary reference tier for intra directional modes [Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI)]

The intra prediction method of the JEM is composed of various kinds of intra prediction modes, i.e., the planar mode, DC mode, and different intra directional modes. Traditionally, only one reference line of samples adjacent to the current prediction unit (PU) is exploited to predict the samples inside the PU. This contribution proposes to increase the number of the exploited reference lines for intra directional modes. Compared to JEM 2.0, the simulation results reportedly show that the proposed method with 2 reference lines can improve up to 0.73% BD-rate for overall test sequence classes and 1.21% BD-rate for the A2 class, featuring 4K sequences with unmanned content. It is further reported that the proposed method with 3 / 4 reference lines can improve up to 1.05% / 1.22% BD-rate for overall classes and 1.73% / 1.98% BD-rate for A2 class.

Reference lines as described above are additional lines/columns from boundary or non-boundary samples of reference blocks. These are used instead of only boundary samples. Computation at the decoder is not increased, but more line buffers are necessary.

Results were only reported for AI. RA simulations were not completed. The method significantly increases encoding time (400% and more for N=4).

There were also results with a “fast” version with 200% for N=4 where the average gain is 0.76%.

Generally, several experts expressed an interest to study this technology, as it gives good gain while not significantly increasing decoder computer decoder.

It was agreed to establish an EE; the EE should also compare against the JEM with a similar complexity of encoder search for conventional intra prediction, where a comparable number of intra prediction modes are tested.

JVET-C0090 Cross-check of JVET-C0043 (Arbitrary reference tier for intra directional modes) [T. Ikai (Sharp)] [late]

JVET-C0098 Cross-check of arbitrary reference tier for intra directional modes (JVET-C0043) [B. Li, J. Xu (Microsoft)] [late]

JVET-C0055 Neighbour based intra most probable modes list derivation [V. Seregin, X. Zhao, A. Said, M. Karczewicz (Qualcomm)]

This contribution presents intra mode coding with modified most probable mode (MPM) list derivation in order to simplify the current JEM derivation process. In the proposed derivation of the MPM list, intra prediction modes of the already coded neighbour blocks are included into the list in a manner similar to building a merge motion vector candidate list. Additionally, the number of contexts used to code most probable mode index is reduced from nine to three. Simulation results reportedly show −0.1%, −0.1%, −0.1% BD rate impacts for luma and the two chroma colour components for JEM-2.0 with All Intra (AI) coding configuration, and −0.2%, −0.1%, −0.2% for QTBT with AI coding configuration, respectively.

A slight decrease in encoder and decoder run time was reported.

Several experts expressed support, as the approach is a clear simplification, removing 6 contexts and saving a counter in the MPM derivation, while also giving gain.

Additional results were also presented that reported that the BR reduction can go up to 0.6% on average if more modes are tested; although increasing encoder runtime to 150%.

Decision: Adopt to JEM3.

JVET-C0081 Crosscheck of neighbour based intra most probable modes list derivation in JVET-C0055 [H. Huang (MediaTek)] [late]

JVET-C0061 Decoder-side intra mode derivation [X. Xiu, Y. He, Y. Ye (InterDigital)]

In this contribution, a decoder-side intra mode derivation (DIMD) approach is proposed based on the JEM. The DIMD approach derives the intra prediction mode and is asserted to reduce the overhead of intra mode signalling. It is asserted that because the only mode signalling needed in DIMD is the indication to use the DIMD mode and the level at which the prediction operates, intra prediction can be performed at higher granularity than in the JEM. For an intra CU with 2Nx2N partition, one DIMD flag is signalled. When the flag is true, the intra mode of the CU is derived using a template-based method at both encoder and decoder. Additionally, when the DIMD mode is selected, another control flag is signalled to indicate whether the intra mode is derived at PU-level or TU-level. For the intra CU with NxN partition, the intra modes derived by DIMD are used to replace one exiting MPM candidate for the intra mode coding of four PUs in this CU. It is reported that, compared to the JEM-2.0 anchor, the proposed method achieves on average 0.81%, 0.47%, 0.21% and 0.23% luma BD-rate reductions for AI, RA, LDB and LDP configurations, respectively; when compared to the anchor by disabling all JEM tools, the corresponding average BD-rate reductions are reported as 1.37%, 0.72%, 0.27% and 0.40%.

A one-line-wide reference is used to determine an intra prediction direction for predicting using an L-shaped template which has a width of 2 or 4 samples, depending on the block size.

A fast search is used; however, the encoder and decoder runtime is still increased by roughly 50%. In the worst case, 20 modes have to be tested.

DIMD, in the case of 2Nx2N is signalled by an additional flag, no MPM is used, and another additional flag is also added that indicates whether it is invoked at the TU or PU level. For NxN, DIMD is put in the first place of the MPM list. A question was raised why this is not done equal? It is reported that it would have some loss.

The technology gives interesting gain, but increases decoder (and encoder) complexity significantly. The main aspect would be reducing the number of searches at the decoder side, to achieve a better tradeoff of complexity vs. compression benefit (and still retain the gain in compression).

Further investigate in EE was planned, with the following aspects to be investigated:

- limitation of number of lines used in the search (1, 2, 4)

- restriction of not using it for small blocks

- restricting number of candidates further

- removing parsing dependencies (so it shall not rely on reconstructed samples)

- SIMD for the decoder (but should be same for intra and inter)

- align with QTBT; if it is intended to still retain two different ways of treating for larger and smaller blocks (as currently for 2Nx2N n NxN), the benefit of this should also be reported

- comparison against an anchor with similar encoder complexity (using other than the fast search for intra mode), and also identifying whether decoder-side derivation still gives similar gains when the encoder would check more modes

JVET-C0094 Cross-check of JVET-C0061: Decoder-side intra mode derivation [X. Ma, H. Yang (Huawei)] [late]

JVET-C0069 Direction-dependent scan order with JEM tools [S. Iwamura, A. Ichigaya (NHK)] [late]

This contribution proposes a change of direction-dependent sub-TU scan order (DDSO) by reference-position-dependent residual flipping. This contribution also analyzes an interaction with intra-related tools integrated on JEM software, and proposes change of PDPC tools to harmonize with DDSO. The intra boundary filter is disabled since the top/left reference samples utilized for the filtering process may not be available when the scan order is changed by the proposed DDSO. The proposed algorithms are implemented on top of JEM1.0 without intra boundary filtering and experimental results reportedly show −0.16% (Y), −0.21% (U), −0.36% (V) BD-rate changes for the AI configuration. Additional experiments are conducted to compare with JEM1.0, which showed 0.02% BD-rate loss for luma with 13% decoding time decrease.

The reported increase of decoder runtime (3%) may be due to inefficient implementation, as it is unlikely that the additional logic for switching the scan order and flipping the residual are requiring much additional computation. The cases where decoder runtime was reduced were those where boundary filtering was disabled. Generally, it would be desirable to find a solution for running the tool also in combination with IBF.

Further study in combination with QTBT was encouraged.

JVET-C0071 Multiple line-based intra prediction [J. Li (Peking Univ.), B. Li, J. Xu (Microsoft), R. Xiong (Peking Univ.), G. J. Sullivan (Microsoft)] [late]

This was presented Sun. morning (chaired by JRO).

This document presents a multiple line-based intra prediction method. In the proposed method, not just the nearest reference line but also further reference lines are utilized in intra prediction. The prediction generated from a further reference line will compete with the prediction generated from the nearest reference line in order to choose the best prediction for each CU. When a further reference line is used, a residue compensation procedure is introduced to further refine the prediction. In addition, this proposal also includes several optional acceleration algorithms to mitigate the encoding complexity. The proposed method is integrated on top of JEM-2.0, and the experimental results reportedly show that for all-intra coding, the fast version of the proposed scheme saves 0.9% bits on average, while increasing encoding time about 66%.

This is imilar to C0043 (using up to four lines/columns), with additional residue compensation to avoid discontinuities. With full search, the encoding runtime goes up to approximately 250%, BD-rate reduction is 1.1% in AI.

Some RD penalty is introduced for farther lines in the encoding selection process.

Investigate in an EE (along with C0043) was planned. In addition to the information already mentioned under C0043, it was requested to investigate the benefit of residual compensation, which would introduce some additional decoder complexity.

JVET-C0082 Cross-check of JVET-C0071 (Multiple line-based intra prediction) [T. Ikai (Sharp)] [late]

JVET-C0096 Cross-check of JVET-C0071 (Multiple line-based intra prediction) [L. Zhang, V. Seregin (Qualcomm)] [late]

JVET-C0097 Cross-check of JVET-C0071 (Multiple line-based intra prediction) [P.-H. Lin, Y.-J. Chang, J.-S. Tu, C.-C. Lin, C.-L. Lin (ITRI)] [late]

## Partitioning (3)

JVET-C0039 Decoupled Luma/Chroma Transform Trees for Intra [F. Urban, T. Poirier, F. Le Léannec (Technicolor)]

In this contribution, a chroma transform tree decoupled from the luma transform tree is proposed for intra CUs. A chroma split transform flag is proposed to be added, which indicates whether the current chroma block is further split into 4 transform units. In addition, the chroma transform tree cannot be split if the luma transform tree is not split. It is reported that the proposed method provides 0.14% BD-Rate saving in luma and around 1.5% gain in chroma in AI and RA coding conditions, with encoding and decoding time identical to those of the JEM2.0.

In QTBT, luma and chroma have separate trees for intra anyway, and the bug that is reported for JEM2 is not present anymore.

It was claimed that the dependency of the luma and chroma trees might give some compression benefit; however, the contribution does not give evidence on that, as only results for separate trees with dependent coding are presented, but it could be the case that the same gain is achieved with independent coding (note that QTBT performs independent coding of luma and chroma trees, which also has been reported to provide gain compared to using a common tree).

Further study was encouraged to provide evidence that the dependent coding would provide benefit.

JVET-C0079 Cross-check of JVET-C0039 Decoupled Luma/Chroma Transform Trees for Intra [O. Nakagami (Sony)] [late]

JVET-C0054 Grouped signalling for transform in QTBT [X. Zhao, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

This contribution presents a grouping method for transform-related syntax elements. It is applied to the QTBT framework, where an EMT flag is signalled for the group of blocks. The block area size, at which signalling happens, can be configured; the size of the area is sent in the slice header. After the EMT flag is signalled for the area, the blocks included into the area will share the same signalled EMT flag value. Simulation results reportedly show 0.2% luma BD rate saving for All Intra (AI) configuration on average.

This has a small modification in signalling. It was agreed to include it in an EE on primary transform (see under C0022).

## Other (6)

JVET-C0026 Tiles coding improvement for Inter pictures by improved merge list at tile boundaries [S. B. Raut (Samsung)]

This contribution presents a method to change the merge list at tile boundaries when encoding inter picture with multiple tiles. The proposed method derives new temporal candidates from reference pictures for each unavailable spatial merge candidate at tile boundaries. The new proposed candidates are appended at the end after the normally available candidates in the merge list. This proposed method is asserted to help the merge list by making up for the unavailable spatial candidates at tile boundaries with new temporal candidates. BD-rate improvements of 0.02% and 0.04% were reported for LDB and LDP configurations, respectively, with the proposed method, when picture is encoded with JEM2.0 using four tiles. It was asserted that the BD-rate improvement will further increase with the increase in the number of tiles used to encode the picture.

This seemed not relevant in the current exploration, as only a single tile is used in the CTC. No need for action was identified.

JVET-C0032 Experiment on polyphase subsampled sequence coding [E. Thomas (TNO)]

This contribution presents some test results of the technique presented in JVET-B0043. The proposed technique decomposes the input video signal (luma and chroma components) by subsampling into lower resolution descriptions, possibly using polyphase subsampling. The multiple lower resolution versions of the signal are encoded and transported in the same video bitstream. This enables the decoder to select and decode the appropriate number of resolution components for the desired output resolution, i.e. one for half resolution of the original stream or all of them for high resolution. Use cases for zero footprint network transcoding and a bi-resolution broadcast channel are also described. This contribution presents a BD-rate comparison between encodings of temporally frame packed sequences for random access and low-delay for classes C and D. The reference for these results is the simulcast encoding, i.e., encodings of original test sequences combined with encodings of the test sequence in half resolution. The provided results are only partial and reflect the worst-case scenario where no optimization of the common test conditions was applied to the encoding of the TFP sequences. For instance, the intra period and the GOP size have not been modified. The contributor reported that the TFP technique outperforms the simulcast for some test sequences, namely for RaceHorse class C for random access by 33.9%.

The results were considerably worse than simulcast for all sequences except RaceHorse.

Probably, due to the missing filtering, aliasing problems occur. It is reported that for some sequences (e.g. BQ square), artefacts were visible, however less for 4K material.

The CTC was used, which is probably not appropriate here, since e.g. the quantizer settings are different for the different polyphases.

It is pointed out that H.261 has a still image mode which works with a similar polyphase technique in a temporal sequence.

A relationship with multiple description coding is also pointed out.

The advantage compared to SHVC-style scalable coding comes by the fact that fewer samples need to be processed. However, the rate penalty is significant.

Better clarification is needed of which requirements are intended to be addressed by the method.

It is also not clear that the method proposed gives benefit (in terms of reconstruction quality or lower bit rate) over other multi-resolution methods that are existing.

This was considered not to be in the scope of current JVET mandates, which are targeting exploration of improved compression.

JVET-C0078 Cross-check of C0032 (polyphase subsampled sequence coding) [P. Philippe (Orange), V. Lorcy (bcom), T. Biatek (TDF)] [late]

JVET-C0040 Adaptive Clipping in JEM2.0 [F. Galpin, P. Bordes, F. Le Léannec (Technicolor)]

In this contribution, a clipping process within the JEM2.0 coding and decoding process is proposed.

The clipping bounds are determined from the original signal characteristics and encoded in the bitstream.

BD-rate savings of 0.35% is reported in Random Access Configuration, 0.17% in All-Intra and 0.23% in low-delay B. No encoding or decoding runtime increase is reported compared to the JEM2.0 anchor.

Adaptive clipping is performed in all stages where currently clipping to the normal range happens (including the prediction).

During the discussion, it was pointed out that similar methods had been proposed in HEVC standardization (e.g. JCTVC-C146, JCTVC-D123). Gains reported at that time were in a similar range (0.4–0.6%).

Adaptive clipping at various stages is slightly more complex than fixed clipping.

Investigation in an EE was planned , where additionally the following investigations should be performed:

- Clipping only as post processing, not in the loop (which could be indicated by an SEI message)

- Investigate possible impact on visual quality

JVET-C0084 Cross-check of C0040 (Adaptive Clipping) [P. Philippe (Orange), V. Lorcy (bcom), T. Biatek (TDF)] [late]

JVET-C0049 Extended deblocking-filter process for large block boundary [K. Kawamura, Q. Yao, S. Naito (KDDI Corp.)] [late]

This contribution proposes an extension of the deblocking filter process for large block boundaries. In HEVC, both large transform blocks and large prediction blocks were introduced. When a large block is used for a smooth area where luma value has little variation and/or gradation, blocking artefacts become more visible at low bit rates. In addition, when a texture region is coded by a large block with neighbouring blocks, the large block boundary is sometimes visible due to the difference of texture coding. For improving the visual quality, this contribution proposes an extension of the edge filtering process from six filtered samples to 14 filtered samples only for the boundary of large blocks. While BD-rate is degraded, the visual quality on large QP is reportedly subjectively improved.

Proponents were requested to organize a viewing session, such that other (independent) experts could confirm that quality improvement is visible. This should also include those sequences of class E LDB where a huge BD drop was observed. If a subjective benefit is reported, further study should be performed in the context of QTBT block structures.

The viewing session was organized, and the results are described as follows:

During the JVET-C meeting, a viewing session was conducted from 2 p.m. to 3 p.m. on 3 May, in the viewing room. Roughly 25 experts attended the viewing session. Two sequences from class B and one from class C with random access and low-delay B conditions were shown. One sequence from class E with low-delay B condition was also shown. Other sequences and conditions were shown based on further requests. It is noted that the display refresh rate is 59.94, and the displaying frame rates were set as 59.94 or 29.97 which was independent of the source sequences’ frame rate like 24 and 50fps. Experts noticed small differences between the video coded by JEM2.0 and the proposal; however, no clear judgement was possible whether those differences provide improvement or degradation in quality (both cases were observed). In class E, most experts had the opinion that some degradation like additional blockiness with small block sizes was recognized.

Further clarification (particularly in the context of QTBT) is needed whether a problem with deblocking of large block sizes exists, and if yes, whether it can be resolved without compromising quality in the case of small block sizes.

# Perceptual metrics and evaluation criteria (2)

JVET-C0030 Perceptual Quality Assessment Metric MS-SSIM [H. Zhang, X. Ma, Y. Zhao, H. Yang (Huawei)]

Objective video quality assessment metrics were noted to play an important role in a broad range of video signal processing applications, such as video compression, video restoration and so on. The most widely used quality assessment metric is PSNR, but it does not take into account the response of the HVS to the distortions, and sometimes cannot align with the judgement that a human would make when watching the video. A higher PSNR value may correspond to a poorer subjective experience in some video sequences, which is not desired. In this proposal, MS-SSIM, which was reported to have been proved more reliable in judging visual quality than PSNR, is suggested being used as an additional metric for quality assessment. MS-SSIM is a multi-resolution version of SSIM.

Comparison of the metrics SNR, SSIM and MS-SSIM was made with 5 image databases and 3 video databases. In most cases, it reportedly had best correlation with subjective quality; however, in one of the video databases, SSIM reportedly performed better.

The performance was reported to seem to be dependent on the bit rate/quality range.

The range of SSIM in the JVET CTC would be between 0.9 to 1. It seemed questionable whether it would be meaningful to conclude about the real quality from this. The proponents reported that they have found some meaningful results in computing BD-rate based on MS-SSIM, however BD-SSIM does not indicate much.

Results might be very different depending on whether fundamentally different coding algorithms are compared, and what the quality differences are. If the comparison is just made for a tool-on/tool-off experiment, it might be questionable how much we could conclude from that.

One option was discussed that UBC would be willing to run a subjective test to generate a database that could be used to investigate the performance of different metrics (including MS-SSIM). This would, however, mean that bitstreams have to be generated at the same bit rates (e.g. HEVC vs JEM, or JEM with different tools enabled). This would require availability of 6–8 sequences at 4 rates. Volunteers are needed to generate such sequences. See further notes under BoG C0104.

JVET-C0033 On comparison criteria for Virtual Reality video coding schemes [E. Thomas (TNO)]

Reviewed in BoG JVET-C0104.

# Joint meetings, BoG reports, and summary of actions taken

## General

The setup of Exploration Experiments was discussed, and an initial draft of the EE description document was reviewed in the plenary (chaired by JRO). This included the list of all tools that are intended to be investigated in EEs during the subsequent meeting cycle:

- EE1: Secondary transform (HYT) and comb. PDPC/NSST – from old EE3/EE7 / C0042, C0053, C0063

- EE2: Adaptive primary transform – from C0022, C0054

- EE3: Generalized bi-prediction – from C0047

- EE4: Improved affine motion prediction – from C0062

- EE5: Improved MV coding – from C0068

- EE6: Extended intra pred reference – from C0043, C0071

- EE7: Adaptive clipping – from C0040

- EE8: Decoder side intra mode derivation – from C0061

- EE9: Adaptive scaling for extended colour volume material – from C0066, C0095, C0102

The EE document was requested to be prepared by Elena Alshina (coordinated with Jill Boyce) – to be reviewed on Tuesday.

It was agreed to give the editors the discretion to finalize the document during the two weeks after the meeting, and to circulate/discuss it on the reflector appropriately.

## Joint meetings

One joint meeting of the parent bodies was held to discuss JVET activities.

Tuesday 0900–1000 Joint discussion of JVET, Q6/16, MPEG Video & Requirements

- Topics suggested for potential discussion included VR in JVET, HDR in JVET, the m38766 RF baseline proposal contribution, and application requirements for JVET

- The m38766 RF baseline proposal was designated for further study by parent bodies

- About the scope of JVET work and application requirements for future video:

  - HDR, yes, the coding of HDR video is within the scope of JVET work

  - There should be a 10 bit emphasis

  - Screen content – 4:4:4? – yes, this should be studied in JVET, but it is not the emphasis

  - JVET should be trying to gather information in a broad sense

  - A suggested scope for JVET was outlined by Q6/16 (VCEG)

  - The following were agreed to be in scope for JVET (based on the scope suggested by Q6/16; which seems acceptable pending further review, with no need to explicitly describe what is out-of-scope):

    - HDR/WCG as well as SDR

    - Panoramic and spherical video

    - Stereo panoramic and spherical video

    - Study of projection types

    - Camera and computer graphics generated (computer-rendered, i.e., screen) content

  - It was noted that scalability, VR, and projections are identified in the current MPEG requirements document for future video coding as well as being proposed as in-scope by Q6/16.

BoGs

JVET-C0100 Report of BoG on JEM software and SCC tools [X. Li]

This is the report of BoG activity on JEM software and SCC tools. The mandates of the BoG were as follows

- Investigate the implementation of SCC tools in JEM

- Identify potential difficulties that may occur during integration

A meeting of the BoG was held on Friday, May 26, 2016 at 2:30 p.m.

General discussion:

- SCC tools to be integrated

  - Current Picture Referencing (IntraBC)

  - Palette mode

  - Motion vector resolution control

    - Slice level tool. Potential overlapped with block level adaptive MV resolution

  - Residual adaptive colour transform

    - For 4:4:4 content only.

- It was agreed to focus on two tools (Current Picture Referencing/intraBC and palette mode) for now.

- Implementation difficulties of each tool

|  | CPR (IntraBC) | Palette Mode |
| --- | --- | --- |
| Independent module or not | A lot of changes may be needed in code | Relative independent module |
| Encoder only | Many encoder only optimizations which may not be efficient for natural content. May be implemented with lower priority. | - |
| Memory consumption | Hash-based search (encoder only) may be with lower priority | It should not be an issue in the latest SCM software |
| Interaction with other tools | A) On top of QTBT, there are separated trees for luma and chroma. Then luma component and chroma components may have different block vectors, which may lead to visual artefact. Solution: When CPR (intraBC) is enabled, I slice will be regarded as P/B slice so that no separated trees will be there. Then no issue. B) Potential interaction with other inter coding tools, e.g. ATMVP. | A) Similar to CPR (intraBC), when I slice is regarded as P/B slice, no issue. A sequence level flag is needed to enable/disable separated trees in I slice. A question here, if we enable palette mode but disable CPR, what to do for palette mode? - One solution would be to disable separated tree when palette is enabled. - Another solution would be separated palette modes for luma and chroma in I slices with separated trees. |

It was noted that not all related code for an SCC tool are marked by macros in SCM-7, which increases porting difficulty when changes are needed in many modules.

It was remarked that there may be many ways to implement the details of SCC tools on top of JEM software with QTBT adopted. It would be better to discuss the details before integration. It may be desirable to have a defined process for such implementation.

Due to the adoption of QTBT, a simple transfer of SCC tool software does not seem to be possible.

It was planned to conduct further study of these aspects in an AHG (as a new mandate of the software AHG).

JVET-C0101 Report of BoG on QTBT configuration setting [K. Choi]

This document provides a report on the study of QTBT configuration settings. The meeting was held in C2 at 9:00–10:00.

Mandates

- To further study detailed results of the EE

- To study possibilities of reducing the encoder complexity

- To define default settings of QTBT for CTC (e.g. max CTU size, separate or non-separate trees for luma and chroma in case of intra).

Mandates 1 & 2

Parameters regarding QTBT

- CTUSize: Basic unit structure of CBs by a QTBT structure

- MinQTLumaISlice: Quadtree leaf node size for luma in I slice

- MinQTChromaISlice: Quadtree leaf node size for chroma in I slice

- MinQTNonISlice: Quadtree leaf node size in B or P slice

- MaxBTDepthISliceL: Binary tree depth for luma in I slice

- MaxBTDepthISliceC: Binary tree depth for chroma in I slice

- MaxBTDepth: Binary tree depth in B or P slice

During the EE study of QTBT, three tests (i.e., Test 1, Test 2, and Test 3) were tested and some of the results were available. The configuration set and results are shown in the tables below.

- Anchor: JEM 2.0

- Test Sets: QTBT code in EE branch with Test 1, 2, and 3

Configuration sets of QTBT

| Param. | Test1 | Test2 | Test3 | Test4 | Test5 |
| --- | --- | --- | --- | --- | --- |
| CTUSize | 128 | 128 | 128 | 256 | 256 |
| MinQTLumaISlice | 16 | 8 | 8 | 8 | 8 |
| MinQTChromaISlice | 4 | 4 | 4 | 4 | 4 |
| MinQTNonISlice | 16 | 8 | 8 | 8 | 8 |
| MaxBTDepthISliceL | 4 | 2 | 3 | 2 | 3 |
| MaxBTDepthISliceC | 0 | 2 | 3 | 2 | 3 |
| MaxBTDepth | 4 | 2 | 3 | 2 | 3 |
| MaxBTSizeISliceL | 32 | 32 | 32 | 32 | 32 |
| MaxBTSizeISliceC | 16 | 16 | 16 | 16 | 16 |
| MaxBTSizeBPSlice | 128 | 128 | 128 | 128 | 128 |

Available test results

|  | Measure | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| AI | BD-rate (%) | −3.3%/−5.6%/−4.6% | −2.0%/−10.4%/−9.8% |  |
|  | Encoding time (%) | 561% | 251% |  |
|  | Decoding time (%) | 110% | 110% |  |
| RA | BD-rate (%) | −3.8%/−8.9%/−8.3% | −1.6%/−9.9%/−9.7% |  |
|  | Encoding time (%) | 2xx% | 137% |  |
|  | Decoding time (%) | 1xx% | 108% |  |
| LD | BD-rate (%) | −4.5%/−5.0%/−5.8% | −2.9%/−4.6%/−5% | −3.9%/−5.6%/−6.0% |
|  | Encoding time (%) | 241% | 131% | 194% |
|  | Decoding time (%) | 106% | 109% | 106% |
| LDP | BD-rate (%) | −4.4%/−5.3%/−5.6% | −2.8%/−4.7%/−5.0% | −3.8%/−5.8%/−6.3% |
|  | Encoding time (%) | 221% | 118% | 175% |
|  | Decoding time (%) | 118% | 118% | 118% |

Note that the better performance of test 2 in chroma, compared to test 1, is due to the increase of MaxBTDepthISliceC from 0 to 2. It might be interesting to modify test 2 with this parameter also set to 0 to bring the encoder runtime further down. (denoted as “Test 2a”) further below.

Test1 (the configuration of the EE) had been cross checked by two companies, and the results were matched with the result of proponent. Test 2 with AI had been cross checked, but the other scenarios had not been cross checked. The proponent provided additional test result based on Test 3 configuration, but the set has not been cross checked yet.

Two additional test sets were suggested to verify the performance of QTBT (i.e., Test 4 and 5). However, these would likely have even higher runtime than Test 2 and Test 3.

During the BoG meeting, people agreed that practically testing all test sets rapidly is impossible especially for Class A sequences (4k) because testing Class A requires more than 2 weeks.

It is suggested that the other test sets are evaluated at the next meeting by establishing an ad hoc group regarding QTBT encoder configuration.

After the BoG meeting, one company said that they will Test 3 during this meeting. Table 3 shows the volunteers to test configuration sets from Test 2 to 4.

Testers of QTBT during this meeting

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| Tester 1 | Samsung (Available) | Samsung | Qualcomm |
| Tester 2 | Qualcomm (Available) |  |  |

Mandate 3

Default settings of QTBT for CTC were agreed to be decided based on the test results obtained during this meeting

Initial recommendations by the break-out group

- To test and verify Test 2 and Test 3 during this meeting.

- To define default settings of QTBT in an additional BoG meeting during this meeting

- To establish an ad hoc group to get the best configuration setting for QTBT

Initial results were presented in JVET Fri afternoon. It was confirmed that the BoG should further investigate the Test 2 and Test 3 cases and report results later. It was agreed that if it is unrealistic to get full results for class A during the meeting, results with partial sequences should be reported along with an analysis of how homogeneous the performance in Test 1 was over the entire sequence.

If possible, additional results should be reported for Test 2a.

The BoG met again on Monday afternoon and provided the following results and recommendations.

Configuration sets of QTBT

| Param. | Test1 | Test2 | Test3 | Test4 | Test5 | Test6 |
| --- | --- | --- | --- | --- | --- | --- |
| CTUSize | 128 | 128 | 128 | 256 | 256 | 128 |
| MinQTLumaISlice | 16 | 8 | 8 | 8 | 8 | 8 |
| MinQTChromaISlice | 4 | 4 | 4 | 4 | 4 | 4 |
| MinQTNonISlice | 16 | 8 | 8 | 8 | 8 | 8 |
| MaxBTDepthISliceL | 4 | 2 | 3 | 2 | 3 | 2 |
| MaxBTDepthISliceC | 0 | 2 | 3 | 2 | 3 | 0 |
| MaxBTDepth | 4 | 2 | 3 | 2 | 3 | 2 |
| MaxBTSizeISliceL | 32 | 32 | 32 | 32 | 32 | 32 |
| MaxBTSizeISliceC | 16 | 16 | 16 | 16 | 16 | 16 |
| MaxBTSizeBPSlice | 128 | 128 | 128 | 128 | 128 | 128 |

Available test results

|  | Measure | Test1 | Test2 | Test3 |
| --- | --- | --- | --- | --- |
| AI | BD-rate | −3.3%/−5.6%/−4.6% | −2.0%/−10.4%/−9.8% |  |
|  | Encoding time | 561% | 251% |  |
|  | Decoding time | 110% | 110% |  |
| RA | BD-rate | −3.8%/−8.9%/−8.3% | −1.6%/−9.9%/−9.7% | *−3.0%/−11.4%/−12.6% |
|  | Encoding time | *259% | 137% | *206% |
|  | Decoding time | *105% | 108% | *107% |
| LD | BD-rate | −4.5%/−5.0%/−5.8% | −2.9%/−4.6%/−5% | −3.9%/−5.6%/−6.0% |
|  | Encoding time | 241% | 131% | 194% |
|  | Decoding time | 106% | 109% | 106% |
| LDP | BD-rate | −4.4%/−5.3%/−5.6% | −2.8%/−4.7%/−5.0% | −3.8%/−5.8%/−6.3% |
|  | Encoding time | 221% | 118% | 175% |
|  | Decoding time | 118% | 118% | 118% |

(*: Average result of Class B, C, and D)

Test 1 had been cross checked by two companies and the results were matched with the result of proponent. Test 2 with AI had been cross checked, but the other scenarios had not been cross checked. The proponent provided additional test results based on the Test 3 configuration, but the set had not been cross checked yet.

Two additional test sets were suggested to verify the performance of QTBT (i.e., Test 4 and 5).

Among 5 test sets, Test 2 is expected to show the lowest encoding time. The group agreed on testing Test 2 only during the meeting and see the results in additional BoG meeting.

It was suggested that the other test sets be evaluated at the next meeting after establishing an ad hoc group regarding QTBT encoder configuration to generate the results.

After BoG meeting, one company said that they could run Test 3 during this meeting. The table below shows the volunteers to test configuration sets from Test 2 to Test 6.

Testers of QTBT during this meeting

|  | Test 1 | Test 2 | Test 3 | Test 6 |
| --- | --- | --- | --- | --- |
| Tester 1 | Samsung (Available) | Samsung | Qualcomm | Ericsson |
| Tester 2 | Qualcomm (Available) |  |  |  |

Two companies confirmed the test results of Test 2 and Test 3 which were provided by the proponent. The results were not fully covered by the test results of proponent, but the available results were matched with the proponent results. Additionally, one company tested the Test6 case voluntarily.

The analysis of complexity and coding performance was done in Tests 1, 2, 3 and 6. Test results are available in an Excel file attached to the BoG report. The following graph shows the trade-off between encoding time and coding performance in RA.

Complexity order:

- Test 1 > Test 3 > Test 2 > Test 6

Coding performance order:

- Test 1 > Test 3 > Test 2 > Test 6

Trade-off between performance and encoding time

- Test 3 was considered to show the best trade-off if luma & chroma results are calculated

Trafficflow in Test 2 shows 1.5% coding loss in RA.

The QTBT proponent said that the updated result of Test 1 in RA (Class A1) showed 256% of encoding time which is less than previous results.

Opinions were surveyed and discussed.

Opinion 1: Using Test 1 as the default setting:

- One expert thinks that coding performance of Test 1 is attractive.

- It was suggested to use Test 1 as default setting for JEM 3.0, because there are several months before the next meeting, and we can have some fast algorithm developed by the next meeting.

Opinion 2: Using Test 2 as default setting:

- The increase of encoding time seems to be fine.

- Using Test 2 or 3 is more desirable due to the encoding time.

Opinion 3: Using Test 3 as default setting:

- Some experts said that compromised solution would be Test 3.

- Some experts suggested to use Test 3 with reduced number of frames for Class A1 and A2 (RA and LD) for testing for the next meeting.

- It is suggested to use a modified intra period for test sequences.

The group consensus was to use the Test 3 configuration setting as the default setting for CTC of JEM 3.0.

Mandate4

It was confirmed that the current QTBT version in the software branch for the EE does not support adaptive QP in JEM 2.0.

Recommendation by BoG group

- To use the Test 3 configuration as default settings of QTBT for JEM 3.0

- To establish an ad hoc to get the best configuration setting and develop a fast method for QTBT

- To fix localized delta QP signalling in the current QTBT version in the software branch for the EE for JEM 3.0

From the follow-up discussion in JVET:

Some concern was expressed about encoder complexity, but no objection was made against the recommendation.

Decision: Establish “Test 3” configuration as CTC for JEM3.

The software coordinators are tasked to additionally report results on the “Test 1” case for JEM3.

Sharp (T. Ikai) volunteered to report results on “Test 2” additionally for JEM3.

These additional test points will be valuable to assess the achievements of the AHG.

Delta QP will be implemented by Mediatek as a separate branch 2 weeks after the release of JEM3. The method of signalling at the PPS and block level shall be identical with the current method in HEVC (where a quantization group indicates the granularity of QP sharing by blocks).

JVET-C0104 Report of BoG on test material [T. Suzuki]

The BoG on test material selection was generated with following mandates:

- Review the class A1/A2 selection made by the last meeting, and propose possible changes

- Establish work plan towards the next meeting for investigating the 1080p sequences (with the goal to establish new class or replace class B by the next meeting

- Summarize the material offered for VR, identify whether it covers all common methods of projection/rendering/stitching, and discuss possible methods of quality assessment

For the VR material, it should also be discussed with parent bodies how to coordinate the different activities in this area. The work of JVET should not be dominated by VR.

The new material for screen content should be brought to the attention of JCT-VC, and could be used in the SCC verification test. Currently, the development of higher compression technology specifically for screen content is not in the focus of JVET.

This document reports the results of the BoG discussion.

Summary of activities

The BoG met at Room C2 (ITU-T building) between 2:00 PM – 6:00 PM, May.29 2016. The BoG reviewed the following contributions. The discussion for each contribution is summarized in the Annex of the BoG report.

- JVET-C0021 GoPro test sequences for Virtual Reality video coding [A.Abbas (GoPro)]

- JVET-C0028 Suggested 1080P Test Sequences Downsampled from 4K Sequences H. Zhang, X. Ma, H. Yang (Huawei)]

- JVET-C0029 Surveillance sequences for video coding development [H. Zhang, X. Ma, H. Yang (Huawei), W.Qiu (Hisilicon)]

- JVET-C0033 On comparison criteria for Virtual Reality video coding schemes [E. Thomas (TNO)]

- JVET-C0041 Proposed test sequences for 1080p class [A. Norkin (Netflix)]

- JVET-C0044 Response to B1002 Call for test materials: Five test sequences for screen content video coding [J. Guo, L. Zhao, T. Lin (Tongji Uni.),

- H. Yu (Futurewei)]

- JVET-C0048 Lens distorted test sequence by an action camera for future video coding [K. Kawamura, S. Naito (KDDI Corp.)]

- JVET-C0050 Test sequence formats for virtual reality video coding [K. Choi, V. Zakharchenko, M. Choi, E. Alshina (Samsung)]

- JVET-C0064 Nokia test sequences for virtual reality video coding [J. Ridge, M. M. Hannuksela (Nokia)]

- JVET-C0067 Ultra High Resolution (UHR) 360 Video [C. J. Murray (Panoaction)]

VR test sequences:

- JVET-C0021 (GoPro): 9 VR test sequences are proposed. For each sequence, both equirectangular and cube-4x3 formats are provided. All contents proposed at this meeting are compressed. Those are captured as a compressed bitstream (AVC). But the proponent can provide an uncompressed version. Until uncompressed files are available, the bitstreams at the above sites can be used to understand the nature of VR test sequences.

- JVET-C0064 (Nokia): This contribution offers the stereoscopic equirectangular panorama sequences. The camera-captured sequences are several tens of seconds long, out of which Nokia is willing to provide 10-second excerpts (with start points as agreed by JVET) for standardization. There was a question on the possibility to provide whole portion of video, and the proponent needs to confirm the permission to provide complete sequences. The proponent can provide original fish eye content (8 cameras). The resolution of each camera video is 2Kx2K (8 bit)

  - Proponent will select 10 sec for each sequences and will upload to the ftp site. Availability of complete sequences for original fish eye camera need to be confirmed.

- JVET-C0067 (Panoaction): No presentation. 8kx4k and 14x7k equirectangular sequences are proposed.

- JVET-C0033 (TNO): This is not a proposal of test material, but proposes to consider spatial random access for the evaluation of virtual reality test sequences. There are several representation of omnidirectional video, e.g. cube, tile, etc). Full HD per view point.

- JVET-C0050 (Samsung): This is not a proposal of test material, but proposes representations of virtual reality test sequences (equirectangular, cube and icosahedron format). Those were converted from the equirectangular test sequences proposed in JVET-C0021. From test sequence perspective, proposed format can be converted from equirectangular sequences, Proponent can provide conversion tools, for further testing.

BoG recommendations:

- To keep all proposed test sequences as candidates of test sets

- To continue to study by AHG. AHG will investigate the following issues

  - Study format of VR sequences to decide test conditions

    - Study of format also includes the study of random access (both temporal and spatial), low delay, etc.

  - Evaluation method

  - Test conditions

- VR test sequences

  - Equirectangular sequences maybe sufficient at this moment. Tool can convert into other formats

  - Some companies may have camera original (video before stitching) and those are also useful for further study

  - FTP site has sufficient to space to upload all sequences.

    - All portions of sequences can be uploaded, and then discuss appropriate portion by the next meeting.

    - Nokia needs to confirm whether it is allowed to upload full length

    - Nokia will pre-select an appropriate portion (uncompressed), and then upload

    - GoPro test sequences: only compressed sequences are available now. At the next meeting, uncompressed sequences will be available.

- To discuss how we should proceed on VR

  - Scope of JVET on VR issues (was clarified in joint meeting of parent bodies)

From follow-up discussion in JVET:

- It was verbally expressed that the sequences from JVET-C0064 will be provided with acceptable licensing conditions (at least similar to previous cases). This will apply to 10s excerpts from the sequences.

- Among the three contributions on VR material, C0064 provides stereo, the other two (GoPro, Panoaction) are monoscopic.

- Evaluation method for VR sequences should be further studied in AHG. For example, PSNR could be measured after backprojection from equirectangular to 2D.

4K test sequences

BoG recommendations:

- To study 4K sequences proposed in C0029 and some test sequences from the test sets studied in the last meeting.

- The following sequences are pre-selected during BoG and are to be studied further by the next meeting as a replacement of the current class A sequences. (5 sequences)

  - ParkRunning1

  - BuildingHall

  - CrossRoad1

  - Runners (from previous test sets)

  - Crosswalk (from previous test sets)

From the follow-up discussion in JVET

- No clear opinion exists whether some of the test material in class A1/A2 is inappropriate

- It is planned to generate test cases for the A1/A2 classes and the 5 sequences listed above, HM/JEM at approximately same bit rates (Possible additional sequences: Market 2, Time lapse)

  - An initial idea for rates in class A 60 Hz could be 2.5, 4, 7, 12, 18 for RA

  - If impossible to reach for certain sequences, extend the range appropriately

  - For 30 Hz scale down by 1.5, etc.

- Perform subjective viewing at the next meeting, for assessing the subjective quality in terms of appropriateness for a formal subjective viewing and selecting appropriate rate points

- This material could also be used in the context of AHG5 for investigating objective metrics, likely after the next meeting.

1080p test sequences

BoG recommendations:

- To study 1080p sequences proposed in C0028, C0041 and C0048.

  - Encoding time is less than class A and all proposed sequences can be tested.

- To study further on the design of test classes.

  - New test class can be considered. The followings were discussed

- Not necessary to restrict current class B number of sequences

- New class B with 10 bit

- New class for specific applications, e.g. surveillance

- New class for special features of content (wind & nature, toddler fountain, complex motion, water, complex texture (grass), etc)

- New class for smaller picture size (smaller than 1080p)

- New class could be optional (e.g. for subjective test)

- The number of test sequences is

  - 12 sequences from C0028 (up to 600 frames. Huawei will provide which part should be used)

  - 9 sequences from C0041

  - 1 sequences from C0048

  - Totally 22 sequences for 1080p

HDR test sequences

BoG recommendations:

- To study further if HDR test sequences can be added.

  - Technicolor and Netflix can provide HDR sequences for JVET.

- To study evaluation methods before including HDR sequences.

- AHG should study evaluation methods and then discuss at the next meeting if HDR sequences can be added.

A workplan document was issued as an output. Its table with volunteers is to be filled in by contacting T. Suzuki.

## List of actions taken affecting the JEM3

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the JEM3 description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

  - Encoder only or software changes

    - JVET-C0037

      - IRAP-level parallel encoding with precise bitstream concatenation

    - JVET-C0046

      - Bug fixes for enabling of RExt tools in JEM

  - Syntax/semantics/decoding process changes

    - JVET-C0024

      - QTBT replaces quadtree in main branch of JEM3

    - JVET-C0025

      - Simplification/unification of MC filters for affine prediction

    - JVET-C0027

      - Simplification/improvement of BIO

    - JVET-C0035

      - ATMVP simplification

    - JVET-C0038

      - Modifications of ALF: Diagonal classification, geometric transformations of filters, prediction of coefficients from fixed set, alignment of luma and chroma filter shapes, removal of context coded bins for filter coefficient signalling

    - JVET-C0042/JVET-C0053

      - unified binarization of NSST index

    - JVET-C0046

      - Enabling TS with 64x64 transform blocks

    - JVET-C0055

      - Simplified derivation of MPM in intra prediction

    - NSST & TS

      - Disable NSST and do not code NSST index if all components in a block use TS; otherwise, if NSST is on, it shall not be used for a block of a component that uses TS.

Project planning

## JEM description drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and JEM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- JEM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be Wednesday 5 October 2016.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for Experiments

Group coordinated experiments have been planned. These may generally fall into one category:

- "Exploration experiments" (EEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the main branch of the JEM by the next meeting.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. (E. Alshina will edit the document based on input from the proponents, review is performed in the plenary)

- Software for tools investigated in EE is provided in a separate branch of the software repository

- During the experiment, further improvements can be made

- By the next meeting it is expected that independent parties (at least one at a minimum) will report a detailed analysis about the tool, confirm that the implementation is correct, and give reasons to include (or not include) the tool in JEM

- As part of the experiment description, it should be captured whether performance relative to the JEM as well as the HM (with all other tools of the JEM disabled) should be reported by the next meeting.

It is possible to define sub-experiments within particular EEs, for example designated as EEX.a, EEX.b, etc., where X is the basic EE number.

As a general rule, it was agreed that each EE should be run under the same testing conditions using one software codebase, which should be based on the JEM software codebase. An experiment is not to be established as an EE unless there is access given to the participants in (any part of) the EE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-B1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to EE work should identify individuals in addition to company names.

EE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the EE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish an EE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to EE activities were established as follows:

- Only qualified JVET members can participate in an EE.

- Participation in an EE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the EE should be announced and made available to all EE participants in a timely manner.

Further discussed Tuesday AM, chaired by JRO and J. Boyce.

A separate branch under the experimental section will be created for each new tool include in the EE. The proponent of that tool is the gatekeeper for that separate software branch. (This differs from the main branch of the JEM, which is maintained by the software coordinators.)

New branches may be created which combine two or more tools included in the EE document or the JEM. Requests for new branches should be made to the software coordinators.

Don’t need to formally name cross-checkers in the EE document. To promote the tool to the JEM at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

Timeline:

T1 = JEM3.0 SW release + 4 weeks: Integration of all tools into separate EE branch of JEM is completed and announced to JVET reflector.

Initial study by cross-checkers can begin.

Proponents may continue to modify the software in this branch until T2

3rd parties encouraged to study and make contributions to the next meeting with proposed changes

T2: JVET-D meeting start – 3 weeks: Any changes to the exploration branch software must be frozen, so the cross-checkers can know exactly what they are cross-checking. An SVN tag should be created at this time and announced on the JVET reflector.

This procedure was again confirmed during the closing plenary of the third JVET meeting. It was further confirmed that the Common Test Conditions of JVET-B1010 are still valid, however the CTC encoder setting will be reflected in the config file that is attached to the JEM3.0 package.

## Software development

Software coordinators will work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- JEM3.0 including all adoptions from section 9.4 will be released within 3 weeks (2016-06-24)

- JEM3.1 including adaptive QP will be released 2 weeks later.

- The results about coding performance of JEM3.0 will be reported by 2016-07-07

# Output documents and AHGs

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JVET-C1000 Meeting Report of the 3rd JVET Meeting [G. J. Sullivan, J.-R. Ohm] [2016-10-14] (near the next meeting)

Intermediate versions of the meeting notes (d0 … d6) were made available on a daily basis during the meeting.

JVET-C1001 Algorithm description of Joint Exploration Test Model 3 (JEM3) [J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce] [2016-07-07]

See the list of new adoptions in section 9.4. During the closing plenary, no complaints were made about the accuracy of that list.

JVET-B1002 Call for test materials for future video coding standardization [A. Norkin, H. Yang, J.-R. Ohm, G. J. Sullivan, T. Suzuki]

This remains valid (from the 2nd meeting)

JVET-C1002 Work plan for assessment of test material [T. Suzuki] [2016-06-17]

This was reviewed and approved in draft form in the closing plenary. An editing period is granted to further consider the appropriate definition of constant bit rate settings.

It was emphasized that the investigation of HM vs. JEM coding at the same bit rates is very important for assessing the current status.

JVET-B1010 JVET common test conditions and software reference configurations [K. Suehring, X. Li]

This remains valid (from the 2nd meeting).

Note: Encoder settings reflected in the config file are related to the CTC in JEM3.

It was commented that we should create a directory on the ftp site for bitstreams and the results of anchors.

JVET-C1011 Description of Exploration Experiments on coding tools [E. Alshina, J. Boyce, Y. J. Chang, S.-H. Kim, V. Seregin, X. Xiu] [2016-06-23]

See list of EEs in section 9.1

The version reviewed on Tuesday was again approved in the closing plenary on Wednesday. One minor change was made, adding Qualcomm as an additional contributing party in EE9 test 4.

Some affine MC friendly content should be made available, as allowed by content rights, so that it can be used in EE4 tests. X. Ma (Huawei) was requested to contact Mathias Wien to enable uploading of such content to the JVET content repository. Once content has been made available, this should be announced on the JVET reflector.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Tool evaluation (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate the exploration experiments. - Investigate interaction of tools in JEM and exploration experiment branches. - Discuss and evaluate methodologies and criteria to assess the benefit of tool, and how to ease the assessment of single tools in terms of encoder runtime. - Study and summarize new technology proposals. - Prepare visual quality comparison for HEVC and JEM at comparable bit rates. | E. Alshina, M. Karczewicz (co-chairs) | N |
| JEM algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-C1001 Algorithm Description of Joint Exploration Test Model 3 - Gather and address comments for refinement of the document - Coordinate with the JEM software development AHG to address issues relating to mismatches between software and text. | J. Chen (chair) E. Alshina, J. Boyce (vice chairs) | N |
| JEM software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of the JEM3.0 and 3.1 software packages and their distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver JEM3.0 software version and the reference configuration encodings according to JVET-B1010 common conditions. - Coordinate with AHG on JEM model editing and errata reporting to identify any mismatches between software and text, and make further updates to the software as appropriate. - Investigate parallelization for speedup of simulations. - Investigate the implementation of SCC coding tools in JEM. | X. Li, K. Suehring (co-chairs) | N |
| Test material (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of future video coding standards. - Identify and recommend appropriate test materials and corresponding test conditions for use in the development of future video coding standards. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics of video test materials according to the work plan JVET-C1002. | T. Suzuki (chair), J. Chen, J. Boyce, A. Norkin (vice chairs) | N |
| Fast encoding, encoding complexity investigation, and configuration settings (AHG5) (jvet@lists.rwth-aachen.de) - Study encoder configuration settings of JEM3 software and suggest alternative configuration files for different operating points in terms of encoder complexity versus compression benefit. - Investigate and develop fast methods to reduce JEM3 encoding complexity, in particular fast encoder decision methods for QTBT partitioning. | K. Choi (chair), X. Li, H. Huang, T. Ikai, P. Philippe (vice chairs) | N |
| Simplification of decoder-side motion derivation tools (AHG6) (jvet@lists.rwth-aachen.de) - Investigate the potential of unification and simplification of interpolation filters for decoder-side MV derivation techniques in JEM3 (BIO and FRUC). - Investigate the adaptive threshold of MV refinement in BIO. - Investigate other potential simplifications of the decoder-side motion derivation tools. | X. Li, E. Alshina (co-chairs) | N |
| JEM coding of extended colour volume material (AHG7) (jvet@lists.rwth-aachen.de) - Identify HDR content that would be available for JVET testing, and coordinate distribution of such content. - Identify and recommend conversion practices to create versions of the JVET test sequences in extended colour volume containers, such as BT.2020 and SMPTE ST 2084. - Identify and recommend test conditions for the JEM coding of extended colour volumes in coordination with EE9. - Study JEM configuration and anchor creation for extended colour volume content (including local control of QP studied in EE9) as appropriate. - Study objective metrics for quality assessment of extended colour volume material. - Study additional aspects of coding extended colour volume content. | A. Segall (chair), S. Lasserre, D. Rusanovskyy (vice chairs) | N |
| 360 video test conditions (AHG8) (jvet@lists.rwth-aachen.de) - Study the effect on compression when different warping methods are applied to the input 360 video before compression. - Run HM and JEM encodings on available 360 video content in multiple projection formats - Discuss definition of common test conditions, test sequence formats, and evaluation criteria - Study software tools for conversion and rendering of 360 video content, and make it available to JVET as appropriate. | J. Boyce (chair), E. Alshina, G. v. d. Auwera, P. Philippe, Y. Ye (vice chairs) | N |
| Objective quality metrics (AHG9) (jvet@lists.rwth-aachen.de) - Study metrics which are useful to evaluate the quality of video compression algorithms with closer match to human perception. - Collect and make software implementing the computation of such metrics available. | P. Nasiopoulos, M. Pourazad (co-chairs) | N |

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–6.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Saturday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 6.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Sat. 15 – Fri. 21 Oct. 2016, 4th meeting under WG 11 auspices in Chengdu, CN.

- Thu. 12 – Wed. 18 Jan 2017, 5th meeting under ITU-T auspices in Geneva, CH.

- Sat. 1 – Fri. 7 Apr. 2017, 6th meeting under WG 11 auspices in Hobart, AU.

The agreed document deadline for the 4th JVET meeting is Wednesday 5 October 2016. Plans for scheduling of agenda items within that meeting remain TBA.

ITU was thanked for the excellent hosting of the 3rd meeting of the JVET. EBU was thanked for providing viewing equipment.

The 3rd JVET meeting was closed at approximately 1225 hours on Wednesday 1 June 2016.

# Annex A to JVET report:
List of documents

| JVET number | MPEG number | Created | First upload | Last upload | Title | Authors |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-C0001 | m38786 | 2016-05-26 11:19:29 | 2016-05-26 11:20:33 | 2016-05-26 11:20:33 | JVET AHG report: Tool evaluation (AHG1) | M. Karczewicz, E. Alshina |
| JVET-C0002 | m38774 | 2016-05-25 22:11:49 | 2016-05-26 10:23:34 | 2016-05-26 10:23:34 | JVET AHG report: JEM algorithm description editing (AHG2) | J. Chen, E. Alshina, J. Boyce |
| JVET-C0003 | m38761 | 2016-05-25 16:40:49 | 2016-05-26 09:23:39 | 2016-05-26 16:25:47 | JVET AHG report: JEM software development (AHG3) | X. Li, K.Suehring |
| JVET-C0004 | m38744 | 2016-05-25 03:07:29 | 2016-05-26 14:39:09 | 2016-05-26 14:39:09 | JVET AHG report: Test material (AHG4) | T. Suzuki, J. Chen, A. Norkin, J. Boyce |
| JVET-C0005 | m38814 | 2016-05-28 09:19:20 | 2016-05-28 09:29:24 | 2016-05-28 09:29:24 | JVET AHG report: Visual quality metrics (AHG5) | P. Nasiopoulos, M. T. Pourazad |
| JVET-C0010 | m38735 | 2016-05-25 00:28:50 | 2016-05-25 00:32:47 | 2016-05-26 07:29:17 | Exploration Experiments on Coding Tools Report | E. Alshina, J. Boyce, Y.-W. Huang, S.-H. Kim, L. Zhang |
| JVET-C0021 | m38243 | 2016-05-03 02:14:25 | 2016-05-03 02:42:13 | 2016-05-26 20:15:04 | GoPro test sequences for Virtual Reality video coding | A. Abbas (GoPro) |
| JVET-C0022 | m38377 | 2016-05-13 14:13:05 | 2016-05-16 23:00:13 | 2016-05-27 18:49:19 | Proposed improvements to the Adaptive multiple Core transform | P. Philippe (Orange), V. Lorcy (bcom) |
| JVET-C0023 | m38381 | 2016-05-15 13:59:00 | 2016-05-15 14:21:07 | 2016-05-15 14:21:07 | Predictors Elimination Technique for HEVC | M. Korman, O. Prosekov (Synopsys) |
| JVET-C0024 | m38383 | 2016-05-16 11:04:08 | 2016-05-16 11:12:59 | 2016-05-25 23:05:41 | EE2.1: Quadtree plus binary tree structure integration with JEM tools | H. Huang, K. Zhang, Y.-W. Huang, S. Lei (MediaTek) |
| JVET-C0025 | m38384 | 2016-05-16 11:59:52 | 2016-05-16 12:04:25 | 2016-05-27 22:35:25 | Simplification of motion compensation filter for affine inter prediction | J. Nam, H. Jang, J. Lee, B. Lee, J. Lim (LGE) |
| JVET-C0026 | m38385 | 2016-05-16 12:10:28 | 2016-05-16 14:42:27 | 2016-05-24 14:48:44 | Tiles coding improvement for Inter pictures by improved merge list at tile boundaries | S. Biplab Raut (Samsung) |
| JVET-C0027 | m38386 | 2016-05-16 13:13:56 | 2016-05-17 07:09:59 | 2016-05-28 09:40:14 | Simplification and improvements for BIO design in JEM2.0 | E. Alshina, A. Alshin (Samsung) |
| JVET-C0028 | m38387 | 2016-05-16 14:00:35 | 2016-05-16 15:53:45 | 2016-05-25 16:59:25 | Suggested 1080P Test Sequences Downsampled from 4K Sequences | H. Zhang, X. Ma, H. Yang (Huawei) |
| JVET-C0029 | m38388 | 2016-05-16 14:16:53 | 2016-05-16 15:54:57 | 2016-05-26 15:03:03 | Surveillance sequences for video coding development | H.Zhang, X. Ma, H. Yang (Huawei), W.Qiu (Hisilicon) |
| JVET-C0030 | m38389 | 2016-05-16 14:18:57 | 2016-05-16 16:01:07 | 2016-05-29 11:05:04 | Perceptual Quality Assessment Metric MS-SSIM | H. Zhang, X. Ma, Y. Zhao, H. Yang (Huawei) |
| JVET-C0031 | m38390 | 2016-05-16 14:30:40 | 2016-05-16 14:45:50 | 2016-05-27 19:31:37 | BIO improvement to reduce the encoder and decoder complexities | J. Lee, N. Park, J. Nam, J. Lim (LGE) |
| JVET-C0032 | m38391 | 2016-05-16 15:11:40 | 2016-05-16 15:31:05 | 2016-05-29 12:25:33 | Experiment on polyphase subsampled sequence coding | E. Thomas (TNO) |
| JVET-C0033 | m38392 | 2016-05-16 16:24:22 | 2016-05-16 16:28:40 | 2016-05-29 12:49:06 | On comparison criteria for Virtual Reality video coding schemes | E. Thomas (TNO) |
| JVET-C0034 | m38393 | 2016-05-16 16:30:25 | 2016-05-26 01:28:27 | 2016-06-01 11:53:19 | Open-source inspired worflow for JEM maintenance (informative) | E. Thomas (TNO) |
| JVET-C0035 | m38395 | 2016-05-16 18:00:41 | 2016-05-16 22:42:18 | 2016-05-26 14:23:36 | EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning | S. Lee, W.-J. Chien, L. Zhang, J. Chen, M. Karczewicz (Qualcomm) |
| JVET-C0036 | m38396 | 2016-05-16 18:43:00 | 2016-05-20 19:28:26 | 2016-05-20 19:28:26 | Cross-check of JVET-C0038 (EE2.5 Improvements on adaptive loop filter) | R. Chernyak (Huawei) |
| JVET-C0037 | m38397 | 2016-05-16 18:57:19 | 2016-05-16 21:11:24 | 2016-06-01 00:15:51 | Sequential/Parallel bistreams unification for JVET CTC | R. Mullakhmetov, I. Sharonov, M. Sychev (Huawei) |
| JVET-C0038 | m38398 | 2016-05-16 20:23:59 | 2016-05-17 04:41:40 | 2016-05-26 13:44:16 | EE2.5: Improvements on adaptive loop filter | M. Karczewicz, L. Zhang, W.-J. Chien, X. Li (Qualcomm) |
| JVET-C0039 | m38403 | 2016-05-16 22:24:47 | 2016-05-16 23:21:35 | 2016-05-25 05:21:11 | Decoupled Luma/Chroma Transform Trees for Intra | F. Urban, T. Poirier, F. Le Léannec (Technicolor) |
| JVET-C0040 | m38405 | 2016-05-16 22:55:28 | 2016-05-16 23:27:08 | 2016-05-27 16:38:36 | Adaptive Clipping in JEM2.0 | F. Galpin, P. Bordes, F. Le Léannec (Technicolor) |
| JVET-C0041 | m38409 | 2016-05-16 23:53:40 | 2016-05-17 05:48:54 | 2016-05-17 05:48:54 | Proposed test sequences for 1080p class | A. Norkin (Netflix) |
| JVET-C0042 | m38418 | 2016-05-17 02:03:24 | 2016-05-17 03:52:57 | 2016-05-24 02:25:12 | EE2.3: NSST-PDPC Harmonization | S.-H. Kim, A. Segall (Sharp) |
| JVET-C0043 | m38424 | 2016-05-17 03:19:44 | 2016-05-17 03:43:16 | 2016-05-25 21:32:26 | Arbitrary reference tier for intra directional modes | Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI) |
| JVET-C0044 | m38427 | 2016-05-17 03:44:50 | 2016-05-17 04:17:42 | 2016-05-28 04:47:21 | Response to B1002 Call for test materials: Five test sequences for screen content video coding | J. Guo, L. Zhao, T. Lin (Tongji Univ.), H. Yu (Futurewei) |
| JVET-C0045 | m38429 | 2016-05-17 04:09:26 | 2016-05-17 04:46:51 | 2016-05-25 07:27:51 | EE2.7-related: On secondary transform when primary transform is skipped | T. Tsukuba, O. Nakagami, T. Suzuki (Sony) |
| JVET-C0046 | m38430 | 2016-05-17 04:11:03 | 2016-05-23 06:09:27 | 2016-05-27 11:09:17 | RExt coding tools support on JEM | T. Tsukuba, O. Nakagami, T. Suzuki (Sony) |
| JVET-C0047 | m38431 | 2016-05-17 04:14:44 | 2016-05-17 06:42:08 | 2016-05-28 10:26:57 | Generalized bi-prediction for inter coding | C.-C. Chen, X. Xiu, Y. He, Y. Ye (InterDigital) |
| JVET-C0048 | m38432 | 2016-05-17 04:23:51 | 2016-05-17 05:48:54 | 2016-05-17 05:48:54 | Lens distorted test sequence by an action camera for future video coding | K. Kawamura, S. Naito (KDDI Corp.) |
| JVET-C0049 | m38433 | 2016-05-17 04:27:50 | 2016-05-24 03:52:24 | 2016-05-31 10:42:15 | Extended deblocking-filter process for large block boundary | K. Kawamura, Q. Yao, S. Naito (KDDI Corp.) |
| JVET-C0050 | m38434 | 2016-05-17 05:01:35 | 2016-05-24 10:18:30 | 2016-05-29 14:58:57 | Test sequence formats for virtual reality video coding | K. Choi, V. Zakharchenko, M. Choi, E. Alshina (Samsung) |
| JVET-C0051 | m38438 | 2016-05-17 06:04:21 | 2016-05-18 13:47:54 | 2016-05-18 13:47:54 | The cross-check for proposed in JVET-C0022 improvements to the Adaptive multiple Core transform | E. Alshina (Samsung) |
| JVET-C0052 | m38439 | 2016-05-17 06:06:03 | 2016-05-19 11:43:01 | 2016-05-23 06:42:50 | The cross-check for BIO improvement to reduce the encoder and decoder complexities in JVET-C0031 | E. Alshina (Samsung) |
| JVET-C0053 | m38440 | 2016-05-17 06:12:37 | 2016-05-17 09:14:16 | 2016-05-26 09:51:55 | EE2.7: TU-level non-separable secondary transform | X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi (Qualcomm) |
| JVET-C0054 | m38441 | 2016-05-17 06:12:48 | 2016-05-17 09:17:24 | 2016-05-26 11:14:01 | Grouped signalling for transform in QTBT | X. Zhao, V. Seregin, A. Said, M. Karczewicz (Qualcomm) |
| JVET-C0055 | m38442 | 2016-05-17 06:13:02 | 2016-05-17 09:21:27 | 2016-05-28 10:31:07 | Neighbor based intra most probable modes list derivation | V. Seregin, X. Zhao, A. Said, M. Karczewicz (Qualcomm) |
| JVET-C0056 | m38443 | 2016-05-17 06:41:05 | 2016-05-24 14:55:33 | 2016-05-24 14:55:33 | Cross-check of JVET-C0024 (QTBT) | M. W. Park, B. Jin, E. Alshina, C. Kim (Samsung) |
| JVET-C0057 | m38444 | 2016-05-17 06:43:29 | 2016-05-24 14:13:29 | 2016-05-26 15:12:59 | Cross-check of JVET-C0038 (GALF) | K. Choi, M. Park, E. Alshina, C. Kim (Samsung) |
| JVET-C0058 | m38445 | 2016-05-17 06:46:26 | 2016-05-24 14:14:56 | 2016-05-26 15:13:26 | Cross-check of JVET-C0053 (TU-NSST) | K. Choi, M. Park, E. Alshina, C. Kim (Samsung) |
| JVET-C0059 | m38447 | 2016-05-17 06:48:45 | 2016-05-24 14:15:32 | 2016-05-26 15:13:48 | Cross-check of JVET-C0042 (NSST-PDPC) | K. Choi, M. Park, E. Alshina, C. Kim (Samsung) |
| JVET-C0060 | m38448 | 2016-05-17 06:50:00 | 2016-05-24 12:15:28 | 2016-05-24 12:15:28 | Cross-check of JVET-C0035 (Modification of merge candidate derivation) | A. Tamse, E. Alshina, C. Kim (Samsung) |
| JVET-C0061 | m38449 | 2016-05-17 07:00:53 | 2016-05-17 08:59:22 | 2016-05-26 08:14:06 | Decoder-side intra mode derivation | X. Xiu, Y. He, Y. Ye (InterDigital) |
| JVET-C0062 | m38450 | 2016-05-17 07:41:56 | 2016-05-17 08:23:11 | 2016-05-28 10:48:32 | Improved affine motion prediction | F. Zou, J. Chen, M. Karczewicz, X. Li, H.-C. Chuang, W.-J. Chien (Qualcomm) |
| JVET-C0063 | m38452 | 2016-05-17 07:47:54 | 2016-05-17 10:14:00 | 2016-05-26 10:53:21 | EE2.7 related: Improved non-separable secondary transform | X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen (Qualcomm) |
| JVET-C0064 | m38453 | 2016-05-17 08:12:40 | 2016-05-17 18:41:22 | 2016-05-27 14:36:09 | Nokia test sequences for virtual reality video coding | J. Ridge, M. M. Hannuksela, E. B. Aksu, J. Lainema, A. Aminlou (Nokia) |
| JVET-C0065 | m38454 | 2016-05-17 08:24:24 | 2016-05-26 10:20:56 | 2016-05-26 10:20:56 | Cross-check of JVET-C0027 | J. Lee, S. Yoo, J. Nam, J. Lim (LGE) |
| JVET-C0066 | m38457 | 2016-05-17 09:28:39 | 2016-05-17 10:53:18 | 2016-05-28 11:26:31 | On Coefficient Scaling | D. B. Sansli, D. Rusanovskyy, J. Sole, A.K. Ramasubramonian, M. Karczewicz (Qualcomm) |
| JVET-C0067 | m38458 | 2016-05-17 09:55:32 | 2016-05-18 18:21:07 | 2016-05-18 18:21:07 | Ultra High Resolution (UHR) 360 Video | C. J. Murray (Panoaction) |
| JVET-C0068 | m38460 | 2016-05-17 10:38:47 | 2016-05-17 11:59:04 | 2016-05-28 15:50:03 | Motion vector coding optimizations | J. Samuelsson, P. Wennersten, R. Yu, U. Hakeem (Ericsson) |
| JVET-C0069 | m38461 | 2016-05-17 15:26:18 | 2016-05-18 02:44:32 | 2016-05-28 12:45:39 | Direction-dependent scan order with JEM tools | S. Iwamura, A. Ichigaya (NHK) |
| JVET-C0070 | m38465 | 2016-05-17 17:13:06 | 2016-05-17 17:17:01 | 2016-05-26 23:57:39 | NEXT test model software | A. Hallapuro, M. Hannuksela, J. Ridge, J. Lainema, A. Aminlou (Nokia) |
| JVET-C0071 | m38466 | 2016-05-17 17:48:48 | 2016-05-17 17:54:54 | 2016-05-26 16:22:02 | Multiple line-based intra prediction | J. Li (Peking Univ.), B. Li, J. Xu (Microsoft), R. Xiong (Peking Univ.), G.-J. Sullivan (Microsoft) |
| JVET-C0072 | m38467 | 2016-05-17 18:09:36 | 2016-05-25 16:38:12 | 2016-05-25 16:38:12 | Cross-check of JVET-C0027 (BIO simplification and improvements) | X. Li (Qualcomm) |
| JVET-C0073 | m38472 | 2016-05-18 03:18:42 | 2016-05-23 11:01:35 | 2016-05-23 11:01:35 | Cross-check of JVET-C0035 (EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning) | Y. Yasugi, T. Ikai (Sharp) |
| JVET-C0074 | m38473 | 2016-05-18 03:25:12 | 2016-05-23 11:32:18 | 2016-05-24 03:22:55 | Cross-check of JVET-C0038(EE2.5: Improvements on adaptive loop filter) | Y. Yamamoto, T. Ikai (Sharp) |
| JVET-C0075 | m38486 | 2016-05-18 20:03:25 | 2016-05-27 09:12:19 | 2016-05-27 09:12:19 | Cross-check of JVET-C0063 EE2.7 related: Improved non-separable secondary transform | S.-H. Kim (Sharp) |
| JVET-C0076 | m38516 | 2016-05-19 16:24:45 | 2016-05-25 15:21:43 | 2016-05-25 15:21:43 | Cross-check of JVET-C0053 (TU-NSST) | V. Lorcy (bcom), P. Philippe (Orange), T. Biatek (TDF) |
| JVET-C0077 | m38524 | 2016-05-20 04:11:07 | 2016-05-20 04:14:37 | 2016-05-20 04:14:37 | EE2.2: Non Square TU Partitioning | K. Rapaka, J. Chen, L. Zhang, W. -J. Chien, M. Karczewicz (Qualcomm) |
| JVET-C0078 | m38528 | 2016-05-20 11:17:05 | 2016-05-25 18:46:02 | 2016-05-25 18:46:02 | Cross-check of C0032 (polyphase subsampled sequence coding) | P. Philippe (Orange), V. Lorcy, T. Biatek (Bcom) |
| JVET-C0079 | m38529 | 2016-05-20 11:25:44 | 2016-05-25 02:18:33 | 2016-05-30 17:12:52 | Cross-check of JVET-C0039 Decoupled Luma/Chroma Transform Trees for Intra | O. Nakagami (Sony) |
| JVET-C0080 | m38530 | 2016-05-20 15:31:51 | 2016-05-25 18:20:00 | 2016-05-25 18:20:00 | Cross-check of JVET-C0027 (Simplification and improvements for BIO design in JEM2.0) | T. Biatek (TDF), V. Lorcy (bcom), P. Philippe, P. Boissonade (Orange) |
| JVET-C0081 | m38562 | 2016-05-23 01:58:22 | 2016-05-25 08:37:17 | 2016-05-26 14:32:37 | Crosscheck of neighbor based intra most probable modes list derivation in JVET-C0055 | H. Huang (MediaTek) |
| JVET-C0082 | m38569 | 2016-05-23 04:27:30 | 2016-05-24 10:58:39 | 2016-05-24 10:58:39 | Cross-check of JVET-C0071 (Multiple line-based intra prediction) | T. Ikai (Sharp) |
| JVET-C0083 | m38575 | 2016-05-23 08:48:08 | 2016-05-27 08:56:09 | 2016-05-30 17:13:34 | Cross-check of JVET-C0062 Improved affine motion prediction | O. Nakagami (Sony) |
| JVET-C0084 | m38582 | 2016-05-23 10:40:46 | 2016-05-26 09:55:01 | 2016-05-26 09:55:01 | Cross-check of C0040 (Adaptive Clipping) | P. Philippe (Orange), V. Lorcy, T. Biatek (Bcom) |
| JVET-C0085 | m38597 | 2016-05-23 14:00:35 | 2016-05-25 17:35:09 | 2016-05-25 17:35:09 | Cross-check of JVET-C0035: Modification of Merge candidate derivation | H. Chen, H. Yang (Huawei) |
| JVET-C0086 | m38661 | 2016-05-23 18:29:41 | 2016-05-27 09:12:51 | 2016-05-27 09:12:51 | Cross-check of EE2.7: TU-level non-separable secondary transform | S.-H. Kim (Sharp) |
| JVET-C0087 | m38711 | 2016-05-24 01:30:34 | 2016-05-26 10:22:45 | 2016-05-26 10:22:45 | EE: Cross-check of EE2.3 NSST-PDPC Harmonization (JVET-C0042) | V. Seregin (Qualcomm) |
| JVET-C0088 | m38712 | 2016-05-24 01:31:36 | 2016-05-26 10:51:38 | 2016-05-26 10:51:38 | EE: Cross-check of EE2.1 QTBT (JVET-C0024) | V. Seregin, J. Chen (Qualcomm) |
| JVET-C0089 | m38713 | 2016-05-24 01:49:30 | 2016-05-29 09:44:05 | 2016-05-29 09:44:05 | Cross-check of JVET-C0045 On secondary transform when primary transform is skipped | J. Chen, K. Rapaka (Qualcomm) |
| JVET-C0090 | m38716 | 2016-05-24 03:41:14 | 2016-05-26 09:01:00 | 2016-05-28 03:28:35 | Cross-check of JVET-C0043 (Arbitrary reference tier for intra directional modes) | T. Ikai (Sharp) |
| JVET-C0091 | m38718 | 2016-05-24 07:31:53 | 2016-05-26 03:25:09 | 2016-05-26 03:25:09 | Crosscheck for EE2.5 Improvements on adaptive loop filter (JVET-C0038) | X. Cai, L. Xu, Y. Chiu (Intel) |
| JVET-C0092 | m38734 | 2016-05-24 20:41:32 | 2016-05-27 09:42:41 | 2016-05-27 09:42:41 | Cross-check of JVET-C0047: Generalized bi-prediction for inter coding | S.-H. Kim (Sharp) |
| JVET-C0093 | m38739 | 2016-05-25 02:35:08 | 2016-05-26 14:47:07 | 2016-05-26 14:47:07 | Cross-check of JVET-C0025: Simplification of motion compensation filter for affine inter prediction | H. Zhang, H. Yang (Huawei) |
| JVET-C0094 | m38740 | 2016-05-25 02:45:06 | 2016-05-26 14:48:25 | 2016-05-26 14:48:25 | Cross-check of JVET-C0061: Decoder-side intra mode derivation | X. Ma, H. Yang (Huawei) |
| JVET-C0095 | m38751 | 2016-05-25 09:19:16 | 2016-05-25 09:22:28 | 2016-05-26 17:06:30 | EE2.4: De-quantization and Scaling for Next Generation Containers | J. Zhao, A. Segall, S.-H. Kim (Sharp) |
| JVET-C0096 | m38765 | 2016-05-25 18:16:11 | 2016-05-26 14:30:53 | 2016-05-28 09:54:43 | Cross-check of JVET-C0071 (Multiple line-based intra prediction) | L. Zhang, V. Seregin, N. Hu (Qualcomm) |
| JVET-C0097 | m38772 | 2016-05-25 21:21:56 | 2016-05-25 21:29:56 | 2016-05-25 21:29:56 | Cross-check of JVET-C0071 (Multiple line-based intra prediction) | P.-H. Lin, Y.-J. Chang, J.-S. Tu, C.-C. Lin, C.-L. Lin, (ITRI) |
| JVET-C0098 | m38782 | 2016-05-26 10:18:08 | 2016-05-26 10:22:58 | 2016-05-26 10:22:58 | Cross-check of arbitrary reference tier for intra directional modes (JVET-C0043) | B. Li, J. Xu (Microsoft) |
| JVET-C0099 | m38789 | 2016-05-26 13:12:04 | 2016-05-26 17:28:33 | 2016-05-26 18:36:36 | Further JVET CTC simplification for RA | M. Sychev (Huawei) |
| JVET-C0100 | m38807 | 2016-05-27 12:01:27 | 2016-05-27 17:14:01 | 2016-05-27 17:14:01 | Report of BoG on JEM software and SCC tools | X. Li |
| JVET-C0101 | m38809 | 2016-05-27 12:21:17 | 2016-05-27 12:22:09 | 2016-05-31 08:13:53 | Report of BoG on QTBT configuration setting | K. Choi |
| JVET-C0102 | m38810 | 2016-05-27 12:25:45 | 2016-05-27 12:31:45 | 2016-05-28 09:17:33 | Luma delta QP adjustment based on video statistical information | J. Kim, J. Lee, E. Alshina, Y. Park |
| JVET-C0103 | m38812 | 2016-05-27 17:32:34 | 2016-05-29 10:12:23 | 2016-05-29 10:12:23 | Cross-check of JVET-C0046 RExt coding tools support on JEM | J. Chen, F. Zou |
| JVET-C0104 | m38830 | 2016-05-30 12:11:59 | 2016-05-30 22:54:31 | 2016-05-31 13:28:05 | Report of BoG on test material | T. Suzuki |
| JVET-C0105 | m38836 | 2016-05-30 23:44:53 | 2016-05-30 23:46:35 | 2016-05-30 23:46:35 | Request for actions regarding JEM complexity | Guillaume Barroux (Fujitsu), Tomohiro Ikai (Sharp), Yukihiro Bandoh (NTT), Haitao Yang (Huawei), Atsuro Ichigaya (NHK), Wassim Hamidouche (INSA), Ryoji Hashimoto (Renesas) |
| JVET-C1000 | m38862 | 2016-06-03 20:42:16 |  |  | Meeting Report of the 3rd JVET Meeting | J.-R. Ohm, G. J. Sullivan |
| JVET-C1001 | m38864 | 2016-06-03 20:45:44 |  |  | Algorithm description of Joint Exploration Test Model 3 (JEM3) | J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce |
| JVET-C1002 | m38846 | 2016-06-01 10:14:27 | 2016-06-01 10:16:00 | 2016-06-01 10:16:00 | Work plan for assessment of test material | T. Suzuki |
| JVET-C1011 | m38838 | 2016-05-31 10:08:52 | 2016-05-31 10:09:08 | 2016-06-02 14:47:58 | Description of Exploration Experiments on Coding Tools | E. Alshina, J. Boyce, X. Xiu, Y.J. Chang, V. Seregin, S.-H. Kim |

# Annex B to JVET report:
List of meeting participants

The participants of the third meeting of the JVET, according to badge pick-up records of the host registration desk (approximately 130 people in total), were as follows:

- Yong-Jo Ahn (Kwangwoon Univ.)

- Elena	Alshina (Samsung Electronics)

- Alireza Aminlou (Nokia)

- Peter Amon (Siemens AG)

- Kenneth Andersson (LM Ericsson)

- Yukihiro Bandoh (NTT)

- Guillaume Barroux (Fujitsu Labs)

- Philippe Bordes (Technicolor)

- Jill Boyce (Intel)

- Benjamin Bross (Fraunhofer HHI)

- Done Bugdayci Sansli (Qualcomm Tech.)

- Yao-Jen Chang (ITRI Intl.)

- Chun-Chi Chen (NCTU/ITRI)

- Jianle	Chen (Qualcomm)

- Weizhong Chen (Huawei)

- Yi-Jen Chiu (Intel)

- Kiho Choi (Samsung Electronics)

- Olena Chubach (RWTH Aachen Univ.)

- Takeshi Chujoh (Toshiba)

- Amith DSouza (Samsung)

- Jana Ehmann (LG electronics)

- Arild Fuldseth (Cisco Systems Norway)

- Onur Guleryuz (LG electronics)

- Antti Hallapuro (Nokia)

- Wassim Hamidouche (IETR/INSA)

- Miska Hannuksela (Nokia)

- Ryoji Hashimoto (Renesas)

- Yuwen He (InterDigital Commun.)

- Per Hermansson (Ericsson)

- Ted Hsieh (Qualcomm Tech.)

- Han Huang (Mediatek)

- Yu-Wen Huang (MediaTek)

- Atsuro Ichigaya (NHK Japan Broadcasting Corp.)

- Tomohiro Ikai (Sharp)

- Shunsuke Iwamura (NHK Japan Broadcasting Corp.)

- Euee S. Jang (Hanyang Univ.)

- Hyeongmoon Jang (LG Electronics)

- Byeungwoo Jeon (Sungkyunkwan Univ., SKKU)

- Seungsu Jeon (Sungkyunkwan Univ., SKKU)

- Bora Jin (Samsung)

- Rajan Joshi (Qualcomm)

- Dongsan Jun (ETRI)

- Joël Jung (Orange Labs)

- Jung Won Kang (ETRI)

- Marta Karczewicz (Qualcomm Tech.)

- Kei Kawamura (KDDI)

- Dae Yeon Kim (Chips & Media)

- Jae-Gon Kim (Korea Aerosp. Univ.)

- Jim-Pil Kim (LG Electronics)

- Joo Young Kim (KT)

- Kibaek Kim (Hanyang Univ.)

- Namuk Kim (Sungkyunkwan Univ., SKKU)

- Seung-Hwan Kim (Sharp)

- Rob Koenen (TNO)

- Jani Lainema (Nokia)

- Sebastien Lasserre (Technicolor)

- Bae-Keun Lee (KT)

- Bumshik Lee (LG Electronics)

- Jae Yung Lee (Sejong Univ.)

- Jaeho Lee (LG Electronics)

- Jinho Lee (ETRI)

- Soon-Jin Lee (Sejong Univ.)

- Sungwon Lee (Qualcomm)

- Xiang Li (Qualcomm Tech.)

- Chongsoon Lim (Panasonic)

- Jaehyun Lim (LG Electronics)

- Po Han Lim (ITRI Intl.)

- Sung-Chang Lim (ETRI)

- Sung-Won Lim (Sejong Univ.)

- Ching-Chieh Lin (ITRI Intl.)

- Shan Liu (MediaTek)

- Victorien Lorcy (B-Com)

- Ajay Luthra (Arris)

- Xiang Ma (Huawei)

- Kiran Misra (Sharp)

- Matteo Naccari (BBC R&D)

- Takayuki Nakachi (NTT)

- Ohji Nakagami (Sony)

- Panos Nasiopoulos (Univ. British Columbia)

- Andrey Norkin (Netflix)

- Jens-Rainer Ohm (RWTH Aachen Univ.)

- Patrice Onno (Canon Research Centre France)

- Krit Panusopone (Motorola Mobility)

- Min Woo Park (Samsung Electronics)

- Stéphane Patëux (Orange Labs)

- Tangi Poirier (Technicolor)

- Mahsa Pourazad (TELUS Commun. & UBC)

- Adarsh Krishnan Ramasubramonian (Qualcomm Tech.)

- S. Biplab Raut (Samsung)

- Justin Ridge (Nokia)

- Christopher Rosewarne (CiSRA / Canon)

- Dmytro Rusanovskyy (Qualcomm)

- Amir Said (Qualcomm Tech.)

- Jesus Sampedro (Polycom)

- Jonatan Samuelsson (LM Ericsson)

- Yago Sanchez De La Fuente (Fraunhofer HHI)

- Andrew Segall (Sharp)

- Vadim Seregin (Qualcomm)

- Masato Shima (Canon)

- Donggyu Sim (Kwangwoon Univ.)

- Karsten Sühring (Fraunhofer HHI)

- Gary Sullivan (Microsoft)

- Teruhiko Suzuki (Sony)

- Maxim Sychev (Huawei Tech.)

- Yasser Syed (Comcast Cable)

- Anish Tamse (Samsung)

- Han Boon Teo (Panasonic)

- Emmanuel Thomas (TNO)

- Uday Thakur (RWTH Aachen Univ.)

- Tadamasa Toma (Panasonic)

- Jih-Sheng Tu (ITRI international)

- Yi-Shin Tung (ITRI USA / MStar Semi.)

- Jiyoon Um (Y. P. Lee & Mocks)

- Sebastiaan Van Leuven (Magic Pony Technologies)

- Rahul Vanam (InterDigital Commun.)

- Zheng Wang (Fujitsu R&D Center)

- Stephan Wenger (Vidyo)

- Mathias Wien (RWTH Aachen Univ.)

- Jianhua Wu (Panasonic)

- Ping Wu (ZTE UK)

- Haitao Yang (Huawei Tech.)

- Qiang Yao (KDDI)

- Jiangli Ye (Fujitsu R&D Center)

- Yan Ye (InterDigital Commun.)

- Haoping Yu (Futurewei / Huawei R&D USA)

- Lu Yu (Zhejiang Univ.)

- Li Zhang (Qualcomm Tech.)

- Louis (Lei) Zhang (AMD)

- Jianhua Zheng (Huawei Tech.)

- Minhua Zhou (Broadcom)