| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0001 |
| --- | --- |

| Title: | JVET AHG report: Tool evaluation (AHG1) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Qualcomm Inc. Elena Alshina Samsung Electronics | Email: Email: | martak@qti.qualcomm.com elena_a.alshina@samsung.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on Tool evaluation (AHG1) between the 2nd JVET meeting at San Diego, USA (20–26 February 2016) and the 3rd Meeting at Geneva, Switzerland (26 May – 1 June 2016).

# Mandates

At the 2nd JVET meeting, AHG1 on Tool evaluation was established with the following mandates:

- Coordinate the exploration experiments.

- Investigate interaction of tools in JEM and exploration experiment branches.

- Discuss and evaluate methodologies and criteria to assess the benefit of tool.

- Study and summarize new technology proposals

# Summary of Activities

AhG1 kick-off message was sent out at March, 4. There are more than 20 e-mails related to AhG1 activity in JVET reflector. Mainly Exploration Experiments activity was discussed.

# JEM Status

Joint Exploration Test Model Software (HM-16.6-JEM-2.0) was released 19th of March, 2016. The software can be downloaded at:

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-2.0

Algorithms included into JEM2.0 are described in [1]. Among them

- Block structure

  - Larger Coding Tree Unit (up to 256x256) and transforms (up to 64x64)

  - Quadtree plus binary tree (QTBT) block structure  only in EE SW branch

- Intra prediction improvements

  - 65 intra prediction directions  modified in JEM2.0 compared to JEM1.0

  - 4-tap interpolation filter for intra prediction

  - Boundary filter applied to other directions in addition to horizontal and vertical ones

  - Cross-component linear model (CCLM) prediction

  - Position dependent intra prediction combination (PDPC)

  - Adaptive reference sample smoothing

- Inter prediction improvements

  - Sub-PU level motion vector prediction

  - Locally adaptive motion vector resolution (AMVR)

  - 1/16 pel motion vector storage accuracy modified in JEM2.0 compared to JEM1.0

  - Overlapped block motion compensation (OBMC)

  - Local illumination compensation (LIC)

  - Affine motion prediction  modified in JEM2.0 compared to JEM1.0

  - Pattern matched motion vector derivation

  - Bi-directional optical flow (BIO)

- Transform

  - Explicit multiple core transform

  - Mode dependent non-separable secondary transforms

  - Signal dependent transform (SDT)  disabled by default

- Adaptive loop filter (ALF)

- Enhanced CABAC design

  - Context model selection for transform coefficient levels

  - Multi-hypothesis probability estimation

  - Initialization for context models

At the 2nd JVET meeting common test conditions were modified and test sequences were updated. So direct performance comparison between JEM1.0 and JEM2.0 is difficult.

Table 1 shows JEM1.0 and JEM2.0 performance compared to HM if both GOP-size and QP/lambda selection are the same for test and reference.

Table 1: JEM coding performance summary in RA test configuration compared to HEVC.

|  | JEM1.0 vs HM16.6 GOP-size =8 for both |  |  | JEM2.0 vs HM16.9 GOP-size =16 for both |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A1 |  |  |  | -22.4% | -21.1% | -24.2% |
| Class A2 |  |  |  | -28.8% | -29.1% | -22.8% |
| Class B | -21.3% | -13.2% | -9.2% | -21.8% | -21.5% | -15.8% |
| Class C | -20.7% | -14.8% | -18.2% | -20.7% | -19.4% | -21.7% |
| Class D | -20.5% | -9.8% | -12.1% | -21.2% | -16.4% | -17.1% |
| Overall | -20.8% | -16.7% | -15.4% | -22.9% | -21.5% | -20.1% |
| Enc. Time | 6.0 |  |  | 5.3 |  |  |
| Dec. Time | 8.0 |  |  | 8.4 |  |  |

Net effect of enlarging of GOP size, QP/lambda selection modification and tools up-date in JEM2.0 compared to JEM1.0 is shown in Table 2. Test data were provided by JEM SW coordinators in [2].

Table 2: JEM2.0 (GOP-size =16) vs JEM1.0 (GOP-size =8) performance in RA test.

|  | JEM2.0 vs JEM1.0 GOP-size is different |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | -3.8% | -17.9% | -16.4% |
| Class A2 | -4.4% | -17.7% | -15.7% |
| Class B | -7.2% | -22.8% | -24.5% |
| Class C | -8.2% | -18.7% | -18.1% |
| Class D | -7.9% | -20.8% | -20.3% |
| Overall | -6.4% | -19.7% | -19.3% |
| Enc. Time | 1.0 |  |  |
| Dec. Time | 1.0 |  |  |

# Exploration Experiments activity

At the 2nd JVET meeting Exploration Experiments practice was established. For each new coding tool under consideration special SW branch was created. After implementation of each tool announcement via JVET reflector was done. There were 7 exploration experiments on new coding tools. For all of them input contribution for this meeting were submitted.

| JVET-VC number | Title | Authors |
| --- | --- | --- |
| JVET-C0024 | EE2.1: Quadtree plus binary tree structure integration with JEM tools | H. Huang, K. Zhang, Y.-W. Huang, S. Lei (MediaTek) |
| JVET-C0035 | EE2.6: Modification of Merge candidate derivation: ATMVP simplification and Merge pruning | S. Lee, W.-J. Chien, L. Zhang, J. Chen, M. Karczewicz (Qualcomm) |
| JVET-C0038 | EE2.5: Improvements on adaptive loop filter | M. Karczewicz, L. Zhang, W.-J. Chien, X. Li (Qualcomm) |
| JVET-C0042 | EE2.3: NSST-PDPC Harmonization | S.-H. Kim, A. Segall (Sharp) |
| JVET-C0053 | EE2.7: TU-level non-separable secondary transform | X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi (Qualcomm) |
| JVET-C0077 | EE2.2: Non Square TU Partitioning | K. Rapaka, J. Chen, L. Zhang, W.-J. Chien, M. Karczewicz (Qualcomm) |
| JVET-C0095 | EE2.4: De-quantization and Scaling for Next Generation Containers | J. Zhao, A. Segall, S.-H. Kim (Sharp) |

Summary of exploration experiments is provided in [3].

# Contributions to this Meeting

In total 20 contributions proposing new coding tools for JEM or improvements of JEM design were submitted in following categories:

- Intra (5),

- Inter (6),

- Transform (2),

- Partitioning (2) ,

- HDR/WCG (1),

- De-blocking (1),

- Other (3).

| JVET-VC number | Title | Authors | Category |
| --- | --- | --- | --- |
| JVET-C0022 | Proposed improvements to the Adaptive multiple Core transform | P. Philippe (Orange), V. Lorcy (bcom) | Transform |
| JVET-C0023 | Predictors Elimination Technique for HEVC | M. Korman, O. Prosekov (Synopsys) | Intra |
| JVET-C0025 | Simplification of motion compensation filter for affine inter prediction | J. Nam, H. Jang, J. Lee, B. Lee, J. Lim (LGE) | Inter |
| JVET-C0026 | Tiles coding improvement for Inter pictures by improved merge list at tile boundaries | S. Biplab Raut (Samsung) | Inter |
| JVET-C0031 | BIO improvement to reduce the encoder and decoder complexities | J. Lee, N. Park, J. Nam, J. Lim (LGE) | Inter |
| JVET-C0039 | Decoupled Luma/Chroma Transform Trees for Intra | F. Urban, T. Poirier, F. Le Léannec (Technicolor) | Partitioning |
| JVET-C0040 | Adaptive Clipping in JEM2.0 | F. Galpin, P. Bordes, F. Le Léannec (Technicolor) | Other |
| JVET-C0043 | Arbitrary reference tier for intra directional modes | Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI) | Intra |
| JVET-C0046 | RExt coding tools support on JEM | T. Tsukuba, O. Nakagami, T. Suzuki (Sony) | Other |
| JVET-C0047 | Generalized bi-prediction for inter coding | C.-C. Chen, X. Xiu, Y. He, Y. Ye (InterDigital) | Inter |
| JVET-C0049 | Extended deblocking-filter process for large block boundary | K. Kawamura, Q. Yao, S. Naito (KDDI Corp.) | De-blocking |
| JVET-C0054 | Grouped signalling for transform in QTBT | X. Zhao, V. Seregin, A. Said, M. Karczewicz (Qualcomm) | Partitioning |
| JVET-C0055 | Neighbor based intra most probable modes list derivation | V. Seregin, X. Zhao, A. Said, M. Karczewicz (Qualcomm) | Intra |
| JVET-C0061 | Decoder-side intra mode derivation | X. Xiu, Y. He, Y. Ye (InterDigital) | Intra |
| JVET-C0062 | Improved affine motion prediction | F. Zou, J. Chen, M. Karczewicz, X. Li, H.-C. Chuang, W.-J. Chien (Qualcomm) | Inter |
| JVET-C0063 | EE2.7 related: Improved non-separable secondary transform | X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen (Qualcomm) | Transform |
| JVET-C0066 | On Coefficient Scaling | D. B. Sansli, D. Rusanovskyy, J. Sole, A.K. Ramasubramonian, M. Karczewicz (Qualcomm) | HDR/WCG |
| JVET-C0068 | Motion vector coding optimizations | J. Samuelsson, P. Wennersten, R. Yu, U. Hakeem (Ericsson) | Inter |
| JVET-C0069 | Direction-dependent scan order with JEM tools | S. Iwamura, A. Ichigaya (NHK) | Other |
| JVET-C0071 | Multiple line-based intra prediction | J. Li (Peking Univ.), B. Li, J. Xu (Microsoft), R. Xiong (Peking Univ.), G.-J. Sullivan (Microsoft) | Intra |

# Recommendations

The AHG recommends:

- To review all the related contribution.

- To continue Exploration Experiments practice.

# References

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce (JEM editors) “Algorithm description of Joint Exploration Test Model 2”, JVET-B1001, San-Diego, Feb. 2016.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, 2nd Meeting, Feb. 2016.

- E. Alshina, J. Boyce, Y.-W. Huang, S.-H. Kim, L. Zhang “Exploration Experiments on Coding Tools Report”, JVET-C0010, Geneva, May. 2016