| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0010 |
| --- | --- |

| Title: | Exploration Experiments on Coding Tools Report. |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Report |  |  |
| Author(s) or Contact(s): |  | E. Alshina, J. Boyce, Y.-W. Huang, S.-H. Kim, L. Zhang | Email: | elena_a.alshina@samsung.com jill.boyce@intel.com yuwen.huang@mediatek.com kimse@sharplabs.com lizhang@qti.qualcomm.com |
| Source: |  | EE coordinators |  |  |

____________________________

# Abstract

Seven experiments on coding tools were agreed to carry out between JVET-B and JVET-C meetings in order to get better understanding of technologies considered for inclusion to the next version of JEM, analyse and verify their performance, complexity and interaction with existing JEM tools. This report summarises the status of each experiment.

# Introduction

JVET group coordinated experiments were conducted following principles were agreed in [1].

Due to the JEM2.0 reference SW release delay development braches for each EE were created later than expected and so first release of EE s/w happened at April, 19. It was announced via JVET reflector.

Table 1. Summary of Exploration Experiments.

| # | Main test and sub-tests | Document | Y-BD-rate (Enc/DecTime) | Cross-check |
| --- | --- | --- | --- | --- |
| 2.1 | Quad-tree plus binary-tree (QTBT) (*) SW released at April, 19, modified during EE. | JVET-C0024 | AI: -3.3% (ET 5.4, DT 1.0) RA: -3.8% (ET ?, DT ?) LD: -4.5% (ET 2.4, DT 1.1) LDP: -4.5% (ET 2.2, DT 1.2) | JVET-C0056 Samsung |
|  | - Low-complexity Intra configuration |  | AI: -2% (ET 2.5, DT 1.0) |  |
| 2.2 | Non Square TU Partitioning(**) SW released and unchanged since April, 19 | JVET-C0077 JVET-B0047 | AI: -1.5% (ET 1.9, DT 1,0) RA: -1.0% (ET 1.1, DT 1.0) LD: -0.7% (ET 1.1, DT 1.0) LDP: -0.8% (ET 1.1, DT 1.0) | JVET-B0068 Sony |
| 2.3 | NSST and PDPC index coding (all modifications enabled) - w/o removing PDPC restriction | JVET-C0042 | AI: -0.6% (ET 1.5, DT 1,0) RA: -0.2% (ET 1.1, DT 1.0) LD: -0.1% (ET 1.1, DT 1.0) LDP: -0.1% (ET 1.2, DT 1.0) AI: -0.2% (ET 0.9, DT 1,0) RA: -0.1% (ET 1.0, DT 1.0) LD: -0.0% (ET 1.1, DT 1.1) LDP: -0.0% (ET 1.0, DT 1.0) | JVET-C0059 Samsung JVET-C0087 Qualcomm |
| 2.4 | De-quantization and scaling for next generation containers SW released and unchanged since April, 19 | JVET-C0095 registered May.25 |  |  |
| 2.5 | Improvements on adaptive loop filter SW released and unchanged since April, 19 | JVET-C0038 | AI: -1,0% (ET 1.0, DT 1.1) RA: -1,2% (ET 1.0, DT 1.0) LD: -1.1% (ET 1.0, DT 1.0) LDP: -1.5% (ET 1.0, DT 1.0) | JVET-C0036 Huawei JVET-C0057 Samsung JVET-C0074 Sharp JVET-C0091 Intel |
|  | - W/o chroma filter vs whole package |  | AI: 0,1% (ET 1.0, DT 1.0) RA: 0,0% (ET 1.0, DT 1.0) LD: 0.0% (ET 1.0, DT 1.0) LDP: 0.0% (ET 1.0, DT 1.0) |  |
|  | - W/o prediction from fixed filters vs whole package |  | AI: 0,3% (ET 1.0, DT 1.0) RA: 0,2% (ET 1.0, DT 1.0) LD: 0.1% (ET 1.0, DT 1.0) LDP: 0.1% (ET 1.0, DT 1.0) |  |
| 2.6 | Modification of Merge candidate derivation SW released and unchanged since April, 19 | JVET-C0035 | RA: -0.1% (ET 1.0, DT 1.0) LD: -0.2% (ET 1.0, DT 1.0) LDP: -0.2% (ET 1.0, DT 1.0) | JVET-C0060 Samsung JVET-C0073 Sharp JVET-C0085 Huawei |
|  | - ATMVP simplification |  | RA: -0.0% (ET 1.0, DT 1.0) LD: -0.0% (ET 1.0, DT 1.0) LDP: -0.0% (ET 1.0, DT 1.0) |  |
|  | - Merge pruning |  | RA: -0.1% (ET 1.0, DT 1.0) LD: -0.2% (ET 1.0, DT 1.0) LDP: -0.2% (ET 1.0, DT 1.0) |  |
| 2.7 | TU-level non-separable secondary transform (***) SW released at April, 19, modified during EE. | JVET-C0053 | AI: -0,5% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: -0.1% (ET 1.0, DT 1.0) LDP: -0.0% (ET 1.0, DT 1.0) | JVET-C0058 Samsung JVET-C0086 Sharp JVET-C0076 Orange, B-com |
|  | - W/o HyGT |  | AI: -0.1% (ET 0.8, DT 1.0) RA: ?% (ET 1.0, DT 1.0) LD: -0.1% (ET 1.0, DT 1.0) LDP: -0.0% (ET 1.0, DT 1.0) |  |
|  | - All coeff. sub-groups use secondary transform |  | AI: -0.5% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: -0.1% (ET 1.0, DT 1.0) LDP: -0.1% (ET 1.0, DT 1.0) |  |
|  | - Secondary transform is applied for all non-zero TUs (default 2 non zero coeff) |  | AI: -0.2% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: -0.1% (ET 1.0, DT 1.0) LDP: -0.1% (ET 1.0, DT 1.0) |  |
|  | - Secondary transform is applied for transform-skip and LM mode |  | AI: -0.5% (ET 0.8, DT 1.0) RA: ?% (ET ?, DT ?) LD: -0.1% (ET 1.0, DT 1.0) LDP: -0.1% (ET 1.0, DT 1.0) |  |
|  | - CU-level signalling with HyGT |  | AI: -0.6% (ET 1.5, DT 1.1) RA: ?% (ET ?, DT ?) LD: -0.1% (ET 1.1, DT 1.0) LDP: -0.0% (ET 1.1, DT 1.0) |  |

Comments:

(*) Full tests data available in cross-check report (not in original contribution), significant Chroma gain is observed (~5% AI, LDB and LDP, ~8,5% in RA), significant Class F gain (not included in the previous average number by the CTC template) is observed (AI Y: ~4%, UV: ~7%, RA Y: ~5%, UV: ~9%, LDB Y: ~8%, UV: ~10%, LDP Y: ~8%, (**) Tested vs HM16.6.

(***) Luma BD-rate gain is accompanied by Chroma drop. Only partial test data are available by May, 24.

# List of Experiments

## . Quad-tree plus binary-tree (QTBT)

All tools described in [2] (except “Signal Dependent Transform”) are ported into QTBT framework.

Questions recommended to be answered during EE tests.

[Q]: What is the performance of QTBT on top of HEVC?

[A]: 4,3%(AI) / 6.0% (RA) / 6.4%(LDB) / 6.8%(LDP), significant Chroma gain is observed (~15% AI,RA,LDB, and LDP)

[Q]: What is the performance of QTBT on top of JEM2.0?

[A]: 3,3%(AI) / 3.8% (RA) / 4.5%(LDB) / 4.5%(LDP), significant Chroma gain is observed (~5%AI,LDB, and LDP, ~8,5% in RA), significant Class F gain (not included in the previous average number by the CTC template) is observed (AI Y: ~4%, UV: ~7%, RA Y: ~5%, UV: ~9%, LDB Y: ~8%, UV: ~10%, LDP Y: ~8%,UV: 9%)

[Q]: It is suggested that the software should be able to support the capability to enable and disable each individual tool.

[A]: For most of the tools it is possible to turn on and off (but not for all, similar to JEM2.0 s/w)

## . Non Square TU Partitioning

Implementation was provided on top of HM16.6 only (not on top of JEM2.0). There is no cross-check.

Questions recommended to be answered during EE tests.

[Q]: Provide performance results on top of JEM.

[A]: Have been studied

[Q]: Compare Non-square TU partitioning performance with QTBT.

[A]: Have been studied

## . NSST and PDPC index coding

This experiment includes 4 modifications of JEM:

- Using TU binarization for NSST index coding and adjusts context modeling based on intra prediction mode and partition type.

- Removing the restriction disallowing NSST restriction based on PDPC flag

- Coding NSST index in CU level and send TU level flag to indicate whether NSST is applied

- HyGT transform from EE2.7

Questions recommended to be answered during EE tests.

[Q]:  Report performance and complexity of modifications (1) and (2) separately.

[A]: Whole package results and results without removing PDPC restriction, i.e., (2), are provided

[Q]: Study potential interaction with TU based NSST (i.e. JVET-B0059)

[A]: Have been studied (HyGT from JVET-B0059 was included in to proposed design)

## . De-quantization and scaling for next generation containers

S/w implementation was provided, but no contribution with test results submitted. Test content to be used still needs to be discussed (BoG activity is suggested). One contribution (on a same topic (JVET-C0066) was identified.

Questions recommended to be answered during EE tests.

[Q]:  How complex is the function f() in the proposal?

[A]: Pseudo-code has been provided in contribution.

[Q]:  How much overhead is required for delta QP signaling at different granularities?

[A]: 0,4% (CTU 6464).

[Q]:  What is performance of tool for SDR content in an ST-2084 container?

[A]: ~2% better that deltaQP approach.

[Q]:  Compare with other techniques studied in HDR JCTVC activity.

## . Improvements on adaptive loop filter

The number of changes were proposed to ALF to further increase its coding efficiency:

- Modified classification with the diagonal gradients taken into consideration,

- Geometric transformations of filter coefficients (diagonal, vertical flip and rotation)

- Prediction from fixed filters for intra-coded slices.

Questions recommended to be answered during EE tests.

[Q]:  Simulation results are only provided for the combination of changes. Some of the changes are inherently grouped together, but some could be separated. Specifically, the following test results are requested to be provided:

1) The whole proposal

2) The whole proposal with prediction from fixed filters disabled: to check the impact of fixed filters

3) The whole proposal without the alignment between chroma filter shape and luma filter shape: to check the impact of the cleanup to the chroma filter shape.

[A]: All requested sub-tests were performed

## . Modification of Merge candidate derivation

First, the derivation of ATMVP merge candidate of the current PU is simplified in order to  avoid the repetitive scanning process of neighboring blocks. Second, the pruning of the merge candidates is modified: RefIdx might point to the same RefPicture if it exists in both RefLists. It si proposed to use actual POC instead RefIdx during merge candidates derivation.

Questions recommended to be answered during EE tests.

[Q]:  Four different modifications are proposed but only combined performance results are available. Individual tool performance numbers need to be provided.

[A]: All requested sub-tests were performed

## . TU-level non-separable secondary transform

Secondary transform signaling is moved to the TU level. This saves encoder run time compare to CU-level signaling (there is no need to repeat prediction). New secondary transform (Hypercube-Givens Transform) is introduced. In JEM secondary transform is applied for Luma and Chroma if there is at least one no-zero coefficient in whole CU. Proposed TU signaling is done only if there are at least 2 non-zero coefficients is TU. In JEM secondary transform is applied to each 44 coefficients sub-group. In this experiment only to four top-left coefficients sub-group goes through secondary transform.  Application of secondary transform to Chroma depends on Chroma Intra prediction mode. Additionally implementation on top of EE2.1 (QTBT) was provided.

Questions recommended to be answered during EE tests.

[Q]:  What is performance and complexity impact recently introduced Hypercube-Givens Transform (HyGT)?

[A]: Addressed (partial test results are available in first version of the contribution).

[Q]:  How much performance change comes from restricting NSST to only four top-left coefficients sub-groups?

[A]: Addressed (partial test results are available in first version of the contribution).

[Q]:  Is this performance effect the same at low and high bit-rates?  Additionally to JEM CTC tests with QP =1, 5, 9, 13 recommended to be performed.

[A]: Not addressed yet.

[Q]:  What is the performance impact of restriction of NSST only for TUs with no less than 2 non-zero coefficients?

[A]: Addressed (partial test results are available in first version of the contribution).

[Q]:  Interaction with JVET-B0051 should be studied.

[A]: Has been studied. Some aspects of JVET-B0051 (NSST and PDPC can be enabled at the same time) become the part of proposed design.

# Recommendation

- Review EE tests’ contribution and cross-checks.

- Make decisions on each part on technology under consideration based on EE results and cross-checker’s opinion.

- Discontinue EE 2.2 (Non Square TU Partitioning ).

- Discuss HDR test materials, performance assesment and related contribution prior making decision on EE2.4 (De-quantization and scaling for next generation containers)

# Acknowledgement

- Thanks all proponents for providing EE s/w in time with proper announcement and explanation about modification in JVET reflector.

- Thanks all volunteers  provided comprehensive cross-check.

# References

- E. Alshina, J. Boyce, Y.-W. Huang, S.-H. Kim, L. Zhang (EE coordinators) Description of Exploration Experiments on Coding ToolsJVET-B1011, 2nd Meeting, Feb. 2016.

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce (JEM editors) “Algorithm description of Joint Exploration Test Model 2”, JVET-B1001, San-Diego, Feb. 2016.