| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0033 |
| --- | --- |

| Title: | On comparison criteria for Virtual Reality video coding schemes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Anna van Buerenplein 1 2595 DA Den Haag Netherlands | Tel: Email: | +31 6 52 77 91 48 emmanuel.thomas@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution discusses challenges of streaming 360 video for Virtual Reality applications. Especially, the bottleneck of the available bandwidth is the main issue towards a high quality video streaming. Many new schemes to represent a spherical video have been proposed such that current video coding schemes perform better. In addition, the selective streaming of the user viewpoint emerges in the literature as a tool to further reduce the bandwidth requirement. However, this selective download and decoding requires the video encoding to produce spatial random access point in the videos.

This contribution argues that video compression efficiency cannot be the only criteria to judge omnidirectional video coding schemes and spatial random access in the video is at least as important as the compression efficiency criterion.

# Problem Statement

As shown in [1] , state-of-the-art omnidirectional video coding schemes usually start with mapping the spherical video onto a cube. Next, the faces of cubes are rearranged in a rectangular fashion. This way, visual distortions are reduced which yield better encoding efficiency with traditional video coding scheme.

In [1], the expert presents another rearrangement that preserves continuity at the edges of the cube. This way, a higher compression efficiency is achieved compared to the arrangement presented in Figure 1.

However, the bandwidth requirement of streaming 360 videos can be dramatically reduced by streaming only what is necessary to recreate the user’s viewpoint. It is indeed not needed to stream a high resolution view of the scene behind the user’s back. There are many initiatives investigating this technique as for instance the one proposed by the Facebook team [2]. But for this to work, it is fundamental to allow spatial random access in the video of each face of the cube. With the HEVC video coding standard, this is can achieved by using the HEVC tiles with a proper configuration of the encoder, e.g. constraining motions vectors to tile boundaries, as shown in Figure 2.

Therefore, it appears that considering spatial random access becomes a key comparison criterion while compression efficiency remains important but the unique one.

Figure 1- Cubic video representation presented in [1]

Figure 2 - HEVC tiles for spatial random access of cube faces

# Conclusion

This contribution advocates for considering spatial random access for comparing omnidirectional video coding schemes. For Virtual Reality applications, this can mean more bitrate reduction that a more efficient coding scheme for the entire spherical video. In practice, it is proposed that when cubic formats are considered for comparison they should also be encoded using features similar to the HEVC tiles such that spatial random access is enabled. We believe that this will be a key aspect for mass deployments of VR 360 video streaming.

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-C0021, “GoPro test sequences for Virtual Reality video coding”, Adeel Abbas, Geneva, CH, 26–31 May 2016

- “Under the hood: Building 360 video”, source: https://code.facebook.com/posts/1638767863078802/under-the-hood-building-360-video/