| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0045 |
| --- | --- |

| Title: | EE2.7-related: On secondary transform when primary transform is skipped |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 takeshi.tsukuba@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to (a) disable secondary transform when primary transform is skipped, and (b) omit signaling transform_skip_flag when secondary transform is signalled.

Experimental results show that:

- BDrate changes for (a) are 0.00%, 0.XX%, -0.01% and 0.00% for AI, RA, LB and LP for class A to class E, respectively.

- BDrate changes for (a+b) are 0.AA%, 0.BB%, 0.CC% and 0.DD% for AI, RA, LB and LP for class A to class E, respectively.

It is noted that EE2.7 includes the same aspect discussed in the contribution. It is recommended to adopt the proposal or the same aspect of EE2.7 to JEM.

Introduction

In JEM-2.0 [1], the secondary transform can be applied when the transform skip flag is enabled as shown in Figure 1. This occurs when transform_skip_flag (TU-level) is 1 and NSST_idx (CU-level) is greater than 0.

Figure 1: JEM-2.0’s decoder-side diagram

The first and second transform selection in JEM-2.0 is summarized below.

Table 1-1 transform selection in JEM-2.0

| case | NSST_IDX (CU) | Transform_Skip_flag (TU) | 1st transform | 2nd transform |
| --- | --- | --- | --- | --- |
| 0 | >0 | 0 | DCT/DST | NSST |
| 1 | >0 | 1 | - | NSST |
| 2 | 0 | 0 | DCT/DST | - |
| 3 | 0 | 1 | - | - |

In the case 1, NSST is applied to residual signal directly since the first transform is skipped. Considering that NSST is originally designed for DCT (or DST) coefficients, the impact should be studied.

If NSST without DCT/DST is less impact for the coding gain, the authors think that removing the case 1 is an improvement. Because, signalling transform_skip_flag in TU-level is redundant. In addition, the encoding run-time can be saved.

From the idea, this contribution studies two items;

- the impact of the second transform when the first transform is not applied

- the impact of omitting the transform_skip_flag in TU-level when NSST_IDX is greater than 0

In EE2.7, the decision chart is modified as below.

Table 1-2 transform selection in EE2.7

| case | Transform_Skip_flag(TU) | NSST_IDX (TU) | 1st transform | 2nd transform |
| --- | --- | --- | --- | --- |
| 0 | 0 | >0 | DCT/DST | NSST |
| 1 | 0 | 0 | DCT/DST | - |
| 2 | 1 | Not signalled (inferred to 0) | - | - |

Since NSST_idx signalling is changed from CU-level to TU-level, the issue pointed by the authors (the case 1 in Table 1-1) is considered.

Proposal

Proposals are summarized as follows:

- Disable both the first and the secondary transform when transform skip is enabled

- Omit transform_skip_flag in TU-level when nsst_idx in CU-level is 0

The proposed decoder-side diagram is shown in Figure 2 and the transform selection is modified as shown in Table 2-1.

Figure 2: Proposed decoder-side diagram

Table 2-1: transform selection in proposal

| case | NSST_IDX (CU) | Transform_Skip_flag (TU) | 1st transform | 2nd transform |
| --- | --- | --- | --- | --- |
| 0 | >0 | Not signalled (inferred to 0) | DCT/DST | NSST |
| 1 | 0 | 0 | DCT/DST | - |
| 2 | 0 | 1 | - | - |

Change related to modification of signaling transform_skip_flag is highlighted in yellow.

// CU-level NSST index signaling case (JEM-2.0 [1])

if ( transform_skip_enabled_flag && !cu_transquant_bypass_flag &&
  (log2TrafoSize<=Log2MaxTransformSkipSize) && nsst_idx==0)

decode transform_skip_flag

Note that in JEM-2.0 nsst_idx syntax is signalled at the end of residual coding. In order to control signaling transform_skip_flag, nsst_idx is relocated before transform_skip_flag.

Experiments

The proposals were implemented on top of JEM-2.0 [1]. Two tests were performed on common test condition [2] as bellow:

TEST1: only disabling secondary transform when primary transform is skipped

TEST2: TEST1 + signaling modification

Table 3-1 to 3-2 summarises the results of experiments. For Random Access, the simulations were conducted with sequential encoding.

Cross-check will be provided by XXX (JVET-CXXXX).

Table 3-1: Results of TEST1 (only disabling secondary transform when primary transform is skipped) (TBD)

Table 3-2: Results of TEST2 (TEST1 + signaling modification) (TBD)

Conclusion

It is proposed to a) disable secondary transform when primary transform is skipped and b) omit signaling transform_skip_flag when secondary transform is signalled.

It is recommended to adopt the proposals or the same aspects of EE2.7.

Reference

[1] JEM-2.0, https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-2.0

[2] K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-B1010, Apr. 2016

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).