| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0046_r1 |
| --- | --- |

| Title: | RExt coding tools support on JEM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 takeshi.tsukuba@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a) software fix to support RExt coding tools on top of JEM-2.0 and b) transform skip support for 64x64 transform block. Experimental results show that, by enabling RExt tools and 64x64 transform skip (Fast Search is off), average coding gains of Class A to Class E are -0.05 %, -0.07%, -0.07% and -0.04% for AI, RA, LB and LP, respectively. For Class F, coding gains are -3.42%, -2.98%, -2.57% and -2.70% for AI, RA, LB and LP, respectively.

A patch to support RExt coding tools (including 64x64 Transform Skip) except Cross Component Prediction and Single Significance Map Context is provided.

In revision 1, the experimental results (Table 3-1) are revised.

Introduction

RExt coding tools are disabled on the JVET common test conditions [2]. In the contribution, coding performance of the RExt tools on top of JEM-2.0 is investigated.

It is reported that there are several problems in JEM-2.0 as follows:

- Assertion failed occurs at the encoder when Explicit Residual DPCM is enabled.

- 64x64 Transform skip doesn’t work appropriately when Max Transform Skip Size is set to 64x64 because high frequency components of 64x64 TB are zero out. The same problem occurs when transquant bypass is enabled.

Proposal

To solve those problems, it is proposed to a) fix the encoder code to support Explicit Residual DPCM and b) support transform skip and transquant bypass for 64x64 Transform Block to be consistent with the JEM-2.0 coding design.

Proposed changes are highlighted in yellow.

Explicit Residual DPCM support (only encoder side)

Void TEncSbac::codeExplicitRdpcmMode( …)

{

…

assert(tuHeight == tuWidth);

assert(tuHeight < 4);

…

}

64x64 transform skip and transquant bypass support

// encoder side

Void TEncSbac::codeCoeffNxN(…)

{

…

if (iSubSet == iLastScanSet || iSubSet == 0)

{

uiSigCoeffGroupFlag[iCGBlkPos] = 1;

}

#if COM16_C806_T64

else if (uiWidth >= 64 && (iCGPosY >= (codingParameters.heightInGroups / 2) || iCGPosX >= (codingParameters.widthInGroups / 2)) 	&& (!pcCU->getCUTransquantBypass(uiAbsPartIdx) && !pcCU->getTransformSkip(uiAbsPartIdx, compID))

// if trans/quant bypass is on or TransformSkipLog2MaxSize is set to 6 (64x64), it should be disabled to truncate high frequency components.

)

{

assert( 0 == uiSigCoeffGroupFlag[ iCGBlkPos ] );

}

…

}

// decoder side

Void TDecSbac::parseCoeffNxN(…)

{

…

if( iSubSet == iLastScanSet || iSubSet == 0)

{

uiSigCoeffGroupFlag[ iCGBlkPos ] = 1;

}

#if COM16_C806_T64

else if( uiWidth>=64 && ( iCGPosY>=(codingParameters.heightInGroups/2) || iCGPosX>=(codingParameters.widthInGroups/2) )

&& ( !pcCU->getCUTransquantBypass(uiAbsPartIdx) && !pcCU->getTransformSkip(uiAbsPartIdx, compID) )

// if trans/quant bypass is on or TransformSkipLog2MaxSize is set to 6 (64x64), it should be disabled to truncate high frequency components.

)

{

uiSigCoeffGroupFlag[ iCGBlkPos ] = 0;

}

#endif

…

}

Simulation Results

The proposals were implemented on top of JEM-2.0 [1]. The simulations were performed on the JVET common test conditions [2], where to enable RExt tools, following parameters are applied to the JVET common test conditions in accordance with the common text conditions for RExt [3].

- Profile=main-RExt

- CrossComponentPrediction=0

- TransformSkipLog2MaxSize=6

- ImplicitResidualDPCM=1

- ExplicitResidualDPCM=1

- ResidualRotation=1

- SingleSignificanceMapContext=1

- IntraReferenceSmoothing=1

- GolombRiceParameterAdaptation=1

- HighPrecisionPredictionWeighting=1

Note that Cross Component Prediction (CCP) is disabled because CCP is only available for 4:4:4 format , Single Significance Map Context doesn’t work because it is not ported on top of context modeling for transform coefficient levels changes (VCEG_AZ07_CTX_RESIDUALCODING) and TransformSkipLog2MaxSize is set to 6.

For Random Access, the simulations were conducted with sequential encoding.

Table 3-1 to 3-2 summarises the results of experiments when TransfromSkip Fast Search algorithm is on/off.

Note that 2Nx2N case Transform Skip is not used when TransformSkip Fast Search algorithm is utilized.

Table 3-1: Results of coding performance (--TransformSkipFast=0)

Table 3-2: Results of coding performance (--TransformSkipFast=1)

Conclusion

It is proposed to a) support Explicit Residual DPCM and b) omit zeroing out high frequency components of large Transform Block (over 32x32) for transform skip and transquant bypass.

It is recommended to adopt proposal b to enable transform skip and transquant bypass up to 64x64 transform block size to the next JEM.

Reference

[1] JEM-2.0, https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-2.0

[2] K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-B1010, Apr. 2016

[3] X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi, “TU-level non-separable secondary transform,”  JVET-B0059, Feb. 2016

[4] C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, Feb. 2014

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).