| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0078 |
| --- | --- |

| Title: | Cross-check of C0032 (Polyphase subsampled sequence coding) |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Report |  |  |
| Author(s) or Contact(s): |  | P. Philippe V. Lorcy T. Biatek | Email: | pierrick.philippe@orange.com |
| Source: |  | Orange, b<>com, TDF |  |  |

____________________________

# Abstract

This contribution verifies the elements described in contribution JVET-C0032. It essentially reproduces the signal paths proposed in the contribution and presents some coding results. Some recommendations are produced for the experimental set-up.

# Introduction

A polyphase decomposition of a video sequence on a frame basis is proposed in JVET-C0032. The original sequence of pictures is subsampled, without anti-alias filters, into 4 sub-resolutions sequences.

The first of the sub-resolution sequences can serve as a base layer in a spatial scalable coding scheme. The original sequence resolution can be restored when the 4 sub-resolution sequences are available.

In JVET-C0032 it is proposed to evaluate this scalable coding method in a JEM2 coding environment. For this purpose it is proposed to compare the simulcast transmission of the sequence at its original resolution along with a down-sampled version with the polyphase sequence that provides implicitly access to the two resolutions in a hierarchical fashion.

For that purpose, the technology proponent provided a software package. This package allows the processing of an original video to derive two alternate sequences:

- Sequence A denotes the original video keep untouched

- Sequence B denotes the sequence A down-sampled by a factor of 2 in vertical and horizontal directions. This down-sampling operation is performed using ffmpeg using a bicubic interpolation filter (option -sws_flags", "bicubic").

- Sequence C denotes the sequence A decomposed into 4 polyphase versions, this is performed using an Octave script that directly retain one sample over four in the original video to feed alternatively each of the four sub-resolution sequences.

The flow graph is schematically presented on the figure below.

Figure 1: Derivation of the B and C sequences from the original sequence A.

It is proposed to encode each of the alternative sequences A, B or C with the JEM2 at QP 22, 27, 32 and 37 and perform the following comparison under the current JEM2 common test conditions.

- The simulcast scenario is evaluated by measuring the sum of the bitrates of A and B with the PSNR of A.

- The spatial scalability scenario is evaluated by measuring the bitrate and PSNR after encoding C.

Note that sequence C has 4 times more images than the sequence A. For this experiment it was decided to keep the IntraPeriod identical for A and C, although the random access period is changed increased from A to C. The C intra period is consequently in the range of about 0.25 second.

The GOP structure was kept untouched and identical for the coding of A, B and C configurations. As C is an interleaving of spatial decomposition of A, it is questionable whether the current GOP structure is appropriate for the purpose of offering the best quality while offering the proposed scalability (in C one should be able to extract one frame over four to enable the scalable feature).

Also, sequence C has a specific temporal behaviour as it hasn't a regular frame rate: e.g. frames 0,1,2,3 represent the original sequence of frame 0 and 4,5,6,7 represent frame 1. Consequently the GOP/QP hierarchy should be adapted to this particular framing structure.

# Visual observations

On the left hand side is provided a crop of a B sequence excerpt of BQTerrace. B is a down-sampled version of A using bicubic interpolation filters. On a right hand side is displayed one frame of C (the polyphase decomposition of A). As there is no decimation filter, the content is richer in high frequency content but also exhibits some aliasing components.

Since C serves as the first level of scalability, it should not exhibit clearly visible visual artefacts. Currently this is not guaranteed with the current polyphase decomposition.

The simulations were performed under JVET common test condition to determine the performance of the proposal in a rate distortion sense. Test results are summarized in the table below. Only the RA access test case is addressed here.

The numbers represent the bdrate between the simulcast of A and B (using the bit rate of A+B and PSNR of A) and the proposal C.

A negative number means that simulcast is better than the proposal.

|  | JEM2_SimCast vs TestC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -53.21% | -74.44% | -74.77% |
| Class C | -75.20% | -86.64% | -87.61% |
| Class D | -69.13% | -84.18% | -83.56% |
| Class E |  |  |  |
| Overall (Ref) | -64.02% | -80.74% | -80.91% |
| Class F (optional) | -81.78% | -88.75% | -88.96% |

As it can be noticed, the proposal is not competitive with the simulcast yet.

For information a plot of the rate distorsion behaviour for Kimono is drawn. The performance of this sequence is close (-11 %) to the performance of the simulcast.

A second plot displays the behavior for BQTerrace for which the BDrate is in the range of -80%:

# Conclusion

In this report we experimented the signal path described in contribution JVET-C0032.

The proposed scalability structure has been partly evaluated and its initial performance assessed in BDRate. Currently, it does not seem to be competitive with the simulcast approach but we believe that the coding structure need further study to be conclusive on this aspect.

Generally the following points are raised during this first bench of experiments and we recommend addressing them in a dedicated group:

- The GOP structure for coding the proposed hierarchical stream should be revisited in order to better match the specifics of the polyphase decomposition.

- The QP hierarchy should be carefully considered during coding.

- Intra Period should be made consistent among coding scenarios.

Also the quality of the base layer of the proposed scalable approach should be assessed by a visual inspection to evaluate the impact of aliasing after coding.

The simulcast anchors generation should be revisited, namely the following aspects need to be considered:

- it is believed that the base layer for simulcast should be identical to on of the scalable system

- also different bit rate distribution trade-offs should be considered in this experiment.

# Patent rights declaration(s)

Orange, b<>com, TDF may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).