| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0022 |
| --- | --- |

| Title: | AHG8: A study on compression efficiency of cube projection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo Drive San Diego, CA-92127, USA | Tel: Email: | Minhua.zhou@broadcom.com |
| Source: | Broadcom Limited. |  |  |

_____________________________

# Abstract

This contribution reports compression efficiency difference between a 4x3 native and a compact 3x2 cube projection layout format by using HM16.9.  The experimental results revealed that the 4x3 native cube projection layout consumes on average 1.4% in AI, 1.3% in RA, 1.7% in LD-B and 1.7% in LD-P more bit-rate than the 3x2 compact cube projection layout format. Among four 3x2 compact cube projection layout formats tested, no significant BD-rate difference was found. In the test, the 3x2 compact layouts use 50% memory footprint when compared to the 4x3 native cube projection layout format.

# Introduction

Cube projection [2] uses six cube faces to represent 360 video after projection. A native 4x3 layout format of cube projection is illustrated in Figure 1. Such a layout format has 50% of wasted area, which makes the projected sequences unfriendly to compression in terms of both compression efficiency and memory footprint.

Figure.1.  A native 4x3 layout format of cube projection

3x2 layouts are a better way to represent 360 video in cube projection format. Some examples of compact 3x2 cube projection layout formats are depicted in Figure 2. Many other layout formats for cube projection are possible.

- 3x2 –rot0                                                                        (b) 3x2 –left-rot270

- 3x2 –top-front-bottom-rot270                                    (d) 3x2 –top-front-bottom-rot90

Figure 2. Compact 3x2 layout formats of cube projection

As shown Figure 2, the following four compact 3x2 cube projection layout formats were tested against the native 4x3 cube projection layout format:

- Figure 2 (a) (3x2-rot0), the left and top faces are placed at bottom right potion of image without rotation.

- Figure 2 (b)  (3x2-left-rot270), the left and top faces are placed at bottom right potion of image, but the left cube face is rotated by 270 degree to avoid discontinuous face boundary between the left and top faces.

- Figure 2 (c) (3x2-top-front-bottom-rot270), the top, front and bottom cube faces are placed in the bottom half of the image and rotated by 270 degree.

- Figure 2 (d) (3x2-top-front-bottom-rot90), the top, front and bottom cube faces are placed in the bottom half of the image and rotated by 90 degree.

# Test Conditions

| Source | Sequence | Frame-rate | No. of frames | Resolution (ERP) | Resolution (native 4x3 cube projection) | Resolution (compact 3x2 cube projection) |
| --- | --- | --- | --- | --- | --- | --- |
| Nokia | BearAttack-equirect-left | 30 | 300 | 3840x1920 | 3840x2880 | 2880x1920 |
|  | GT_Sheriff-equirect-left | 30 | 300 | 4320x2160 | 4320x3240 | 3240x2160 |
|  | LRRH-equirect-left | 30 | 300 | 3840x1920 | 3840x2880 | 2880x1920 |
| GoPro | abyss_great_shark_vr | 30 | 300 | 3840x1920 | 3840x2880 | 2880x1920 |
|  | bicyclist_vr | 25 | 250 | 3840x1920 | 3840x2880 | 2880x1920 |
|  | glacier_vr | 24 | 240 | 3840x1920 | 3840x2880 | 2880x1920 |
|  | paramotor_training_vr | 25 | 250 | 3840x1920 | 3840x2880 | 2880x1920 |
|  | skateboarding_vr | 60 | 600 | 4096x2048 | 4096x3072 | 3072x2048 |
|  | timelapse_basejump_vr | 25 | 250 | 3840x1920 | 3840x2880 | 2880x1920 |
|  | timelapse_building_vr | 25 | 250 | 3840x1920 | 3840x2880 | 2880x1920 |
| Etri | Stitched_left_Dancing360 | 60 | 600 | 4096x2048 | 4096x3072 | 3072x2048 |
|  | Stitched_left_Driving360 | 30 | 300 | 4096x2048 | 4096x3072 | 3072x2048 |

Table 1. List of 360 video test sequences

Table 1 lists the test sequences used for simulation. The cube projection sequences are converted from the ERP (equirectangular) sequences by using a 4x4 bi-cubic rendering method.  The native and compact cube projection sequences carry the exactly same content except for the cube faces placed different locations in layout. As shown in Table 1, the compact 3x2 cube projection sequences require only half of memory footprint when compared to the sequences in native 4x3 cube projection layout format.

The HM16.9 Main-10 reference configurations specified in [1] were followed except for that in AI configuration all frames are coded.

# Experimental Results on Cube Projection Layouts

The test results are summarized in Table 2. The native cube projection  layout format consumes on average 1.4% in AI, 1.3% in RA, 1.7% in LD-B and 1.7% in LD-P more bit-rate than the compact cube projection layout format (i.e. 3x2 –top-front-bottom-rot270).   Note that the PSNR values are directly computed in the cube projection domain, and the wasted area in the 4x3 native cube projection is not included in the PSNR calculation.

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Nokia | 0.7% | 4.8% | 6.5% |
| GoPro | 1.5% | 13.5% | 15.8% |
| Etri | 2.0% | 23.5% | 17.6% |
| Overall | 1.4% | 13.0% | 13.8% |
|  | 1.4% | 12.9% | 13.6% |
| Enc Time[%] | 151% |  |  |
| Dec Time[%] | 179% |  |  |
|  | Random Access Main 10 |  |  |
|  | Y | U | V |
| Nokia | 0.8% | 3.7% | 5.4% |
| GoPro | 1.5% | 9.3% | 10.5% |
| Etri | 1.4% | 17.4% | 11.4% |
| Overall | 1.3% | 9.2% | 9.3% |
| Enc Time[%] | 177% |  |  |
| Dec Time[%] | 186% |  |  |
|  | Low delay B Main 10 |  |  |
|  | Y | U | V |
| Nokia | 1.5% | 4.9% | 8.9% |
| GoPro | 2.0% | 14.5% | 20.0% |
| Etri | 1.1% | 14.7% | 22.9% |
| Overall | 1.7% | 12.1% | 17.8% |
| Enc Time[%] | 158% |  |  |
| Dec Time[%] | 168% |  |  |
|  | Low delay P Main 10 |  |  |
|  | Y | U | V |
| Nokia | 1.5% | 5.0% | 9.1% |
| GoPro | 2.0% | 14.6% | 20.0% |
| Etri | 1.1% | 14.9% | 23.0% |
| Overall | 1.7% | 12.3% | 17.8% |
| Enc Time[%] | 169% |  |  |
| Dec Time[%] | 176% |  |  |

Table 2. Summary of test results (tested: 4x3 native layout, anchor: 3x2 –top-front-bottom-rot270)

There can be many compact cube projection layout formats. Experimental results (see Table 3) revealed that the compression efficiency among different compact cube layout formats is comparable as along as the number of discontinuous face boundaries in the composed cube projection sequences is kept at minimum.

|  | 3x2-rot0 |  |  | 3x2-left-rot270 |  |  | 3x2-top-front-bottom-rot90 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main10 |  |  | All Intra Main10 |  |  | All Intra Main10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Nokia | -0.1% | -0.1% | 0.1% | -0.2% | -0.3% | -0.2% | 0.0% | 0.0% | 0.1% |
| GoPro | 0.4% | 1.0% | 0.8% | 0.3% | 0.6% | 0.5% | 0.0% | -0.2% | -0.3% |
| Etri | 0.1% | 1.1% | 0.2% | -0.1% | 0.2% | -0.5% | -0.1% | 1.1% | 0.8% |
| Overall | 0.2% | 0.8% | 0.5% | 0.1% | 0.3% | 0.2% | 0.0% | 0.1% | 0.0% |
|  | 0.2% | 0.8% | 0.5% | 0.1% | 0.3% | 0.2% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 105% |  |  | 105% |  |  | 104% |  |  |
| Dec Time[%] | 106% |  |  | 108% |  |  | 107% |  |  |
|  | Random Access Main 10 |  |  | Random Access Main 10 |  |  | Random Access Main 10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Nokia | 0.0% | 0.0% | 0.1% | -0.1% | -0.3% | -0.1% | 0.0% | 0.1% | 0.2% |
| GoPro | 0.5% | 1.4% | 1.1% | 0.2% | 0.5% | 0.4% | -0.1% | -0.2% | 0.1% |
| Etri | 0.2% | 1.8% | 0.9% | 0.1% | 0.2% | 0.4% | 0.3% | 0.5% | 0.7% |
| Overall | 0.3% | 1.1% | 0.8% | 0.1% | 0.2% | 0.3% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 105% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  | 106% |  |  |
|  | Low delay B Main 10 |  |  | Low delay B Main 10 |  |  | Low delay B Main 10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Nokia | 0.4% | 0.5% | 0.6% | 0.1% | 0.2% | 0.2% | 0.3% | 0.4% | 0.5% |
| GoPro | 0.7% | 1.5% | 1.1% | 0.0% | 0.6% | 0.2% | -0.1% | -0.2% | -0.1% |
| Etri | 0.0% | -1.1% | -2.4% | 0.1% | -1.9% | -2.5% | 0.0% | -1.1% | 0.4% |
| Overall | 0.5% | 0.8% | 0.4% | 0.0% | 0.1% | -0.3% | 0.0% | -0.2% | 0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 97% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  | 98% |  |  |
|  | Low delay P Main 10 |  |  | Low delay P Main 10 |  |  | Low delay P Main 10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Nokia | 0.4% | 0.2% | 0.6% | 0.1% | 0.0% | 0.3% | 0.3% | 0.4% | 0.8% |
| GoPro | 0.6% | 1.6% | 1.3% | -0.1% | 0.5% | 0.3% | -0.1% | -0.2% | 0.1% |
| Etri | -0.1% | -0.8% | -2.4% | 0.0% | -1.7% | -2.1% | -0.1% | -0.7% | 0.4% |
| Overall | 0.5% | 0.9% | 0.5% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  | 101% |  |  |

Table 3. Summary of test results for different 3x2 compact cube projection layouts

# Experimental Results on ERP compared to Cube Projection

Assuming that an end device only takes cube projection as 360 video input format, but the content is provided in ERP format, a test was carried out to determine which of the following two ways is more efficient:

- Compress and transmit 360 video in ERP directly, and have the end device convert the video into cube projection format before rendering.

- Or convert the 360 video in ERP into cube projection format before transmission, then compress and transmit 360 video in cube projection format.

The PSNRs are all computed in cube projection domain. For ERP, both the original and reconstructed ERP sequences are converted into cube projection by using a same 4x4 bi-cubic rendering method and the PSNRs are then calculated in cube projection domain after rendering.

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Nokia | 3.0% | 22.0% | 21.7% |
| GoPro | -3.6% | 8.0% | 6.2% |
| Etri | -3.2% | 9.5% | 9.8% |
| Overall | -1.9% | 11.7% | 10.6% |
|  | -1.8% | 11.6% | 10.5% |
| Enc Time[%] | 134% |  |  |
| Dec Time[%] | 136% |  |  |
|  | Random Access Main 10 |  |  |
|  | Y | U | V |
| Nokia | -2.6% | 14.1% | 14.1% |
| GoPro | 0.4% | 10.3% | 9.7% |
| Etri | -15.7% | 0.1% | -1.1% |
| Overall | -3.0% | 9.5% | 9.0% |
| Enc Time[%] | 137% |  |  |
| Dec Time[%] | 144% |  |  |
|  | Low delay B Main 10 |  |  |
|  | Y | U | V |
| Nokia | -10.0% | 10.5% | 11.0% |
| GoPro | -3.8% | 5.9% | 5.9% |
| Etri | -15.3% | -4.0% | -2.7% |
| Overall | -7.3% | 5.4% | 5.7% |
| Enc Time[%] | 129% |  |  |
| Dec Time[%] | 132% |  |  |
|  | Low delay P Main 10 |  |  |
|  | Y | U | V |
| Nokia | -10.5% | 10.1% | 10.8% |
| GoPro | -5.1% | 6.1% | 6.4% |
| Etri | -16.1% | -3.8% | -3.2% |
| Overall | -8.3% | 5.4% | 5.9% |
| Enc Time[%] | 128% |  |  |
| Dec Time[%] | 123% |  |  |

Table 4. Summary of test results (tested: ERP, anchor: 3x2 –top-front-bottom-rot270)

Summary results in Table 4 revealed that directly transmitting video in ERP seems to be more efficient although it has 25% more active samples when compared to cube projection.

# Conclusions

The experimental results revealed that the compact cube projection layout formats are more efficient than the native cube projection layout formats in terms of compression efficiency and memory footprint.  Among different 3x2 compact cube projection layout formats, the difference in compression efficiency is insignificant.

# References

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, JVET-B1010, February, 2016.

- Cube, https://en.wikipedia.org/wiki/Cube

# Patent rights declaration(s)

Broadcom Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).