| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0023 |
| --- | --- |

| Title: | AHG8: A study on compression efficiency of icosahedral projection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo Drive San Diego, CA-92127, USA | Tel: Email: | Minhua.zhou@broadcom.com |
| Source: | Broadcom Limited. |  |  |

_____________________________

# Abstract

This contribution reports compression efficiency difference between a native and a compact icosahedral projection layout format by using HM16.9.  The experimental results revealed that the native icosahedral projection layout consumes on average 9.5% in AI, 4.9% in RA, 3.0% in LD-B and 3.3% in LD-P more bit-rate than the compact layout format. In the test, the compact ISP format uses 60.6% of memory footprint to store the projected 360 video when compared to the native ISP format.

# Introduction

Icosahedral projection [2] uses 12 vertexes and 20 triangles to represent 360 video after projection. A native layout format of icosahedral projection (ISP) is illustrated in Figure 1. Such a layout format has lots of wasted area, which makes the projected sequences unfriendly to compression in terms of both compression efficiency and memory footprint.

Figure.1.  A native layout format of icosahedral projection

An alternative way of representing 360 video in icosahedral projection is to eliminate the wasted area in the storage. A compact layout format of icosahedral projection is depicted in Figure 2, in which the bottom five triangles are moved to the top, and the right two triangles are warped around during the projection.

Figure 2. A compact layout format of icosahedral projection

This contribution reports the compression efficiency difference between those two layout formats of icosahedral projection. The software used is HM16.9 and the test sequences are those UHD 360 video test sequences submitted to JVET by Nokia, GoPro and Etri.

# Test Conditions

| Source | Sequence | Frame-rate | No. of frames | Resolution (ERP) | Resolution (native ISP) | Resolution (compact ISP) |
| --- | --- | --- | --- | --- | --- | --- |
| Nokia | BearAttack-equirect-left | 30 | 300 | 3840x1920 | 4224x1920 | 3840x1280 |
|  | GT_Sheriff-equirect-left | 30 | 300 | 4320x2160 | 4752x2160 | 4320x1440 |
|  | LRRH-equirect-left | 30 | 300 | 3840x1920 | 4224x1920 | 3840x1280 |
| GoPro | abyss_great_shark_vr | 30 | 300 | 3840x1920 | 4224x1920 | 3840x1280 |
|  | bicyclist_vr | 25 | 250 | 3840x1920 | 4224x1920 | 3840x1280 |
|  | glacier_vr | 24 | 240 | 3840x1920 | 4224x1920 | 3840x1280 |
|  | paramotor_training_vr | 25 | 250 | 3840x1920 | 4224x1920 | 3840x1280 |
|  | skateboarding_vr | 60 | 600 | 4096x2048 | 4504x2048 | 4096x1368 |
|  | timelapse_basejump_vr | 25 | 250 | 3840x1920 | 4224x1920 | 3840x1280 |
|  | timelapse_building_vr | 25 | 250 | 3840x1920 | 4224x1920 | 3840x1280 |
| Etri | Stitched_left_Dancing360 | 60 | 600 | 4096x2048 | 4504x2048 | 4096x1368 |
|  | Stitched_left_Driving360 | 30 | 300 | 4096x2048 | 4504x2048 | 4096x1368 |

Table 1. List of 360 video test sequences

Table 1 lists the test sequences used for simulation. The ISP sequences are converted from the ERP (equirectangular) sequences by using a 4x4 bi-cubic rendering method.  The native ISP and compact ISP sequences carry the exactly same content except for the triangle faces are placed differently in the layout format. As shown in Table 1, the compact ISP layout consumes about 60.6% of memory footprint when compared to the native ISP layout.

The HM16.9 Main-10 reference configurations specified in [1] were followed except for that in AI configuration all frames are coded.

# Test Results

The test results are summarized in Table 2. The native ISP layout format consumes on average 9.5% in AI, 4.9% in RA, 3.0% in LD-B and 3.3% in LD-P more bit-rate than the compact ISP layout forma, respectively.   Note that the PSNR values are directly computed in the ISP domain, and the wasted area in the native ISP projection is not included in the PSNR calculation.

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Nokia | 5.1% | 10.3% | 10.1% |
| GoPro | 10.3% | 24.2% | 25.2% |
| Etri | 13.2% | 23.8% | 25.7% |
| Overall | 9.5% | 20.6% | 21.5% |
|  | 9.5% | 20.2% | 21.2% |
| Enc Time[%] | 138% |  |  |
| Dec Time[%] | 154% |  |  |
|  | Random Access Main 10 |  |  |
|  | Y | U | V |
| Nokia | 3.8% | 9.8% | 9.9% |
| GoPro | 5.6% | 20.2% | 22.0% |
| Etri | 4.0% | 14.8% | 17.4% |
| Overall | 4.9% | 16.7% | 18.2% |
| Enc Time[%] | 143% |  |  |
| Dec Time[%] | 145% |  |  |
|  | Low delay B Main 10 |  |  |
|  | Y | U | V |
| Nokia | 3.0% | 12.4% | 12.3% |
| GoPro | 3.7% | 19.5% | 21.4% |
| Etri | 0.8% | 12.8% | 15.2% |
| Overall | 3.0% | 16.6% | 18.1% |
| Enc Time[%] | 138% |  |  |
| Dec Time[%] | 149% |  |  |
|  | Low delay P Main 10 |  |  |
|  | Y | U | V |
| Nokia | 3.2% | 13.3% | 13.0% |
| GoPro | 4.1% | 19.3% | 20.9% |
| Etri | 0.7% | 12.6% | 15.0% |
| Overall | 3.3% | 16.7% | 18.0% |
| Enc Time[%] | 147% |  |  |
| Dec Time[%] | 156% |  |  |

Table 2. Summary of test results

# Conclusion

The experimental results revealed that the compact ISP layout format is significantly more efficient than the native ISP layout format in terms of both compression efficiency and memory footprint.

# References

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, JVET-B1010, February, 2016.

- Icosahedron, https://en.wikipedia.org/wiki/Icosahedron

# Patent rights declaration(s)

Broadcom Limited does not have current or pending patent rights relating to the technology described in this contribution.