| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0024 |
| --- | --- |

| Title: | AHG8: A study on JEM3.0 compression efficiency on 360 video content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo Drive San Diego, CA-92127, USA | Tel: Email: | Minhua.zhou@broadcom.com |
| Source: | Broadcom Limited. |  |  |

_____________________________

# Abstract

This contribution reports JEM3.0 performance on 12 4K 360 video sequences submitted to the JVET.  Three projection formats were tested, namely, equirectangular projection, compact 3x2 cube projection and compact ISP projection. Compared to HM16.9, the BD-rate reduction is as follows:

- For equirectangular projection, the JEM3.0 outperforms HM16.9 on average by 16.9%   in AI, 27.4% in RA, 21.9% in LD-B and 24.1% in LD-P, respectively.

- For cube projection, the JEM3.0 provides an average BD-rate reduction of 17.5% in AI, 24.8% in RA, 22.5% in LD-B and 25.0% in LD-P, respectively.

- Foe compact ISP, the JEM3.0 BD-rate reductions are 22.4% in AI, 24.5% in RA, 21.8% in LD-B and 25.1% in LD-P, respectively.

The reported BD-rate reduction is less than the BD-rate reduction of class A1 25.5% and class A2 31.93% in the JVET CTC.

# Introduction

This contribution reports JEM3.0 compression efficiency related to HM16.9 on the 360 video content submitted to JVET. The projection formats tested are equirectanular (ERP), 3x2 cube projection layout and compact ISP layout. An example of compact 3x2 cube and ISP projection layout is shown in Figure 1 and Figure 2, respectively.

Figure.1.  A compact 3x2 layout format of cube projection

Figure 2. A compact layout format of icosahedral projection

# Test Conditions

| Source | Sequence | Frame-rate | No. of frames | Resolution (ERP) | Resolution (compact 3x2 cube projection) | Resolution (compact ISP) |
| --- | --- | --- | --- | --- | --- | --- |
| Nokia | BearAttack-equirect-left | 30 | 300 | 3840x1920 | 2880x1920 | 3840x1280 |
|  | GT_Sheriff-equirect-left | 30 | 300 | 4320x2160 | 3240x2160 | 4320x1440 |
|  | LRRH-equirect-left | 30 | 300 | 3840x1920 | 2880x1920 | 3840x1280 |
| GoPro | abyss_great_shark_vr | 30 | 300 | 3840x1920 | 2880x1920 | 3840x1280 |
|  | bicyclist_vr | 25 | 250 | 3840x1920 | 2880x1920 | 3840x1280 |
|  | glacier_vr | 24 | 240 | 3840x1920 | 2880x1920 | 3840x1280 |
|  | paramotor_training_vr | 25 | 250 | 3840x1920 | 2880x1920 | 3840x1280 |
|  | skateboarding_vr | 60 | 600 | 4096x2048 | 3072x2048 | 4096x1368 |
|  | timelapse_basejump_vr | 25 | 250 | 3840x1920 | 2880x1920 | 3840x1280 |
|  | timelapse_building_vr | 25 | 250 | 3840x1920 | 2880x1920 | 3840x1280 |
| Etri | Stitched_left_Dancing360 | 60 | 600 | 4096x2048 | 3072x2048 | 4096x1368 |
|  | Stitched_left_Driving360 | 30 | 300 | 4096x2048 | 3072x2048 | 4096x1368 |

Table 1. List of 360 video test sequences

Table 1 lists the test sequences used for simulation. The cube and ISP projection sequences are converted from the ERP (equirectangular) sequences by using 4x4 bi-cubic rendering.

The reference configurations specified in [1] for both JEM3.0 and HM16.9 were followed.

# Experimental Results

The test results are summarized in Table 2. When compared to HM16.9,

- For equirectangular projection, the JEM3.0 outperforms HM16.9 on average by 16.9%   in AI, 27.4% in RA, 21.9% in LD-B and 24.1% in LD-P, respectively.

- For cube projection, the JEM3.0 provides BD-rate reduction of 17.5% in AI, 24.8% in RA, 22.5% in LD-B and 25.0% in LD-P, respectively.

- For compact ISP projection, the similar gain was observed, namely, 22.4% in AI, 24.5% in RA, 21.8% in LD-B and 25.1% in LD-P, respectively.

Note that PSNRs here are directly computed in ERP or in cube projection domain.

|  | Equirectangular Projection |  |  | 3x2 Cube Projection |  |  | Compact ISP Projection |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main10 |  |  | All Intra Main10 |  |  | All Intra Main10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Nokia | -17.1% | -29.9% | -29.3% | -17.1% | -23.3% | -22.8% | -16.8% | -25.6% | -25.8% |
| GoPro | -15.6% | -25.5% | -16.5% | -16.7% | -23.5% | -17.9% | -25.9% | -35.4% | -30.4% |
| Etri | -21.4% | -35.8% | -31.8% | -20.8% | -34.1% | -30.6% | -19.4% | -32.6% | -29.7% |
| Overall | -16.9% | -28.3% | -22.3% | -17.5% | -25.2% | -21.3% | -22.5% | -32.5% | -29.2% |
|  | -16.9% | -28.2% | -22.3% | -17.5% | -25.2% | -21.8% | -22.4% | -32.7% | -29.6% |
| Enc Time[%] | 207% |  |  | 213% |  |  | 190% |  |  |
| Dec Time[%] | 4421% |  |  | 4891% |  |  | 4560% |  |  |
|  | Random Access Main 10 |  |  | Random Access Main 10 |  |  | Random Access Main 10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Nokia | -22.0% | -38.8% | -38.8% | -22.6% | -30.8% | -30.6% | -23.6% | -37.5% | -38.1% |
| GoPro | -28.6% | -40.5% | -33.8% | -23.7% | -32.9% | -27.6% | -23.5% | -33.4% | -29.1% |
| Etri | -31.2% | -47.8% | -44.9% | -31.9% | -47.6% | -44.6% | -29.4% | -43.5% | -41.8% |
| Overall | -27.4% | -41.3% | -36.9% | -24.8% | -34.8% | -31.2% | -24.5% | -36.1% | -33.5% |
| Enc Time[%] | 689% |  |  | 738% |  |  | 757% |  |  |
| Dec Time[%] | 661% |  |  | 722% |  |  | 681% |  |  |
|  | Low delay B Main 10 |  |  | Low delay B Main 10 |  |  | Low delay B Main 10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Nokia | -18.4% | -28.1% | -26.5% | -19.4% | -26.9% | -24.7% | -19.5% | -27.6% | -26.5% |
| GoPro | -22.1% | -41.0% | -36.1% | -22.9% | -39.1% | -35.5% | -22.0% | -37.3% | -33.5% |
| Etri | -26.3% | -46.8% | -44.9% | -26.0% | -48.3% | -45.1% | -24.7% | -40.2% | -39.8% |
| Overall | -21.9% | -38.8% | -35.2% | -22.5% | -37.5% | -34.4% | -21.8% | -35.4% | -32.8% |
| Enc Time[%] | 474% |  |  | 483% |  |  | 528% |  |  |
| Dec Time[%] | 501% |  |  | 510% |  |  | 540% |  |  |
|  | Low delay P Main 10 |  |  | Low delay P Main 10 |  |  | Low delay P Main 10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Nokia | -21.8% | -33.5% | -32.6% | -21.8% | -31.4% | -29.9% | -21.8% | -32.0% | -31.4% |
| GoPro | -24.2% | -44.2% | -39.2% | -25.5% | -42.1% | -38.7% | -26.2% | -41.2% | -37.7% |
| Etri | -27.3% | -47.9% | -46.8% | -28.2% | -49.2% | -46.9% | -25.9% | -41.2% | -40.1% |
| Overall | -24.1% | -42.2% | -38.8% | -25.0% | -40.6% | -37.9% | -25.1% | -38.9% | -36.5% |
| Enc Time[%] | 300% |  |  | 300% |  |  | 336% |  |  |
| Dec Time[%] | 472% |  |  | 436% |  |  | 484% |  |  |

Table 2. Summary of test results

# Conclusion

The JEM3.0 provides significant compression efficiency improvements on 360 video content over HM16.9, but still falls short of expectation. It is recommended more work be done in this area to further improve the compression efficiency of future video coding.

# References

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, JVET-B1010, February, 2016.

- Cube, https://en.wikipedia.org/wiki/Cube

- Icosahedron, https://en.wikipedia.org/wiki/Icosahedron

# Patent rights declaration(s)

Broadcom Limited does not have current or pending patent rights relating to the technology described in this contribution.