| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0040 |
| --- | --- |

| Title: | AHG8: WS-PSNR for 360 video objective quality evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yule Sun, Ang Lu, Lu Yu Zhejiang University Hangzhou, Zhejiang 310027,China | Tel: Email: | +86 57187953107 sunyule@zju.edu.cn luangss@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

This document describes the derivation and application of Weighted to Spherically uniform PSNR (WS-PSNR) for objective quality evaluation on 360 degree videos. Some comparison test and analysis between WS-PSNR and S-PSNR, CPP-PSNR are discussed. Along with the document, WS-PSNR software is provided.

Overview of WS-PSNR

Since the way we see the VR video in head-mounted display (HMD) is omnidirectional, the average distortion of VR video in all directions can be calculated in spherical surface by projecting omnidirectional views to spherical surface if the viewing plane and spherical surface are considered as continuous domain. So it is more reasonable to compute the PSNR in the spherical surface as objective quality evaluation method without considering the dominating direction of VR video.

However the commonly used representations of VR video are rectangular plane, e.g. equirectangular, cubic. For better evaluating the objective quality of VR video, Weighted to Spherically uniform PSNR (WS-PSNR) is proposed to evaluate the objective quality of VR video in the meaning of sphere.

First, we regard the rectangular plane and spherical surface as continuous domain, and a patch's importance (weight) in rectangular plane is proportion to its area of projection region in spherical space. The mapping of patch from rectangular plane to spherical surface is shown in Figure 1.

Figure 1. mapping from rectangular plane to spherical surface

Second, considering the digital image is consist of sampling points, the discretization of weight should be done. For example, the weight of rectangular format is in (1).

wi,j=Wi,ji=0width-1j=0height-1Wi,j           (1)

where W(i,j) is the scaling factor of area from equirectangular to unit spherical surface in (2). (The resolution of equirectangular format is defined as 2a*a.)

Wi,j=cosj-a2+12*πa              (2)

And for each face in cubic with b*b resolution, the scaling factor is (3+(i+1)2+(j+1)2-i+j*aa2/4)-3/2. Figure 2 illustrates the scaling factor of Equi-rectangular, cubic, OHP and ISP, the pixel value 255 represents the scaling factor equal to 1 and pixel value 0 represents 0.

Figure 2. scaling factor of equirectangular, cubic (3*2), OHP and ISP projection

Finally the WS-PSNR for different formats can be obtained using (3).

WS_PSNR=10logMAXI2WMSE                 (3)

WMSE=i=0width-1j=0height-1(yi,j-y'(i,j))2*w(i,j)         (4)

where MAXI is the maximum possible intensity of the image. MAXI2 is the maximum possible power of a signal and WMSE in (4) is the average power of corrupting noise per unit area in the meaning of sphere. And yi,j and y'(i,j) are intensity in i,j for the reference and test image.

Test and analysis

Test condition:

- Codec: HM 16.11 with RA

- QP setting: 22, 27, 32, 37

- S-PSNR: omnieval [1]

- WS-PSNR：WS-PSNR software

- Format transformation, CPP-PSNR software: 360tools provided by Samsung

- Test sequences: abss_great_shark_vr_30p_3840x1920, glacier_vr_24p_3840x1920 [2]

Comparison of S-PSNR, CPP-PSNR and WS-PSNR

Some experimental results comparing WS-PSNR, S-PSNR and CPP-PSNR [3] are shown in Table 1 and 2. Table 1 depicts quality evaluation on simple artificial distortion patterns while Table 2 reports HEVC coding distortion with different quantization roughness.

In the experiment of table 1, different patterned errors with same strength are added to one tested frame of an equirectangular sequence glacier_vr_24p_3840x1920. Although power of noise is the same for all patterns, which is 1 per pixel, the distortion reported by S-PSNR and CPP-PSNR differs a lot (3.1303dB for S-PSNR, 2.8543dB for CPP-PSNR) under different patterns. The reason is that resampling in S-PSNR and CPP-PSNR need interpolation which filters some high frequent signal and reduces the power of noise. Especially when the noise is in high frequency, S-PSNR or CPP-PSNR losses its ability to reflect power of noise. WS-PSNR shows stable under different error patterns.

The results in table 2 show both S-PSNR, CPP-PSNR and WS-PSNR have ability to distinguish quality variation of video coded with different quantization roughness. However the reported test results of S-PSNR and CPP-PSNR are always larger than that of WS-PSNR. It is noticed that the difference between S-PSNR and WS-PSNR enlarged along with the fineness of quantization. This can be explained by that high frequency distortion is more dominant in case of finer quantization with lower quantization parameter.

Here the objective quality evaluation method is conducted to calculate the distortion caused by compression in the meaning of sphere. WS-PSNR can fairly evaluate the distortion with all frequency. However, the results of S-PSNR and CPP-PSNR will be influenced by noise frequency. In other word, S-PSNR and CPP-PSNR introduces extra format transformation distortion by resampling.

Table 1 Comparison of S-PSNR, CPP-PSNR and WS-PSNR with different error patterns

| Error Pattern | S-PSNR | CPP-PSNR | WS-PSNR |
| --- | --- | --- | --- |
| All pixels +1 | 48.1308 | 48.1563 | 48.1308 |
| All pixels -1 | 48.1308 | 48.1563 | 48.1308 |
| Odd rows -1 Even rows +1 | 51.2611 | 51.0106 | 48.1308 |
| Odd rows +1 Even rows -1 | 51.2611 | 51.0047 | 48.1308 |

Table 2 Comparison of S-PSNR, CPP-PSNR and WS-PSNR with HEVC coding distortion

| 1s | S-PSNR | CPP-PSNR | WS-PSNR | Diff between S-PSNR and WS-PSNR | Diff between CPP-PSNR and WS-PSNR |
| --- | --- | --- | --- | --- | --- |
| abss_great_shark QP=22 | 45.4552 | 44.9707 | 44.5720 | 0.8832 | 0.3987 |
| abss_great_shark QP=27 | 40.8400 | 40.4361 | 40.1876 | 0.6524 | 0.2485 |
| abss_great_shark QP=32 | 36.9142 | 36.6151 | 36.4808 | 0.4334 | 0.1343 |
| abss_great_shark QP=37 | 34.0197 | 33.7858 | 33.6431 | 0.3766 | 0.1427 |
| glacier QP=22 | 44.2271 | 43.7686 | 43.5309 | 0.6962 | 0.2377 |
| glacier QP=27 | 40.8675 | 40.518 | 40.4020 | 0.4655 | 0.116 |
| glacier QP=32 | 37.8981 | 37.6282 | 37.5763 | 0.3218 | 0.0519 |
| glacier QP=37 | 35.0936 | 34.8833 | 34.8656 | 0.228 | 0.0177 |

The application of WS-PSNR

Figure 3. Quality evaluation framework of WS-PSNR

WS-PSNR supports evaluating the quality of 360 degree video in the meaning of sphere. And original and test videos shares the same format and resolution, as show in Figure 3, in which Video X’ means Video X with some distortion but the format of X’ and X is same.

Table 3 shows the format transformation distortion: mapping equirectangular (ERP) (3840*1920) to cubic (3840*2880) and inversely mapping to ERP (3840*1920).

Table 4 shows coding distortion with cubic format (3840*2880).

Table 5 shows format transformation and coding distortion: mapping ERP (3840*1920) to cubic (3840*2880), encode (3840*2880), decode (3840*2880), and inversely mapping to ERP (3840*1920).

Table 3 Format transformation distortion

| great_shark (ERP) | WS-PSNR | glacier | WS-PSNR |
| --- | --- | --- | --- |
|  | 50.3345 |  | 52.7068 |

Table 4 Coding distortion

| great_shark (Cubic) | WS-PSNR | glacier | WS-PSNR |
| --- | --- | --- | --- |
| QP=22 | 45.1079 | QP=22 | 44.6946 |
| QP=27 | 41.3429 | QP=27 | 41.6796 |
| QP=32 | 37.9040 | QP=32 | 38.8796 |
| QP=37 | 35.1127 | QP=37 | 36.2201 |

Table 5 Format transformation and coding distortion

| great_shark (ERP) | WS-PSNR | glacier | WS-PSNR |
| --- | --- | --- | --- |
| QP=22 | 42.8520 | QP=22 | 42.9149 |
| QP=27 | 39.4333 | QP=27 | 40.0405 |
| QP=32 | 36.1220 | QP=32 | 37.3015 |
| QP=37 | 33.4040 | QP=37 | 34.6381 |

Conclusion

WS-PSNR is an objective quality evaluation metric to fairly evaluate the difference (quality) between two video clips in the meaning of spherical domain without resampling. So we propose to use WS-PSNR metric to evaluate the 360/VR sequences. And it supports to evaluate the quality of VR video where original and test videos share the same format and resolution.

References

- M. Yu, H. Lakshman, B. Girod, “A Framework to Evaluate Omnidirectional Video Coding Schemes”, IEEE International Symposium on Mixed and Augmented Reality, 2015.

- JVET-C0050, “Test sequence formats for virtual reality video coding”, International Telecommunication Union, June 2016

- V. Zakharchenko, K.P. Choi, J.H. Park, Video quality metric for spherical panoramic video, Optics and Photonics, SPIE, 9970, 2016

Patent rights declaration(s)

Zhejiang university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).