| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15 Oct – 21 Oct 2016 | Document: JVET-D0045 |
| --- | --- |

| Title: | Reference Line Generation For Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qiang Yao Kei Kawamura Sei Naito 2-1-15 Ohara Fujimino-shi, Saitama Japan | Tel: Email: | +81 49 278 7429 qi-yao@kddi-research.jp |
| Source: | KDDI Corp. (KDDI Research Inc.) |  |  |

_____________________________

- Abstract

In HEVC and JEM3.0, for a coding unit, only the closest reference line in neighboring blocks is adopted for intra prediction. This contribution proposes to generate another reference line using multiple lines the neighboring blocks. The selection of the newly generated reference line or the closest reference line depends on the rate-distortion cost, and a flag is added to signal the result of reference line selection at CU level. Furthermore, initial value of the CABAC model for coding the flag is also modified. The contribution is simulated on top of JEM 3.0 only for Luma component, and the averaged 0.29% BD reduction is achieved in the condition of All-Intra with averaged 75% complexity increase in encoder side. There is no complexity increase in decoder side.

Introduction

Intra prediction is a key issue to achieve high compression gain by considering spatial correlation among samples of a picture. In the intra prediction framework of HEVC, only the closest reference line the neighboring block is adopted based on the assumption that prediction accuracy is inversely proportional to the distance between a prediction sample and the reference line. In JEM3.0, except for the number of angular prediction modes is extended from 35 to 67, the prediction scheme that uses the closest reconstructed sample line in top and left neighboring blocks is directly inherited from HEVC.

However, as the resolution of video is increasing, the maximum block size of each coding unit has been extended from 64x64 in HEVC to 128x128 in JEM 3.0. Regarding larger block size in high resolution video, the closest reference line might not be always the best choice for prediction. Therefore, this contribution proposes to generate another reference line by averaging multiple reference lines in a neighboring block. The selection of the newly generated reference line and the closest reference line depends on the rate-distortion cost, and one flag is added to signal the result of the selection at CU level.

Description of the proposed method

Basically speaking, there are two steps for determining the final prediction mode in the intra prediction of an encoder. A rough check based on sum of transformed coefficients of prediction residual is first conducted to select candidates of prediction modes, and a full check based on rate distortion cost is conducted following the rough check. Finally, the prediction mode that yields to the minimum RD cost is selected and signalled into the bitstream.

In the contribution, a new reference line is generated from multiple lines of the neighboring block just after the rough check, and the procedure is shown in Figure-1. Therefore, there are two types of reference lines, the closest reference line (same as JEM 3.0) and the newly generated reference line, in the full check process. According to the cost of the full check, a new flag, new_refer_line, in CU level, is added to indicate which reference line is finally adopted, and the flag is signalled into the bitstream.

Figure-1 Encoder description for reference line selection in the proposed method

As shown in Figure-2, four lines in the top neighboring block and the left neighboring block are used to generate a top reference line and a left reference line, respectively. During the generation, the coefficients of the four reference lines are selected as [¼, ¼, ¼, ¼]. Furthermore, in order to generate the new reference line along the corresponding prediction angle, a shift modification is also implemented among the four reference lines, and the illustration of the shift modification is shown in Figure-3. To be specific, for DC and planar mode, there is no shift modification. For a prediction mode that is larger than 34 (pred_diag), there is only shift modification for top reference line; for a prediction that is smaller than 34, there is only shift modification for left reference line.

Figure-2 Illustration of new reference line generation using four lines in neighboring blocks.

Figure-3 Illustration of the shift modification for generating a new reference line using four lines in neighboring blocks according to the prediction angle.

Moreover, because the frequency of using the newly generated reference line and the closest reference line is different, the initial value of CABAC model for coding the flag, new_refer_line, is also modified.

Experimental results

The proposed method is implemented on top of JEM 3.0, and our test condition follows the JVET common test condition [1]. The summary of BD-rate and running time are shown in the following table. The averaged 0.29% BD reduction is achieved with 75% averaged complexity increase in encoder side, and there is no complexity increase in decoder side.

Table-1 Simulation Results by the proposed intra prediction method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.03% | 0.04% | 184% | 98% |
| Class A2 | -0.60% | -0.15% | -0.14% | 181% | 98% |
| Class B | -0.29% | -0.06% | -0.12% | 185% | 99% |
| Class C | -0.26% | -0.04% | -0.05% | 161% | 99% |
| Class D | -0.14% | -0.02% | 0.10% | 160% | 98% |
| Class E | -0.26% | 0.01% | -0.10% | 182% | 98% |
| Overall | -0.29% | -0.05% | -0.05% | 175% | 98% |

Conclusion

This contribution proposed to generate another reference line using multiple lines in the neighboring blocks for intra prediction, and the proposed method was simulated on top of JEM 3.0 only for Luma component. Basically, averaged 0.29% BD reduction was achieved in the condition of All-Intra with averaged 75% complexity increase in encoder side, and there is no complexity increase in decoder side. We recommend including the proposed method to the next version of JEM and studying it in the exploration experiments (EEs) or core experiments (CEs).

References

- K. Suehring and X. Li, "JVET common test conditions and software reference configurations", JVET-B1010, San Diego, USA, 20-26 February 2016.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).