| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0060 |
| --- | --- |

| Title: | Aerial photography sequences for video coding standard development |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng, Zisheng Cao, Ferdinand Wolf | Tel: Email: | xiaozhen.zheng@dji.com |
| Source: | DJI |  |  |

_____________________________

- Abstract

This document proposes four clips video test sequences for aerial photography. The sequence content description, RD performance and copyright are also included at the document.

Introduction

Unmanned aerial vehicle (UAV) becomes a popular photography and video capture device recently. Aerial photography shows wide applications in film-making, professional photography, outdoor travelling, industry device inspection, agricultural liquid fertilizers and herbicides and etc. Compared to the traditional scenarios, aerial photography scenario shows some different characterises. For examples, aerial photography sequences contain larger scenes with dramatically zoom-in, zoom-out, pan, rotation, pull-up and pull-down actions.

For the purpose of coding tools’ efficiency evaluation on aerial photography scenes, DJI provides four test sequences, which cover typical aerial photography characterises, captured by DJI’s UAV device. Detailed information is show in the next three sections.

Sequences descriptions

All four test sequences are captured by DJI’s inspire 1 RAW UAV with 12bit cinemaDNG linear raw format. In order to get 4:20 YUV file from cinemaDNG file, the conversion processes are performed by the following two steps:

- DNG to TIF conversion: cinemaDNG files are converted to TIFF files with sRGB color domain and 16bits for every color channel. The format conversion between cinemaDNG and TIFF is performed by Adobe LightRoom Photoshop CC software.

- TIF to YUV conversion: TIFF files are converted to 4:2:0 10bit files by ffmpeg. The command line for this conversion is shown as below:

ffmpeg.exe -i input.tif -vcodec rawvideo -s 3840x2160 -r 30 -pix_fmt yuv420p10le output.yuv

All sequences after the above conversion have the same video parameters which are described in Table 1. The thumbnail of every sequence and its description are shown in Table2.

Table 1: Specification of aerial photography test sequences

| Resolution | Frame count | Frame rate(fps) | Bit Depth | Color Space |
| --- | --- | --- | --- | --- |
| 3840x2160 | 300 | 30 | 10 | 4:2:0 |

Table 2: Description of aerial photography test sequences

| Scenario | Description | Thumbnail | Comments |
| --- | --- | --- | --- |
| iceAerial_3840x2160 | The scenario shows the global aerial scene of an ice land , the rivers and waterfalls at the land. The scenario contains the rotation and pull-down motion. |  |  |
| iceRive_3840x2160 | The scenario contains ice river and mountain with the pull-up motion and the slow rotation from southeast to northwest orientation. |  |  |
| iceRock_3840x2160 | The scenario contains ice river, rock and sunrise scene with back forward flight (zoom-out) and pull-up motions. |  |  |
| iceRock2_3840x2160 | The scenario contains ice river, rock and sunrise scene with forward flight (zoom-in) and pull-up motions. |  |  |

RD performance

RD performance of those test sequences are evaluated by HM16.12 and JEM2.0 platform. All of the test conditions are followed by the description in JVET-C1002. A brief summary of the test conditions are described as follows:

- Encoder: JEM2, HM16.12

- Encoded format: 4:2:0 yuv

- Encoded bitdepth: 10 bit

- Test length: 300 frames

- QP: 5 QP is used (22, 27, 32, 37 and 42).

- HM GOP size: 16, JCTVC-W0062 is enabled.

Considering 5 QPs (22, 27, 32, 37, 42) are used for the evaluation, both lowQP (22, 27, 32, 37) and highQP (27, 32, 37, 42) test results are provided. The RD performance of JEM2.0 compared to HM16.12 over those four sequences is shown below.

Copyright information

The proposed test sequences in this document and all intellectual property rights therein remain the property of SZ DJI Technology Co., Ltd.

The following uses are allowed for the proposed sequences:
·         Can be published in technical papers, played at technology research and development events.
·         Can be used by standards committees.

The following uses are not allowed for the proposed sequences:
·         Do not publish snapshots in product brochures.
·         Do not redistribute video with a commercial product.
·         Do not use in television shows, commercials, or movies.

Patent rights declaration(s)

SZ DJI Technology Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).