| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0073 |
| --- | --- |

| Title: | AHG8: Interpolation filters for 360 video geometry conversion and coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Yan Ye, Bharath Vishwanath 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

At the 3rd JVET meeting, AHG8 on 360 video test conditions was established. One of AHG8’s mandates is to “study the effect on compression when different warping methods are applied to the input 360 video before compression.” When performing projection format conversion, interpolation filters are deployed to generate sample values at fractional sample locations. In this document, we study the effect of using different interpolation filters on conversion only and conversion plus coding processes. Based on our simulation results, it is reported that using Lanczos-3 for luma and Lanczos-2 for chroma could offer a good performance vs. complexity tradeoff.

# Introduction

360 video allows user to view the content on the entire sphere. Spherical content can be represented in different projection formats on a 2D plane, for example, using equirectangular projection (ERP) [1], cubemap projection (CMP) [2], equal-area projection (EAP) [3], and octahedron projection (OHP) [4]. Implementations of these projection formats are available in the PCT360 software tool described in JVET-D0021 [5]. When converting from a source projection format (e.g. ERP) to a destination projection format (e.g. CMP), samples that are located at fractional positions need to be interpolated. The PCT360 software tool supports different interpolation filter options: nearest neighbor, bilinear, bicubic [10], Lanczos-2 and Lanczos-3 [11]. The Lanczos filter family is defined as in Equation (1) with two sinc() functions within a window, where “a” is the window size. Lanczos-2 is defined with “a” being equal to 2, and Lanczos-3 is defined with “a” being equal to 3.

| Lx= sincxsincx/aif-a<x<a0otherwise | (1) |
| --- | --- |

In this contribution, we study the impact on conversion and conversion plus coding performance when different interpolation filters are used. To reduce simulation time, we focus on conversion from ERP to CMP and back to ERP, and we use 4K ERP instead of 8K ERP. Our work flow is depicted in Figure 1.

Figure 1. Work flow used to study interpolation filters

# Experimental Results

We use the six new 4K ERP sequences from GoPro [7] in our tests. The PCT360 tool is used to perform format conversion. For conversion + coding results, we use HM16.12 and the Random Access coding configuration. For quality metrics, we use the SPSNR [8] and AW-SPSNR [9].

Following the work flow in Figure 1, the conversion between ERP and CMP (forward and backward) is performed using different interpolation filters. Our limited test showed that nearest neighbor and bilinear filters do not perform well (which is perhaps not surprising). Therefore, in this contribution, we focus on comparisons among bicubic, Lanczos-2 and Lanczos-3 filters.

## Geometry Conversion without Coding

Table 1, Table 2, Table 3 provide the conversion-only results using bicubic, Lanczos-2 and Lanczos-3 filters, respectively. On average, bicubic and Lanczos-2 filters have similar performance, with the latter being slightly better, offering less than 0.1 dB gain in luma SPSNR and AW-SPSNR. Lanczos-3 performs much better, offering almost 2 dB gain in luma SPSNR and AW-SPSNR.

In terms of complexity, bicubic and Lanczos-2 filters are both 4-tap filters, thus their complexity is comparable.  Thus Lanczos-2 is considered a better choice than bicubic, given it provides slightly better performance. Lanczos-3 is a 6-tap filter and has much higher complexity, at the same time it offers much higher conversion quality.

Table 1. Bicubic filters

|  | SPSNR |  |  | AW-SPSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sequence | Y | U | V | Y |  | U |  | V |
| Basketball | 43.13 | 66.01 | 64.08 | 43.13 | 66.03 |  | 64.10 |  |
| Chairlift | 45.54 | 60.31 | 61.05 | 45.54 | 60.32 |  | 61.06 |  |
| Jam Session | 49.86 | 61.72 | 61.41 | 49.88 | 61.73 |  | 61.44 |  |
| Skate in lot | 44.63 | 61.62 | 61.60 | 44.62 | 61.64 |  | 61.60 |  |
| Skate trick | 49.71 | 62.16 | 62.61 | 49.66 | 62.18 |  | 62.64 |  |
| Train | 49.98 | 61.92 | 62.12 | 50.00 | 61.96 |  | 62.14 |  |
| Average | 47.14 | 62.29 | 62.14 | 47.14 | 62.31 |  | 62.16 |  |

Table 2. Lanczos-2 filters

|  | SPSNR |  |  | AW-SPSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sequence | Y | U | V | Y |  | U |  | V |
| Basketball | 43.19 | 66.05 | 64.11 | 43.19 | 66.07 |  | 64.13 |  |
| Chairlift | 45.61 | 60.35 | 61.08 | 45.61 | 60.36 |  | 61.09 |  |
| Jam Session | 49.94 | 61.77 | 61.45 | 49.96 | 61.77 |  | 61.48 |  |
| Skate in lot | 44.71 | 61.67 | 61.64 | 44.69 | 61.69 |  | 61.65 |  |
| Skate trick | 49.78 | 62.20 | 62.64 | 49.74 | 62.21 |  | 62.67 |  |
| Train | 50.06 | 61.95 | 62.15 | 50.09 | 61.99 |  | 62.17 |  |
| Average | 47.22 | 62.33 | 62.18 | 47.22 | 62.35 |  | 62.20 |  |

Table 3. Lanczos-3 filters

|  | SPSNR |  |  | AW-SPSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sequence | Y | U | V | Y |  | U |  | V |
| Basketball | 44.66 | 66.89 | 64.77 | 44.65 | 66.90 |  | 64.79 |  |
| Chairlift | 47.25 | 61.22 | 61.78 | 47.24 | 61.23 |  | 61.79 |  |
| Jam Session | 52.02 | 62.88 | 62.43 | 52.03 | 62.89 |  | 62.46 |  |
| Skate in lot | 46.35 | 63.02 | 62.61 | 46.33 | 63.03 |  | 62.61 |  |
| Skate trick | 51.79 | 62.94 | 63.22 | 51.77 | 62.96 |  | 63.26 |  |
| Train | 52.62 | 62.63 | 62.81 | 52.66 | 62.67 |  | 62.84 |  |
| Average | 49.11 | 63.26 | 62.94 | 49.11 | 63.28 |  | 62.96 |  |

## Conversion + coding results

Based on the conversion-only results, we further narrow down our filters to Lanczos-2 and Lanczos-3, given these showed better performance in conversion-only tests. Table 4 shows the conversion plus coding results comparing Lanczos-3 with Lanczos-2. As can be observed, compared to using Lanczos-2 filters, Lanczos-3 gives on average 5.4% BD rate reduction for luma, but causes 4.3% and 4.5% BD rate loss for the two chroma components. A closer look at the detailed RD data reveals that the Lanczos-3 filters cause a small increase (~2%-6%) in coded bit rates, a large luma SPSNR and AW-SPSNR gain, and almost no difference in chroma SPSNR and AW-SPSNR. This seems to indicate that Lanczos-3 can better preserve the details of the luma component in the converted content, whereas the chroma components don’t have enough details (high frequency information) and therefore don’t need as complex an interpolation filter. Therefore, we test another setting, where Lanczos-3 filters are used for luma, and Lanczos-2 filters are used for chroma. The conversion plus coding results are provided in Table 5. Compared to using the more complex Lanczos-3 filters for all three color components, using the less complex Lanczos-2 filters achieve effectively the same RD performance. Given such results, we suggest that Lanczos-3 for luma and Lanczos-2 for chroma achieve a good performance vs. complexity tradeoff, and should be used in future 360 video conversion and coding tests.

Table 4. Lanczos-3 filters (all 3 color components) vs. Lanczos-2 filters (all 3 color components)

|  | SPSNR |  |  | AW-SPSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Basketball | -7.8% | 6.9% | 6.8% | -7.8% | 6.9% | 6.8% |
| Chairlift | -5.5% | 4.6% | 4.9% | -5.5% | 4.6% | 4.9% |
| Jam Session | -6.9% | 3.0% | 2.6% | -7.1% | 2.9% | 2.6% |
| Skate in lot | -6.4% | 3.7% | 4.1% | -6.4% | 3.7% | 4.1% |
| Skate trick | -2.2% | 3.7% | 4.4% | -2.2% | 3.7% | 4.4% |
| Train | -3.7% | 4.0% | 4.2% | -3.7% | 4.1% | 4.2% |
| Average | -5.4% | 4.3% | 4.5% | -5.5% | 4.3% | 4.5% |

* Anchor: Lanczos2 for all 3 color components

Table 5. Lanczos-3 for luma + Lanczos-2 for chroma vs. Lanczos-3 filters for all 3 color components

|  | SPSNR |  |  | AW-SPSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Basketball | 0.0% | -0.3% | 0.3% | 0.0% | -0.3% | 0.4% |
| Chairlift | 0.0% | -0.3% | -0.2% | 0.0% | -0.3% | -0.2% |
| Jam Session | 0.1% | -0.3% | -0.6% | 0.2% | -0.3% | -0.6% |
| Skate in lot | 0.1% | 0.1% | 0.4% | 0.1% | 0.1% | 0.3% |
| Skate trick | 0.0% | 1.1% | 0.6% | 0.0% | 1.1% | 0.6% |
| Train | 0.0% | 0.3% | -0.1% | 0.0% | 0.4% | -0.1% |
| Average | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |

* Anchor: Lanczos3 for all 3 color components

# References

- Equirectangular projection, https://en.wikipedia.org/wiki/Equirectangular_projection

- Cubemap, https://en.wikipedia.org/wiki/Cube_mapping

- Lambert cylindrical equal-area projection, https://en.wikipedia.org/wiki/Lambert_cylindrical_equal-area_projection

- Octahedron, https://en.wikipedia.org/wiki/Octahedron

- Y. He, B. Vishwanath, X. Xiu, Y. Ye, “AHG8: InterDigital's projection format conversion tool”, JVET-D0021, Oct. 2016, Chengdu, China.

- B. Vishwanath,Y. He, X. Xiu, Y. Ye, “AHG8: Suggested test methodology for evaluating projection formats for 360 video coding”, JVET-D0074, Oct. 2016, Chengdu, China.

- A. Abbas, B. Adsumilli, “New GoPro Test Sequences for Virtual Reality Video Coding”, JVET-D0026, Oct. 2016, Chengdu, China.

- M. Yu, H. Lakshman, B. Girod, “A Framework to Evaluate Omnidirectional Video Coding Schemes”, IEEE International Symposium on Mixed and Augmented Reality, 2015.

- B. Vishwanath, Y. He, Y. Ye, “AHG8: Area Weighted Spherical PSNR for 360 video quality evaluation”, JVET-D0072, Oct. 2016, Chengdu, China.

- Bicubic, https://en.wikipedia.org/wiki/Cubic_Hermite_spline

- Lanczos resampling, https://en.wikipedia.org/wiki/Lanczos_resampling

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).