| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0075 |
| --- | --- |

| Title: | AHG8: Geometry padding for 360 video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Yan Ye, Philippe Hanhart, Xiaoyu Xiu 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | 858-210-4819 Yuwen.He@interdigital.com Yan.Ye@interdigital.com Philippe.Hanhart@interdigital.com Xiaoyu.Xiu@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

At the 3rd JVET meeting, AHG8 on 360 video test conditions was established. Different projection formats and frame packing formats can be used for 360 video coding. This document investigates a new geometry padding method for 360 video coding. Unlike the conventional padding method for 2D video, which extends samples outside of a picture’s boundary by simply copying (repeating) those from the boundaries, the proposed geometry padding method considers the spherical nature of the 360 video when extending samples outside of a picture’s boundary. Specifically, the proposed geometry padding method considers the projection format when performing padding. The simulation results using equirectangular (ERP) and cubemap (CMP) projection formats for random access configuration are provided. It is reported that, when compared with the conventional 2D padding method in HEVC, geometry padding performs better: for 8K sequences, the proposed geometry padding gives on average Y BD-rate reduction of 0.3% for ERP projection format coding and 0.8% for CMP projection format coding; for 4K sequences, the proposed geometry padding gives on average Y BD-rate reduction of 0.2% for ERP projection format coding and 1.0% for CMP projection format coding, all in terms of the spherical PSNR (SPSNR) metric.

# Introduction

In the existing workflow of 360 spherical video coding, the 360 video is projected onto the 2D plane with a projection format, such as equirectangular (ERP) [1], cubemap (CMP) [2], equal-area (EAP) [3], octahedron (OHP) [4], etc. Currently, all available test sequences are provided in ERP format defined in longitude and latitude. Figure 1 shows an example. Projection format conversion may be performed to convert the ERP format to another format. When the projection format contains more than one face, for example, CMP or OHP, the faces in that projection format is packed to a 2D rectangular picture with a frame packing method. For example, six faces of CMP can be packed with 4x3 configuration shown in Figure 2, or 3x2 configuration shown in Figure 3. Finally the frame packed picture is coded as a 2D conventional video.

Figure 1. ERP picture as input

Figure 2. CMP 4x3 frame packing: red dotted line indicates face boundary

Figure 3. CMP 3x2 frame packing: red dotted line indicates face boundary

Existing video codecs are designed only considering conventional 2D video captured on a plane. When motion compensated prediction uses any samples outside of a reference picture’s boundaries, padding will be performed by simply copying the sample values from the picture boundaries. However, the 360 video encompass video information on a whole sphere, and thus intrinsically has a cyclic property. When considering this cyclic property, the reference pictures of a 360 video no longer have “boundaries”, as the information they contain is all wrapped around a sphere. This cyclic property holds regardless of which projection format is used to represent the 360 video on a 2D plane. The proposed geometry padding considers this very specific 360 video property, and introduces a different padding method in motion compensated prediction for 360 video coding.

# Geometry padding for motion compensated prediction

## Repetitive padding

The repetitive padding method is designed for conventional 2D video coding. In HEVC inter coding, if the temporal reference block is outside of the reference picture boundary, the reference sample at integer position is derived with a repetitive padding method. The sample position outside of the picture is clipped to a nearest sample position within the picture in both horizontal and vertical directions. Figure 4 shows the repetitive padding method. For example, block B0 is partially outside of the reference picture. The part P0 is filled with the top-left sample of part P3. The part P1 is filled line by line with the top line of part P3. The part P2 is filled column by column with the left column of part P3. Figure 5, Figure 6 and Figure 7 shows the extended reference picture padded with repetitive padding for ERP, CMP 4x3 and CMP 3x2, respectively. The reference picture is within the red box, and extended boundary is outside of the red box. From those figures, it clearly shows that the extended boundary does not contain any information from the spherical source, and is thus not appropriate for the coding of 360 video having all-around neighboring information.

Figure 4. Existing padding method

Figure 5. Extended ERP picture with repetitively padded boundary

Figure 6. Extended CMP 4x3 picture with repetitively padded boundary

Figure 7. Extended CMP 4x3 picture with repetitively padded boundary

## Geometry padding

Geometry padding for 360 video coding is designed specifically for video having 3D geometry structure. It pads the sample value by considering the video’s 3D geometry structure.

Geometry padding for ERP is relatively simple because the ERP is defined on the sphere with longitude and latitude. Given a point (u, v) to be padded (outside of the ERP picture), the point (u’, v’) used to derive the padding sample can be calculated as follows.

| if (u<0 or u≥W) and (0≤v <H), then u'=u%W, v'=v | (1) |
| --- | --- |
| Otherwise, if (v<0), then v'=-v-1, u'=u+W2%W; | (2) |
| Otherise, if (v≥H), then v'=2*H-1-v, u'=u+W2%W; | (3) |

where W and H are the width and height of the ERP picture.

Figure 8 shows the geometry padding process for ERP. In the left and right boundary, those samples at A, B, C, D, E and F are padded with the sample at A’, B’, C’, D’, E’ and F’. In the top boundary, those samples at G, H, I and J are padded with the sample at G’, H’, I’ and J’. In the bottom boundary, those samples at K, L, M and N are padded with the sample at K’, L’, M’ and N’. Figure 9 shows an example of the extended ERP picture using geometry padding. The original ERP picture is shown in Figure 1. Compared to the repetitive padding method currently used in HEVC, the proposed geometry padding can provide meaningful samples and improve continuity of neighboring samples for areas outside of the ERP picture boundaries.

Figure 8. Geometry padding for ERP: the padding to be filled in at arrow (A) would be taken along arrow (A’), and so forth. The alphabetic labels show the correspondence.

Figure 9. Extended ERP picture with geometry padding: padded boundary is outside of the red box

If the coded picture is in CMP format, then we pad each face of CMP using geometry padding. Figure 10 is an example for CMP 3x2 frame packing. All six faces are padded with geometry padding. Figure 11 shows how the geometry padding is performed for a given face. Point P is on face F1, but outside of face F1’s boundaries. Therefore, it needs to be padded. Point O is the center of the sphere. R is the left boundary point closest to P and is inside face F1. Point Q is the projection point of point P on face F2 from the center point O. Rather than using sample value at point R to fill the sample value at point P, geometry padding uses the sample value at point Q to fill the sample value at point P. Figure 12 show the extended faces with geometry padding for the CMP 3x2 picture in Figure 3. Compared to the repetitive padding method currently used in HEVC, the proposed geometry padding can provide more meaningful samples for areas outside of the CMP face boundaries.

Figure 10. Extended face pictures of CMP 3x2 with geometry padding: the left part is the CMP 3x2 frame packing configuration, and right part is the extended face picture. The shadow parts are padded with geometry padding.

Figure 11. Geometry padding for CMP. Point P is outside of face F1 and needs to be padded. Point Q is the projected point on face F2 of point P from the center of the sphere point O. The sample value at point P will be filled with the sample value at point Q on face F2.

Figure 12. Example: extended face pictures of CMP 3x2 with geometry padding. Padded boundary is outside of the red box.

# Simulation results

We implemented geometry padding based on the HM-16.12 version of our PCT360 projection conversion software [5]. Two projection formats were tested to compare geometry padding and repetitive padding: (1) ERP format; (2) CMP 3x2 format. We use the random access configuration. Two sets of test material are used:

- 4K ERP test sequences: 7 old sequences [9] and 6 new sequences [8] from GoPro, 1 sequence from Nokia [10], and 2 sequences from ETRI

- 8K ERP test sequences: 6 new sequences from GoPro [8] and 2 sequences from ETRI

Table 1 lists our four test settings using these two sets of test material and two projection formats (ERP and CMP). For quality metrics, we include sphere based PSNR (SPSNR), SPSNR weighted by latitude access frequency (L-SPSNR), both of which proposed in [6], and area weighted spherical PSNR (AW-SPSNR) proposed in [7], which are calculated in ERP domain. There metrics are already implemented in the PCT360 software [5]. BD rate results for Test-1, Test-2, Test-3, and Test-4 are provided in Table 2, Table 3, Table 4, and Table 5, respectively.

The simulation results show that, compared to repetitive padding, on average the proposed geometry padding achieves Y BD-rate reduction of 0.3% for ERP projection format coding and 0.9% for CMP projection format coding on average. Comparing the gains in ERP format with the gains in CMP format, we find the improvement for CMP to be larger. This is because CMP has six faces; therefore the geometry padding method affects more out-of-boundary samples. We find that particularly for those sequences with faster moving objects and objects that move across faces, the proposed geometry padding can be quite effective. For example, it achieves BD rate reductions of 4.3%, 2.7%, 1.9%, and 2.5% for Glacier, Chairlift, Sb_in_lot and Driving, respectively. For information, the BD rate reduction using PSNR as reported by the HM encoder (in CMP domain) is also provided in Table 4. The BD rate saving is larger than those measured by those spherical PSNR in original ERP domain. For example, it achieves BD rate reductions of 4.9%, 3.3%, 2.4%, and 2.8% for Glacier, Chairlift, Sb_in_lot and Driving, respectively.

Based on these findings, it is suggested that JVET establish focused group (such as EE and/or AHG) to explore 360 video coding tools.

Table 1. Test settings

| Test | Source | Coding projection format | Metrics calculation |
| --- | --- | --- | --- |
| Test-1 | 4K ERP | 4K ERP | Calculate metric in 4K ERP domain |
| Test-2 | 8K ERP | 4K ERP | Converted back to 8K ERP to calculate the metrics in 8K ERP domain |
| Test-3 | 4K ERP | 75% CMP 3x2 | Converted back to 4K ERP to calculate the metrics in 4K ERP domain |
| Test-4 | 8K ERP | 4K CMP 3x2 | Converted back to 8K ERP to calculate the metrics in 8K ERP domain |

Table 2. Test1 - geometry padding compared to repetitive padding, coding projection format is in ERP 4K, source is in 4K ERP

|  | SPSNR |  |  | AW-SPSNR |  |  | L-SPSNR |  |  | PSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Shark | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Bicyclist | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | 0.2% |
| Glacier | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% | -0.5% | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% | -0.4% |
| Paramotor | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Skate | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| Basejump | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Building | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Sheriff | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Basketball | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | -0.3% |
| Charilift | -0.7% | -1.0% | -0.7% | -0.7% | -1.0% | -0.8% | -0.7% | -1.0% | -0.8% | -0.7% | -1.1% | -0.9% |
| Sb_in_lot | -0.3% | -0.9% | -1.3% | -0.4% | -0.9% | -1.3% | -0.4% | -0.9% | -1.1% | -0.3% | -0.9% | -1.4% |
| Jam | -0.2% | -0.4% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.4% | -0.3% |
| Train | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% |
| Sb_trick | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Dancing | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Driving | -0.5% | -0.6% | -0.7% | -0.5% | -0.6% | -0.8% | -0.5% | -0.7% | -0.7% | -0.5% | -0.6% | -0.8% |
| Overall | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% |

Table 3. Test2 - geometry padding compared to repetitive padding, coding projection format is in ERP 4K, source is in 8K ERP

|  | S-PSNR |  |  | AW-SPSNR |  |  | L-SPSNR |  |  | PSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Basketball | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.3% |
| Charilift | -0.7% | -1.0% | -0.7% | -0.7% | -1.0% | -0.7% | -0.7% | -0.9% | -0.7% | -0.7% | -1.1% | -0.9% |
| Sb_in_lot | -0.4% | -0.9% | -1.2% | -0.4% | -0.9% | -1.2% | -0.4% | -0.8% | -1.1% | -0.4% | -0.9% | -1.4% |
| Jam | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.4% | -0.3% |
| Train | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% |
| Sb_trick | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Dancing | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Driving | -0.5% | -0.3% | -0.6% | -0.5% | -0.3% | -0.6% | -0.5% | -0.4% | -0.6% | -0.5% | -0.3% | -0.5% |
| Overall | -0.3% | -0.4% | -0.4% | -0.3% | -0.4% | -0.4% | -0.3% | -0.4% | -0.4% | -0.3% | -0.4% | -0.5% |

Table 4. Test3 – geometry padding compared to repetitive padding, coding projection format is 75% CMP 3x2, source is in 4K ERP

|  | SPSNR |  |  | AW-SPSNR |  |  | L-SPSNR |  |  | PSNR |  |  | PSNR(codec only) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Shark | -0.4% | -0.7% | -0.6% | -0.4% | -0.7% | -0.7% | -0.4% | -0.6% | -0.7% | -0.4% | -0.7% | -0.6% | -0.5% | -0.8% | -0.7% |
| Bicyclist | -1.4% | -1.8% | -1.8% | -1.4% | -1.8% | -1.8% | -1.4% | -1.9% | -1.9% | -1.4% | -1.7% | -1.8% | -1.4% | -2.4% | -2.2% |
| Glacier | -4.3% | -4.4% | -3.9% | -4.3% | -4.4% | -3.9% | -4.2% | -4.4% | -4.1% | -4.2% | -4.0% | -3.4% | -4.9% | -5.2% | -4.4% |
| Paramotor | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% |
| Skate | -0.3% | -0.4% | -0.2% | -0.3% | -0.4% | -0.2% | -0.3% | -0.4% | -0.2% | -0.3% | -0.4% | -0.2% | -0.4% | -0.5% | -0.3% |
| Basejump | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| Building | -1.3% | -1.4% | -1.5% | -1.3% | -1.4% | -1.5% | -1.3% | -1.4% | -1.5% | -1.2% | -1.2% | -1.6% | -1.5% | -1.8% | -1.9% |
| Sheriff | 0.3% | 0.2% | 0.4% | 0.2% | 0.2% | 0.3% | 0.3% | 0.1% | 0.3% | 0.3% | 0.4% | 0.3% | 0.2% | 0.3% | 0.4% |
| Basketball | -0.4% | -0.5% | -0.3% | -0.4% | -0.5% | -0.3% | -0.4% | -0.5% | -0.4% | -0.4% | -0.5% | -0.3% | -0.5% | -0.7% | -0.6% |
| Charilift | -2.7% | -3.1% | -2.8% | -2.7% | -3.1% | -2.8% | -2.7% | -3.2% | -2.9% | -2.6% | -2.8% | -2.7% | -3.3% | -4.0% | -3.6% |
| Sb_in_lot | -1.9% | -2.6% | -3.2% | -1.9% | -2.6% | -3.2% | -1.8% | -2.8% | -3.1% | -2.1% | -2.4% | -3.5% | -2.4% | -3.6% | -4.4% |
| Jam | -0.5% | -0.6% | -0.6% | -0.5% | -0.6% | -0.6% | -0.5% | -0.6% | -0.6% | -0.5% | -0.6% | -0.6% | -0.6% | -0.8% | -0.7% |
| Train | -0.6% | -0.8% | -0.8% | -0.6% | -0.8% | -0.8% | -0.6% | -0.8% | -0.8% | -0.6% | -0.8% | -0.7% | -0.7% | -1.1% | -1.0% |
| Sb_trick | -0.2% | -0.4% | -0.4% | -0.3% | -0.4% | -0.4% | -0.2% | -0.5% | -0.4% | -0.3% | -0.4% | -0.5% | -0.3% | -0.7% | -0.7% |
| Dancing | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% |
| Driving | -2.5% | -3.0% | -2.8% | -2.5% | -2.9% | -2.8% | -2.5% | -3.0% | -2.8% | -1.9% | -2.9% | -2.7% | -2.8% | -3.6% | -3.3% |
| Overall | -1.0% | -1.2% | -1.2% | -1.1% | -1.2% | -1.2% | -1.0% | -1.3% | -1.2% | -1.0% | -1.2% | -1.2% | -1.2% | -1.6% | -1.5% |

Table 5. Test4 – geometry padding compared to repetitive padding, coding projection format is 4K CMP 3x2, source is in 8K ERP

|  | SPSNR |  |  | AW-SPSNR |  |  | L-SPSNR |  |  | PSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Basketball | -0.3% | -0.1% | -0.2% | -0.3% | 0.0% | -0.2% | -0.3% | -0.4% | -0.3% | -0.3% | 0.1% | 0.6% |
| Charilift | -2.4% | -2.6% | -2.2% | -2.4% | -2.6% | -2.1% | -2.4% | -2.8% | -2.2% | -2.4% | -2.2% | -2.0% |
| Sb_in_lot | -1.6% | -2.3% | -3.1% | -1.6% | -2.2% | -3.1% | -1.6% | -2.3% | -3.0% | -1.7% | -2.4% | -3.4% |
| Jam | 0.1% | -0.4% | -0.2% | 0.1% | -0.4% | -0.3% | 0.0% | -0.4% | -0.4% | 0.1% | -0.4% | 0.5% |
| Train | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% | -0.4% | -0.3% | -0.2% | -0.3% | -0.5% | -0.4% |
| Sb_trick | 0.0% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | 0.0% | -0.4% |
| Dancing | 0.3% | 0.2% | 0.2% | 0.3% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.3% | 0.2% | 0.6% |
| Driving | -1.9% | -3.4% | -3.8% | -1.9% | -3.3% | -3.7% | -2.0% | -2.5% | -2.9% | -1.3% | -5.7% | -4.6% |
| Overall | -0.8% | -1.1% | -1.2% | -0.8% | -1.1% | -1.2% | -0.8% | -1.1% | -1.1% | -0.7% | -1.3% | -1.1% |

# References

- Equirectangular projection, https://en.wikipedia.org/wiki/Equirectangular_projection

- Cubemap, https://en.wikipedia.org/wiki/Cube_mapping

- Lambert cylindrical equal-area projection, https://en.wikipedia.org/wiki/Lambert_cylindrical_equal-area_projection

- Octahedron, https://en.wikipedia.org/wiki/Octahedron

- Y. He, B. Vishwanath, X. Xiu, Y. Ye, “AHG8: InterDigital’s projection format conversion tool”, JVET-D0021, Oct. 2016, Chengdu, China.

- M. Yu, H. Lakshman, B. Girod, “A Framework to Evaluate Omnidirectional Video Coding Schemes”, IEEE International Symposium on Mixed and Augmented Reality, 2015.

- B. Vishwanath, Y. He, Y. Ye, “AHG8: Area Weighted Spherical PSNR for 360 video quality evaluation”, JVET-D0072, Oct. 2016, Chengdu, China.

- A. Abbas, B. Adsumilli, “New GoPro Test Sequences for Virtual Reality Video Coding”, JVET-D0026, Oct. 2016, Chengdu, China.

- A. Abbas, “GoPro test sequences for Virtual Reality video coding”, JVET-C0021, May 2016, Geneva, Switzerland.

- J. Ridge, M. M. Hannuksela, E. B. Aksu, J. Lainema, A. Aminlou, “Nokia test sequences for virtual reality video coding”, JVET-C0064, May 2016, Geneva, Switzerland.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).