| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0092 |
| --- | --- |

| Title: | AHG8: Reference samples derivation using geometry padding for intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | +1 858 210 4841 Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

At the 3rd JVET meeting, AHG8 on 360 video test conditions was established. Different projection formats and frame packing formats can be used for 360 video coding. This document investigates a new reference sample derivation method for intra prediction in 360 video coding. Unlike the conventional reference sample derivation method for intra coding in HEVC, which uses the samples located directly above and on the left of the current block, the proposed reference samples derivation method relies on the geometry padding method proposed in JVET-D0075 when deriving reference samples located outside the current face to which the block belongs. The simulation results using cubemap (CMP) projection formats for all intra configuration are provided. It is reported that, when compared with the conventional reference sample derivation method for intra coding, the proposed method gives on average Y BD-rate reduction of 0.3% in terms of the spherical PSNR (SPSNR) metric.

# Introduction

In the existing workflow of 360 spherical video coding, the 360 video is projected onto the 2D plane with a projection format, such as equirectangular (ERP) [1], cubemap (CMP) [2], equal-area (EAP) [3], octahedron (OHP) [4], etc. Currently, all available test sequences are provided in ERP format defined in longitude and latitude. Figure 1 shows an example. Projection format conversion may be performed to convert the ERP format to another format. When the projection format contains more than one face, for example, CMP or OHP, the faces in that projection format is packed to a 2D rectangular picture with a frame packing method. For example, six faces of CMP can be packed with 4x3 configuration shown in Figure 2, or 3x2 configuration shown in Figure 3. Finally the frame packed picture is coded as a 2D conventional video.

Figure 1. ERP picture as input

Figure 2. CMP 4x3 frame packing: red dotted line indicates face boundary

Figure 3. CMP 3x2 frame packing: red dotted line indicates face boundary

Existing video codecs are designed only considering conventional 2D video captured on a plane. When considering projection geometries composed of several faces, continuity between faces is broken because each face is defined on a different place. Moreover, discontinuities across different faces can be increased in the frame packed picture, as illustrated in Figure 3 between the first and second rows. Therefore, for intra coding, special care must be applied when using reference samples located outside the face that current block belongs to. The proposed reference samples derivation method relies on the geometry padding method proposed in [5] to derive reference samples located outside the face that current block belongs to.

# Reference samples derivation method for intra prediction

For motion compensated prediction for 360 video coding, geometry padding is proposed in [5] to replace the conventional repetitive padding for 2D video coding. For intra coding, the same geometry padding method can be applied to derive a reference sample when its position is located outside the face that current block belongs to. Compared to the conventional reference sample derivation method in HEVC, using the geometry padding proposed in [5] to derive reference samples for intra prediction can provide more meaningful sample values for reference sample locations outside of the face boundaries.

For example, let us consider the CMP format and 3x2 frame packing, as illustrated in Figure 4. Note that for better visualization, face index rotation is aligned with face rotation. The face arrangement and raster scan processing order are the two factors determining which blocks can take advantage of the proposed method. In our example, if a block is located at the face boundary in one of the hatched areas (see Figure 4b), then geometry padding may be used for the sample locations that are outside the face that current block belongs to. Note that unlike geometry padding for inter prediction [5], where geometry padding is applied on previously coded frames where all samples are available to the decoder, in our case, geometry padding is only applied to reference samples where all samples used for padding have already been coded. In the example depicted in Figure 4, the left reference samples for block “A” may be derived from face #0 (see Figure 5a) instead of using mid grey samples (as the block is located at the picture boundary). Additionally, the above reference samples for block “B” should be derived from face #5 (see Figure 5b) instead of using samples located directly above the block in the frame packed picture, which are located in face #0. Similarly, the above reference samples for block “C” should be derived from samples located at the top of face #5 instead of samples located at the bottom (see Figure 5b). Even for blocks located at the boundary of two faces that are aligned, such as block “D” in Figure 4b, a phase shift occurs between the reference samples that are derived using the conventional method and the reference samples in the extended face.

| (a) | (b) |
| --- | --- |

Figure 4. CMP: 3D representation (a) and 3x2 frame packing configuration (b).

| (a) | (b) |
| --- | --- |

Figure 5. The reference samples in the hatched areas are derived using geometry padding, whereas the reference samples in the non-hatched areas are derived using the conventional method.

# Simulation results

We implemented the proposed reference sample derivation using geometry padding based on the JEM3.0 version of our PCT360 projection conversion software [6]. The CMP 3x2 format was tested to compare coding performance between the proposed method and the conventional method. We use the all intra configuration. Two sets of test material are used:

- 4K ERP test sequences: 7 old sequences from GoPro [10] and 1 sequence from Nokia [11]

- 8K ERP test sequences: 6 new sequences from GoPro [9] and 2 sequences from ETRI

Table 1 lists our two test settings using these two sets of test material. For the Sheriff sequence, the coding face size was set to 1088 pixels instead of 1080 pixels, i.e., 76.1% resolution when compared to 4K ERP instead of 75%, such that the face size is a multiple of 64. Note that for the other 4K ERP sequences, each 75% CMP version also has a coding face size that is a multiple of 64. For encoding, the CTU size was changed from the default value, i.e., 128, to 64 for Test-1 to ensure that no CTU would overlap over two or more faces. For the 8K ERP test sequences, the coding face size was set to 1152 pixels. This value corresponds to 23.7% resolution when compared to 8K ERP, which yields the closest resolution to 25% using a coding face size value that is a multiple of 64 (and also a multiple of 128 in this case). For Test-2, the default CTU size was used for encoding.

For quality metrics, we include sphere based PSNR (SPSNR), SPSNR weighted by latitude access frequency (L-SPSNR), both of which proposed in [7], and area weighted spherical PSNR (AW-SPSNR) proposed in [8]. There metrics are already implemented in the PCT360 software [6]. BD rate results for Test-1 and Test-2 are provided in Table 2 and Table 3, respectively.

The simulation results show that, compared to the conventional reference sample derivation method for intra coding, the proposed method using geometry padding achieves Y BD-rate reduction of about 0.3% on average. Results are quite consistent over the different metrics and sets of test sequences, and show that there is consistent luma performance gain for all sequences, and chroma performance again for all sequences except Basejump.

Based on these findings, it is suggested that JVET establish focused group (such as EE and/or AHG) to explore 360 video coding tools.

Table 1. Test settings

| Test | Source | Coding projection format | Metrics calculation |
| --- | --- | --- | --- |
| Test-1 | 4K ERP | 75% CMP 3x2 | Converted back to 4K ERP to calculate the metrics in 4K ERP domain |
| Test-2 | 8K ERP | 4K CMP 3x2 | Converted back to 8K ERP to calculate the metrics in 8K ERP domain |

Table 2. Test1 – reference samples derivation using geometry padding compared to conventional reference samples derivation method, coding projection format is 75% CMP 3x2, source is in 4K ERP

|  | SPSNR |  |  | AW-SPSNR |  |  | L-SPSNR |  |  | PSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Shark | -0.30% | -0.23% | -0.32% | -0.33% | -0.25% | -0.32% | -0.30% | -0.24% | -0.35% | -0.37% | -0.30% | -0.36% |
| Bicyclist | -0.20% | -0.31% | -0.41% | -0.22% | -0.29% | -0.42% | -0.19% | -0.31% | -0.41% | -0.19% | -0.15% | -0.48% |
| Glacier | -0.24% | -0.23% | -0.52% | -0.23% | -0.20% | -0.53% | -0.24% | -0.27% | -0.55% | -0.16% | -0.26% | -0.17% |
| Paramotor | -0.21% | -0.04% | -0.37% | -0.21% | -0.01% | -0.39% | -0.19% | -0.10% | -0.37% | -0.17% | -0.08% | -0.34% |
| Skate | -0.52% | -0.38% | -0.58% | -0.52% | -0.37% | -0.58% | -0.53% | -0.45% | -0.46% | -0.50% | -0.38% | -0.73% |
| Basejump | -0.33% | 0.24% | -0.11% | -0.29% | 0.27% | -0.09% | -0.35% | 0.05% | -0.13% | -0.22% | 0.19% | -0.14% |
| Building | -0.47% | -0.48% | -0.49% | -0.41% | -0.50% | -0.42% | -0.46% | -0.46% | -0.43% | -0.34% | -0.56% | -0.72% |
| Sheriff | -0.14% | -0.02% | -0.03% | -0.12% | -0.04% | 0.02% | -0.15% | -0.13% | 0.12% | -0.13% | 0.11% | -0.11% |
| Overall | -0.30% | -0.18% | -0.35% | -0.29% | -0.17% | -0.34% | -0.30% | -0.24% | -0.32% | -0.26% | -0.18% | -0.38% |

Table 3. Test2 – reference samples derivation using geometry padding compared to conventional reference samples derivation method, coding projection format is 4K CMP 3x2, source is in 8K ERP

|  | SPSNR |  |  | AW-SPSNR |  |  | L-SPSNR |  |  | PSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Basketball | -0.07% | -0.37% | -0.76% | -0.11% | -0.36% | -0.71% | -0.10% | -0.38% | -0.67% | -0.10% | -0.68% | 0.28% |
| Charilift | -0.28% | -0.27% | -0.36% | -0.27% | -0.26% | -0.35% | -0.29% | -0.31% | -0.46% | -0.29% | -0.12% | -0.22% |
| Sb_in_lot | -0.20% | -0.11% | -0.11% | -0.24% | -0.10% | -0.10% | -0.21% | -0.06% | -0.19% | -0.23% | -0.22% | -0.08% |
| Jam | -0.41% | -0.09% | -0.20% | -0.35% | -0.08% | -0.20% | -0.40% | -0.16% | -0.18% | -0.36% | -0.02% | -0.26% |
| Train | -0.36% | -0.59% | -0.46% | -0.37% | -0.62% | -0.46% | -0.37% | -0.58% | -0.47% | -0.37% | -0.65% | -0.58% |
| Sb_trick | -0.21% | -0.29% | -0.24% | -0.22% | -0.30% | -0.24% | -0.21% | -0.29% | -0.24% | -0.22% | -0.31% | -0.16% |
| Dancing | -0.32% | -0.45% | -0.17% | -0.29% | -0.46% | -0.15% | -0.34% | -0.42% | -0.20% | -0.28% | -0.59% | -0.05% |
| Driving | -0.32% | -0.09% | -0.96% | -0.30% | -0.07% | -0.94% | -0.32% | -0.18% | -0.58% | 0.05% | 0.01% | -1.22% |
| Overall | -0.27% | -0.28% | -0.41% | -0.27% | -0.28% | -0.39% | -0.28% | -0.30% | -0.37% | -0.22% | -0.32% | -0.29% |

# References

- Equirectangular projection, https://en.wikipedia.org/wiki/Equirectangular_projection

- Cubemap, https://en.wikipedia.org/wiki/Cube_mapping

- Lambert cylindrical equal-area projection, https://en.wikipedia.org/wiki/Lambert_cylindrical_equal-area_projection

- Octahedron, https://en.wikipedia.org/wiki/Octahedron

- Y. He, Y. Ye, P. Hanhart, X. Xiu, “AHG8: Geometry padding for 360 video coding”, JVET-D0075, Oct. 2016, Chengdu, China.

- Y. He, B. Vishwanath, X. Xiu, Y. Ye, “AHG8: InterDigital’s projection format conversion tool”, JVET-D0021, Oct. 2016, Chengdu, China.

- M. Yu, H. Lakshman, B. Girod, “A Framework to Evaluate Omnidirectional Video Coding Schemes”, IEEE International Symposium on Mixed and Augmented Reality, 2015.

- B. Vishwanath, Y. He, Y. Ye, “AHG8: Area Weighted Spherical PSNR for 360 video quality evaluation”, JVET-D0072, Oct. 2016, Chengdu, China.

- A. Abbas, B. Adsumilli, “New GoPro Test Sequences for Virtual Reality Video Coding”, JVET-D0026, Oct. 2016, Chengdu, China.

- A. Abbas, “GoPro test sequences for Virtual Reality video coding”, JVET-C0021, May 2016, Geneva, Switzerland.

- J. Ridge, M. M. Hannuksela, E. B. Aksu, J. Lainema, A. Aminlou, “Nokia test sequences for virtual reality video coding”, JVET-C0064, May 2016, Geneva, Switzerland.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).