| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0093 |
| --- | --- |

| Title: | AHG8: High level syntax extensions for signaling of 360-degree video information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | +1 858 210 4841 Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

This document proposes high level syntax extensions for signaling of 360-degree video information. In particular, the proposed syntax elements are used to specify the projection geometry and frame packing parameters. It is asserted that these syntax elements are essential to enabling low level coding tools to achieve more efficient compression of 360-degree video.

# Introduction

The 360-degree video data need to be projected onto a 2D plane to encode the information using conventional 2D planar video coding. We note that there are many geometric projections that can be used to represent 360-degree data, including equirectangular (ERP), equal-area (EAP), cubemap (CMP), octahedron (OHP), icosahedron (ISP), and so on. Further, for some of these projection formats, such as CMP, OHP, and ISP, the projected data consist of multiple faces, which can be packed in different configurations.

Low level coding tools are proposed in [1] and [2] to enable better compression efficiency for 360-degree video sequences. However, these low level coding tools rely on information regarding the geometry and frame packing parameters. Correspondingly, the geometry and frame packing parameters should be known by the decoder to unpack the data and project it back from the 2D space to the 3D space. For example, the CMP format has six square faces, and can be represented using different arrangements, such as 3x2, 4x3, 1x6, or 6x1, with different face orders or different face rotations. And the same is true for OHP and ISP formats.

This document suggests to signal the geometry and frame packing parameters in the bitstream by means of additional high level syntax elements. In particular, the projection geometry type can be specified, including different parameters for the geometry faces to locate them on the 2D planar video. The 360-video parameters can be signaled at different levels. In particular, we propose to signal a set of projection format parameters in the VPS that can be shared among different layers and/or sequences and/or picture. At SPS level, the projection format parameter index identifies the geometric parameters used for the current video sequence.

Another aspect of this document is to enable encoding of the different geometry faces with different quality factors. For example, in the CMP format, the front, back, left, and right faces may be coded with higher quality, whereas the top and bottom faces may be coded with lower quality. This is because the viewers are more likely to watch areas around the horizon than near the poles. In this way, the 360-video can be coded more efficiently. To achieve this, the proposed syntax allows delta QP values to be specified for each face.

# Proposed syntax elements

## Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_360_extension_flag | u(1) |
| if( vps_360_extension_flag ) |  |
| vps_360_extension( ) |  |
| … |  |
| } |  |

| vps_360_extension ( ) { | Descriptor |
| --- | --- |
| vps_num_360_formats_minus1 | ue(v) |
| for( i = 0; i <= vps_num_360_formats_minus1; i++ ) |  |
| 360_format() |  |
| if( vps_num_360_formats_minus1 > 0 ) |  |
| 360_format_idx_present_flag | u(1) |
| if( 360_format_idx_present_flag ) |  |
| for( i = 0; i <= MaxLayersMinus1; i++ ) |  |
| vps_360_format_idx[ i ] | ue(v) |
| } |  |

| 360_format( ) { | Descriptor |
| --- | --- |
| projection_geometry | ue(v) |
| compact_representation_enabled_flag | u(1) |
| loop_filter_across_faces_enabled_flag | u(1) |
| num_face_rows_minus1 | ue(v) |
| num_face_columns_minus1 | ue(v) |
| if( (num_face_rows_minus1 + 1) * (num_face_columns_minus1 + 1) > 1 ) |  |
| face_qp_offset_enabled_flag | u(1) |
| for( i = 0; i <= num_face_rows_minus1; i++ ) { |  |
| for( j = 0; j <= num_face_columns_minus1; j++ ) { |  |
| face_idx[ i ][ j ] | ue(v) |
| face_rotation_idc[ i ][ j ] | ue(v) |
| if( face_qp_offset_enabled_flag ) |  |
| face_qp_offset[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |

## Video parameter set RBSP semantics

vps_360_extension_flag: specifies whether the video is a 360-degree video, in which case specific parameters and tools for efficient representation and compression of 360-video may be used. When not present, the value of vps_360_extension_flag can be inferred to be equal to 0.

vps_num_360_formats_minus1: specifies the number (minus 1) of projection formats used in the different sequences and/or layers. When not present, the value of vps_num_360_formats_minus1 can be inferred to be equal to 0, i.e., only one projection format is used.

360_format_idx_present_flag: specifies whether the syntax elements vps_360_format_idx[ i ] are present or not. When not present, the value of 360_format_idx_present_flag can be inferred to be equal to 0.

vps_360_format_idx[ i ]: specifies the index, into the list of 360_format( ) syntax structures in the VPS, of the 360_format( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. When not present, the value of vps_rep_format_idx[ i ] can be inferred to be equal to Min( i, vps_num_rep_formats_minus1 ).

projection_geometry: specifies the mapping index in Table 1 of the used projection geometry.

Table 1. Projection geometry indices

| Index | Projection geometry |
| --- | --- |
| 0 | Equirectangular |
| 1 | Cubemap |
| 2 | Equal-area |
| 3 | Octahedron |
| 4 | Icosahedron |

compact_representation_enabled_flag: specifies whether the samples or blocks used for padding the frame packed image into a rectangular picture are skipped by the encoder. When not present, the value of compact_representation_enabled_flag can be inferred to be equal to 0.

loop_filter_across_faces_enabled_flag: specifies whether in-loop filtering operations may be performed across face boundaries. When not present, the value of loop_filter_across_faces_enabled_flag can be inferred to be equal to 1.

num_face_rows_minus1: specifies the number of face rows (minus 1) in the frame packed picture. When not present, the value of num_face_rows_minus1 can be inferred to be equal to 0.

num_face_columns_minus1: specifies the number of face columns (minus 1) in the frame packed picture. When not present, the value of num_face_columns_minus1 can be inferred to be equal to 0.

face_qp_offset_enabled_flag: specifies whether different QPs are used for the different faces. When not present, the value of face_qp_deltaoffset_enabled_flag can be inferred to be equal to 0.

face_idx[ i ][ j ]: specifies the index of the face located at the i-th row and j-th column in the frame packed picture. For simple geometries with only a single face, such as equirectangular or equal-area, the only face is face #0. For other geometries, a default numbering and positioning of the faces can be used. For example, the following face numbering can be used for the cubemap projection: 0: PX, 1: NX, 2: PY, 3: NY, 4: PZ, 5: NZ (see Figure 1).

Figure 1. Cubemap physical layout.

face_rotation_idc[ i ][ j ]: specifies the mapping index in Table 2 of the rotation between the face coordinate system and the picture coordinate system of the face located at the i-th row and j-th column in the frame packed picture. When not present, the value of face_rotation_idc[ i ][ j ] can be inferred to be equal to 0.

Table 2. Rotation indices

| Index | Rotation in counter-clockwise |
| --- | --- |
| 0 | 0 |
| 1 | 90 |
| 2 | 180 |
| 3 | 270 |

face_qp_offset[ i ][ j ]: specifies the difference to be added to the sequence level QP when determining the QP value of the face located at the i-th row and j-th column in the frame packed picture.

The width and height of all faces in the frame packed picture can be inferred based on the projection geometry. For example, with the cubemap projection, the width in luma samples of all faces in the frame packed picture can be inferred to be equal to pic_width_in_luma_samples / (num_face_columns_minus1 + 1), while the height in luma samples of all faces in the frame packed picture can be inferred to be equal to pic_height_in_luma_samples / (num_face_rows_minus1 + 1). Note that the width and height in luma samples of all faces in the frame packed picture shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

Considering the frame packed picture as a face grid, these parameters can be used for a very flexible signaling of the geometry format. For projection geometry resulting in a single face, such as ERP or EAP, the parameters num_face_rows_minus1, num_face_columns_minus1, face_idx, and face_rotation can be simply inferred from the geometry and the picture size. However, for other geometries, such as CMP, OHP, and ISP, it is necessary to specify these parameters as the faces may be arranged in different ways or have different sizes. For example, as illustrated in Figure 2, the same cubemap projection can be frame packed in different ways, such as a 3x4 grid (a) or a 2x3 grid (b). In the case of the 3x4 grid, face_idx can be set to a value higher than the actual face number, which can be inferred from the geometry, to indicate positions in the grid that do not contain actual faces. For example, the parameters can be set as follows

| projection_geometry = 1 | // cubemap |
| --- | --- |
| face_idx[0][0] = 2 | // face #2 |
| face_idx[0][1] = 6 | // invalid face |
| face_idx[0][2] = 7 | // invalid face |
| face_idx[0][3] = 8 | // invalid face |
| face_idx[1][0] = 1 | // face #1 |
| face_idx[1][1] = 4 | // face #4 |
| face_idx[1][2] = 0 | // face #0 |
| face_idx[1][3] = 5 | // face #5 |
| face_idx[2][0] = 3 | // face #3 |
| face_idx[2][1] = 9 | // invalid face |
| face_idx[2][2] = 10 | // invalid face |
| face_idx[2][3] = 11 | // invalid face |

Faces can be arranged with different orientations. For example, as illustrated in Figure 2 for the cubemap projection, faces “2”, “1”, and “3” are rotated by 90-degree counter-clockwise in the 2x3 grid (b) when compared to the 3x4 grid (a). The face_rotation_idc parameter can be used to specify the rotation between the face coordinate system and the frame packed picture coordinate system.

| (a) | (b) |
| --- | --- |

Figure 2. Cubemap projection represented in the 4x3 (a) and 3x2 (b) frame packing formats.

Finally, the face_qp_delta parameter can be used to specify if a particular face is encoded at higher or lower quality. Similar results can be obtained for example by adapting the quality at a slice or Coding Unit level. However, a slice could cover several faces and a face will most likely contain several Coding Units, so it is more efficient and more intuitive to signal the quality difference for each face directly.

## General sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_360_extension_flag | u(1) |
| if( sps_360_extension_flag ) |  |
| sps_360_extension( ) |  |
| … |  |
| } |  |

| sps_360_extension( ) { | Descriptor |
| --- | --- |
| sps_360_format_idx | ue(v) |
| } |  |

## General sequence parameter set RBSP semantics

sps_360_extension_flag: specifies whether the sequence is a 360- video, in which case specific parameters and tools for efficient compression of 360-video may be used.

sps_360_format_idx: specifies the index into the list of 360_format( ) syntax structures in the VPS of the 360_format( ) syntax structure that is used in the sequence. The value of sps_360_format_idx shall be in the range of 0 to vps_num_360_formats_minus1, inclusive. When not present, the value of pps_360_format_idx can be inferred to be equal to 0.

# References

- Y. He, Y. Ye, P. Hanhart, X. Xiu, “AHG8: Geometry padding for 360 video coding”, JVET-D0075, Oct. 2016, Chengdu, China.

- P. Hanhart, Y. He, Y. Ye, “AHG8: Reference samples derivation using geometry padding”, JVET-D0092, Oct. 2016, Chengdu, China.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).