| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0095 |
| --- | --- |

| Title: | AHG5: Fast QTBT encoding configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshiya Yamamoto Tomohiro Ikai | Email: | yamamoto.yoshiya@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation. |  |  |

_____________________________

- Abstract

This contribution proposes a fast QTBT encoding configuration, which utilizes different MaxBTDepth settings depending on Temporal ID layer. Simulation results show around xx % encoding speed up with 0.xx % BD-rate loss, which claims to show better encoding-time performance balance compared with constant MaxBTDepth settings. i.e. MaxBTDepth is 2 or 3.

Introduction

At the last meeting, many participants expressed concerns on massive JEM encoding time as reported in JVET-C0105 [2] and an AHG on fast encoding configuration has been established.

To realize this move, we has investigated the QTBT partitioning and found that some QTBT partitioning is related to Temporal ID (TID) layer. Fig. 1 and Fig. 2 show an example of QTBT block partitioning of frame #18 (TID = 3) and #17 (TID = 4) of BQMall sequence. It was observed that bock sizes tend to be larger as Temporal ID is higher, which may indicate that deep MaxBTDepth is not likely needed for frames with high Temporal ID.

Note: It has been understood that MaxBTDepth has an impact on encoding time and performance [1]. In CTC, test3 configuration (MaxBTDepth equal to 3) was chosen.

Fig. 1. BQMall frame #18 (TID = 3)

Fig. 2. BQMall frame #17 (TID = 4)

Proposed Method

The proposed method is to set MaxBTDepth depending on Temporal ID. In other words, key frames or more relevant frames with low Temporal ID uses deep MaxBTDepth to utilize flexible block size selection, while less important frames with high Temporal ID use smaller MaxBTDepth which speeds up encoding time. In the proposal, MaxBTDepth values for each TID are signaled in SPS.

Experimental results

The proposed method is implemented on top of JEM-3.0 reference software. Two tests, testA, test B with different TID base MaxBTDepth settings as well as test 2(MaxBTDepth equal to 2) in Table1 were performed for RA configuration. The results against JEM3.0 CTC anchor are shown in Table 2, Table 3 and Table 4.

Table 1. MaxBTDepth settings

| TID | MaxBTDepth |  |  |  |
| --- | --- | --- | --- | --- |
|  | testA | testB | CTC | test2 |
| 0 | 3 | 3 | 3 | 2 |
| 1 | 3 | 3 | 3 | 2 |
| 2 | 3 | 3 | 3 | 2 |
| 3 | 3 | 2 | 3 | 2 |
| 4 | 2 | 2 | 3 | 2 |

Table 2. Results on testA

|  | testA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.43% | 0.44% | 0.54% | 87% | 100% |
| Class A2 | 0.27% | 0.19% | 0.16% | 92% | 99% |
| Class B | 0.22% | 0.07% | 0.07% | 90% | 106% |
| Class C | 0.26% | 0.19% | 0.20% | 87% | 95% |
| Class D | 0.18% | 0.03% | 0.03% | 92% | 100% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 0.27% | 0.18% | 0.19% | 90% | 100% |
| Class F (optional) | 0.35% | 0.22% | 0.25% | 89% | 98% |

Table 3. Results on testB

|  | testB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.49% | 0.22% | 0.24% | 82% | 101% |
| Class C | 0.52% | 0.40% | 0.45% | 80% | 98% |
| Class D | 0.37% | 0.06% | 0.00% | 83% | 99% |
| Class E |  |  |  |  |  |
| Overall (Ref) |  |  |  |  |  |
| Class F (optional) | 0.64% | 0.54% | 0.62% | 81% | 99% |

Table 4. Results on test2

|  | test2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.46% | 2.47% | 2.72% | 63% | 100% |
| Class A2 | 1.98% | 2.62% | 2.25% | 68% | 102% |
| Class B | 1.41% | 1.55% | 1.40% | 64% | 95% |
| Class C | 1.32% | 1.71% | 1.89% | 63% | 95% |
| Class D | 1.14% | 1.38% | 1.74% | 66% | 100% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 1.46% | 1.93% | 1.97% | 65% | 98% |
| Class F (optional) | 1.37% | 1.26% | 1.31% | 65% | 105% |

The following figure shows the trade-off between encoding time and coding performance of JEM3.0 and proposals. The figure shows both test results are better than JEM3.0 in terms of the balance.

CTC (MaxBTdepth = 3)

test2 (MaxBTdepth = 2)

Discussions

We observed TID depending MaxBTdepth achieves better encoding time / performance balance. The constant configuration with MaxBTDepth = 2 shows good speed up and comparable encoding time / performance balance compared to MaxBTDepth = 3. But the loss is huge with1.5 %, which would not be acceptable. On the other hand, our proposed testX shows better encoding time / performance balance and the loss is limited with around 0.xx %.

Conclusion

This contribution introduces signaling MaxBTDepth for each Temporal ID. It is reported that TID depending MaxBTDepth shows better encoding time / performance balance compared with constant MaxBTDepth setting. We recommend to include TID depending MaxBTDepth software in JEM and to discuss which encoding configuration would be preferred for the next CTC.

References

- Kiho Choi, “Report of BoG on QTBT configuration setting”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-C0101, 4th Meeting

- G. Barroux (Fujitsu), T. Ikai (Sharp), Y. Bandoh (NTT), H. Yang (Huawei), A. Ichigaya (NHK), W. Hamidouche (INSA), R. Hashimoto (Renesas), “Request for actions regarding JEM complexity” , Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-C0105, 4th Meeting

Patent rights declaration(s)

SHARP Corporation. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).