| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0096 |
| --- | --- |

| Title: | EE2: Cross-check for new adaptive primary transform w/o block size dependency (test 4) and symmetrical (test 5). |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Seoul R&D Campus F-10 A Zone, Seongchon-gil 34, Seocho-gu, Seoul, Korea | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This contribution reports verification test results for EE2 (on adaptive primary transforms) tests 4 and 5. If mapping between transforms and Intra prediction modes the same for all block sizes the gain in AI test 0.1%(Y)/0.1%(Cb)/0.1%(Cr). With block size dependency the gain is 0.3%(Y)/0.2%(Cb)/0.2%(Cr) in AI. Significant performance improvement is demonstrated for screen content (class F gain is over 1%).

# Introduction

Two additional transform kernels were suggested to be added into set of adaptive primary transforms of JEM in [1]. New transform kernels are

  - DST-IV (which is similar to DST-VII but has a fast algorithm);

  - ID (no transform, but not TS as it can be selected separately per direction).

Improves performance over existing JEM primary transforms set:

Table 1. JEM3.0 transforms

| Transform Set | Transform Candidates |
| --- | --- |
| 0 | DST-VII, DCT-VIII |
| 1 | DST-VII, DST-I |
| 2 | DST-VII, DCT-VIII |

In original contribution mapping between Intra prediction modes and transforms was different depending on block size and also designed differently for vertical and horizontal directional Intra predictions with the symmetrical angles. EE2 studied both those aspects.

In both verified tests mapping between Intra prediction modes and transforms is symmetrical (for vertical and horizontal modes of same angles mirroring symmetric of LUT is implied). Additionally in EE2-T4 mapping between Intra prediction modes and transforms is same for all block sizes.

# Memory usage of primary transforms in JEM

HEVC standards uses integer approximation of DST for blocks 44 and DCT-II sizes 4, 8, 16 and 32. The magnitude of transform coefficients in HEVC doesn’t exceed 64 (7 bits). So, all HEVC transforms coefficients storage requires (4*4*2+8*8+16*16+32*32)*7 bits= 1204 bytes.

In JEM3.0 transform coefficients are storied in g_aiTrX [NUM_TRANS_TYPE][ X][ X]  matrices, NUM_TRANS_TYPE=5 in JEM3.0, X=2,4,8,16,32,64,128 (for blocks 22 and 128128 only one transform is used). Values of primary transform coefficients are within 10 bits. Total memory size required for coefficients of all primary transforms is 54585 bytes. In EE2 test 2 more primary transforms are added, but ID transform is just scaling, so memory size required for transform coefficients is 61405bytes.

Those data were provided by experts from Qualcomm and Orange during discussion of EE results among subscribers of EE mail list.

Table 1. Memory usage for JEM transforms compared to HEVC

| Tr. size | Matrix size | HEVC |  |  | JEM |  |  | JEM+EE2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | # kernels | bits | memory | # kernels | bit per element | total memory | # kernels | bit per element | total memory |
| 2 | 4 | 0 | 7 | 0 | 1 | 10 | 5 | 1 | 10 | 5 |
| 4 | 16 | 2 | 7 | 28 | 5 | 10 | 100 | 6 | 10 | 120 |
| 8 | 64 | 1 | 7 | 56 | 5 | 10 | 400 | 6 | 10 | 480 |
| 16 | 256 | 1 | 7 | 224 | 5 | 10 | 1600 | 6 | 10 | 1920 |
| 32 | 1024 | 1 | 7 | 896 | 5 | 10 | 6400 | 6 | 10 | 7680 |
| 64 | 4096 | 0 | 7 | 0 | 5 | 10 | 25600 | 6 | 10 | 30720 |
| 128 | 16384 | 0 | 7 | 0 | 1 | 10 | 20480 | 1 | 10 | 20480 |
| Total |  |  |  | 1204 |  | Total | 54585 |  | Total | 61405 |

Increment of memory for primary coefficients is depicted on Fig. 1.

# Experimental Results

Code for verified EE tests 4 and 5 was provided in time, availability of the code and test results was properly announced on JVET reflector. Performance was tests vs JEM3.0 under JVET common test conditions [2]. Performance matches the data reported by proponent. SW implementation matches EE description. Test results are summarized in Table1.

We have not found noticeable run time increment compared to JEM3.0

Table 1. Test results for EE2-T4 and EE2-T5.

# Conclusions

Performance of tests 4 and 5 in EE2 is verified in this contribution. Performance improvement from additional 2 kernels in the set of adaptive primary transforms is 0.3%(Y)/0.2%(Cb)/0.2%(Cr) in AI test is mapping between Intra perdition mode and transform kennel depends on block size. W/o dependency on block size performance is 0.1%(Y)/0.1%(Cb)/0.1%(Cr).

# References

- Pierrick Philippe, Victorien Lorcy “Proposed improvements to the Adaptive multiple Core transform”, JVET-C0022

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, JVET-B1010, February, 2016.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).