| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0098 |
| --- | --- |

| Title: | EE8-related: Decoupling decoder-side intra mode derivation from entropy decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

In the current Exploration Experiment (EE) 8, one decoder-side intra mode derivation (DIMD) method is proposed, which derives the intra prediction mode at decoder-side and reduces the overhead of intra mode signaling. It has been found that the current DIMD design can cause parsing dependency issue due to the interdependency between intra mode derivation and entropy decoding. In this contribution, one intra mode propagation method is proposed to address this issue. The method uses the intra modes of normal intra blocks (i.e., being explicitly signaled) to DIMD blocks at the entropy decoding stage when parsing transform coefficients, and thus does not require the intra modes of DIMD-coded blocks to be derived during entropy decoding. At the reconstruction stage, the intra mode of each DIMD block is derived based on the template-based intra mode derivation using reconstructed neighboring samples. It is reported that compared to the HM-16.6-JEM-3.0 anchor results the combination of the proposed method and DIMD provides average BD-rate savings of X.X% for AI configuration.

# Problem statement

A decoder-side intra mode derivation (DIMD) approach was proposed in [1]. Instead of signaling intra mode explicitly, the DIMD derives the information at both encoder and decoder from the neighboring reconstructed samples of one block. It is included in Exploration Experiments (EEs) [2] as EE8 for further evaluation. The current DIMD design is reported to have parsing dependency issues because of the interdependency between intra mode derivation and entropy decoding, which is caused by the following reasons:

- For intra-coded blocks, the scanning order of transform coefficients is dependent on the intra mode. For DIMD-coded blocks, the intra modes are derived from the reconstructed samples of the neighboring blocks. Therefore, the determination of scanning order of transform coefficients for DIMD blocks require fully reconstruction of the neighboring samples.

- For intra mode prediction of each normal intra block, MPM candidates are derived from the intra modes of the spatial neighbors of the block. If the selected MPM candidate is coded by DIMD mode, decoding the intra mode of the current block is dependent on the reconstruction of the intra mode of the neighboring DIMD block. As the decoded intra mode is used for parsing the transform coefficients of the current block, it operates in the same manner as the parsing dependency of scanning order for DIMD blocks.

# Proposal

To resolve the DIMD parsing dependency issue as discussed above, one intra mode propagation method is proposed to remove the interdependency between the DIMD intra mode derivation and entropy decoding. At the parsing stage, the proposed method uses the intra modes of normal intra blocks, which are explicitly signaled, to generate one intra mode of the DIMD block. The intra mode is then used to determine the scanning order of transform coefficients for the DIMD block and used as the MPM candidate to predict the intra modes of neighboring normal intra blocks. In this way, decoder is capable of parsing the bit-stream without knowing the derived intra mode of DIMD blocks. Additionally, the intra mode of the DIMD block is only used at the parsing stage. At the reconstruction stage, the intra mode of the DIMD block, which is used for intra prediction, will be derived based on the template-based intra mode derivation of DIMD by using the reconstructed samples of neighboring blocks [1]. Figure 1 shows the decoding flowchart of DIMD blocks with the proposed intra mode propagation method.

Figure 1 Decoding flowchart of DIMD blocks with the proposed intra mode propagation

To generate the intra mode of one DIMD block at the parsing stage, five spatial neighbor candidates are considered. The locations of the five neighbors are defined in the same way as the merge mode, i.e., left (L), above (A), below-left (BL), above-right (AR) and above-left (AL), as shown in Figure 2. The intra mode that is most frequently used among the five spatial neighbors is selected for the DIMD block.

Figure 2 Positions of spatial neighbors used by the intra mode propagation

In current HM16.6-JEM-3.0 encoder, in addition to the intra mode candidates that are roughly selected based on sum of absolute transformed differences (SATD) of prediction residual, a number of MPM candidates are also checked for full rate-distortion (R-D) calculation to select the optimal intra mode. When the proposed intra mode propagation is enabled, one encoder-only change is used by additionally checking the R-D cost of the intra modes derived by the intra mode propagation if they are different from the MPM candidates.

# Simulation results

The proposed method has been implemented on top of the current HM16.6-JEM-3.0 EE8 branch, which was download from https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-3.0-EE8-DIMD/. All the simulations are conducted by following the JVET common test conditions (CTCs) [3]. To validate the performance of the proposed method, three sets of simulations are conducted as described below. For all the tests, the results of HM-16.6-JEM-3.0 are used as anchor.

- Test #1: DIMD

- Test #2: DIMD + intra mode propagation

- Test #3: DIMD + intra mode propagation + encoder change

Table I. The performance of DIMD (Test #3 of EE8)

|  | Over HM-16.6-JEM-2 (parallel) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -0.34% | -0.28% | 162% | 115% |
| Class A2 | -0.88% | -0.58% | -0.57% | 170% | 119% |
| Class B | -0.61% | -0.27% | -0.41% | 165% | 118% |
| Class C | -0.86% | -0.56% | -0.64% | 161% | 132% |
| Class D | -0.73% | -0.54% | -0.44% | 158% | 132% |
| Class E | -1.23% | -1.03% | -1.06% | 170% | 117% |
| Overall | -0.78% | -0.52% | -0.54% | 164% | 122% |

Table II. The performance of DIMD + intra mode propagation

|  | Over HM-16.6-JEM-2 (parallel) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.54% | -0.05% | -0.14% | 155% | 110% |
| Class C | -0.74% | -0.20% | -0.42% | 152% | 120% |
| Class D | -0.61% | -0.39% | -0.25% | 154% | 116% |
| Class E | -1.03% | -0.48% | -0.62% | 161% | 113% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table III. DIMD + intra mode propagation + encoder change

|  | Over HM-16.6-JEM-2 (parallel) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.82% | -0.29% | -0.43% | 160% | 116% |
| Class D | -0.68% | -0.29% | -0.40% | 162% | 115% |
| Class E | -1.12% | -0.54% | -0.72% | 172% | 111% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# References

- X. Xiu, Y. He, Y. Ye, “Decoder-side intra mode derivation,” JVET-C0061, Geneva, Switzerland, June, 2016.

- J. Chen, E. Alshina, G.-J. Sullivan, J.-R. Ohm, J, Boyce, “Algorithm description of joint exploration test mode 3 (JEM3),” JVET-C1001, Geneva, Switzerland, June, 2016.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-B1010, San Diego, USA, Feb. 2016.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).