| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0133 |
| --- | --- |

| Title: | Peak Sample Adaptive Offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Li Zhang, Jianle Chen, Wei-Jung Chien 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents an in-loop filtering method, named Peak Sample Adaptive Offset (SAO). Similar to the SAO adopted in HEVC, in Peak SAO, each pixel may be modified by adding an offset. A Pixels is first classified into one of three categories. The pixel is then applied with an offset based on both category and pixel difference with its neighbors. Simulation results reportedly show that under all intra and random access configurations, the proposed method brings 0.2% bitrate savings with almost the same encoding running time.

# Introduction

The Sample Adaptive Offset (SAO) technology was adopted in HEVC [1] and also employed in JEM [2]. SAO supports two types: edge offset (EO) and band offset (BO). For BO, the classification is based on pixel values. For EO, the classification is based on comparison between current pixel and neighboring pixels according to 1-D directional patterns: horizontal, vertical, 135° diagonal, and 45° diagonal, as depicted in Figure 1.

Figure 1. Four 1-D directional patterns for EO classification: horizontal (EO class = 0), vertical (EO class = 1), 135° diagonal (EO class = 2), and 45° diagonal (EO class = 3). [1]

According to the selected EO pattern, five categories in Table I are further defined. For each category, an offset is signaled. A pixel which is classified to one of the five categories will be modified according to the signaled offset.

Table I: classification for EO in SAO

| Category | Condition |
| --- | --- |
| 0 | c < a && c < b |
| 1 | (c < a && c==b ) ¦¦ (c==a && c<b) |
| 2 | (c > a && c==b) ¦¦ (c == a && c > b) |
| 3 | c > a && c > b |
| 4 | None of the above |

# Technology description

In Peak SAO, four neighboring pixels are utilized to classify the current pixel into one of the three categories. The relative positions between the current pixel (denoted by C) and its four neighboring pixels (denoted by Ci with i being 0...3) are depicted in Figure 2. Depending on how many neighboring pixel values being larger or smaller than the current pixel value, three categories are defined, as tabulated in Table II. The lgr () and sml () functions are given as follows:

| lgr ( x ) = (x > 0) ? 1 : 0 | (1) |
| --- | --- |
| sml ( x ) = (x < 0) ? 1 : 0 | (2) |

Figure 2. Neighbouring samples used for Peak SAO classification.

Table II: classification for Peak SAO

| Category | Condition |
| --- | --- |
| 0 | i=03lgr Ci-C==3 ¦¦ i=03sml Ci-C==3 |
| 1 | i=03lgr Ci-C==4 ¦¦ i=03sml Ci-C==4 |
| 2 | None of the above |

If the pixel falls to category 2, the pixel value remains unchanged. Otherwise, its value is modified with a signaled offset. The offset is selected based on the average pixel value difference between the current pixel and its selected neighboring pixels and a normalization factor. Here, only neighboring samples with the same behavior (larger or small) as the whole block are utilized to calculate the average value. More specifically, the classification is defined as follows:

| (m=0m=M-1(¦Cm-C¦))/M(1≪NF) | (3) |
| --- | --- |

wherein Cm denotes the selected neighboring pixel value, M indicates the total number of pixels involved in average calculation and NF indicates the normalization factor. To avoid too many offsets to signal, a maximum class index is further signaled. When the derived class index using equation (3) exceeds a signaled maximum class index, the derived class index is replaced by the maximum class index.

The proposed filtering process is only applied to luma component. For each picture, a flag is firstly coded to indicate whether Peak SAO is enabled or disabled. If Peak SAO is enabled, other side information, such as NF, maximum class index and offsets are further signaled.

# Simulation results

The proposed method has been implemented on top of HM16.6 JEM-3.0 [3]. All the simulations below were conducted following the JVET common test conditions [4]. Note that for the RA configuration, ‘parallel’ coding is employed. Detailed coding performance results are tabulated below.

Table 1 Coding gain of proposed MDM on top of HM16.6 JEM3.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.08% | -0.03% | 100% | 113% |  |  |  |  |  |  |
| Class A2 | -0.15% | -0.05% | -0.04% | 103% | 114% |  | -0.18% | -0.06% | -0.02% |  |  |
| Class B | -0.12% | -0.02% | -0.02% | 100% | 113% |  | -0.34% | -0.02% | -0.18% |  |  |
| Class C | -0.32% | 0.01% | -0.02% | 100% | 109% |  | -0.33% | 0.17% | 0.05% |  |  |
| Class D | -0.15% | 0.03% | 0.04% | 99% | 105% |  | -0.30% | -0.05% | 0.17% |  |  |
| Class E | -0.21% | 0.00% | 0.00% | 100% | 112% |  |  |  |  |  |  |
| Overall (Ref) | -0.16% | -0.02% | -0.01% | 100% | 111% |  |  |  |  |  |  |

# Conclusion

This contribution proposed an in-loop filtering method based on HM-16.6-JEM-3.0 software. We suggest to further study it in Exploration Experiment.

# References

- C. Fu, E. Alshina, A. Alshin, Y. Huang, C. Chen, Chia. Tsai, C. Hsu, S. Lei, J. Park, W. Han, “Sample adaptive offset in the HEVC standard,” IEEE Trans. Circuits Syst. Video Technol., 22(12): 1755–1764 (2012)

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, JVET-C1001, “Algorithm description of Joint Exploration Test Model 3 (JEM3),” July, 2016.

- HM16.6 JEM-3.0, https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-3.0/.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-C1010, 2nd Meeting, San Diego, USA, 20–26 February 2016.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).