| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0155 |
| --- | --- |

| Title: | EE2 related: On transform coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | E. Alshina Seoul R&D Campus F-10 A Zone, Seongchon-gil 34, Seocho-gu, Seoul, Korea | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This contribution reports results for the test w/o extra precision for primary transform coefficients magnitude in JEM3.0. This modification saves 20% memory for transform coefficients storage w/o performance degradation.

# Introduction

EE2 studied potential extension for adaptive primary transforms set. One of the questions raised during EE2 discussion was about memory usage for multiple primary transforms storage.

Memory usage estimation provided by experts from Qualcomm and Orange during discussion among subscribers of EE mail list is shown in Table 1.  So memory usage for primary transform coefficients storage of JEM3.0 is 45 of HEVC. For JEM3.0+EE2 it becomes is 51 of HEVC.

Table 1. Memory usage for JEM3.0 and EE2 transforms compared to HEVC

| Tr. size | Matrix size | HEVC |  |  | JEM |  |  | JEM+EE2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | # kernels | bits | memory | # kernels | bit per element | total memory | # kernels | bit per element | total memory |
| 2 | 4 | 0 | 7 | 0 | 1 | 10 | 5 | 1 | 10 | 5 |
| 4 | 16 | 2 | 7 | 28 | 5 | 10 | 100 | 6 | 10 | 120 |
| 8 | 64 | 1 | 7 | 56 | 5 | 10 | 400 | 6 | 10 | 480 |
| 16 | 256 | 1 | 7 | 224 | 5 | 10 | 1600 | 6 | 10 | 1920 |
| 32 | 1024 | 1 | 7 | 896 | 5 | 10 | 6400 | 6 | 10 | 7680 |
| 64 | 4096 | 0 | 7 | 0 | 5 | 10 | 25600 | 6 | 10 | 30720 |
| 128 | 16384 | 0 | 7 | 0 | 1 | 10 | 20480 | 1 | 10 | 20480 |
| Total |  |  |  | 1204 |  | Total | 54585 |  | Total | 61405 |

Default settings of JEM3.0 is COM16_C806_TRANS_PREC=2. It means 2 extra bits for primary transform coefficients representation. If is COM16_C806_TRANS_PREC=0 then memory for coefficient storage is 20% smaller (as shown in Table 2). But it is still 36 of HEVC for JEM3.0 set of multiple transforms 36 of HEVC for  JEM3.0+EE2.

Table 2. Memory usage for JEM3.0 and EE2 transforms compared to HEVC w/o extra precision

| Tr. size | Matrix size | HEVC |  |  | JEM |  |  | JEM+EE2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | # kernels | bits | memory | # kernels | bit per element | total memory | # kernels | bit per element | total memory |
| 2 | 4 | 0 | 7 | 0 | 1 | 8 | 4 | 1 | 8 | 4 |
| 4 | 16 | 2 | 7 | 28 | 5 | 8 | 80 | 6 | 8 | 96 |
| 8 | 64 | 1 | 7 | 56 | 5 | 8 | 320 | 6 | 8 | 384 |
| 16 | 256 | 1 | 7 | 224 | 5 | 8 | 1280 | 6 | 8 | 1536 |
| 32 | 1024 | 1 | 7 | 896 | 5 | 8 | 5120 | 6 | 8 | 6144 |
| 64 | 4096 | 0 | 7 | 0 | 5 | 8 | 20480 | 6 | 8 | 24576 |
| 128 | 16384 | 0 | 7 | 0 | 1 | 8 | 16384 | 1 | 8 | 16384 |
| Total |  |  |  | 1204 |  | Total | 43668 |  |  | 49124 |

# Test results

The only modification in this test is COM16_C806_TRANS_PREC = 0. Default value is 2. In this case primary transforms in use 8 bits for coefficients magnitude (instead of 10 in JEM3.0 CTC).

Table 1. Summary of test results for disabled extra precision for primary transforms. (tested: COM16_C806_TRANS_PREC =0, anchor: JEM3.0)

Performance impact of this modification is negligible while memory saving is desirable.

# Conclusions

This contribution reports results for the test w/o extra precision for primary transform coefficients magnitude in JEM3.0. This modification saves 20% memory for transform coefficients storage w/o performance degradation.

# References

- JVET AHG report: JEM algorithm description editing (AHG2) .

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, JVET-B1010, February, 2016.

- A. Alshin, E. Alshina, “Bi-directional optical flow,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C204, Guangzhou, China, 10-15 October, 2010.

- Shunichi Sekiguchi, Yusuke Itani (Mitsubishi), “Report of CE1 :Decoder-Side Motion Vector Derivation,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D098, Daegu, Jan. 2011

- A. Alshin, E. Alshina, T. Lee “Bi-directional optical flow for improving motion compensation,” Picture Coding Symposium (PCS), 2010.

- A.Alshin, E.Alshina, M.Budagavi, K. Choi, J. Min, M. Mishourovsky, Y. Piao, A.  Saxena “Coding efficiency improvements beyond HEVC with known tools”, SPIE Optics + Photonics 2015, San Diego.

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).