| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0177 |
| --- | --- |

| Title: | AHG8: GoPro VR Player: A Tool for VR Content Playback |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Informative |  |  |
| Author(s) or Contact(s): | Adeel Abbas | Tel: Email: | aabbas@gopro.com |
| Source: | GoPro |  |  |

_____________________________

# Abstract

Virtual Reality (VR) content is generally submitted in equirectangular projection (ERP) format after stitching. Playing back VR videos in ERP format is unpleasant because viewers do not feel immersion and observe extreme levels of distortion at poles. We feel that the GoPro VR player [1] serves as a useful tool to play back VR files on mobile, desktops, laptops and head mounted displays like Oculus Rift.

# Introduction

Otfen times, one feels a need to quickly test subjective visual quality of a particular encoding technique and because it has huge distortion at poles, playing back video in ERP projection is not a good choice to do this testing. Playing back content on Head Mounted Displays (HMD) devices like Oculus Rift or HTC Vive is another option, but these devices are expensive, require a powerful PC and don’t lend well with the workflow of implementing->quick encoding->viewing when one is working on a regular laptop. Hence, there is a need for a VR player that can 1) play VR files with native (i.e uncompressed) quality 2) support viewport changes when one wants to pan and zoon 3) run on most mobile devices, laptops and regular PCs 4) support high resolutions (up to 8K) 5) supports HMD devices if one is available. This contribution serves to propose that GoPro VR player (formerly Kolor Eyes Desktop) provides all of these features and thus is a useful tool evaluate subjective quality of 360/VR videos. The player is available for free on all major platforms like iOS, Android, Mac, Windows and Linux platforms. Using GoPro VR player, playing 4K content on regular laptops and desktops is possible and even playing back of 8K content is also  possible.

# Playing Files with GoPro VR Player

The player can play back most modern image/video formats like TIF, PNG, MOV, MP4 and AVI. Playback of uncompressed file formats like YUV is not supported. In order to play any file, simply drag and drop it on the player window. Space bar toggles between pause and play operations. As the player is playing, one can press “H” key to look at statistics like field of view (FOV), decoded FPS and viewport parameters. Viewport can be changed by simple mouse or swipe gestures. Zoom in/out can be achieved using pinch gestures or +/- keys.

A simple screen shot of the GoPro VR Player is shown in Fig. 1 and Fig. 2.

# Playing YUV Files

The GoPro VR player on desktop and laptop platforms can natively play back 8-bit YUV files when packaged in an AVI container. This allows one to evaluate video at uncompressed image quality. On mobile platforms, like iOS and Android, YUV files can be encoded and packaged in mp4 container at very high bitrates.

Ffmpeg commands to package YUV into an AVI container are given below:

For 10-bit YUV (resolution 4096x2048 and 60fps)

ffmpeg.exe -s:v 4096x2048 -r 60 -pix_fmt yuv420p10le -i input.yuv -pix_fmt yuv420p -vcodec rawvideo output.avi

For 8-bit YUV (resolution 8192x4096 and 60fps)

ffmpeg.exe -s:v 8192x4096 -r 60 -pix_fmt yuv420p -i input.yuv -pix_fmt yuv420p -vcodec rawvideo output.avi

On modern Android phones like Samsung Galaaxy s7 and s7 edge, playback of HEVC encoded bitstreams at very high bitrate (up to 4096x2048 resolution at 60FPS is possible). An example command to achieve this is shown below:

ffmpeg.exe –s:v 4096x2048 –r 60 –i test.yuv –c:v libx265 –crf 8 test_hevc.mp4

Once the mp4 files are generated, place them in /Movies/GoPro VR/ folder. They should be automatically visible to the player in “My Media” section.

On modern iOS phones like iPhone 6s and 7, the only option available at the moment is to encode mp4 files with H.264 at high bitrates. And these devices are limited to 4096x2048 resolution at 30FPS. An example command for this platform is shown below:

ffmpeg.exe –s:v 4096x2048 –r 60 –i test.yuv –c:v libx264 –crf 8 test_hevc.mp4

Once the mp4 files are generated, drag and drop them to GoPro VR icon in iTunes, then press sync button Files should be automatically visible to the player.

# Oculus Support

The player also supports rendering directly to Oculus Rift [2] head mounted display. Once Oculus software is installed and the HMD is paired with computer, the VR player will automatically detect Oculus and start rendering to it. This makes it easy to test things because if someone wants to look at video quality on Oculus, they can just use the same player that works on desktop.

# Viewing In Different Projections

The player supports rendering of output 360 image in GoPro VR, Fish Eye[4], Rectillinear[5] and Little Planet (which is actually a Stereographic Projection[6]) Projection formats (this option is available in Preferences->General Settings). Fish Eye projection projects the 360 image to fish eye view, which has extreme barrel distortion at the corners. Rectillinear projects a 360 image such that straight lines appear straight. For example, the six faces in cubic projection are projected rectilinearly. The drawback of rectilinear projection is that for wide field of views (FOV greater than 90 degrees), stretching around the edges becomes extreme. This is illustrated by the image in Fig 1, which has a FOV of 120 degrees:

Fig 1

Little planet projects image such that look appears closer to barrel distortion effect that is observed when capturing images from wide angle cameras. Thus, corners do not have same amount of stretching as rectilinear projection but objecs are slightly distorted  (e.g. straight lines do not appear straight). The viewport of Fig. 1, when projected onto little planet at FOV 30 is shown in Fig. 2.

Fig 2

It can be seen from Fig. 1 and Fig. 2 that rectilinear projection preserves straight lines, but causes extreme stretching on the corners. Because people have now become more comfortable seeing videos from wide angle action cameras, we believe that Little Planet projection is artistically a better choice to view and test 360 content, especially at higher FOV angles). At lower FOV angles (FOV less than 40 degrees), both of these projections work well and rectilinear projection can still be selected.

It also needs to be mentioned that the default rendering choice of many online VR platforms like YouTube and Facebook is rectilinear projection. This has limitations in our opinion because they do not allow the ability to zoom out into wider FOV (because rectilinear projection shows high stretching on corners at larger FOV). Also since they limit to low FOV, vidoes seem soft (and with low resolution) because most of the content that they are getting is in 4K resolution (if they were getting their uploaded content in 8K resolution, this might have been all right). We contend that for 360 content that is in 4K original resolution, a wider FOV (something like what a GoPro sees) with little planet projection gives not only more immersion, but a higher resolution experience than what is possible with Rectillinear projection.

In the player, the GoPro VR projection gradually switches from rectilinear to little planet at around FOV 90 (91 is not stereographic but there is a small range before the projection is purely stereographic).

# Patent rights declaration(s)

GoPro may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- GoPro VR Player: http://www.kolor.com/gopro-vr-player/download/

- Oculus Rift https://www.oculus.com

- GoPro VR: https://vr.gopro.com/browse

- Fish Eye Projection: https://en.wikipedia.org/wiki/Fisheye_lens

- Rectillinear Projection: https://en.wikipedia.org/wiki/Rectilinear_lens

- Stereographic Projection: https://en.wikipedia.org/wiki/Stereographic_projection