| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D1030_v1 |
| --- | --- |

| Title: | JVET common test conditions and evaluation procedures for 360° video |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce, Elena Alshina, Adeel Abbas, Yan Ye | Email: | jill.boyce@intel.com, elena_a.alshina@samsung.com, aabbas@gopro.com, Yan.Ye@InterDigital.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document defines common test conditions and software reference configurations to be used in video codec performance evaluation for 360° video. These 360° video test conditions are also recommended for use in technical contributions to the 5th JVET meetings.

# Introduction

360° video common test conditions are desirable to conduct experiments in a well-defined environment and ease the comparison of the outcome of experiments.

Version 16.15 of HM HEVC reference software is agreed to be used for most experiments. For 360-specific coding tools proposal JEM4.1 software is recommended. Performance for both HM16.15 and JEM4.1 under test conditions specified below will be prepared by AhG8 and published.

Input contributions should provide a set of results as complete as possible that apply to the proposal. Results should be reported using the Excel sheets prepared and distributed by AhG8.

# Testing procedure

## Definitions

Here and after following abbreviations are used to specify formats (projections) representing 360 video:

ERP – Equi-rectangular projection

ISP – Icosahedral projection (described in [6])

CMP – Cube Map projection

OHP – Octahedron projection

TSP – Truncated square pyramid projection (described in [8])

SSP – Segmented sphere projection (described in [7])

EAP – Equal-area projection ([5])

There are two categories of frame packing for three projections ISP, CMP and OHP listed above. They are known as “native” and “compact” layouts.

Additionally to PSNR specific quality metric for spherical video listed below will be used:

WS-PSNR – as described in [2],

CPP-PSNR  – as described in [3],

S-PSNR-I  – as described in [4],

S-PSNR-NN  (w/o interpolation) – as described in [5].

## Processing chain

High fidelity test materials are provided in 4:2:0 YUV format representing 360 video in equirectangular projection (ERP). Prior to the encoding those materials are converted to the one of the formats listed above according to the testing procedure specified by Figure 1.

Fig. 1. 360 video testing procedure.

# Conversion software

For conversions and quality measurement specified on Figure 1 software package located at https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/ is expected to be used. Patches for HM16.15 and JEM4.1 are included into the package.

# Test sequences and sizes

## High fidelity input

Table 1 defines the set of test sequences agreed to be used. They are all in 4:2:0 YUV format representing 360 video in equirectangular projection (ERP). Height (H) of the picture in ERP projection is half of the width (W) reported in Table 1. All frames (as defined by frame count in the table) shall be processed and encoded for all sequences.

Test sequences are available on ftp://jvet@ftp.ient.rwth-aachen.de and ftp://jvet@ftp.hhi.fraunhofer.de in /testsequences/testset360 directory. Please contact the JVET chairs for login information.

Table 1 High fidelity input test sequences in ERP format

| Class | Sequence name | Frame count | Resolution@FPS | Bit-depth | Proposed in |
| --- | --- | --- | --- | --- | --- |
| 8K | Train_le | 600 | 8192x4096@60 | 8 | JVET-D0026 |
| 8K | SkateboardingTrick_le | 600 | 8192x4096@60 | 8 | JVET-D0026 |
| 8K | SkateboardInLot | 300 | 8192x4096@30 | 10 | JVET-D0026 |
| 8K | ChairLift | 300 | 8192x4096@30 | 10 | JVET-D0026 |
| 8K | KiteFlite | 300 | 8192x4096@30 | 8 | JVET-D0039 |
| 8K | Harbor | 300 | 8192x4096@30 | 8 | JVET-D0039 |
| 4K | PoleVault_le | 300 | 3840x1920@30 | 8 | JVET-D0143 |
| 4K | AerialCity | 300 | 3840x1920@30 | 8 | JVET-D0179 |
| 4K | DrivingInCity | 300 | 3840x1920@30 | 8 | JVET-D0179 |
| 4K | DrivingInCountry | 300 | 3840x1920@30 | 8 | JVET-D0179 |

For stereoscopic video “le” stands for Left Eye.

The final versions of the test sequence files names and md5 check sums are given in Table 2:

Table 2 Test sequences and Md5 checksums

| Sequence | Md5 |
| --- | --- |
| Train_le_8192x4096_60fps_8bit_420_erp.yuv | 90287c1b8cb03c71973e1e10949f75ad |
| SkateboardTrick_le_8192x4096_60fps_8bit_420_erp.yuv | 24029fe5bf03eca813952e2a6c700ba3 |
| SkateboardInLot_8192x4096_30fps_10bit_420_erp.yuv | e8eae04c43e959060f641fec4892fced |
| ChairliftRide_8192x4096_30fps_10bit_420_erp.yuv | 9126f753bb216a73ec7573ecc4a280c3 |
| KiteFlite_8192x4096_30fps_8bit_420_erp.yuv | 18c0ea199b143a2952cf5433e8199248 |
| Harbor_8192x4096_30fps_8bit_420_erp.yuv | aa827fdd01a58d26904d1dbdbd91a105 |
| PoleVault_le_3840x1920_30fps_8bit_420_erp.yuv | d038b3b1497df2ce5894a9f28cb467a8 |
| AerialCity_3840x1920_30fps_8bit_420_erp.yuv | a9eb9e7d44722c76dcbb6a56a8dd7248 |
| DrivingInCity_3840x1920_30fps_8bit_420_erp.yuv | 909667e65a9a6ad67780b6ada28328d8 |
| DrivingInCountry_3840x1920_30fps_8bit_420_erp.yuv | d982a6d1d186f8015b4f2eae080b2b61 |

## Coding projections

Prior to the encoding, high fidelity test materials are converted to the one of the coding projections listed above. Recommended frame sizes for each projections are listed in Table 3. For the 8K ERP source, the coding size is half width and half height of the high fidelity ERP, and for the remaining coding projections the sizes are selected to have the same amount of active samples as for the coded ERP (that is, 25% of the high fidelity 8K ERP). For 4K ERP source, encoding size is 75% of the high fidelity ERP for all projections.

For all conversions between projections prior to the encoding and after decoding conversion software shall use 6-taps Lanczos filter for Luma and 4-taps Lanczos filter for Chroma.

Table 3 Frame sizes for different projections

| Class Format | 8K |  |  |  | 4K |  |  |  | Bit-depth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | faceW | faceH | W | H | faceW | faceH | W | H |  |
| ERP high fidelity | NA | NA | 8192 | 4096 | NA | NA | 3840 | 1920 | 10 or 8 Table 1 |
| ERP for coding | NA | NA | 4096 | 2048 | NA | NA | 3328 | 1664 | 10 |
| ISP | 952 | 824 | 2400 | 3392 | 776 | 672 | 1960 | 2784 | 10 |
| CMP native | 1184 | 1184 | 4736 | 3552 | 960 | 960 | 3840 | 2880 | 10 |
| CMP compact | 1184 | 1184 | 3552 | 2368 | 960 | 960 | 2880 | 1920 | 10 |
| OHP | 1544 | 1336 | 3096 | 2672 | 1264 | 1096 | 2536 | 2192 | 10 |
| TSP | 1184 | 1184 | 2368 | 1184 | 960 | 960 | 1920 | 960 | 10 |
| SSP | 1216 | 1216 | 7296 | 1216 | 1008 | 1008 | 6048 | 1008 | 10 |
| EAP | NA | NA | 4096 | 2048 | NA | NA | 3328 | 1664 | 10 |

First coding projection should be generated. The output of conversion process is 4:2:0 YUV 10 bits video. So input bit-depth for encoding is 10 bits. Encoding and decoding is performed using 10 bits internal bit-depth.

After encoding and decoding WS-PSNR is calculated between original and reconstructed video. WS-PSNR is currently available for ERP, CMP, ISP, EAP and OHP projections only.

Between reconstructed coding projection and high fidelity ERP CPP-PSNR and S-PSNRs are calculated. CPP-PSNR is currently available for all projections except EAP.

Two variants of S-PSNR calculation will be performed: S-PSNR-I original metric developed in Stanford with 6-taps Lanczos interpolation for matching between positions on sphere and S-PSNR-NN w/o interpolation (with rounding to the nearest integer sample position).

Reconstructed coding projection is converted to ERP format with picture size equal to high fidelity ERP original video. WS-PSNR are measured between original and reconstructed ERP (high fidelity resolution).

For quality comparison within specific Field Of View (FoV) 2D rectilinear viewport picture shall be generated from high fidelity ERP (original viewport). It will be compared with viewport rendered from reconstructed coding projection (reconstructed viewport). For each test sequences 2 viewports will be generated, the size is 7575. Viewport parameters (pitch, yaw) for each test sequence are listed in Table 4.

For viewport generation conversion software should use bi-linear interpolation. The output of viewport generation is 10 bits 4:2:0 YUV video of size listed in Table 4 according to [9].

Table 4 View port parameters

| Class | Sequence name | Viewport Size | Viewport 1 (yaw, pitch) center (in degrees)_ | Viewport 2 (yaw, pitch) center (in degrees) |
| --- | --- | --- | --- | --- |
| 8K | Train | 1816x1816 | 73, −9 | 163, −17 |
| 8K | SkateboardingTrick | 1816x1816 | −64, −20 | 110, −7 |
| 8K | SkateboardInLot | 1816x1816 | 0, 0 | −76, −28 |
| 8K | ChairLift | 1816x1816 | 0, 0 | −156, −58 |
| 8K | KiteFlite | 1816x1816 | −69, −38 | 58, −26 |
| 8K | Harbor | 1816x1816 | −38, −41 | −95, 27 |
| 4K | PoleVault | 856x856 | 0, 0 | 77, −38 |
| 4K | AerialCity | 856x856 | 0, 0 | 122, −39 |
| 4K | DrivingInCity | 856x856 | 14, −29 | 90, 0 |
| 4K | DrivingInCountry | 856x856 | 0, 0 | 136, −32 |

Between original view port and reconstructed view port PSNR will be measured.

In total 8 different metrics will be reported: both PSNR reported by codec and WS-PSNR between input and output of the codec, CPP-PSNR between decoded output and input high fidelity ERP, S-PSNR-I, S-PSNR-NN (w/o interpolation) and WS-PSNR between original and reconstructed ERP in highest resolution, PSNR for 2 view ports.

Among all 360 video projection formats test results for ERP are mandatory to be reported. Other projection formats are optional.

# Encoder configurations and quantization parameter values

Encoder configuration files and their usage, as well as software configuration for both HM and JEM are described in [1]. For each video sequence four quantization parameter values are to be used: 22, 27, 32 and 37.

Aligned with JVET common test condition document [1] 4 tests, reflecting intra-only, random-access, and low-delay settings are defined:

- Intra, 10 bit

- Random access, 10 bit

- Low delay, 10 bit

- Low delay, P slices only, 10 bit

Among them, Random access is mandatory while remaining are optional. To be aligned with JVET ctc in random access test motion search range is 256, GOP size is 16 for both HM and JEM.

# Test results

# Templates for reporting test results for HM and JEM coding are attached to this document, they also can be found in https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/.

# References

[1] K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, 2nd Meeting, Feb. 2016.

[2] Y. Sun, A. Lu, L. Yu (Zhejiang Univ.), “AHG8: WS-PSNR for 360 video objective quality evaluation”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0040, 4th Meeting, Oct. 2016.

[3] V. Zakharchenko, E. Alshina, A. Singh, A. Dsouza (Samsung), “AhG8: Suggested testing procedure for 360-degree video”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0027, 4th Meeting, Oct. 2016.

[4] Matt Yu, Haricharan Lakshman, Bernd Girod, “A frame-work to evaluate omnidirectional video coding schemes, in Mixed and Augmented Reality”, 2015 IEEE International Symposium on Mixed and Augmented Reality (ISMAR).

[5] Y. He, B. Vishwanath, X. Xiu,Y. Ye (InterDigital), “AHG8: InterDigital's projection format conversion tool”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0021, 4th Meeting, Oct. 2016.

[6] V. Zakharchenko, E. Alshina, K. P. Choi,A. Singh, A. Dsouza (Samsung), “AhG8: Icosahedral projection for 360-degree video content”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0028, 4th Meeting, Oct. 2016.

[7] C. Zhang, Y. Lu, J. Li, Z. Wen (OwlReality), “AhG8: segmented sphere projection (SSP) for 360-degree video content”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0030, 4th Meeting, Oct. 2016.

[8] G. Van der Auwera, M. Coban,Hendry, M. Karczewicz (Qualcomm), “AHG8: Truncated Square Pyramid Projection (TSP) For 360 Video”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0071, 4th Meeting, Oct. 2016.

[9] A.Abbas, “Viewport Recommendations for CTC Content” , Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0195, 4th Meeting, Oct. 2016.