| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E_Notes_d0 |
| --- | --- |

| Title: | Meeting Report of the 5th meeting of the Joint Video Exploration Team (JVET), Geneva, CH, 12–20 January 2017 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from responsible coordinators of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Responsible coordinators |  |  |

_____________________________

- Summary

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its fourth meeting during 12 – 20 January 2017 at the ITU-T premises in Geneva, CH. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations. For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.13 of this document.

The JVET meeting sessions began at approximately 1400 hours on Thursday 12 January 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 20 January 2017. Approximately XXX people attended the JVET meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. The JVET meeting also performed an evaluation of compression technology designs proposed in this area, and refined the definition of test cases for evaluating such technology in a rigid manner.

One primary goal of the meeting was to review the work that was performed in the interim period since the fourth JVET meeting in producing the Joint Exploration Test Model 4 (JEM4). In this context, results from six exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM5 were planned.

The JVET produced 5 output documents from the meeting (update):

- Algorithm description of Joint Exploration Test Model 4 (JEM4)

- Work plan for assessment of test materials

- Description of Exploration Experiments on coding tools

- Common test conditions and evaluation procedures for HDR/WCG video

- Common test conditions and evaluation procedures for 360 video

For the organization and planning of its future work, the JVET established X "ad hoc groups" (AHGs) to progress the work on particular subject areas. X Exploration Experiments (EE) were defined on particular subject areas of coding tool testing. The next four JVET meetings are planned for Fri. 31 Mar. – Fri. 7 Apr. 2017 under WG 11 auspices in Hobart, AU, during Fri. 14 – Fri. 21 Jul. 2017 under WG 11 auspices in Torino, IT, during Wed. 18. – Wed. 25 Oct. 2017 under ITU-T auspices in Macao, CN, and during … .

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Exploration Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its fifth meeting during 12 – 20 January 2017 at the ITU premises in Geneva, CH. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations.

## Meeting logistics

The JVET meeting sessions began at approximately 1400 hours on Thursday 12 January 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 20 January 2017. Approximately XXX people attended the JVET meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. The JVET meeting also performed an evaluation of compression technology designs proposed in this area, and refined the definition of test cases for evaluating such technology in a rigid manner.

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2017_01_E_Geneva/.

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the fourth JVET meeting in producing the Joint Exploration Test Model 4 (JEM4). In this context, results from nine exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM5 were planned.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the JEM software but have no normative effect are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the JEM description (an error, oversight, or messiness) or in the software are marked by the string "Decision (BF):".

This meeting report is based primarily on notes taken by the responsible leaders. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 03 January 2017. Any documents uploaded after 2359 hours Paris/Geneva time on Wednesday 04 January were considered "officially late", giving a grace period of 24 hrs to those living in different time zones of the world.

All contribution documents with registration numbers JVET-E0095 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "E0095+" range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- JVET-E0XXX (a proposal on …), uploaded 01-XX,

- ..

The following technical design proposal contributions were both registered late and uploaded late:

- JVET-E0XXX (a proposal on …), uploaded 01-XX,

- ..

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JVET-E0XXX (information documents on …), uploaded 01-XX,

- ...

The following cross-verification reports were registered on time but were uploaded late: JVET-E0XXX [uploaded 01-XX], … .

(Documents that were both registered late and uploaded late, other than technical proposal documents, are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-E0XXX, … .

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable rejected in the document management system. The initial uploads of the following contribution documents were rejected as "placeholders" and were not corrected until after the upload deadline: JVET-D0049 [improved on 10-15], JVET-D0053 [improved on 10-10], JVET-D0098 [improved on 10-13].

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, EE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly the meeting report JVET-D1000, JEM4 algorithm description JVET-D1001, the work plan for assessment of test material JVET-D1002, the description of exploration experiments JVET-D1011, the JVET common test conditions and evaluation procedures for HDR/WCG video JVET-D1020, and the JVET common test conditions and evaluation procedures for 360 video JVET-D1030, were approved. The JEM4 software implementation (versions 4.0 and 4.1) was also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Review of results of previous meeting

- Review of AHG reports

- Reports of exploration experiments

- Consideration of contributions and communications on project guidance

- Consideration of video technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the JEM software uses the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the JEM software, and for evaluating proposals for technology to be included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

Clarify status of 360 video conversion software – as it is part of the HM/JEM package, does it follow the same copyright header?

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It is reminded to send notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers is 610.

For distribution of test sequences, a password protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI.

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT: Adaptive multi-core transform.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP: Advanced temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BIO: Bi-directional optical flow.

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EE: Exploration Experiment – a coordinated experiment conducted toward assessment of coding technology.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MDNSST: Mode dependent non-separable secondary transform.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- QTBT: Quadtree plus binary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in JEM:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in P/B slice, and one CTB per luma CTU and two CTBs per chroma CTU in I slice.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size to a CU.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size to a CU.

## Opening remarks

- Reviewed logistics, agenda, working practices

- Results of previous meeting: JEM, meeting report, etc.

- Goals of the meeting: New version of JEM, evaluation of status progress in EEs and new proposals, provide summary to parent bodies, define new EEs.

- …

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Thu. 12 Jan, 1st day

  - 1400-XXXX Opening, AHG reports (chaired by JRO and GJS)

  - XXXX-XXXX EE review (chaired by JRO and GJS)

- Fri. 13 Jan, 2nd day

  - 0900-… (chaired by …)

  - …

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized (final number counts tbd)

- AHG reports (9) (section 2)

- Analysis, development and improvement of JEM (3) (section 3)

- Test material (12) (section 4)

- Exploration experiments (42) (section 5)

  - EE1 and related: Residual coefficients coding (4)

  - EE2 and related: Nonlinear in-loop filters (10)

  - EE3 and related: Decoder-side motion vector derivation (11)

  - EE4 and related: MV coding (5)

  - EE5 and related: Chroma coding (8)

  - EE6 and related: Adaptive scaling for HDR/WCG material (4)

- Non-EE technology proposals (17) (section 6)

  - Transforms and coefficient coding (6)

  - Motion compensation and vector coding (4)

  - Intra prediction and coding (2)

  - QTBT improvements and other partitioning schemes (0)

  - Loop filters (3)

  - Other (2)

- Extended colour volume coding (5) (section 7)

- 360 video (20) (section 8)

- Encoder optimization (5) (section 9)

- Metrics and evaluation criteria (0) (section 10)

- Withdrawn (3) (section 11)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 12)

- Project planning (section 13)

- Output documents, AHGs (section 14)

# AHG reports (9)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0001 JVET AHG report: Tool evaluation (AHG1) [M. Karczewicz, E. Alshina] [miss]

JVET-E0002 JVET AHG report: JEM algorithm description editing (AHG2) [J. Chen, E. Alshina, J. Boyce] [miss]

JVET-E0003 JVET AHG report: JEM software development (AHG3) [X. Li, K. Suehring] [miss]

JVET-E0004 JVET AHG report: Test material (AHG4) [T. Suzuki, V. Baroncini, J. Chen, J. Boyce, A. Norkin] [miss]

JVET-E0005 JVET AHG report: Fast encoding, encoding complexity investigation, and configuration settings (AHG5) [K. Choi, Y.-J. Chang, H. Huang, L. Xiang, P. Philippe, Y. Yukinobu]

JVET-E0006 JVET AHG report: Simplification of decoder-side motion derivation tools (AHG6) [X. Li, E. Alshina] [miss]

JVET-E0007 JVET AHG report: JEM coding of HDR/WCG material (AHG7) [A. Segall (chair), S. Lasserre, D. Rusanovskyy (vice chairs)] [miss]

JVET-E0008 JVET AHG report: 360 video coding tools and test conditions (AHG8) [J. Boyce (chair), A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye (vice chairs)] [miss]

JVET-E0009 JVET AHG report: 360 video conversion software development (AHG9) [Y. He, V. Zakharchenko (co-chairs)] [miss]

# Analysis, development and improvement of JEM (3)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0030 JEM bug fix for ALF cross-picture prediction [R. Sjöberg, M. Pettersson (Ericsson)]

JVET-E0059 Floating point QP support for parallel encoding in RA configuration [X. Ma, H. Chen, H. Yang, M. Sychev (Huawei)]

JVET-E0064 On JEM Binary Arithmetic Engine Design [M. Zhou, Y. Hu (Broadcom)]

# Test material (12)

## New test material proposals (1)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0086 New HDR 4K test sequences with Hybrid Log-Gamma transfer characteristics [Shunsuke Iwamura, Atsuro Ichigaya (NHK)]

## Test material evaluation (11)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0022 Evaluation report of 1080P Test Sequences from Sharp [T. Hashimoto, Y. Yasugi (Sharp)]

JVET-E0040 AHG4: Evaluation report of new 4K test sequences [K. Choi, E. Alshina (Samsung)] [late]

JVET-E0041 AHG4: Evaluation report of new HDR test sequences [K. Choi, E. Alshina (Samsung)] [late]

JVET-E0042 AHG4: Cross-check of 4K test sequences [K. Choi, E. Alshina (Samsung)] [late]

JVET-E0053 Evaluation report of SDR test sequences (4K5-9 and 1080p1-5) [S. Cho, S.-C. Lim, J. Kang (ETRI)]

JVET-E0082 AHG4: Evaluation report of partial 4K sequences from DJI [X. Zheng (DJI)] [late]

JVET-E0087 AHG4: Evaluation report of 4K test sequences (ClassA1/A2) [H.-C. Chuang, J. Chen, X. Li, M. Karczewicz (Qualcomm)] [late]

JVET-E0095 Evaluation report of 1080p test sequences [O. Nakagami, T. Suzuki (Sony)] [late]

JVET-E0110 AHG4: Evaluation report of SDR test sequences (4K8-9 and 1080p1-5) [Y.-H. Ju, C.-C. Lin, C.-L. Lin, Y.-J. Chang, P.-H. Lin (ITRI)] [late]

JVET-E0112 AHG4: Evaluation report of aerial photography sequences [Y.-H. Ju, C.-C. Lin, C.-L. Lin, Y.-J. Chang, P.-H. Lin (ITRI)] [late] [miss]

JVET-E0121 AHG4: Evaluation report of Netflix HDR test sequences [T. Lu, F. Pu, P. Yin, T. Chen, W. Husak (Dolby)] [late]

# Exploration experiments (42)

## General (1)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0010 Exploration Experiments on Coding Tools Report [E. Alshina, J. Boyce, L. Zhang]

## EE1: Residual coefficients coding (4)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

### Primary (4)

JVET-E0051 EE1: Residual Coefficient Sign Prediction [Gordon Clare, Felix Henry (Orange)]

JVET-E0038 EE1: Cross-check results for residual coefficient sign prediction (JVET-E0051) [Y.Piao, E.Alshina (Samsung)]

JVET-E0072 EE1: Cross-check of Residual Coefficient Sign Prediction (tests 5 and 6) (JVET-E0051) [G. Barroux, K. Kazui, K. Takeuchi (Fujitsu)]

JVET-E0102 EE1: Cross-check of JVET-E0051 on Residual Coefficients coding [T. Hashimoto, T. Ikai (Sharp)] [late]

### Related (0)

## EE2: Nonlinear in-loop filters (10)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

### Primary (7)

JVET-E0031 EE2-JVET-D0069 Bilateral filter Test1, Test2, Test3 [J. Ström, P. Wennersten, K. Andersson, J. Enhorn (Ericsson)]

JVET-E0043 EE2: Cross-check of EE2 (Bilateral filter after inverse transform) [K. Choi, E. Alshina (Samsung)] [late]

JVET-E0091 EE2: Cross-check of EE2 (Bilateral filter after inverse transform) [L. Zhang (Qualcomm)] [late] [miss]

JVET-E0066 EE2: Peak Sample Adaptive Offset [M. Karczewicz, L. Zhang, J. Chen, W.-J. Chien (Qualcomm)]

JVET-E0034 Cross-check of EE2-JVET-D0133 Test4 and Test5 [K. Andersson (Ericsson)]

JVET-E0044 EE2: Cross-check of EE2 (Peak Sample Adaptive Offset) [K. Choi, E. Alshina (Samsung)] [late]

JVET-E0120 EE2: Cross-check of JVET-E0066 on Peak Sample Adaptive Offset [Roman Chernyak (Huawei)] [late]

### Related (3)

JVET-E0032 Bilateral filter strength based on prediction mode [J. Ström, P. Wennersten, K. Andersson, J. Enhorn (Ericsson)]

JVET-E0092 Cross-check of JVET-E0032 Bilateral filter strength based on prediction mode [L. Zhang (Qualcomm)] [late] [miss]

JVET-E0109 Cross-check of JVET-E0032 on Bilateral filter strength based on prediction mode [K. Choi, E. Alshina (Samsung)] [late]

## EE3: Decoder-side motion vector derivation (11)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

### Primary (9)

JVET-E0028 EE3: bi-directional optical flow w/o block extension [A. Alshin, E. Alshina (Samsung)]

JVET-E0063 Cross-check of EE3-JVET-D0042 On BIO memory bandwidth [A. Robert, F. Le Léannec, T. Poirier (Technicolor)] [late]

JVET-E0124 EE3: Crosscheck of JVET-E0028 bi-directional optical flow w/o block extension [Y.-W. Chen, X. Li (Qualcomm)] [late]

JVET-E0052 EE3: Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching [X. Chen, J. An, J. Zheng (HiSilicon)]

JVET-E0049 EE3: Cross-check for Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching [A. Alshin, E. Alshina (Samsung)]

JVET-E0088 EE3: Cross-check of Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching [Han Boon Teo, Ru Ling Liao (Panasonic)] [late]

JVET-E0060 EE3-JVET-D0046: High precision FRUC with additional candidates [A. Robert, F. Le Léannec, T. Poirier (Technicolor)]

JVET-E0048 Cross-check for high precision FRUC with additional candidates [E. Alshina, A. Alshin (Samsung)]

JVET-E0100 EE3: Cross-check of JVET-E0060 on High precision FRUC with additional candidates [T. Ikai (Sharp)] [late]

### Related (2)

JVET-E0035 Enhanced Template Matching in FRUC Mode [Y. Lin, X. Chen, J. An, J. Zheng (HiSilicon)]

JVET-E0096 Crosscheck of JVET-E0035 Enhanced Template Matching in FRUC Mode [Y.-W. Chen (Qualcomm)] [late] [miss]

## EE4: MV coding (5)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

### Primary (5)

JVET-E0076 EE4: Enhanced Motion Vector Difference Coding [J. Chen, W.-J. Chien, M. Karczewicz, X. Li (Qualcomm)]

JVET-E0101 EE4: Cross-check of JVET-E0076 on Enhanced Motion Vector Difference Coding [T. Ikai (Sharp)] [late]

JVET-E0122 EE4: Cross-check of JVET-E0076 on Enhanced Motion Vector Difference Coding [Roman Chernyak, Sergey Ikonin (Huawei)] [late]

JVET-E0046 EE4: Cross-check of EE4 (MV coding: switchable binarization) [S. Jeong, E. Alshina (Samsung)] [late]

JVET-E0111 EE4: Cross-check of Enhanced Motion Vector Difference Coding (JVET-E0076 Test 2) [H.-B. Teo, R.-L. Liao (Panasonic)] [late]

### Related (0)

## EE5: Chroma coding (8)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

### Primary (7)

JVET-E0062 EE5: Multiple Direct Modes for chroma intra coding [L. Zhang, W.-J. Chien, J. Chen, X. Zhao, M. Karczewicz (Qualcomm)]

JVET-E0099 EE5 #5: Cross-check of JVET-E0062 on Multiple Direct Modes for chroma intra coding [Q. Yao, K. Kawamura, S. Naito (KDDI)] [late]

JVET-E0077 EE5: Enhanced Cross-component Linear Model Intra-prediction [K. Zhang, J. Chen, L. Zhang, M. Karczewicz (Qualcomm)]

JVET-E0045 EE5: Cross-check for Enhanced Cross-Component Linear Model Intra Prediction [B. Jin, E. Alshina (Samsung)] [late]

JVET-E0080 EE5: Cross-check of EE5 Multiple-filter LM [J. Lee, H. Lee, J. W. Kang (ETRI)] [late]

JVET-E0097 EE5: Cross-check of Multiple linear models (JVET-E0077 Test 1) [Y. Yasugi, T. Ikai (Sharp)] [late]

JVET-E0098 EE5 #4: Cross-check of JVET-E0077 on Enhanced Cross-component Linear Model Intra-prediction [Q. Yao, K. Kawamura, S. Naito (KDDI)] [late]

### Related (1)

JVET-E0103 [EE5 related] Block adaptive CCLM residual prediction [K. Kawamura, S. Naito (KDDI) [late]

## EE6: Adaptive scaling for HDR/WCG material (4)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

### Primary (4)

JVET-E0055 EE6: Performance evaluation of adaptive scaling of JVET-D0118 for extended color volume material [D. Rusanovskyy, A.K. Ramasubramonian, J.R. Sole, M. Karczewicz (Qualcomm)]

JVET-E0123 EE6: Cross-check and Supplemental result of JVET-E0055 [J. Zhao, A. Segall] [late]

JVET-E0081 EE6: On Dequantization and Scaling for Extended Colour Volume Materials [J. Zhao, A. Segall, K. Misra (Sharp)]

JVET-E0125 EE6: Cross-check of JVET-E0081 results [D. Rusanovskyy (Qualcomm)] [late] [miss]

### Related (0)

# Non-EE Technology proposals (17)

## Transforms and coefficient coding (6)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0036 On Adaptive Multiple Core Transform for Chroma [T. Tsukuba, O. Nakagami, M. Ikeda, T. Suzuki (Sony)]

JVET-E0073 Cross-check of JVET-E0036 On Adaptive Multiple Core Transform for Chroma [X. Zhao, J. Chen, V. Seregin (Qualcomm)] [late] [miss]

JVET-E0037 Signaling for primary transform and transform skip [H. Jang, J. Lim, J. Nam, S. Kim (LGE)]

JVET-E0089 Cross-check of JVET-E0037 on Signaling for primary transform and transform skip [P. Philippe (Orange)] [late]

JVET-E0047 Adaptive NSST Kernel Size Selection [H. Jang, J. Nam, J. Lim, S. Kim (LGE)]

JVET-E0114 Crosscheck of JVET-E0047 on adaptive NSST kernel size selection [M.-S. Chiang, C.-W. Hsu (MediaTek)] [late]

## Motion compensation and motion parameter coding (4)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

### Decoder-side estimation methods (2)

JVET-E0027 Decoder-Side Direct Mode Prediction [Y. Han, J. An, J. Zheng (HiSilicon)]

JVET-E0115 Crosscheck of JVET-E0027 on decoder-side direct mode prediction [C.-M. Tsai, C.-W. Hsu (MediaTek)] [late] [miss]

### MV and motion parameter coding (2)

JVET-E0039 Simplification of affine flag signaling condition [J. Lee, S. Kim, J. Lim (LGE)]

JVET-E0127 Cross-check of Simplification of affine flag signaling condition (JVET-E0039) [H. Zhang, H. Chen, H. Yang] [late] [miss]

## Intra prediction and coding (2)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

### Intra prediction (2)

JVET-E0068 Unequal Weight Planar Prediction and Constrained PDPC [K. Panusopone, S. Hong, L. Wang (ARRIS)]

JVET-E0117 Cross-check of JVET-E0068 on Unequal Weight Planar Prediction and Constrained PDPC [T. Ikai (Sharp)] [late]

### Intra mode coding (0)

## Partitioning schemes (0)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

## Loop filters (3)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0079 Unified Adaptive Loop Filter for Luma and Chroma [J. An, J. Zheng (HiSilicon)]

JVET-E0093 Cross-check of JVET-E0079 Unified Adaptive Loop Filter for Luma and Chroma [L. Zhang (Qualcomm)] [late] [miss]

JVET-E0104 ALF temporal prediction with temporal scalability [L. Zhang, W.-J. Chien, M. Karczewicz] [late]

## Other (2)

JVET-E0074 Dynamic Textures Synthesis Based on Motion Distribution Statistics [O. Chubach, M. Wien (RWTH Aachen Univ.)]

JVET-E0119 Binary arithmetic coding with small table or short multiplications [A. Said, T. Hsieh, R. Joshi, V. Seregin, M. Karczewicz (Qualcomm)]	[late]

# Extended colour volume coding (5)

## Test conditions and evaluation (3)

All contributions of this section were reviewed in BoG E0XXX (chaired by XXX), unless indicated otherwise.

JVET-E0067 AHG7: On coding of extended colour volume materials [E. François, C. Chevance, F. Hiron (Technicolor)]

JVET-E0069 AHG7/EE6: Comments on JVET common test conditions and evaluation procedures for HDR/WCG video [D. Rusanovskyy, A.K. Ramasubramonian, J.R. Sole, M. Karczewicz (Qualcomm)] [late] [miss]

JVET-E0094 AHG7: On JVET common test conditions and evaluation procedures for HDR/WCG video [Andrey Norkin (Netflix)] [late] [miss]

## Tools (2)

JVET-E0105 A new tool for Color Gamut Analysis of MPEG video content [A.M. Tourapis, M. Meyer, D. Singer (Apple)] [late]

JVET-E0106 HDRTools: Generalized Scaling and Tone Mapping Support [A.M. Tourapis, T. Baar, Y. Su, D. Singer] [late]

# Coding of 360 video projection formats (20)

All contributions of this section were reviewed in BoG E0XXX (chaired by XXX), unless indicated otherwise.

## Conversion tools (1)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0084 AHG8: Algorithm description of projection format conversion in 360Lib [Y. He, X. Xiu, Y. Ye (InterDigital), V. Zakharchenko, A. Singh, A. Dsouza (Samsung), C.-C. Huang, J.-L. Lin (MediaTek), Y. Sun, A. Lu, L. Yu (Zhejiang Univ.), G. V. Auwera (Qualcomm), C. Zhang, Y. Lu (OwlReality)]

## Packing and Projection formats (9)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0025 AHG8: Segmented Sphere Projection for 360-degree video [C. Zhang, Y. Lu, J. Li, Z. Wen (Owl Reality)]

JVET-E0085 AHG8: Crosscheck of JVET-E0025 Segmented Sphere Projection for 360-degree video [Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital)]

JVET-E0029 AHG8: Efficient Frame Packing for Icosahedral Projection [S.N. Akula, A. Singh, A. Dsouza, R.K. K??, C. Pujara, R.N. Gadde, V. Zakharchenko, E. Alshina, K.P. Choi, (Samsung)]

JVET-E0056 AHG8: An improvement on the compact OHP layout [H.-C. Lin, C.-C. Huang, C.-Y. Li, Y.-H. Lee, J.-L. Lin, S.-K. Chang (MediaTek)]

JVET-E0057 AHG8: Face-based padding for cube projection [C.-H. Shih, H.-C. Lin, J.-L. Lin, S.-K. Chang (MediaTek)]

JVET-E0058 AHG8: A viewport-based pyramid projection for VR360 video streaming [P. Wang, H.-C. Lin, C.-Y. Li, Y.-H. Lee, J.-L. Lin, S.-K. Chang (MediaTek)]

JVET-E0061 AHG8: Yaw-roll-pitch orientation for VR360 video content [H.-C. Lin, C.-H. Shih, J.-L. Lin, S.-K. Chang (MediaTek)]

JVET-E0090 AHG8: Nested polygonal chain packing of 360-degree ERP pictures [K. Kammachi-Sreedhar, M. M. Hannuksela (Nokia)]

JVET-E0113 AHG8: Supplemental Test Results on Segmental Sphere Projection (SSP) [M. Zhou (Broadcom)] [late]

## Testing procedure and metrics (6)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0021 AHG8: On 360 video testing procedure [V. Zakharchenko, C. Kim, E. Alshina (Samsung)]

JVET-E0024 AHG8: Dynamic viewport generation for 360° video evaluation [T. Ikai, Y. Yasugi, T. Aono (Sharp)]

JVET-E0070 AHG8: TSP Evaluation With Viewport-Aware Quality Metric For 360 Video [Geert Van der Auwera, Muhammed Coban, Hendry, Marta Karczewicz (Qualcomm)]

JVET-E0071 AhG8: Viewport-based subjective evaluation of 360-degree video coding [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-E0083 AHG8: On 360 Video Quality Evaluation [Hendry, M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-E0107 AHG8: Requirements and proposed method for viewport-adaptive quality assessment [A. Aminlou, M. Hannuksela (Nokia)] [late]

## Coding tools (3)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0026 Results for Geometry correction for motion compensation of planar-projected 360VR video with JEM4.1 and 360Lib [J. Sauer, M. Wien (RWTH Aachen Univ.)]

JVET-E0050 AhG8: Coding performance impact of omnidirectional projection rotation [V. Zakharchenko, E. Alshina, K.P. Choi, C. Pujara, A. Dsouza (Samsung)]

JVET-E0065 AHG8: Unrestricted Motion Compensation for 360 Video in ERP Format [M. Zhou (Broadcom)]

## HL syntax (1)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0075 AHG8: Spherical rotation orientation SEI for coding of 360 video [J. Boyce, Q. Xu (Intel)]

# Encoder optimization (5)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

JVET-E0023 AHG5: Improved fast encoding setting [Y. Yamamoto, T. Ikai, Y. Yasugi (Sharp)]

JVET-E0054 Cross-check of AHG5: Improved fast encoding setting (Test A) [X. Chen, J. Zheng (HiSilicon)] [late] [miss]

JVET-E0118 Cross-check of AHG5: Improved fast encoding setting (test B) [V. Lorcy (bcom), P. Philippe (Orange), T. Biatek (TDF)] [late] [miss]

JVET-E0078 AHG5: improved fast algorithm in JEM-4.0 [P.-H. Lin, Y.-J. Chang, C.-L. Lin, C.-C. Lin (ITRI)]

JVET-E0116 Cross-check of AHG5: improved fast algorithm in JEM-4.0 [K. Choi (Samsung)] [late] [miss]

# Metrics and evaluation criteria (0)

All contributions in this category discussed XXX XX00-XX00 (chaired by …)

# Withdrawn (3)

See under 1.4.2.

JVET-E0033 Withdrawn

JVET-E0108 Withdrawn

JVET-E0126 Withdrawn

# Joint Meetings, BoG Reports, and Summary of Actions Taken

## Exploration Experiments (update)

The setup of Exploration Experiments was discussed, and an initial draft of the EE document was reviewed in the plenary (chaired by JRO). This included the list of all tools that are intended to be investigated in EEs during the subsequent meeting cycle:

EE1: Residual Coefficients coding

JVET-D0031 Residual Coefficient Sign Prediction [F. Henry, G. Clare (Orange)]

EE2: Nonlinear in-loop filters

JVET-D0069 Bilateral filter after inverse transform [J. Ström, P. Wennersten, Y. Wang, K. Andersson, J. Samuelsson (Ericsson)]

JVET-D0133 Peak Sample Adaptive Offset [M. Karczewicz, L. Zhang, J. Chen, W.-J. Chien (Qualcomm)]

EE3: Decoder Side Motion Vector Derivation

JVET-D0046 High precision FRUC with additional candidates [A. Robert, F. Le Léannec, T. Poirier (Technicolor)]

JVET-D0029 Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching [X. Chen, J. An, J. Zheng (HiSilicon)]

JVET-D0042 AHG6: On BIO memory bandwidth [A.Alshin, E.Alshina (Samsung)].

EE4: MV coding

JVET-D0123 Enhanced Motion Vector Difference Coding [J. Chen, W.-J. Chien, N. Hu, V. Seregin, M. Karczewicz, X. Li (Qualcomm)]

EE5: Chroma coding

JVET-D0110 Enhanced Cross-component Linear Model Intra-prediction [K. Zhang, J. Chen, L. Zhang, M. Karczewicz (Qualcomm)]

JVET-D0111 Multiple Direct Modes for chroma intra coding [L. Zhang, W.-J. Chien, J. Chen, X. Zhao, M. Karczewicz (Qualcomm)]

EE6: Adaptive Scaling for HDR/WCG Material

JVET-D0118 On adaptive scaling for extended colour volume materials

JVET-D0124 On De-quantization and Scaling for Extended Colour Volume Materials

An initial version of the EE document had been prepared by Elena Alshina and circulated on Thursday.

It was agreed to give the editors the discretion to finalize the document during the two weeks after the meeting, and circulate/discuss it on the reflector appropriately.

## Joint meetings

## BoGs

## List of actions taken affecting the JEM5

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the JEM5 description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

### Encoder only or software changes (update)

### Syntax/semantics/decoding process changes (update)

# Project planning

## JEM description drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- JEM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be Tuesday 03 Jan 2017.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for Experiments

Note: This section was drafted during the second JVET meeting, and is kept here for information about the EE procedure.

Group coordinated experiments have been planned. These may generally fall into one category:

- "Exploration experiments" (EEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the main branch of JEM by the next meeting.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. (E. Alshina will edit the document based on input from the proponents, review is performed in the plenary)

- Software for tools investigated in EE is provided in a separate branch of the software repository

- During the experiment, further improvements can be made

- By the next meeting it is expected that at least one independent party will report a detailed analysis about the tool, confirms that the implementation is correct, and gives reasons to include the tool in JEM

- As part of the experiment description, it should be captured whether performance relative to JEM as well as HM (with all other tools of JEM disabled) should be reported by the next meeting.

It is possible to define sub-experiments within particular EEs, for example designated as EEX.a, EEX.b, etc., where X is the basic EE number.

As a general rule, it was agreed that each EE should be run under the same testing conditions using one software codebase, which should be based on the JEM software codebase. An experiment is not to be established as a EE unless there is access given to the participants in (any part of) the TE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-B1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to EE work should identify individuals in addition to company names.

EE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the EE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish an EE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to EE activities were established as follows:

- Only qualified JVET members can participate in an EE.

- Participation in an EE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the EE should be announced and made available to all EE participants in a timely manner.

A separate branch under the experimental section will be created for each new tool include in the EE. The proponent of that tool is the gatekeeper for that separate software branch. (This differs from the main branch of the JEM, which is maintained by the software coordinators.)

New branches may be created which combine two or more tools included in the EE document or the JEM. Requests for new branches should be made to the software coordinators.

Don’t need to formally name cross-checkers in the EE document. To promote the tool to the JEM at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

Timeline:

T1 = JEM5.0 SW release + 4 weeks: Integration of all tools into separate EE branch of JEM is completed and announced to JVET reflector.

Initial study by cross-checkers can begin.

Proponents may continue to modify the software in this branch until T2

3rd parties encouraged to study and make contributions to the next meeting with proposed changes

T2: JVET-F meeting start – 3 weeks: Any changes to the exploration branch software must be frozen, so the cross-checkers can know exactly what they are cross-checking. An SVN tag should be created at this time and announced on the JVET reflector.

This procedure was again confirmed during the closing plenary of the third JVET meeting. It was further confirmed that the Common Test Conditions of JVET-B1010 are still valid, however the CTC encoder setting will be reflected in the config file that is attached to the JEM4.0 package.

## Software development

Software coordinators will work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- JEM4.0 including all adoptions from section 12.4 will be released within 2 weeks (2016-11-04)

- The results about coding performance of JEM4.0 will be reported by 2016-11-11

- Further versions may be released for additional bug fixing, as appropriate

- Encoder software optimized for luma-dependent quantizer adaptation will be developed as separate branch by the AHG on HDR/WCG within two weeks after JEM4.0 release.

For 360 video, the two available conversion software packages, 360tools in JVET-D0027 and PCT360 in JVET-D0021, will be merged. Software coordinators are Yuwen He and Vlad Zakharchenko. The merged software will be in C++ and follow the HM style conventions, and be hosted along with the HM and JEM software repositories, structured as a separate library. Direct path from conversion to encoding will be supported.

Timeline of conversion software release:

First release: (3 weeks – Nov 11) Emphasize Mandatory CTC, separate library, hosted on HHI site, HM 16.9 integration, remove neighboring sample pattern, CPP-PSNR, S-PSNR-I, calculation of multiple metrics in single pass of encoding, ISP

Second release: (6 Weeks – Dec 2) Viewport in same pipeline, TSP and SSP including confirmation by proponents, WS-PSNR metric for TSP and SSP (optional), JEM 4.0 integration

# Output documents and AHGs (update)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JVET-D1000 Meeting Report of the 4th JVET Meeting [G. J. Sullivan, J.-R. Ohm] [2017-01-11] (near next meeting)

Intermediate versions of the meeting notes (d0 … d7) were made available on a daily basis during the meeting.

JVET-D1001 Algorithm description of Joint Exploration Test Model 4 (JEM4) [J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce] [2016-11-18] (MPEG N16511)

See list of new adoptions under 12.4. During the closing plenary, no complaints were made about the accuracy of that list.

JVET-B1002 Call for test materials for future video coding standardization [A. Norkin, H. Yang, J.-R. Ohm, G. J. Sullivan, T. Suzuki]

stays valid (from 2nd meeting)

JVET-D1002 Work plan for assessment of test material [T. Suzuki] [2016-11-04] (MPEG N16512)

An initial version was reviewed and approved in the closing plenary.

- Update with new sequences

- Plan to make visual assessment with both classes A and B, longer sequences

- Plan to test more bit rates, to identify interesting test cases for CfE/CfP

- Upload bitstreams prior to next meeting, make pre-selection (suggested by Test chair) prior to next meeting

JVET-B1010 JVET common test conditions and software reference configurations [K. Suehring, X. Li]

remains valid (from 2nd meeting).

Note: Encoder settings reflected in the config file related to CTC in JEM4 (see changes under 12.4)

A directory had been installed in the ftp for bitstreams and results of anchors, but in the previous meeting cycle this had not yet been used. It is planned to upload bitstreams and Excel sheets after completion of anchors.

JVET-D1011 Description of Exploration Experiments on coding tools [E. Alshina, J. Boyce, L. Zhang] [2016-11-11] (MPEG N16513)

See list of EEs under Error: Reference source not found

Reviewed and approved during closing plenary on Friday.  Following changes were suggested relative to the first version:

- Change title of EE2 (as bilateral filter is not SAO style)

- For EE2, using bilateral filter as postprocessing (not preprocessing) should be investigated

- For EE4, another variant just adding one more context model should be investigated

EE1: Residual coefficients coding

EE2: Non-linear in-loop filters

EE3: Decoder-side motion vector derivation

EE4: MV coding

EE5: Chroma coding

EE6: Adaptive scaling for HDR/WCG material

(EE6 added during closing plenary, see notes under D0192)

JVET-D1020 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, S. Lasserre, D. Rusanovskyy] [2016-11-11] (MPEG N16514)

Approved in context of BoG D0192 presentation Friday morning.

JVET-D1030 JVET common test conditions and evaluation procedures for 360 video [E. Alshina, J. Boyce, A. Abbas, Y. Ye] [2016-11-11] (MPEG N16515)

A draft of this document was presented to the plenary Thu afternoon. During the closing plenary on Friday, the document was approved without further review, no expert had complaints about the procedure.

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header should be generated. This version should be sent to GJS and JRO for upload.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Tool evaluation (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate the exploration experiments. - Investigate interaction of tools in JEM and exploration experiment branches. - Discuss and evaluate methodologies and criteria to assess the benefit of tools, and how to ease the assessment of single tools in terms of encoder runtime. - Study and summarize new technology proposals. | E. Alshina, M. Karczewicz (co-chairs) | N |
| JEM algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-D1001 Algorithm Description of Joint Exploration Test Model 4 - Gather and address comments for refinement of the document - Coordinate with the JEM software development AHG to address issues relating to mismatches between software and text. | J. Chen (chair) E. Alshina, J. Boyce (vice chairs) | N |
| JEM software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of the JEM4.0 software packages and their distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver JEM4.0 software version and the reference configuration encodings according to JVET-B1010 common conditions. - Coordinate with AHG on JEM model editing and errata reporting to identify any mismatches between software and text, and make further updates to the software as appropriate. - Investigate parallelization for speedup of simulations. - Investigate the implementation of SCC coding tools in JEM. - Coordinate with AHG9 for integration of 360 video software. | X. Li, K. Suehring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of future video coding standards. - Identify and recommend appropriate test materials and corresponding test conditions for use in the development of future video coding standards. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Prepare visual quality comparison for HM16.13 and JEM4 at comparable bit rates, and make bitstreams available prior to the next meeting. - Study coding performance and characteristics of video test materials according to the work plan JVET-D1002. - Prepare for the visual assessment planned in the next meeting. | V. Baroncini, T. Suzuki (co-chairs), J. Chen, J. Boyce, A. Norkin (vice chairs) | N |
| Fast encoding, encoding complexity investigation, and configuration settings (AHG5) (jvet@lists.rwth-aachen.de) - Study encoder configuration settings of JEM4 software and suggest alternative configuration files for different operating points in terms of encoder complexity versus compression benefit. - Investigate and develop fast methods to reduce JEM4 encoding complexity. | K. Choi (chair), Y.-J. Chang, H. Huang, X. Li, P. Philippe, Y. Yasugi (vice chairs) | N |
| Simplification and harmonization of motion derivation tools (AHG6) (jvet@lists.rwth-aachen.de) - Investigate the potential of unification and simplification of interpolation filters for decoder-side MV derivation techniques in JEM3 (BIO and FRUC). - Investigate the adaptive threshold of MV refinement in BIO. - Investigate other potential simplifications of the decoder-side motion derivation tools. - Harmonization of affine, OBMC and motion derivation and compensation processes. | X. Li, E. Alshina (co-chairs) | N |
| JEM coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Study and refine conversion practices to create sequences in HDR/WCG containers with emphasis on BT.2020/BT.2100 and SMPTE ST 2084 - Study and refine test conditions for the JEM coding of HDR/WCG content - Create and release software supporting recommended conversion practices and test conditions as per JVET-D0192 and JVET-D1020 - Study objective metrics for quality assessment of HDR/WCG material. - Study coefficient scaling methods for HDR/WCG coding - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), S. Lasserre, D. Rusanovskyy (vice chairs) | N |
| 360 video coding tools and test conditions (AHG8) (jvet@lists.rwth-aachen.de) - Study the effect on compression of different projections formats, resolutions, and layouts of the input 360 video. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Generate anchors and a reporting template for the common test conditions. - Study viewpoint generation methods and evaluation criteria, and viewport-dependent video coding and streaming. - Study coding tools dedicated to 360 video, and their impact on compression efficiency and subjective quality. | J. Boyce (chair), A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye (vice chairs) | N |
| 360 video conversion software development (AHG9) (jvet@lists.rwth-aachen.de) - Prepare and deliver 360Lib software version and common test condition configuration files according to JVET-D1030 common conditions. - Produce documentation of software usage for distribution with the software. - Coordinate with JEM and HM software coordinators for integration of 360Lib within most recent versions of HM and JEM software packages. | Y. He, V. Zakharchenko (co-chairs) | N |

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Wednesday or Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–7.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 7.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Fri. 31 Mar. – Fri. 7 Apr. 2017, 6th meeting under WG 11 auspices in Hobart, AU.

- Fri. 14 – Fri. 21 Jul. 2017, 7th meeting under WG 11 auspices in Torino, IT.

- Wed. 18 – Wed. 25 Oct. 2017, 8th meeting under ITU-T auspices in Macao, CN.

The agreed document deadline for the 6th JVET meeting is XXday XX March 2017. Plans for scheduling of agenda items within that meeting remain TBA.

Sichuan University, Peking University and University of Electronic Science and Technology of China were thanked for the excellent hosting of the 4th meeting of the JVET. Samsung, Sichuan University, GBTech, and Huawei were thanked for providing viewing equipment.

The 4th JVET meeting was closed at approximately 1205 hours on Friday 21 October 2016.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the fifth meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately XXX people in total), were as follows:

- …