| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0010_r1 | Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0010 |
| --- | --- | --- | --- |

| Title: | Exploration Experiments on Coding Tools Report. |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Report |  |  |
| Author(s) or Contact(s): |  | Elena Alshina, Jill Boyce, Li Zhang | Email: | elena_a.alshina@samsung.com, jill.boyce@intel.com, lizhang@qti.QUALCOMM.com |
| Source: |  | EE coordinators |  |  |

____________________________

# Abstract

Six experiments on coding tools were agreed to carry out between JVET-D and JVET-E meetings in order to get better understanding of technologies considered for inclusion to the next version of JEM, analyse and verify their performance, complexity and interaction with existing JEM tools. This report summarises the status of each experiment.

# Introduction

JVET group coordinated experiments were conducted following principles were agreed in [1].

Key announcement related to EE activities were done using JVET reflector. Mainly discussion was carried on EE mail list which has 97 subscribers from 26 companies.

Exploration experiments were conducted according to the agreed during JVET-C time-line:

T1= 3 weeks after JVET-D meeting = Nov. 11: EE description was finalized.

T2 = JEM4.0 SW release + 4 weeks= Dec. 4: Initial integration for all tools into separate EE branch of JEM is available.

T3: JVET-E meeting start – 3 weeks = Dec. 23: Final version for each tool is available, start of official cross-check.

Contributions [2]-[12] were included into Exploration experiments. Proposals [2]-[10] were tested under JVET CTC compared to JEM4.0. EE6 on HDR and WCG used its own anchor and followed test procedure [13].

Figure 1 shows performance and complexity in terms of encoder run time for AI and RA configurations, respectively.

Table 1 summarizes results of all 6 exploration experiments.

- Under All Intra configuration (Y component)

- Under Random Access configuration (Y component)

Fig. 1. Complexity performance tradeoff in EE experiments.

Table 1. Summary of Exploration Experiments.

| # | Tests and sub-tests | Document | Y-BD-rate (Enc/DecTime) | Cross-check |
| --- | --- | --- | --- | --- |
| 1 | EE1: Residual Coefficients coding - Sing prediction for n=1, 2, 3, 4, 5 - Simplified encoder | JVET-E0051 (Orange) | N=4 AI: -0.5% (ET 1.49, DT 1.33) RA: -0.6% (ET 1.20, DT 1.04) AI: -0.5% (ET 1.41, DT 1.38) RA: -0.5% (ET 1.11, DT 1.03) | JVET-E0038 Samsung JVET-E0072 Fujitsu JVET-E0102 Sharp |
| 2 | EE2: Non-linear in-loop filters - Bilateral filter after inverse transform | JVET-E0031 (Ericsson) | AI: -0.4% (ET 1.07, DT 1.05) RA: -0.4% (ET 1.02, DT 1.00) LD: 0.4% (ET 1.02, DT 1.01) LDP: 0.2% (ET 1.03, DT 1.01) | JVET-E0043 Samsung JVET-E0091 Qualcomm JVET-E0044 Samsung JVET-E0034 Ericsson JVET-E0120 Huawei |
|  | - Peak SAO | JVET-E0066 (Qualcomm) | AI: -0.1% (ET 1.00, DT 1.08) RA: -0.2% (ET 1.00, DT 1.03) LD: 0.1% (ET 1.02, DT 1.03) LDP: 0.0% (ET 0.99, DT 1.07) |  |
| 3 | EE3: Decoder Side Motion Vector Derivation - High precision FRUC with additional candidates | JVET-E0060 Technicolor | RA: -0.2% (ET 1.01, DT 0.99) LD: -0.6% (ET 1.04, DT 0.98) LDP: -0.8%(ET 1.03, DT 1.00) | JVET-E0100 Sharp JVET-E0048 Samsung |
|  | - Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching | JVET-E0052 Hi-Silicon | RA: -0.4% (ET 1.02, DT 1.02) | JVET-E0049 Samsung JVET-E0088 Panasonic |
|  | - BIO w/o block extension | JVET-E0028 Samsung | RA: 0.0% (ET 0.92, DT 0.83) LD: 0.1% (ET 0.98, DT 0.95) | JVET-E0063 Technicolor JVET-E0124 Qualcomm |
| 4 | EE4: MV coding - Enhanced Motion Vector Difference Coding | JVET-E0076 (Qualcomm) | RA: -0.4% (ET 1.02, DT 0.99) LD: -0.2% (ET 1.02, DT 0.99) | JVET-E0046 Samsung JVET-E0101 Sharp JVET-E0122 Huawei JVET-E0111 Panasonic |
| 5 | EE5: Chroma coding - Multiple sets for CCLM - Multiple filters for CCLM - Average CCLM and angular | JVET-E0077 Qualcomm | AI: -0.4%(Y) -3.9% (U) -4.1% (V) (ET 1.03, DT 1.02) RA: -0.2%(Y)-3.4% (U)-3.2% (V) (ET 1.01, DT 1.00) | JVET-E0045 Samsung JVET-E0080 ETRI JVET-E0097 Sharp JVET-E0098 KDDI |
|  | - Multiple Direct Modes | JVET-E0062 Qualcomm | AI: -0.2% (Y)-1.5% (U) -1.4% (V) (ET 1.00, DT 1.00) RA: -0.0%(Y)-1.1%(U) -1.0% (V) (ET 0.99, DT 1.00) | JVET-E0099 KDDI |
| 6 | EE6: Adaptive scaling for HDR/WCG material - Adaptive scaling | JVET-E0055 Qualcomm | Multiple metrics defined in HDR/WCG test conditions JVET-D1020 | JVET-E0123 Sharp |
|  | - Dequantization and Scaling | JVET-E0081 Sharp |  | JVET-E0125 Qualcomm |

# List of Experiments

## EE1: Residual Coefficients coding

Sign of luma coefficients is predicted. Difference between prediction and actual sign is coded with CABAC contexts. The number of additional CABAC contexts is 2.

If n signs are predicted the n+1 partial inverse transforms and 2n border-cost measurements (number of sign combination hypotheses) are performed.

Decoder needs to have the reconstructed coefficients available before parsing the signs.

Questions recommended to be answered during EE tests.

[Q]: How gain and complexity increase with increment for number of predicted signs?

[A]: Performance gain in RA configuration and encoding run time are shown in Table below.

| n | RA Y-BD-rate gain | Enc. Run time, % |  |
| --- | --- | --- | --- |
| 1 | 0.3 | 110 |  |
| 2 | 0.4 | 114 |  |
| 3 | 0.5 | 117 |  |
| 4 | 0.6 | 120 |  |
| 5 | 0.6 | 123 |  |

[Q]: What is the complexity and gain impact if residue sign prediction processing is moved from it's current position in RDO to final encode?

[A]: Simplified version provides almost identical performance when hiding 4 signs (0.6%  0.5% in RA test) with 9% reduction for encoder run time (120% -> 111%).11.

Summary: Gain increases roughly linearly if more signs are predicted for n<5. Maximum gain demonstrated in RA configuration is 0.5% comes with 20% extra encoder run time. Similar gain is observed in AI configuration, but run time increment ~50% for encoder and ~30% for decoder.

## EE2: Non-linear in-loop filters

Bilateral after inverse transform. The bilateral 5-taps filter operates on decoded sample values after inverse transform. The shape of filter is a plus sign. The strength of the filter is based only on the TU size and QP. The size of the table is 3(TU size)*1024(pixel diff)*35(QP)*2 bytes (per element) = 210K bytes. No additional parameters are determined during encoding and no new syntax elements are proposed for turning on or off the filter.

Questions recommended to be answered during EE tests.

[Q]: It was asked whether it would be better to perform the filtering not in the intra prediction loop, but rather before de-blocking?

[A]: Test #2 was designed to answer this question. Performance drop (0.5%, 0.5%, 2.1%, 1.7% BD rate increase for AI, RA, LDB, LDP, respectively according to cross-check report) was observed. Performance of Test #2 will be updated after bug fix.

[Q]: What is visual quality effect?

[A]: Proponent has provided examples in contribution. Viewing test is needed for confirmation.

[Q]: Could we achieve similar effect by residual smoothing prior to the encoding?

[A]: Instead this proponent investigated application of this tool as post-filter in Test #3. Drop was observed (1.2% in AI, 0.4% in MC configuration)

Summary: 0.4% gain is observed for AI and RA. But coding loss is observed in low-delay configuration. Proponent provided solution for this drop in new contribution. Run time increment is 2-3% (encoder) and ~0% (decoder) for motion compensation test scenarios but 7% (encoder) and 5% (decoder) for AI.

Peak SAO. Peak SAO, four neighboring pixels are utilized to classify the current pixel into one of the three categories. Samples in two categories are corrected with a signaled offset. The offset is selected based on the average pixel value difference between the current pixel and its selected neighboring pixels and a normalization factor. The offset parameters are signaled at picture level.

Questions recommended to be answered during EE tests.

[Q]: What is the impact on subjective quality?

[A]: No data provided in contribution. Cross-check reports provide examples for visual quality improvement. Viewing test is needed for confirmation.

[Q]: Could new method replace the edge offset in SAO?

[A]: Test #5 was designed to answer this question. Proponent provides the results of disabling the whole SAO which shows 0.1% coding gain for AI, 0.0% for RA, and 0.3% loss for LD. As was reported in cross-check, Peak SAO and EO are both helpful, but there is some overlap between these two methods.

[Q]: Can it be added as an additional SAO type and signaled on CTU level?

[A]: Test #6 was designed to answer this question. Not tested, but based on Test#5 it seems the answer is “not”.

Summary: 0.1% and 0.2% gain is observed for AI and RA configuration with no encoder run-time increment, but 1…8% decoder run-time increment. Coding loss is observed in low-delay configuration (mostly class E).

## EE3: Decoder Side Motion Vector Derivation

High precision FRUC with additional candidates Technology contains:

- Increase of the precision in FRUC until the finer internal one (currently 1/16);

- Addition of Motion Vector Candidates in FRUC, by adding the 2 AMVP candidates in the front of the FRUC candidate list for AMVP CUs, and up to 5 Spatial Candidate(s) in the end of FRUC candidate list, both for entire CUs, and FRUC CUs’ sub-blocks.

- Remove some FRUC candidates from the list

Questions recommended to be answered during EE tests.

[Q]: What is the performance impact from increment the precision in FRUC" up to 1/16?

[A]: Test #1 was designed to answer this question. Performance impact from this change is relatively low: 0.0%(RA)/0.1%(LD)/-0.1%(LDP).

[Q]: How many "Additional of Motion Vector Candidates" in total?

[A]: Based on test 3, the numbers of added candidates are:

-          +4 for AMVP CUs

-          Between +2 and -31 for Merge CUs (a negative number meaning that less candidates are used than in JEM-4.0).

[Q]: Can some candidates (ex. spatial) be re-ordered?

[A]: Indeed, for sub-blocks of merge CUs, test 2 re-orders the candidate list as follows, compared to test 1.2:

-          top and left neighboring motion vectors are moved from ending place to 2nd place.

-          added top-left and top-right neighboring motion vectors are moved from ending place to 3rd place.

RA: -0.2% (ET 1.01, DT 1.01)  RA: -0.2% (ET 0.97, DT 1.00)

LD: -0.6% (ET 1.05, DT 1.02)  LD: -0.7% (ET 1.01, DT 1.01)

LDP: -0.8% (ET 1.03, DT 1.01)  LDP: -0.8% (ET 0.99, DT 1.01)

[Q]: Can some candidates (among initial FRUC candidates or added spatial candidates) be removed?

[A]: Test #3 was designed to answer this question:

RA: -0.2% (ET 0.97, DT 1.00)  RA: -0.2% (ET 1.00, DT 0.99)

LD: -0.7% (ET 1.01, DT 1.01)  LD: -0.6% (ET 1.04, DT 0.98)

LDP: -0.8% (ET 0.99, DT 1.01)  LDP: -0.8% (ET 1.03, DT 1.00)

Despite the low complexity decrease obtained with the removals of test 3 (1% enc and 2% dec time), the number of candidates for sub-blocks is drastically reduced and smoothed.

Test 3 reduces the theoretical complexity and simplifies the design of the FRUC tool.

Summary: 0.2%(RA) / 0.7% (LD) / 0.8%(LDP) gain is observed with run-time increment -3% to 1% (encoder) and 0-1% (decoder. (test2)

0.2%(RA) / 0.6% (LD) / 0.8%(LDP) gain is observed with run-time increment 0/4% (encoder) and -2%/0% (decoder) (test3)

Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching “The third” decoder side MV derivation technique for JEM. Applied under conditions:

- Not OBMC

- L0 and L1 reference are from opposite time directions

- Merge

- Not Local Illumination Compensation

- Not Affine MC

- Not FRUC

It similarly to FRUC it operates at sub-PU level and BIO is applied on top.

First motion compensation for luminance is done with MV signalled in the bit-stream for block extended by one row and one column on each size. So the memory access required for this method is (W+2+7) (H+2+7) instead (W+7) (H+7) in HEVC motion compensation. This is 28% higher for HEVC the worst case (88 bi-predicted PU).

Then decoder searches for MV refinement in L0 and L1. In first round of search up to 8 MV candidates (1 int-pel displacement in vertical and horizontal directions) are checked. So up to 9 calculations for SAD and up to 8 comparisons are needed. The second round of MV search at decoder-side refines MV with 1/2-pel precision. Additionally up to 8 calculations for SAD and up to 8 comparisons are needed. Additional gain from this 1/2-pel refinement is 0.1%. But this second round doesn’t require additional memory access.

After MV refinement is found MC for both three colour components is performed with new MV.

Questions recommended to be answered during EE tests.

[Q]: Test performance and complexity on JEM4.0 platform.

[A]: 0.4% gain is observed in RA case with ~2% encoder and decoder run-time increment.

[Q]: How does performance depend on number of MV0’ and MV1’candidates checked on decoder side?

[A]: Proponent provided 2 sets of test data: with int-pel and with 1/2-pel MV refinement precision. Later one estimates roughly twice smaller number of MV candidates and provides the most part of the gain (0.3%).

Summary: 0.4%(RA)  gain is observed ~2%  encoder and decoder run-time increment. Major source of the gain is MV refinement with int-pel precision (0.3%).

BIO w/o block extension JEM4.0 BIO performs prediction and calculates gradients for the extended block WH  (W+4)  (H+4). Block extension has been removed. After this modification the memory bandwidth of BIO is equal to regular bi-directional motion compensation. Computational complexity also has been reduced which results in both encoder and decoder run time reduction.

Questions recommended to be answered during EE tests.

[Q]: Test performance and complexity on JEM4.0 platform.

[A]: Performance change relatively to JEM4.0 is 0.0%(RA)/0.1%9(LD). Encoder run time reduction is 8%, decoder run time reduction is 17% (In RA). Contribution provides memory access and computational complexity analysis. For corner case (44 block) number of multiplications has been reduced by factor 3, and memory access has been reduced by factor 2.

Summary: W/o performance drop in RA encoder run time can be reduced by 8%, decoder run time can be reduced by 17% if BIO doesn’t use block extension. Memory for modified BIO is equivalent to the regular bi-directional MC.

## EE4: MV coding

Modified MVD coding is tested here. The proposed method includes two elements, a) 4-pel accuracy for MVD signaling (in addition to ¼ pel and integer-pel MVD accuracy, and b) switchable binarization and context model selection.

First a flag is signaled to indicate whether ¼ luma sample MV precision is used in a CU. When the first flag indicates that ¼ luma sample is not used, another flag is signaled to indicate whether integer luma sample MV precision or four luma samples MV precision is used.

The binarization and context modeling are dependent on the MVD precision and the POC distance between the current frame and the reference frame.

Number of CABAC contexts for MVD coding 2  5.

Questions recommended to be answered during EE tests.

[Q]: What is the performance impact of 4-pel MVD accuracy?

[A]: Test #1 was designed to answer this question. 4-pel MVD provides 0.3%(RA)/0.1%(LD) gain.

[Q]: What is the performance impact of switchable binarization and context model selection?

[A]: Test #2 was designed to answer this question. New binarization and contexts selection 4 provides 0.1%(RA)/0.0%(LD)

Summary: 0.4% (RA) and 0.2% (LDB) gain can be achieved with ~2% increment of  encoder run-time, w/o increment of decoder run  variation. Major gain comes from 4-pel MVD accuracy. Switchable binarization and context model selection requires 3 additional CABAC contexts.

## EE5: Chroma coding

Enhanced Cross-component Linear Model Intra-prediction includes following new elements:

- Multiple linear models, samples are grouped in multiple sets;

  - Threshold is calculated as the average value of the neighboring reconstructed Luma samples. A neighboring sample with Rec’L[x,y] <= Threshold Rec'Lx,y≤Thresholdis classified into group 1; while a neighboring sample withRec'Lx,y>Threshold Rec’L[x,y] > Threshold is classified into group 2 and two CCLM models are used for 2 groups of samples.

- multiple-filter LM where filter is used prior to feeding samples into the model;

  - Rec'Lx,y=(RecL2x,2y+RecL2x+1,2y+1)≫1

- Average of angular and LM mode.

Weights are {1/2,/1/2}

The number of additional CABAC context is 3.

Questions recommended to be answered during EE tests.

[Q]: Report about the contribution to gain, and the complexity of the three different elements of the proposal.

[A]: Tests #1,2,3 were designed to answer this question.  BD-rate gain in AI test is summarized in Table below

| Test | Y | U | V | Enc | Dec |
| --- | --- | --- | --- | --- | --- |
| Multiple sets in CCLM | -0.25% | -1.75% | -1.87% | 100% | 100% |
| Multiple sets in CCLM+ multiple filters | -0.36% | -2.64% | -3.11% | 102% | 101% |
| Multiple sets in CCLM + averaging | -0.33% | -3.17% | -3.02% | 102% | 101% |

[Q]: Question is raised whether it is a problem that the number of samples in the two linear models may not be a power of two.

[A]: A look-up table is utilized to replace the shift-operation when the number of samples is not in the form of 2N.

Summary: Maximum gain could be achieved in AI configuration is 0.4% (Y), 3.9% (U)  and 4.1% (V) comes with ~2% (encoder) and ~1% decoder run time  increment. Major gain comes from multiple CCLM models 0.3% (Y),  1.8% (U)  and 1.9% (V).

Multiple Derived Modes The list of Chroma Intra prediction modes was modified as follows (changes compared to JEM4.0 are highlighted):

- 5 chroma prediction modes from left, above, Planar and DC modes, below-left, above-right, and above-left spatial neighboring blocks of merge mode

- Planar and DC modes

- derived modes are added, those intra modes are obtained by adding -1 or +1 to the angular modes which are already included into the list

- default modes are added in the order of: Vertical (Mode 18), Horizontal (Mode 50), Mode 2, Mode 34, Mode 66, Mode 10, Mode 26

- if any of the four default modes (Planar, Horizontal, Vertical and DC modes) is not included in the list, the missing default modes are used to replace the last one or more candidates

Overall number the chroma intra mode candidates is 10. For fair comparison with JEM4.0 encoder performs the same number (6) of rate-distortion estimations.

Questions recommended to be answered during EE tests.

[Q]: What is the performance of multi direct modes for chroma coding?

[A]: Major gain is observed for Chroma 1.4% in AI , 1.1% in RA and 0.3% in LD configurations.

Summary: 0.2%(Y) 1.2%(U) 1.2%(V) gain in AI and 0.0%(Y) 1.1%(U) 1.0%(V) gain in RA can be achieved w/o increment of run-time (0% for both encoder and decoder) by extension of number Chroma intra modes candidates up to 10.

## EE6: Adaptive scaling for HDR/WCG material

“HDR super-anchor” was developed during HDR/WCG materials coding by HEVC study by JCTVC. The key element of super-anchor is enabling of delta QP signalling. Delta QP is selected  at encoder side based on average luminance value in a block

Delta QP = s = LUT [Mean (OrigY)].

In addition to delta QP adaptation, HDR super-anchor also implemented chroma QP offset.

This encoding technique anchor was used in EE6.

Adaptive scaling in D0118 is an alternative to the residual scaling. The motivation is to remove the overhead of delta QP that is signaled by the encoder to compensate for the reshaping of quantization noise as a function of brightness in next generation containers such as ST 2084. This is accomplished by adapting the scaling of AC coefficients based on the DC coefficient and reconstructed prediction mean. In the new proposal, scale determination method is modified to allow the scaling of all coefficients and scaling operation can be moved after dequantization block in decoder for simpler design

s = LUT [Mean (Prediction)].

In addition to this change to the scale derivation process, adaptive scaling is also applied to the DC coefficient, in contrast to approach of D0124.

Dequantization and Scaling in D0124 In this test instead of delta QP signaling scaling factor is decided per each block based on average value of prediction and de-quantized DC coefficient:

s = LUT [Mean (Prediction)+invQuant(DC)].

Resulting scale is not applied to the DC coefficient.

Results summary

Seq 1 (JVET test set in HDR representation)

|  | wPSNR |  |  |  |  |  |  |  |  |  |  | PSNR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  |  | V | tPSNRY | DE100 |  | PSNRL100 |  |  | Y | U |  |  |  | V |  |  |  |  |  |  | EncT |  |  | DecT |  |  |  |  |
|  | All Intra JVET10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E0081 | -1.7% |  | -1.5% | -0.8% |  | 1.2% | -0.8% |  |  |  | -1.3% | 1.2% |  | 0.8% |  |  |  | 1.5% |  |  |  |  | 108% |  |  |  | 104% |  |  |  |  |
| E0055 | -2.0% |  | -1.2% | -1.1% |  | 1. 5% | -0.3% |  |  |  | -1.2% | 1.7% |  | 1.3% |  |  |  | 1.5% |  |  |  |  | 106% |  |  |  | 102% |  |  |  |  |
|  | Random Access JVET 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E0081 | -2.8% |  | -3.0% | -2.0% |  | -0.4% |  | -1.9% |  | -2.4% |  | -0.3% |  |  | 0.5% |  |  |  |  | 1.3% |  |  | 102% |  |  |  | 98% |  |  |  |  |
| E0055 | -2.3% |  | -2.6% | -2.6% |  | 0.2% |  | -1.4% |  | -1.9% |  | 0.3% |  |  | 0.8% |  |  |  |  | 1.0% |  |  | 103% |  |  |  | 100% |  |  |  |  |
|  | Low Delay-B JVET 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E0081 | -3.5% |  | -4.5% | -3.0% |  | -0.8% | -2.9% |  |  |  | -2.9% | -0.8% |  |  | 0.1% |  |  |  |  | 0.6% |  |  | 100% |  |  |  | 96% |  |  |  |  |
| E0055 | -2.9% |  | -3.8% | -3.3% |  | -0.4% | -1.7% |  |  |  | -2.2% | -0.4% |  |  | 0.2% |  |  |  |  | 0.5% |  |  | 103% |  |  |  | 100% |  |  |  |  |

Seq 2 (HDR test set)

|  |  | wPSNR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PSNR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  |  |  | V |  |  |  |  | tPSNRY |  |  |  |  | DE100 |  |  |  | PSNRL100 |  |  |  |  |  |  |  |  |  | Y |  |  |  | U |  |  |  |  |  |  |  |  |  |  |  |  | V |  |  |  |  |  |  |  |  | EncT |  |  |  |  |  |  | DecT |  |  |  |  |  |  |  |  |  |  |
|  |  | All Intra JVET10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E0081 |  | -0.8% |  |  |  | -1.5% |  | -2.3% |  |  |  |  |  |  | 0.2% |  | -0.8% |  |  |  |  |  | -0.9% |  |  |  |  |  |  |  | 0.3% |  |  |  |  |  |  |  | 0.5% |  |  |  |  |  |  |  | 0.3% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E0055 |  | -1.6% |  |  |  | -2.0% |  | -2.9% |  |  |  |  |  |  | 1.0% |  | -0.5% |  |  |  |  |  | -2.1% |  |  |  |  |  |  |  | 1.0% |  |  |  |  |  |  |  | 2.5% |  |  |  |  |  |  |  | 2.9% |  |  |  |  |  |  |  | 104% |  |  |  |  |  |  |  |  |  |  |  | 102% |  |  |  |  |  |  |  |  |
|  |  | Random Access JVET 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E0081 |  | -1.0% |  |  |  | -3.2% |  | -5.0% |  |  |  |  |  |  | 0.2% |  |  | -1.3% |  |  |  |  | -1.0% |  |  |  |  |  |  |  |  | 0.2% |  |  |  |  |  | 0.2% |  |  |  |  |  |  |  | -0.7% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E0055 |  | -1.3% |  |  |  | -3.1% |  | -5.2% |  |  |  |  |  |  | 1.1% |  |  | -0.5% |  |  |  |  | -1.6% |  |  |  |  |  |  |  |  | 1.1% |  |  |  |  |  | 2.6% |  |  |  |  |  |  |  | 1.8% |  |  |  |  |  |  |  |  |  | 105% |  |  |  |  |  |  |  |  |  |  |  |  | 101% |  |  |  |  |  |  |

Questions recommended to be answered during EE tests.

[Q]: What is performance of JVET-D0118 and JVET-D0124 using the content and test conditions described in JVET-D1020 JVET common test conditions and evaluation procedures for HDR/WCG video?

[A]: EE081/E0055 provide:

Shown in tables above.

[Q]: What is the visual quality assessment of JVET-D0118 and JVET-D0124?

[A]: Viewing is planned during the meeting,

[Q]: What is the benefit of combining different aspects of studied technologies?

[A]: One is the sub-set of another.

Summary: Based on results and knowledge obtained during EE it is suggested to refine HDR/WCG test procedure, unify excel template for reporting results, get agreement on anchor and metrics, conduct viewing and agree on visual assessment procedure for content in PQ2100 representation during the meeting

# Recommendation

- Review ALL EE tests’ contribution and cross-checks.

- Make decisions on each part on technology under consideration by reviewing EE results.

- It is important to make EE results reproducible. Which means that the following information should be provided:

  - the exact JEM macro settings

  - the configuration files for HDRConvert, HDRMetrics and for the JEM

  - the objective results

Even EE description requests full test results to be available by T3 this is not always the case.

# Acknowledgement

- Thanks all proponents for providing EE s/w in time with proper announcement and explanation about modification in JVET reflector.

- Thanks all volunteers provided comprehensive cross-check.

# References

[1] E. Alshina, J. Boyce, Xiaoyu Xiu, Yao Jen Chang, Vadim Seregin, S.-H. Kim (EE coordinators) Description of Exploration Experiments on Coding Tools JVET-C1010, 3nd Meeting, May. 2016

[2] Residual Coefficient Sign Prediction [F. Henry, G. Clare (Orange)] JVET-D0031

[3] Bilateral filter after inverse transform [J. Ström, P. Wennersten, Y. Wang, K. Andersson, J. Samuelsson (Ericsson)] JVET-D0069

[4] Peak Sample Adaptive Offset [M. Karczewicz, L. Zhang, J. Chen, W.-J. Chien (Qualcomm)] JVET-D0133

[5] High precision FRUC with additional candidates [A. Robert, F. Le Léannec, T. Poirier (Technicolor)] JVET-D0046

[6] Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching [X. Chen, J. An, J. Zheng (HiSilicon)] JVET-D0029

[7] AHG6: On BIO memory bandwidth [A.Alshin, E.Alshina (Samsung)] JVET-D0042

[8] Enhanced Motion Vector Difference Coding [J. Chen, W.-J. Chien, N. Hu, V. Seregin, M. Karczewicz, X. Li (Qualcomm)] JVET-D0123

[9] Enhanced Cross-component Linear Model Intra-prediction [K. Zhang, J. Chen, L. Zhang, M. Karczewicz (Qualcomm)] JVET-D0110

[10] Multiple Direct Modes for chroma intra coding [L. Zhang, W.-J. Chien, J. Chen, X. Zhao, M. Karczewicz (Qualcomm)] JVET-D0111

[11] EE9: On adaptive scaling for extended colour volume materials [D. Rusanovskyy, D. B. Sansli, A. K. Ramasubramonian, J. Sole, M. Karczewicz  (Qualcomm)] JVET-D0118

[12] EE9/AHG7: On Dequantization and Scaling for Extended Colour Volume Materials [J. Zhao, S.-H. Kim, K. Mira and A. Segall (Sharp)] JVET-D0124

[13] A. Segall, S. Lasserre, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-D1020, Oct.2016.