| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0028 |
| --- | --- |

| Title: | EE3: bi-directional optical flow w/o block extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A.Alshin, E.Alshina Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

In EE3 Bi-directional optical flow algorithm was modified in order not to access samples on reference frame other than motion uses. This contribution reports test results of this modification. Average performance change is 0.0% in RA and 0.1% in LDB. This simplification leads to the 8% encoder and 17% decoder run-time reduction in RA test. For corner case (44 bi-predicted sub-blocks) proposed modification reduces the number of multiplications by factor 2 and memory access by factor 3.

# Introduction

HEVC standard uses 8-taps interpolation filter for Luma and 4-taps filter for Chroma motion compensation. Bi-prediction is restricted for blocks smaller than 88. MC for small blocks 44 is not allowed in HEVC. With those restrictions memory bandwidth of HEVC is ~15% higher than H.264/AVC, and this was considered to be acceptable at the time HEVC was developed.

JEM [1] includes multiple tools which increase memory bandwidth compared to HEVC. This contributions studies one of JEM4.0 tools namely bi-directional optical flow (BIO). It was found that in a form BIO implemented in JEM4.0 small block extension (2 lines from each side of block) is needed for blocks compensated with BIO, which results in some memory band-width increment (despite BIO uses shorter interpolation filters). The reason for this was found. Modification of BIO which doesn’t require block extension has been tested in EE3.

# Test description

So, in order to perform BIO calculations (4) for both texture and gradients are calculated for the block which is 2 lines extended on each side (Fig. 1). Additionally block extension for interpolation is needed. This block extension depends on interpolation filter length T.

Since because Luma interpolation filter length is 8, for luminance W×HWH  block motion compensation HEVC requires accessing the memory (W+7)(H+7) in reference frame.

Due to the averaging all values for BIO in 55 (5=2M+1) surrounding of predicted sample the total memory access is (W+T-1+2M)(H+T-1+2M)= (W+11)(H+11).

Figure 1: BIO memory access: JEM4.0. (left), tested (right).

In order to equalize memory access between regular motion compensation and BIO in [2] the way of averaging was modified and only texture and gradients inside predicted block are used in averaging.

σ1i,j=j',j'∈Ω; 0≤i'≤W-1;0≤j'≤H-1Ix1i',j'+Ix0i',j'2 wi',j'

σ3i,j=i',j'∈Ω; 0≤i'≤W-1;0≤j'≤H-1Ix1i',j'+Ix0i',j'I1i',j'-I0i',j'wi',j'

σ2i,j=i',j'∈Ω; 0≤i'≤W-1;0≤j'≤H-1Ix1i',j'+Ix0i',j'Iy1i',j'+Iy0i',j'  wi',j'

σ5i,j=i',j'∈Ω;0≤i'≤W-1;0≤j'≤H-1Iy1i',j'+Iy0i',j'2wi',j';

σ6i,j=i',j'∈Ω;0≤i'≤W-1;0≤j'≤H-1Iy1i',j'+Iy0i',j'I1i',j'-I0i',j'wi',j'.

Changes compared to the BIO design in [1] are highlighted in yellow. This additional weights wi',j' allow compensating the lack of neighboring pixels if current pixel near boundary. Weights wi',j'  are all zeros outside of predicted block, so instead i',j'∈Ω in (4) we have  i',j'∈Ω;0≤i'≤W-1;0≤j'≤H-1 in (5). It means amount of summand in (5) is fewer than in (4) and both texture and gradients values outside of predicted block are not needed.

Table  1. Complexity of bi-directional motion compensation in NN block.

|  | HEVC | JEM4.0 BIO | Proposed BIO |
| --- | --- | --- | --- |
| Memory access | 2*(N+7)(N+7) | 2*(N+4+7)(N+4+7) | 2*(N+7)(N+7) |
| Multiplications | 2*8*{(N+7)N+NN} | 2*8*{(N+4+7)(N+4)+(N+4)(N+4)} 2*6*{(N+4+5)(N+4)+(N+4)(N+4)} 2*6*{(N+4+5)(N+4)+(N+4)(N+4)} | 2*8*{(N+7)N+NN} 2*6*{(N+5)N+NN} 2*6*{(N+5)N+NN} |
| Divisions | 0 | 2*NN | 2*NN |

The complexity of bi-directional motion compensation in HEVC, BIO in JEM4.0 and proposed variant is summarized in Table 1. Complexity associated with block extension is highlighted in yellow.

With proposed modification memory access of BIO is identical to the bi-directional motion compensation in HEVC.

For corner case (44 sub-blocks) the number of multiplication in bi-predicted block has been reduced by factor 2, memory access has been reduced by factor 3.

Additionally in proposed modification BIO implementation the size of additional buffers for BIO has been reduced by factor 3.

# Test results

Implementation for changes described above was uploaded https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-4.0-EE3-D0042/ and tested under JVET CTC for random access and low-delay configurations (only those configurations are affected by BIO).

Summary of test results is shown in Table 2.

Table 2. Test results summary.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM-16.6-JEM-4.0rc1_r328 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.0% | -0.1% | -0.1% | 93% | 84% |
| Class A2 | 0.0% | 0.0% | -0.1% | 91% | 81% |
| Class B | 0.0% | 0.0% | 0.0% | 92% | 83% |
| Class C | 0.1% | 0.0% | 0.0% | 92% | 82% |
| Class D | 0.0% | 0.0% | 0.1% | 91% | 83% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 0.0% | 0.0% | 0.0% | 92% | 83% |
| Class F (optional) | -0.1% | -0.1% | 0.0% | 96% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over HM-16.6-JEM-4.0rc1_r328 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.1% | 97% | 94% |
| Class C | 0.2% | 0.2% | -0.1% | 98% | 94% |
| Class D | 0.2% | 0.2% | 0.1% | 99% | 97% |
| Class E | 0.1% | 0.6% | 0.1% | 98% | 92% |
| Overall (Ref) | 0.1% | 0.3% | 0.0% | 98% | 95% |
| Class F (optional) | 0.0% | -0.1% | -0.4% | 101% | 99% |

We believe the main reason for run-time reduction is significantly smaller number of computations proposed modification of BIO requires.

Proponents would like to thank experts from Technicolor for comprehensive cross-check.

# Conclusions

This contribution eliminates block extension in BIO and so equalizes memory access in BIO and HEVC bi-directional motion compensation. Contribution verifies that w/o block extension BIO performance remains roughly the same (performance change 0.0% in RA and 0.1% in LDB) while run-time of JEM can be reduced significantly (8% encoder and 17% decoder run time reduction in RA). For corner case (44 bi-predicted sub-blocks) proposed modification reduces the number of multiplications by factor 2 and memory access by factor 3. It is proposed to adopt proposed modification of BIO to the next version of JEM.

# References

[1] J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 3 (JEM3),” JVET-C1001, May. 2016, Geneva

[2] A.Alshin, E. Alshina “AHG6: On BIO memory bandwidth,” JVET-D0042, Oct. 2016, Chengdu

# Patent rights declaration

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).