| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0032 |
| --- | --- |

| Title: | Bilateral filter strength based on prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström Per Wennersten Kenneth Andersson Jack Enhorn Färögatan 6 16480 Stockholm Sweden | Tel: Email: | +46702672192 jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

An updated version of the bilateral filter proposed in JVET-D0069 is presented. The filter strength is now lower for blocks using inter prediction, and it is reported that this results in additional BD rate decreases of 0% / -0.04% / -0.88% / -0.74% for AI / RA / LDB / LDP at no measurable added complexity. In comparison with JEM-4.0 the BD rate results are reported to be -0.42% / -0.46% / -0.52% / -0.54% with encoding time increase of 7% / 3% / 5% / 4% and decoding time increase of 5% / 3% / 5% / 3%. For screen content (class F) the BD rate improvements are -1.84% / -1.27% / -1.31% / -1.62%.

# Introduction

At the JVET meeting in Chengdu the use of bilateral filtering after the inverse transform was proposed in JVET-D0069 [1] and was also tested in EE2 on top of JEM-4.0 in JVET-E0031 [2].

A bilateral filter works by basing the filter weights not only on the distance to neighboring samples but also on their values. A sample located at (i, j), will be filtered using its neighboring sample (k, l). The weight ωi,j,k,l is the weight assigned for sample (k, l) to filter the sample (i, j), and it is defined as

ωi,j,k,l=e- i-k2+j-l22σd2 - Ii,j-Ik,l22σr2.                (Eq.1)

Here, I(i, j ) and I(k, l) are the original reconstructed intensity value of samples (i, j) and (k,l) respectively.

σd is the spatial parameter, and σr is the range parameter. The properties (or strength) of the bilateral filter are controlled by these two parameters. Samples located closer to the sample to be filtered, and samples having smaller intensity difference to the sample to be filtered, will have a larger weight than samples further away and with larger intensity difference. In the original contribution JVET-D0069 it is proposed to set σd based on the transform unit size (Eq. 2), and σr to be set based on the QP used for the current block (Eq. 3) according to

σd=0.92-minTU block width, TU block height40                 (Eq.2)

σr=maxQP-172, 0.01.                                                              (Eq.3)

The application of the bilateral filter after inverse transform can improve the objective coding efficiency for all intra and random access configuration. In this contribution we further improve the bilateral filter performance by reducing the filter strength for inter prediction blocks compared to intra prediction blocks.

# Filter strength dependent of prediction mode

Inter predicted blocks typically have less residual than intra predicted blocks and therefore it makes sense to filter the reconstruction of inter predicted blocks less.

We therefore set the filter strength for intra predicted blocks as before but for inter predicted blocks we use the following spatial weight:

σd= 0.72 – minTU block width,  TU block height40                        (Eq. 4)

# Results

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,17% | 0,17% | 0,09% | 107% | 106% |
| Class A2 | -0,37% | 0,02% | 0,09% | 106% | 105% |
| Class B | -0,17% | 0,18% | 0,20% | 111% | 109% |
| Class C | -0,81% | 0,09% | 0,05% | 107% | 104% |
| Class D | -0,56% | 0,10% | 0,05% | 106% | 100% |
| Class E | -0,56% | 0,03% | 0,10% | 107% | 106% |
| Overall | -0,42% | 0,11% | 0,10% | 107% | 105% |
| Class F (optional) | -1,84% | -0,28% | -0,27% | 104% | 105% |

|  | Random access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,24% | 0,01% | 0,05% | 105% | 102% |
| Class A2 | -0,60% | -0,36% | -0,29% | 103% | 103% |
| Class B | -0,45% | 0,06% | -0,13% | 104% | 104% |
| Class C | -0,59% | -0,13% | -0,19% | 102% | 101% |
| Class D | -0,44% | -0,23% | 0,15% | 104% | 105% |
| Class E |  |  |  |  |  |
| Overall (Ref) | -0,46% | -0,12% | -0,08% | 103% | 103% |
| Class F (optional) | -1,27% | -0,24% | -0,05% | 104% | 104% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,37% | 0,34% | 0,26% | 108% | 107% |
| Class C | -0,55% | 0,13% | 0,09% | 105% | 106% |
| Class D | -0,64% | 0,43% | 0,14% | 104% | 104% |
| Class E | -0,58% | 0,98% | -0,57% | 102% | 103% |
| Overall (Ref) | -0,52% | 0,43% | 0,03% | 105% | 105% |
| Class F (optional) | -1,31% | -0,04% | 0,20% | 105% | 104% |

|  | Low delay P |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,46% | -0,09% | -0,12% | 105% | 104% |
| Class C | -0,61% | -0,08% | 0,12% | 106% | 106% |
| Class D | -0,66% | 0,19% | 0,17% | 100% | 95% |
| Class E | -0,41% | -0,31% | 0,58% | 108% | 108% |
| Overall (Ref) | -0,54% | -0,06% | 0,14% | 104% | 103% |
| Class F (optional) | -1,62% | 0,00% | -0,48% | 107% | 106% |

# Conclusions

Due to the large improvements primarily seen for low delay settings, we suggest that this modified version of the bilateral filter is considered for inclusion instead of the original version proposed in JVET-D0069.

# References

[1] J. Ström, P. Wennersten, Y. Wang, K. Andersson, J. Samuelsson,  ”Bilateral Filter After Inverse Transform”, JVET-D0069, Chengdu, China, 15–21 October 2016

[2] J. Ström, P. Wennersten, K. Andersson, J. Enhorn, ”EE2-JVET-D0069 Bilateral Filter Test1, Test2 and Test3”, JVET-E0031, Geneva, Switzerland, January 2017

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).