| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0036 |
| --- | --- |

| Title: | On Adaptive Multiple Core Transform for Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Ohji Nakagami Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 takeshi.tsukuba@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to apply Adaptive Multiple Core Transform (AMT) without additional signaling to chroma inter predicted blocks, where the transform type for each chroma transform block (TB) is determined based on chroma TB size and luma AMT mode of the collocated TB. Experimental results show that, without encoding and decoding time increase, an average chroma BD-rate gain is approximately 0.5% & 0.6% for RA, 1.6% & 1.3% for LB and 1.8% & 1.2% for LP for U & V, respectively.

Introduction

In JEM-4.0 [1], AMT is used for 4x4 to 64x64 luma TBs. Firstly, a flag (EmtFlag) is signaled for the non-zero luma TB to indicate whether AMT is applied or not. If the EmtFlag is 0, DCT-2 is selected. Otherwise, an index (EmtIdx) is further signaled to choose an alternative transform from the given transform set for each horizontal and vertical transform. In inter coding, DST-7 or DCT-8 can be chosen for luma TBs, while DCT-2 is always used for chroma ones as shown in Table 1-1.

Table 1-1: The decision table on the core transform type for inter predicted blocks

| EmtFlag | EmtIdx | Luma |  | Chroma |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Hor. Tr. | Ver. Tr. | Hor. Tr. | Ver. Tr. |
| 0 | - | DCT-2 | DCT-2 | DCT-2 | DCT-2 |
| 1 | 0 | DCT-8 | DCT-8 |  |  |
|  | 1 | DST-7 | DCT-8 |  |  |
|  | 2 | DCT-8 | DST-7 |  |  |
|  | 3 | DST-7 | DST-7 |  |  |

Proposal

Proposed method is to apply AMT to chroma inter predicted blocks as follows:

- The same transform type as for luma block is used for the chroma one in inter coding if the transform size (TrSize) is equal to or larger than 4.

- Otherwise (TrSize==2), existing DCT-2 is used.

Table 2-1 shows the proposed chroma transform variations according to TrSize and luma transform type. Alternative transform for chroma TB can be chosen from existing transform sets for AMT without additional signaling and additional transform type as shown in Table 2-1.

The benefits of the proposed method are:

- Improvement of chroma coding efficiency for inter predicted blocks

- No encoding and decoding time increase

Table 2-1: Proposed decision table on the core transform type for inter predicted blocks

| EmtFlag | EmtIdx | Luma |  | Chroma |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Hor. Tr. | Ver. Tr. | TrSize>=4 |  | TrSize==2 |  |
|  |  |  |  | Hor. Tr. | Ver. Tr. | Hor. Tr. | Ver. Tr. |
| 0 | - | DCT-2 | DCT-2 | DCT-2 | DCT-2 | DCT-2 | DCT-2 |
| 1 | 0 | DCT-8 | DCT-8 | DCT-8 | DCT-8 |  |  |
|  | 1 | DST-7 | DCT-8 | DST-7 | DCT-8 |  |  |
|  | 2 | DCT-8 | DST-7 | DCT-8 | DST-7 |  |  |
|  | 3 | DST-7 | DST-7 | DST-7 | DST-7 |  |  |

Simulation Results

The proposed method was implemented on top of JEM-4.0 [1]. The simulations were performed on the JVET common test conditions [2]. The coding performance is summarized in Table 3-1. On average, 0.5% and 0.6% reduction for RA, 1.6% and 1.3% reduction for LB, 1.8% and 1.2% reduction for LP in terms of chroma BD-rate over JEM-4.0 are observed without encoding and decoding time increase.

Table 3-1: Coding Performance over JEM-4.0

Conclusion

It is proposed to apply AMT without additional signaling to chroma inter predicted blocks, where the  transform type for each chroma TB is determined based on chroma TB size and luma AMT mode of the collocated TB. The proposed method can approximately provide 0.5% and 0.6% reduction for RA, 1.6% and 1.3% reduction for LB, 1.8% and 1.2% reduction for LP in terms of chroma BD-rate over JEM-4.0 without encoding and decoding time increase.

It is recommended to adopt the proposed method into the next JEM.

Reference

[1] JEM-4.0, https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-4.0

[2] K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-B1010, Apr. 2016

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).