| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, Jan.11–20 January 2017 | Document: JVET-E0038 |
| --- | --- |

| Title: | EE1: Cross-check results for residual coefficient sign prediction (JVET-E0051) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yinji Piao Elena Alshina | Email: | yj1003.piao@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of EE1 subtest 1-3 described in JVET-D1011 [1]. Signs of some luma coefficients are predicted and the differences from predicted signs are signalled with context modelling instead of bypass coding. The software implementation is consistent with the algorithm description and the crosscheck results match the results from proponent.

# Code-level cross-check

This proposal was implemented on top of JEM4.0. Macro SIGNPRED is used for the proposed modification. Maximum number of predicted sign is read from configuration file and set in SPS. Sign prediction is not considered in RDOQ but at the end of coefficient coding when hidden sign positions are fixed. The only implementation difference from sign prediction proposed in last meeting [2] and JVET-E0051[3] is cost calculation of border distortion after hypothesis reconstruction. In EE1, the border distortion is evaluated by sum of absolute difference from 1-pixel distance reconstructed neighboring pixels, while in JVET-D0031, two-pixel wide reconstructed neighboring pixels are considered in computation of border distortion.

# Experimental Results

Three tests were carried out under JVET common test conditions [4]. In addition to subtest 3, subtest1 and 2 were also tested to compare the trade-off between performance and complexity.

- Test 1: Number of predicting signs n=1

- Test 2: Number of predicting signs n=2

- Test 3: Number of predicting signs n=3

S/w provided by proponent was compiled on 64-bit Linux and executed on Linux cluster. The summary results are presented in following tables, and more details can be found in accompanying excel sheets. Performance matches the data reported by proponent for subtest3 in RA (except decoder running time).

Table 1. Test results for EE1 subtest 1

Table 2. Test results for EE1 subtest 2

Table 1. Test results for EE1 subtest 3

# Conclusions

Test results from JVET-E0051 are verified in this report. The simulation results matched the data provided by the proponents.

# References

- E.Alshina, J. Boyce, L. Zhang “Description of Exploration Experiment on Coding Tools”, JVET-D1011

- F. Henry, G. Clare “Residual Coefficient Sign Prediction”, JVET-D0031

- F. Henry, G. Clare “EE1: Residual Coefficient Sign Prediction”, JVET-E0051

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, JVET-B1010, February, 2016.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).