| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0049 |
| --- | --- |

| Title: | EE3: Cross-check for Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A.Alshin, E.Alshina Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This contribution verifies performance results of EE3 tests 4 on Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching. It is confirmed that major performance improvement 0.3% in RA comes from extension MV refinement with int-pel precision. This is questionable whether the similar gain can be achieved by improving encoder side MV derivation.

# Introduction

JEM4.0 includes 2 decoder-side motion derivation techniques: Pattern matched motion vector derivation technique (FRUC) and bi-directional optical flow (BIO). First one operates on sub-block, the second is sample-wise MV refinement.

In exploration experiment under verification the third decoder side MV derivation technique was tested. So-called Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching [1] is applied under conditions:

- Not OBMC

- L0 and L1 reference are from opposite time directions

- Merge

- Not Local Illumination Compensation

- Not Affine MC

- Not FRUC

It similarly to FRUC it operates at sub-PU level and BIO is applied on top.

First motion compensation for luminance is done with MV signalled in the bit-stream for block extended by one row and one column on each size. So the memory access required for this method is (W+1+7) (H+1+7) instead (W+7) (H+7) in HEVC motion compensation. This is 40% higher for corner case (44 sub-PU in JEM4.0).

Then decoder searches for MV refinement in L0 and L1. In first round of search up to 8 MV candidates (1 int-pel displacement in vertical and horizontal directions) are checked. So up to 9 calculations for SAD and up to 8 comparisons are needed. The refinement with int-pel precision contributes the major part the gain (0.3%).

The second round of MV search at decoder-side refines MV with 1/2-pel precision. Additionally up to 8  calculations for SAD and up to 8 comparisons are needed. Additional gain from this 1/2-pel refinement is 0.1%. But this second round doesn’t require additional memory access.

After MV refinement is found MC for both three colour components is performed with new MV.

MV refinement for lists L0 and L1 are not restricted to have same magnitude but opposite sign (as in BIO). This is true in only ~30% of cases.

Based on collected statistics in ~15% of the cases MV refinement is zero.

This is questionable whether the similar gain can be achieved by improving encoder side MV derivation.

# Test results

The code EE3 tests 1-3 is available at  https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-4.0-EE3-D0029 and tested under JVET CTC for random access configuration compared to the JEM4.0 as anchor. Additionally to the full capability test we tested only int-pel MV refinement.

Summary of test results is shown in Table 1. Major performance improvement is observed in the supplementary test with int-pel MV refinement.

Test results perfectly match to data provided by proponent. SW implementation is clear and follows original proposal [1].

Table 1. Test results summary: 1/2-pel MV refinement (on the left), int-pel MV refinement (on the right).

|  |  | Random Access Main 10 |  |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  | -0.7% | -0.6% | -0.6% | 111% | 108% |  |  |  |  |  |  |
| Class B |  | -0.3% | -0.4% | -0.4% | 108% | 115% |  | -0.3% | -0.3% | -0.4% | 103% | 104% |
| Class C |  | -0.4% | -0.6% | -0.6% | 102% | 102% |  | -0.3% | -0.5% | -0.4% | 102% | 100% |
| Class D |  | -0.4% | -0.4% | -0.3% | 103% | 103% |  | -0.3% | -0.3% | -0.3% | 102% | 101% |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |
| Class F |  | 0.0% | 0.0% | -0.1% | 102% | 107% |  | -0.1% | -0.1% | -0.1% | 101% | 102% |

# Conclusions

Verified proposal provides reasonable gain with moderate run-time increment. The major source of the performance improvement is int-pel MV refinement. This is questionable whether the similar gain can be achieved by improving encoder side MV derivation.

# References

[1] X. Chen, J. An, J. Zheng (HiSilicon) “Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching,” JVET-D0029, Oct. 2016, Chengdu

# Patent rights declaration

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).