| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0064 |
| --- | --- |

| Title: | On JEM Binary Arithmetic Engine Design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou , Yi Hu 16340 West Bernardo Drive San Diego, CA-92127, USA | Tel: Email: | minhua.zhou@broadcom.com yi.hu@broadcom.com |
| Source: | Broadcom Limited |  |  |

_____________________________

# Abstract

The current JEM4.0 CABAC design uses a LPS (Least probable Symbol) range table of 36,834 bytes which is expensive from the implementation point of view. This contribution provides a new LPS range table design for the JEM binary arithmetic engine. The new design not only maintains the ability of doing the LPS range update via table look up (similar to what the current JEM4.0 CABAC does, but with the half of LPS range table size), but also enables the LPS range update by using on-the-fly calculation that employs a 7bit by 8-bit multiply plus shifts. The on-the-fly LPS update avoids the need of buffering huge LPS range table and is in the order of magnitude cheaper than the table loop up based update. Experimental results revealed that the proposed change preserved the same compression efficiency as the current JEM4.0 design.

# Introduction

This contribution advocates designing the LPS (Least probable Symbol) range table of the JEM CABAC binary arithmetic engine (BAE) in a way that allows the LPS range update not only by using the table look up (same as the current JEM4.0 does) but also by using a 7-bit by 8-bit multiply plus shifts. It is asserted that the proposed change significantly reduces the implementation cost of the JEM CABAC engine.

# Problem Statement

Fig. 1 Diagram of the JEM4.0 CABAC binary arithmetic coding engine of context-coded bins

Fig.1 depicts the diagram of the JME4.0 CABAC binary arithmetic coding engine of context coded bins. Compared to the MPEG/ITU-T HEVC/H.265 and AVC/H.264 CABAC BAE design, the noticeable changes include

- Increased precision for range (in 9-bit) and (MPS) probability state (in 15-bit), resulting in a LPS range table (i.e. rTab512[64] in Fig. 1) size of 36,384 bytes (i.e. 512 x 64 x 9bit).

- Probability update with a so-called multi-hypothesis probability update (not a topic of this contribution).

The JEM4.0 CABAC BAE is more efficient than that of MPEG/ITU-T HEVC/H.265 and AVC/H.264, but with a cost. In MPEG/ITU-T HEVC/H.265, the LPS range table size is only 64x4 (i.e. 256) bytes, but in JEM4.0 the same table has been increased to 36,864 bytes, which is a significant cost increase for CABAC implementation and may not be justified.

# Proposed Solution

In order to address the LPS range table size issue highlighted above, we propose an architecture flexible LPS table design, so that the LPS range (i.e. RLPS) calculation can not only be implemented in table look up fashion as the current JEM4.0 design does, but also be computed on-the-fly with multiply and bit shifts.

To replace the table-look-up based RLPS calculation highlighted in red lines in Fig. 1, the proposed on-the-fly  RLPS calculation is defined in the following two steps:

- quantize range R and probability state Ps by 2 and 7 bits, respectively, and by

qR=R≫2qP=Ps≫7

- based on the quantized range qR and the quantized probability state qP to compute the RLPS by

RLPS=Clip3(2, 511,(qR∙256-maxqP,1)≫6)

Since R is in 9-bit, qR will be in 7-bit. It is obvious that the LSP table look up can be replaced by a 7 bit by 8 bit multiply plus shifts in the proposed solution, which is in the order of magnitude cheaper than storing a LPS range table of 36,384 bytes.

The proposed solution maintains the ability of computing RLPS based on table-look-up like the JPEM4.0 CABAC does, but the LPS range table rTab25664 has to be computed and pre-stored according to the pseudo code defined in Table 1 below:

for qP=0;qP<256;qP++ { for qR=64;qR<128;qR++ { rTabqPqR-64=Clip3(2,511,(qR∙256-maxqP,1)≫6); } }

Table 1 Pseudo code for computing the LPS range table based on proposed solution

Note that the LPS range table size in the proposed solution is only the half of that of JEM4.0.

In the following functions of JEM4.0

- TEncBinCABAC::encodeBin( UInt binValue, ContextModel &rcCtxModel)

- TDecBinCABAC::decodeBin( UInt& ruiBin, ContextModel &rcCtxModel )

The proposed solution is realized by the following code change:

Replace the table loop up based LPS range calculation

UShort uiLPS = TComCABACTables::sm_aucLPSTable[rcCtxModel.getState()>>6][(m_uiRange>>2)-64];

With the on-the-fly LPS range calculation

UShort uiLPS;

if ((rcCtxModel.getState()>>7) >=1)

uiLPS  = ((m_uiRange>>2)*(256 - (rcCtxModel.getState()>>7)))>>6;

else

uiLPS  =  ((m_uiRange>>2)*255)>>6;

if (uiLPS > 511) uiLPS = 511; else if (uiLPS < 2) uiLPS = 2;

# Experimental Results

The simulation was carried out by using the JEM4.0 and following the common test conditions specified in [1].

To quantify the quality gain of using large LPS range table, a test was carried out by disabling following two macros in In JEM4.0:

VCEG_AZ07_BAC_ADAPT_WDOW           ///< Multi-hypothesis probability estimation

VCEG_AZ05_MULTI_PARAM_CABAC  ///< CABAC probability estimation with 2 windows

Table 2 Summary results of disabling dual window and multi-hypothesis probability estimation in JEM4.0

As shown in Table 2, the dual window and multi-hypothesis probability estimation together with the large LPS range table provided around 0.8% to 0.9% BD-rate gain in JEM4.0.

Table 3 Summary results of the proposed solution

Table 3 summarizes the experimental results for the proposed solution. The test results revealed that  the proposed solution provided almost identical results to the current JEM4.0 design in terms of compression efficiency.

Note that the run time in Table 2 and Table 3 may not be accurate due to the heterogeneous nature of LSF farm.

# Conclusions

This contribution advocates offering an option of implementing the LPS range update by using multiply and shifts to combat the LPS range table size issue in the current JEM4.0 CABAC design. The proposed change does not comprise the compression efficiency.

# References

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, JVET-B1010, February, 2016.

- J. Chen, E. Alshina, G.J. Sullivan, J.-R. Ohm and J. Boyce, “Algorithm Description of Joint Exploration Test Model 4”, JVET-D1001, October 2016.

# Patent rights declaration(s)

Broadcom Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).