| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0071 |
| --- | --- |

| Title: | AhG8: Viewport-based subjective evaluation of 360-degree video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | +1 858 210 4841 Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

At the 4th JVET meeting, common test conditions and evaluation procedures for 360-degree video coding were established (JVET-D1030). Different viewports (VPs) were defined for each 360-degree video sequence for evaluation on 2D displays. This document reports preliminary results of a subjective evaluation comparing VPs rendered from 360-degree video sequences encoded with HM-16.14 and JEM-4.1 in the equirectangular projection (ERP) format at low bit rates. The stimulus comparison method was used to compare the quality difference between HM and JEM at the lowest bit rate. Two video sequences were presented side-by-side on a 4K TV and expert subjects were asked to rate which video has better overall quality. Results show that JEM achieves some visual quality improvements over HM at low bit rate.

# Introduction

An important question is whether JEM can provide better visual quality than HM for 360-degree video sequences. At the 4th JVET meeting, common test conditions and evaluation procedures for 360-degree video coding were established [1]. In total, eight different metrics should be reported: six PSNR-based metrics computed on the whole 360-degree video and the PSNR computed on two static rendered VPs. Correlation between objective quality metrics and perceived visual quality is still an open question for 360-degree video. Even if PSNR is well-known and widely used for regular video quality assessment, its usage for VP-based quality assessment for 360-degree video is also an open question, especially considering the potential impact of VP selection. The field of view (FOV) size of VP may be different from conventional 2D video. The PSNR may not be accurate when the FOV size is large, which corresponds to wide-angle view. Therefore, subjective quality assessment remains the ultimate means to quantify or qualify improvements in 360-degree video compression.

Currently there is an open debate on whether subjective assessment for 360-degree video should be performed using head mounted display (HMD) or regular 2D display using VP rendering. Ultimately, the consumption of 360-degree video would be on HMD devices, where the viewer will be presented a rendered VP based on the viewer’s head orientation. However, considering currently available technologies, the quality of experience provided by HMD devices is far from perfect, mostly due to relatively low resolution and framerate. Therefore, most of current and short-term consumption of 360-degree video will be on standard 2D displays using VP rendering.

Considering these aspects, we have designed a subjective evaluation to assess the quality difference between JEM and HM encoded 360-degree video sequences. For this aim, we selected a test method that allows a direct comparison of the two codecs. In particular, we selected the stimulus comparison method [4]. To minimize visual working memory limitations, the video sequences were presented in pairs in side-by-side fashion for a direct comparison. Additionally, each pair of video sequences were displayed twice before asking the subjects to rate the quality difference between the pair. This method was selected to achieve more reliable results and reduce the impact of stitching artefacts, which are not due to video compression but are already visible in the source material.

# Methodology

To evaluate the performance of JEM-4.1 over HM-16.14, an informal subjective evaluation was conducted with five expert viewers.

## Dataset

The dataset was built following the common test conditions defined in [1]. In total, ten 360-degree video sequences were used: six sequences in 8K ERP original format and four sequences in 4K ERP original format. The sequences were encoded with HM-16.14 and JEM-4.1 following the common test conditions defined in [2]. The encoding was done in the ERP format, using the Random Access configuration, at QP=37. Then, for each sequence, two VPs were rendered from the decoded bit stream using the 360Lib software [5] according to the parameters defined in [1]. Table 1 reports the resulting bit rates for HM and JEM encoded at QP=37, as well as the PSNR measured on the luma channel of the rendered VPs.

Table 1. Bit rates and VP PSNR for HM and JEM.

| Sequence name | bit rate [kbps] |  |  | luma PSNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VP1 |  | VP2 |  |
|  | HM | JEM | JEM-rate / HM-rate (%) | HM | JEM | HM | JEM |
| Train_le | 1519.4 | 1291.9 | 85.0 | 33.49 | 34.20 | 33.02 | 33.69 |
| SkateboardingTrick_le | 2009.6 | 1769.9 | 88.1 | 30.16 | 30.62 | 38.47 | 39.08 |
| SkateboardInLot | 1865.0 | 1598.9 | 85.7 | 32.84 | 33.41 | 34.10 | 34.69 |
| ChairLift | 1508.5 | 1295.2 | 85.9 | 36.43 | 37.00 | 33.90 | 34.46 |
| KiteFlite | 2676.6 | 2506.7 | 93.7 | 34.37 | 34.86 | 32.93 | 33.38 |
| Harbor | 910.4 | 854.3 | 93.8 | 34.33 | 34.79 | 36.17 | 36.79 |
| PoleVault_le | 1649.8 | 1512.4 | 91.7 | 33.89 | 34.33 | 32.81 | 33.24 |
| AerialCity | 822.0 | 709.8 | 86.4 | 34.57 | 35.19 | 33.91 | 34.53 |
| DrivingInCity | 1118.7 | 907.3 | 81.1 | 33.34 | 34.08 | 33.28 | 33.90 |
| DrivingInCountry | 2129.8 | 1806.8 | 84.8 | 30.73 | 31.48 | 29.88 | 30.47 |

## Test environment

The experiments were conducted in a dark room with grey walls, black curtains, and no ambient light. To display the test stimuli, a Samsung UN55HU6950 55” 4K LCD TV with a native resolution of 3840×2160 pixels was used. The monitor was calibrated to the following profile: sRGB gamut, D65 white point, 120 cd/m2 brightness, and minimum black level. The experiment involved one subject per test session. The subject was seated in line with the center of the monitor at a distance of 1.6 times the image height, as suggested in [3].

## Test method

The stimulus-comparison method [4] was chosen to compare the performance of JEM over HM. To minimize visual working memory limitations, the video sequences were presented in pairs in side-by-side fashion. One sequence was always the HM anchor whereas the other sequence was the same content but encoded with JEM. The position of the HM and JEM versions was randomized. The two video sequences were separated by a 20-pixels black border. Each pair was presented twice, with a one-second message before each playback. After the second presentation of each video pair, a five-second voting time followed. The following semantic adjectival categorical judgement scale was used: left is much better, left is better, left is slightly better, the same, right is slightly better, right is better, right is much better. Subjects were asked to rate the quality difference between the pair of video sequences to be evaluated and to express these judgments in terms of the wordings used to define the rating scale.

Before the test starts, oral instructions were provided to the subject to explain his/her task. Additionally, a brief training session was organized to allow subjects to familiarize with the assessment procedure. The training materials were presented to subjects exactly as for the test materials.

# Results

Figure 1. Subjective results.

Figure 1 reports the mean opinion scores (MOSs) and associated 95% confidence intervals (CIs) from the subjective experiment comparing HM and JEM at QP=37. Results demonstrate that JEM improves visual quality in most cases. Table 2 depicts the viewport sequences used in the subjective evaluation and the areas where visual quality difference are noticeable between HM and JEM. The improvements are particularly observed for sequence Harbor, where the rendering of the water appears less blocky in the JEM encoded sequence when compared to HM (see Figure 2). For this sequence, improvements can also be perceived in the flag floating on the boat as well as on the different ropes attached to the boat (see Figure 3). For sequence KiteFlite, improvements can be observed in the pavement, the sign boards, some textures in the grass, as well as in the weathercock. In ChairLift, improvements are observed on the face of one of the persons seating on the lift, as well as some textures on the ground below the lift, particularly in the grass and on the dirt road. For SkateboardingTrick, more details can be observed in the pavement and there are less artifacts in the JEM version in the structure on which the skateboarder is sliding. Regarding sequence SkateboardInLot (see Figure 4), improvements are mostly visible on the face, in the sky, in the buildings behind, as well as in the road. Improvements in the road are even more visible when pausing the video. However, because of the relatively high motion in the sequence, these improvements are mostly masked by motion during playback. Similarly, for the Train sequence, improvements from JEM can be perceived in the moving train when displayed as still frames, but are rather small during playback. In general, improvements are less visible for the 4K ERP source sequences because of the relatively small size of the rendered VPs (856x856 for 4K ERP source versus 1816x1816 for 8K ERP source). In particular, for sequence PoleVault, there are very few details because most of the objects are relatively far from the camera. The same observation applies for sequence AerialCity, where improvements can mostly be observed in the road. Improvements are also mostly observed in the road in DrivingInCity, where blocking artefacts can be observed in the HM encoded sequence when compared to JEM. For this sequence, results are significantly different for the two selected VPs. Finally, regarding DrivingInCountry, there is a lot of camera motion probably due to a lack of camera stabilization. As a result, the VP looking at the river is rather unpleasant to watch and tends to create some sort of simulator sickness.

Table 2. Representative frames of the viewport sequences used in the subjective evaluation. The red circles indicate areas where visual quality difference are noticeable between HM and JEM.

|  | VP1 | VP2 |
| --- | --- | --- |
| Train |  |  |
| SkateboardingTrick |  |  |
| SkateboardInLot |  |  |
| ChairLift |  |  |
| KiteFlite |  |  |
| Harbor |  |  |
| PoleVault |  |  |
| AerialCity |  |  |
| DrivingInCity |  |  |
| DrivingInCountry |  |  |

| (a) HM | (b) JEM |
| --- | --- |

Figure 2. Harbor – VP1: comparison between HM (a) and JEM (b).

| (a) HM | (b) JEM |
| --- | --- |

Figure 3. Harbor – VP2: comparison between HM (a) and JEM (b).

| (a) HM | (b) JEM |
| --- | --- |

Figure 4. SkateboardInLot – VP1: comparison between HM (a) and JEM (b).

# Discussion

In general, the stitching artefacts, e.g., geometric distortions and color differences, observed in some VPs can be disturbing, especially for non-expert viewers or viewers not familiar with the content, as similar artifacts can also result from compression, which may influence the viewers’ judgement. Based on this observation, we recommend trying to select VP parameters such that stitching artefacts are minimized in the rendered VPs. When selecting a test methodology, it is also important to take this aspect into account such that the influence of stitching artefacts can be minimized. This can be achieved by avoiding to rate a sequence independently from any reference or other sequence to compare to, i.e., avoiding single stimulus test methods.

The rendered VPs with horizontal and vertical FOV set to 75 degrees appear a little blurred in the corners as the content is further from the sphere center. It is suggested that when using rectilinear projection, such as currently supported by 360Lib, FOV values larger than 75 degrees should not be used in order to avoid over-blurring in the corners.

For rather static contents, dynamic viewports may be a good alternative to static viewports to explore a larger portion of the 360-degree video content. However, for sequences having relatively large amount of motion, dynamic viewports might introduce some visual discomfort or create simulator sickness if not selected carefully. For example, we should avoid placing fast-moving dynamic viewports in sequences with large motion, especially avoiding fast-moving viewports along the latitude direction. Instead, we may consider placing dynamic viewports along the horizontal direction to mimic more natural viewer behavior. Dynamic viewports can also change the relative motion of the content. For example, in the Train sequence, if a dynamic viewport is defined by changing the yaw linearly and keeping the pitch constant, the relative motion of the train may appear either slower or faster depending on the direction of the camera pan. As a results, considering motion masking, the visibility of video compression artefacts might be affected by the change in relative motion. Table 1 reports a comparison of the different viewport types in terms of space and time coverage, as well as visual comfort. With static viewports, the selected viewport location might influence the results. With dynamic viewports, there is a tradeoff between the area that one wants to cover and the visual comfort of the resulting video sequence.

Table 3. Viewports comparison.

| Viewport type | Space coverage | Time coverage | Visual comfort |
| --- | --- | --- | --- |
| Static | Poor | Good | Good |
| Dynamic | Average – Good | Good | Poor – Good |

# References

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360° video”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D1030, 4th Meeting, Oct. 2016.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, 2nd Meeting, Feb. 2016.

- ITU-R BT.2022, “General viewing conditions for subjective assessment of quality of SDTV and HDTV television pictures on flat panel displays.” International Telecommunication Union, Aug. 2012.

- ITU-R BT.500-13, “Methodology for the subjective assessment of the quality of television pictures.” International Telecommunication Union, Jan. 2012.

- 360Lib-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-1.0

# Patent rights declaration(s)

InterDigital Communications Inc. does not have any current or pending patent rights relating to the technology described in this contribution.