| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0103_r1 |
| --- | --- |

| Title: | Block adaptive CCLM residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes an extension of CCLM Cb-to-Cr prediction. In this contribution, Cr-to-Cb prediction is also introduced to make a symmetric mechanism in CCLM residual prediction. One flag for each chroma CU is added to identify the reference chroma component. The combination is decided by RD cost check. Compared with the top of JEM4.0, BD-rate and running times are 0.0%/-1.0%/-1.5% for Y/Cb/Cr and 103.0%/99.9% for Enc/Dec, respectively, in all intra condition with a full RDO case. When a fast RDO is utilized, BD-rate and running times are 0.0%/-0.5%/-1.0% for Y/Cb/Cr and 100.1%/99.9% for Enc/Dec, respectively, in all intra condition.

Introduction

Chroma intra prediction is enhanced in JEM4.0 from HEVC. One of the enhanced chroma intra prediction is a cross component linear model (CCLM) method [1]. CCLM consists of luma-to-chroma prediction and Cb-to-Cr prediction. In this contribution, we focuses on the Cb-to-Cr as CCLM residual prediction.

The Cb-to-Cr prediction is enhancement of Cr prediction. Cr component is then predicted from Cb component. Instead using the reconstructed luma sample signal in luma-to-chroma, the Cb-to-Cr prediction is applied in residual domain. A combination of reference and target component is Cb-to-Cr only. In JEM, CCLM luma-to-chroma prediction mode is added as one additional chroma intra prediction mode, while CCLM Cb-to-Cr prediction is applied for all chroma intra prediction mode except CCLM luma-to-chroma mode.

Asymmetric prediction mechanism is remained in CCLM residual prediction as Cb-to-Cr. To make the symmetric mechanism, we proposed the Cr-to-Cb prediction with fast RDO method.

Description of the proposed method

Cr-to-Cb prediction

As similar to the CCLM Cb-to-Cr prediction in JEM4.0, CCLM Cr-to-Cb prediction is proposed to enhance the Cb component prediction. These predictions are exclusive each other. One flag for each chroma CU is added to identify the reference chroma component. This is implemented by adding a weighted reconstructed Cb residual to the original Cr intra prediction to form the final Cr prediction:

predCr*i,j=predCri,j+α⋅resiCb'i,j

The scaling factor α is derived in the same way of CCLM Cb-to-Cr prediction.

When the intra prediction modes other than CCLM luma-to-chroma prediction mode is used for chroma component of a CU, CCLM Cb-to-Cr or Cr-to-Cb prediction is used to enhance the Cr or Cb component prediction.

Decision with full RDO

At encoder side, an additional RD cost check for chroma components in each mode is added for selecting the optimal reference chroma component. In JEM4.0, chroma intra prediction has six modes, DM_Chroma, Planar, Vertical, Horizontal, DC, and CCLM. Since CCLM Cb-to-Cr/Cr-to-Cb is exclusive to CCLM mode, the number of RD cost check is increased from six to 11.

Decision with fast RDO (encoder only optimization)

For a fast encoding, a reduction of the number of RD cost check is proposed. The key idea is to decide the reference chroma component by only one time RD cost check. To realize the idea, the decision is based on the RD cost of DM_Chroma since a probability of DM_Chroma is highest. At the same time, DM_Chroma is moved to the first position in chroma RD-cost-check order. It is noted that a signal processing based decision without RD cost check does not work well in our experiments.

Experimental results

The proposed method is implemented to JEM4.0. A test condition follows the JVET common test condition [2]. The summary of BD-rate is shown in the following table. The running time evaluation is reliable.

Table 1 Results by the block adaptive chroma residual prediction with full RDO

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.0% | -1.1% | -0.6% | 103.2% | 99.7% |
| Class A2 | 0.0% | -2.5% | -2.7% | 103.6% | 99.9% |
| Class B | 0.0% | -0.8% | -1.3% | 102.9% | 99.9% |
| Class C | 0.1% | -0.5% | -1.4% | 102.9% | 99.9% |
| Class D | 0.0% | -0.9% | -1.4% | 102.5% | 99.8% |
| Class E | 0.1% | -0.1% | -1.9% | 103.0% | 100.0% |
| Overall | 0.0% | -1.0% | -1.5% | 103.0% | 99.9% |

Table 2 Results by the block adaptive chroma residual prediction with fast RDO

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.0% | -0.5% | -0.4% | 100.1% | 99.6% |
| Class A2 | 0.0% | -1.8% | -2.2% | 100.0% | 100.0% |
| Class B | 0.0% | -0.4% | -0.7% | 100.0% | 99.9% |
| Class C | 0.1% | -0.2% | -0.9% | 100.1% | 100.1% |
| Class D | 0.1% | -0.4% | -0.8% | 100.5% | 100.1% |
| Class E | 0.1% | 0.5% | -1.0% | 99.9% | 100.0% |
| Overall | 0.0% | -0.5% | -1.0% | 100.1% | 99.9% |

Bit rate is almost no change due to the additional flag. By introducing the fast RDO, encoding run time is reduced from 103.0% to 100.1%. The modification of the chroma RD-cost-check order also contributes the time reduction since unnecessary memory copy can be avoided.

Conclusion

In this contribution, we propose to enhance the Cr prediction by introducing CCLM Cb-to-Cr prediction. One flag for each chroma CU is added to identify the reference chroma component.

References

- Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm, and Jill Boyce, “”, JVET-D1001, Chengdu, CN, 15—21 October 2016.

- K. Suehring and X. Li, “JVET common test conditions and software reference configurations”, JVET-B1010, San Diego, USA, 20—26 February 2016.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).