| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0112 |
| --- | --- |

| Title: | AHG4: Evaluation report of aerial photography sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | You-Huei Ju, 1Ching-Chieh Lin, 2Chun-Lung Lin, Yao-Jen Chang and Po-Han Lin 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-5758 1JackLin@itri.com 2Chunlung@itri.com |
| Source: | ITRI international |  |  |

_____________________________

- Abstract

This document reports the encoding results of aerial test sequences proposed by DJI. Two 4K and four 1080p sequences are tested with float QP to match a specific target bitrate. Bitstreams were encoded both by HM-16.13 and JEM-4.0 software under the conditions specified in the work plan. Objective and subjective evaluations are provided in this document for assessment of test material.

Introduction

In Chengdu meeting, DJI proposed four aerial test sequences and provided with both 4K and 1080p resolution [1]. Since the work plan [2] included two 4K aerial sequences (iceAerial and iceRiver), this document tested the rest of the aerial test sequences. Therefore, four 1080p sequences: iceAerial, icrRiver, iceRock, iceRock2 and two 4K sequences: iceRock and iceRock2 were encoded by HM-16.13 and JEM-4.0 at approximately the same bitrate.

The encoding conditions and target bitrates are the same as JVET-E0110 [3] described. Since both iceRock and iceRock2 are new 4k test sequences, the two test sequences were encoded from rate 1 to rate 6 to study the encoding performance.

Test sequences evaluation

Target bitrate matching using a float QP

After performing the same QP selection step described in [3], base QP points were obtained for each test sequence. The steps and rules in [3] were used to derive float QP values. The encoding results of 1080p and 4K test sequences are summarized below. Details of the coding results are attached in the accompanying excel files.

Table 1: Encoding results of 4K test sequences by using HM-16.13 and JEM-4.0 with float QP

| iceRock (30p) |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | target rate [Mbps] | QP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R6 | 8 | 19.81333 | 8.1119056 | 1.40% | 3 | 19.4848 | 7.9999384 | 0.00% | -1.38% |
| R5 | 5 | 21.68 | 4.9290776 | -1.42% | 6 | 21.48485 | 4.9707344 | -0.59% | 0.85% |
| R4 | 2.7 | 24.78667 | 2.6789128 | -0.78% | 1 | 23.48485 | 2.678252 | -0.81% | -0.02% |
| R3 | 1.6 | 28.73333 | 1.5906416 | -0.58% | 8 | 27.48485 | 1.5972672 | -0.17% | 0.42% |
| R2 | 1 | 32.78667 | 0.9959496 | -0.41% | 0 | 31 | 0.9963792 | -0.36% | 0.04% |
| R1 | 0.6 | 37.14667 | 0.5961224 | -0.65% | 4 | 35.48485 | 0.5984296 | -0.26% | 0.39% |
| iceRock2 (30p) |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
|  | target rate [Mbps] | QP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R6 | 8 | 20.36 | 7.9184208 | -1.02% | 0 | 19.48485 | 7.8599072 | -1.75% | -0.74% |
| R5 | 5 | 22.69333 | 5.094744 | 1.89% | 8 | 22.4848 | 4.875116 | -2.50% | -4.31% |
| R4 | 2.7 | 26.36 | 2.6949856 | -0.19% | 7 | 25.48485 | 2.6916048 | -0.31% | -0.13% |
| R3 | 1.6 | 30.25333 | 1.5874096 | -0.79% | 1 | 28.48485 | 1.5859464 | -0.88% | -0.09% |
| R2 | 1 | 34.09333 | 0.9940376 | -0.60% | 5 | 32.48485 | 1.0417248 | 4.17% | 4.80% |
| R1 | 0.6 | 38.14667 | 0.5945056 | -0.92% | 4 | 36.48485 | 0.5951912 | -0.80% | 0.12% |

Table 2: Encoding results of 1080p test sequences by using HM-16.13 and JEM-4.0 with float QP

| iceAerial (30p) |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | target rate [Mbps] | QP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R5 | 2.9 | 27.36 | 2.8788112 | -0.73% | 0 | 26 | 2.8654056 | -1.19% | -0.47% |
| R4 | 1.7 | 32 | 1.6839632 | -0.94% | 7 | 30.48485 | 1.689432 | -0.62% | 0.32% |
| R3 | 1 | 37 | 0.9920832 | -0.79% | 0 | 35 | 0.9977192 | -0.23% | 0.57% |
| R2 | 0.6 | 41.89333 | 0.5950944 | -0.82% | 0 | 39.48485 | 0.5981928 | -0.30% | 0.52% |
| R1 | 0.4 | 45.78667 | 0.3953256 | -1.17% | 1 | 43.48485 | 0.39606 | -0.99% | 0.19% |
| iceRiver (30p) |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
|  | target rate [Mbps] | QP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R5 | 2.9 | 21 | 2.873718 | -0.91% | 4 | 19.4848 | 2.9097568 | 0.34% | 1.25% |
| R4 | 1.7 | 25.36 | 1.690995 | -0.53% | 2 | 23.48485 | 1.6857936 | -0.84% | -0.31% |
| R3 | 1 | 29.52 | 0.997558 | -0.24% | 0 | 27.4848 | 1.0022232 | 0.22% | 0.47% |
| R2 | 0.6 | 33.36 | 0.594577 | -0.90% | 0 | 32 | 0.5965752 | -0.57% | 0.34% |
| R1 | 0.4 | 36.09333 | 0.398014 | -0.50% | 0 | 34.48485 | 0.39888 | -0.28% | 0.22% |
| iceRock (30p) |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
|  | target rate [Mbps] | QP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R5 | 2.9 | 17.84 | 2.882455 | -0.60% | 1 | 16.4848 | 2.8965136 | -0.12% | 0.49% |
| R4 | 1.7 | 21.78667 | 1.689317 | -0.63% | 5 | 20.48485 | 1.6920048 | -0.47% | 0.16% |
| R3 | 1 | 26.14667 | 0.995109 | -0.49% | 1 | 24.48485 | 0.9926776 | -0.73% | -0.24% |
| R2 | 0.6 | 30.57333 | 0.593791 | -1.03% | 7 | 29.48485 | 0.5993256 | -0.11% | 0.93% |
| R1 | 0.4 | 33.96667 | 0.398138 | -0.47% | 2 | 32.4848 | 0.3997968 | -0.05% | 0.42% |
| iceRock2 (30p) |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
|  | target rate [Mbps] | QP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R5 | 2.9 | 19.60333 | 2.921876 | 0.75% | 4 | 18.48485 | 2.885436 | -0.50% | -1.25% |
| R4 | 1.7 | 23.36 | 1.6994656 | -0.03% | 8 | 22.4848 | 1.713176 | 0.78% | 0.81% |
| R3 | 1 | 27.46667 | 0.9942824 | -0.57% | 8 | 26.4848 | 0.999844 | -0.02% | 0.56% |
| R2 | 0.6 | 31.46667 | 0.5970264 | -0.50% | 7 | 30.48485 | 0.593864 | -1.02% | -0.53% |
| R1 | 0.4 | 34.57333 | 0.4006336 | 0.16% | 5 | 33.48485 | 0.3980416 | -0.49% | -0.65% |

The R-D curves of the encoding results by using float QPs are presented as follows. More details of the PSNR values and bitrate can be found in the attached excel files “Aerial_4K_evaluation_ITRI.xlsm” and “Aerial_1080p_evaluation_ITRI.xlsm.”

From the objective evaluation of iceRock and iceRock2 with 4K resolution, the R-D curves show that HM-16.19 can provide good coding performance. In addition, it is observed that the PSNR Y values from each rate points are all higher than 38 dB.

Subjective test

In most cases, differences between JEM-4.0 and HM16.13 at the same rate point could be easily observed when QP values are big. However, for small QP values, it may be not easy to observe the differences. Therefore, this document provides some decoding pictures at rate 1 to compare the compression performance.

Figure 1-1: #15 of iceRock 4K in HM-16.13 with QP37.14667 (Rate 1)

Figure 1-2: #15 of iceRock 4K in JEM-4.0 with QP35.48485 (Rate 1)

Details of the surface of the rock are missing in the HM-16.13.

Figure 2-1: #299 of iceRock2 4K in HM-16.13 with QP38.14667 (Rate 1)

Figure 2-2: #299 of Rock2 4K in JEM-4.0 with QP36.48485 (Rate 1)

In HM-16.13, the surface of the rock was blurred and details are missing.

Figure 3-1: #275 iceAerial 1080p in HM-16.13 with QP45.78667 (Rate 1)

Figure 3-2: #275 iceAerial 1080p in JEM-4.0 with QP43.48485 (Rate 1)

In the area marked by red rectangular, some reviewers commented that chromatic aberrations were observed in HM-16.13.

Figure 4-1: #299 of iceRiver 1080p in HM-16.13 with QP34.48485 (Rate 1)

Figure 4-2: #299 of iceRiver 1080p in JEM-4.0 with QP36.09333 QP (Rate 1)

In summary, for the aerial series except iceAerial, the differences between the hightest rate and the lowest rate are not really conspicuous compared with the other test sequences in the work plan [2]. The aerial series of test sequences are realatively easier to encode. The qualities at rate 1 could be good enough only serveral frames were somewhat blurred.

Conclusion

The contribution reports both objective and subjective evaluation results of the four 1080p and two 4K aerial test sequences proposed at Chengdu meeting. Bitstreams were generated by HM-16.13 and JEM-4.0 according to the test conditions specified in the test plan. From the objective and subjective evaluation results, it is suggested to consider iceAerial as an candidate for further testing.

Reference

- “Aerial photography sequences for video coding standard development”, JVET-D0060, October 2016

- “Work plan for assessment of test material,” JVET-D1002, November 2016

- “AHG4: Evaluation report of SDR test sequences (4K8-9 and 1080p1-5)”, JVET-E0110, January, 2017

- ftp://ftp.ient.rwth-aachen.de/

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.13/

- https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-4.0/

Patent rights declaration(s)

ITRI international does not have any current or pending patent rights relating to the technology described in this contribution.