| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0113 |
| --- | --- |

| Title: | AHG8: Supplemental Test Results on Segmental Sphere Projection (SSP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo Drive San Diego, CA-92127, USA | Tel: Email: | minhua.zhou@broadcom.com |
| Source: | Broadcom Limited |  |  |

_____________________________

# Abstract

This contribution reports supplemental test results on SSP. Under the JVET 360 video common test conditions, on average over 30% BD-rate increase relative to ERP was observed for viewport quality along the North and South Pole. Further experiments reviewed that viewport quality loss is common to other alternative projection formats such as CMP and CISP. This contribution also reports inconsistent behaviors of 360 video quality metric WS-PSNR observed in some cases.

# Introduction

The Segmental Sphere Projection (SSP) (see Fig. 1) [2] allocates less number of samples to North and South Pole when projecting sphere to a 2-D image, this may greatly undermine the viewport quality along the North and South Pole. This contribution provides supplemental test results on SSP by intentionally setting the viewing direction (yaw, pitch) angles to (0, 90) and (0, -90) in the JVET 360 video common test conditions [1].

Fig. 1 A layout of SSP

# Experimental Results

The simulation was carried out by using the HM16.14 that was integrated with the 360 video library. The JVET 360 video common test conditions specified in [1] were followed except that the viewing direction angles for two viewports were set to (0, 90) and (0, -90), respectively.

Table 1 Summary results of using HM16.14

Table 1 summarizes the experimental results by using HM16.14. On average over 30% BD-rate increase relative to ERP was observed for viewport quality along the North and South Pole.

Note that the run time in Table 1 may not be accurate due to the heterogeneous nature of LSF farm.

The detailed results can be found in the attached spreadsheet HM-16.14-360Lib-ERP_vs_SSP.

Further experiments were carried out to investigate whether viewport quality loss around the North and South Pole is common to other alternative projection formats such as CMP and CISP. In this case, the viewing directions were set to (0, 0) and (0, 90), so that information about to what extent the viewport quality loss around the North and South Pole is converted to viewport quality gain in typical viewing directions can also be studied.

The RA results for SSP, CMP, and CISP are summarized in Table 2, 3, and 4. The detailed results can be found in the attached spreadsheets HM-16.14-360Lib-ERP_vs_SSP_suppl, HM-16.14-360Lib-ERP_vs_CMP_suppl and HM-16.14-360Lib-ERP_vs_CISP_suppl. The viewport quality loss around the North and South Pole seems to be common to alternative projection formats.

Table 2 RA summary results for SSP

Table 3 RA summary results for CMP

Table 4 RA summary results for CISP

It has been noticed that WS-PSNR behaves inconsistently in some cases. In the first case, the WS-PSNR reports an average gain of 2.5% and 15.8% for SSP vertical layout and SSP horizontal layout of  RA configuration, respectively (see Table 5).

Table 5 Comparison of RA results between SSP vertical and horizontal layout

The second example of inconsistent behavior of WS-PSNR is shown in Table 6, in which it reports 0.6% loss for CMP vs. ERP in RA configuration, but reports an average gain of 15.9% in LD_B configuration.

Table 6 CMP summary results (RA vs. LD-B)

# References

[1] E. Alshina, J. Boyce, A. Abbas, Y. Ye , “JVET common test conditions and evaluation procedures for 360° video”, JVET-B1030, February, 2016.

[2] C. Zhang, Y. Lu, J. Li, Z. Wen, “AHG8: Segmented Sphere Projection for 360-degree video”, JVET-E0025, January 2017.

# Patent rights declaration(s)

Broadcom Limited does not have current or pending patent rights relating to the technology described in this contribution.