| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F_Notes_d0 |
| --- | --- |

| Title: | Meeting Report of the 6th meeting of the Joint Video Exploration Team (JVET), Hobart, AU, 31 March – 7 April 2017 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from responsible coordinators of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Responsible coordinators |  |  |

_____________________________

- Summary

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its sixth meeting during 31 March – 7 April 2017 Wrest Point Hotel, Hobart, Tasmania, AU. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations. For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.13 of this document.

The JVET meeting sessions began at approximately 0900 hours on Friday 31 March 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 7 April 2017. Approximately XXX people attended the JVET meeting, and approximately XX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. The JVET meeting also performed an evaluation of compression technology designs proposed in this area, and refined the definition of test cases for evaluating such technology in a rigid manner, including the planning of a Call for Evidence for which responses are expected by the 7th meeting in July 2017.

One primary goal of the meeting was to review the work that was performed in the interim period since the fifth JVET meeting in producing the Joint Exploration Test Model 5 (JEM5). In this context, results from three exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM6 were planned.

The JVET produced XX output documents from the meeting:

- Algorithm description of Joint Exploration Test Model 6 (JEM6)

- Joint Call for Evidence on video compression with capability beyond HEVC

- Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (new version?)

- Description of Exploration Experiments on coding tools

- Common test conditions and evaluation procedures for HDR/WCG video (new version?)

- Common test conditions and evaluation procedures for 360 video (new version?)

For the organization and planning of its future work, the JVET established X "ad hoc groups" (AHGs) to progress the work on particular subject areas. X Exploration Experiments (EE) were defined on particular subject areas of coding tool testing. The next four JVET meetings are planned for Fri. 14 – Fri. 21 Jul. 2017 under WG 11 auspices in Torino, IT, during Wed. 18. – Wed. 25 Oct. 2017 under ITU-T auspices in Macao, CN, during Fri. 19 – Fri. 26 Jan. 2018 under WG 11 auspices in Gwangju, KR, and … .

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Exploration Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its sixth meeting during 31 March – 7 April 2017 Wrest Point Hotel, Hobart, Tasmania, AU. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations.

## Meeting logistics

The JVET meeting sessions began at approximately 0900 hours on Friday 31 March 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 7 April 2017. Approximately XXX people attended the JVET meeting, and approximately XX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. The JVET meeting also performed an evaluation of compression technology designs proposed in this area, and refined the definition of test cases for evaluating such technology in a rigid manner, including the planning of a Call for Evidence for which responses are expected by the 7th meeting in July 2017.

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2017_03_F_Hobart/.

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the fifth JVET meeting in producing the Joint Exploration Test Model 5 (JEM5). In this context, results from three exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM5 were planned.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the JEM software but have no normative effect are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the JEM description (an error, oversight, or messiness) or in the software are marked by the string "Decision (BF):".

This meeting report is based primarily on notes taken by the responsible leaders. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Thursday, 23 March 2017. Any documents uploaded after 2359 hours Paris/Geneva time on Friday 24 March were considered "officially late", giving a grace period of 24 hrs to those living in different time zones of the world.

All contribution documents with registration numbers JVET-F0068 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "F0068+" range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- JVET-F0XXX (a proposal on …), uploaded 03-XX,

- …

The following technical design proposal contributions were both registered late and uploaded late:

- JVET-F0XXX (a proposal on …), uploaded 03-XX,

- …

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JVET-F0XXX (an information document on on …), uploaded 03-XX,

- ...

The following cross-verification reports were registered on time but were uploaded late: JVET-F0XXX [uploaded 03-XX], … .

(Documents that were both registered late and uploaded late, other than technical proposal documents, are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-F0XXX, ….

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable rejected in the document management system. The initial uploads of the following contribution documents were rejected as "placeholders" and were not corrected until after the upload deadline: (This case did not happen at the current meeting).

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, EE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly the meeting report JVET-E1000, JEM5 algorithm description JVET-E1001, the Preliminary Joint Call for Evidence JVET-E1002, the algorithm descriptions of projection format conversion and video quality metrics in 360Lib JVET-E1003, the description of exploration experiments JVET-E1011, the JVET common test conditions and evaluation procedures for HDR/WCG video JVET-E1020, and the JVET common test conditions and evaluation procedures for 360 video JVET-E1030, were approved. The JEM5 software implementation (versions 5.0 and 5.01), and the 360Lib software implementation (versions 2.0.1 and 2.1) were also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Review of results of previous meeting

- Review of AHG reports

- Reports of exploration experiments

- Consideration of contributions and communications on project guidance

- Consideration of video technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the JEM software uses the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the JEM software, and for evaluating proposals for technology to be included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

Note: This applies also to the 360Lib video conversion software as well as the JEM and HM.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It is reminded to send notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers was XXX.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT: Adaptive multi-core transform.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP: Advanced temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BIO: Bi-directional optical flow.

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EE: Exploration Experiment – a coordinated experiment conducted toward assessment of coding technology.

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MDNSST: Mode dependent non-separable secondary transform.

- MMLM: Multi-model (cross component) linear mode.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- QTBT: Quadtree plus binary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- UCBDS: Unrestricted center-biased diamond search.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in JEM:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in P/B slice, and one CTB per luma CTU and two CTBs per chroma CTU in I slice.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size to a CU.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size to a CU.

## Opening remarks

- Reviewed logistics, agenda, working practices

- Results of previous meeting: JEM, meeting report, etc.

- Goals of the meeting: New version of JEM, evaluation of status progress in EEs and new proposals, selection of test sequences for testing, expert viewing assessment of JEM status, improved 360Lib software, provide summary to parent bodies, define new EEs.

- Produce Draft Call for Evidence (to be issued by parent bodies)

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Fri. 31 Mar, 1st day

  - 0900-XXXX Opening, AHG reports (chaired by JRO and GJS)

  - …

- Sat. 1 Apr, 2nd day

  - …

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized (final number counts tbd)

- AHG reports (8) (section 2)

- Analysis, development and improvement of JEM (4) (section 3)

- Test material and Call for Evidence (6) (section 4)

- Exploration experiments (23) (section 5)

  - EE1 and related: Intra Prediction (11)

  - EE2 and related: Nonlinear in-loop filters (4)

  - EE3 and related: Decoder-side motion vector derivation (7)

- Non-EE technology proposals (7) (section 6)

- Extended colour volume coding (0) (section )

- 360 video (23) (section 8)

- Encoder optimization (XX) (section 9)

- Metrics and evaluation criteria (XX) (section 10)

- Withdrawn (XX) (section )

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 12)

- Project planning (section 13)

- Output documents, AHGs (section 14)

# AHG reports (8)

Contributions in this category were discussed Thursday 12 January 1420-1545 (chaired by GJS & JRO).

JVET-F0001 JVET AHG report: Tool evaluation (AHG1) [M. Karczewicz, E. Alshina]

JVET-F0002 JVET AHG report: JEM algorithm description editing (AHG2) [J. Chen, E. Alshina, J. Boyce] [miss]

JVET-F0003 JVET AHG report: JEM software development (AHG3) [X. Li, K. Sühring] [miss]

JVET-F0004 JVET AHG report: Test material (AHG4) [T. Suzuki, V. Baroncini, J. Chen, J. Boyce, A. Norkin]

Fast encoding, encoding complexity investigation, and configuration settings (AHG5) [K. Choi (chair), Y.-J. Chang, H. Huang, X. Li, P. Philippe, Y. Yasugi (vice chairs)]

JVET-F0006 JVET AHG Report: 360 video conversion software development [Y. He, V. Zakharchenko]

JEM coding of HDR/WCG material (AHG7) [A. Segall (chair), E. Francois, D. Rusanovskyy (vice chairs)]

JVET-F0008 JVET AHG report: 360 video coding tools and test conditions (AHG8) [J. Boyce, A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye]

# Analysis, development and improvement of JEM (4)

Contributions in this category were discussed XXday. XXth XXXX-XXXX (chaired by XXX).

JVET-F0023 AHG3: JEM coding results for adaptive search range [K. Suehring, K. Sharman, B. Bross]

JVET-F0029 On memory bandwidth of JEM5.0 tools [E.Alshina (Samsung)]

JVET-F0044 AHG5: Performance evaluation of Adaptive Search Range on JEM-5.0.1 [Y. Yasugi, T. Ikai (Sharp)]

JVET-F0076 Request for 4:4:4 support in the JEM software [M. Meyer, A. M. Tourapis, D. Singer (Apple)] [late]

# Test material and Call for Evidence(6)

## New test material proposals (1)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-F0062 New aerial photography sequences for video coding standard development [?? (??)] [miss] [late]

TBP

## Call for Evidence (5)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-F0040 Revised Preliminary Joint Call for Evidence on Video Compression with Capability beyond HEVC [M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki (eds.)]

JVET-F0060 AHG4: Report on CfE anchor generation for SDR content with HM [K. Choi, E. Alshina (Samsung)]

JVET-F0061 AHG4: Report on CfE anchor generation for SDR content with JEM [K. Choi, E. Alshina (Samsung)]

JVET-F0069 AHG4: SDR anchor generation for Preliminary Joint Call for Evidence by Qualcomm [H.-C. Chuang, J. Chen, M. Karczewicz] [late]

(also see contributions related to 360 degree video under 8.3)

JVET-F0086 AHG7: Information on CfE anchor generation for HDR content and comments on coding conditions [D. Rusanovskyy (Qualcomm), E. Francois (Technicolor)]

# Exploration experiments (23)

## General (1)

Contributions in this category were discussed XXday XXth XXXX-XXXX (chaired by XXX).

JVET-F0010 Exploration Experiments on Coding Tools Report [E. Alshina, L. Zhang]

## EE1: Intra prediction (11)

Contributions in this category were discussed XXday XXXX-XXXX (chaired by XXX).

From JVET-F0010 summary report:

…

### Primary (5)

JVET-F0024 EE1: Unequal Weight Planar Prediction, Constrained PDPC, and Constrained ARSS [K. Panusopone, S. Hong, Y. Yu, L. Wang (ARRIS)]

JVET-F0045 EE1: Cross-check of JVET-F0024 on Unequal Weight Planar Prediction, Constrained PDPC, and Constrained ARSS [T. Ikai (Sharp)]

JVET-F0047 EE1: Cross-check of test1 on unequal weight planar [J. Lee, H. Lee, J. Kang (ETRI)] [miss] [late]

JVET-F0056 EE1: Cross-check of Test 6 with UW planar and restricted ARSS [V. Seregin (Qualcomm)] [miss] [late]

JVET-F0070 EE1: Cross-check of Test 4 (UW-Planar with PDPC restricted for different number of modes) [S.-H. Kim, H. Jang, J. Lim (LGE)] [miss] [late]

### Related (6)

JVET-F0033 EE1-Related: UWP with Constrained PDPC and Constrained ARSS [H. Jang, J. Lim, J. Nam, S.-H. Kim (LGE)]

JVET-F0080 Cross-check of JVET-F0033: UWP with Constrained PDPC and Constrained ARSS [K. Panusopone, S. Hong, Y. Yu, L. Wang (ARRIS)] [miss] [late]

JVET-F0054 Non-EE1: Alternative setting for PDPC mode [M. Karczewicz, N. Hu, X. Zhao, V. Seregin (Qualcomm)]

JVET-F0078 Cross-check of JVET-F0054: Non-EE1: Alternative setting for PDPC mode [F. Le Léannec, T. Poirier, E. François (Technicolor)] [miss] [late]

JVET-F0055 Non-EE1: Explicit flag signalling for ARSS [V. Seregin, X. Zhao, M. Karczewicz (Qualcomm)]

JVET-F0081 Cross-check of JVET-F0055: Explicit flag signalling for ARSS [K. Panusopone, Y. Yu, L. Wang (ARRIS)] [miss] [late]

## EE2: Nonlinear in-loop filters (4)

Contributions in this category were discussed XXday XXXX-XXXX (chaired by XXX).

From JVET-F0010 summary report:

…

### Primary (4)

JVET-F0034 EE2-JVET-E0032 Bilateral filter Test 1, Test2 [J. Ström, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. Sjöberg (Ericsson)]

JVET-F0048 EE2: Cross-check of bilateral filter (JVET-F0034 test 2) [S.-C. Lim, J. Kang (ETRI)] [miss] [late]

JVET-F0057 EE2: Cross-check of bilateral filter (JVET-F0034 Test1, Test2) [L. Zhang (Qualcomm)] [miss] [late]

JVET-F0073 EE2: Cross-check of JVET-F0034 Bilateral filter Test 1, Test 2 [Z. Deng (Intel)] [miss] [late]

### Related (7)

## EE3: Decoder-side motion vector derivation (7)

Contributions in this category were discussed XXday XXXX-XXXX (chaired by XXX).

From JVET-F0010 summary report:

…

### Primary (4)

JVET-F0028 EE3: BIO w/o block extension [A.Alshin, E. Alshina (Samsung)]

JVET-F0059 EE3: Crosscheck of JVET-F0028 BIO w/o block extension [Y.-W. Chen, X. Li (Qualcomm)] [miss] [late]

JVET-F0032 EE3: Enhanced FRUC Template Matching Mode [Y. Lin, X. Chen, J. An, J. Zheng (HiSilicon)]

JVET-F0071 EE3: Crosscheck for Enhanced FRUC Template Matching Mode (JVET-F0032) [W. Zhang (Hulu)] [miss] [late]

### Related (3)

JVET-F0022 EE3-related: A block-based design for Bi-directional optical flow (BIO) [H.-C. Chuang, J. Chen, X. Li, Y.-W. Chen, M. Karczewicz, W.-J. Chien (Qualcomm)]

JVET-F0030 Cross-check for a block-based design for Bi-directional optical flow (JVET-F0022) [E. Alshina (Samsung)] [late] [miss]

JVET-F0082 Cross-check of JVET-F0022 (EE3-related: A block-based design for Bi-directional optical flow) [O. Nakagami (Sony)] [miss] [late]

# Non-EE Technology proposals (7)

Contributions in this category were discussed XXday XXth XXXX-XXXX (chaired by XXX).

JVET-F0031 Reduction redundant syntax signaling for transform skip [H. Jang, J. Lim, J. Nam, S.-H. Kim (LGE)]

JVET-F0079 Cross-check of JVET-F0031 (Redundant syntax signaling reduction for transform skip) [V. Lorcy (b-com), P. Philippe (Orange)] [miss] [late]

JVET-F0043 NSST memory reduction [S.-H. Kim, H. Jang, J. Lim (LGE)]

JVET-F0075 Crosscheck of JVET-F0043 NSST memory reduction [?? (??)] [miss] [late]

JVET-F0051 Unified Adaptive Loop Filter for Luma and Chroma [J. An, J. Zheng (HiSilicon)]

JVET-F0058 Cross-check of JVET-F0051 Unified Adaptive Loop Filter for Luma and Chroma [L. Zhang (Qualcomm)] [miss] [late]

JVET-F0068 An Update to the Tone Mapping Information SEI Message [P. Topiwala, M. Krishnan, W. Dai (FastVDO)] [miss] [late]

Off-topic?

# Extended colour volume coding (0)

Contributions in this category were discussed XXday XXth XXXX-XXXX (chaired by XXX).

## Test conditions and evaluation (0)

## Tools (0)

# Coding of 360 video projection formats (23)

Contributions in this category were discussed XXday XXth XXXX-XXXX (chaired by XXX).

## Conversion tools, 360lib (2)

JVET-F0041 AHG8: Platform independent floating point to integer conversion for 360Lib [Y. He, P. Hanhart, Y. Ye (InterDigital)]

JVET-F0065 360Lib modifications for spherical rotation [Q. Xu, J. Boyce (Intel), Y. He, Y. Ye (InterDigital)]

## Packing and Projection formats (10)

JVET-F0025 AHG8: Adjusted cubemap projection for 360-degree video [M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-F0026 AHG8: Equatorial cylindrical projection for 360-degree video [G. Van der Auwera, M. Coban, M. Karczewicz (Qualcomm)]

JVET-F0035 AHG8: Additional test results of JVET-E0090 on nested polygonal chain packing of 360-degree ERP pictures [K. Kammachi-Sreedhar, M. M. Hannuksela (Nokia)] [miss] [late]

JVET-F0036 AHG8: Rotated Sphere Projection for 360 Video [A. Abbas, D. Newman (GoPro)]

JVET-F0085 AHG8: Crosscheck of JVET-F0036 Rotated sphere projection for 360 video [Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital)]

JVET-F0037 AHG8: Padding method for Segmented Sphere Projection [C. Zhang, Y. Lu, J. Li, Z. Wen (??)] [miss] [late]

JVET-F0052 AHG8: EAP-based segmented sphere projection with padding [Y.-H. Lee, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

JVET-F0077 AHG8: Crosscheck of JVET-F0052 EAP-based segmented sphere projection with padding [Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital)] [miss] [late]

JVET-F0053 AHG8: An improvement on compact octahedron projection with padding [Y.-H. Lee, H.-C. Lin, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

JVET-F0084 Cross-check of JVET-F0053 on An improvement on compact octahedron projection with padding [V. Zakharchenko (Samsung)]

## Testing procedure and metrics (7)

JVET-F0021 AHG8: Subjective testing of 360º video projection/packing formats [J. Boyce, Z. Deng (Intel)]

JVET-F0027 AhG8: JEM vs HM performance for 360-video content under CfE test condition [A. Singh, C. Pujara, S. N. Akula, A. Dsouza, Ramkumaar K. K., R. N. Gadde, V. Zakharchenko, E. Alshina, K. P. Choi (Samsung)]

JVET-F0042 AHG8: On cross-format S-PSNR-NN [Y. He, X. Xiu, Y. Ye (InterDigital)]

JVET-F0050 AhG8: Dynamic viewport based subjective evaluation of 360-degree video coding under CfE test conditions [P. Hanhart, Y. He, Y. Ye (InterDigital)] [miss] [late]

JVET-F0066 AHG8: On inactive pixels in evaluation of 360 video projections [G. Van der Auwera, M. Coban (Qualcomm)] [miss] [late]

JVET-F0067 Comments on subjective testing procedure of 360 video [K. Kawamura, S. Naito (KDDI)]

JVET-F0083 AHG8: Subjective test pilot study of 360º video projection/packing formats [Z. Deng, L. Xu, J. Boyce (Intel)] [miss] [late]

## Coding tools (4)

JVET-F0038 AHG8: adaptive QP for 360 video coding [F. Racape, F. Galpin, G. Rath, E. Francois (Technicolor)]

JVET-F0039 AHG8: Codec behavior on 360 videos [F. Galpin, F. Racape, E Francois (Technicolor)]

JVET-F0049 AHG8: Adaptive QP for ERP 360º video [Hendry, M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-F0072 AHG8: Stretching ratio based adaptive quantization for 360 video [Y. Sun, L. Yu (??)] [miss] [late]

## HL syntax (0)

# Encoder optimization (1)

Contributions in this category were discussed XXday XXth XXXX-XXXX (chaired by XXX).

JVET-F0063 AHG5: Enhanced fast algorithm of JVET-E0078 [P.-H. Lin, C.-L. Lin, Y.-J. Chang, C.-C. Lin, Y.-H. Ju (ITRI)]

# Metrics and evaluation criteria (1)

Contributions in this category were discussed XXday XXth XXXX-XXXX (chaired by XXX).

JVET-F0064 MS-SSIM as an additional mandatory metric to PSNR for future video coding [M. Pettersson, R. Sjöberg, P. Wennersten, K. Andersson, J. Ström, J. Enhorn (Ericsson)]

# Withdrawn (2)

See under 1.4.2.

JVET-F0046 Withdrawn

JVET-F0074 Withdrawn

# Joint Meetings, BoG Reports, and Summary of Actions Taken

## Exploration Experiments (update)

The setup of Exploration Experiments was discussed, and an initial draft of the EE document was reviewed in the plenary (chaired by JRO). This included the list of all tools that are intended to be investigated in EEs during the subsequent meeting cycle:

EE1: Intra prediction

JVET-E0068 Unequal Weight Planar Prediction and Constrained PDPC [K. Panusopone, S. Hong, L. Wang (ARRIS)]

EE2: Nonlinear in-loop filters

JVET-E0032 Bilateral filter strength based on prediction mode [J. Ström, P. Wennersten, K. Andersson, J. Enhorn (Ericsson)] (also test cases with reduced lookup table memory)

EE3: Decoder Side Motion Vector Derivation

JVET-E0028 EE3: bi-directional optical flow w/o block extension [A. Alshin, E. Alshina (Samsung)] (investigate version without block size dependent weighting)

JVET-E0035 Enhanced Template Matching in FRUC Mode [Y. Lin, X. Chen, J. An, J. Zheng (HiSilicon)]

Li Zhang is mandated to compile the EE document with remote assistance by Elena Alshina, to be circulated by Thursday and reviewed Friday.

It was agreed to give the editors the discretion to finalize the document during the two weeks after the meeting, and circulate/discuss it on the reflector appropriately.

## Joint meetings

## BoGs

## List of actions taken affecting JEM6 (update)

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the JEM5 description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

Was presented and confirmed to be complete Thu morning in the JVET plenary.

### Encoder only or software changes

JVET-E0059 Floating point QP support for parallel encoding in RA configuration [X. Ma, H. Chen, H. Yang, M. Sychev (Huawei)]

Also replace “floating point QP” by base QP, and frame position.

Replace previous COHP1 by this, still keep COHP2.

JVET-E0023 AHG5: Improved fast encoding setting [Y. Yamamoto, T. Ikai, Y. Yasugi (Sharp)]

Adopt JVET-E0023 test case B (skip depth 2 always for LDB and LDP, skip depth 2 for highest temporal layer in RA, other layers skip depth 3).

### Syntax/semantics/decoding process changes

JVET-E0052 EE3: Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching [X. Chen, J. An, J. Zheng (HiSilicon)]

Adopt JVET-E0052, with integer step search (8 positions around the start position). Also implement a high-level flag (in SPS) to disable the tool.

JVET-E0060 EE3-JVET-D0046: High precision FRUC with additional candidates [A. Robert, F. Le Léannec, T. Poirier (Technicolor)]

Adopt JVET-E0060 Test 3 configuration

JVET-E0076 EE4: Enhanced Motion Vector Difference Coding [J. Chen, W.-J. Chien, M. Karczewicz, X. Li (Qualcomm)]

Adopt the 4-luma sample precision aspect of the proposal

JVET-E0062 EE5: Multiple Direct Modes for chroma intra coding [L. Zhang, W.-J. Chien, J. Chen, X. Zhao, M. Karczewicz (Qualcomm)]

Adopt a modified version, where the number of MPM for chroma is still kept as 6, and only the list construction is modified with the first six as proposed. This means that the current bitstream syntax is not changed, only the semantics.

JVET-E0077 EE5: Enhanced Cross-component Linear Model Intra-prediction [K. Zhang, J. Chen, L. Zhang, M. Karczewicz (Qualcomm)]

Adopt the combination of test 2 (MMLM+MFLM).

JVET-E0104 ALF temporal prediction with temporal scalability [L. Zhang, W.-J. Chien, M. Karczewicz]

### Changes in 360lib

JVET-E0025 AHG8: Segmented Sphere Projection for 360-degree video [C. Zhang, Y. Lu, J. Li, Z. Wen (Owl Reality)]

Add the new layout to the 360Lib software.

JVET-E0029 AHG8: Efficient Frame Packing for Icosahedral Projection [S. N. Akula, A. Singh, A. Dsouza, Ram Kumaar K. K., C. Pujara, R. N. Gadde, V. Zakharchenko, E. Alshina, K. P. Choi, (Samsung)]

Replace the current ISP packing scheme by the one proposed in JVET-E0029, version with padding of 64 luma samples width (horizontally), and 32 luma samples height (vertically). Only used for the discontinuous triangles. Padding shall be included in the description.

JVET-E0056 AHG8: An improvement on the compact OHP layout [H.-C. Lin, C.-C. Huang, C.-Y. Li, Y.-H. Lee, J.-L. Lin, S.-K. Chang (MediaTek)]

Replace previous COHP1 by this, still keep COHP2.

# Project planning (update)

## JEM description drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- JEM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XXday XX April 2017.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for Experiments

Note: This section was drafted during the second JVET meeting, and is kept here for information about the EE procedure.

Group coordinated experiments have been planned. These may generally fall into one category:

- "Exploration experiments" (EEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the main branch of JEM by the next meeting.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. (E. Alshina will edit the document based on input from the proponents, review is performed in the plenary)

- Software for tools investigated in EE is provided in a separate branch of the software repository

- During the experiment, further improvements can be made

- By the next meeting it is expected that at least one independent party will report a detailed analysis about the tool, confirms that the implementation is correct, and gives reasons to include the tool in JEM

- As part of the experiment description, it should be captured whether performance relative to JEM as well as HM (with all other tools of JEM disabled) should be reported by the next meeting.

It is possible to define sub-experiments within particular EEs, for example designated as EEX.a, EEX.b, etc., where X is the basic EE number.

As a general rule, it was agreed that each EE should be run under the same testing conditions using one software codebase, which should be based on the JEM software codebase. An experiment is not to be established as a EE unless there is access given to the participants in (any part of) the TE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-B1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to EE work should identify individuals in addition to company names.

EE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the EE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish an EE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to EE activities were established as follows:

- Only qualified JVET members can participate in an EE.

- Participation in an EE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the EE should be announced and made available to all EE participants in a timely manner.

A separate branch under the experimental section will be created for each new tool include in the EE. The proponent of that tool is the gatekeeper for that separate software branch. (This differs from the main branch of the JEM, which is maintained by the software coordinators.)

New branches may be created which combine two or more tools included in the EE document or the JEM. Requests for new branches should be made to the software coordinators.

Don’t need to formally name cross-checkers in the EE document. To promote the tool to the JEM at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

Timeline:

T1 = JEM5.0 SW release + 4 weeks: Integration of all tools into separate EE branch of JEM is completed and announced to JVET reflector.

Initial study by cross-checkers can begin.

Proponents may continue to modify the software in this branch until T2

3rd parties encouraged to study and make contributions to the next meeting with proposed changes

T2: JVET-F meeting start – 3 weeks: Any changes to the exploration branch software must be frozen, so the cross-checkers can know exactly what they are cross-checking. An SVN tag should be created at this time and announced on the JVET reflector.

This procedure was again confirmed during the closing plenary of the third JVET meeting. It was further confirmed that the Common Test Conditions of JVET-B1010 are still valid, however the CTC encoder setting will be reflected in the config file that is attached to the JEM4.0 package.

## Software development and anchor generation

Software coordinators will work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- JEM5.0 including all adoptions from section 12.4 will be released by 2017-02-06.

- The results about coding performance of JEM5.0 will be reported by 2017-02-17.

- Further versions may be released for additional bug fixing, as appropriate

- Encoder software optimized for luma-dependent quantizer adaptation will be ported as separate branch by the AHG on HDR/WCG within two weeks after JEM5.0 release.

Bug tracker for 360 will be established as part of JEM bug tracker.

Timeline of 360Lib2.0: 4 weeks after the meeting (2017-02-17).

Further versions may be released as appropriate for bug fixing.

Timelines and volunteers for CfE anchors:

- For SDR: See under BoG JVET-E0132

- For 360 HM anchors: Samsung, InterDigital will provide them until Feb. 15, Qualcomm does cross-check. Generation of JEM anchors will be done under AHG mandates until next meeting.

- For HDR, Qualcomm, Sharp, Technicolor and Netflix will provide/crosscheck HM anchors by Feb. 15. Generation of JEM anchors will be done under AHG mandates until next meeting.

As it is unlikely that HM 16.15 would be available in time, HM 16.14 should be used with appropriate config file change

# Output documents and AHGs (update)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JVET-E1000 Meeting Report of the 5th JVET Meeting [G. J. Sullivan, J.-R. Ohm] [2017-03-30] (near next meeting)

Intermediate versions of the meeting notes (d0 … d8) were made available on a daily basis during the meeting.

JVET-E1001 Algorithm description of Joint Exploration Test Model 5 (JEM5) [J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce] [2017-02-13] (MPEG N16698)

See list of new adoptions under 12.4. During the closing plenary, no complaints were made about the accuracy of that list.

JVET-E1002 Preliminary Joint Call for Evidence on Video Compression with Capability beyond HEVC [M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki] [2017-01-27] (MPEG N16697)

Draft was discussed Thu 1130 and in joint meeting with parent bodies Thu 1400.

The companies responsible for providing the HM anchors will also provide the corresponding Excel templates for the cases of SDR, HDR and 360.

Note: Work plan should be in the mandate of the AHG, and any planning for responsibilities should appear in the BoG reports.

In the closing plenary, it was further decided that Tango should be moved to the set of rates starting at 1.5 Mbps, since HM is not capable of encoding 1 Mbps.

JVET-E1003 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib [Y. Ye, E. Alshina, J. Boyce] [2017-03-03] (MPEG N16699)

First version to be made available by 2017-02-10

JVET-B1010 JVET common test conditions and software reference configurations [K. Suehring, X. Li]

remains valid (from 2nd meeting).

Note: Encoder settings reflected in the config file related to CTC in JEM5 (see changes under 12.4)

A directory had been installed in the ftp for bitstreams and results of anchors, but in the previous meeting cycle this had not yet been used. It is planned to upload bitstreams and Excel sheets after completion of anchors.

JVET-E1011 Description of Exploration Experiments on coding tools [E. Alshina, L. Zhang] [2017-02-03] (MPEG N16700)

Initial version was presented in the closing plenary on Friday 20th Jan, and minor changes were made: Bilateral filter test3 (CTU based on/off) and investigation on normalization was removed from the initial draft.

See list of EEs under 12.1.

JVET-E1020 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. Francois, D. Rusanovskyy] [2017-03-02] (MPEG N16701)

JVET-E1030 JVET common test conditions and evaluation procedures for 360 video [E. Alshina, J. Boyce, A. Abbas, Y. Ye] [2017-03-02 ] (MPEG N16702)

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header should be generated. This version should be sent to GJS and JRO for upload.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Tool evaluation (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate the exploration experiments. - Investigate interaction of tools in JEM and exploration experiment branches. - Discuss and evaluate methodologies and criteria to assess the benefit of tools, and how to ease the assessment of single tools in terms of encoder runtime. - Study and summarize new technology proposals. | E. Alshina, M. Karczewicz (co-chairs) | N |
| JEM algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-E1001 Algorithm Description of Joint Exploration Test Model 5. - Gather and address comments for refinement of the document. - Coordinate with the JEM software development AHG to address issues relating to mismatches between software and text. | J. Chen (chair) E. Alshina, J. Boyce (vice chairs) | N |
| JEM software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of the JEM5.0 software packages and their distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver JEM5.0 software version and the reference configuration encodings according to JVET-B1010 common conditions. - Coordinate with AHG on JEM model editing and errata reporting to identify any mismatches between software and text, and make further updates to the software as appropriate. - Investigate parallelization for speedup of simulations. - Investigate the implementation of SCC coding tools in JEM. - Coordinate with AHG6 for integration of 360 video software. | X. Li, K. Suehring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of future video coding standards. - Identify and recommend appropriate test materials and corresponding test conditions for use in the development of future video coding standards. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Prepare HM and JEM SDR anchors defined in the preliminary CfE. - Discuss further visual assessment plan to confirm CfE anchors. - Prepare for the visual assessment in the next meeting. | V. Baroncini, T. Suzuki (co-chairs), J. Chen, J. Boyce, A. Norkin (vice chairs) | N |
| Fast encoding, encoding complexity investigation, and configuration settings (AHG5) (jvet@lists.rwth-aachen.de) - Study encoder configuration settings of JEM5 software and suggest alternative configuration files for different operating points in terms of encoder complexity versus compression benefit. - Investigate and develop fast methods to reduce JEM5 encoding complexity. | K. Choi (chair), Y.-J. Chang, H. Huang, X. Li, P. Philippe, Y. Yasugi (vice chairs) | N |
| 360 video conversion software development (AHG6) (jvet@lists.rwth-aachen.de) - Prepare and deliver 360Lib2.0 software version and common test condition configuration files according to JVET-E1030 common conditions. - Produce documentation of software usage for distribution with the software. - Coordinate with JEM and HM software coordinators for integration of 360Lib2.0 within most recent versions of HM and JEM software packages, and set up a bug tracker. - Review integration framework and refine the interface to minimize the needed software changes in HM and JEM. | Y. He, V. Zakharchenko (co-chairs) | N |
| JEM coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Study and refine conversion practices to create sequences in HDR/WCG containers with emphasis on BT.2020/BT.2100 and SMPTE ST 2084. - Study and refine test conditions and anchors for the JEM coding of HDR/WCG content. - Coordinate generation of HM and JEM anchors for the CfE. - Create and release software supporting recommended conversion practices and test conditions JVET-E1020. - Study and evaluate available HDR/WCG test content including both HLG and PQ content. - Study and evaluate visual quality assessment methods including rendering content with a peak brightness larger than available displays. - Study objective metrics for quality assessment of HDR/WCG material. - Study methods for increased coding efficiency of HDR/WCG material. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. Francois, D. Rusanovskyy (vice chairs) | N |
| 360 video coding tools and test conditions (AHG8) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Study consistency of and potential improvements to the objective quality metrics in CTC. - Coordinate effort to prepare for finalized CfE, including anchor generation, selection of sequences and bit rates, and subjective quality evaluation. - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Define subjective testing methodology, in coordination with the MPEG OMAF AhG. - Produce and finalize JVET-E1003 algorithm descriptions of projection format conversion process and objective quality metrics in 360Lib. - Produce and finalize JVET-E1030 JVET common test conditions and evaluation procedures for 360 video. Generate CTC anchors and a reporting template for the common test conditions. - Study coding tools dedicated to 360 video, and their impact on compression. | J. Boyce (chair), A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye (vice chairs) | Y (1-3 phone meetings) |

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Wednesday or Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–7.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 7.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Fri. 14 – Fri. 21 Jul. 2017, 7th meeting under WG 11 auspices in Torino, IT.

- Wed. 18 – Wed. 25 Oct. 2017, 8th meeting under ITU-T auspices in Macao, CN.

- Fri. 19 Jan. – Fri. 26 Jan. 2018, 9th meeting under WG 11 auspices in Gwangju, KR.

- …

The agreed document deadline for the 7th JVET meeting is XXday XX July 2017. Plans for scheduling of agenda items within that meeting remain TBA.

ITU-T was thanked for the excellent hosting of the 5th meeting of the JVET. EBU, GBTech, ITU-T, and Technicolor were thanked for providing viewing equipment. Vittorio Baroncini was thanked for conducting visual tests, Teruhiko Suzuki, Maxim Sychev and Roman Chernyak were thanked for their great help in organizing the tests. The participants in the expert viewing were also thanked.

The 6th JVET meeting was closed at approximately XXXX hours on Friday 7 April 2017.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the sixth meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately XXX people in total), were as follows:

- …