| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F_Notes_dD |
| --- | --- |

| Title: | Meeting Report of the 6th meeting of the Joint Video Exploration Team (JVET), Hobart, AU, 31 March – 7 April 2017 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from responsible coordinators of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Responsible coordinators |  |  |

_____________________________

- Summary

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29/‌WG 11 held its sixth meeting during 31 March – 7 April 2017 Wrest Point Hotel, Hobart, Tasmania, AU. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations. For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section  of this document.

The JVET meeting sessions began at approximately 0900 hours on Friday 31 March 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1155 hours on Friday 7 April 2017. Approximately 138 people attended the JVET meeting, and approximately 85 input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. The JVET meeting also performed an evaluation of compression technology designs proposed in this area, and refined the definition of test cases for evaluating such technology in a rigid manner, including the planning of a Call for Evidence for which responses are expected by the 7th meeting in July 2017.

One primary goal of the meeting was to review the work that was performed in the interim period since the fifth JVET meeting in producing the Joint Exploration Test Model 5 (JEM5). In this context, results from three exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM6 were planned.

The JVET produced 7 output documents from the meeting:

- Algorithm description of Joint Exploration Test Model 6 (JEM6)

- Joint Call for Evidence on video compression with capability beyond HEVC

- Algorithm descriptions of projection format conversion and video quality metrics in 360Lib

- Description of Exploration Experiments on coding tools

- Common test conditions and evaluation procedures for HDR/WCG video

- Common test conditions and evaluation procedures for 360° video

- Subjective testing method for 360° Video projection formats using HEVC

For the organization and planning of its future work, the JVET established 9 "ad hoc groups" (AHGs) to progress the work on particular subject areas. Four Exploration Experiments (EE) were defined on particular subject areas of coding tool testing. The next four JVET meetings are planned for Fri. 13 – Fri. 21 Jul. 2017 under WG 11 auspices in Torino, IT, during Wed. 18. – Wed. 25 Oct. 2017 under ITU-T auspices in Macao, CN, during Fri. 19 – Fri. 26 Jan. 2018 under WG 11 auspices in Gwangju, KR, and 13–20 April 2018 in San Diego, US.

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Exploration Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29/‌WG 11.

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29/‌WG 11 held its sixth meeting during 31 March – 7 April 2017 Wrest Point Hotel, Hobart, Tasmania, AU. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations.

## Meeting logistics

The JVET meeting sessions began at approximately 0900 hours on Friday 31 March 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1155 hours on Friday 7 April 2017. Approximately 138 people attended the JVET meeting, and approximately 85 input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. The JVET meeting also performed an evaluation of compression technology designs proposed in this area, and refined the definition of test cases for evaluating such technology in a rigid manner, including the planning of a Call for Evidence for which responses are expected by the 7th meeting in July 2017.

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2017_03_F_Hobart/.

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the fifth JVET meeting in producing the Joint Exploration Test Model 5 (JEM5). In this context, results from three exploration experiments were also reviewed. Another important goal was to review the work that had been conducted for investigating the characteristics of new test material in the assessment of video compression technology. Furthermore, technical input documents were reviewed, and modifications towards JEM5 were planned.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that are considered especially important (e.g., if they could affect the normative content of a future standard) are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the JEM software but have no normative effect are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the JEM description (an error, oversight, or messiness) or in the software are marked by the string "Decision (BF):".

This meeting report is based primarily on notes taken by the responsible leaders. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Thursday, 23 March 2017. Any documents uploaded after 2359 hours Paris/Geneva time on Friday 24 March were considered "officially late", giving a grace period of 24 hrs to those living in different time zones of the world.

All contribution documents with registration numbers JVET-F0068 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "F0068+" range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- JVET-F0035 (a proposal providing additional results on nested polygonal chain packing or 360-degree video), uploaded 03-27,

- JVET-F0037 (a proposal on padding for segmented sphere projection), uploaded 03-28.

The following technical design proposal contributions were both registered late and uploaded late:

- JVET-F0072 (a proposal on adaptive quantization for 360-degree video), uploaded 03-30,

- JVET-F0096 (a proposal on division-free bilateral filter), uploaded 04-02,

- JVET-F0104 (a proposal on weighted angular intra prediction), uploaded 04-04,

- JVET-F0108 (a proposal on padded ERP projection format for 360-degree video), uploaded 04-06.

The following other documents not proposing normative technical content were uploaded late:

- JVET-F0050 (an information document on dynamic viewports for 360-degree video), uploaded 03-28,

- JVET-F0062 (a contribution proposing new test material of drone sequences), uploaded 03-29,

- JVET-F0066 (an information document on active pixel count for 360-degree video), uploaded 03-28,

- JVET-F0069 (an information document on SDR anchor generation for CfE), uploaded 03-27,

- JVET-F0076 (a contribution proposing 4:4:4 support in JEM), uploaded 03-28,

- JVET-F0083 (an information document on a subjective study for 360-degree video), uploaded 03-29,

- JVET-F0089 (an information document on quality metrics for extended colour volume content), uploaded 03-31,

- JVET-F0094 (an information document on analysis of HLG sequence characteristics), uploaded 03-31,

- JVET-F0102 (a project guidance advocacy document on industry recommendations for standards development), uploaded 04-03.

The following cross-verification reports were registered on time but were uploaded late: JVET-F0030 [uploaded 03-28], JVET-F0047 [uploaded 03-28], JVET-F0048 [uploaded 03-30], JVET-F0056 [uploaded 03-29], JVET-F0057 [uploaded 03-28], JVET-F0058 [uploaded 03-28]. Cross-check documents that were both registered late and uploaded lateare not listed in this section, in the interest of brevity.

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-F0046, JVET-F0068, JVET-F0074, JVET-F0130.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable rejected in the document management system. The initial uploads of the following contribution documents were rejected as "placeholders" and were not corrected until after the upload deadline: (This case did not happen at the current meeting).

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, EE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly the meeting report JVET-E1000, JEM5 algorithm description JVET-E1001, the Preliminary Joint Call for Evidence JVET-E1002, the algorithm descriptions of projection format conversion and video quality metrics in 360Lib JVET-E1003, the description of exploration experiments JVET-E1011, the JVET common test conditions and evaluation procedures for HDR/WCG video JVET-E1020, and the JVET common test conditions and evaluation procedures for 360° video JVET-E1030, were approved. The JEM5 software implementation (versions 5.0 and 5.01), and the 360Lib software implementation (versions 2.0.1 and 2.1) were also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Review of results of previous meeting

- Review of AHG reports

- Reports of exploration experiments

- Preparation of final Joint Call for Evidence

- Consideration of contributions and communications on project guidance

- Consideration of video technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the JEM software uses the HEVC reference software copyright license header language is the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the JEM software, and for evaluating proposals for technology to be included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

Note: This applies also to the 360Lib video conversion software as well as the JEM and HM.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It is reminded to send notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers was 657.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT: Adaptive multi-core transform.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP: Advanced temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BIO: Bi-directional optical flow.

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EE: Exploration Experiment – a coordinated experiment conducted toward assessment of coding technology.

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ECV: Extended Colour Volume (up to WCG).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- HDR: High dynamic range.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLG: Hybrid log-gamma.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MDNSST: Mode dependent non-separable secondary transform.

- MMLM: Multi-model (cross component) linear mode.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- QTBT: Quadtree plus binary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- UCBDS: Unrestricted center-biased diamond search.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in JEM:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in P/B slice, and one CTB per luma CTU and two CTBs per chroma CTU in I slice.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size to a CU.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size to a CU.

## Opening remarks

- Reviewed logistics, agenda, working practices

- Results of previous meeting: JEM, meeting report, etc.

- Goals of the meeting: New version of JEM, evaluation of status progress in EEs and new proposals, selection of test sequences for testing, expert viewing assessment of JEM status, improved 360Lib software, define new EEs.

- Produce Call for Evidence (to be issued by parent bodies)

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Fri. 31 Mar, 1st day

  - 0900–1300 Opening, AHG reports (chaired by JRO and GJS)

  - 1500–1700 BoG on subjective testing SDR/HDR (chaired by T. Suzuki)

- Sat. 1 Apr, 2nd day

  - 0900–1315 and 1430–1600 EE review (chaired by JRO)

  - 1615–1930 360° degree video subjective testing (chaired by JRO)

- Sun. 2 Apr, 3rd day

  - 0900 BoG report on test material and subjective assessment

  - 0930–1100 section 6 non-EE review (chaired by JRO)

  - 1115–1230 section 3 analysis & improvement of JEM (chaired by JRO)

  - 1230–1300 section 8 encoder optimization (chaired by JRO)

  - 1400–1500 Viewing session 360° video for CfE

  - 1500–1615 section 9 metrics and evaluation criteria (chaired by JRO)

  - 1615–1800 360° video subjective testing (chaired by JRO)

- Mon. 3 Apr, 4th day

  - 1600–1800 Joint meeting with parent bodies and JCT-VC: 360° video

  - 1500– BoG work on HDR

  - 1800–1900 360° video viewing

- Tue. 4 Apr, 5th day

  - 0900–1030 Review of BoG & viewing activities (chaired by JRO)

  - 1030–1245 section 7.4 360° video coding tools, discussion CTC (chaired by JRO)

  - 1430–1545 section 7.2 360° video projection formats (chaired by JRO)

  - 16:00– BoG on 360° video (chaired by Jill Boyce): 8.1 360Lib Software, remaining 8.2 documents, discussion of CTC, CfE1400–1430 Viewing session 360° video

  - 1500–1700 Viewing session HDR, new sequences

- Wed. 5 Apr, 6th day

  - 1400–1600 JVET plenary: BoG reports, Revisits, etc. (chaired by JRO)

  - 1600–1730 Joint meeting with parent bodies: CfE

  - 1730–1800: Viewing session 360° Video

- Thu. 6 Apr, 7th day

  - 0900–1230 JVET plenary: Review remaining docs, Revisits, EE planning, 360° video, etc. (chaired by JRO)

  - 1000–1300 BoG on HDR (chaired by A. Segall)

  - 1400–1600 JVET plenary: BoG report ECV/HDR, EE doc, AHG planning, etc. (chaired by JRO)

- Fri. 7 Apr, 8th day

  - 0900–1230 JVET closing plenary: Review and approve output docs, resolutions, setup AHGs, AOB, etc. (chaired by GS and JRO)

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized (estimated contribution counts are noted in parenthesis)

- AHG reports (8) (section 2)

- Analysis, development and improvement of JEM (5) (section 3)

- Test material and Call for Evidence (8) (section 4)

- Exploration experiments (29) (section 5)

  - General (1)

  - EE1 and related: Intra Prediction (13)

  - EE2 and related: Nonlinear in-loop filters (8)

  - EE3 and related: Decoder-side motion vector derivation (7)

- Non-EE technology proposals (7) (section 6)

- 360° video (30) (section 7)

- Encoder optimization (2) (section 8)

- Metrics and evaluation criteria (2) (section 9)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 10)

- Project planning (section 11)

- Output documents, AHGs (section 12)

# AHG reports (8)

Contributions in this category were discussed Thursday 12 January 1420–1545 (chaired by GJS & JRO).

JVET-F0001 JVET AHG report: Tool evaluation (AHG1) [M. Karczewicz, E. Alshina]

This document reports the work of the JVET ad hoc group on Tool evaluation (AHG1) between the 5th JVET meeting at Geneva, Switzerland (12–20 January 2017) and the 6th Meeting at Hobart, Australia (31 March – 7 April 2017).

Algorithms included into JEM5.0 are listed in the report. The biggest change is the addition of the third decoder side motion derivation technique (DMVR). Additionally to the luma BD-rate gain some performance improvement of JEM5.0 over JEM4.0 is observed for chroma due to the improvement both intra mode coding and cross-colour prediction for chroma.

Performance progress for JEM (HM-KTA) in terms of BD-rate gain vs. encoder time increase in random access test configuration is demonstrated in the figure below. Results are based on Software Development AHG reports. Noticeable encoder run-time reduction is observed for JEM5.0 compared to JEM4.0 (due to adopted SW optimization, JVET-E0023), but still encoder run time is more than a factor of 10 compared to the HM.

The progress of JEM performance in the RA test configuration is illustrated in the figure below.

JEM to HM encoding time ratio

KTA-1  KTA-2    JEM1     JEM2      JEM3     JEM4     JEM5

JVET Common Test Conditions use integer QP settings QP=22, 27, 32, 37 for all video sequences in the test set. The Preliminary CfE document JVET-E1002 requires bit-matching. The table below shows BD-rate performance JEM5.0.1 for the video test set from the CfE. The left part contains performance for QP =22, 27, 32, 37 and the right part is BD-rate calculated for bit-rates matching target rate within 2%. Under CfE test conditions, JEM5.0.1 achieves 33.7% BD-rate gain over HM with Encoder run time ×11 and decoder run time ×9.

JEM5.0 (5th meeting)

| Test configuration | BD-rate |  |  | Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. | Dec. |
| All Intra | −20% | −28% | −28% | 63 | 2 |
| Random Access | −29% | −35% | −34% | 12 | 10 |
| Low Delay-B | −22% | −29% | −29% | 10 | 8 |
| Low Delay-P | −26% | −31% | −32% | 7 | 5 |

JEM4.0 (4th meeting)

| Test configuration | BD-rate |  |  | Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. | Dec. |
| All Intra | −19% | −26% | −25% | 62 | 2 |
| Random Access | −28% | −33% | −32% | 13 | 11 |
| Low Delay-B | −22% | −27% | −28% | 10 | 8 |
| Low Delay-P | −25% | −30% | −31% | 8 | 5 |

Significant gain was observed in all three colour components. In random access testing, the highest gain over HEVC was observed for the CatRobot test sequence (38.7%), and the lowest gain the JEM showed was for the ToddlerFountain sequence (14.9% only).

JEM5.0.1 vs HM under CfE test conditions

| Reference: HM |  | QP=22, 27, 32, 37 |  |  |  |  | bit-matching |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V |
|  |  | Random Access CfE Main10 |  |  |  |  |  |  |  |
| A1 | Crosswalk1 | −33.8% | −31.8% | −35.7% | 1324% | 763% | −37.1% | −42.3% | −46.0% |
|  | FoodMarket3 | −31.4% | −32.3% | −36.4% | 1300% | 745% | −34.2% | −46.3% | −48.7% |
|  | Tango1 | −33.6% | −44.3% | −44.6% | 1532% | 673% | −35.7% | −54.6% | −48.6% |
|  | CatRobot | −38.9% | −48.1% | −40.5% | 1371% | 945% | −39.8% | −52.2% | −45.1% |
| A2 | DaylightRoad | −38.5% | −45.2% | −30.8% | 1521% | 503% | −40.2% | −53.3% | −37.9% |
|  | BuildingHall1 | −30.4% | −28.5% | −32.2% | 949% | 752% | −32.8% | −40.9% | −46.0% |
|  | ParkRunning2 | −27.1% | −17.7% | −18.9% | 1364% | 812% | −31.4% | −26.3% | −29.5% |
|  | CampfireParty | −34.1% | −36.4% | −57.9% | 1924% | 371% | −37.3% | −35.5% | −56.1% |
| 2K | BQTerrace | −29.0% | −44.4% | −52.0% | 1368% | 823% | −29.7% | −50.8% | −61.1% |
|  | RitualDance | −25.9% | −31.0% | −35.7% | 1490% | 710% | −26.9% | −37.8% | −41.1% |
|  | Timelapse | −22.7% | −42.8% | −47.1% | 1043% | 982% | −26.4% | −45.4% | −50.7% |
|  | BasketballDrive | −30.2% | −43.1% | −42.2% | 1660% | 661% | −31.5% | −46.7% | −43.2% |
|  | Cactus | −31.9% | −40.1% | −32.4% | 1563% | 892% | −35.6% | −48.9% | −44.5% |
|  | CfE | −31.3% | −37.4% | −38.9% | 1096% | 911% | −33.7% | −44.7% | −46.0% |

Here, “bit matching” refers to a BD interpolation from other QP settings – specifically, using those selected for the CfE.

At the 2nd JVET meeting, an Exploration Experiments practice was established. In the 5th JVET meeting, 3 EEs were created. For each new coding tool under consideration, a special software branch was created. After implementation of each tool, an announcement on JVET reflector was made. For all 3 EEs, an input contribution for this meeting were submitted. A summary of the exploration experiments is provided in JVET-F0010.

A reduction for the number of technical contributions was observed. In total, 16 contributions proposing new coding tools for the JEM were noted, improvements of JEM design or including comments on the existing design were submitted in following categories:

- Structure (2)

- Intra (4),

- Inter (5),

- Transform (3),

- Entropy (0)

- In-loop filter (1)

The AHG recommended to:

- Conduct viewing for visual quality comparison of JEM and HEVC during the meeting.

- Consider encoder complexity as one of the criteria when evaluating the tools. Encourage further encoder and decoder complexity reduction.

- Review all the related contribution.

- Continue the Exploration Experiments practice.

The use of a separate email list was noted in the EE description document, for people to contact the EE coordinators to get access to this reflector. One participant commented that they had difficulty getting access to that reflector. Such a separate email reflector will not be used in the future. EE discussions must take place on the main JVET reflector. Availability of large data files can be announced with the data available as an input document to the next meeting (or announced for interested parties to reply to obtain the data).

JVET-F0002 JVET AHG report: JEM algorithm description editing (AHG2) [J. Chen, E. Alshina, J. Boyce]

This document reports the work of the JVET ad hoc group on JEM algorithm description editing (AHG2) between the 5th JVET meeting at Geneva, Switzerland (12–20 January 2017) and the 6th meeting in Hobart, AU (31 March – 7 April 2017).

During the editing period, on top of the JVET-D1001 Algorithm Description of Joint Exploration Test Model 4, the editors worked on the following aspects to produce the final version of JVET-E1001 Algorithm Description of Joint Exploration Test Model 5.

- Integrating adoptions of the 5th JVET meeting that changed the encoding or decoding process,

  - JVET-E0052, decoder-side motion vector refinement based on bilateral template matching with integer pel step search only (8 positions around the start position with integer pel MV offset)

  - JVET-E0060, test 3 case, FRUC with additional candidates

  - JVET-E0076, MVD coding in unit of four luma sample precision

  - JVET-E0062, multiple Direct Modes for chroma intra coding: the total number of chroma intra modes is kept as 6 (unchanged), the list construction of chroma mode is modified with the first six as proposed

  - JVET-E0077, enhanced Cross-component Linear Model Intra-prediction, includes

    - Multiple-model Linear Model intra prediction

    - Multiple-filter Linear Model intra prediction

  - JVET-E0104, ALF parameters temporal prediction with temporal scalability

  - JVET-E0023, improved fast algorithm test case B: skip depth is set equal to 2 always for LDB and LDP, skip depth is set equal to 2 for highest temporal layer in RA, and is set equal to 3 for other temporal layers

- Overall text refinement and quality improvement (esp. FRUC description enhancement with details)

- Fix of text and software bug related to transformed coefficients zero-out for large transform (Ticket#44)

  - Note: This aspect is not in the output of the previous meeting – planned for incorporation into the next JEM description

Currently the document contains the algorithm description as well as encoding logic description for all new coding features in JEM5.0 beyond HEVC. Compared to HEVC, the following new coding features are included in JEM5 (where boldface indicates aspects that had technical changes incorporated as the result of the previous meeting):

- Block structure

  - Larger CTUs (software supports 256×256, CTC use 128×128)

  - Quadtree plus binary tree (QTBT) block structure

- Intra prediction

  - 65 intra prediction directions with improved intra mode coding

  - 4-tap interpolation filter for intra prediction

  - Boundary filter applied to other directions in addition to horizontal and vertical ones

  - Cross-component linear model (CCLM) prediction

  - Position dependent intra prediction combination (PDPC)

  - Adaptive reference sample smoothing

- Inter prediction

  - Sub-PU level motion vector prediction

  - Locally adaptive motion vector resolution (AMVR)

  - 1/16 pel motion vector storage accuracy

  - Overlapped block motion compensation (OBMC)

  - Local illumination compensation (LIC)

  - Affine motion prediction

  - Pattern matched motion vector derivation

  - Bi-directional optical flow (BIO)

  - Decoder-side motion vector refinement

- Transform

  - Larger transform block sizes (with zeroing of high frequencies)

  - Explicit multiple core transform

  - Mode dependent non-separable secondary transforms

  - Signal dependent transform (SDT, disabled by default)

- Loop fillter

  - Adaptive loop filter (ALF)

  - Content adaptive clipping

- Enhanced CABAC design

  - Context model selection for transform coefficient levels

  - Multi-hypothesis probability estimation

  - Initialization for context models

Among all of them, the decoder-side motion vector refinement (DMVR) method was added at the 5th JVET meeting. The AMVR method is modified by adding MVD coding in units of four luma samples, in addition to quarter luma sample and integer luma sample. The CCLM method is modified by adding multiple-model Linear Model intra prediction and multiple-filter Linear Model intra prediction.

The AHG recommended to:

- Continue to edit the algorithm description of the Joint Exploration Model document to ensure that all agreed elements of the JEM are described

- Continue to improve the editorial quality of the algorithm description of Joint Exploration Model document and address issues relating to mismatches between software and text.

- Identify and improve the algorithm description for critically important parts of JEM design for better understanding of complexity.

It was asked whether we need to maintain proper operation of the JEM with QTBT disabled. It was agreed that this remains a highly desirable capability, but has lower priority than other matters.

It was commented that removing this capability would make the code much cleaner and substantially reduce the size of the codebase.

Another participant suggested that pseudo-profiling (i.e., supporting on/off switches in syntax) rather than the use of a macro can make it easier to maintain proper functioning. However, an AHG co-chair said that this is difficult to do for aspects that affect basic structure, like QTBT.

It was commented that some bug fixes are found in the software with macros and that in such cases the macro should just be removed.

It was commented that we still need to integrate screen content coding tools and enable non-4:2:0 operation.

Decision (CTC): It was agreed that in the CTC when Class F is tested, the SCM (with SCC tools enabled) should be substituted for the HM anchor.

JVET-F0003 JVET AHG report: JEM software development (AHG3) [X. Li, K. Sühring]

This report summarized the activities of the AhG3 on JEM software development that had taken place between the 5th and 6th JVET meetings.

Software development was continued based on the HM-16.6-JEM-4.2 version. A branch was created in the software repository to implement the JEM-5 tools based on the decisions noted in section 12.4 in the notes of the 5th JVET meeting. All integrated tools were included in macros to highlight the changes in the software related to that specific tool.

HM-16.6-JEM-5.0 was released on Feb. 13th, 2017.

HM-16.6-JEM-5.0.1 was released on Feb. 17th, 2017. This version fixed a bug in rate calculation and was used as the anchor for EE tests.

Several branches were created for exploration experiments. Note that these branches are maintained by the proponents of exploration experiments.

The JEM software is developed using a Subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/

The implementation of JEM-5 tools has been performed on the branch

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/HM-16.6-JEM-4.2-dev

The released version of HM-16.6-JEM-5.0 can be found at

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-5.0

The released version of HM-16.6-JEM-5.0.1 can be found at

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-5.0.1

The branches of exploration experiments can be found at

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates

The performance of HM-16.6-JEM-5.0.1 over HM-16.6-JEM-4.0 and HM-16.14 under test conditions defined in JVET-B1010 is summarized in the following table.

It was reported by several companies that minor bit rate differences from the released JEM-5.0.1 anchor were observed. It turned out that level information was not properly set in those tests. It should be emphasized that level setting is a part of the test conditions and shall be followed during tests. The correct settings are provided in per sequence cfg files within the JEM package.

The JEM bug tracker is located at

https://hevc.hhi.fraunhofer.de/trac/jem

It uses the same accounts as the HM software bug tracker. For spam fighting reasons, account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

All issues related to the JEM should be entered into the bug tracker. All details that are necessary to reproduce the issue should be provided. Patches for solving issues and improving the software are appreciated.

The AHG recommended to:

- Continue software development on the HM-16.6 based version

- Encourage people to test JEM software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bitstreams/test cases that trigger bugs in JEM.

JVET-F0004 JVET AHG report: Test material (AHG4) [T. Suzuki, V. Baroncini, J. Chen, J. Boyce, A. Norkin]

The test sequences used for the CfE (JVET-E1002) are available on ftp://jvet@ftp.ient.rwth-aachen.de in the directory “/jvet-cfe” (please contact the JCT-VC chairs for login information).

HM/JEM anchors were generated and verified by a cross checker.

HM anchors:

ftp://jvet@ftp.ient.rwth-aachen.de/jvet-cfe/anchors-hm

JEM anchors:

ftp://jvet@ftp.ient.rwth-aachen.de/jvet-cfe/anchors-jem

There were questions on the encoder configuration for anchors, the HM/JEM version, floating point QP (it is part of HM16.15 but it is still not part of JEM5), etc.

The precise encoder configuration to generate anchors was requested to be clearly finalized during the Hobart meeting.

There was one input contribution on new test sequence (aerial photography). It was suggested that this should be evaluated and discussed whether this is to be used in test conditions (in CTC/CfE, etc).

Contributions to this meeting were noted as follows.

CfE anchor generation (as defined in JVET-E1002)

- JVET-F0060 "AHG4: Report on CfE anchor generation for SDR content with HM", K. Choi, E. Alshina (Samsung).

- JVET-F0061 "AHG4: Report on CfE anchor generation for SDR content with JEM", K. Choi, E. Alshina (Samsung).

- JVET-F0069 "AHG4: SDR anchor generation for Preliminary Joint Call for Evidence by Qualcomm", H.-C. Chuang, J. Chen, M. Karczewicz (Qualcomm).

New test sequence

- JVET-F0062 "New aerial photography sequences for video coding standard development", X. Zheng (DJI).

Proposed draft Call for Evidence

- JVET-F0040 "Revised Preliminary Joint Call for Evidence on Video Compression with Capability beyond HEVC", M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki (editors).

It was suggested that a visual assessment to check HM/JEM anchors for the CfE (JVET-E1002) should be conducted during the meeting. In addition, it was suggested that the new test sequences proposed by JVET-F0062 should be evaluated.

The AHG recommended to:

- Review all related contributions.

- Evaluate visual quality of HM/JEM anchors.

- Clarify and to finalize the precise test conditions for HM/JEM anchors

- Perform viewing of new test sequences

- Discuss further actions to select new test materials for JVET activity, especially to issue CfE.

Notes from the discussion:

The highest priority is to select bit rates for CfE and plan what to test in Turin. It was emphasized that the purpose of the CfE is not a competition between proposals but an evaluation of potential beyond HEVC. There is no commitment that everything submitted to the Turin meeting will be evaluated in formal subjective testing.

Comparison of the JEM with the HM is also desirable. It was commented that it would be desirable to have BD-MOS or something of that sort to enable estimation of bit rate savings rather than quality differences at given bit rates.

A BoG (coordinated by Teruhiko Suzuki) was asked to discuss CfE planned test sequences (incl. HDR) Fri 31 March p.m.

JVET-F0005 JVET AHG report: Fast encoding, encoding complexity investigation, and configuration settings (AHG5) [K. Choi (chair), Y.-J. Chang, H. Huang, X. Li, P. Philippe, Y. Yasugi (vice chairs)]

This document reports the work of the JVET ad hoc group on fast encoding, encoding complexity investigation, and configuration settings between the 5th Meeting at Geneva, CH and the 6th JVET meeting at Hobart, AU.

An AhG5 kick-off message was sent out at February 9, and encoder optimizations (i.e., JVET-E0023) had been implemented for the JEM 5.0 SW package. There was no email discussion during this period.

The following table shows the reduced encoding time with coding loss in JEM-5.0.1 when enabling JVET-E0023.

Test result on JEM-5.0.1 with JVET-E0023

The following contributions were identified as relevant to AHG5.

- JVET-F0044 AHG5: Performance evaluation of Adaptive Search Range on JEM-5.0.1 [Y. Yasugi, T. Ikai (Sharp)]

- JVET-F0063 AHG5: Enhanced fast algorithm of JVET-E0078 [P.-H. Lin, C.-L. Lin, Y.-J. Chang, C.-C. Lin, Y.-H. Ju (ITRI)]

The AHG recommended to:

- Review all the related contributions

- Continue work related to fast encoding, encoding complexity investigation, and configuration settings

JVET-F0006 JVET AHG report: 360° video conversion software development [Y. He, V. Zakharchenko]

The document summarized activities on 360-degree video content conversion software development between the 5th and the 6th JVET meetings.

The 360Lib Software package integrated all adoptions about projection format and metrics calculation:

- Metrics:

  - End-to-end S-PSNR-NN, end-to-end S-PSNR-I, end-to-end CPP-PSNR;

  - Cross format S-PSNR-NN, cross-format S-PSNR-I, cross-format CPP-PSNR;

  - Dynamic viewport PSNR (JVET-E0133);

- Projection formats and frame packing:

  - Vertical frame packing for Segmented Sphere Projection (JVET-E0025);

  - New frame packing for Icosahedron Projection (JVET-E0029);

  - New frame packing for Octahedron Projection (JVET-E0056);

- Software interface:

  - New 360Lib hook interface used for integrating 360Lib in HM and JEM;

  - VS-2015 and xcode project support;

360Lib-2.0.1 with support of HM-16.14 and JEM-5.0.1 was released on Feb. 17th, 2017;

360Lib-2.1 with support of HM-16.15 and JEM-5.0.1 was released on Feb. 23rd, 2017.

The 360Lib software is developed using a Subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

The released version of 360Lib_2.0.1 can be found at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-2.0.1/

The released version of 360Lib_2.1 can be found at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-2.1/

Patch available to integrate 360Lib conversion software on top of HM-16.15

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-2.1/patches/HM-16.15-360Lib-2.1.patch

Patch available to integrate 360Lib conversion software on top of HM-16.6-JEM-5.0.1

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-2.1/patches/HM-16.6-JEM-5.0.1-360Lib-2.1.patch

360Lib bug tracker

https://hevc.hhi.fraunhofer.de/trac/jem/newticket?component=360Lib

The following input contributions on software algorithm description are related to this AHG:

- JVET-F0041 AHG8: Platform independent floating point to integer conversion for 360Lib [Y. He, P. Hanhart, Y. Ye (InterDigital)]

- JVET-F0065 360Lib modifications for spherical rotation [Q. Xu, J. Boyce (Intel), Y. He, Y. Ye (InterDigital)

The AHG recommended to:

- Review input contributions to this meeting

- Continue software development of the 360Lib software package.

- Discuss and improve the quality and structure of the software.

- Discuss software architecture modification to reduce changes required in corresponding JEM software package

JVET-F0007 JVET AHG report: JEM coding of HDR/WCG material (AHG7) [A. Segall (chair), E. Francois, D. Rusanovskyy (vice chairs)]

The document summarized activities related to the AHG on JEM coding of HDR/WCG material between the 5th and the 6th JVET meetings.

Activities related to the mandates of this AHG include

- Sequences generated with the recommended conversion practices decided at the previous meeting were generated and released for study.  The sequences were made available at:

ftp://ftp.ient.rwth-aachen.de/testsequences/testset_hdr/

- Candidate software integrating the HDR/WCG specific tools required for the anchor defined in JVET-E1020 was released for study. The software included support for luma and chroma adaptive quantization parameters. The software was made available at:

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-5.0.1-AHG7-Anchor/

- Anchors for the Call for Evidence HDR test were created and cross-checked. The results were reported to the reflector on March 20, 2017.

- Anchors for the Common Test Conditions were created and cross-checked. (Note that this was side activity and not officially reported to JVET at the time of this report.)

During the generation of the CfE anchors, the following observations were reported to the AhG. It is suggested to further discuss these during the 6th meeting in combination with related input documents.

- It was commented that during anchor production and visual assessment of the results that the bit rates for the HDR3 (EBU_05_Hurdles) and HDR4 (EBU_06_Starting) test sequences may be swapped.

- The original capture frame rates for these same sequences is 100 fps. However earlier study on HDR in MPEG and JCT-VC, including verification testing in JCTVC-X1018, suggested that these sequences should be coded and viewed as if they were 50 fps.

In JVET discussion, the suggested fixes for these two issues were agreed.

Two input contributions were noted to be related:

- JVET-F0086AHG7: Information on CfE anchor generation for HDR content and comments on coding conditions [D. Rusanovskyy, A. K. Ramasubramonian (Qualcomm), E. François (Technicolor)]

- JVET-F0089 AHG7: Comments on metrics for extended colour volume content E. François, C. Chevance, F. Hiron (Technicolor), D. Rusanovskyy (Qualcomm)]

The AHG recommended to:

- Review all related contributions.

- Identify and discuss open issues related to the coding of HDR/WCG material.

JVET-F0008 JVET AHG report: 360° video coding tools and test conditions (AHG8) [J. Boyce, A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye]

This document summarized the activity of AHG8 on 360° video coding tools and test conditions between the 5th JVET meeting at Geneva, Switzerland (12–20 January 2017) and the 6th meeting in Hobart, Australia (31 March – 7 April 2017).

The initial version of this document provides a report of the AHG8 conference call held on 21 February 2017.

The v2 version of this document provides a summary of the input contributions to the 6th meeting, and recommends actions.

One conference call was held. The key motivation of the call was to work to define subjective testing methodology, in coordination with the MPEG OMAF AhG. The MPEG OMAF AhG has asked JVET to help define a subjective testing methodology which can be used to evaluate proposed projection/packing formats for possible inclusion in the OMAF specification.

One input contribution was reviewed during the AHG call.

- JVET-F0021 AHG8: Subjective testing of 360º video projection/ packing formats [J. Boyce, Z. Deng (Intel)]

Four methods of subjective viewing of 360º video are proposed to be used together for the comparison of the video quality associated with various projection/packing formats.

- Static viewport with forced discontinuous edges at picture boundary

- Static viewport with forced edges

- Pre-published dynamic viewports

- Non pre-published dynamic viewports

Detailed notes of the conference call were provided in the AHG report.

There were 21 relevant contributions to the current meeting of JVET related to 360° video coding, which were classified in the AHG report as listed below.

- 360Lib updates

  - JVET-F0041 AHG8: Platform independent floating point to integer conversion for 360Lib [Y. He, P. Hanhart, Y. Ye (InterDigital)]

  - JVET-F0065 360Lib modifications for spherical rotation [Q. Xu, J. Boyce (Intel), Y. He, Y. Ye (InterDigital)]

- Packing and Projection formats

  - JVET-F0025 AHG8: Adjusted cubemap projection for 360-degree video [M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

  - JVET-F0026 AHG8: Equatorial cylindrical projection for 360-degree video [G. Van der Auwera, M. Coban, M. Karczewicz (Qualcomm)]

  - JVET-F0035 AHG8: Additional test results of JVET-E0090 on nested polygonal chain packing of 360-degree ERP pictures [K. Kammachi-Sreedhar, M. M. Hannuksela (Nokia)]

  - JVET-F0036 AHG8: Rotated Sphere Projection for 360° Video [A. Abbas, D. Newman (GoPro)]

  - JVET-F0037 AHG8: Padding method for Segmented Sphere Projection [C. Zhang, Y. Lu, J. Li, Z. Wen (OwlReality)]

  - JVET-F0052 AHG8: EAP-based segmented sphere projection with padding [Y.-H. Lee, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

  - JVET-F0077 AHG8: Crosscheck of JVET-F0052 EAP-based segmented sphere projection with padding [Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital)] [late]

  - JVET-F0053 AHG8: An improvement on compact octahedron projection with padding [Y.-H. Lee, H.-C. Lin, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

- Subjective testing

  - JVET-F0021 AHG8: Subjective testing of 360º video projection/packing formats [J. Boyce, Z. Deng (Intel)] [NOTE: already discussed during AHG conference call.]

  - JVET-F0050 AhG8: Dynamic viewport based subjective evaluation of 360-degree video coding under CfE test conditions [P. Hanhart, Y. He, Y. Ye (InterDigital)] [late]

  - JVET-F0067 Comments on subjective testing procedure of 360° video [K. Kawamura, S. Naito (KDDI)]

  - JVET-F0083 AHG8: Subjective test pilot study of 360º video projection/packing formats [Z. Deng, L. Xu, J. Boyce (Intel)]

- Objective metrics

  - JVET-F0042 AHG8: On cross-format S-PSNR-NN [Y. He, X. Xiu, Y. Ye (InterDigital)]

  - JVET-F0066 AHG8: On inactive pixels in evaluation of 360° video projections [G. Van der Auwera, M. Coban (Qualcomm)]

- Performance

  - JVET-F0027 AhG8: JEM vs HM performance for 360° video content under CfE test condition [A. Singh, C. Pujara, S. N. Akula, A. Dsouza, Ramkumaar K. K., R. N. Gadde, V. Zakharchenko, E. Alshina, K. P. Choi (Samsung)]

  - JVET-F0039 AHG8: Codec behaviour on 360° videos [F. Galpin, F. Racape, E François (Technicolor)]

- Coding tools

  - JVET-F0038 AHG8: adaptive QP for 360° video coding [F. Racape, F. Galpin, G. Rath, E. Francois (Technicolor)]

  - JVET-F0049 AHG8: Adaptive QP for ERP 360º video [Hendry, M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

  - JVET-F0072 AHG8: Stretching ratio based adaptive quantization for 360° video [Y. Sun, L. Yu (Zhejiang University)]

The AHG recommended to:

- Review input contributions

- Prepare an output document defining a subjective testing methodology to compare coding of projection formats, proposed for use by the MPEG OMAF activity

- Refine common test conditions for 360° video, including objective metrics

- Update the 360° video section of the Call for Evidence

# Analysis, development and improvement of JEM (5)

Contributions in this category were discussed Sunday 2nd 11:15–12:30 (chaired by JRO).

JVET-F0023 AHG3: JEM coding results for adaptive search range [K. Suehring, K. Sharman, B. Bross]

This contribution provides results of RA simulations using the JCT-VC test conditions (JCTVC-Z1100) in combination with JEM 5.0.1, enabling adaptive search range and the minimum search window parameter associated with the adaptive search range. With an average encoder run-time decrease of 5%, the two sequences TrafficFlow and Rollercoaster of class A2 show significant loss of 0.84% and 1.01% luma BD rate, while the RD-performance most others is not changed.

See notes under JVET-F0044.

JVET-F0029 On memory bandwidth of JEM5.0 tools [E. Alshina (Samsung)]

Memory bandwidth information for several JEM tools compared to HEVC MC is summarized in this contribution. The power consumption is key complexity metric for mobile devices manufacturers. Since there is very clear correlation between power and memory bandwidth, the contributor suggested that this complexity metric be carefully considered during next generation video coding standard development. Currently the worst case memory usage in the JEM is reportedly 15 times that of HEVC.

Consideration of this would be relevant in context of standard development. No immediate action was necessary at this moment.

One expert raised awareness that such a study should also include the amount of cache memory available. A general (known) problem is the high dependency on the target implementation platform.

In the context of preparing for a CfP, it might be useful to require proponents to bring information about the memory bandwidth requirements of their proposal, as one of the aspects of complexity assessment.

It was agreed to establish an AHG on assessment of memory bandwidth consumption of coding tools (X. Li and E. Alshina, coordinating)

JVET-F0044 AHG5: Performance evaluation of Adaptive Search Range on JEM-5.0.1 [Y. Yasugi, T. Ikai (Sharp)]

This contribution reports the performance of Adaptive Search Range (ASR) on JEM-5.0.1. The authors tested the performance on JEM-5.0.1 using several combinations of minimum search range (MinSearchWindow) and maximum search range (SearchRange), where the minimum search range setting implementation was ported from HM 16-15. It is reported that the following configurations (MinSearchWindow_SearchRange) can be found as good operation points in terms of performance and encoding time balance: 128_256, 96_384 and 64_512.

From the results shown here, the combination 96_384 gives best tradeoff. However, the CTC of JEM and HM should be kept aligned, and HM results have not been investigated yet. HM results were provided in an update of the document, and indicated a minor improvement with a minor reduction of runtime, such that it seems to be beneficial for HM as well. This was reported to JCT-VC in JCTVC-AA0043.

As it would otherwise be necessary to generate new HM anchors for the CfE, it would be undesirable to implement this change at the current meeting. The change to the combination 96_384 adaptive search range should be implemented in CTC by the July meeting in both HM and JEM CTC.

JVET-F0095 AHG5: Cross check of JVET-F0044 on Performance evaluation of Adaptive Search Range on JEM-5.0.1 [P.-H. Lin, Y.-H. Ju, C.-C. Lin, Y.-J. Chang, C.-C. Lin (ITRI)] [late]

JVET-F0076 Request for 4:4:4 support in the JEM software [M. Meyer, A. M. Tourapis, D. Singer (Apple)] [late]

It has been observed that currently the JEM reference software is only able to handle 4:2:0 material. In particular, a variety of tools have been introduced over the original HM codebase, that unfortunately cannot properly handle 4:4:4 video material. This contribution requests that any issues impacting 4:4:4 encoding in the JEM are identified and resolved, and that any future development properly accounts for 4:4:4 material from this point onward.

4:4:4 is probably an important application area of future standardization, and we might even consider to include 4:4:4 material in a CfP.

In the current exploration for which JEM is designed, 4:4:4 is not in the central focus (and not in included in common test conditions). It would nevertheless be beneficial to have an idea about the level of 4:4:4 compliance of certain JEM tools. For example, some tools may have implemented the colour subsampling hard coded.

It was agreed to establish an AHG to study the difficulty of enabling 4:4:4 capability in the JEM (A. Tourapis, X. Li, coordinating)

# Test material and Call for Evidence (8)

## New test material proposals (2)

JVET-F0062 New aerial photography sequences for video coding standard development [X. Zheng, W. Li, Z. Cao, W. Su, C. Zhao, Y. Li, Z. Lorenz, H. Wu, Z. Du, D. A. Hoang (DJI)] [late]

Reviewed in BoG JVET-F0103.

JVET-F0094 Analysis of 4k Hybrid Log-Gamma test sequences [Shunsuke Iwamura, S. Nemoto, A. Ichigaya, M. Naccari (NHK)] [late]

Reviewed in BoG JVET-F0103.

## Call for Evidence (6)

JVET-F0040 Proposed Revision of Preliminary Joint Call for Evidence on Video Compression with Capability beyond HEVC [M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki]

Reviewed in plenary, and recommended as the basis for the final CfE.

JVET-F0060 AHG4: Report on CfE anchor generation for SDR content with HM [K. Choi, E. Alshina (Samsung)]

Reviewed in BoG JVET-F0103.

JVET-F0061 AHG4: Report on CfE anchor generation for SDR content with JEM [K. Choi, E. Alshina (Samsung)]

Reviewed in BoG JVET-F0103.

JVET-F0069 AHG4: SDR anchor generation for Preliminary Joint Call for Evidence by Qualcomm [H.-C. Chuang, J. Chen, M. Karczewicz] [late]

Reviewed in BoG JVET-F0103.

JVET-F0087 AHG4: Cross-check of SDR anchor generation (JVET-0069) [K. Choi, E. Alshina (Samsung)] [late]

Reviewed in BoG JVET-F0103.

(Also see contributions related to 360° video in section 7.3 and BoG JVET-F0106.)

JVET-F0086 AHG7: Information on CfE anchor generation for HDR content and comments on coding conditions [D. Rusanovskyy (Qualcomm), E. François (Technicolor)]

Reviewed in BoG JVET-F0107.

# Exploration experiments (29)

## General (1)

Contributions in this category were discussed Saturday April 1 0900–11:00 (chaired by JRO).

JVET-F0010 Exploration Experiments on Coding Tools Report [E. Alshina, L. Zhang]

Three experiments on coding tools were agreed to be carried out between the JVET-E and JVET-D meetings to get a better understanding of technologies considered for inclusion in the next version of the JEM, and analyse and verify their performance, complexity and interaction with existing JEM tools. This report summarizes the status of each experiment.

Summary of Exploration Experiments.

| # | Tests and sub-tests | Document | Y-BD-rate (Enc/DecTime) | Cross-check |
| --- | --- | --- | --- | --- |
| 1 | EE1: Intra prediction | JVET-F0024 (Arris) |  | JVET-F0045 (Sharp) JVET-F0047 (ETRI) JVET-F0056 (Qualcomm) |
|  | - Unequal Weight Planar (UWP) |  | AI: −0.1% (ET 0.99, DT 1.00) |  |
|  | - UWP + Constrained PDPC4 (Dec) |  | AI: 0.0% (ET 0.91, DT 1.00) |  |
|  | - UWP + Constrained PDPC4 (Enc) |  | AI: 0.2% (ET 0.91, DT 1.00) |  |
|  | - UWP + Constrained PDPC3 (Dec) |  | AI: 0.0 % (ET 0.91, DT 1.00) |  |
|  | - UWP + Constrained PDPC3 (Enc) |  | AI: 0.2% (ET 0.91, DT 1.00) |  |
|  | - UWP + Constrained ARSS (Dec) |  | AI: 0.0% (ET 0.85, DT 1.00) |  |
|  | - UWP + Constrained ARSS (Enc) |  | AI: 0.2% (ET 0.85, DT 1.00) |  |
| 2 | EE2: Non-linear in-loop filters - Reduced LUT size | JVET-F0034 (Ericsson) | AI: −0.4% (ET 1.06, DT 1.03) RA: −0.5% (ET 1.02, DT 0.99) LD: −0.5% (ET 1.02, DT 1.00) LDP: −0.6% (ET 1.04, DT 1.01) | JVET-F0048 (ETRI) JVET-F0057 (Qualcomm) JVET-F0073 (Intel) |
| 3 | EE3: Decoder Side Motion Vector Derivation |  |  |  |
|  | - Enhanced Template Matching in FRUC (with aspect 1 and 2 ) - Enhanced Template Matching in FRUC (with aspect 2 only) | JVET-F0032 (Hi-Silicon) | RA: −0.4% (ET 0.99, DT 1.02) LD: −0.1% (ET 1.00, DT 1.02) RA: −0.3% (ET 0.99, DT 1.01) LD: −0.1% (ET 1.00, DT 1.01) | JVET-F0048 (ETRI) JVET-F0071 (Hulu) |
|  | - BIO w/o block extension | JVET-F0028 (Samsung) | RA: 0.1% (ET 0.95, DT 0.88) LD: 0.1% (ET 0.98, DT 0.95) | JVET-F0059 (Qualcomm) |

## EE1: Intra prediction (13)

Contributions in this category were discussed Saturday 1200–1300 and 1430–1545 (chaired by JRO).

### Primary (5)

JVET-F0024 EE1: Unequal Weight Planar Prediction, Constrained PDPC, and Constrained ARSS [K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris)]

This contribution reports the results of EE1 for Test 1 (UW-Planar), Test 2 (UW-Planar with 4 angular modes with PDPC), Test 3 (UW-Planar with 4 angular modes with PDPC without syntax modification), Test 4 (UW-Planar with 3 angular modes with PDPC), Test 5 (UW-Planar with 3 angular modes with PDPC without syntax modification), Test 6 (UW-Planar with constrained ARSS), and Test 7 (UW-Planar with constrained ARSS without syntax modification). Test 1 shows the BD rates of −0.15%, −0.08%, and −0.01% with the encoding time of 99%, 100%, and 96% for AI, RA and LB, respectively, and no change in decoding time. Test 6 gives the fastest encoding times with the BD rates of −0.04%, −0.04%, and −0.01% and the encoding time of 85%, 98%, and 99% for AI, RA, and LB, respectively, and little change (101%, 100% and 100% for AI, RA and LB) in the decoding time.

The original contribution proposes replacement of planar mode in JEM by unequal weight planar prediction (UW-Planar). Compared to HEVC Planar prediction 2 multiplications per sample are needed. 2 multiplications per sample in Bottom Row and Right Column calculation also added. Unequal weight parameters are MaxblockSize*2+1 10 bits unsigned constants.

Additionally restriction on Position Dependent Intra Prediction combination mode (constrained PDPC) and Adaptive reference sample smoothing (constrained ARSS) are studied. Specifically, UW-Planar employs bottom-right position adjustment and unequal weight assignment for final predictor calculation.

Constrained PDPC allows PDPC mode only for intra CU that employs one of the following 4 angular modes: modes 2, 18, 34, and 50 (test 2 and 3) of one of the following 3 angular modes: 2, 18 and 50 (Test 4 and 5). Constrained PDPC was tested with syntax change (such that PDPC flag is only sent in the case of the mentioned four modes) and as encoder only modification.

Constrained PDPC reduces the size of PDPC parameter storage by factor ~9 ([5][35][6]  [5][4][6]).

Constrained Adaptive reference sample smoothing allows ARSS under certain condition:

diff > m_aucIntraFilter[toChannelType(compID)][sizeIndex]

− ( (compID==COMPONENT_Y && enableRSAF && sizeIndex==1) ? 2 : 0)

Constrained ARSS was tested with syntax change (such that an ARSS flag is hidden in parity of coefficients magnitudes sum) and as encoder only modification.

Questions recommended to be answered during EE tests.

[Q]: Investigate options of encoder-only restriction of PDPC?

[A]: For both variant of constrained PDPC (3 or 4 modes use PDPC) syntax modification shows 0.2% better performance compared to encoder only modification.

[Q]: Check the tradeoff with different numbers of restricted modes.

[A]: Tests 2 & 3 examine PDPC constrained for 4 modes, in tests 4 & 5 PDPC was allowed for 3 angular modes.

Test 2 (UW-Planar+CPDPC4) shows a coding gain of 0.01% with encoder time of 91% as compared to the anchor.

Test 4 (UW-Planar+CPDPC3) shows a coding loss of 0.01% with encoder time of 91% as compared to the encoder.

In addition, without UW planar, it is also observed that if PDPC is restricted for 4 modes, the average performance drops by 0.38% (with encoder time 92%) for AI as compared to the anchor

[Q] Also check tradeoff with ARSS, and different numbers of restricted modes

[A] Tests 6 & 7 examine constrained ARSS.

If ARSS is restricted, the average performance drop in AI test is 0.11% (86% Enc.time)

Summary from EE report: Use of unequal weight Planar, instead of current Planar, improves JEM performance by 0.15% (in AI test), with little increase in computations and memory. Constrained PDPC and ARSS with syntax modification is 0.2% better than encoder only change (w/o syntax). Combination of unequal weight planar constraining PDPD for only 4 angular modes with syntax modification leads to 9% Encoder run time reduction and 0.01% BD-rate gain in AI test as compared to the JEM5.0. Combination of unequal weight Planar and constraining ARSS with syntax modification leads to 14% Encoder run time reduction and 0.04% BD-rate gain in AI test as compared to the anchor.

- Test 1: UWP standalone, provides 0.1% gain

- Test 2/4: UWP plus normative change of PDPC (4/3 modes), roughly no impact on compression, encoder reduction to 90%

- Test 3/5: UWP plus encoder-only change of PDPC (4/3 modes), 0.2% loss, encoder reduction to 90%

- Test 6: UWP plus normative change of ARSS, roughly no impact on compression, encoder reduction to 85%

- Test 7: UWP plus encoder-only change of ARSS, 0.2% loss, encoder reduction to 85%

UWP standalone gives 0.1% gain. If combined with the constrained PDPC, no gain or loss is observed, but encoder runtime significantly decreased. When only constraining PDPC, bit rate increases more significantly (around 0.6%).

Though the constrained PDPC reduces the amount of memory, UWP as additional tool again introduces additional complexity, as it also requires a table of 257x10 bits to implement a division.

Related contributions with new ideas to simplify ARSS and PDPC without introducing a new tool –those were reviewed first before further discussion.

UWP as replacement of current planar mode might be attractive, as it provides 0.1% gain but does not increase the encoder/decoder runtime (not currently considering the problem of the additional memory).

The other aspects of reducing complexity of PDPC and ARSS require better understanding about the interaction of the different tools, and there are more contributions on these aspects.

Other documents in EE related category (in particular F0054, F0033, etc.) were reviewed but did not give clear evidence whether UWP would still have an advantage when other modifications would be made to existing tools. Further investigation in ongoing EE1, with extension of unequal weight to mode 66, as proposed in JVET-F0104.

JVET-F0045 EE1: Cross-check of JVET-F0024 on Unequal Weight Planar Prediction, Constrained PDPC, and Constrained ARSS [T. Ikai (Sharp)]

JVET-F0047 EE1: Cross-check of test1 on unequal weight planar [J. Lee, H. Lee, J. Kang (ETRI)] [late]

JVET-F0056 EE1: Cross-check of Test 6 with UW planar and restricted ARSS [V. Seregin (Qualcomm)] [late]

JVET-F0070 EE1: Cross-check of Test 4 (UW-Planar with PDPC restricted for different number of modes) [S.-H. Kim, H. Jang, J. Lim (LGE)] [late]

### Related (8)

JVET-F0033 EE1-Related: UWP with Constrained PDPC and Constrained ARSS [H. Jang, J. Lim, J. Nam, S.-H. Kim (LGE)]

This contribution reported a variation of a different number of constrained PDPC on top of UWP with constrained ARSS which was discussed in EE1. 3 variations for constrained PDPC were studied. From the experimental results, the proposed experiment achieve some “trade-off point” for the constrained PDPC based on UWP with constrained ARSS.

Three tests are performed:

- “Test 8”: Combining tests 2&6 of EE1

- “Test 9”: Combining tests 4&6 of EE1

- “Test 10”; On top of test 6, disable only planar mode from PDPC (not encoder only, syntax/semantics change required)

Tests 8 and 9 reduce encoding time by little more than 20%, increase bit rate by approx. 0.15.

Test 10 reduces encoding time by 16%, and saves 0.1% bit rate.

It is commented that the test 10 configuration is meaningful, as PDPC and UWP might not be combined anyway.

From results reported in JVET-E0068, the benefit by completely removing ARSS might even be larger than the Test8/9 configurations.

Further investigate in EE: Test 10 definitely, Test 8/9 if resources allow. A test case completely removing ARSS might be more interesting.

JVET-F0080 Cross-check of JVET-F0033: UWP with Constrained PDPC and Constrained ARSS [K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris)] [late]

JVET-F0054 Non-EE1: Alternative setting for PDPC mode [M. Karczewicz, N. Hu, X. Zhao, V. Seregin (Qualcomm)]

In EE1, a new planar mode is tested along with PDPC and ARSS mode constraints for encoder complexity reduction. This contribution proposes an alternative setting for encoder speed-up based on PDPC mode and EE1 planar mode, and also the interaction between PDPC mode and new planar mode is studied. In the first test, PDPC mode and EE1 planar mode are both used while ARSS is disabled, and simulation results reportedly show −0.2% luma BD-rate reduction with 79% encoding and 100% decoding time on average for AI configuration. In the second test, EE1 planar mode is disabled compared to the first test, and simulation results reportedly provide −0.1% luma BD-rate reduction with 79% encoding and 100% decoding time on average for AI configuration.

In the first test, Test1 of EE (UWP) is used, PDPC mode is applied with following changes:

- PDPC can be used for NSST

- PDPC mode is not applied and not signalled for small block sizes: 4x4, 4x8 and 8x4

- PDPC flag is signalled only when a block has at least 2 non-zero coefficients

- PDPC mode is not applied to planar mode

In the second test, the conventional planar mode is retained in combination with PDPC.

Both tests show similar performance, which seems to indicate that with the given modifications of PDPC (which indeed make PDPC more complex by releasing constraints), UWP does not provide much gain any more.

Since disabling ARSS would save around 30% encoding time, the modifications in PDPC obviously increase the runtime compared to the PDPC of JEM5. E.g. the combination of PDPC and NSST had been proposed previously, but not adopted due to the encoding time increase (see JVET-C0042). This was however not combined with disabling PDPC for small blocks by that time.

The position of the PDPC flag also needs to be changed.

Further study was planned to be conducted in an EE, in particular also to investigate the benefit of the different elements of PDPC modification in terms of compression / complexity.

JVET-F0078 Cross-check of JVET-F0054: Non-EE1: Alternative setting for PDPC mode [F. Le Léannec, T. Poirier, E. François (Technicolor)] [late]

JVET-F0055 Non-EE1: Explicit flag signalling for ARSS [V. Seregin, X. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes to indicate ARSS mode with a flag that is explicitly signalled instead of hiding it in the transform coefficients. Simulation results reportedly show 0.0% luma BD-rate change with 93% encoder running time on average for AI configuration. A combination with an ARSS explicit flag and EE1 Test1 and Test6 is also tested in this contribution.

The ARSS flag is signalled explicitly, and only signalled if the block has at least 3 coefficients

Following results are reported:

- Using the method standalone, reducing enc RT by 7%, no loss

- Using in combination with UWP (EE1 test 1), reducing enc RT by 7%, 0.1% BR reduction

- Using in combination with UWP and ARSS restrictions of EE1 test 6, enc. RT −20%, no loss.

Combination of the method with ARSS restrictions from EE1 test 6 seems to be an interesting tradeoff point. Further study in an EE was suggested.

Side activity (organized by E. Alshina) was requested to further discuss test cases in a next cycle of EE, including elements of F0024, F0033, F0054, F0055.

JVET-F0081 Cross-check of JVET-F0055: Explicit flag signalling for ARSS [K. Panusopone, Y. Yu, L. Wang (Arris)] [late]

JVET-F0104 Weighted angular prediction [K. Panusopone, Y. Yu, L. Wang (Arris)] [late]

This contribution proposes changes to intra prediction process in JEM-5.0.1, with modifications to angular mode 66 (weight angular prediction). Specifically, weighted angular prediction employs weighted average of top and left neighbours to compute final predictor for angular mode 66. Simulation results reportedly show that weighted angular prediction mode only and weighted angular prediction without PDPC have luma BD-Rate of approximately −0.21% and 0.11% compared with JEM-5.0.1 anchor, respectively, for AI configuration (cases of PDPC on and PDPC off). It is proposed to study weighted angular prediction as EE activities.

(Presentation deck not available at the time of presentation.)

This was implemented on top of JVET-F0024, i.e. unequal weighting is applied to both planar mode and mode 66. Several experts supported to investigate this in EE1. Only this combination should be tested in the EE.

The EE should also test cases with PDPC off/on, and ARSS off/on.

JVET-F0105 Cross-check of Weighted angular prediction (JVET-F0104 Test 1 & Test 2) [H. M. Jang, S.-H. Kim, J. Lim LGE)] [late]

## EE2: Nonlinear in-loop filters (8)

Contributions in this category were discussed Saturday 1000–1030 (chaired by JRO).

### Primary (4)

JVET-F0034 EE2-JVET-E0032 Bilateral filter Test 1, Test2 [J. Ström, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. Sjöberg (Ericsson)]

This document describes the result of the EE2 activity on reducing the size of the look-up table (LUT) used to implement the bilateral filter from JVET-E0032. This work is divided into two parts; test 1 and test 2. For test 1, the contribution describes techniques to reduce the LUT while retaining bit exact behaviour of the algorithm. The BD rate differences of test 1 against JEM 5.0.1 are reported to be −0.4% / −0.5% / −0.5% / −0.6% for AI/RA/LD/LDP. A LUT size of 56148 bytes is reported in this case. For test 2, the bilateral filter coefficients are stored with 5-bit precision resulting in a stated LUT size of 2202 bytes. The BD rate figures are reported to be −0.4% / −0.5% / −0.5% / −0.6% for test 2. Reported encoding complexity for test 2 is 106% / 102% / 102% / 104% and reported decoding complexity is 103% / 99% / 100% / 101%.

The bilateral filter operation is suggested to be performed on decoded sample values directly after the inverse transform. The proposed bilateral filter is a five-tap filter in the shape of a plus sign.

,

The strength of the filter is based on coded mode (intra/inter), TU size and QP. The size of the table is 56148 bytes (16 bits precision of filter coefficients). The filter is automatically disabled when there are no non-zero transforms coefficients. LUT operation implementation is used to reduce the complexity of the described operations. No additional parameters are determined during encoding and no new syntax elements are proposed. LUT operation makes complexity manageable.

Questions recommended to be answered during EE tests.

[Q]: What the size of the lookup table would be?

[A]: Test 1 examines the performance of the original proposal (LUT size 56148 bytes). Test 2 examines performance of the algorithm for reduced size of LUT 2202 bytes (5 bits precision per coefficient).

|  | LUT size = 56148 bytes |  |  |  |  | LUT size = 2202 bytes |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| AI | −0.4 | 0.1 | 0.1 | 106% | 104% | −0.4 | 0.2 | 0.1 | 106% | 103% |
| RA | −0.5 | 0.1 | 0.1 | 103% | 98% | −0.5 | −0.2 | −0.2 | 102% | 99% |
| LD | −0.5 | 0.4 | 0.2 | 103% | 100% | −0.5 | 0.3 | 0.2 | 102% | 100% |
| LDP | −0.6 | 0.1 | 0.3 | 104% | 101% | −0.6 | 0.4 | 0.4 | 104% | 101% |

[Q]: It is recommended to further reduce the size of LUT

[A]: Test 2 was designed for those purposes.

Summary from EE report: 0.4%, 0.5%, 0.5% and 0.6% gain is observed for AI, RA, LD and LD-P respectively from bi-lateral filtering after inverse transformation with LUT size 2202 bytes. Encoder run-time increment ranges from 2% to 6% and decoder run time increment ranges from 0% to 3%.

The LUT table size has been significantly reduced, without significant loss in compression as compared to previous reported results. During initial review, it was verbally reported that another even more simplified version exists which avoids divisions but increases the table size by another 1000 bytes. A new input document was to be registered with software attached, Qualcomm planned to perform a crosscheck. Bitstreams were to be made available upon request to interested parties.

Generally, it was expressed by several experts that the method provides some interesting gain without significantly increasing encoder runtime. It was, however, pointed out that most of the gain is obtained in intra mode, where this would be yet another step in the prediction pipeline between neighbouring blocks.

It was also proposed by one expert that the interdependency with block boundary filtering might be an aspect to be further investigated.

JVET-F0048 EE2: Cross-check of bilateral filter (JVET-F0034 test 2) [S.-C. Lim, J. Kang (ETRI)] [late]

JVET-F0057 EE2: Cross-check of bilateral filter (JVET-F0034 Test1, Test2) [L. Zhang (Qualcomm)] [late]

JVET-F0073 EE2: Cross-check of JVET-F0034 Bilateral filter Test 1, Test 2 [Z. Deng (Intel)] [late]

### Related (4)

JVET-F0096 EE2-JVET related: Division-free bilateral filter [J. Ström, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. Sjöberg (Ericsson)] [late]

During the Hobart meeting, it was requested to have a division-free implementation of the bilateral filter from JVET-F0034 test 2 demonstrated. It was also concluded that a smaller division-lookup-table (division-LUT) of about 1 kB was preferred. This contribution was a response to these requests. To keep the size of the division-LUT small, it is important to reduce the size of both the numerator and the denominator of the division. To reduce the numerator, the filter equation can be recast in a mathematically equivalent manner to work with differences instead of absolute values. To reduce the denominator, bilateral filtering is turned off for inter blocks of size 16×16 and larger, which avoids a large center weight coefficient. With these changes, this contribution asserts that it is possible to make a division-free implementation where the division-LUT has a size of 576 bytes. The result by using the division-LUT is reportedly identical to the result from using divisions. The reported BD rate numbers are −0.4% / −0.5% / −0.5% / −0.5% for AI / RA / LD / LDP, respectively.

Some concern was raised about complexity in particular for the AI case a) in terms of encoder runtime (though less than other tools such as PDPC); b) in terms of decoder side where the tool is in the innermost prediction loop and may require switching at each sample position. However, the expert who raised this concern did not strongly object against adoption to the experimental JEM framework, though his opinion was that this would not be a good tradeoff yet when put into a final standard.

Other experts (in particular, cross-checkers) expressed the opinion that with the new modifications, the tool provides a reasonable tradeoff between complexity and compression, and its inclusion in JEM was supported.

Decision: Adopt JVET-F0096.

JVET-F0099 Cross-check of JVET-F0096 on EE2-JVET related: Division-free bilateral filter [L. Zhang (Qualcomm)] [late]

JVET-F0100 EE2 related: Cross-check of JVET-F0096 Division-free bilateral filter [Z. Deng (Intel)] [late]

JVET-F0101 EE2 related: Cross-check of division-free bilateral filter (JVET-F0096) [S.-C. Lim, J. Kang (ETRI)] [late]

## EE3: Decoder-side motion vector derivation (7)

Contributions in this category were discussed Saturday 1130–1200 (chaired by JRO).

### Primary (4)

JVET-F0028 EE3: BIO w/o block extension [A. Alshin, E. Alshina (Samsung)]

In EE3 Bi-directional optical flow algorithm was modified in order not to access samples on reference frames other than what regular motion compensation uses. This contribution reports test results of this modification. The reported average performance change is 0.1% in RA and 0.1% in LD-B. This simplification reportedly leads to the 5% encoder and 12% decoder run-time reduction in RA test. For a corner case (44 bi-predicted sub-blocks), the proposed modification reduces the number of multiplications by a factor of two and memory access by factor of three.

In JEM5.0, BIO performs prediction and calculates gradients for the extended block WH  (W+4)  (H+4). Block extension has been removed. After this modification, the memory bandwidth of BIO is equal to that of regular bi-predictive motion compensation. Computational complexity also has been reduced which results in both encoder and decoder run time reduction.

Questions recommended to be answered during EE tests.

[Q]: Test results with weights not depending on PU size are requested,

[A]: Proposal was tested with all weights equal to 1. Original proposal has no performance drop in RA and 0.1% performance drop for LD. With all weight equal to 1 drop is 0.1% in both R and LD test.

Summary from EE report: With 0.1%performance drop in RA and LD encoder run time can be reduced by 5% and 2%, respectively, decoder run time can be reduced by 12% (RA) and 5% (LD) if BIO doesn’t use block extension. Memory for modified BIO is equivalent to the regular bi-directional MC.

During the discussion, some concern was expressed that the method of padding that is replacing the block extension would disallow sharing gradient computation between adjacent blocks and subblocks with identical motion vectors. However, the worst case memory bandwidth which would occur when all 4x4 blocks have different motion vectors. Also the results clearly indicate an encoder and decoder runtime reduction (confirmed by cross-check).

The description of the method is also consistent and does not cause any mismatch between encoder and decoder, as the padding and subsequent gradient calculation are executed once the block size and the motion vector for the block are known and BIO is invoked.

Decision: Adopt F0028.

JVET-F0059 EE3: Crosscheck of JVET-F0028 BIO w/o block extension [Y.-W. Chen, X. Li (Qualcomm)] [late]

JVET-F0032 EE3: Enhanced FRUC Template Matching Mode [Y. Lin, X. Chen, J. An, J. Zheng (HiSilicon)]

This contribution reports test results of JVET-E0035. The following two aspects were tested: 1) bi-directional template matching; 2) selection between bi-prediction and uni-prediction. EE Test1 is designed to test combination of the two aspects while EE Test2 is designed to test aspect 2 only. It is reported that EE Test1 brings 0.36% and 0.14% bit rate savings for RA and LDB configurations, respectively. EE Test2 brings 0.32% and 0.08% bit rate savings for RA and LDB configurations. Both EE tests have almost the same encoding and decoding time as JEM5.0.1.

A modification to the FRUC template matching mode is proposed. An identified issue is that motion information derived by the existing uni-directional template matching is used for bi-prediction in the FRUC template matching mode (allegedly unreasonably). This issue can be solved by the following proposed methods: 1) bi-directional template matching which jointly uses list 0 and list 1 reference pictures; 2) selection between uni-prediction and bi-prediction based on template matching distortion.

get template T0 matched to current template Tc from list0 reference pictures ( motion info MV0, cost0)

get template T1 matched to updated template T’C from list1 reference pictures (motion info MV1, cost1)

Bi-prediction based on MV0 and MV1

Update current template:

T’C = 2*TC – T0

cost0 < 0.5*cost1

Uni-prediction based on MV0

get template T0 (matched to current template) Tc from list0 reference pictures (motion info MV)

get template T1 matched to current template Tc from list1 reference pictures (motion info MV1)

Bi-prediction based on MV0 and MV1

Proposed FRUC template matching mode

Existing FRUC template matching mode

Y

N

Aspect 1

Aspect 2

One difference compared to current FRUC is the fact that the template based search cannot be implemented in parallel and must be performed sequentially for the two reference pictures.

Note: The Figure above relates to both Aspect 1&2 (Test 1). This requires sequential operation in determining the starting position of the second search.

Test 2 only uses Aspect 2, and determines templates T0 and T1 independently from List 0 and List 1 as in current FRUC, but then determines to use biprediction or uniprediction from one of the lists, depending on the cost criterion:

- Select the bi-, uni-L0-, or uni-L1-prediction according to the min (1.1xcost0, 1.1xcost1, costBi)

Note: Current FRUC always uses biprediction.

Questions recommended to be answered during EE tests.

[Q]: Check gain on top of JEM5.0 (is it additive to JVET-E meeting adoptions)?

[A]: original contribution tested vs JEM4.0 provided 0.38% (RA) / 0.12%(LDB) gain.

On top of JEM5.0, the gain is 0.36% (RA) and 0.14% (LDB) (aspects 1 and 2).

On top of JEM5.0, the gain is 0.32% (RA) and 0.08% (LDB) (only aspect 2).

Summary: Major gain come from selection between uni-prediction and bi-prediction based template matching: 0.3%(RA) / 0.1% (LD); no encoder run-time increment and 1–2% increment for decoder run-time.

Decision: Adopt F0032 Aspect 2. However, the multiplication by factor 1.1 should be implemented in fixed point precision.

JVET-F0071 EE3: Crosscheck for Enhanced FRUC Template Matching Mode (JVET-F0032) [W. Zhang (Hulu)] [late]

### Related (3)

JVET-F0022 EE3-related: A block-based design for Bi-directional optical flow (BIO) [H.-C. Chuang, J. Chen, X. Li, Y.-W. Chen, M. Karczewicz, W.-J. Chien (Qualcomm)]

This contribution presents a block-based design for the Bi-directional optical flow (BIO). On JEM-5.0.1, it reportedly shows that on average, 0.0% BD-rate saving with 9%/14% reduction in encoding/decoding time for Random Access configuration, and 0.0% BD-rate saving with 4%/10% reduction in encoding/decoding time for Low Delay configuration. The source code is made available along with the submitted document.

In this contribution, a block-based design of BIO is proposed. Instead of pixel level motion refinement in JEM5, the motion refinement is done based on 4x4 block. In the block-based BIO the weighted summation of gradients for the samples in a 4x4 block is used to derive BIO motion vector offsets for the block.

The other processes, such as calculation of gradients, BIO motion vectors and offsets, follow the same procedure as done in the current JEM.

After the 4x4 MV for each MV is obtained with block-based BIO, the MV buffer is updated and used for subsequent CU coding. OBMC is applied without BIO operation.

The method is still computing gradients with block extension (as in JEM5 BIO), with no reduction of worst case memory bandwidth. The contribution has several aspects

- disable BIO on top of OBMC (i.e., do not perform bio with the additional references from neighbour block MVs)

- computing motion vector refinement per 4x4 block (instead of per sample)

- use motion vector refinement for prediction of subsequent MV.

Further investigation in an EE was planned:

- investigate contributions of the different aspects to runtime and coding performance

- investigate in combination with the memory bandwidth reduction (as per adoption of F0028 to JEM6)

JVET-F0030 Cross-check for a block-based design for Bi-directional optical flow (JVET-F0022) [E. Alshina (Samsung)] [late]

JVET-F0082 Cross-check of JVET-F0022 (EE3-related: A block-based design for Bi-directional optical flow) [O. Nakagami (Sony)] [late]

# Non-EE Technology proposals (7)

Contributions in this category were discussed Sunday 2nd 0930–1100 (chaired by JRO).

JVET-F0031 Reduction redundant syntax signalling for transform skip [H. Jang, J. Lim, J. Nam, S.-H. Kim (LGE)]

This contribution proposes to remove redundant syntax signalling of transform skip when the primary transform is enabled. It has been observed that the flag for transform skip is still signalled when the primary transform is enabled. Therefore, in the proposed method, the flag signalling for the transform skip is skipped when the primary transform is enabled. From the experimental results, no loss was observed in both AI and RA configuration. And also 0.17% and 0.16% BD-rate saving in Class F was observed in AI and RA configuration, respectively.

This is a follow-up of contribution E0037. The previous contribution showed some loss, and the syntax position was changed. In the new proposal, the syntax position is unchanged, and transform skip flag is parsed depending on whether primary transform flag is 0.

The change is small, and was supported by several experts.

Decision: Adopt F0031.

JVET-F0079 Cross-check of JVET-F0031 (Redundant syntax signalling reduction for transform skip) [V. Lorcy (b-com), P. Philippe (Orange)] [late]

JVET-F0043 NSST memory reduction [S.-H. Kim, H. Jang, J. Lim (LGE)]

This contribution proposes a method to reduce the NSST memory requirement by replacing the third kernels of 8x8 NSST with the first kernels of 4x4 NSST. Specifically, 22 out of 35 third 8x8 kernels are replaced by 4x4 kernels based on intra prediction mode. From experimental results, 0.13% and 0.05% BD-rate increases were reported in AI and RA configuration, respectively, while reducing the NSST memory by 20%.

It seemed not of high importance at this stage to optimize memory usage, in particular if it implies coding losses. Further study was encouraged.

JVET-F0075 Crosscheck of JVET-F0043 NSST memory reduction [X. Zhao, V. Thirumalai, V. Seregin, A. Said, M. Karczewicz (Qualcomm)] [late]

JVET-F0051 Unified Adaptive Loop Filter for Luma and Chroma [J. An, J. Zheng (HiSilicon)]

This contribution proposes a unified adaptive loop filter for luma and chroma. In the proposed unified ALF mode, the chroma sample classification and on/off decision directly re-use the results of the collocated luma sample so that there is no complexity increase for the chroma classification. The chroma ALF coefficients also re-use those of luma but with a constant 5x5 diamond region of support to reduce the complexity of the chroma filtering. Besides the proposed unified ALF mode, the original separate ALF with a newly added CTU-level on/off control for chroma ALF is also supported as an alternative mode at the picture level. The proposed method reportedly achieves around 1% chroma BD-rate gain for AI configuration and 4.6% chroma BD-rate gain for RA, LB, LP configurations based on JEM5.0 without the decoding complexity increase for chroma ALF (as measured, although percentage gains measured only for chroma are typically not valid).

This is a follow-up of contribution E0079. In the new contribution, a classification and block-level on/off decision of luma is also used for chroma. Further, the filter coefficients are used for both luma and chroma in case of 5x5, in cases of 7x7 and 9x9 filter coefficients are reduced to a 5x5 matrix for chroma (by some rules of summing filter coefficients). In the encoder, both luma and chroma are used to determine the coefficients.

In some cases, small losses were observed in luma (for RA and AI), but there were also some cases with small gains (for LD).

In the presentation, chroma pictures were shown which indicate that compression artefacts are reduced, but also some loss in detail was observed. Reconstructed colour pictures were not demonstrated.

Some concern was expressed whether the properties of luma and chroma are matching in order to use common filters, and whether this might lead to artefacts (in one of the chroma pictures, a player’s number 19 was changed to 10).

It was also remarked that using the same filter for luma and chroma in the 5x5 case could be compromising because chroma is subsampled.

Furthermore, it was remarked that for chroma, the luma classification at the 2x2 level means that in chroma the switching of filter coefficients may necessary at sample level.

The BD gains in chroma indicate that chroma PSNR is improved, but the overall bit rate gains are low.

After cross-checks were finished, cross checkers reported that they observed some chroma artefacts, such as local colour losses or colour bleeding.

Due to the fact that the same filter is applied at least in the 5x5 case for luma and chroma, and the chroma is subsampled, effectively a stronger lowpass filter is applied to chroma than for the luma. In particular, it may happen that edges are smoothed. The joint design may be compromising both components. It may be a better idea for the same class to optimize luma and chroma filters separately (and use always 5x5 filters for chroma, as already done in the proposal). Further study was recommended.

JVET-F0058 Cross-check of JVET-F0051 Unified Adaptive Loop Filter for Luma and Chroma [L. Zhang (Qualcomm)] [late]

# Coding of 360° video projection formats (30)

## Conversion tools, 360lib (3)

Contributions in this category were discussed Tuesday 4th in BoG JVET-F0106 (chaired by Jill Boyce). Recommendations of the BoG were approved in JVET plenary Wed. 5th.

JVET-F0041 AHG8: Platform independent floating point to integer conversion for 360Lib [Y. He, P. Hanhart, Y. Ye (InterDigital)]

Reviewed in BoG JVET-F0106.

The 360Lib software supports projection format conversion among different projection formats and spherical metrics calculation. To keep high precision, floating point computation is applied in 360Lib-2.1 when computing geometry coordinates for both projection format conversion and quality metric calculation. While generating and cross checking the 360° CTC simulation results as defined in JVET-E1030, it was found that 360Lib-2.1 may generate slightly different results depending on the OS and also the compiler version of the simulation platform. This document proposes a fix to the platform-dependent problem by converting floating point precision to fixed point precision and then converting to integer precision, rather than converting directly from floating point to integer. It was reported that, with the proposed fix, the simulations on Windows and on Linux platforms, as well as using different gcc versions on Linux, all achieve the same results. It is therefore asserted that this fix resolves the platform-dependence problem in the 360Lib-2.1 software. Compared to the 360Lib-2.1 anchors, in terms of end-to-end spherical quality metrics, there is no BD rate loss for luma for all projection formats. For chroma, there is no BD loss for all projection formats except the OHP projection format, where a small chroma BD loss of 0.1% was observed.

Decision (SW): Adopt JVET-F0041 (confirmed in JVET plenary).

JVET-F0088 AHG8: Crosscheck for JVET-F0041 [Hendry (Samsung)] [late]

JVET-F0065 360Lib modifications for spherical rotation [Q. Xu, J. Boyce (Intel), Y. He, Y. Ye (InterDigital)]

Reviewed in BoG JVET-F0106.

Modifications to the 360Lib software were proposed to support spherical rotation aligned with the omnidirectional projection indication SEI message in JCTVC-Z1005. A software patch based on 360Lib- 2.1 was provided with the contribution.

A change in order of the rotation is included.

An inverse rotation function is needed to properly compute objective metrics.

Decision (SW): Adopt JVET-F0065 (confirmed in JVET plenary).

## Packing and Projection formats (15)

Contributions in this category were discussed Tuesday 4th 1430–1700 (chaired by JRO, unless otherwise noted).

JVET-F0025 AHG8: Adjusted cubemap projection for 360-degree video [M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

This contribution proposes an adjusted cubemap projection for coding of 360-degree video. The proposed projection targets uniform sphere sampling with limited texture deformation. It uses a function to perturb the cubemap lookup vector. The proposed projection can reportedly reuse existing cubemap hardware lookup for efficient rendering. The proposed scheme reportedly provides −11.3% RA coding impact (E2E WS-PSNR).

The rendering is performed in two steps: first mapping the adjusted cubemap into the conventional cubemap, and then performing conventional cubemap rendering.

No additional interpolation is performed, just re-computing the position.

According to several experts’ opinion, the method is not actually compatible with existing GPUs.

It was asked what is the end-to-end PSNR? This was not known, but should be comparable.

Decision (SW): Include in 360Lib software.

It was noted that this projection format was also requested to be included in OMAF; therefore OMAF should exercise an “evil viewport” experiment. The evil viewports of this format would be around the same vertices as in CMP. The effects might be different, since the density of samples is also varied close to the boundaries. Subjective testing was to be performed by the OMAF activity.

JVET-F0091 Cross-check of JVET-E0025 on Adjusted cubemap projection for 360-degree video [V. Zakharchenko, K. P. Choi (Samsung)] [late]

JVET-F0097 AHG8: Cross-check of JVET-F0025 Adjusted cubemap projection with tile coding for 360-degree video [Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital)] [late]

JVET-F0026 AHG8: Equatorial cylindrical projection for 360-degree video [G. Van der Auwera, M. Coban, M. Karczewicz (Qualcomm)]

The equatorial cylindrical projection (ECP) is proposed for efficient compression of 360-degree video:

- projecting the equatorial region of the sphere using the Lambert cylindrical equal-area projection,

- projecting the poles of the sphere onto square faces in the video frame to avoid inactive pixels.

The RA coding impact was −11.4% (e2e WS-PSNR) compared with the equirectangular projection in accordance with the JVET common test conditions for 360-degree video.

The functions of coordinate mapping to faces seem to be more complicated than in F0026.

It was noted by experts that rendering could be complex.

It was noted that this projection format was also requested to be included in OMAF; therefore OMAF should exercise an “evil viewport” experiment. The evil viewports of this format would be around the same vertices as in SSP. Subjective testing was to be performed by the OMAF activity.

No action was taken on this regarding the 360Lib software.

JVET-F0092 Cross-check of JVET-E0026 on equatorial cylindrical projection for 360-degree video [V. Zakharchenko, K. P. Choi (Samsung)] [late]

JVET-F0098 AHG8: Cross-check of JVET-F0026 equatorial cylindrical projection with tile coding for 360-degree video [Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital)] [late]

JVET-F0035 AHG8: Additional test results of JVET-E0090 on nested polygonal chain packing of 360-degree ERP pictures [K. Kammachi-Sreedhar, M. M. Hannuksela (Nokia)] [late]

This contribution reports test results of JVET-E0090 on a nested polygonal chain packing method using the JVET common test conditions and evaluation procedures for 360° video of JVET-E1030. The nested polygonal chain packing method presented in JVET-E0090 reportedly provides on average 8.2% bit rate reduction in Y-PSNR according to all JVET 360-degree end-to-end metrics (S-PSNR-NN, S-PSNR-I, CPP-PSNR, and WS-PSNR) compared to equirectangular panorama projection.

Compared to SSP, this has approximately 1% loss.

The contribution was for information; no specific action was requested.

It was later reported that this projection method is also one of the candidates in OMAF.

JVET-F0036 AHG8: Rotated Sphere Projection for 360° Video [A. Abbas, D. Newman (GoPro)]

This contribution provides a description of rotated sphere projection (RSP) for 360° video. RSP represents 360° video using only two faces that are arranged in two rows. The top face is directly cropped from the middle 270°x90° of ERP, and the bottom face is cropped from the middle 270°x90° of ERP after performing a spherical rotation by 180° along the Y-axis and 90° along the X-axis. Average coding gains of 10.5%, 4.5% and 5.2% were reported for WS-PSNR-E2E metrics.

It was claimed that the method requires 6% less pixels to be coded; however the cutoff towards uncoded pixels has a circular shape and cannot well be covered by the square blocks of HEVC.

Several experts expressed interest in this.

Decision (SW): Include in the 360Lib software.

It was noted that this projection format was also requested to be included in OMAF; therefore OMAF should exercise an “evil viewport” experiment. The evil viewports of this format need to be defined. Subjective testing was to be performed by the OMAF activity.

JVET-F0085 AHG8: Crosscheck of JVET-F0036 Rotated sphere projection for 360° video [Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital)]

JVET-F0037 AHG8: Padding method for Segmented Sphere Projection [C. Zhang, Y. Lu, J. Li, Z. Wen (Owl Reality)] [late]

Discussion chaired by J. Boyce

This contribution describes a padding method for the SSP format in order to mitigate the seam artefact. Experiments results are listed, comparing usage with and without padding, including subjective comparison and objective metrics. Objective gains are reduced with the method, but subjective improvements were demonstrated.

This would consider the padding region to count as “active pixels”, and downsample the original regions to fit in the same coded picture size. It was proposed to have a configurable padding width parameter. Experiments used 8 pixels padding width for most sequences, and 16 pixels for moving camera sequences (Chairlift, Skateboarding_in_lot). It was noted that having an adjustable padding width would violate the terms of the current CfE draft, which requires the same projection format to be used for all sequences and all bit rates.

Further study was encouraged of padding with SSP projection, but without changing the padding width per sequence.

It was agreed to include this in an EE with JVET-F0052 on SSP projection modifications and padding.

(Presentation deck requested to be uploaded.)

JVET-F0052 AHG8: EAP-based segmented sphere projection with padding [Y.-H. Lee, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

Discussion chaired by J. Boyce

This contribution proposes an EAP-based segmented sphere projection which uses equal-area projection (EAP) on the equatorial segment and applies padding on the north and south poles. The experimental results reportedly demonstrate that the proposed EAP-based segmented sphere projection achieves 11.33%, 11.21%, 11.28% and 11.35% BD-rate reduction in S-PSNR-NN, S-PSNR-I, CPP-PSNR, and WS-PSNR, respectively, as compared to ERP.

The padding width is 32 pixels. The face size is reduced to fit in same coded picture size. More coding gains than SSP were reported, with reduced subjective artefacts. It seemed difficult to separate the impact of the EAP center from the padding.

This has some similarity with JVET-F0026, which also had EAP in the center region.

It was agreed to start an EE on SSP projection modifications and padding, to compare SSP with this proposal.

Aspects to study in the EE:

- EAP vs ERP in the center section

- Padding widths for SSP-based projections. Including some informal subjective testing.

JVET-F0077 AHG8: Crosscheck of JVET-F0052 EAP-based segmented sphere projection with padding [Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital)] [late]

Noted.

JVET-F0053 AHG8: An improvement on compact octahedron projection with padding [Y.-H. Lee, H.-C. Lin, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

Discussion chaired by J. Boyce

This contribution proposes a modification on the compact octahedron projection layout (COHP) by padding additional pixel samples at the discontinuous edges. The padded samples reportedly improve the compression efficiency and significantly reduce the appeared artefact when rendering around discontinuous edges. As compared to the COHP without padding, the experimental results reportedly demonstrate that the BD bit rate reduction provided by the proposed padding scheme is 0.7%, 1.6%, and 1.5% in end-to-end WS-PSNR-Y, WS-PSNR-U, and WS-PSNR-V, respectively, while the virtual artefacts on the discontinuous edges are also reportedly reduced.

This adds 16 pixel wide padding to discontinuous edges. It doesn’t reduce face size, because even with the padding, the increased coded picture size fits into the previous limit of coded picture size. It is similar in concept to the padding used for CISP.

Decision (SW): Change the COHP1 design in the 360Lib software and update the CTC accordingly.

JVET-F0084 Cross-check of JVET-F0053 on An improvement on compact octahedron projection with padding [V. Zakharchenko (Samsung)]

Noted.

JVET-F0108 Padded ERP (PERP) projection format [J. Boyce, A. Tourapis, C. Fogg] [late]

Presented Thu 6th afternoon (chaired by JRO)

It was proposed to add a new projection format to the 360Lib software for a variation of the equirectangular projection (ERP) format that adds padding, referred to as padded ERP format (PERP). This would enable the 360Lib software to support a feature included in JCTVC-AA0044 for the omnidirectional projection indication SEI message that allows the value of the yaw_range to represent more than 360 degrees for ERP sequences.

It was commented that now cropping has to be done in rendering. Furthermore, to prevent artefacts, it is necessary to discard samples from both left and right hand sides. It was suggested that therefore, it might be better to symmetrically pad samples both on left and right hand side, because otherwise the 0 degree angle would not be in the center of the picture.

It was also mentioned that using SCC profile could be more efficient by using CPR (aka IBC) for coding overlapping regions.

It was proposed to add this to the 360Lib software.

It was planned to include this in EE4 (symmetric center-based as described above) and a related branch of 360Lib; visual testing is to be done in the EE context.

## Testing procedure and metrics (8)

### Subjective testing contributions (5)

Contributions in this area were discussed Saturday 1615–1930, chaired by JRO.

JVET-F0021 AHG8: Subjective testing of 360º video projection/packing formats [J. Boyce, Z. Deng (Intel)]

Four methods of subjective viewing of 360º video are proposed to be used together for the comparison of the video quality associated with various projection/packing formats.

- Static viewport with forced discontinuous edges at picture boundaries

- Static viewport with forced edges

- Pre-published dynamic viewports

- Non pre-published dynamic viewports

The first proposed method applies pre-rotation to force maximizing the discontinuous edges of all tested projection/packing formats to be placed in the same location in a static viewport, to enable more direct comparisons of subjective artefacts between the formats. It requires a separate encoding than would be used for the other proposed. The second proposed method similarly requires pre-rotation to force edges at the same location in a static viewport, but allows avoiding the picture boundary discontinuous edges. The third and fourth methods allow but do not require pre-rotation, and use dynamic viewports, with pre-published and non- pre-published position tracks.

F0083 is related; see further notes for that.

It was reported that static viewports may be quite different in results, depending on where edges are.

One suggestion made in the contribution was to use pre-rotation to enforce face boundaries or picture boundaries to lie within the viewport.

For the CfE, methods 3 and 4 would be used.

Methods 1 and 2 are requiring rotation prior to encoding, based on a definition of “worst case view position”; therefore it seemed unpractical to define by discussion during a meeting what the worst case is and get results already within the same meeting. Therefore, a delay by one meeting cycle would be unavoidable to make decisions. Furthermore, debates could occur on which position establishes the worst case.

Alternatively, it appears possible to define a rule as part of the procedure, such that anybody who proposes a new projection format should be forced at the same time to bring results that allow identifying whether there are problems with seams at face boundaries, e.g. by requesting

- that a rotation is performed prior to encoding, which brings the defined static viewports to a most critical area of the given projection format,

- where the most critical area is an area around a vertex (or picture boundary point) with the most discontinuities around that point

- for this case, encoded results must be provided with QP values t.b.d. (e.g. QP37 was used in contribution F0083)

It was also noted that this procedure is mainly meant as kind of “sanity check” whether there may be inherent problems in a projection format, producing visible seams between face boundaries when compression artefacts appear.

JVET-F0050 AhG8: Dynamic viewport based subjective evaluation of 360-degree video coding under CfE test conditions [P. Hanhart, Y. He, Y. Ye (InterDigital)] [late]

A preliminary Joint Call for Evidence (CfE) on Video Compression with Capability beyond HEVC was issued at the 5th JVET meeting (JVET-E1002). The preliminary CfE covers 360-degree video, among other test cases, and defines the test conditions, e.g., coding configuration, sequences, and bit rates, for the evaluations. For subjective evaluation, it is specified that dynamic viewports will be viewed on ordinary monitors and assessed using the expert viewing protocol (EVP) method (Rec. ITU-R BT.2095-0). This document reports results of a subjective evaluation comparing dynamic viewports as defined in JVET-E1030 rendered from 360-degree video sequences encoded with HM-16.15 and JEM-5.0.1 in the equirectangular projection (ERP) format under CfE test conditions. The EVP method was used to assess the compression efficiency of the HM and JEM. The results reportedly showed that the JEM achieves some visual quality improvements over the HM at low and middle bit rates. Finally, based on observations made during the experiment, this document discusses issues regarding the use of the EVP method.

Due to the fact that already the original has impairments (stitching artefacts), additional training is necessary to only judge coding artefacts

The results indicate that for most cases the rate definitions are appropriate.

Further action requested during the meeting was to:

- Investigate the coded results with Vittorio Baroncini

- Perform a further (informal) viewing session to finally decide the rate points for the CfE. These were requested to be selected in a way that for the lowest rate, HEVC starts becoming poor quality, and for the highest rate point HEVC is not yet transparent.

- In case when we come to the opinion that certain rate points need to be aligned, additional results are available with a fixed QP

JVET-F0067 Comments on subjective testing procedure of 360° video [K. Kawamura, S. Naito (KDDI)]

This contribution presents comments on subjective testing methodology of 360º video. In the current preliminary call for evidence, the subjective evaluation of 360º video utilizes the 2D rectilinear viewpoints, following the method for SDR content, as defined in Rec. ITU-R BT.2095-0. However, a field of view (FOV) of a viewport of 360º video is much wider than that of SDR content. In this contribution, the view distances of them are considered from the FOV context. Suggested changes of text for the call for evidence is also shown.

Due to the fact that HMDs have wider field of view than prescribed by the viewing distance of ITU-R BT.2095-0, and also pixels are visible in case of 4K 360° video, it makes sense to define a shorter viewing distance. However, it may not be desirable to make a too-precise definition, as the assessment of 360° video is novel and may require changes on short notice. Therefore, the CfE text was agreed to be modified as follows: “… following the method described above for SDR content with slightly shorter viewing distance that better corresponds to the field of view of HMDs.”

JVET-F0083 AHG8: Subjective test pilot study of 360º video projection/packing formats [Z. Deng, L. Xu, J. Boyce (Intel)] [late]

A mini subjective test pilot study was conducted for the two methods described in F0021.

- Static viewport with forced discontinuous edges at picture boundary

- Static viewport with forced vertexes

The first method applies pre-rotation to force discontinuous edges at the picture boundary to be placed in a static viewport. The second proposed method similarly requires pre-rotation to force a static viewport center to a vertex inside the picture.

By pre-rotating each sequence prior to projection/packing format mapping and encoding, discontinuous edges will be included in the static viewport. After lossy encoding, especially with high QP, the compression distortion would make the discontinuous edges visible in the decoded static viewport. In the reported mini subjective viewing test pilot study, these seam/edge artefacts were observed in the decoded viewport for both Method 1 and Method 2, and the artefacts were observed with different extent across different projection formats. To evaluate the visual quality associated with various projection/packing formats, it was suggested to be meaningful to include the proposed pre-rotation based methods into the common test conditions and evaluation procedures for 360º video coding and that the proposed methods could also be used for the quality assessment of those techniques that would possibly be developed to alleviate or eliminate those seam artefacts.

Method 1 defines position at picture boundary, method 2 within the picture.

Subjective testing was performed with three subjects, and a MOS scale was obtained. They detected visible artefacts for several projection formats, but the correlation with viewport PSNR are rather low.

During the discussion, it had also been suggested that testing various viewports and measuring PSNR might be an alternative, but results show that viewport PSNR does not reliably detect the seam problems.

Both methods seemed needed, since some projection formats might have more critical areas at boundary, others in the inner part.

From video demonstrations given during the presentation, obvious visibility of artefacts was confirmed. It seemed, however, unclear whether this could be avoided by some encoder optimization (e.g. decreasing QP at face boundaries).

See further notes under JVET-F0021.

JVET-F0027 AhG8: JEM vs HM performance for 360° video content under CfE test condition [A. Singh, C. Pujara, S. N. Akula, A. Dsouza, Ramkumaar K. K., R. N. Gadde, V. Zakharchenko, E. Alshina, K. P. Choi (Samsung)]

Discussed Sunday 16:15, chaired by JRO and GJS.

This document reports a comparison of an HM-based anchor and an additional JEM-based anchor in the 360° video category under test CfE conditions. The average Y-BD-rate gain that the JEM showed over the HM for 360° content was reportedly 24.2% in PSNR metric; and from 22.5 to 24.5% in VR-specific quality metrics. Chroma BD-rate gain was reportedly around 50%. Correlation of subjective visual quality and objective metrics was also discussed in this contribution.

It was generally found that the different metrics are providing similar results. This may be explainable by the fact that only ERP is tested here. Generally, the PSNR results can also give some hints about necessary alignments of rate points (as from the viewing session, see below). For example, the difference QP24/QP27 for Harbour can be expected to give similar visual quality.

Some discussion was also provided about metrics, but in the current exploration all metrics should be retained.

A first viewing session showing results of F0050 and F0027 was held Sunday 14–15. The following initial observations were made:

- Lowest rate point should be made lower for Kite Flite

- For Harbour, all rate points should be shifted down (R3->R4, R2->R3, etc.)

- For Chairlift, rate points are OK

- Skateboard in lot is too fast, difficult to see coding artefacts

Possible candidates for Skateboard would be Gaslamp and Trolley. Rate/QP points needed to be identified (and a BoG was planned for Monday).

The test chair also suggested to use approximately the same QP for the lowest rate of all sequences.

### Objective testing contributions (3)

Contributions in this area were discussed Sun 16:45–18:00, chaired by JRO and GJS.

JVET-F0042 AHG8: On cross-format S-PSNR-NN [Y. He, X. Xiu, Y. Ye (InterDigital)]

Cross-format spherical quality metrics were included into the 360° common test conditions as defined in JVET-E1030. For cross-format quality metrics, the original video and the reconstructed video to be evaluated have different projection formats and/or different resolutions. S-PSNR-NN is one of the cross-format metrics. In order to avoid the impact due to interpolation filters, S-PSNR-NN measures the distortion between samples at nearest neighbour integer positions in the projection plane, instead of interpolating at fractional sample positions. However, when S-PSNR-NN is measured across two projection planes with different formats and/or resolutions, the nearest neighbour rounding operation in each projection plane may cause the reference sample and the reconstructed sample to no longer correspond to the same position on the unit sphere. In such case, cross-format S-PSNR-NN may not provide a reliable quality measurement and coding performance comparison. This document proposes a modified S-PSNR-NN calculation method to try to solve this problem. It was asserted that simulation results validate that the BD bit rates reported by the modified cross-format S-PSNR-NN are consistent with other metrics, such as end to end S-PSNR-NN and WS-PSNR.

The method is modifying the set of points on the sphere that are compared, such that each maps to an integer sample position in the original. The number of points is around 655K. The set of points compared is the same for each projection format, and interpolation is performed to compare each point against the corresponding fractional sample position in the reconstruction.

It was pointed out that the method is not symmetric, as the results would be different when the original and reconstruction are interchanged (unless they are in the same projection format).

Decision (SW360): Replace the previous cross format S-PSNR-NN by the new method.

JVET-F0090 AHG8: Crosscheck for JVET-F0042 on cross-format S-PSNR-NN [C.-H. Shih, J.-L. Lin (MediaTek)] [late]

JVET-F0066 AHG8: On inactive pixels in evaluation of 360° video projections [G. Van der Auwera, M. Coban (Qualcomm)] [late]

The common test conditions for 360-degree video allow for coded frame sizes that are larger than the ERP anchor coding size, since only the number of active samples of the projection are counted. This coding size increase can be 6–10% depending on the projection, which significantly increases decoder complexity. A coding result reportedly indicates that there is a coding gain advantage associated with the increased coding size. The concern is that this may put compact projection formats at a disadvantage in the evaluation. Furthermore, the preliminary Call for Evidence document that includes 360-degree video would allow the active pixel count to increase up to 3% beyond that of the ERP anchor. It was suggested to use the ERP anchor coding size as the upper limit for the coding size for all projections under evaluation in the common test conditions and Call for Evidence for 360-degree video.

The main motivation of this contribution is to take into account the number of inactive (e.g. black) samples. Hypothetically, if some contribution is able to represent the 8K with low bit rate, this would be welcome. On the other hand, it should be prevented that some kind of downsampling or other preprocessing is hidden in the projection format, since this might make comparison at low bit rates useless, where coding at lower resolution with subsequent interpolation could improve the quality.

The definition of “active/inactive pixel” seemed not fully clear. Inactive pixels could be black, padded, etc.; if a decoder pads on the fly, would the padding be considered an “inactive” region? Would the low resolution in a scalable representation be inactive? It might be better to know number of coded pixels.

It was agreed that the restriction of +/−3% should be removed from the CfE and the following restrictions should be imposed:

- Report end-to-end performance (without compression) of projection format (since large deviations might unveil some kind of subsampling, pre/postprocessing etc.

- Report the coded resolution (number of coded pixels)

The CTC currently also has a similar restriction (vaguely), with an additional table of the data for different projection formats of 360Lib; some of them have a higher number of active+inactive samples, but do not exceed ERP in the number of coded samples.

It was agreed to distinguish two cases of CTC:

- use HEVC as is (the “OMAF case”), in this case the number of coded samples should not be larger than in the ERP HM anchors. This should also include a definition of “evil viewport” for each known projection format (included in 360Lib), and some definition how such “evil viewports” might be defined for unknown formats (see notes elsewhere on that)

- when adding new coding tools (“JEM case”), no restriction on number of active/inactive/coded samples, but report these number, make comparison against ERP JEM anchor, and additionally, if another projection format is used, comparison against a JEM anchor of the same projection format with not larger number of coded samples than ERP JEM anchor

Adding new coding tools to the HM only would not be desirable, since they would be expected to be added in the context of a new standard, such that their interaction with other coding tools is also relevant.

In PSNR calculation, “active samples” are relevant. Active samples are those that are used for rendering.

Further update of CTC document was conducted in a BoG (coordinated by Jill Boyce) Tue 16:00.

## Coding tools (4)

Contributions in this category were discussed Tuesday 4th 1030–1245 (chaired by JRO).

JVET-F0038 AHG8: adaptive QP for 360° video coding [F. Racape, F. Galpin, G. Rath, E. François (Technicolor)]

This contribution presents the results of encoding ERP sequences with a vertically adapted QP. On HM-16.15, it was reported that this method brings, 4.3%, 2.9% coding gains in RA main 10, AI 1 frame main 10 configurations, w.r.t WS-PSNR. It is proposed to include such an adaptive QP when generating the anchors for an asserted more fair comparison between the different proposed projections.

This uses a similar formula for QP adaptation as used in WS-PSNR, therefore the improvement on that criterion is not surprising. The QP adaptation is signalled by the encoder.

Typically, this ends up in QP+12 from the equator to the poles (4× the quantization step size), clipped if the value 51 would be surpassed. Some experts raised a concern that this might be too aggressive.

No assessment of subjective effects had been done so far.

The proponents requested including this in the CTC, but more study seemed necessary.

If used for ERP, it should also be considered for other projection formats.

JVET-F0039 AHG8: Codec behaviour on 360° video [F. Galpin, F. Racape, E. François (Technicolor)]

This contribution presents some informative results on the JEM-4.1 behaviour for different mapping projections used in 360° video coding. As quantitative comparisons using objective metrics are still difficult between two different mappings, the goal of this contribution was to give an additional, qualitative evaluation of the mapping effects on compression performance, by observing the behaviour on picture partitioning and coding modes distribution of CUs.

One idea suggested here was mapping the partitioning and intra/inter mode decisions into EAP (equiangular) which has the most homogeneous geometric distribution of samples. What is analysed is the percentage of intra coded areas, average partition size, etc.

This was interesting information, and could be used to better understand properties of projection formats; further study was encouraged.

JVET-F0049 AHG8: Adaptive QP for ERP 360º video [Hendry, M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

This contribution proposes a method for encoding 360º video with the ERP projection by using adaptive QP at the CTU level based on the location of the CTU in the picture. The QP offset is calculated based on the WS-PSNR weight. The proposed method is an encoder change only. Simulation results reportedly show that the proposed method provides average gain of −4.3% RA coding impact (E2E WS-PSNR).

A revision of the contribution updates the viewport quality for AerialCity sequence with ISP projection.

This presented the same idea as F0038, but the QP at the equator is decreased by 2 (and thus clipping to 51 may occur less often in the CTC).

Comparison was also performed against ISP for viewports in the poles, and gain was shown. It was however pointed out that in most sequences the amount of detail in the poles is low, and this might change when there was more detail. Experiments with rotating high-detailed content to the poles could show the effect better.

JVET-F0072 AHG8: Stretching ratio based adaptive quantization for 360° video [Y. Sun, L. Yu (Zheijang Univ.)] [late]

To make the coding distortion in a spherical space more uniform, a stretching ratio based adaptive quantization for 360° video was proposed. Different CTUs in images are assigned with different quantization according to the stretching ratio. The experimental results reportedly show that this proposed method has average impacts of −2.15% and −2.22% for E2E WS-PSNR Y and WS-PSNR (encoder) Y, respectively.

It was not fully clear why the results reported show less gain than for contributions F0038 and F0049. The adaptation formula is similar. From the results, some sequences, e.g. Trolley, Gaslamp, have a bit rate increase (whereas the other contributions showed gains over all sequences).

It was agreed to start an EE to further investigate adaptive QP methods, as follows:

- Both for HM and JEM

- Comparing against other formats with equivalent adaptation of QP

- For the JEM, also a method could be studied without signalling (where the QP adaptation is derived at the decoder from knowledge of the projection format)

- Investigate rotations where higher detailed content is brought to the pole.

- Investigate the impact on subjective quality

It would be desirable not to disallow methods like that. In the case of 360° video, local adaptation of the quantization setting should be allowed if it is exclusively based on geometric position, and applied equally over all pictures.

Due to the urgency of providing the anchors shortly after the meeting, this cannot be included; however, later CfP anchors would likely have geometry based quantization.

# Encoder optimization (2)

Contributions in this category were discussed Sunday 2nd at 12:30 (chaired by JRO).

JVET-F0063 AHG5: Enhanced fast algorithm of JVET-E0078 [P.-H. Lin, C.-L. Lin, Y.-J. Chang, C.-C. Lin, Y.-H. Ju (ITRI)]

This contribution proposes a fast algorithm for the QTBT structure which is based on the method of E0078. The method in E0078 skips some partition processes in QTBT and the conditions are modified in the contribution. The simulation results reportedly show that up to 10% encoding time reduction over JEM-5.0 can be achieved in the RA condition with less than 0.2% BD-rate loss.

The cross-checkers proposed another threshold 0.4 which gives a different tradeoff (only 0.1% loss, but only 6% coding time reduction).

It was pointed out that this specific proposal also gives a run time reduction at low QP (however, also with more losses in these cases).

Currently, a further small reduction of encoding run time was not considered to be of the highest priority. No action was taken on this.

JVET-F0093 Cross-check of JVET-F0063 on AHG5: Enhanced fast algorithm of JVET-E0078 [T. Hashimoto, T. Ikai (Sharp)] [late]

This contribution is a cross-check report of JVET-F0063: AHG5: Enhanced fast algorithm of JVET-E0078. The simulation was carried out based on the common test condition (CTC). The simulation results of bit rate and PSNR reportedly matched with those provided by the proponents. It was reported that the average loss and average relative encoding time (EncT) impact were the following:

- Threshold = 0.40; 0.18 % loss with EncT 91 % in RA

- Threshold = 0.45; 0.13 % loss with EncT 93 % in RA

- Threshold = 0.5; 0.08 % loss with EncT 94 %in RA

Note: The proponent uses the value 0.4.

# Metrics and evaluation criteria (2)

Contributions in this category were discussed Sunday 2nd 1500–1615 (chaired by JRO & GJS).

JVET-F0064 MS-SSIM as an additional mandatory metric to PSNR for future video coding [M. Pettersson, R. Sjöberg, P. Wennersten, K. Andersson, J. Ström, J. Enhorn (Ericsson)]

In JVET-C0030, it was reported that the MS-SSIM metric correlated better with subjective data compared to PSNR and SSIM. During the presentation of that document, a question was raised of how well MS-SSIM was adapted to correctly identify small changes in a video, e.g. turning a tool on/off. The current contribution attempted to answer this question. BD-rate results for MS-SSIM between recent JEM versions were provided. The contribution claimed that the numbers indicate that MS-SSIM is also stable for small changes in subjective quality. The proponents argued that including MS-SSIM during a future video coding development can help creating a more efficient standard.

Version 2 of this contribution adds patches for MS-SSIM integrated into JEM-5.0.1 and HM-16.15.

This was discussed Sunday at 15:00 (GJS & JRO).

MS-SSIM correlation to MOS was asserted to be better – some limited test results were shown for video.

It was asked whether there are examples where a different conclusion would be reached for PSNR and MSSIM. It was reported that C0030 had an example using BQSquare.

It was suggested to test this on something that doesn’t help PSNR, such as deblocking or SAO. It was remarked that for some prior test this had not been done.

It was originally designed for images, not video, and commented that it would not capture some types of distortion – e.g., consistency of motion – that are better captured by PSNR.

It was commented that MS-SSIM seems to have a tendency to encourage excessive blurriness.

A software patch had previously been provided. It was asked why we hadn’t previously put the software into the JEM then.

The scheme has adjustable parameters and asked what values to use. Values used in the contribution were trained on some content.

This includes a use of the variance of the input signal.

Decision (SW): Adopt into JEM software, but not CTC (optional), and not referenced in the Excel template.

Further study was also encouraged.

JVET-F0089 AHG7: Comments on metrics for extended colour volume content [E. François, C. Chevance, F. Hiron (Technicolor), D. Rusanovskyy, A. K. Ramasubramonian (Qualcomm)] [late]

This contribution discussed the metrics specified in the JEM Common Test Conditions on HDR (JVET-E1020) for evaluating coding performance on extended colour volume materials and addressed in AHG7. It reported observations made on the metrics currently considered in the AHG7, and made several suggestions on the potential usage of the wPSNR metric.

The proposed wPSNR performs weighting based on the same criterion that is used for QP adaptation in the HDR anchor. Therefore, it basically gives a preference to algorithms which do it similarly. This is not desirable.

The wPSNR is not symmetric because the original and decoded may have different luminance.

A BoG (coordinated by A. Segall) was asked to work on the following:

- to further discuss metrics for HDR quality assessment

- to coordinate the preparation of the CfE

- to review the available new content

# Exploration experiments, joint meetings, BoG reports, and summary of actions taken

## Exploration experiments

The setup of Exploration Experiments was discussed, and an initial draft of the EE document was reviewed in the plenary (chaired by JRO). This included the list of all tools that are intended to be investigated in EEs during the subsequent meeting cycle:

- EE1: Intra prediction (continuation)

  - JVET-F0033, JVET-F0054, JVET-F0055, JVET-F0104

  - The 10 aspects listed in the EE document are the minimum set expected; if additional information is brought about better combinations of the tools investigated in the EE, such combinations should be considered as well, but add-on elements not known so far shall be considered like new contributions.

- EE2: Decoder Side Motion Vector Derivation (continuation)

  - JVET-F0022

- EE3: Adaptive QP for 360° video

  - JVET-F0038, JVET-F0049, JVET-F0072

  - It was further suggested to add the case of rotated projection, which is known to improve coding efficiency.

EE4: 360° Projection Modifications and Padding (HM only)

  - JVET-F0037, JVET-F0052, JVET-F0108,

Li Zhang was asked to compile the EE document with remote assistance by Elena Alshina, to be circulated by Thursday and reviewed Friday.

It was agreed to give the editors the discretion to finalize the document during the two weeks after the meeting, and to circulate/discuss it on the reflector appropriately.

## Joint meetings

Two joint meeting sessions were held with the parent bodies.

- Joint meeting Monday 1600–1800 on 360° video and tile sets (JCT-VC, JVET, VCEG, MPEG R & S)

(Also reported in the JCT-VC report.)

Liaison statements

- From SG 16 to MPEG (m40692), with remarks on

  - Approved texts

  - Preliminary CfE issuance

  - Requirements

- From MPEG to SG 16 TD 61/Gen

  - Approved texts

  - Prelim CfE

360° video

- Coordination on mathematical conventions JCTVC-AA0033

  - The OMAF WG 11 N 16636 has the x axis pointing to the right, z pointing back, and y pointing up

  - In the draft SEI message, pitch (θ) should be airplane tilt up, yaw (ϕ) is anti-clockwise rotation (as viewed from the origin), and roll is anti-clockwise rotation (as viewed from the origin)

  - Software alignment may be difficult, but we will try to proceed on that basis

- For cube map projection JCTVC-AA0034 / JCTVC-AA0035 (& arrangement of the faces) – the software has a default, which seems like a good candidate for selection – but no action was taken on this yet, although it reportedly has some desirable properties

  - Widely supported in VR applications

  - Removes extreme warping of the poles – more “sensible”, spatially uniform coding format

  - Suitable for tiled streaming

  - Memory bandwidth consumption reduction for rendering

  - Coding efficiency results vary from sequence to sequence, with more reported maximum benefit than maximum loss in some test, but seams cause visual artefacts, so it was suggests that we cannot make a firm conclusion

  - For assignment of future type codes:

    - OMAF has written criteria currently in N 16639 (industry support, visual quality benefit, complexity), which could be the basis of establishing a plan

- Evaluation of projection mappings was discussed – JVET was continuing to work on a subjective testing methodology, and planned to have some method documented as an output of this meeting

- Decision: Angular syntax values should use 32 bits in increments of 2−16.

- Geometry type and projection type are unused JCTVC-AA0022 – remove geometry type, no action at this time on projection type

- Rectilinear viewport (equations like face of cube map, essentially indicating the position and orientation of an ordinary camera) JCTVC-AA0035. The bitstream would have an ordinary view, but with an indication of the location it represents in a spherical space – For further study

- Director’s suggested view JCTVC-AA0036 – (since updated with ID values – Decision: OK)

Motion-constrained tile sets

- Errata issues identified – corrective action planned

- MCTS extraction with slice reordering JCTVC-AA0029 – For further study

- MCTS extraction of non-neighbouring regions JCTVC-AA0024 #11 (item withdrawn)

- MCTS hash JCTVC-AA0028 – No action.

- Maximum number of MCTS tile sets JCTVC-AA0024 (note: already standardized) – consider some fix of the extraction SEI message

- Joint meeting Wednesday 1600–1730 on CfE preparation (JVET, VCEG, MPEG R & S)

In preliminary perceptual testing, in some cases, the JEM with up to 35% lower data rate performed visually better than HEVC HM.

An updated draft of the CfE was reviewed. One aspect was to request bitstreams to be submitted by 6 July. Additionally, JEM 6.0 anchors were planned to be provided by June 10:

- Samsung and Qualcomm were asked to provide anchors for SDR and perform mutual cross-check.

- Samsung was asked to provide the anchors for 360°, and InterDigital to cross-check them. Furthermore, HM anchors for 360° need modification of adaptive search range (InterDigital).

- Sharp, Qualcomm and Technicolor were asked to provide HM and JEM anchors for HDR, and to mutually perform crosschecks.

Small changes were made in the test conditions. An SDR bit rate point was modified. A different part of one sequence was used. Some bit rates were adjusted for HDR. For 360° video, one test sequence was added and some biy rate points were changed.

JVET-F0102 Industry recommendation for Future Video Coding Standard Development [E. Alshina (Samsung), J. Zhou (Huawei), S. Lei (MediaTek), C. Luo (China Telecom), A. Hinds (CableLabs)] [late]

This document made recommendations for Future Video Coding standard development and suggestions regarding standardization practices. This contribution was described as being intended to serve as a starting point for discussing improvements for an improved future video coding standard process to meet market requirements in an efficient and timely matter.

This was reviewed in the joint meeting.

In JVET-F0102, a late contribution commenting on process, planning, and requirements, was presented. Regarding HEVC development, it praised the test model methodology and advocated for “one tool, one functionality”, the principle that clear design and sufficient gain need to be shown to add something to the design, and the incorporation of strong complexity analysis. It advocated holding to a firm two-year timeline upon adoption of the first test model, and “freezing” the low-level design 6 months prior to completion. It advocated having a large and diverse CTC test set, perhaps changing the selected sequences frequently, and ensuring that testing bit rates are aligned to application requirements. It requested emphasis on simpler coding features with modest complexity needs, and evaluation of complexity on GPU and MPU platforms as well as reference software.

Some experts expressed concern that they did not have a sufficient chance for studying this document before it was presented. It was also mentioned that some of the aspects may be premature to discuss, while other aspects are probably valid and reflect the best practices of past standardization efforts rather than being new suggestions.

## Break-out Groups

JVET-F0103 BoG report on test material and subjective assessment [T. Suzuki]

The review of the initial report of the BoG on test material and subjective assessment included discussion of the following:

- New test sequences: Aerial, HLG HDR

- Setup of the test: investigating lowest/highest rate points

- Also making comparisons for various cases with JEM bit rate x vs. HM bit rate x+1

- Informal viewing was suggest for bit rates as follows: 360° video Sun 1400–1700, SDR Sun 1700

- The CfE will be using a double stimulus method, and Vittorio Baroncini was asked to draft the description

- Create 4 more rate points for HM anchors (3x half way between the target rates, and one more higher than highest rate) will be tested, for closer matching MOS with proposals, and better estimation of rate saving. These additional anchors should only be applied for the SDR case, not HDR, not 360°.

A follow-up review was conducted Tuesday:

- In the CfE, a bigger 4K monitor should be used for viewing; the difference was more visible on the large display used at the last meeting than on the small studio monitors that were used at the current meeting.

- For the following sequences, differences were hardly visible: Cross Walk, Timelapse, and Tango.

- For Timelapse, the bit rates should be made lower (the new part is easier to code, it was asked to find another lowest rate point with approximately the same QP as at the last meeting, and otherwise R3->R4).

- Add 4 more bit rate points for HM.

- Submission prior to meeting (July 6) to an ftp site was planned

- For several test sequences, the JEM was visually better than the HM even with 35% lower bit rate; for other sequences it was equivalent

- To be done: Viewing session 3 PM, SDR new sequences, HDR anchor viewing, new HDR sequences.

- 360°: An additional viewing session was planned for 14:00, comparing original and coded

- Skateboard: Use lower bit rates (r3->r4 etc.), select a different dynamic viewport

- Add Trolley as a fifth sequence, Interdigital generated HM results, and based on this, rates were defined for the CfE

- Mathias Wien and Minhua Zhou were asked to define dynamic viewports by July 5, such that Vittorio Baroncini can decode bitstreams and generate viewable sequences.

- CfE document preparation coordinators were agreed as follows: Andrew Segall (HDR), Jill Boyce (360°), Teruhiko Suzuki (SDR)

Another version of the BoG report was presented Wed. afternoon.

New test sequences were reviewed:

- From JVET-F0062: Beach Mountain and Mountain Bay are more appropriate aerial sequences. These could be considered in the future for CTC or CfP (probably one of them, to be further evaluated). These need more evaluation to define a set of bit rates (similar to CfE), to be decided in the next meeting. Additional study of these was planned under mandates of a test sequence AHG.

- From JVET-F0094: This had some test sequences which are easy to encode, but show colour artefacts (e.g. cranes, butterfly), and others that are more challenging in terms of texture and motion (e.g. water sequences). These should be further studied in terms of properties and bit rate ranges for subjective evaluation (under mandates of an HDR AHG). Coding was done in HLG, with no adaptive quantization. This could be remapped in the PQ domain.

JVET-F0106 BoG report on 360° Video [J. Boyce]

This was presented Wed. afternoon.

Recommendation and Decision: Remove TSP from CTC table; only compare projection formats with comparable resolution.

Recommendation and Decision: Restrict the number of coded pixels not to be larger than 1% higher than ERP in the CTC (in both OMAF and JEM test cases, as far as projection formats are concerned; for tools, additional results may be reported which give up such restrictions). Note that in the CfE and CfP such restrictions shall not apply.

The question was raised whether postprocessing should be allowed. It was agreed to be mandatory to deliver results without postprocessing across face boundaries, but optionally additional results may be provided. A question in this context was whether post-processing would be before, after or during rendering – the OMAF activity should decide if they want it.

An initial version of the CTC document was reviewed Thursday morning, and an initial version was made available in the document repository.

JVET-F0107 BoG Report on Extended Colour Volume Material [A. Segall]

The following activities and discussions were reported:

- Performed viewing, redefining CfE rate points (lower)

- New test sequences were viewed

- Potential removal of tPSNR

- Further work: Check new bit rates with HM encoding, discuss metrics further, establish work plan for investigating new sequences

- Follow-up BoG meeting Thursday

The BoG reported again Thursday afternoon:

- No recommendation of removing metrics at this meeting

- Work was planned for investigation of new test sequences

- Defined new rate points based on viewing

- To update the CTC document, to precisely point out version of HDRtools, and improve definition of metrics

- Decision (SW): Include the HDR anchor software (encoder optimization of quantization) in JEM 6.0

## List of actions taken affecting JEM6

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the JEM6 or 360Lib3.0 description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

This was presented and confirmed to be complete Friday morning in the JVET plenary.

### Encoder-only or software changes

JVET-F0064 MS-SSIM as an additional mandatory metric to PSNR for future video coding [M. Pettersson, R. Sjöberg, P. Wennersten, K. Andersson, J. Ström, J. Enhorn (Ericsson)]

Adopted into JEM software, but not CTC (optional, off by default). No reference in the Excel template.

Under BoG report JVET-F0107:

Include the HDR anchor software (encoder optimization of quantization) in JEM 6.0. By default off in the SDR and 360° CTC, by default on in the HDR CTC.

### Syntax/semantics/decoding process changes

JVET-F0028 EE3: BIO w/o block extension [A. Alshin, E. Alshina (Samsung)]

JVET-F0031 Reduction redundant syntax signalling for transform skip [H. Jang, J. Lim, J. Nam, S.-H. Kim (LGE)]

JVET-F0032 EE3: Enhanced FRUC Template Matching Mode [Y. Lin, X. Chen, J. An, J. Zheng (HiSilicon)]

Adopt F0032 Aspect 2. However, the multiplication by factor 1.1 should be implemented in fixed point precision.

JVET-F0096 EE2-JVET related: Division-free bilateral filter [J. Ström, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. Sjöberg (Ericsson)]

### Changes in 360lib

JVET-F0025 AHG8: Adjusted cubemap projection for 360-degree video [M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

Include in 360Lib.

JVET-F0036 AHG8: Rotated Sphere Projection for 360° Video [A. Abbas, D. Newman (GoPro)]

Include in 360Lib.

JVET-F0041 AHG8: Platform independent floating point to integer conversion for 360Lib [Y. He, P. Hanhart, Y. Ye (InterDigital)]

JVET-F0042 AHG8: On cross-format S-PSNR-NN [Y. He, X. Xiu, Y. Ye (InterDigital)]

Replace the previous cross format S-PSNR-NN by the new method.

JVET-F0053 AHG8: An improvement on compact octahedron projection with padding [Y.-H. Lee, H.-C. Lin, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

Change the COHP1 design in the 360Lib software and update the CTC accordingly.

JVET-F0065 360Lib modifications for spherical rotation [Q. Xu, J. Boyce (Intel), Y. He, Y. Ye (InterDigital)]

# Project planning

## JEM description drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- JEM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be Wednesday 5 July 2017.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for Experiments

Note: This section was drafted during the second JVET meeting, and is kept here for information about the EE procedure.

Group coordinated experiments have been planned. These may generally fall into one category:

- "Exploration experiments" (EEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the main branch of JEM by the next meeting.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. (E. Alshina will edit the document based on input from the proponents, review is performed in the plenary)

- Software for tools investigated in EE is provided in a separate branch of the software repository

- During the experiment, further improvements can be made

- By the next meeting it is expected that at least one independent party will report a detailed analysis about the tool, confirms that the implementation is correct, and gives reasons to include the tool in JEM

- As part of the experiment description, it should be captured whether performance relative to JEM as well as HM (with all other tools of JEM disabled) should be reported by the next meeting.

It is possible to define sub-experiments within particular EEs, for example designated as EEX.a, EEX.b, etc., where X is the basic EE number.

As a general rule, it was agreed that each EE should be run under the same testing conditions using one software codebase, which should be based on the JEM software codebase. An experiment is not to be established as a EE unless there is access given to the participants in (any part of) the TE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-B1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to EE work should identify individuals in addition to company names.

EE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the EE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish an EE – a single proponent is not enough. It is highly desirable have more than just one cross-checker.

Some agreements relating to EE activities were established as follows:

- Only qualified JVET members can participate in an EE.

- Participation in an EE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the EE should be announced and made available to all EE participants in a timely manner.

A separate branch under the experimental section will be created for each new tool included in the EE. The proponent of that tool is the gatekeeper for that separate software branch. (This differs from the main branch of the JEM, which is maintained by the software coordinators.)

New branches may be created which combine two or more tools included in the EE document or the JEM. Requests for new branches should be made to the software coordinators.

We don’t need to formally name cross-checkers in the EE document. To promote the tool to the JEM at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

Timeline:

T1 = JEM5.0 SW release + 4 weeks: Integration of all tools into separate EE branch of JEM is completed and announced to JVET reflector.

Initial study by cross-checkers can begin.

Proponents may continue to modify the software in this branch until T2

3rd parties encouraged to study and make contributions to the next meeting with proposed changes

T2: JVET-F meeting start – 3 weeks: Any changes to the exploration branch software must be frozen, so the cross-checkers can know exactly what they are cross-checking. An SVN tag should be created at this time and announced on the JVET reflector.

This procedure was again confirmed during the closing plenary of the third JVET meeting. It was further confirmed that the Common Test Conditions of JVET-B1010 are still valid, however the CTC encoder setting will be reflected in the config file that is attached to the JEM4.0 package.

## Software development and anchor generation

Software coordinators will work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- JEM6.0 including all adoptions from section 10.4 will be released by 2017-04-21.

- The results about coding performance of JEM6.0 will be reported by 2017-05-02.

- Further versions may be released for additional bug fixing, as appropriate

Timeline of 360Lib3.0: 2 weeks after the meeting (2017-04-21).

Further versions may be released as appropriate for bug fixing.

Timelines and volunteers for CfE anchors: See notes under section 10.2

New HM anchors will be generated using HM 16.15. JEM anchors will be based on JEM 6.0.

# Output documents and AHGs

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header needs to be generated. This version should be sent to GJS and JRO for upload.

JVET-F1000 Meeting Report of the 6th JVET Meeting [G. J. Sullivan, J.-R. Ohm (JVET coordinators)] [2017-07-14] (near next meeting)

Intermediate versions of the meeting notes (d0 … d6) were made available on a daily basis during the meeting.

JVET-F1001 Algorithm description of Joint Exploration Test Model 6 (JEM6) [J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce (editors)] [2017-04-28] (WG 11 N 16887)

See list of new adoptions under section 10.4. During the closing plenary, no complaints were made about the accuracy of that list.

JVET-F1002 Joint Call for Evidence on Video Compression with Capability beyond HEVC [M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki (editors)] [2017-04-14] (WG 11 N 16886)

Draft was discussed Wednesday 1400 and in a joint meeting with parent bodies Wednesday 1600. It was again presented during the closing JVET plenary on Friday.

The companies responsible for providing the HM and JEM anchors were asked to also provide the corresponding Excel templates for the cases of SDR, HDR and 360°.

(See responsibilities for generating anchors under section 10.2.)

JVET-F1003 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib [Y. Ye, E. Alshina, J. Boyce (editors)] [2017-04-28] (WG 11 N 16888)

See list of new adoptions under section 10.4.3. During the closing plenary, no complaints were made about the accuracy of that list.

JVET-F1004 Subjective testing method for 360º video projection formats using HEVC [J. Boyce, E. Alshina, Z. Deng (editors)] [2017-04-21] (WG 11 N 16892)

Remains valid – not updated: JVET-B1010 JVET common test conditions and software reference configurations [K. Suehring, X. Li (editors)]

This remains valid (from 2nd meeting).

Note: Encoder settings reflected in the config file related to CTC in JEM5 (see changes under section 10.4)

A directory had been installed in the ftp for bitstreams and results of anchors, but in the previous meeting cycle this had not yet been used. It was planned to upload bitstreams and Excel sheets after completion of anchors.

JVET-F1011 Description of Exploration Experiments on coding tools [E. Alshina, L. Zhang (editors)] [2017-04-28] (WG 11 N 16889)

See list of EEs under section 10.1.

JVET-F1020 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, D. Rusanovskyy (editors)] [2017-04-21] (WG 11 N 16890)

JVET-F1030 JVET common test conditions and evaluation procedures for 360º video [E. Alshina, J. Boyce, A. Abbas, Y. Ye (editors)] [2017-04-21] (WG 11 N 16891)

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Tool evaluation (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate the exploration experiments. - Investigate interaction of tools in JEM and exploration experiment branches. - Discuss and evaluate methodologies and criteria to assess the benefit of tools, and how to ease the assessment of single tools in terms of encoder runtime. - Study and summarize new technology proposals. | E. Alshina, M. Karczewicz (co-chairs) | N |
| JEM algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-F1001 Algorithm Description of Joint Exploration Test Model 6. - Gather and address comments for refinement of the document. - Coordinate with the JEM software development AHG to address issues relating to mismatches between software and text. | J. Chen (chair), E. Alshina, J. Boyce (vice chairs) | N |
| JEM software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of the JEM6.0 software packages and their distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver JEM6.0 software version and the reference configuration encodings according to JVET-B1010 common conditions. - Coordinate with AHG on JEM model editing and errata reporting to identify any mismatches between software and text, and make further updates to the software as appropriate. - Investigate the implementation of SCC coding tools in JEM. - Coordinate with AHG6 for integration of 360° video software. | X. Li, K. Suehring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of future video coding standards. - Identify and recommend appropriate test materials and corresponding test conditions for use in the development of future video coding standards. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Discuss and prepare HM anchors at 4 additional rate points for the Call for Evidence. - Evaluate new test sequences, e.g. sequences proposed in JVET-F0062, and prepare for the visual assessment in the next meeting. - Make logistic arrangements for the forthcoming Call for Evidence. | V. Baroncini, T. Suzuki (co-chairs), J. Chen, J. Boyce, A. Norkin (vice chairs) | N |
| Memory bandwidth consumption of coding tools (AHG5) (jvet@lists.rwth-aachen.de) - Study the methodology of measuring decoder memory bandwidth consumption, including cache models. - Develop software tools for measuring both average and worst case of memory bandwidth. - Make analysis for examples of JEM coding tools. - Study the impact of memory bandwidth on specific application cases. | X. Li, E. Alshina, T. Ikai, H. Yang | N |
| 360º video conversion software development (AHG6) (jvet@lists.rwth-aachen.de) - Prepare and deliver 360Lib-3.0 software version and common test condition configuration files according to JVET-F1030. - Generate CTC anchors and a reporting template for the common test conditions. - Produce documentation of software usage for distribution with the software. - Coordinate with JEM software coordinators for integration of 360Lib-3.0 within most recent versions of JEM software packages. - Review integration framework and refine the interface to minimize the needed software changes in the JEM. | Y. He, V. Zakharchenko (co-chairs) | N |
| JEM coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Study and refine test conditions and anchors for the JEM coding of HDR/WCG content. - Coordinate generation of HM and JEM anchors for the CfE. - Study and evaluate available HDR/WCG test content. - Study and evaluate the application of QP adaptation in the context of an HLG container. - Study and evaluate visual quality assessment methods including rendering content with a peak brightness larger than available displays. - Study objective metrics for quality assessment of HDR/WCG material. - Study methods for increased coding efficiency of HDR/WCG material. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. Francois, D. Rusanovskyy (vice chairs) | N |
| 360º video coding tools and test conditions (AHG8) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Study consistency of and potential improvements to the objective quality metrics in CTC. - Coordinate effort to prepare for CfE testing, including anchor generation, in collaboration with AHG5. - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Produce and finalize JVET-F1003 algorithm descriptions of projection format conversion process and objective quality metrics in 360Lib. - Produce and finalize JVET-F1004 Subjective testing method for comparison of 360° video projection formats using HEVC. - Produce and finalize JVET-F1030 JVET common test conditions and evaluation procedures for 360° video. - Study coding tools dedicated to 360° video, and their impact on compression. | J. Boyce (chair), A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye (vice chairs) | N |
| 4:4:4 support in JEM (AHG9) (jvet@lists.rwth-aachen.de) - Evaluate JEM6.0 software in terms of 4:4:4 chroma sampling support: Identify the tools in JEM6.0 that are not able to support 4:4:4 chroma sampling appropriately. Also identify whether this is due to implementation limitations or tool characteristics. | A. Tourapis (chair), X. Li (vice chair) | N |

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Wednesday or Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–7.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 7.5 meeting days).

In cases where high workload is expected for a meeting, an earlier starting date may be defined.

Some specific future meeting plans (to be confirmed) were established as follows:

- Thu. 13 – Fri. 21 Jul. 2017, 7th meeting under WG 11 auspices in Torino, IT.

- Wed. 18 – Wed. 25 Oct. 2017, 8th meeting under ITU-T auspices in Macao, CN.

- Fri. 19 Jan. – Fri. 26 Jan. 2018, 9th meeting under WG 11 auspices in Gwangju, KR.

- Fri. 13 Apr. – Fri. 20 Apr. 2018, 10th meeting under WG 11 auspices in San Diego, US.

The agreed document deadline for the 7th JVET meeting is Wednesday 5 July 2017. Plans for scheduling of agenda items within that meeting remained TBA.

Kenzler Conference Management was thanked for the excellent hosting of the 6th meeting of the JVET, and Justin Ridge was thanked for exceptional Tasmanian hospitality. NHK and GBTech were thanked for providing viewing equipment. Vittorio Baroncini was thanked for conducting visual tests, Teruhiko Suzuki, Edouard François, and Yuwen He were thanked for their great help in organizing the tests. The participants in the expert viewing were also thanked.

The 6th JVET meeting was closed at approximately 1155 hours on Friday 7 April 2017.

# Annex A to JVET report:
List of documents

| JVET-VC number | MPEG number | Created | First upload | Last upload | Title | Authors |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-F0001 | m40594 | 2017-03-29 07:00:40 | 2017-03-29 07:01:34 | 2017-03-29 07:01:34 | JVET AHG report: Tool evaluation (AHG1) | M. Karczewicz, E. Alshina |
| JVET-F0002 | m40566 | 2017-03-28 07:17:51 | 2017-03-30 12:17:38 | 2017-03-31 01:23:36 | JVET AHG report: JEM algorithm description editing (AHG2) | J. Chen, E. Alshina, J. Boyce |
| JVET-F0003 | m40581 | 2017-03-28 19:16:43 | 2017-03-30 06:46:09 | 2017-03-30 06:46:09 | JVET AHG report: JEM software development (AHG3) | X. Li, K. Sühring |
| JVET-F0004 | m40572 | 2017-03-28 10:38:32 | 2017-03-30 23:52:09 | 2017-03-30 23:52:09 | JVET AHG report: Test material (AHG4) | T. Suzuki, V. Baroncini, J. Chen, J. Boyce, A. Norkin |
| JVET-F0005 | m40629 | 2017-03-30 11:06:41 | 2017-03-30 11:21:30 | 2017-03-31 03:33:14 | JVET AHG report: Fast encoding, encoding complexity investigation, and configuration settings (AHG5) | K. Choi, Y.-J. Chang, H. Huang, X. Li, P. Philippe, Y. Yasugi |
| JVET-F0006 | m40565 | 2017-03-28 05:58:09 | 2017-03-28 05:59:59 | 2017-03-31 02:15:08 | JVET AHG Report on 360° video conversion software development (AHG6) | Y. He, V. Zakharchenko |
| JVET-F0007 | m40636 | 2017-03-31 00:15:55 | 2017-03-31 00:18:18 | 2017-03-31 00:47:34 | JVET AHG Report on JEM coding of HDR/WCG material (AHG7) | A. Segall, E. François, D. Rusanovskyy |
| JVET-F0008 | m40226 | 2017-02-23 01:17:07 | 2017-02-23 17:27:20 | 2017-03-30 08:58:55 | JVET AHG report: 360° video coding tools and test conditions (AHG8) | J. Boyce, A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye |
| JVET-F0010 | m40428 | 2017-03-27 10:40:30 | 2017-03-27 10:41:12 | 2017-03-31 08:50:26 | Exploration Experiments on Coding Tools Report | E. Alshina, L. Zhang |
| JVET-F0021 | m40219 | 2017-02-18 03:11:33 | 2017-02-21 23:08:33 | 2017-02-21 23:08:33 | AHG8: Subjective testing of 360° video projection/packing formats | J. Boyce, Z. Deng (Intel) |
| JVET-F0022 | m40259 | 2017-03-15 00:05:10 | 2017-03-15 00:17:14 | 2017-03-15 00:17:14 | EE3-related: A block-based design for Bi-directional optical flow (BIO) | H.-C. Chuang, J. Chen, X. Li, Y.-W. Chen, M. Karczewicz, W.-J. Chien (Qualcomm) |
| JVET-F0023 | m40283 | 2017-03-21 19:59:48 | 2017-03-21 21:13:53 | 2017-03-29 13:02:49 | AHG3: JEM coding results for adaptive search range | K. Sühring, K. Sharman, B. Bross (HHI) |
| JVET-F0024 | m40294 | 2017-03-22 17:35:16 | 2017-03-22 17:38:21 | 2017-04-01 20:36:22 | EE1: Unequal Weight Planar Prediction, Constrained PDPC, and Constrained ARSS | K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris) |
| JVET-F0025 | m40300 | 2017-03-23 01:54:53 | 2017-03-24 07:17:28 | 2017-04-02 04:12:28 | AHG8: Adjusted cubemap projection for 360-degree video | M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm) |
| JVET-F0026 | m40301 | 2017-03-23 01:56:54 | 2017-03-24 07:20:49 | 2017-04-01 09:34:42 | AHG8: Equatorial cylindrical projection for 360-degree video | G. Van der Auwera, M. Coban, M. Karczewicz (Qualcomm) |
| JVET-F0027 | m40303 | 2017-03-23 09:23:28 | 2017-03-24 11:09:18 | 2017-04-02 08:57:26 | AhG8: JEM vs HM performance for 360° video content under CfE test condition | A. Singh, C. Pujara, S. Nitchith Akula, A. Dsouza, Ramkumaar K. K., R. Narayana Gadde, V. Zakharchenko, E. Alshina, K. P. Choi (Samsung) |
| JVET-F0028 | m40304 | 2017-03-23 09:26:15 | 2017-03-24 06:19:53 | 2017-03-28 12:13:36 | EE3: BIO w/o block extension | A. Alshin, E. Alshina (Samsung) |
| JVET-F0029 | m40305 | 2017-03-23 09:27:02 | 2017-03-24 09:09:10 | 2017-03-24 09:09:10 | On memory bandwidth of JEM5.0 tools | E. Alshina (Samsung) |
| JVET-F0030 | m40306 | 2017-03-23 09:27:53 | 2017-03-28 03:43:53 | 2017-03-28 03:43:53 | Cross-check for block-based design for Bi-directional optical flow (JVET-F0022) | E. Alshina (Samsung) |
| JVET-F0031 | m40315 | 2017-03-23 12:50:23 | 2017-03-23 12:54:35 | 2017-04-02 01:14:26 | Redundant syntax signalling reduction for transform skip | H. Jang, J. Lim, J. Nam, S.-H. Kim (LGE) |
| JVET-F0032 | m40317 | 2017-03-23 14:38:11 | 2017-03-23 14:44:50 | 2017-03-23 14:44:50 | EE3: Enhanced FRUC Template Matching Mode | Y. Lin, X. Chen, J. An, J. Zheng (HiSilicon) |
| JVET-F0033 | m40318 | 2017-03-23 15:08:03 | 2017-03-23 15:11:45 | 2017-03-31 23:46:37 | EE1-Related : UWP with Constrained PDPC and Constrained ARSS | H. Jang, J. Lim, J. Nam, S.-H. Kim (LGE) |
| JVET-F0034 | m40320 | 2017-03-23 15:23:57 | 2017-03-23 20:30:14 | 2017-03-23 20:30:14 | EE2-JVET-E0032 Bilateral filter Test 1, Test2 | J. Ström, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. Sjöberg (Ericsson) |
| JVET-F0035 | m40321 | 2017-03-23 15:37:46 | 2017-03-27 09:53:26 | 2017-03-27 09:53:26 | AHG8: Additional test results of JVET-E0090 on nested polygonal chain packing of 360-degree ERP pictures | K. Kammachi-Sreedhar, M. M. Hannuksela (Nokia) |
| JVET-F0036 | m40325 | 2017-03-23 17:12:59 | 2017-03-23 17:22:35 | 2017-04-02 04:43:42 | AHG8: Rotated Sphere Projection for 360° Video | A. Abbas, D. Newman (GoPro) |
| JVET-F0037 | m40326 | 2017-03-23 17:34:02 | 2017-03-28 06:26:19 | 2017-04-03 16:06:37 | AHG8: Padding method for Segmented Sphere Projection | C. Zhang, Y. Lu, J. Li, Z. Wen (Owl Reality) |
| JVET-F0038 | m40327 | 2017-03-23 19:38:32 | 2017-03-23 19:44:16 | 2017-03-31 15:23:53 | AHG8: adaptive QP for 360° video coding | F. Racape, F. Galpin, G. Rath, E. François (Technicolor) |
| JVET-F0039 | m40328 | 2017-03-23 19:59:07 | 2017-03-23 20:00:57 | 2017-03-31 15:49:15 | AHG8: Codec behaviour on 360° videos | F. Galpin, F. Racape, E. François (Technicolor) |
| JVET-F0040 | m40330 | 2017-03-23 21:24:59 | 2017-03-23 21:28:14 | 2017-04-05 06:36:39 | Proposed Revision of Preliminary Joint Call for Evidence on Video Compression with Capability beyond HEVC | M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki |
| JVET-F0041 | m40331 | 2017-03-23 21:40:56 | 2017-03-24 04:45:07 | 2017-04-02 10:02:35 | AHG8: Platform independent floating point to integer conversion for 360Lib | Y. He, P. Hanhart, Y. Ye (InterDigital) |
| JVET-F0042 | m40332 | 2017-03-23 21:50:48 | 2017-03-24 04:46:10 | 2017-04-02 09:02:32 | AHG8: On cross-format S-PSNR-NN | Y. He, X. Xiu, Y. Ye (InterDigital) |
| JVET-F0043 | m40334 | 2017-03-24 00:54:33 | 2017-03-24 05:05:30 | 2017-04-02 01:14:50 | NSST memory reduction | S.-H. Kim, H. Jang, J. Lim (LGE) |
| JVET-F0044 | m40335 | 2017-03-24 01:43:24 | 2017-03-24 09:43:24 | 2017-04-04 04:35:59 | AHG5: Performance evaluation of Adaptive Search Range on JEM-5.0.1 | Y. Yasugi, T. Ikai (Sharp) |
| JVET-F0045 | m40336 | 2017-03-24 01:43:51 | 2017-03-24 02:10:48 | 2017-03-24 02:10:48 | EE1: Cross-check of JVET-F0024 on Unequal Weight Planar Prediction, Constrained PDPC, and Constrained ARSS | T. Ikai (Sharp) |
| JVET-F0046 | m40337 | 2017-03-24 01:45:13 |  |  | Withdrawn | Y. Yasugi, T. Ikai |
| JVET-F0047 | m40338 | 2017-03-24 02:09:41 | 2017-03-28 05:04:49 | 2017-03-28 05:04:49 | EE1: Cross-check of test1 on unequal weight planar | J. Lee, H. Lee, J. Kang (ETRI) |
| JVET-F0048 | m40339 | 2017-03-24 02:19:37 | 2017-03-30 04:20:19 | 2017-03-30 04:20:19 | EE2: Cross-check of bilateral filter (JVET-F0034 test 2) | S.-C. Lim, J. Kang (ETRI) |
| JVET-F0049 | m40340 | 2017-03-24 02:22:19 | 2017-03-24 23:08:51 | 2017-04-04 02:41:15 | AHG8: Adaptive QP for ERP 360Âº video | Hendry, M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm) |
| JVET-F0050 | m40341 | 2017-03-24 02:45:08 | 2017-03-27 22:45:57 | 2017-03-31 07:54:19 | AhG8: Dynamic viewport based subjective evaluation of 360-degree video coding under CfE test conditions | P. Hanhart, Y. He, Y. Ye (InterDigital) |
| JVET-F0051 | m40342 | 2017-03-24 02:53:54 | 2017-03-24 03:19:42 | 2017-03-28 12:22:50 | Unified Adaptive Loop Filter for Luma and Chroma | J. An, J. Zheng (HiSilicon) |
| JVET-F0052 | m40343 | 2017-03-24 03:45:37 | 2017-03-24 04:23:10 | 2017-04-04 06:43:53 | AHG8: EAP-based segmented sphere projection with padding | Y.-H. Lee, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek) |
| JVET-F0053 | m40344 | 2017-03-24 03:46:13 | 2017-03-24 04:50:58 | 2017-04-04 06:45:59 | AHG8: An improvement on compact octahedron projection with padding | Y.-H. Lee, H.-C. Lin, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek) |
| JVET-F0054 | m40346 | 2017-03-24 04:53:41 | 2017-03-24 06:58:03 | 2017-03-31 12:27:30 | Non-EE1: Alternative setting for PDPC mode | M. Karczewicz, N. Hu, X. Zhao, V. Seregin (Qualcomm) |
| JVET-F0055 | m40347 | 2017-03-24 04:53:56 | 2017-03-24 06:59:11 | 2017-03-31 12:29:18 | Non-EE1: Explicit flag signalling for ARSS | V. Seregin, X. Zhao, M. Karczewicz (Qualcomm) |
| JVET-F0056 | m40348 | 2017-03-24 04:54:13 | 2017-03-29 05:10:50 | 2017-03-29 05:10:50 | EE1: Cross-check of Test 6 with UW planar and restricted ARSS | V. Seregin (Qualcomm) |
| JVET-F0057 | m40349 | 2017-03-24 06:26:00 | 2017-03-28 20:31:36 | 2017-04-04 01:48:52 | EE2: Cross-check of bilateral filter (JVET-F0034 Test1, Test2) | L. Zhang (Qualcomm) |
| JVET-F0058 | m40350 | 2017-03-24 06:54:09 | 2017-03-28 22:31:29 | 2017-04-02 01:20:35 | Cross-check of JVET-F0051 Unified Adaptive Loop Filter for Luma and Chroma | L. Zhang (Qualcomm) |
| JVET-F0059 | m40351 | 2017-03-24 07:02:47 | 2017-03-27 18:34:51 | 2017-03-27 18:34:51 | EE3: Crosscheck of JVET-F0028 BIO w/o block extension | Y.-W. Chen, X. Li (Qualcomm) |
| JVET-F0060 | m40352 | 2017-03-24 08:38:53 | 2017-03-24 08:45:53 | 2017-03-31 06:37:06 | AHG4: Report on CfE anchor generation for SDR content with HM | K. Choi, E. Alshina (Samsung) |
| JVET-F0061 | m40353 | 2017-03-24 08:40:18 | 2017-03-24 08:46:34 | 2017-03-31 06:37:41 | AHG4: Report on CfE anchor generation for SDR content with JEM | K. Choi, E. Alshina (Samsung) |
| JVET-F0062 | m40354 | 2017-03-24 08:59:50 | 2017-03-29 12:36:53 | 2017-04-06 06:34:46 | New aerial photography sequences for video coding standard development | X. Zheng, W. Li, Z. Cao, W. Su, C. Zhao, Y. Li, Z. Lorenz, H. Wu, Z. Du, D. A. Hoang (DJI) |
| JVET-F0063 | m40355 | 2017-03-24 09:46:39 | 2017-03-24 10:45:26 | 2017-04-04 04:34:01 | AHG5: Enhanced fast algorithm of JVET-E0078 | P.-H. Lin, C.-L. Lin, Y.-J. Chang, C.-C. Lin, Y.-H. Ju (ITRI) |
| JVET-F0064 | m40356 | 2017-03-24 10:16:20 | 2017-03-24 10:29:15 | 2017-04-02 08:38:23 | MS-SSIM as an additional mandatory metric to PSNR for future video coding | M. Pettersson, R. Sjöberg, P. Wennersten, K. Andersson, J. Ström, J. Enhorn (Ericsson) |
| JVET-F0065 | m40358 | 2017-03-24 15:19:14 | 2017-03-24 15:28:51 | 2017-03-24 15:28:51 | 360Lib modifications for spherical rotation | Q. Xu, J. Boyce (Intel), Y. He, Y. Ye (InterDigital) |
| JVET-F0066 | m40361 | 2017-03-24 18:56:31 | 2017-03-28 20:06:51 | 2017-04-01 09:39:03 | AHG8: On inactive pixels in evaluation of 360° video projections | G. Van der Auwera, M. Coban (Qualcomm) |
| JVET-F0067 | m40372 | 2017-03-24 22:46:03 | 2017-03-24 22:47:58 | 2017-03-24 22:47:58 | Comments on subjective testing procedure of 360° video | K. Kawamura, S. Naito (KDDI) |
| JVET-F0068 | m40390 | 2017-03-26 17:34:18 | 2017-03-27 16:09:05 | 2017-03-31 01:19:46 | Withdrawn | P. Topiwala, M. Krishnan, W. Dai (FastVDO) |
| JVET-F0069 | m40392 | 2017-03-27 02:20:00 | 2017-03-27 02:21:30 | 2017-03-31 00:38:56 | AHG4: SDR anchor generation for Preliminary Joint Call for Evidence by Qualcomm | H.-C. Chuang, J. Chen, M. Karczewicz |
| JVET-F0070 | m40395 | 2017-03-27 04:13:42 | 2017-03-27 23:59:58 | 2017-03-27 23:59:58 | EE1: Cross-check of Test 4 (UW-Planar with PDPC restricted for different number of modes) | S.-H. Kim, H. Jang, J. Lim (LGE) |
| JVET-F0071 | m40396 | 2017-03-27 04:48:45 | 2017-03-31 01:57:36 | 2017-03-31 01:57:36 | EE3: Crosscheck for Enhanced FRUC Template Matching Mode (JVET-F0032) | W. Zhang (Hulu) |
| JVET-F0072 | m40397 | 2017-03-27 04:58:31 | 2017-03-30 22:54:18 | 2017-04-07 01:18:13 | AHG8: Stretching ratio based adaptive quantization for 360° video | Y. Sun, L. Yu (Zhejiang Univ.) |
| JVET-F0073 | m40421 | 2017-03-27 09:02:14 | 2017-03-28 17:34:17 | 2017-03-28 17:34:17 | EE2: Cross-check of JVET-F0034 Bilateral filter Test 1, Test2 | Z. Deng (Intel) |
| JVET-F0074 | m40433 | 2017-03-27 11:45:30 |  |  | Withdrawn | I. Marzuki, Y.-J. Ahn, S. Park, D. Sim (KWU) |
| JVET-F0075 | m40549 | 2017-03-28 01:42:28 | 2017-03-30 12:20:29 | 2017-03-30 12:20:29 | Crosscheck of JVET-F0043 NSST memory reduction | X. Zhao, V. Thirumalai, V. Seregin, A. Said, M. Karczewicz (Qualcomm) |
| JVET-F0076 | m40553 | 2017-03-28 02:47:15 | 2017-03-28 03:33:43 | 2017-03-28 03:33:43 | Request for 4:4:4 support in the JEM software | M. Meyer, A. M. Tourapis, D. Singer (Apple) |
| JVET-F0077 | m40567 | 2017-03-28 08:25:14 | 2017-03-31 15:03:05 | 2017-03-31 15:03:05 | AHG8: Crosscheck of JVET-F0052 EAP-based segmented sphere projection with padding | Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital) |
| JVET-F0078 | m40568 | 2017-03-28 08:50:43 | 2017-03-30 17:22:19 | 2017-03-30 17:22:19 | Cross-check of JVET-F0054 Non-EE1: Alternative setting for PDPC mode | F. Le Léannec, T. Poirier, F. Urban, E. François (Technicolor) |
| JVET-F0079 | m40573 | 2017-03-28 13:21:33 | 2017-03-31 01:42:40 | 2017-04-01 00:05:51 | Cross-check of JVET-F0031 (Redundant syntax signalling reduction for transform skip) | V. Lorcy (b-com), P. Philippe (Orange) |
| JVET-F0080 | m40589 | 2017-03-29 05:23:07 | 2017-04-01 00:17:19 | 2017-04-02 01:40:37 | Cross-check of JVET-F0033 : UWP with Constrained PDPC and Constrained ARSS | K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris) |
| JVET-F0081 | m40590 | 2017-03-29 05:25:14 | 2017-04-01 00:11:37 | 2017-04-02 01:35:30 | Cross-check of JVET-F0055: Explicit flag signalling for ARSS | K. Panusopone, Y. Yu, L. Wang (Arris) |
| JVET-F0082 | m40591 | 2017-03-29 06:21:14 | 2017-03-31 00:27:28 | 2017-03-31 00:27:28 | Cross-check of JVET-F0022 (EE3-related: A block-based design for Bi-directional optical flow) | O. Nakagami (Sony) |
| JVET-F0083 | m40595 | 2017-03-29 07:30:42 | 2017-03-29 15:26:36 | 2017-04-02 01:59:18 | AHG8: Subjective test pilot study of 360Âº video projection/packing formats | Z. Deng, L. Xu, J. Boyce (Intel) |
| JVET-F0084 | m40622 | 2017-03-30 07:34:45 | 2017-03-30 07:41:07 | 2017-03-30 07:41:07 | Cross-check of JVET-E0053 on An improvement on compact octahedron projection with padding | V. Zakharchenko |
| JVET-F0085 | m40623 | 2017-03-30 08:43:05 | 2017-03-31 15:06:40 | 2017-03-31 15:06:40 | AHG8: Crosscheck of JVET-F0036 Rotated sphere projection for 360° video | Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital) |
| JVET-F0086 | m40628 | 2017-03-30 10:27:13 | 2017-03-30 22:39:28 | 2017-03-30 22:39:28 | AHG7: Information on CfE anchor generation for HDR content and comments on coding conditions | D. Rusanovskyy, A. Ramasubramonian (Qualcomm), E. François (Technicolor) |
| JVET-F0087 | m40630 | 2017-03-30 11:17:08 | 2017-03-30 11:19:57 | 2017-04-02 07:31:37 | AHG4: Cross-check of SDR anchor generation (JVET-0069) | K. Choi, E. Alshina (Samsung) |
| JVET-F0088 | m40633 | 2017-03-30 22:13:30 | 2017-04-01 18:57:28 | 2017-04-01 18:57:28 | AHG8: Crosscheck for JVET-F0041 | Hendry (Qualcomm) |
| JVET-F0089 | m40635 | 2017-03-31 00:13:15 | 2017-03-31 00:38:02 | 2017-03-31 00:38:02 | AHG7: Comments on metrics for extended colour volume content | E. François, C. Chevance, F. Hiron (Technicolor), D. Rusanovskyy, A. K. Ramasubramonian (Qualcomm) |
| JVET-F0090 | m40637 | 2017-03-31 00:31:23 | 2017-03-31 05:56:25 | 2017-03-31 05:56:25 | AHG8: Crosscheck for JVET-F0042 on cross-format S-PSNR-NN | C.-H. Shih, J.-L. Lin (MediaTek) |
| JVET-F0091 | m40652 | 2017-03-31 08:41:47 | 2017-03-31 08:47:33 | 2017-04-02 02:31:46 | Cross-check of JVET-F0025 on Adjusted cubemap projection for 360-degree video | V. Zakharchenko, K. P. Choi (Samsung) |
| JVET-F0092 | m40653 | 2017-03-31 08:46:09 | 2017-03-31 08:47:08 | 2017-04-04 03:55:35 | Cross-check of JVET-F0026 on Equatorial cylindrical projection for 360-degree video | V. Zakharchenko, K. P. Choi (Samsung) |
| JVET-F0093 | m40655 | 2017-03-31 10:07:54 | 2017-03-31 10:12:41 | 2017-04-02 01:24:24 | Cross-check of JVET-F0063 on AHG5: Enhanced fast algorithm of JVET-E0078 | T. Hashimoto, T. Ikai (Sharp) |
| JVET-F0094 | m40663 | 2017-04-01 03:10:24 | 2017-04-01 06:21:25 | 2017-04-01 08:41:37 | Analysis of 4k Hybrid Log-Gamma test sequences | S. Iwamura, S. Nemoto, A. Ichigaya, M. Naccari (NHK) |
| JVET-F0095 | m40668 | 2017-04-01 13:32:39 | 2017-04-02 01:13:51 | 2017-04-02 01:13:51 | AHG5: Cross check of JVET-F0044 on Performance evaluation of Adaptive Search Range on JEM-5.0.1 | P.-H. Lin, Y.-H. Ju, C.-C. Lin, Y.-J. Chang, C.-C. Lin (ITRI) |
| JVET-F0096 | m40673 | 2017-04-01 14:37:25 | 2017-04-02 01:01:25 | 2017-04-06 01:50:10 | EE2-JVET related: Division-free bilateral filter | J. Ström, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. Sjöberg (Ericsson) |
| JVET-F0097 | m40676 | 2017-04-02 04:05:34 | 2017-04-03 04:10:31 | 2017-04-03 04:10:31 | AHG8: Cross-check of JVET-F0025 Adjusted cubemap projection with tile coding for 360-degree video | Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital) |
| JVET-F0098 | m40678 | 2017-04-02 04:13:03 | 2017-04-03 04:10:54 | 2017-04-03 04:10:54 | AHG8: Cross-check of JVET-F0026 Equatorial cylindrical projection with tile coding for 360-degree video | Y. He, X. Xiu, P. Hanhart, Y. Ye (InterDigital) |
| JVET-F0099 | m40679 | 2017-04-02 04:39:59 | 2017-04-04 01:34:06 | 2017-04-04 01:34:06 | Cross-check of JVET-F0096 on EE2-JVET related: Division-free bilateral filter | L. Zhang (Qualcomm) |
| JVET-F0100 | m40689 | 2017-04-02 09:25:36 | 2017-04-03 02:24:32 | 2017-04-03 02:24:32 | EE2 related: Cross-check of JVET-F0096 Division-free bilateral filter | Z. Deng (Intel) |
| JVET-F0101 | m40691 | 2017-04-02 12:45:21 | 2017-04-03 12:11:35 | 2017-04-05 03:47:53 | EE2 related: Cross-check of division-free bilateral filter (JVET-F0096) | S.-C. Lim, J. Kang (ETRI) |
| JVET-F0102 | m40693 | 2017-04-03 00:58:26 | 2017-04-03 01:00:11 | 2017-04-05 06:35:39 | Industry recommendation for Future Video Coding Standard Development | E. Alshina (Samsung), J. Zhou (Huawei), S. Lei (MediaTek), C. Luo (China Telecom), A. Hinds (CableLabs) |
| JVET-F0103 | m40700 | 2017-04-04 02:30:26 | 2017-04-05 06:03:26 | 2017-04-06 03:59:23 | BoG report on test material and subjective assessment | T. Suzuki |
| JVET-F0104 | m40701 | 2017-04-04 02:58:17 | 2017-04-04 03:05:14 | 2017-04-04 03:05:14 | Weighted angular prediction | K. Panusopone, Y. Yu, L. Wang (Arris) |
| JVET-F0105 | m40702 | 2017-04-04 09:02:22 | 2017-04-05 08:48:53 | 2017-04-05 08:48:53 | Cross-check of Weighted angular prediction (JVET-F0104 Test 1 & Test 2) | H. M. Jang, S.-H. Kim, J. Lim (LGE) |
| JVET-F0106 | m40706 | 2017-04-04 14:28:51 | 2017-04-04 20:13:32 | 2017-04-04 20:13:32 | BoG report on 360° Video | J. Boyce |
| JVET-F0107 | m40709 | 2017-04-05 06:31:40 | 2017-04-05 06:33:17 | 2017-04-06 06:04:29 | BoG Report on Extended Colour Volume Material | A. Segall |
| JVET-F0108 | m40721 | 2017-04-06 06:13:22 | 2017-04-06 06:20:06 | 2017-04-06 07:25:26 | Padded ERP (PERP) projection format | J. Boyce, A. Tourapis, C. Fogg |
| JVET-F0130 | m40722 | 2017-04-06 06:41:38 |  |  | Withdrawn |  |
| JVET-F1000 | m40749 | 2017-04-17 22:18:05 | (this document) | (this document) | Meeting Report of the 6th JVET Meeting | G. J. Sullivan, J.-R. Ohm (JVET coordinators) |
| JVET-F1001 | m40750 | 2017-04-17 22:22:49 | 2017-04-29 01:01:31 | 2017-05-31 15:20:43 | Algorithm description of Joint Exploration Test Model 6 (JEM6) | J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce (editors) |
| JVET-F1002 | m40751 | 2017-04-17 22:24:35 | 2017-04-20 05:47:24 | 2017-04-20 05:47:24 | Joint Call for Evidence on Video Compression with Capability beyond HEVC | M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki (editors) |
| JVET-F1003 | m40752 | 2017-04-17 22:25:56 | 2017-04-28 06:24:53 | 2017-04-28 06:24:53 | Algorithm descriptions of projection format conversion and video quality metrics in 360Lib | Y. Ye, E. Alshina, J. Boyce (editors) |
| JVET-F1004 | m40739 | 2017-04-07 04:05:37 | 2017-04-07 04:06:12 | 2017-05-18 20:06:49 | Subjective testing method for comparison of 360° video projection formats using HEVC | J. Boyce, E. Alshina, Z. Deng (editors) |
| JVET-F1011 | m40720 | 2017-04-06 06:10:07 | 2017-04-06 06:10:25 | 2017-04-29 01:39:43 | Description of Exploration Experiments on Coding Tools | E. Alshina, L. Zhang (editors) |
| JVET-F1020 | m40753 | 2017-04-17 22:30:21 | 2017-06-19 23:44:49 | 2017-06-19 23:44:49 | JVET common test conditions and evaluation procedures for HDR/WCG video | A. Segall, E. François, D. Rusanovskyy (editors) |
| JVET-F1030 | m40725 | 2017-04-06 07:27:01 | 2017-04-06 09:16:35 | 2017-05-17 05:37:20 | JVET common test conditions and evaluation procedures for 360° video | E. Alshina, J. Boyce, A. Abbas, Y. Ye (editors) |

# Annex B to JVET report:
List of meeting participants

The participants of the sixth meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately 138 people in total), were as follows:

- Anne Aaron (Netflix)

- Adeel Abbas (GoPro)

- Kiyofumi Abe (Panasonic)

- Yong-Jo Ahn (Kwangwoon Univ.)

- Elena Alshina (Samsung Electronics)

- Alireza Aminlou (Nokia)

- Jicheng An (HiSilicon)

- Kenneth Andersson (LM Ericsson)

- Aram Baek (Hanbat Univ.)

- Yukihiro Bandoh (NTT)

- Gun Bang (ETRI)

- Vittorio Baroncini (GBTech)

- Guillaume Barroux (Fujitsu Labs)

- Jill Boyce (Intel)

- Benjamin Bross (Fraunhofer HHI)

- Soo-Ik Chae (Seoul Natl. Univ.)

- Eric (Chi W.) Chai (Real Commun.)

- Yao-Jen Chang (ITRI Intl.)

- Jianle Chen (Qualcomm)

- Jie Chen (Samsung)

- Yi-Jen Chiu (Intel)

- Kiho Choi (Samsung Electronics)

- Hsiao-Chiang Chuang (Qualcomm Tech.)

- Tzu-Der Chuang (MediaTek)

- Olena Chubach (RWTH Aachen Univ.)

- Muhammed Coban (Qualcomm)

- Zhipin Deng (Intel)

- Virginie Drugeon (Panasonic)

- Amith DSouza (Samsung)

- Xavier Ducloux (Harmonic)

- David Flynn (Blackberry)

- Edouard François (Technicolor)

- Alexandre Gabriel (TNO)

- Dae-hyeok Gwon (Hanbat Univ.)

- Ryoji Hashimoto (Renesas)

- Yuwen He (InterDigital Commun.)

- Cornelius Hellge (Fraunhofer HHI)

- Seungwook Hong (Arris)

- Yu-Wen Huang (MediaTek)

- Atsuro Ichigaya (NHK)

- Tomohiro Ikai (Sharp)

- Masaru Ikeda (Sony)

- Shunsuke Iwamura (NHK)

- Euee S. Jang (Hanyang Univ.)

- Hyeongmoon Jang (LG Electronics)

- Wonkap Jang (Vidyo)

- Byeungwoo Jeon (Sungkyunkwan Univ.)

- Rajan Joshi (Qualcomm)

- Jung Won Kang (ETRI)

- Ryuichi Kanoh (Panasonic)

- Kei Kawamura (KDDI)

- Michel Kerdranvat (Technicolor)

- Hyosong Kim (Pixtree)

- Jae-Gon Kim (Korea Aerosp. Univ.)

- Nam-Uk Kim (Sejong Univ. DMS Lab)

- Seung-Hwan Kim (LG Electronics)

- Bae-Keun Lee (KT)

- Jinho Lee (ETRI)

- Sunyoung Lee (Pixtree)

- Shawmin Lei (MediaTek)

- Ling Li (LG Electronics)

- Xiang Li (Qualcomm Tech.)

- Ru Ling Liao (Panasonic)

- Chongsoon Lim (Panasonic)

- Jaehyun Lim (LG Electronics)

- Jeongyeon Lim (SK Telecom)

- Ching-Chieh Lin (ITRI Intl.)

- Jian-Liang Lin (MediaTek)

- Po Han Lin (ITRI Intl.)

- Victorien Lorcy (B-Com)

- Ajay Luthra (Arris)

- Akira Minezawa (Mitsubishi Electric)

- Kiran Misra (Sharp)

- Karsten Müller (Fraunhofer HHI)

- Ohji Nakagami (Sony)

- Shimpei Nemoto (NHK)

- Andrey Norkin (Netflix)

- Hyun Mook Oh (LG Electronics)

- Jens-Rainer Ohm (RWTH Aachen Univ.)

- Patrice Onno (Canon Research France)

- Jun Ono (Socionext)

- Jörn Ostermann (Univ. Hannover)

- Krit Panusopone (Arris)

- Dohyeon Park (Korea Aerosp. Univ.)

- Seanae Park (Kwangwoon Univ.)

- Martin Pettersson (Ericsson)

- Pierrick Philippe (Orange Labs FT)

- Shouxing Qu (Blackberry)

- Fabien Racape (Technicolor)

- Adarsh Krishnan Ramasubramonian (Qualcomm Tech.)

- Justin Ridge (Nokia)

- Christopher Rosewarne (CiSRA / Canon)

- Yago Sanchez De La Fuente (Fraunhofer HHI)

- Eiichi Sasaki (Socionext)

- Andrew Segall (Sharp)

- Vadim Seregin (Qualcomm)

- Cheng-Hsuan Shih (MediaTek)

- Masato Shima (Canon)

- Jae-Seob Shin (Pixtree)

- Donggyu Sim (Kwangwoon Univ.)

- Anubhav Singh (Samsung)

- Robert Skupin (Fraunhofer HHI)

- Sehoon Son (Pixtree)

- Jacob Ström (Ericsson)

- Kazuo Sugimoto (Mitsubishi Electric)

- Jong-Yeul Suh (LG Electronics)

- Gary Sullivan (Microsoft)

- Yu-Chen Sun (Qualcomm Tech.)

- Yule Sun (Zhejiang Univ.)

- Teruhiko Suzuki (Sony)

- Maxim Sychev (Huawei Tech.)

- Han Boon Teo (Panasonic)

- Uday Thakur (RWTH Aachen Univ.)

- Herbert Thoma (Fraunhofer IIS)

- Emmanuel Thomas (TNO)

- Tadamasa Toma (Panasonic)

- Alexandros Tourapis (Apple)

- Yi-Shin Tung (ITRI USA / MStar Semi.)

- Fabrice Urban (Technicolor)

- Geert Van der Auwera (Qualcomm)

- Ye-Kui Wang (Qualcomm)

- Stephan Wenger (Vidyo)

- Haitao Yang (Huawei Tech.)

- Jar-Ferr (Kevin) Yang (National Cheng Kung Univ.)

- Ji Yao (Fujitsu R&D Center)

- Yukinobu Yasugi (Sharp)

- Yan Ye (InterDigital Commun.)

- Peng Yin (Dolby Labs)

- Haoping Yu (Futurewei / Huawei R&D USA)

- Lu Yu (Zhejiang Univ.)

- Chuanyi Zhang (OwlReality)

- Li Zhang (Qualcomm Tech.)

- Wenhao Zhang (Hulu)

- Xin Zhao (Qualcomm Tech.)

- Zhijie Zhao (Huawei Tech.)

- Jianhua Zheng (Huawei Tech.)

- Xiaozhen Zheng (DJI)

- Jian Qing Zhu (Fujitsu R&D Center)