| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0025 |
| --- | --- |

| Title: | AHG8: Adjusted cubemap projection for 360-degree video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | [+1 858 658 3973] mcoban@qti.qualcomm.com, geertv@qti.qualcomm.com, martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an adjusted cubemap projection for coding of 360-degree video. The proposed projection targets uniform sphere sampling with limited texture deformation. It uses a simple function to perturb the cubemap lookup vector. The proposed projection can reuse existing cubemap hardware lookup for efficient rendering. The proposed scheme provides -11.3% RA coding gain (E2E WS-PSNR).

# Proposal

The cubemap projection offers relatively simple projection scheme resulting in 6 square faces that are suitable for 2D frame packing.  However, the cubemap projection non-uniformly samples the sphere resulting in inefficient representation of 360-degree video for coding [1][2]. The regions closer to the cube face edges are more densely sampled as shown in Figure 1.

Figure 1. Cubemap sampling.

The cubemap lookup vectors can be modified to approximate uniform sphere sampling for addressing the non-uniform sampling problem while preserving the packing scheme. A function given by

| f(x)=sgn(x)0.34-0.342-0.09x0.18 | (1) |
| --- | --- |

is used to modify the lookup vectors in the forward 2D to 3D conversion. For the inverse 3D to 2D conversion, its inverse function

| g(x)=sgn(x)(-0.36x2+1.36xx) | (2) |
| --- | --- |

is used to modify the lookup vectors. The mapping functions are illustrated in Figure 2

Figure 2. Forward and inverse mapping functions

The mapping enlarges the center of the cubemap with respect to the cubemap projection. Figure 2 shows cubemap projection (CMP) and the adjusted cubemap projection (ACMP) applied to PoleVault sequence.

Figure 3. (a) 3x2 Cubemap projection (CMP), (b) 3x2 Adjusted cubemap projection (ACMP).

| (a) | (b) |
| --- | --- |

# Adjusted cubemap projection (ACMP) format conversion

The adjusted cubemap projection has 6 square faces in total, labelled as PX, PY, PZ, NX, NY, NZ (with “P” standing for “positive” and “N” standing for “negative”), in Figure 3. Table 1 specifies the face index values corresponding to each of the six cubemap faces [3].

The uv plane definition for each cubemap face is shown in Figure 3. Each face in the uv plane is a 2x2 square, with u and v being defined in the range of [−1, 1].

Figure 4. Coordinates definition for cubemap

Table 1. Face index of ACMP

| Face index | Face label | Notes |
| --- | --- | --- |
| 0 | PX | Front face with positive X axis value |
| 1 | NX | Back face with negative X axis value |
| 2 | PY | Top face with positive Y axis value |
| 3 | NY | Bottom face with negative Y axis value |
| 4 | PZ | Right face with positive Z axis value |
| 5 | NZ | Left face with negative Z axis value |

Denote the dimension of any square face as A×A. For 2D-to-3D coordinate conversion, given the position (m, n) on a given face f, (u’, v’) is first calculated as:

| u’ = (m + 0.5) * 2 / A − 1, 0≤ m <A | (3) |
| --- | --- |
| v’ = (n + 0.5) * 2 / A − 1, 0 ≤ n < A | (4) |

The (u,v) values are adjusted to perturb the lookup vector by using Equation (1):

| u=sgn(u')0.34-0.342-0.09u'0.18 | (5) |
| --- | --- |
| v=sgn(v')0.34-0.342-0.09v'0.18 | (6) |

Then, the 3D coordinates (X, Y, Z) as defined in [3] are derived using Table 2 given the position (u’, v’) and the face index f.

Table 2. (X, Y, Z) derivation given (u, v) and the face index f

| f | X | Y | Z |
| --- | --- | --- | --- |
| 0 | 1.0 | −v | −u |
| 1 | −1.0 | −v | u |
| 2 | u | 1.0 | v |
| 3 | u | −1.0 | −v |
| 4 | u | −v | 1.0 |
| 5 | −u | −v | −1.0 |

For 3D-to-2D coordinate conversion, given (X, Y, Z), the (u’, v’) and face index f is calculated per Table 3.

Table 3. Derivation of (u’, v’) and the face index f given (X, Y, Z)

| Condition |  | f | u’ | v’ |
| --- | --- | --- | --- | --- |
| ¦X¦ ≥ ¦Y¦ and ¦X¦ ≥ ¦Z¦ and X >0 |  | 0 | −Z/¦X¦ | −Y/¦X¦ |
| ¦X¦ ≥ ¦Y¦ and ¦X¦ ≥ ¦Z¦ and X <0 |  | 1 | Z/¦X¦ | −Y/¦X¦ |
| ¦Y¦ ≥ ¦X¦ and ¦Y¦ ≥ ¦Z¦ and Y >0 |  | 2 | X/¦Y¦ | Z/¦Y¦ |
| ¦Y¦ ≥ ¦X¦ and ¦Y¦ ≥ ¦Z¦ and Y <0 |  | 3 | X/¦Y¦ | −Z/¦Y¦ |
| ¦Z¦ ≥ ¦X¦ and ¦Z¦ ≥ ¦X¦ and Z >0 |  | 4 | X/¦Z¦ | −Y/¦Z¦ |
| ¦Z¦ ≥ ¦X¦ and ¦Z¦ ≥ ¦Y¦ and Z <0 |  | 5 | −X/¦Z¦ | −Y/¦Z¦ |

The final (u,v) values obtained by adjusting (u’,v’) by

| u=sgnu'(-0.36u'2+1.36u'u'), | (7) |
| --- | --- |
| v=sgn(v')(-0.36v'2+1.36v'v'). | (8) |

Then, (m, n) on the face f is calculated by solving the following equations,

| u = (m + 0.5) * 2 / A − 1, 0≤ m <A | (9) |
| --- | --- |
| v = (n + 0.5) * 2 / A − 1, 0 ≤ n < A. | (10) |

# Coding results

The proposed adjusted cubemap projection was implemented into the JVET 360Lib-2.1 [4] library for evaluation. Per common test conditions described in [5], the random access (RA) coding gain of ACMP with 3×2 packing is enumerated per sequence in Table 4. The ACMP face sizes are set equal to cube map face sizes in [5]. Frame sizes are 3552×2368 for 8K test sequences and 2880×1920 for 4K.

| ACMP vs. ERP |  |  | SPSNR-NN (End to End) |  |  | SPSNR-I (End to End) |  |  | CPP-PSNR (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FW | FH | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 1184 | 1184 | -7.5% | 2.9% | -1.5% | -7.8% | 2.8% | -1.6% | -7.8% | 2.8% | -1.6% | -7.6% | 2.9% | -1.6% |
| GasLamp | 1184 | 1184 | -4.3% | 2.2% | 7.3% | -4.4% | 1.9% | 7.1% | -4.3% | 2.0% | 7.1% | -4.2% | 2.2% | 7.3% |
| Skateboarding_in_lot | 1184 | 1184 | -17.7% | -16.7% | -17.6% | -17.8% | -16.9% | -17.8% | -17.7% | -16.8% | -17.8% | -17.6% | -16.6% | -17.7% |
| Chairlift | 1184 | 1184 | -25.2% | -18.3% | -18.8% | -25.2% | -18.4% | -18.9% | -25.3% | -18.4% | -18.9% | -25.2% | -18.3% | -18.9% |
| KiteFlite | 1184 | 1184 | -9.8% | -3.1% | -4.6% | -10.1% | -3.3% | -4.8% | -9.9% | -3.3% | -4.7% | -9.8% | -3.1% | -4.6% |
| Harbor | 1184 | 1184 | -9.2% | -3.4% | -2.0% | -9.4% | -3.5% | -2.1% | -9.7% | -3.5% | -2.1% | -9.5% | -3.4% | -2.0% |
| PoleVault | 960 | 960 | -9.6% | -9.6% | -9.8% | -9.2% | -9.5% | -9.8% | -9.2% | -9.6% | -9.9% | -9.6% | -9.6% | -9.8% |
| AerialCity | 960 | 960 | -3.1% | -2.0% | -1.7% | -3.3% | -2.3% | -2.0% | -3.2% | -2.3% | -2.1% | -3.0% | -2.0% | -1.8% |
| DrivingInCity | 960 | 960 | -4.9% | 3.8% | 3.7% | -4.3% | 3.4% | 3.3% | -4.1% | 3.5% | 3.4% | -4.8% | 3.8% | 3.8% |
| DrivingInCountry | 960 | 960 | -21.3% | -18.4% | -19.2% | -20.6% | -18.6% | -19.4% | -20.6% | -18.6% | -19.3% | -21.3% | -18.3% | -19.1% |
| Overall |  |  | -11.3% | -6.3% | -6.4% | -11.2% | -6.4% | -6.6% | -11.2% | -6.4% | -6.6% | -11.3% | -6.3% | -6.4% |

Table 4. BD-rates for RA per common test conditions for 360-degree video.

# Conclusions

The adjusted cubemap projection with 3×2 frame packing was proposed. The proposed projection targets uniform sphere sampling with limited texture deformation. The reported coding gain is -11.3% (E2E WS-PSNR) for RA conditions. It is suggested to include ACMP into the next version of 360Lib for further exploration of 360-degree video coding.

# References

- R. Lerbour, J.-E. Marvie, P. Gautron, “Adaptive real-time rendering of planetary terrains”, Proc. Int. Conf. Computer Graphics, Visualization and Computer Vision (WSCG), Feb. 2010, pp 89-96.

- Chan, F.K. and O'Neill, “Feasibility Study of a Quadrilateralized Spherical Cube Earth Data Base”, Computer Sciences Corp., EPRF Tech. Report 2-75. Prepared for the Environmental Prediction Research Facility, Monterey, Calif., 1975.

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib”, JVET-E1003, Jan. 2017.

- 360Lib-2.1: https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-2.1/

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360-degree video”, JVET-E1030.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).