| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0026 |
| --- | --- |

| Title: | AHG8: Equatorial cylindrical projection for 360-degree video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Muhammed Coban, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | geertv@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The equatorial cylindrical projection (ECP) is proposed for efficient compression of 360-degree video:

- projecting the equatorial region of the sphere using the Lambert cylindrical equal-area projection,

- projecting the poles of the sphere onto square faces in the video frame to avoid inactive pixels.

The RA coding gain is -11.44% (e2e WS-PSNR) compared with the equirectangular projection in accordance with the JVET common test conditions for 360-degree video.

# Proposal

It is proposed to apply the Lambert cylindrical equal-area projection to the equatorial sphere region as illustrated in Figure 1a. The latitude interval of the equatorial region is ±sin-12/3≈±41.81°±sin-123≈±41.81°±sin-123≈±41.81°±sin-123≈±41.81°±sin-123≈±41.81°, which is chosen such that the equatorial region is 2/3 of the total sphere area, hence, one pole area is 1/6 of the sphere area. Similarly, the number of samples utilized by the equatorial region when mapped to square faces is equal to 2/3 of the video frame packed as in Figure 2b. The number of samples of the video frame that are utilized by both pole segments is equal to 1/3 if the pole discs are mapped to square faces as illustrated in Figure 1b. This way, all available pixels are utilized in the video frame, hence, there are no inactive samples in the packed video frame that would require additional decoder complexity. Section 3 formulates the conversion equations involved in the proposed equatorial cylindrical projection (ECP).

The equatorial and pole faces are arranged 3×2 into a video frame as illustrated in Figure 2b. The pole faces are numbered 0 and 1, while the equatorial faces are numbered 2 to 5. Figure 2a is a frame in the equirectangular projection (ERP) format with corresponding faces indicated. In Figure 2b, the top row consists of the first three equatorial faces (2-4) and the bottom row contains the rotated 5th equatorial face in between the rotated square pole faces. Figure 3 illustrates the packing with a frame from the ‘PoleVault’ sequence.

| (a) |  |  | (b) |
| --- | --- | --- | --- |

Figure 1. (a) Mapping of equatorial sphere region onto cylinder. (b) Disc ↔ square mapping.

| (a) | (b) |
| --- | --- |

Figure 2. (a) ERP partitioning into equatorial and pole regions. (b) 3x2 packing after ECP projection.

Figure 3. 3×2 packed frame from ‘PoleVault’ sequence.

# Coding Results

The proposed equatorial cylindrical projection was implemented into the JVET 360Lib-2.1 [3] library for evaluation. Per common test conditions described in [2], the random access (RA) coding gain of ECP and 3×2 packing is enumerated per sequence in Table 1. The ECP face sizes are set equal to cube map face sizes in [2]. Frame sizes are 3552×2368 for 8K test sequences and 2880×1920 for 4K.

| Proposed vs. ERP |  |  | SPSNR-NN (End to End) |  |  | SPSNR-I (End to End) |  |  | CPP-PSNR (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Face W | Face H | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 1184 | 1184 | -7.6% | 2.5% | -2.5% | -7.8% | 2.2% | -2.7% | -7.7% | 2.3% | -2.7% | -7.6% | 2.4% | -2.7% |
| GasLamp | 1184 | 1184 | -7.8% | -1.0% | 2.7% | -7.9% | -1.3% | 2.5% | -7.9% | -1.3% | 2.4% | -7.8% | -1.2% | 2.6% |
| Skateboarding_in_lot | 1184 | 1184 | -18.3% | -19.7% | -19.4% | -18.5% | -19.9% | -19.6% | -18.3% | -19.8% | -19.6% | -18.2% | -19.7% | -19.5% |
| Chairlift | 1184 | 1184 | -22.0% | -14.2% | -14.5% | -22.1% | -14.3% | -14.6% | -22.1% | -14.3% | -14.7% | -22.0% | -14.2% | -14.6% |
| KiteFlite | 1184 | 1184 | -8.2% | -2.4% | -3.8% | -8.3% | -2.6% | -4.0% | -8.2% | -2.6% | -3.9% | -8.1% | -2.5% | -3.8% |
| Harbor | 1184 | 1184 | -11.4% | -3.9% | -4.1% | -11.7% | -4.1% | -4.4% | -11.8% | -4.2% | -4.5% | -11.6% | -4.1% | -4.3% |
| PoleVault | 960 | 960 | -9.0% | -10.8% | -10.7% | -8.3% | -10.8% | -10.8% | -8.3% | -10.8% | -11.0% | -9.0% | -10.9% | -10.9% |
| AerialCity | 960 | 960 | -4.4% | -3.5% | -2.8% | -4.7% | -3.9% | -3.2% | -4.6% | -3.9% | -3.3% | -4.4% | -3.6% | -2.9% |
| DrivingInCity | 960 | 960 | -4.8% | 6.0% | 4.8% | -4.1% | 5.3% | 4.1% | -3.9% | 5.3% | 4.2% | -4.7% | 6.0% | 4.8% |
| DrivingInCountry | 960 | 960 | -21.0% | -16.0% | -18.7% | -20.0% | -16.5% | -19.0% | -20.0% | -16.5% | -19.0% | -21.0% | -16.0% | -18.7% |
| Overall |  |  | -11.5% | -6.3% | -6.9% | -11.3% | -6.6% | -7.2% | -11.3% | -6.6% | -7.2% | -11.4% | -6.4% | -7.0% |

Table 1. BD-rates for RA per common test conditions for 360-degree video.

# Conversion Equations

## 2D-to-3D Conversion

Following the descriptions in [1], denote the dimension of one face as A×A. In 2D-to-3D conversion, a point (m, n) on the face is mapped to a point on the sphere (ϕ, θ) per equations below, depending on the face it belongs to in Figure 2b.

First, the point (x, y) is determined as follows:

x=2Am+12-1

y=2An+12-1

Followed by square-to-disc mapping [4]:

s=x2+y2-x2y2x2+y2

u=A2sx

v=A2sy

For the face f = 0, the following applies:

ϕ=tan-1uv

θ=π2-2rAπ2-sin-123

with r=u2+v2, θ∈sin-123,π2, ϕ∈-π,π.

For the face f = 1, the following applies:

ϕ=tan-1uv+π2

θ=2rAπ2-sin-123-π2

where r=u2+v2 and θ∈-π2,-sin-123, ϕ∈-π,π.

For the square faces with face index f, f = 2…5, the following applies:

ϕ=π2m+12A+f-2

θ=sin-1231-2An+12

with yaw in the range ϕ∈-π+f-2π2,-π2+f-2π2 depending on f, and pitch in the range θ∈-sin-123,sin-123.

## 3D-to-2D Conversion

Following descriptions in [1], a point on the sphere (ϕ, θ) is mapped to a point (m, n) on the face with dimension A×A depending on the face index f per equations below.

For the face f = 0, θ∈sin-123,π2, ϕ∈-π,π, the following applies:

u=π2-θsinϕπ2-sin-123

v=π2-θcosϕπ2-sin-123

Followed by disc-to-square mapping [4]:

w=sgnuv2u2+v2-u2+v2u2+v2-4u2v2

x,y=wv,wu  if w>0u,v,   otherwise

with sgn the sign function. Finally, (m, n) are given by:

m=A2x+1-12

n=A2y+1-12

For the face f = 1, θ∈-π2,-sin-123, ϕ∈-π,π, the following applies:

u=π2+θsinϕπ2-sin-123

v=-π2+θcosϕπ2-sin-123

Followed by disc-to-square mapping:

w=sgnuv2u2+v2-u2+v2u2+v2-4u2v2

x,y=wv,wu  if w>0u,v,   otherwise

Finally, (m, n) are given by:

m=A2x+1-12

n=A2y+1-12

For the square faces with face index f, f = 2…5, yaw in the range ϕ∈-π+f-2π2,-π2+f-2π2 depending on f, and pitch in the range θ∈-sin-123,sin-123, the following applies:

m=2ϕπA+4-fA-12

n=A21-32sinθ-12

# Conclusion

The equatorial cylindrical projection with 3×2 frame packing was proposed. The equatorial region of the sphere is projected using the Lambert cylindrical equal-area projection and the pole regions are projected onto squares. The reported coding gain is -11.44% (e2e WS-PSNR) for RA conditions. It is suggested to include ECP into the next version of 360Lib for further exploration of 360-degree video coding.

# References

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib”, JVET-E1003, Jan. 2017.

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360-degree video”, JVET-E1030.

- 360Lib-2.1: https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-2.1/

- M. Fernandez-Gausti, “Classroom notes: Analytic geometry of some rectilinear figures”, Int. J. Mathematical Education in Science and Technology, Vol. 23, No. 6, 1992, pp. 895–913.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).