| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 3 March – 7 April 2017 | Document: JVET-F0029 |
| --- | --- |

| Title: | On memory bandwidth of JEM5.0 tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E.Alshina Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

Memory bandwidth information for several JEM tools compared to HEVC MC is summarized in this contribution. The power consumption is key complexity metric for mobile devices manufacturers. Since there is very clear correlation between power and memory bandwidth this complexity metric shall be carefully considered during next generation video coding standard development. Currently the worst case memory usage in JEM is 15 of HEVC.

# Introduction

HEVC standard uses 8-taps interpolation filter for Luma and 4-taps filter for Chroma motion compensation. Bi-prediction is restricted for blocks smaller than 88. MC for small blocks 44 is not allowed in HEVC. With those restrictions memory bandwidth of HEVC  was considered to be acceptable at the time of standard development.

For memory bandwidth assessment special addition for the reference SW was developed and widely exploited by multiple CEs during HEVC and its extension development. Example of this SW can be found in [1]. Based on results on provided by that tool the HEVC memory bandwidth compared to AVC/H.264 expected to be~15% higher. This estimation is very close to the actual power consummation test results. One of those power consumption tests was published in [2]. Figure 1 is borrowed from this PCS2016 paper.

Fig. 1 also provides energy consumption by H.263 (4 lower than HEVC) and VP9 (2 higher than HEVC) decoders.

Authors of [2] proposed very interesting methodology for energy consumption estimation based on bit-stream features analysis. Results of their method wonderfully correlate with actual measurement. Using methodology proposed in [2] one can estimate an impact to the power consumption of individual tools in a codec, As Fig.1 shows the majority of battery is eaten during motion compensation.

There is very clear correlation between power consumption which is key parameter for the complexity estimation for mobile devices manufacturers and memory bandwidth. This complexity metric shall be carefully considered during next generation video coding standard development.

Fig. 1 Measured and estimated decoding energies (FFmpeg).

# JEM MC tools

Several new MC tools were added to the JEM on top of HEVC [3]. Among them:

  - Sub-PU level motion vector prediction

  - Locally adaptive motion vector resolution (AMVR)

  - 1/16 pel motion vector storage accuracy

  - Overlapped block motion compensation (OBMC)

  - Local illumination compensation (LIC)

  - Affine motion prediction

  - Pattern matched motion vector derivation (FRUC)

  - Bi-directional optical flow (BIO)

  - Decoder-side motion vector refinement (DMVR)

Also, as a part of Sub-PU level motion vector prediction motion field compression (the part of HEVC standard) is disabled in JEM.

If at least one of the following JEM tools:

  - Quadtree plus binary tree (QTBT) block structure

  - Sub-PU level motion vector prediction

  - Overlapped block motion compensation (OBMC)

  - Affine motion prediction

  - Pattern matched motion vector derivation (FRUC)

is enabled then minimal size of Luma block bi-prediction happens is 44. This would be the worst case from the view point of the memory band-width.

Suppose we are doing bi-prediction for WH block. If T-taps interpolation filter is used then decoder needs to access (W+T-1)(H+T-1) memory on each reference frames, so total memory access is 2 (W+T-1)(H+T-1). For HEVC and most of the tools in JEM (Affine MC for example) T=8.

For BIO in JEM 5.0 predication needs to be calculated in (W+4)(H+4) area for WH block prediction, so the number of samples to access 2 (W+T-1+4)(H+T-1+4). The variant of BIO under study in EE3 [4] doesn’t need block extension so memory access is the same as regular bi-directional MC (W+T-1)(H+T-1).

For DMVR in JEM 5.0 predication needs to be calculated in (W+2)(H+2) area for WH block prediction, so the number of samples to access 2 (W+T-1+2)(H+T-1+2) with T=8.

For MV candidates estimation stage of FRUC 2-taps filter is used, but for template matching prediction is calculated for (W+4)(H) or (W)(H+4) area (let’s assume vertical extension for the simplicity).  In total up to 17 MV candidates are estimated for sub-block in FRUC. The best is selected and for final prediction 8-taps interpolation filter is used. So total memory access is 162 (W+2-1)(H+2-1+4) + 2 (W+8-1)(H+8-1).

Those numbers are summarised in Table 1. Also memory access per one Luma sample prediction is calculated.

Both BIO and DMVR do not require the split of the block to small (up to 44) sub-blocks. The reason they are applied for 44 is the combination with other JEM tools. So for those algorithms memory access in case of 44 is marked with *. For Affine MC, OBMC and FRUC splitting to 44 sub-blocks is part of the algorithms.

Table 1. Memory access for bi-directional MC.

| Block size | HEVC MC | Affine MC ¦¦OBMC | BIO (EE3) | DMVR | BIO JEM | FRUC |
| --- | --- | --- | --- | --- | --- | --- |
| 88 | (8+7) (8+7). | (8+7) (8+7) | (8+7) (8+7) | (8+2+7) (8+2+7) | (8+4+7) (8+4+7). | 16(8+4+1)(8+1) + (8+7)(8+7) |
| 44 | - | (4+7) (4+7) | (4+7) (4+7)* | (4+2+7) (4+2+7)* | (4+4+7) (4+4+7)* | 16(4+4+1)(4+1) + (4+7)(4+7) |
| Per sample (the worst case) |  |  |  |  |  |  |
| 88 | 3.5 | 3.5 | 3.5 | 4.5 | 5.6 | 32.8 |
| 44 | - | 7.6 | 7.6* | 10.6* | 14.1* | 52.6 |
| The worst case |  |  |  |  |  |  |
|  | 3.5 | 7.6 | 7.6* | 10.6* | 14.1* | 52.6 |

From Table 1 one can compute that memory the worst case memory usage in JEM is 15 of HEVC. FRUC makes the major impact to the worst case of memory usage.

Restriction of bi-prediction in blocks smaller than 88, disabling FRUC (or careful restriction of MV candidates considered) and redesigning BIO as suggested in EE3 bring the worst case memory bandwidth to 128% of HEVC (this might be reasonable trade-off).

Excel template attached to this contribution contains all calculations above and can use for memory access estimation for different combinations of JEM tools.

# Conclusions

There is clear correlation between power consumption during decoding and memory bandwidth. Several tools for memory bandwidth estimation are available and provide accurate enough results. It is recommended to use them during next generation video coding standard development and pay special attention to the power consumption as one of the most important for mobile device manufacturers complexity metric, Currently the worst case of memory usage in JEM is 15 of HEVC.

# References

[1] JCTVC-L0440, E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440

[2] Christian Herglotz, Yongjun Wen, Bowen Dai and Matthias Kränzler (Friedrich-Alexander University Erlangen-Nürnberg (FAU), Germany); Andre Kaup (University of Erlangen-Nuremberg, Germany) A Bitstream Feature Based Model for Video Decoding Energy Estimation, PCS 2016, Nuremberg, Dec 3-7, 2016

[3] J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 3 (JEM3),” JVET-C1001, May. 2016, Geneva

[4] A.Alshin, E. Alshina “EE3: BIO w/o block extension,” JVET-E0028, Jan. 2017, Geneva

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).