| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0038_r1 |
| --- | --- |

| Title: | AHG8: adaptive QP for 360 video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fabien Racapé Franck Galpin Gagan Rath Edouard François | Email: | fabien.racape@technicolor.com franck.galpin@technicolor.com gagan.rath@technicolor.com edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution presents the results of encoding ERP sequences with a vertically adapted QP. On HM-16.15, it is reported that this method brings, for instance, 4.3%, 2.9% coding gains in RA main 10, AI 1 frame main 10 configurations, w.r.t WS-PSNR. It is then proposed to include such adaptive QP when generating the anchors for a fair comparison between the different proposed projections.

# Background

In HM and JEM, spatially adaptive QP can be activated at the encoder side. Currently, it is not activated in Common Test Conditions.

# In the following, the adaptation is described with the equi-rectangular projection (ERP) but the method can be adapted to other projections.

The relationship between the Cartesian co-ordinates (normalized) on the XY-plane and the angular co-ordinates on the sphere is given as:

y = φπ, -0.5 <= y <= 0.5, -π2 <= φ <= π2

x =θ2π, 0 ≤ x ≤ 1, 0 ≤ θ ≤ 2π

Let Δ0 denote the sampling interval for φ = 0 (or y = 0). Then, the sampling interval Δ, is defined by

Δ(φ)= Δ0 cosφ

Please note that this relation is use for weighting the MSE when calculating the WS-PSNR.

If Qstep denotes the quantization error, the average quantization error is equal to its variance, which can be denoted by σ2, defined by

σ2 = (1/3)*Qstep2

Therefore, to have similar distortion at φ=d, Qstep must be increased by the factor 1cosφ ,which can be also expressed as a function of the vertical coordinate: √(1cosπy).

In HEVC and in JEM, the quantization step size is determined from the QP parameter using the following relationship:

QstepQP= 2QP-46

where QP can take 52 values from 0 to 51.  The above relationship can be equivalently expressed as:

QP=4+6*log2(Qstep)

From this, if the Qstep needs to be increased by any factor M, this requires that the QP is increased by 6*log2M. Hence to increase the quantization step size by the factor 1cosπy,  QP must be increased by 3* log2(1cosπy). Therefore, the QP can be expressed as a function of y as:

QPy=QP0-3* log2cosπy

where QP0 denotes the base QP chosen at y=0. First, this equation will increase the QP to indefinite values as the logarithmic function will decrease boundlessly for y→ ±π2. QP must then be bounded to its maximum value: 51. Secondly, the QP has an integral value. Therefore, the above expression needs to be modified as:

QP(x,y)=min⁡(QP0-3*log2(cosπy), QPmax).

The delta QP feature has been activated with a target QP computed from the method described above, at CU level. The center of the CU is considered for calculating the current vertical position. Figure 1 illustrates the variation of the QP on the sequence AerialCity, with a base QP of 27.

Figure 1: vertically adaptive QP on the sequence AerialCity with base QP0=27

# Experimental results

The experiments have been carried using HM-16.15 and 360Lib-2.1 for the metrics generation.

Table 1 and Table 2 present resulting bd-rate gains in random access and intra (first frame only), respectively.

The test set of sequences has been corrected in the revision 1, including Trolley and GasLamp, to comply with the test conditions described in JVET-E1030.

| AQ_ERP vs. ANCHOR_ERP |  |  |  |  |  | S-PSNR-NN |  |  |  | WS-PSNR |  |  |  | S-PSNR-I |  |  |  | CPP-PSNR |  |  |  | PSNR(Encoder) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FW | FH | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 4096 | 2048 | -1.0% | -2.2% | -3.9% | -1.1% | -2.2% | -3.9% | -1.2% | -2.2% | -3.9% | -1.3% | -2.2% | -3.9% | 11.2% | 10.6% | 4.2% |
| GasLamp | 4096 | 2048 | -1.6% | -1.0% | 2.9% | -1.7% | -1.0% | 2.9% | -1.6% | -1.0% | 2.9% | -1.7% | -1.0% | 2.9% | 11.0% | 13.9% | 24.5% |
| Skateboarding_in_lot | 4096 | 2048 | -8.6% | -12.7% | -10.5% | -8.5% | -12.7% | -10.5% | -8.3% | -12.7% | -10.4% | -8.3% | -12.7% | -10.4% | 9.8% | -6.0% | 0.9% |
| Chairlift | 4096 | 2048 | -8.9% | -8.2% | -9.0% | -8.9% | -8.2% | -9.0% | -8.8% | -8.1% | -8.9% | -8.7% | -8.1% | -8.9% | 15.9% | 14.3% | 9.5% |
| KiteFlite | 4096 | 2048 | -2.8% | -2.8% | -2.2% | -2.9% | -2.9% | -2.3% | -3.0% | -2.8% | -2.2% | -3.1% | -2.9% | -2.3% | 14.5% | 15.0% | 14.8% |
| Harbor | 4096 | 2048 | -0.8% | -2.3% | -2.2% | -0.9% | -2.3% | -2.2% | -0.8% | -2.3% | -2.1% | -0.9% | -2.3% | -2.1% | 18.7% | 15.4% | 13.6% |
| PoleVault | 3840 | 1920 | -6.4% | -7.0% | -7.3% | -6.4% | -7.0% | -7.4% | -6.4% | -7.0% | -7.4% | -6.5% | -7.0% | -7.5% | 13.0% | 10.4% | 8.4% |
| AerialCity | 3840 | 1920 | -4.1% | -4.6% | -4.3% | -4.1% | -4.6% | -4.4% | -4.2% | -4.6% | -4.3% | -4.2% | -4.7% | -4.3% | 11.9% | 12.7% | 12.4% |
| DrivingInCity | 3840 | 1920 | 0.0% | 3.8% | 3.9% | 0.0% | 3.9% | 3.9% | 0.0% | 3.9% | 4.0% | 0.0% | 3.9% | 4.0% | 6.3% | 18.1% | 19.8% |
| DrivingInCountry | 3840 | 1920 | -9.0% | -10.8% | -11.5% | -9.0% | -10.9% | -11.6% | -8.9% | -10.9% | -11.5% | -9.0% | -11.0% | -11.6% | 8.6% | 0.3% | 1.9% |
| Overall |  |  | -4.3% | -4.8% | -4.4% | -4.4% | -4.8% | -4.4% | -4.3% | -4.8% | -4.4% | -4.4% | -4.8% | -4.4% | 12.1% | 10.5% | 11.0% |

Table 1: results in random access

Table 2: results in intra (1st frame)

| AQ_ERP vs. ANCHOR_ERP |  |  | S-PSNR-NN |  |  | WS-PSNR |  |  | S-PSNR-I |  |  | CPP-PSNR |  |  | PSNR(Encoder) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FW | FH | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 4096 | 2048 | -2.6% | -0.3% | -1.8% | -2.7% | -0.3% | -1.9% | -2.7% | -0.3% | -1.8% | -2.8% | -0.3% | -1.8% | 8.8% | 12.5% | 7.2% |
| GasLamp | 4096 | 2048 | -2.4% | 0.5% | 4.6% | -2.5% | 0.4% | 4.6% | -2.4% | 0.5% | 4.7% | -2.5% | 0.5% | 4.6% | 11.1% | 18.0% | 29.3% |
| Skateboarding_in_lot | 4096 | 2048 | -3.1% | -5.9% | -4.3% | -3.1% | -5.8% | -4.5% | -2.8% | -5.8% | -4.4% | -2.8% | -5.8% | -4.6% | 9.9% | 0.3% | 7.1% |
| Chairlift | 4096 | 2048 | -2.5% | -0.1% | 4.6% | -2.7% | -0.2% | 4.5% | -2.7% | -0.1% | 4.8% | -2.7% | -0.2% | 4.4% | 9.2% | 11.1% | 16.1% |
| KiteFlite | 4096 | 2048 | -3.1% | -0.8% | -0.1% | -3.1% | -0.8% | 0.0% | -3.3% | -0.6% | 0.0% | -3.3% | -0.9% | 0.0% | 12.8% | 15.8% | 15.7% |
| Harbor | 4096 | 2048 | -3.6% | -2.4% | -1.9% | -3.7% | -2.5% | -2.3% | -3.5% | -2.5% | -2.1% | -3.8% | -2.4% | -2.2% | 15.5% | 12.2% | 12.2% |
| PoleVault | 3840 | 1920 | -3.7% | -3.7% | -3.9% | -3.8% | -3.8% | -4.0% | -3.8% | -3.6% | -3.8% | -3.8% | -3.7% | -3.9% | 11.0% | 8.7% | 8.9% |
| AerialCity | 3840 | 1920 | -2.6% | 0.3% | -0.4% | -2.8% | 0.3% | -0.5% | -2.5% | 0.2% | -0.6% | -2.7% | 0.3% | -0.6% | 6.7% | 13.9% | 12.8% |
| DrivingInCity | 3840 | 1920 | -2.3% | 0.1% | 0.6% | -2.3% | 0.1% | 0.9% | -2.2% | 0.2% | 0.8% | -2.2% | 0.2% | 0.8% | 4.6% | 10.0% | 11.9% |
| DrivingInCountry | 3840 | 1920 | -2.8% | -0.5% | 0.5% | -2.9% | -0.7% | 0.2% | -2.9% | -0.6% | 0.3% | -2.9% | -0.9% | 0.1% | 9.6% | 14.0% | 13.6% |
| Overall |  |  | -2.9% | -1.3% | -0.2% | -3.0% | -1.3% | -0.3% | -2.9% | -1.3% | -0.2% | -3.0% | -1.3% | -0.3% | 9.9% | 11.6% | 13.5% |

# Conclusion

A simple spatially adaptive quantization significantly modifies the performances of the codec, at least in the case of ERP. It is then proposed to include such adaptive QP when generating the anchors for a fair comparison between the different proposed projections.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).