| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0042 |
| --- | --- |

| Title: | AHG8: On cross-format S-PSNR-NN |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Xiaoyu Xiu, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | 858-210-4819 yuwen.he@interdigital.com xiaoyu.xiu@interdigital.com yan.ye@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

In Geneva, cross-format spherical quality metrics were included into the 360 common test conditions as defined in JVET-E1030. For cross-format quality metrics, the original video and the reconstructed video to be evaluated have different projection formats and/or different resolutions. S-PSNR-NN is one of the cross-format metrics. In order to avoid the impact due to interpolation filters, S-PSNR-NN measures the distortion between samples at nearest neighbor integer positions in the projection plane, instead of interpolating at fractional sample positions. However, when S-PSNR-NN is measured across two projection planes with different formats and/or resolutions, the nearest neighbor rounding operation in each projection plane may cause the reference sample and the reconstructed sample to no longer correspond to the same position on the unit sphere. In such case, cross-format S-PSNR-NN may not provide reliable quality measurement and coding performance comparison. This document proposes a modified S-PSNR-NN calculation method to solve this problem. It is asserted that simulation results validate that the BD rates reported by the modified cross-format S-PSNR-NN are consistent with other metrics, such as end to end S-PSNR-NN and WS-PSNR.

# Introduction

The cross-format spherical metrics in 360 common test conditions [1] will measure the quality difference between the original video and the reconstructed video without converting the reconstructed video to the same projection format and resolution as the original video. Denote S as the point on the unit sphere. Denote g as the point with fractional precision in the original video project plane, where g corresponds to S on the sphere. Denote r as the point with factional precision in the reconstructed video projection plane, where r also corresponds to the same point S on the sphere. Denote g’ and r’ as the nearest neighbor integer sampling point of g and r, respectively. Due to the fact that the original projection plane and the reconstructed projection plane are in different formats and/or resolutions, after nearest rounding, g’ and r’ may no longer correspond to the same point on the sphere. Thus, calculating the sample error between g’ and r’ may not provide accurate quality measurement between the original and the reconstructed video.

To solve this problem of sample misalignment in cross-format S-PSNR-NN calculation, the proposed cross-format S-PSNR-NN calculation will allow fractional sample position in the reconstruction video, and apply interpolation to derive the sample value at the fractional sample position. There is no interpolation applied to original video; therefore, the characteristics of the original video are not changed during the metric calculation. In contrast, the cross-format S-PSNR-I applies interpolation to both the original video and the reconstructed video, and it changes the sample values in the original video during metric calculation, which is not desirable.

The proposed cross-format S-PSNR-NN calculation follows the steps below (shown in Figure 1):

Step 1) Map point S (S is from the set of points uniformly sampled on sphere) to point g in the original video projection plane;

Step 2) Round point g to nearest neighbor point g’ at integer sampling position in the original video projection plane;

Step 3) Perform inverse coordinate mapping of the point g’ back onto the sphere at S(g’);

Step 4) Perform coordinate mapping from the spherical coordinate S(g’) to the reconstructed video projection plane at the position r;

Step 5) Apply interpolation to derive the sample value at point r using its neighboring sample values at integer sampling positions if r is not located at integer sampling position; use the sample value without any interpolation if r is at integer sampling position;

Step 6) Calculate error between the sample value at point g’ in the original video and the interpolated sample value at point r in the reconstructed video.

Figure 1. Proposed cross-format S-PSNR-NN calculation using interpolation in reconstructed video

It is worth noting that steps 1) to 4) can be pre-calculated at the sequence level, and only steps 5) and 6) need to be performed on-the-fly for metric calculation. If the original video and the reconstructed video have the same projection format and the same resolution, the proposed cross-format S-PSNR-NN will achieve the same result as that calculated with the end-to-end S-PSNR-NN.

# Simulation results

The same interpolation filters as those used for other cross-format spherical metrics are used in the proposed cross-format S-PSNR-NN calculation. The interpolation is only applied to the reconstructed video and no interpolation is applied to original video. The simulation was performed with HM-16.15 based 360Lib-2.1 [2] according to 360 common test conditions [1]. Table 1, Table 2, Table 3 and Table 4 list CMP, OHP1, ISP and SSP coding performance compared to ERP coding in terms of end to end WS-PSNR in 360Lib-2.1, the existing cross-format S-PSNR-NN in 360Lib-2.1, the proposed cross-format S-PSNR-NN and the cross-format S-PSNR-I in 360Lib-2.1. Full results are provided in the attached spreadsheets. From the results, the BD gain reported by the proposed cross-format S-PSNR-NN is consistent with end-to-end WS-PSNR. Compared to the existing cross format S-PSNR-NN implementation in 360Lib-2.1, the proposed modification significantly improves consistency with other metrics.

It is therefore suggested to replace the existing cross-format S-SPSNR-NN with the proposed cross-format S-PSNR-NN calculation in the next release of 360Lib.

Table 1. CMP vs ERP

| CMP vs. ERP | WS-PSNR (End to End) in 360Lib-2.1 |  |  | S-PSNR-NN (Cross Format) in 360Lib-2.1 |  |  | Proposed S-PSNR-NN (Cross Format) |  |  | S-PSNR-I (Cross Format) in 360Lib-2.1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | -2.2% | 4.8% | -0.4% | -48.3% | 7.1% | 1.5% | -2.3% | 4.7% | -0.3% | -2.3% | 4.8% | -0.3% |
| GasLamp | 2.4% | 5.4% | 10.7% | -44.7% | 8.1% | 13.4% | 2.1% | 5.5% | 10.8% | 2.1% | 5.4% | 10.8% |
| Sb_in_lot | -8.5% | -9.6% | -10.8% | -38.2% | -6.5% | -7.8% | -8.8% | -9.8% | -11.0% | -8.8% | -9.8% | -11.0% |
| Chairlift | -18.2% | -13.3% | -14.1% | -44.0% | -11.5% | -12.0% | -18.1% | -13.1% | -13.9% | -18.1% | -13.1% | -14.0% |
| KiteFlite | -4.9% | -1.5% | -3.8% | -48.8% | 0.7% | -2.2% | -5.2% | -1.5% | -3.8% | -5.1% | -1.5% | -3.8% |
| Harbor | 1.1% | -3.0% | -2.1% | -51.9% | -0.4% | 0.3% | 1.0% | -3.1% | -2.2% | 0.9% | -3.1% | -2.2% |
| PoleVault | -2.4% | -5.8% | -4.9% | 9.3% | -3.6% | -3.5% | -2.2% | -5.7% | -4.8% | -1.3% | -5.0% | -4.4% |
| AerialCity | 4.2% | 3.7% | 3.6% | 7.8% | 3.6% | 2.9% | 4.2% | 3.8% | 3.8% | 4.3% | 3.8% | 3.8% |
| DrivingInCity | 4.8% | 7.2% | 9.2% | 9.5% | 6.3% | 7.2% | 4.6% | 7.1% | 9.0% | 5.5% | 7.1% | 9.0% |
| DrivingInCountry | -14.6% | -14.3% | -13.6% | -10.3% | -14.7% | -14.3% | -14.5% | -14.2% | -13.5% | -13.5% | -14.1% | -13.5% |
| Overall | -3.8% | -2.7% | -2.6% | -25.9% | -1.1% | -1.5% | -3.9% | -2.6% | -2.6% | -3.6% | -2.5% | -2.6% |

Table 2. OHP1 vs ERP

| OHP1 vs. ERP | WS-PSNR (End to End) in 360Lib-2.1 |  |  | S-PSNR-NN (Cross Format) in 360Lib-2.1 |  |  | Proposed S-PSNR-NN (Cross Format) |  |  | S-PSNR-I (Cross Format) in 360Lib-2.1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | -2.5% | 8.1% | 3.5% | -47.8% | 10.3% | 5.1% | -2.2% | 8.1% | 3.4% | -2.3% | 8.1% | 3.3% |
| GasLamp | 1.6% | 6.4% | 10.7% | -43.9% | 9.0% | 12.9% | 1.7% | 6.4% | 10.9% | 1.9% | 6.4% | 10.8% |
| Sb_in_lot | -4.0% | -4.2% | -5.2% | -34.8% | -0.9% | -1.9% | -4.2% | -4.7% | -5.6% | -4.3% | -4.7% | -5.6% |
| Chairlift | -13.6% | -4.7% | -7.2% | -39.1% | -2.6% | -4.8% | -13.4% | -4.6% | -7.0% | -13.4% | -4.6% | -7.2% |
| KiteFlite | -4.6% | 2.3% | 0.4% | -47.5% | 4.9% | 2.3% | -4.6% | 2.5% | 0.5% | -4.6% | 2.4% | 0.2% |
| Harbor | 0.6% | 2.8% | 3.9% | -51.7% | 5.7% | 6.6% | 0.9% | 2.7% | 3.8% | 0.8% | 2.6% | 3.8% |
| PoleVault | -3.4% | -5.8% | -5.7% | 5.0% | -3.3% | -3.6% | -3.5% | -5.7% | -5.6% | -2.6% | -4.9% | -5.1% |
| AerialCity | 5.0% | 7.8% | 7.2% | 6.6% | 8.1% | 6.8% | 5.0% | 7.7% | 7.2% | 5.3% | 7.8% | 7.2% |
| DrivingInCity | 14.5% | 25.4% | 22.3% | 20.2% | 24.6% | 21.3% | 14.3% | 25.2% | 22.1% | 15.4% | 25.1% | 22.2% |
| DrivingInCountry | -7.5% | -5.0% | -7.0% | -4.3% | -5.2% | -7.4% | -7.3% | -4.9% | -7.1% | -6.5% | -4.9% | -7.0% |
| Overall | -1.4% | 3.3% | 2.3% | -23.7% | 5.1% | 3.7% | -1.3% | 3.3% | 2.3% | -1.0% | 3.3% | 2.3% |

Table 3. ISP vs ERP

| ISP vs. ERP | WS-PSNR (End to End) in 360Lib-2.1 |  |  | S-PSNR-NN (Cross Format) in 360Lib-2.1 |  |  | Proposed S-PSNR-NN (Cross Format) |  |  | S-PSNR-I (Cross Format) in 360Lib-2.1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | -5.1% | 5.5% | 0.9% | -48.6% | 7.9% | 2.7% | -5.1% | 5.4% | 0.9% | -5.1% | 5.5% | 0.9% |
| GasLamp | -2.4% | 2.3% | 7.7% | -45.5% | 4.7% | 10.0% | -2.4% | 2.3% | 7.7% | -2.4% | 2.3% | 7.6% |
| Sb_in_lot | -8.1% | -7.3% | -10.3% | -36.8% | -3.9% | -6.9% | -8.1% | -7.3% | -10.3% | -8.2% | -7.3% | -10.3% |
| Chairlift | -13.8% | -6.4% | -8.3% | -39.2% | -4.6% | -6.0% | -13.6% | -6.3% | -8.1% | -13.6% | -6.3% | -8.1% |
| KiteFlite | -7.0% | -0.7% | -2.3% | -48.3% | 1.7% | -0.6% | -6.9% | -0.6% | -2.2% | -7.0% | -0.6% | -2.3% |
| Harbor | -6.5% | -0.5% | -1.4% | -53.1% | 2.1% | 1.0% | -6.0% | -0.5% | -1.5% | -6.2% | -0.4% | -1.4% |
| PoleVault | -5.8% | -6.7% | -6.8% | 0.8% | -3.3% | -4.0% | -5.7% | -6.5% | -6.6% | -4.9% | -6.0% | -6.3% |
| AerialCity | 3.3% | 3.9% | 3.9% | -0.9% | 4.2% | 3.5% | 3.4% | 4.0% | 4.0% | 3.8% | 4.0% | 4.1% |
| DrivingInCity | 3.4% | 13.4% | 12.7% | 0.9% | 12.5% | 11.7% | 3.4% | 13.4% | 12.7% | 4.6% | 13.5% | 12.9% |
| DrivingInCountry | -9.4% | -8.8% | -9.1% | -4.7% | -9.2% | -9.7% | -9.3% | -8.7% | -9.0% | -8.2% | -8.6% | -8.9% |
| Overall | -5.2% | -0.5% | -1.3% | -27.5% | 1.2% | 0.2% | -5.0% | -0.5% | -1.2% | -4.7% | -0.4% | -1.2% |

Table 4. SSP vs ERP

| SSP(vert) vs. ERP | WS-PSNR (End to End) in 360Lib-2.1 |  |  | S-PSNR-NN (Cross Format) in 360Lib-2.1 |  |  | Proposed S-PSNR-NN (Cross Format) |  |  | S-PSNR-I (Cross Format) in 360Lib-2.1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | -8.7% | 2.9% | -2.9% | -50.6% | 5.2% | -1.1% | -8.6% | 2.9% | -2.8% | -8.7% | 2.9% | -2.8% |
| GasLamp | -5.5% | 1.3% | 6.7% | -47.0% | 4.4% | 9.5% | -5.3% | 1.4% | 6.9% | -5.2% | 1.4% | 6.8% |
| Sb_in_lot | -16.1% | -14.1% | -14.3% | -41.8% | -10.5% | -10.5% | -16.1% | -14.0% | -14.2% | -16.2% | -14.0% | -14.1% |
| Chairlift | -20.3% | -12.6% | -13.6% | -43.7% | -10.3% | -11.3% | -20.2% | -12.5% | -13.4% | -20.3% | -12.6% | -13.5% |
| KiteFlite | -9.8% | -2.7% | -3.1% | -49.8% | -0.5% | -1.5% | -9.7% | -2.6% | -2.9% | -9.8% | -2.6% | -3.0% |
| Harbor | -8.1% | 0.8% | 1.3% | -52.6% | 4.1% | 4.2% | -7.6% | 0.9% | 1.3% | -7.7% | 1.1% | 1.4% |
| PoleVault | -8.9% | -9.9% | -9.8% | 3.9% | -4.8% | -5.8% | -8.9% | -9.9% | -9.7% | -8.2% | -9.3% | -9.4% |
| AerialCity | -2.4% | 0.5% | -0.5% | 4.6% | 2.5% | 0.2% | -2.4% | 0.6% | -0.4% | -2.1% | 0.9% | -0.2% |
| DrivingInCity | -2.6% | 8.7% | 6.0% | 6.5% | 10.6% | 6.3% | -2.7% | 8.8% | 6.0% | -1.4% | 9.3% | 6.1% |
| DrivingInCountry | -20.3% | -14.9% | -15.9% | -14.2% | -14.4% | -15.8% | -20.3% | -14.8% | -15.8% | -19.1% | -14.8% | -15.7% |
| Overall | -10.3% | -4.0% | -4.6% | -28.5% | -1.4% | -2.6% | -10.2% | -3.9% | -4.5% | -9.9% | -3.8% | -4.4% |

# References

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 3600 video,” JVET-E1030, Jan. 2017, Geneva, CH.

- 360Lib software, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-2.1

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).