| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0053 |
| --- | --- |

| Title: | AHG 8: An improvement on compact octahedron projection with padding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ya-Hsuan Lee, Hung-Chih Lin, Jian-Liang Lin, Shen-Kai Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {louise.lee, hungchih.lin, jl.lin, shenkai.chang, cc.ju }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an improvement on the compact octahedron projection layout (COHP) by padding additional pixel samples at the discontinuous edges. The padded samples profit the compression efficiency and significantly reduce the appeared artifact when rendering around the discontinuous edges. As compared to the COHP without padding, the experimental results reportedly demonstrate that the BD-rate reduction provided by the proposed padding scheme achieves 0.7%, 1.6%, and 1.5% in end-to-end WS-PSNR-Y, WS-PSNR-U, and WS-PSNR-V, respectively, while the virtual artifacts on the discontinuous edges are also efficiently reduced.

# COHP Improvement with Padding Pixel Samples

In the previous meeting, we proposed a compact layout for octahedron projection (COHP) [1][2] with minimum discontinuous egdes. As shown in Fig. 1, the COHP layout has four discontinuous edges, labeled as #1~#4.

Fig. 1 Four discontinuous edges in COHP layout

In conventional video coding, the residuals could not be perfectly reconstructed because of the quantization operation, which introduces error on the DCT coefficients. The error on the quantized DCT coefficients are then spread inside the coding pixels after inverse DCT. The spread pixel error may result in unexpectedly visual artifacts, especially on the coding blocks containing the discontinuous edges.

To efficiently reduce the artifacts, the four discontinuous edges of the COHP layout are padded additionally before encoding. The padded samples are derived by vertically linear interpolation with two active pixels for each side, as shown in Fig. 2. Moreover, sample images for the COHP layout with and without padding samples are depicted in Fig. 3.

Fig. 2 Linear interpolation for padded samples

| (a) | (b) |
| --- | --- |
| (c) |  |
| (d) |  |

Fig. 3 Sample images (a) COHP layout [1][2] (b) COHP layout with padding samples (c) zoom-in of COHP layout [1][2] on discontinuous edge #3 (d) zoom-in of COHP layout with padding samples on discontinuous edge #3

# Experimental Results

The experiments on the COHP layout with the proposed padding scheme are conducted on 360Lib-2.0 [3] and HM version 16.14 [4] followed by the JVET common test conditions and evaluation procedures for 360° video [5][6]. The BD-rate reduction over the ten test sequences with various quality metrics is shown in Table I as the padding size is set to 8 pixels.

Table I. BD-rate performance (Anchor: COHP without padding; Test: Proposed)

|  | SPSNR-NN (End to End) |  |  | SPSNR-I (End to End) |  |  | CPP-PSNR (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | -0.3% | -1.1% | -1.0% | -0.3% | -1.0% | -0.9% | -0.3% | -1.0% | -1.0% | -0.3% | -1.1% | -1.0% |
| GasLamp | -0.8% | -1.6% | -1.6% | -0.7% | -1.5% | -1.5% | -0.7% | -1.6% | -1.5% | -0.7% | -1.6% | -1.5% |
| Skateboarding_in_lot | -0.8% | -1.8% | -2.2% | -0.7% | -1.5% | -2.0% | -0.7% | -1.5% | -2.0% | -0.7% | -1.6% | -2.2% |
| Chairlift | -1.3% | -1.9% | -2.0% | -1.3% | -1.8% | -1.8% | -1.3% | -1.9% | -1.9% | -1.3% | -2.0% | -2.0% |
| KiteFlite | -0.2% | -0.6% | -0.6% | -0.2% | -0.6% | -0.5% | -0.2% | -0.6% | -0.6% | -0.2% | -0.6% | -0.6% |
| Harbor | -1.0% | -2.0% | -1.7% | -1.0% | -1.9% | -1.8% | -0.9% | -1.9% | -1.8% | -0.8% | -2.0% | -1.8% |
| PoleVault | -0.3% | -0.4% | -0.5% | -0.3% | -0.4% | -0.4% | -0.3% | -0.4% | -0.4% | -0.3% | -0.5% | -0.5% |
| AerialCity | -1.4% | -2.4% | -2.1% | -1.3% | -2.2% | -2.0% | -1.3% | -2.2% | -2.0% | -1.3% | -2.5% | -2.3% |
| DrivingInCity | -0.9% | -2.2% | -1.8% | -0.9% | -2.1% | -1.7% | -0.8% | -2.1% | -1.7% | -0.9% | -2.5% | -1.9% |
| DrivingInCountry | -0.4% | -1.7% | -1.3% | -0.4% | -1.5% | -1.1% | -0.4% | -1.5% | -1.1% | -0.4% | -1.8% | -1.4% |
| Overall | -0.7% | -1.6% | -1.5% | -0.7% | -1.4% | -1.4% | -0.7% | -1.5% | -1.4% | -0.7% | -1.6% | -1.5% |

# Conclusion

The coding performance of the COHP layout is further improved in this contribution. The enhanced compression efficiency is achieved by padding pixel samples at the discontinuous edges. With the padding samples, the artifacts appeared at the discontinuous edges after rendering are noticeably reduced. Moreover, as compared to the COHP without padding, the simulation results report that the BD-rate saving provided by the proposed padding scheme accomplishes 0.7%, 1.6%, and 1.5% in end-to-end WS-PSNR-Y, WS-PSNR-U, and WS-PSNR-V, respectively. We thus recommend to adopt the proposed padding scheme on the octahedron projection for the improvement on the coding performance and subjective quality.

# References

- H.-C. Lin, C.-Y. Li, J.-L. Lin, S.-K. Chang, and C.-C. Ju, “AHG8: An efficient compact layout for octahedron format,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0142, Oct. 2016

- H.-C. Lin, C.-C. Huang, C.-Y. Li, Y.-H. Lee, J.-L. Lin, and S.-K. Chang, “AHG8: An improvement on the compact OHP layout,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-E0056, Jan. 2017

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- HM reference software, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/

- K. Suehring and X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, Feb. 2016

- J. Boyce, E. Alshina, A. Abbas, and Y. Ye, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-E1030, Jan. 2017

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).