| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0065 |
| --- | --- |

| Title: | 360Lib modifications for spherical rotation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qian Xu, Jill Boyce Yuwen He, Yan Ye | Tel: Email: | qian.z.xu@intel.com jill.boyce@intel.com yuwen.he@interdigital.com yan.ye@interidgital.com |
| Source: | Intel, InterDigital |  |  |

_____________________________

# Abstract

Modifications to the 360Lib software are proposed to support spherical rotation aligned with the the omnidirectional projection indication SEI message in JCTVC-Z1005. A software patch based on 360Lib- 2.1 is provided with the contribution.

# Proposal

In current 360Lib-2.1 software [3], the 3D XYZ coordinate system is defined as in Figure 1 (from JVET-E1003 [1]). The spherical rotation is performed with three rotation steps: roll counter-clockwise rotation in degrees around the x axis first, then pitch counter-clockwise rotation in degrees around the y axis, and finally yaw counter-clockwise rotation in degrees around the z axis. If the par configuration SVideoRotation is set as (roll, pitch, yaw), the backward rotation matrix is calculated as

R = Rz(yaw)*Ry(pitch)*Rx(roll),						        (1)

where

Rz(yaw) = cos⁡(yaw)-sin⁡(yaw)0sin⁡(yaw)cos⁡(yaw)0001	(2)

Ry(pitch) = cos⁡(pitch)0sin⁡(pitch)010-sin⁡(pitch)0cos⁡(pitch)	(3)

Rx(roll) = 1000cos⁡(roll)-sin⁡(roll)0sin⁡(roll)cos⁡(roll).	(4)

Figure 1. 3D XYZ coordinate and rotation definition used in 360Lib

Figure 2. Region (lighter shade) with yaw, pitch, and roll offsets

We propose to add support for the rotation definition of 360Lib to align with the omnidirectional projection indication SEI message in JCTVC-Z1005 [2], where the rotation is defined as follows (see Figure 2). First, yaw counter-clockwise rotation in degrees around the up vector, restricted to -180.0 to 180.0. Second, pitch counter-clockwise rotation in degrees around the right vector post yaw transform, restricted to -90.0 to 90.0. Last, roll clockwise-rotation in degrees around the forward vector post yaw and pitch transform, restricted to -180.0 to 180.0. Following this definition, the rotation matrix is

R = Rup(yaw)*Rright(pitch)*Rforward(-roll)					        (5)

Assume a right-handed coordinate system where -X is forward, -Z is right and +Y is up as in 360Lib, the rotation matrix above becomes

R = Ry(yaw)*Rz(-pitch)*Rx(roll) 						        (6)

where Ry(), Rz(), and Rx() are defined as in equation (2) to (4).

After the spherical rotation before encoding, the inverse spherical rotation needs to be applied after reconstruction for all end-to-end metrics calculation, and cross-format metrics calculation. The inverse rotation matrix is calculated as

R = Rx(-roll)* Rz(pitch)*Ry(-yaw)					                     (7)

The software patch is provided based on 360Lib-2.1.

# References

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib,” JVET-E1003, Jan. 2017, Geneva, CH.

- C. Fogg, J. Boyce, G. J. Sullivan, A. Tourapis, “Updated proposed draft text for omni-directional projection indication SEI message with equirectangular projection,” JCTVC-Z0051, Jan. 2017, Geneva, CH.

- 360Lib software, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-2.1

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).