| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0066 |
| --- | --- |

| Title: | AHG8: On inactive pixels in evaluation of 360-degree video projections |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | geertv@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The common test conditions for 360-degree video allow for coded frame sizes that are larger than the ERP anchor coding size, since only the number of active samples of the projection are counted. This coding size increase can be 6-10% depending on the projection, which significantly increases decoder complexity. A coding result reportedly indicates that there is a coding gain advantage associated with the increased coding size. The concern is that this may put compact projection formats at a disadvantage in the evaluation. Furthermore, the preliminary call for evidence document that includes 360-degree video, would allow the active pixel count to increase up to 3% beyond the ERP anchor. Therefore, it is suggested to use the ERP anchor coding size as the upper limit for the coding size for all projections under evaluation in the common test conditions and call for evidence for 360-degree video.

# Discussion

Per the common test conditions for 360-degree video [1], the ERP anchor coding size is 25% of the high fidelity 8K source ERP, and it is 75% of the high fidelity 4K source ERP. The coding size of the projections under evaluation is such that the number of active samples is approximately equal to the ERP anchor coding size [1]. Active samples refer to the samples in the projected frame that are utilized by the projection. For example, all samples of the cube map projection [2] (CMP) are active samples if the faces are arranged in 3×2 layout as is the case for the compact CMP [1]. On the other hand, the segmented sphere projection (SSP) [2] and recent proposals submitted to the Hobart meeting [3][4] have inactive samples in the projected frame. This document investigates what the impact of inactive samples is on the evaluation.

|  | 8K Source |  |  | 4K Source |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Projections | Coding Width | Coding Height | Coding Size % | Coding Width | Coding Height | Coding Size % |
| ERP | 4096 | 2048 | 100.0% | 3328 | 1664 | 100.0% |
| SSP [2] | 1216 | 7296 | 105.8% | 1008 | 6048 | 110.1% |
| JVET-F0036 [3] | 3648 | 2432 | 105.8% | 3024 | 2016 | 110.1% |
| JVET-F0052 [4] | 1216 | 7296 | 105.8% | 992 | 5952 | 106.6% |

Table 1. Comparison of coding sizes.

Table 1 enumerates the coding sizes per the common test conditions [1] and the coding sizes used in recent proposals. In summary, the coding sizes of SSP and recent proposals are 6-10% greater than the coding size of the ERP anchor.

The problem is that the coding size difference is significant and significantly increases decoder complexity, which is not the case for other projections that do not require increased coding size. In our opinion, this puts the compact projections without inactive samples at a disadvantage in the evaluation.

As an experiment to verify the impact of the increased coding size of SSP on the coding efficiency, the SSP coding size is decreased to the ERP anchor size: 1184×7104 for 8K and 960×5760 for 4K. The results are enumerated in Table 2. It is observed that the coding gain decreased from -10.3% per [1] to -9.8%. Therefore, the projections under evaluation without inactive samples are again at a disadvantage in the evaluation.

In addition, the preliminary joint call for evidence description [5] would allow the coded active sample count to be met within ±3% of the ERP anchor coded size. This would allow the active pixel count to additionally increase up to 3%, which potentially further disadvantages projections with 100% of the anchor coding size.

Based on this information, it is suggested to use the ERP anchor coding size as the upper limit for the coding size for all projections under evaluation in the common test conditions [1] and call for evidence [5].

| SSP(vert) vs. ERP |  |  | SPSNR-NN (End to End) |  |  | SPSNR-I (End to End) |  |  | CPP-PSNR (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Face W | Face H | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 1184 | 1184 | -8.2% | 3.1% | -2.9% | -8.2% | 3.0% | -2.8% | -8.2% | 3.0% | -2.9% | -8.1% | 3.1% | -2.9% |
| GasLamp | 1184 | 1184 | -5.1% | 1.3% | 7.0% | -5.1% | 1.2% | 6.9% | -5.2% | 1.2% | 6.8% | -5.1% | 1.3% | 6.9% |
| Skateboarding_in_lot | 1184 | 1184 | -15.7% | -13.9% | -14.4% | -15.7% | -13.9% | -14.4% | -15.6% | -13.8% | -14.4% | -15.5% | -13.7% | -14.3% |
| Chairlift | 1184 | 1184 | -19.8% | -12.1% | -13.2% | -19.8% | -12.1% | -13.2% | -19.9% | -12.1% | -13.3% | -19.9% | -12.1% | -13.3% |
| KiteFlite | 1184 | 1184 | -9.2% | -2.7% | -3.7% | -9.3% | -2.7% | -3.7% | -9.3% | -2.7% | -3.7% | -9.2% | -2.6% | -3.6% |
| Harbor | 1184 | 1184 | -7.1% | 0.4% | 0.2% | -7.2% | 0.5% | 0.4% | -7.6% | 0.5% | 0.3% | -7.4% | 0.6% | 0.4% |
| PoleVault | 960 | 960 | -9.6% | -11.0% | -10.3% | -8.4% | -10.9% | -10.2% | -8.4% | -11.0% | -10.5% | -9.6% | -11.0% | -10.3% |
| AerialCity | 960 | 960 | -0.3% | 3.7% | 3.2% | -0.1% | 3.1% | 2.7% | -0.1% | 2.6% | 2.3% | -0.2% | 3.8% | 3.2% |
| DrivingInCity | 960 | 960 | -2.8% | 10.7% | 8.5% | -1.8% | 9.8% | 7.7% | -1.7% | 9.3% | 7.5% | -2.7% | 11.0% | 8.6% |
| DrivingInCountry | 960 | 960 | -20.2% | -11.0% | -12.1% | -18.8% | -11.5% | -12.3% | -18.9% | -11.8% | -12.6% | -20.3% | -10.9% | -12.0% |
| Overall |  |  | -9.8% | -3.2% | -3.8% | -9.4% | -3.3% | -3.9% | -9.5% | -3.5% | -4.1% | -9.8% | -3.1% | -3.7% |

Table 2. BD-rates for SSP with coding size 100% of ERP anchor.

# Conclusion

The current common test conditions for 360-degree video [1] allow for coded frame sizes that are larger than the ERP anchor coding size. The reason is that the number of active samples of the projection are counted while inactive samples that increase the coding size are not counted. This coding size increase can be 6-10% depending on the projection, which significantly increases decoder complexity. A coding efficiency experiment with the SSP coding size configured equal to the ERP anchor coding size also indicates that there is a coding gain advantage of 0.5% associated with the increased coding size. The concern is that this may put compact projection formats at a disadvantage in the evaluation. Furthermore, the preliminary call for evidence document [5] would allow the active pixel count to additionally increase up to 3% beyond the ERP anchor. Therefore, it is suggested to use the ERP anchor coding size as the upper limit for the coding size for all projections under evaluation in the common test conditions [1] and call for evidence [5].

# References

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360-degree video”, JVET-E1030, Jan. 2017.

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib”, JVET-E1003, Jan. 2017.

- A. Abbas, D. Newman, “AHG8: Rotated Sphere Projection for 360 Video”, JVET-F0036, Mar. 2017.

- Y.-H. Lee, J.-L. Lin, S.-K. Chang, C.-C. Ju, “AHG8: EAP-based segmented sphere projection with padding”, JVET-F0052, Mar. 2017.

- M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki, “Revised Preliminary Joint Call for Evidence on Video Compression with Capability beyond HEVC”, JVET-F0040, Mar. 2017.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).